|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.1020 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (11/2003) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Quality of service and performance - Generic and user-related aspects |  |  |  |
|  | Performance parameter definitions for quality of speech and other voiceband applications utilizing IP networks |  |  |  |
|  | ITU-T Recommendation G.1020 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.1020 Performance parameter definitions for quality of speech and other voiceband applications utilizing IP networks

Summary The transmission of speech and other voiceband applications over packet networks brings with it new, sometimes unique forms of quality degradation. There are many existing definitions for packet network performance parameters, yet the desire to control the quality of non-elastic isochronous applications requires additional, complementary information. The purpose of this Recommendation is to define packet network and terminal performance parameters that better reflect the perceived quality of the target applications. It is largely focused on quality impairments resulting from delay variation and packet loss which are peculiar to IP and other packet-based technologies, and which do not appear in traditional TDM networks. It discusses the interactions and trade-offs among these packet impairments, and describes mechanisms such as de-jitter buffers and packet loss concealment for reducing their effects on the quality of speech and other applications. However, this Recommendation avoids overlap by making reference to existing definitions wherever possible. The parameters defined by this Recommendation extend beyond the IP layer in many cases. End-to-end packet system (combination of end terminals and network) parameters are also necessary to determine the speech/voiceband quality. Clauses 5, 6 and 7 collect the parameter definitions for source terminals, packet networks, and destination terminals (with overall parameters), respectively. Appendix I provides information on packet loss distributions and packet loss models. Appendix II gives an example adaptive de-jitter buffer emulator.

Source ITU-T Recommendation G.1020 was approved on 13 November 2003 by ITU-T Study Group 12 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g., interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		3

3	Definitions		3

4	Abbreviations		4

5	Source terminal packet parameters		5

5.1	Analogue/digital conversion clock accuracy (free-running)		5

5.2	Packet information field size		6

5.3	Packet overhead		6

5.4	Source terminal delay		6

5.5	Source terminal delay variation		6

6	Packet network performance parameters		7

6.1	Summary of network performance parameters		7

6.2	Additional network parameters recommended		7

7	Destination terminal and overall packet parameters		9

7.1	Discussion of destination packet processing		9

7.2	Taxonomy of de-jitter buffer types/parameters and models		10

7.3	System frequency offset, using destination clock as reference		13

7.4	Packet loss concealment (type, delay)		14

7.5	Overall delay (including source, network and destination)		14

7.6	Time-scale discontinuities in post-de-jitter and PLC stream		15

7.7	Overall (frame/packet) loss (including network and destination)		15

Appendix I – Packet loss distributions and packet loss models		16

I.1		Introduction		16

I.2		Common packet loss models		16

I.3		Example packet trace		18

I.4		Bibliography to Appendix I		19

Appendix II – Example adaptive de-jitter buffer emulator		20

Appendix III – BIBLIOGRAPHY		22

Introduction

The transmission of speech and other voiceband applications over packet networks brings with it new, sometimes unique forms of quality degradation. There are many existing definitions for packet network performance parameters, yet the desire to control the quality of non-elastic isochronous applications requires additional, complementary information. The purpose of this Recommendation is to define packet network and terminal performance parameters that better reflect the perceived quality of the target applications, extending beyond the IP layer in many cases. End-to-end packet system (combination of end terminals and network) parameters are also necessary to determine the speech/voiceband quality, and this Recommendation defines them as well.

ITU-T Recommendation G.1020

Performance parameter definitions for quality of speech and other voiceband applications utilizing IP networks

# 1	Scope

This Recommendation defines a set of performance parameters for packet networks and end terminals that can assist in quantifying the end-to-end quality of speech and other voiceband applications. It is largely focused on quality impairments resulting from delay variation and packet loss which are peculiar to IP and other packet-based technologies, and which do not appear in traditional TDM networks. It discusses the interactions and trade-offs among these packet impairments, and describes mechanisms such as de-jitter buffers and packet loss concealment for reducing their effects on the quality of speech and other applications.

This Recommendation recognizes existing performance parameter definitions, and avoids duplication. Many factors that determine the quality of speech and voiceband applications are common to both TDM and IP-based networks, and are addressed in existing Recommendations. In the parlance of ITU-T Rec. I.350, the scope of this Recommendation is limited to the information transfer function of the 3  3 matrix, and only to the bearer channel. Call processing aspects of connection access and disengagement (e.g., dialtone delay and post-dialling delay) are not considered in this Recommendation. Furthermore, this Recommendation does not specify numerical objectives for packet networks or end terminals, although this will be the subject of follow-on work.

Figure 1 illustrates this scope, along with some other specifications with their areas of coverage. This Recommendation only defines parameters that describe packet terminal and packet transmission impairments that are unique to speech and voiceband application quality assessment.

Figure 1/G.1020 – Scope of ITU-T Rec. G.1020 as it relates
to other performance specifications

Note that the number of IP networks between terminals is not limited in these definitions.

Other ITU-T Recommendations supplement the parameters provided in this Recommendation. For example, transmission planning over hybrid Internet/PSTN connections is covered in ITU-T Rec. G.177. Still, other Recommendations specify such parameters in the context of assessing the performance of IP packet transfer on international data communication services (e.g., ITU-T Rec. Y.1540). Network performance objectives for different QoS classes of IP-based services are described in ITU-T Rec. Y.1541, and end-to-end one-way delay objectives are specified in ITU-T Rec. G.114.

New Recommendations that complement ITU-T Rec. G.1020 are anticipated. Call processing aspects of the connection are in development. The ITU-T is also currently working on new Recommendations dealing with situations where the network is mainly an IP network with islands of PSTN, and where the network is mainly a PSTN network with islands of IP. Finally, there is ITU-T work to specify the performance objectives for VoIP terminals and gateways, and the methods to assess the performance by measuring metrics related to the end-to-end quality of VoIP.

The definitions of packet transmission parameters that are unique to ATM networks are explicitly out of the scope of this Recommendation.

This Recommendation should be particularly useful to those new to the area of Voice over IP (VoIP) who want to gain a better understanding of the factors affecting the quality of these telecommunication systems. Developers of telecommunication equipment can use the parameters defined in this Recommendation to specify relevant aspects of their contribution to end-to-end performance. Service providers can use these parameters to effectively summarize performance of IP network solutions.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	ITU-T Recommendation G.107 (2003), The E-model, a computational model for use in transmission planning.

–	ITU-T Recommendation G.113 (2001), Transmission impairments due to speech processing.

–	ITU-T Recommendation G.114 (2003), One-way transmission time.

–	ITU-T Recommendation G.177 (1999), Transmission planning for voiceband services over hybrid Internet/PSTN connections.

–	ITU-T Recommendation I.356 (2000), B-ISDN ATM layer cell transfer performance.

–	ITU-T Recommendation P.51 (1996), Artificial mouth.

–	ITU-T Recommendation P.57 (2002), Artificial ears.

–	ITU-T Recommendation Y.1540 (2002), Internet protocol data communication service – IP packet transfer and availability performance parameters.

–	ITU-T Recommendation Y.1541 (2002), Network performance objectives for IP-based services.

# 3	Definitions

This Recommendation defines the following terms:

3.1	ear reference point: A virtual point for geometric reference located at the entrance to the listener's ear, traditionally used for calculating telephonometric loudness ratings [P.57].

3.2	terminal input measurement point: A measurement point in the physical medium connecting a terminal to an IP network that is crossed as IP packets leave the IP network and enter the terminal. This measurement point is as close to the terminal as possible.

3.3	IP terminal: An endpoint device intended for connecting to an IP network to support speech communications. These devices can be dedicated (e.g., a telephone set) or general purpose (e.g., a computer running an application that performs the terminal function).

3.4	terminal output reference point: A measurement point in the physical medium connecting a terminal to an IP network that is crossed as IP packets leave the terminal and enter the IP network. This measurement point is as close to the terminal as possible.

3.5	de-jitter buffer: A buffer designed to remove the delay variation (i.e., jitter) in packet arrival times. Data is put into the de-jitter buffer at a variable rate (i.e., whenever they are received from the network), and taken out at a constant rate.

3.6	mouth reference point: The point on the reference axis, 25 mm in front of the lip plane [P.51].

3.7	real-time signal: A signal accurately representing acoustic or electrical signals in the time domain.

3.8	receive electrical reference point: An electrical point of reference that is equivalent to the ear reference point from a terminal delay measurement perspective.

3.9	send electrical reference point: An electrical point of reference that is equivalent to the mouth reference point from a terminal delay measurement perspective.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ADC	Analogue-to-Digital Converter

ATM	Asynchronous Transfer Mode

DAC	Digital-to-Analogue Converter

DSCP	Differentiated Services Code Point

Dst	Destination

HDLC	High-level Data Link Control

IETF	Internet Engineering Task Force

IP	Internet Protocol

IPErr	Errored IP Packet Count

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IPER	IP Packet Error Ratio

IPLR	IP Packet Loss Ratio

IPPM	IP Performance Metrics working group

IPRE	IP packet transfer Reference Event

IPSLB	IP Packet Severe Loss Block

IPSLBR	IP Packet Severe Loss Block Ratio

IPTD	IP Packet Transfer Delay

MAPDV2	Mean Absolute Packet Delay Variation 2

NA	Not Available

PSTN	Public Switched Telephone Network

QoS	Quality of Service

RFC	Request For Comments

RSVP-TE	Resource Reservation Protocol – Traffic Engineering

RTP	Real-time Transport Protocol

RTPErr	Errored RTP Packet Count

RTCP	Real-time Transport Control Protocol

SPR	Spurious Packet Ratio

Src	Source

TDM	Time Division Multiplex

UDP	User Datagram Protocol

UDPErr	Errored UDP Packet Count

UNI	User-Network Interface

# 5	Source terminal packet parameters

This clause gives the relevant sending terminal packet parameters that have a direct effect on perceived speech and voiceband application quality. Figure 2 indicates the positions of measurement points and system components.

## 5.1	Analogue/digital conversion clock accuracy (free-running)

The relative frequency offset of a clock may be specified as:

Δffnominal=fmeasured−fnominalfnominal

The measured frequency error should be minimized, and use of an atomic frequency reference for measurement is preferred (the nominal frequency should be as close to ideal as practical).

The long-term (or end-of-life) accuracy of the clock oscillator technology (e.g., quartz crystal) may also be given if known, as it establishes an upper bound on the frequency offset.

For example, assume that the nominal frequency of the analogue/digital conversion oscillator is 8000 Hz. A measurement on the source terminal oscillator indicates a frequency of 8000.0027 Hz. The relative frequency offset is then 0.0027/8000 = 3.38 × 10–7. Disciplined quartz oscillators can usually achieve offsets of 1 × 10–6 based on their accuracy specifications.

Note that it should be possible to infer the sending clock frequency from measurements of the source packet rate (under favourable circumstances, for example, when silence suppression is disabled). This would permit a measurement using externally available signals. The method is for further study.

Figure 2/G.1020 – Source terminal diagram and reference points

## 5.2	Packet information field size

The Packet information field size specifies the amount of encoded voiceband waveform that a packet contains. This size must be expressed unambiguously, using as many of the following units of measure as necessary:

1)	The number of 8-bit octets of encoded voiceband signals and supporting information (e.g., bit associated with forward error correction to aid packet loss concealment, or bits associated with encryption).

2)	The number of encoder frames (the specific encoder and native frame size must also be specified).

3)	The amount of continuous waveform time represented by the encoded bits in the field.

Typical information fields contain one or two encoder frames and combine 10 or 20 ms of waveform time in a single packet. Annex A/G.114 provides guidance on the calculation of delay for various coders and packet information field sizes.

## 5.3	Packet overhead

The total octets appended to the packet information field should be counted separately for each protocol layer header. The total octets dedicated to non-information headers may be counted as the packet exits the terminal output reference point, therefore, including the effects of header compression, if any. The octet size of packets dedicated to media flow control, status/performance reports (e.g., RTCP), or other non-media carrying packets should be counted separately. Some typical overhead contributors are (in octets):

RTP	12		UDP	8		IPv4	20		IPv6	40

HDLC Encapsulation	8		Flag	1

## 5.4	Source terminal delay

The source terminal delay is the interval defined by the time that a signal enters the mouth reference point and the time that the first bit of the corresponding encoded, packetized signal exits the terminal output reference point. When appropriate, the send electrical reference point may be substituted for the mouth reference point. By definition, the source terminal delay includes the entire packetization/depacketization time, and test waveforms and methods must retain sufficient information to assess packetization time variability due to alignment between the test signal and the packet boundaries. For example, the test signal must be of sufficient length to span packet boundaries, permitting boundary identification in time. The portion of the signal that is carried by the earliest part of the packet payload should be used.

NOTE – This delay will include source terminal delay variation if present, and appropriate statistics should be applied to summarize the variation.

## 5.5	Source terminal delay variation

The fundamental notion of a 1-point delay variation parameter is the comparison between the actual packet emission pattern and the intended (usually periodic) emission pattern. Some variations of this definition include a "skipping clock" adjustment, as in ITU-T Rec. I.356.

The source terminal delay variation is defined as the time difference between the first bit of a packet emission at the terminal output reference point and the ideal periodic reference time. For the first packet in a flow, the ideal periodic reference time is set equal to the emission time. Subsequent packets emissions are compared to this periodic time reference, as shown below:

Source Terminal Delay Variation =t(packetn)−t(referencepacketn)

where t(reference_packet_n) is the emission time of packet_n of the ideal periodic reference stream. The time interval of measurement, along with appropriate statistics, should be provided.

NOTE – Long interval measurements may include the undesired effects of source frequency offset. Variation due to source frequency offset should be noted and removed as a measurement error, when possible.

# 6	Packet network performance parameters

Standards for IP layer packet transfer performance (see e.g., ITU-T Recs Y.1540, Y.1541 and IETF RFCs 2330, 2678 through 2681, 3357 and 3393) include the following parameters: One-way transfer delay, delay variation, and packet loss. These parameters must be mapped to the application layer to adequately estimate user impact.

## 6.1	Summary of network performance parameters

Table 1 summarizes the IP network performance parameters relevant to this Recommendation, and the relationships among network and terminal parameters that form the basis for overall system parameters. Reading left to right, each row identifies a parameter and indicates how it may be combined with other parameters to derive a specific overall performance parameter for one aspect of the end-to-end or user-user quality (although the exact formulas are given in later clauses).

Table 1/G.1020 – Summary of IP network performance parameters and mapping to overall/user-user parameters

| IP network parameter (Note) | Translation to overall | Overall parameter |
| --- | --- | --- |
| Transfer delay (IPTD, mean) | IPTD + Src delay + Dst delay | Mean user-user delay |
| Delay variation (IPDV, 99.9%tile minus the minimum) | Combine with Src delay variation distribution | Contributes to Dst delay, or audio frame loss |
| Delay jump (possibly captured by RFC 3393, for example) | May come from network path/facility change, or may only appear at de-jitter buffer output | Audio time-scale discontinuity |
| Errored packet (headers) | IPErr + UDPErr + RTPErr  | Audio frame loss (packet or codec frame discard) |
| Reordered packet (Appendix VII/Y.1540) | (may be considered lost)  | Audio frame loss |
| Lost packet | IP loss + (all audio defects)  | Audio frame loss (pre-concealment) |
| IP severe loss block (IPSLB) | (depends on block duration) | Call cut-off |
| Loss patterns (e.g., RFC 3357) | Complete stream loss/arrival  | Burst length/Consecutive loss |
| Packet rate (inferred from other system characteristics) | Difference of source and destination terminal ADC and DAC oscillators | System frequency offset (relative to destination) |
| NOTE – From ITU-T Rec. Y.1540, unless noted. |  |  |

## 6.2	Additional network parameters recommended

All the fundamental single packet outcomes are defined in ITU-T Rec. Y.1540 (and IETF RFCs listed in Bibliography). However, it is possible to derive additional parameters of interest when streams or flows of packets are considered, as in VoIP planning and measurement.

### 6.2.1	Consecutive packet loss event

For cases where successive packets, sent in a periodic stream (according to RFC 3432, for example), are designated as lost according to the definition of lost packet outcome in ITU-T Rec. Y.1540, then the length of the event should be specified as the number of packets lost in sequence. This length should be recorded separately for each event. Following a measurement encountering multiple consecutive loss events, the count for each event length should also be recorded. Sequence numbers contained in packet headers may assist this measurement.

### 6.2.2	Degraded second

A degraded second outcome occurs for a block of packets observed during a 1-second interval when the ratio of lost packets at the egress UNI to total packets in the corresponding second interval at the ingress UNI exceeds D%. Sequence numbers and timestamps contained in packet headers may be used to aid in this measurement.

The value of D is provisionally set at 15%, and may change on the basis of further experience or study. For example, if a flow of packets at 50 packets per second is impaired by 8 losses (16%), then the quality will be degraded whether the losses are consecutive or distributed throughout the second.

### 6.2.3	Short-term IP delay variation/jitter quantification

The following clauses provide two approaches to short-term jitter quantification. When the distribution of delays over short intervals is available, then the first approach, based on short-term range is recommended. However, if the complete time series of delay variation is known, then the approach based on mean absolute packet delay variation may provide additional information.

#### 6.2.3.1	Approach based on short-term range

This definition is consistent with Appendix II/Y.1541.

Short-term IP delay variation is defined as the maximum IPTD minus the minimum IPTD during a given short measurement interval.

IPDVShortTerm=IPTDmax−IPTDmin

where:

IPTDmax	is the maximum IPTD recorded during the short measurement interval;

IPTDmin	is the minimum IPTD recorded during the short measurement interval.

This is a simple and fairly accurate method for calculating IPDV in real-time. The length of the short measurement interval is for further study. The measurement interval influences the ability of the metric to capture low and high frequency variations in the IP packet delay behaviour.

To be consistent with other parameter definitions in this Recommendation, a measurement interval of 1 second is provisionally agreed.

Many values of IPDVShort_Term are measured over a longer time interval (comprising many short measurement intervals). The 99.9th percentile of these IPDVShort_Term values is expected to meet the Y.1541 objective of 50 ms (note that this objective was established for a 1-minute measurement interval and the percentile is evaluated on a per-packet basis, assuming a 50-packet per-second sending rate or higher).

As an example, assume 1200 one-second measurements of IPDVShort_Term, collected over 20 minutes. If two or more measurements of IPDVShort_Term exceed 50 ms, then the Y.1541 objective may not have been met during a few intervals, and a more exact evaluation of the objective is warranted.

#### 6.2.3.2	Approach based on mean absolute packet delay variation

An alternative approach is to determine the mean absolute packet delay variation with regard to a short-term average or minimum value – termed here the adjusted absolute packet delay variation. This may provide a more meaningful relationship to de-jitter buffer behaviour.

The short-term jitter is computed for current packet (i) whose delay is designated ti. Packet (i) is compared to a running estimate of the mean delay (using the 16 previous packet delays), and assigned either a positive or negative deviation value.

mean delay Di = (15Di-1 + ti-1) / 16

positive deviation Pi = ti – Di	if ti > Di (Ni is NA)

negative deviation Ni = Di – ti	if ti < Di (Pi is NA)

if ti = Di then Pi is NA and Ni is NA

We compute mean absolute packet delay variation 2 (MAPDV2) for packet (i) as:

MAPDV2 = mean(Pi) + mean(Ni)

where mean(Pi) is the overall P including the current packet.

# 7	Destination terminal and overall packet parameters

This clause gives the relevant destination terminal packet parameters that have a direct effect on perceived speech and voiceband application quality, and a set of overall packet parameters. Figure 3 indicates the positions of measurement points and system components.

Figure 3/G.1020 – Destination terminal components

## 7.1	Discussion of destination packet processing

Figure 4 depicts the process through which IP packet parameters/impairments (transfer delay, delay variation, and packet loss and errors) can be mapped to application layer performance in terms of overall loss and delay.

Figure 4/G.1020 – Mapping IP packet performance to application layer

At the bottom of the figure, packets arrive with various impairments due to the source terminal and network(s), or never arrive (lost). The arriving packets are processed as they move up the protocol stack to remove as much impairment as possible. We show that some forms of impairment (errors, jitter) map into other impairments (overall loss, overall delay).

Figure 4 captures the trade-off between application level delay and loss as a threshold on the range of delay variation based on the size of the de-jitter buffer. Packets with delay variation in the "white" range are accommodated, while packets with larger variation (in the "black" range) would be discarded. A larger de-jitter buffer can accommodate packets with greater delay variation, hence, fewer packets would be lost overall at the expense of larger overall delay. Conversely, a smaller de-jitter buffer will produce less overall delay, but expose a larger fraction of packets to be discarded by the terminal and increase the overall loss.

## 7.2	Taxonomy of de-jitter buffer types/parameters and models

There are two main types of de-jitter buffers, fixed length and adaptive length. De-jitter buffers can be constructed in many different ways, including the following attributes identified in Table 2. The values of applicable de-jitter buffer parameters must be known when assessing the performance of a system.

Table 2/G.1020 – De-jitter buffer types and parameters

| Type | Attributes | Possibilities |  |
| --- | --- | --- | --- |
| Fixed (and adaptive) | Size (configure maximum and nominal or minimum) | Integer number of packets | Fractional number of packets |
| Adaptive | Control | Timed decay if no over/under flow | Evaluate loss ratio (configure lowest acceptable threshold, and minimum packet count between adjustments) |
|  | Adjustment | Timed | Silence gaps only |
|  | Initialization | First packet | Small sample |
|  | Adjustment granularity | Packet size | Fraction of packet size |
|  | Restores packet order | Yes | No |
|  | Voiceband data mode | Detect 2100 Hz tone, set to maximum length | None |

### 7.2.1	Destination terminal delay and loss assessment

The primary contributors to delay are variable sources. This clause illustrates how the de-jitter buffer size and network IP packet delay variation overlap, and how one must carefully accumulate specific delay statistics to achieve the correct delay totals.

Figure 5/G.1020 – Delay of packet networks and network elements

Figure 5 shows some of the elements of a VoIP path that contribute to end-end delay. At the sender, packetization time can be significant. There is usually a variable delay as packets traverse the network. At the receiver, the de-jitter buffer exists to accommodate the delay variation and deliver a continuous payload stream. We note that packets with the minimum source and network delay spend the maximum time in the de-jitter buffer; likewise packets that encounter delays longer than the minimum spend less time in the buffer. There is also some minimum amount of time each packet must spend in a buffer at the receiver; possibly as long as an entire packet.

The following subclauses give an overview of the process to combine the IP layer loss and delay with the additional contributions from destination terminal higher-layer functions, such as the de-jitter buffer.

#### 7.2.1.1	Loss

Depending on the type of de-jitter buffer, there will be some criteria for determining whether or not each specific packet in a flow is accommodated or discarded. The result can completely change the distribution of overall packet losses. For example, if random bit errors are causing packets to fail the UDP checksum, then packet losses will have a random distribution as they proceed to the application layer. But, if several consecutive packets experience excessive delays, then the additional discards due to the limitations of the de-jitter buffer will make the overall loss distribution appear bursty rather than random. Therefore, categorization of the loss distribution must take place at the application layer (using techniques such as in Appendix I, or the burst ratio, see Appendix I/G.113), before estimation of application performance with tools such as the E-model (see ITU-T Rec. G.107).

There are circumstances where packet order may change during network transfer. Some de-jitter buffers are unable to restore order to these reordered packets (ITU-T Rec. Y.1540) and, in this case, they are designated as discarded packets.

#### 7.2.1.2	Delay

The correct value of buffer delay to combine with the other delays depends on the descriptive statistics available. For example, the mean network delay should be summed with the average de-jitter buffer occupation time (and other delays) to obtain an overall average delay. This method allows for buffer adaptation, needing only the average queuing time for all packets in the assessment time interval. On the other hand, if only the minimum network delay is known, it should be summed with the maximum de-jitter buffer occupation (or size used, and other delays) to give an overall delay.

We next consider initialization for a fixed-size de-jitter buffer. If the first packet to arrive has the minimum transfer delay, then the receiver will buffer the packet for the entire time requested, and buffering size will be as expected. Fortunately, many packets arrive at or near the minimum transfer time, so this case has a fair likelihood.

On the other hand, if the first packet has a rather long delay, then more buffer space will be needed to accommodate the "early" arrival of packets at or near the minimum transfer time, and the de-jitter buffer will contribute more than the expected delay to the overall calculation.

#### 7.2.1.3	Fixed de-jitter buffer and destination terminal model

The simplest effective model of loss due to a fixed de-jitter buffer is to designate as discarded all packets whose delay is greater than the minimum transfer delay for the packet stream plus the (fixed) de-jitter buffer length.

The following procedure provides a mapping between the IP and application layers, assuming fixed length de-jitter buffers for destination terminal performance assessment.

1)	Designate as lost all packets failing the UDP Checksum.

2)	Designate as discarded all packets whose delay is greater than the minimum transfer delay for the packet stream plus the (fixed) de-jitter buffer length, or whose delay is less than the established minimum.

3)	Sum the mean network delay (IPTD) with the average source and destination terminal delay to obtain an overall average delay, OR, sum the minimum source terminal delay and the minimum network delay with the maximum destination terminal delay (reflecting the maximum de-jitter buffer occupation when network jitter is present, or maximum size used).

In step 2 above, the minimum transfer delay should be evaluated over short intervals (provisionally a value of 10 seconds is used). The minimum for the first interval is used throughout, unless the short term minimum grows beyond the accommodation range of the buffer. In this case, no packets will be delivered to the upper layers and the de-jitter buffer must be reset to the new minimum, as would likely occur in practice. Alternatively, if the short term minimum should fall to a value where a high percentage (provisionally 50%) of packets would be designated lost due to early arrival, the de-jitter buffer must be reset to the new minimum.

When calculating the overall impairment contribution of a fixed de-jitter buffer, the distribution of delay variation determines the proportion of packets that would be discarded. The distribution of packet delays that are accommodated (not discarded) can be used to calculate the mean de-jitter buffer occupation delay, as follows:

Mean occupation delay = [De-jitter Buffer Size] – (Mean Delay of Accom. Packets – Min. Delay)

This mean delay can be added with other destination terminal delay constants to produce an estimate of the mean destination terminal delay. If the exact delay distribution is not available, then there is agreement that a value of half the de-jitter buffer size may be substituted for the mean occupation delay in calculations supporting network planning.

If the maximum destination terminal delay is needed in calculations, then the maximum de-jitter buffer size can be added with other destination terminal delay constants to produce an estimate of the maximum delay.

#### 7.2.1.4	Adaptive de-jitter buffer model

The fixed de-jitter buffer in item 2 above may be replaced with an adaptive de-jitter buffer emulation, as described in this clause when a time series of packet stream information is at hand.

The time series of packet arrivals may be used with an adaptive de-jitter buffer emulator to determine the buffer size dynamics and the mean de-jitter buffer occupation time (delay) over the series. This mean delay can be combined with other destination terminal delay constants to produce an estimate of the mean destination terminal delay.

An example of an adaptive de-jitter buffer emulator is provided in Appendix II.

### 7.2.2	Destination terminal delay

The destination terminal delay is the interval defined as beginning when the first bit of a packet representing a waveform signal enters the terminal input reference point and ending when the corresponding decoded, depacketized signal exits the ear reference point. When appropriate, the receive electrical reference point may be substituted for the ear reference point.

NOTE – This delay may vary if an adaptive de-jitter buffer is present, and appropriate statistics should be applied to summarize the variation.

Since, by definition, the source terminal delay includes the entire packetization/depacketization time, destination terminal packet test signals should be constructed such that they occupy the earliest part of the payload. In this way, source and destination terminal delay measurements will be conducted at equivalent moments with respect to packetization time.

## 7.3	System frequency offset, using destination clock as reference

The system frequency offset may be assessed by monitoring sequence number increment per unit time or accumulated time-stamp offset, and is a measure of the difference between source and destination analogue/digital conversion clock accuracy. The relative frequency offset between the source and destination clocks may be specified as:

ΔffDestination=fSource−fDestinationfDestination

This frequency offset may be used to determine the rate of buffer overflow or underflow events at the destination terminal, usually resulting in additional packet losses, by noting that the fractional frequency offset is equivalent to the time shift (t) over an observation interval (T).

ΔffDestination=−ΔtT

(noting that frequency and time period differences have a negative relationship). For example, assume that the source frequency is 7999.997 Hz, the destination frequency is 8000.001 Hz, and the de-jitter buffer length is 20 ms. Since the destination's D/A converter clock is reading information faster than the source supplies it, the de-jitter buffer will eventually empty, or underflow. At a relative offset of

7999.997−8000.0018000.001=−5×10−7

(where the minus sign indicates that the source clock pulses occur slower than the corresponding pulses at the destination), the time shift equal to the entire de-jitter buffer will accumulate in an observation interval of

T=−(Δt=0.02)−5×10−7=40000sec=667min

## 7.4	Packet loss concealment (type, delay)

Many standardized speech coders have a native Packet Loss Concealment (PLC), and it is sufficient to specify whether or not the PLC is on or off, and account for any additional delay. For example, Appendix I/G.711 adds at least 3.75 ms algorithmic delay, and possibly more depending on implementation. This PLC may be used with other waveform coders, such as G.726.

Many forms of non-standardized PLC have emerged in practice, particularly for G.711 and other waveform coders. If these are used, the specific PLC algorithm and delay should be specified.

Note that a PLC that sounds best to human users may not meet the needs of voiceband modem carrier detectors. If there is a signal classifier for voiceband data or fax modems, and a special PLC is selected to improve their operation on packet networks, then the PLC type and signal classification method should be specified.

## 7.5	Overall delay (including source, network and destination)

Following the analysis of the de-jitter buffer and other destination terminal components, as described throughout 7.2.1 and illustrated in Figure 5, it is possible to combine them with the delay of the source terminal and network(s) to determine the system's overall delay. The following formulas are acceptable, and their use is determined by the specific delay statistics available for computation.

When the mean delays for all components are at hand:

Overall_Mean_Delay = mean(source_delay) + mean(net_delay) + mean(destination_delay)

As Figure 5 clearly illustrates, the minimum delays for source terminal and network can be combined with the maximum destination terminal delay to obtain an estimate using constants:

Overall Delay (constant) = min(source_delay) + min(net_delay) + max(destination_delay)

When measured directly, the overall delay is the interval defined from the time that a signal enters the mouth reference point and ending at the time when the corresponding signal exits the ear reference point (or equivalent reference points).

One method for direct measurement of overall delay has been documented in Annex B of [B-11].

Several examples of the overall mean delay calculation are present in Appendix III/Y.1541. These examples utilize various network configurations and reference terminals with several packet sizes, de-jitter buffers, and forms of packet loss concealment. Appendix X/Y.1541 goes on to calculate E-model R values for each of these cases [G.107].

## 7.6	Time-scale discontinuities in post-de-jitter and PLC stream

A time-scale discontinuity is defined as a sudden change in the overall delay, measured from the mouth reference point to the ear reference point. This parameter captures how often the user's time reference shifts, due to the network path, the de-jitter buffer, or both.

## 7.7	Overall (frame/packet) loss (including network and destination)

This parameter may be expressed in terms of packets, or coder frames. It is important to understand the relationship between frame loss and packet loss. For example, when two frames are combined in each packet, then every packet lost implies a burst of two frame losses and the decoder/PLC must attempt recovery under these more difficult circumstances than the isolated single frame loss.

### 7.7.1	Overall (frame/packet) loss ratio

The overall loss ratio for an evaluation interval is defined as follows:

Overall Loss Ratio=1 – (Total_pkt_sent – Lost_net – Lost_error_check – Discarded_de - jitter- buffer – Discarded_reordering)Total _pkt_sent

### 7.7.2	Overall (frame/packet) loss model

In order to assess the impact of losses and discards on VoIP applications, it is useful to consider distribution of these impairments over time. Typical approaches include the Gilbert-Elliott model and similar Markov-based models. [B-10] specifies the use of a Gilbert-Elliott model to describe packet loss and discard distribution and gives an example of a four-state Markov model to derive these parameters. Appendix I provides a description of these models, and gives an example of a typical packet loss/discard distribution. The typical output parameters are the average gap length and loss/discard density, and the average burst length and loss/discard density.

### 7.7.3	Overall consecutive (frame/packet) loss event count

After examining a stream of packets sent according to [B-9], and a set of successive packets have been designated as lost or discarded according to all the relevant loss/discard criteria in the overall loss ratio parameter, then the length of the event should be specified as the number of packets lost in sequence. This length should be specified separately for each event. The count of each event size should also be provided as a result. Sequence numbers contained in packet headers may be used to aid in this measurement.

### 7.7.4	Pitfalls and errors in calculating overall parameters

One simplified approach to obtain the overall end-to-end delay has been to take the mean IP packet transfer delay, and combine it with constants for other elements in the mouth-to-ear path. This procedure may produce errors because of variable delays in some terminal components (e.g., the de-jitter buffer), or because the variable delay elements are ignored.

Another potential pitfall would be to use the packet loss ratio as measured by a test receiver that allows, for example, 3 seconds before declaring a packet lost, thereby underestimating the loss ratio. A typical de-jitter buffer would have much less tolerance for long delays beyond the norm. Hereto, knowledge of the de-jitter buffer figures prominently in the mapping between IP packet performance and loss at the application layer.

Appendix I

Packet loss distributions and packet loss models

## I.1	Introduction

It is generally understood that packet loss distribution in IP networks is "bursty", however, there is less certainty concerning the use of specific loss models and, in fact, some misunderstanding related to some commonly used models, for example the Gilbert Model. This appendix outlines some key packet loss models, provides some analysis of packet loss data, discusses the degree of "fit" of models and data and proposes the use of a 4-state Markov model to represent loss distribution.

## I.2	Common packet loss models

### I.2.1	Historical background

Much of the early work on loss or error modelling occurred in the 1960's in relation to the distribution of bit errors on telephone channels.

One approach used was a Markov or multi-state model. Gilbert [13] appears to be the first to describe a burst error model of this type, later extended by Elliott [10] and [11] and Cain and Simpson [6]. Blank and Trafton [3] produced higher state Markov models to represent error distributions.

Another approach was to identify the statistical distribution of gaps. Mertz [17] used hyperbolic distributions and Berger and Mandelbrot [2] used Pareto distributions to model inter-error gaps. Lewis and Cox [16] found that in measured error distributions there was strong positive correlation between adjacent gaps.

Packet loss modelling in IP networks seems to have followed a similar course, although the root cause of loss (typically congestion) may be different to that of bit errors (typically circuit noise or jitter).

### I.2.2	Bernoulli or independent model

The most widely used model is a simple independent loss channel, in which a packet is lost (or bit error occurs) with a probability Pe. For some large number of packets N, then the expected number of lost packets is N  Pe. The loss probability can be estimated by counting the number of lost packets and dividing this by the total number of packets transmitted.

### I.2.3	Gilbert and Gilbert-Elliott models

The most widely known burst model is the Gilbert model [13] and a variant known as the Gilbert-Elliott model [10] and [11]. These are both two state models that transition between a "good" or gap state 0 and a "bad" or burst state 1 according to state transition probabilities P01 and P11:

i)	Gilbert model

a)	State 0 is a zero loss/error state;

b)	State 1 is a lossy state with independent loss probability Pe1.

ii)	Gilbert-Elliott model

a)	State 0 is a low loss state with independent loss probability Pe0;

b)	State 1 is a lossy state with independent loss probability Pe1.

It is often assumed that the Gilbert model lossy state corresponds to a "loss" state, i.e., that the probability of packet loss in state 1 is 1, however, this is incorrect (it would be more proper to describe this as a 2-state Markov model). This leads to analysis of packet loss burstiness in terms solely of consecutive loss which misses the effects of longer periods of high loss density. As illustrated in [14], these long periods of high loss density can have significant effect on voice over IP services.

For example, consider the following:

Loss pattern	     000001100101010110110000000000000000000

Correct application of Gilbert Model – burst length 15, burst density 60%

Incorrect application of Gilbert Model – mean burst length 1.5 packets

Further examples are given in [21].

### I.2.4	Markov models

A Markov model is a general multi-state model in which a system switches between states i and j with some transition probability p(i, j).

A 2-state Markov model has some merit in that it is able to capture very short-term dependencies between lost packets, i.e., consecutive losses [1], [4], [15] and [19]. These are generally very short duration events (say 1-3 packets in length) but occasional link failures can result in very long loss sequences extending to tens of seconds [5].

By combining the 2-state model with a Gilbert-Elliott model, it is possible to capture both very short duration consecutive loss events and longer lower density events.

Figure I.1/G.1020 – 4-state Markov model

This 4-state Markov model [7] and [12] represents burst periods, during which packets are received and lost according to a first 2-state model and gap periods during which packets are received and lost according to a second 2-state model. The states have the following definition:

a)	State 1 – packet received successfully;

b)	State 2 – packet received within a burst;

c)	State 3 – packet lost within a burst;

d)	State 4 – isolated packet lost within a gap.

For example, using the loss pattern below:

Loss pattern		000001100101010110110000000000000000000000001000000000

State			111113322323232332331111111111111111111111114111111111

It is common to define a gap state with respect to some criteria, for example, a loss rate lower than some limit or some consecutive number of received packets. A convenient definition is that a burst must be a longest sequence beginning and ending with a loss during which the number of consecutive received packets is less than some value Gmin (a suitable value for Gmin for use with Voice over IP services would be 16 whereas, for use with video services, a higher value of say 64 or 128 would be preferable).

## I.3	Example packet trace

There are two charts shown below that were obtained from analysis of an example IP trace. The first chart shows a scatter diagram of burst length versus burst weight (Gilbert model). Burst length is the distance in packets between the first and last lost packets in a burst and burst weight is the number of packets lost within the burst. It can be clearly seen that bursts of up to 100 packets in length occur, and have a typical loss density of 20-25%.

Figure I.2/G.1020 – Trace W3 scatter diagram of burst length vs weight for packet loss only

The second chart shows a scatter diagram of burst length versus burst weight for losses and discards, assuming a 50 ms fixed de-jitter buffer size. This shows a much larger number of bursts indicating that jitter was a significant problem on this trace. Burst density extends out to 500 packets and mean burst density is approximately 30%.

Figure I.3/G.1020 – Trace W3 scatter diagram of burst length vs weight
for packet loss and packet discard (50 ms jitter buffer)

## I.4	Bibliography to Appendix I

[1]	ALTMAN (E.), AVRACHENKOV (K.), BARAKAT (C.): TCP in the Presence of Bursty Losses, Performance Evaluation 42, pp. 129-147, 2000.

[2]	BERGER (J.M.), MANDELBROT (B.): A New Model for Error Clustering in Telephone Circuits, IBM J R&D, July 1963.

[3]	BLANK (H.A), TRAFTON (P.J.): A Markov Error Channel Model, Proc. Nat Telecomm Conference, 1973.

[4]	BOLOT (J.C.), VEGA GARCIA (A.): The case for FEC based error control for packet audio in the Internet, ACM Multimedia Systems, 1997.

[5]	BOUTREMANS (C.), IANNACCONE (G.), DIOT (C.): Impact of Link Failures on VoIP Performance, Sprint Labs Technical Report IC/2002/015.

[6]	CAIN (J.B.), SIMPSON (R.S.): The Distribution of Burst Lengths on a Gilbert Channel, IEEE Trans IT, 15 September 1969.

[7]	CLARK (A.): Modelling the effects of burst packet loss and recency on subjective voice quality, IPtel 2001 workshop.

[8]	DRAJIC (D.), VUCETIC (B.): Evaluation of Hybrid Error Control Systems, IEE Proc. F. Vol. 131, April 1984.

[9]	EBERT (J-P.), WILLIG (A.): A Gilbert-Elliott Model and the Efficient Use in Packet Level Simulation, TKN Technical Report 99-002.

[10]	ELLIOTT (E.O.): Estimates of Error Rates for Codes on Burst Noise Channels, BSTJ 42, September 1963.

[11]	ELLIOTT (E.O.): A Model of the Switched Telephone Network for Data Communications, BSTJ 44, January 1965.

[12]	ETSI TIPHON TS 101 329-5 VI.1.2 (2002), Annex E, QoS measurements for Voice over IP.

[13]	GILBERT (E.N.): Capacity of a Burst Noise Channel, BSTJ, September 1960.

[14]	ITU-T SG 12 D.139, Study of the relationship between instantaneous and overall subjective speech quality for time-varying quality speech sequences, France Telecom.

[15]	JIANG (W.), SCHULZRINNE (H.): Modelling of Packet Loss and Delay and their effect on Real Time Multimedia Service Quality, NOSSDAV, 2000.

[16]	LEWIS (P.), COX (D.): A Statistical Analysis of Telephone Circuit Error Data, IEEE Trans COM-14, 1966.

[17]	MERTZ (P.): Statistics of Hyperbolic Error Distributions in Data Transmission, IRE Trans CS-9, December 1961.

[18]	SANNECK (H.), CARLE (G.): A Framework Model for Packet Loss Metrics Based on Loss Runlengths, Proc. ACM MMCN, January 2000.

[19]	YAJNIK (M.), MOON (S.), KUROSE (J.), TOWSLEY (D.): Measuring and Modelling of the Temporal Dependence in Packet Loss, UMASS CMPSCI Tech. Report 98-78.

[20]	ITU-T SG 12, Delayed Contribution D.22, A framework for setting packet loss objectives for VoIP, AT&T, October 2001.

[21]	ITU-T SG 12, Delayed Contribution D.97, Packet Loss Distributions and Packet Loss Models, Telchemy, January 2003.

Appendix II

Example adaptive de-jitter buffer emulator

This example of a de-jitter buffer emulator operates by tracking the short-term minimum delay and using this to position a time window equivalent in size to the de-jitter buffer size. The actual packet arrival time is compared to the time window to determine if the packet would be discarded or accommodated.

The output from this de-jitter buffer emulator is a packet loss/discard event associated with a count of the number of good packets (i.e., not lost or discarded), which is input to the packet loss distribution model.

The de-jitter buffer emulation algorithm determines the delay variation for each arriving RTP packet, based on the RTP timestamp/sequence number and a local clock. This approach is preferable to measuring packet-to-packet delay variation as it:

i)	handles out-of-order packets without requiring them to be buffered, which reduces computational complexity;

ii)	is able to detect mid-long term delay variations, due to congestion, route changes or timing drift.

The de-jitter buffer emulator operates as follows:

The first arriving RTP packet is the initial reference point with RTP timestamp Rref.

Set Nominal equal to the delay for packets arriving on time (configuration parameter).

Set Maximum Delay equal to the number of packets times the packet size (configuration parameter).

Define early window = Maximum – Nominal.

Define late window = Nominal.

For each RTP packet associated with a stream that passes the monitoring point:

Associate a local timestamp L with the arrival time of the RTP packet

Identify the RTP timestamp R of the packet

Estimate the expected arrival time of the RTP packet based on the reference RTP packet using the expression Lexpected = Lref + (R – Rref)

Estimate the delay variation of the RTP packet as D = L – Lexpected

If D < early window then

mark the packet as discarded

reset the reference point to this packet

If D > late window then

mark the packet as discarded

If packet is a duplicate of an already received packet, then silently discard

Maintain a sliding window of 32 packets, ordered by sequence number, by default marked as lost – mark packets within this window as accommodated or discarded

At the end of the window – identify packets as lost/discarded or accommodated

The early/late window can be dynamically adjusted to suit adaptive de-jitter buffer behaviour.

Adjustment algorithm:

Define threshold T1 equal to the lowest unacceptable rate of discards (a configurable parameter)

Define threshold T2 equal to the period between de-jitter buffer size downward adjustments (in packets, a configurable parameter)

Maintain a running average of late discards C1, with a scaling factor, S (typically 15)

C1 = (C1  (S – 1) + D)/S where D is 1 if packet discarded and 0 if not

Maintain a count of the packets received since the last late discard C2

if C1 exceeds a threshold, T1, and the buffer is less than the maximum then increase the buffer size, and reset C1;

if C2 exceeds a threshold, T2, and the buffer is more than the minimum then reduce the buffer size, and reset C2.

The maximum value of the time window, or de-jitter buffer maximum length, must be specified so that the emulator cannot grow the buffer to extreme values that would not be possible in practice.

Appendix III

BIBLIOGRAPHY

[B-1]	RFC 3550, RTP (2003), A Transport Protocol for Real-Time Applications.

[B-2]	RFC 2330 (1998), Framework for IP Performance Metrics.

[B-3]	RFC 2678 (1999), IPPM Metrics for Measuring Connectivity.

[B-4]	RFC 2679 (1999), A One-way Delay Metric for IPPM.

[B-5]	RFC 2680 (1999), A One-way Packet Loss Metric for IPPM.

[B-6]	RFC 2681 (1999), A Round-trip Delay Metric for IPPM.

[B-7]	RFC 3357 (2002), One-way Loss Pattern Sample Metrics.

[B-8]	RFC 3393 (2002), IP Packet Delay Variation Metric for IP Performance Metrics IPPM.

[B-9]	RFC 3432 (2002), Network performance measurement for periodic streams.

[B-10]	RFC 3611 (2003), RTP Control Protocol Extended Reports (RTPC XR).

[B-11]	ETSI TS 101 329-5 V1.1.2 (2002), TIPHON release 3, End-to-End Quality of Service in TIPHON Systems, Part 5 Quality of Service (QoS) Measurement Methodologies.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |