|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.1020 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (05/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Quality of service and performance - Generic and user-related aspects |  |  |  |
|  | Performance parameter definitions for quality of speech and other voiceband applications utilizing IP networks Amendment 1: New Annex A – VoIP gateway-specific reference points and performance parameters |  |  |  |
|  | ITU-T Recommendation G.1020 (2003) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.1020 Performance parameter definitions for quality of speech and other voiceband applications utilizing IP networks Amendment 1 New Annex A – VoIP gateway-specific reference points and performance parameters

Summary VoIP gateways are usually deployed to interconnect packet and circuit switched networks and require new reference points for delay and other parameters. This annex defines the gateway-specific reference points and parameters.

Source Amendment 1 to ITU-T Recommendation G.1020 (2003) was approved on 14 May 2004 by ITU-T Study Group 12 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

A.1	Introduction		1

A.2	Definitions		1

A.3	Source gateway parameters		1

A.4	Destination gateway parameters		2

A.5	Overall delay		3

ITU-T Recommendation G.1020

Performance parameter definitions for quality of speech and other 
voiceband applications utilizing IP networks

Amendment 1

New Annex A – 
VoIP gateway-specific reference points and
performance parameters

## A.1	Introduction

VoIP gateways are usually deployed to interconnect packet and circuit switched networks and require new reference points for delay and other parameters. This annex defines the gateway-specific reference points and parameters.

## A.2	Definitions

This Recommendation defines the following terms:

A.2.1	packet input reference point: A measurement point in the physical medium connecting an IP network to a gateway that is crossed as IP packets leave the IP network and enter the gateway. This measurement point is as close to the terminal as possible.

A.2.2	packet output reference point: A measurement point in the physical medium connecting a gateway to an IP network that is crossed as IP packets leave the gateway and enter the IP network. This measurement point is as close to the gateway as possible.

A.2.3	TDM input reference point: A measurement point in the physical medium connecting a Time Division Multiplex Network to a VoIP gateway. Signals that cross this point are packetized and enter the IP network. This measurement point is as close to the gateway as possible.

A.2.4	TDM output reference point: A measurement point in the physical medium connecting a gateway to a Time Division Multiplex Network. Signals that cross this point are carried to the end terminal. This measurement point is as close to the gateway as possible.

## A.3	Source gateway parameters

This clause gives the relevant source gateway packet parameters that have a direct effect on perceived speech and voiceband application quality. Figure A.1 indicates the positions of measurement points and system components.

Figure A.1/G.1020 – Source gateway diagram and reference points

We note that some gateways will include coders that provide bit rate compression, while others will simply packetize the PCM waveform, or provide other processing to the voiceband signals such as Fax Demod/Remod.

Most of the source terminal parameters defined in clause 5/G.1020 are relevant to source gateways. Some parameters require the following reference point substitutions in Table A.1.

Table A.1/G.1020 – Source Reference Point Substitution

| Source Terminal | Substitute Source Gateway Reference Point |
| --- | --- |
| Mouth Reference Point | TDM Input Reference Point |
| Send Electrical Reference Point | TDM Input Reference Point |
| Terminal Output Reference Point | Packet Output Reference Point |

The source terminal to source gateway parameter mapping is as follows.

Table A.2/G.1020 – Source Parameter Mapping

| Source Terminal Parameter | Source Gateway Parameter |
| --- | --- |
| Source Terminal Delay | Source Gateway Delay |
| Source Terminal Delay Variation | Source Gateway Delay Variation |

Parameters such as Packet Information Field Size require no modifications.

## A.4	Destination gateway parameters

This clause gives the relevant destination gateway packet parameters that have a direct effect on perceived speech and voiceband application quality. Figure A.2 indicates the positions of measurement points and system components.

Figure A.2/G.1020 – Destination gateway diagram and reference points

Most of the destination terminal parameters defined in clause 7/G.1020 are relevant to destination gateways. Some parameters require the reference point substitutions in Table A.3.

Table A.3/G.1020 – Destination Reference Point Substitution

| Destination Terminal | Substitute Destination Gateway Reference Point |
| --- | --- |
| Ear Reference Point | TDM Output Reference Point |
| Receive Electrical Reference Point | TDM Output Reference Point |
| Terminal Input Reference Point | Packet Input Reference Point |

The destination terminal to destination gateway parameter mapping is as follows.

Table A.4/G.1020 – Destination Parameter Mapping

| Destination Terminal Parameter | Destination Gateway Parameter |
| --- | --- |
| Destination Terminal Delay | Destination Gateway Delay |

Parameters such as Packet Loss Concealment require no modifications.

## A.5	Overall delay

When a gateway is present in the end-end path, the additional delay in the TDM network between the Mouth or Ear Reference point and the gateway must be included in the overall delay. The TDM network elements usually have fixed delays, so they can be added to the delay for the packet network components.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |