|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.108 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (09/2003) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS International telephone connections and circuits – General definitions |  |  |  |
|  | Application of the E-model: A planning guide Amendment 1: New Appendix I: The relationship between and interaction of talker echo and absolute delay |  |  |  |
|  | ITU-T Recommendation G.108 (1999) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| General definitions | G.100–G.109 |
| General Recommendations on the transmission quality for an entire international telephone connection | G.110–G.119 |
| General characteristics of national systems forming part of international connections | G.120–G.129 |
| General characteristics of the 4-wire chain formed by the international circuits and national extension circuits | G.130–G.139 |
| General characteristics of the 4-wire chain of international circuits; international transit | G.140–G.149 |
| General characteristics of international telephone circuits and national extension circuits | G.150–G.159 |
| Apparatus associated with long-distance telephone circuits | G.160–G.169 |
| Transmission plan aspects of special circuits and connections using the international telephone connection network | G.170–G.179 |
| Protection and restoration of transmission systems | G.180–G.189 |
| Software tools for transmission systems | G.190–G.199 |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.108 Application of the E-model: A planning guide Amendment 1 New Appendix I: The relationship between and interaction of talker echo and absolute delay

Source Amendment 1 to ITU-T Recommendation G.108 (1999) was agreed by ITU-T Study Group 12 (2001-2004) on 30 September 2003.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.108

Application of the E-model: A planning guide

Amendment 1

New Appendix I: The relationship between and
interaction of talker echo and absolute delay

This appendix is intended to provide guidance on the relationship between and interaction of talker echo and absolute delay by means of graphs which result from calculations with the E-model as contained in ITU-T Rec. G.107 (03/2003).

Figure I.1 illustrates the combined effects of talker echo in the presence of absolute delay, whereas Figures I.2a and I.2b illustrate the effects of  talker echo only (all three figures have been taken from ITU-T Rec. G.131).

It should be noted that, the upper graph in Figure I.1 illustrates the effect of  absolute delay in the absence of any echo (see also Figure 1 of ITU-T Rec. G.114).

Figures I.3 through I.7 illustrate the comparison of the effects of  talker echo only with the combined effects of talker echo and absolute delay; each for a given value of talker echo loudness rating TELR.

NOTE – All graphs are based on the 2003 version of the E-model. The graphs depicting the effects of talker echo have been calculated with Ta = 0, T = Tr/2 by means of recursive calculation; whereas the graph for absolute delay, in the absence of echo, is based on Ta = 0, Tr = 0 as provided in ITU-T Rec. G.114.

From a transmission planner's perspective, it is important to understand that the effects of talker echo may be eliminated by the proper deployment of an echo cancelling device, which performs in accordance with, or even exceeds, the requirements given in ITU-T Rec. G.168.

In addition, it is also very important to understand that the effect of absolute delay cannot be eliminated by the deployment of technology. Talker echo and absolute delay may effect different sequences of the same conversation, which is the reason why, under normal circumstances, the combination of both effects has to be taken as the basis for transmission planning.

However, the differences, as shown in Figures I.3 through I.7, may be taken as useful brackground information for tutorial purposes; any application, in exceptional cases, is left to the individual transmission planner's discretion.

Figure I.1/G.108 – Combined effects of talker echo in the presence of absolute delay

Figure I.2a/G.108 – Effects of talker echo based on E-model

Figure I.2b/G.108 – Effects of talker echo based on E-model

Figure I.3/G.108 – Comparison of the effects of talker echo with the combined
effects of talker echo and absolute delay; for TELR = 65 dB

Figure I.4/G.108 – Comparison of the effects of  talker echo with the combined
effects of talker echo and absolute delay; for TELR = 55 dB

Figure I.5/G.108 – Comparison of the effects of talker echo with the combined
effects of talker echo and absolute delay; for TELR = 45 dB

Figure I.6/G.108 – Comparison of the effects of talker echo with the combined
effects of talker echo and absolute delay; for TELR = 35 dB

Figure I.7/G.108 – Comparison of the effects of talker echo with the combined
effects of talker echo and absolute delay; for TELR = 25 dB

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |