|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.108 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 2 (03/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS International telephone connections and circuits – General definitions |  |  |  |
|  | Application of the E-model: A planning guide Amendment 2: New Appendix II – Planning examples regarding delay in packet-based networks |  |  |  |
|  | ITU-T Recommendation G.108 (1999) – Amendment 2 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| General definitions | G.100–G.109 |
| General Recommendations on the transmission quality for an entire international telephone connection | G.110–G.119 |
| General characteristics of national systems forming part of international connections | G.120–G.129 |
| General characteristics of the 4-wire chain formed by the international circuits and national extension circuits | G.130–G.139 |
| General characteristics of the 4-wire chain of international circuits; international transit | G.140–G.149 |
| General characteristics of international telephone circuits and national extension circuits | G.150–G.159 |
| Apparatus associated with long-distance telephone circuits | G.160–G.169 |
| Transmission plan aspects of special circuits and connections using the international telephone connection network | G.170–G.179 |
| Protection and restoration of transmission systems | G.180–G.189 |
| Software tools for transmission systems | G.190–G.199 |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.108 Application of the E-model: A planning guide Amendment 2 New Appendix II – Planning examples regarding delay in packet-based networks

Source Amendment 2 to ITU-T Recommendation G.108 (1999) was agreed on 31 March 2004 by ITU-T Study Group 12 (2001-2004).

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.108

Application of the E-model: A planning guide

Amendment 2

New Appendix II – Planning examples regarding delay in
packet-based networks

This appendix provides guidance for the transmission planner on how to deal with the delay occuring in packet-based networks in conjunction with VoIP terminals and gateways.

For the calculations quoted in this appendix, the algorithm of the E-model has been taken from ITU-T Rec. G.107 at the time of publication. In case a later revision of ITU-T Rec. G.107 does contain an improved version of the algorithm, this appendix will still provide valid guidance for tutorial purposes. However, for actual transmission planning tasks it should, in any case, be referred to the most recent version of ITU-T Rec. G.107.

For illustratory purposes, the following scenarios have been investigated:

1)	two VoIP terminals interconnected via a packet-based network that complies with ITU-T Rec. Y.1541, Class 0;

2)	two VoIP islands interconnected via the digital PSTN;

3)	a mixed connection between a VoIP terminal (connected to a LAN) and an analogue phone (connected to the PSTN).

In addition, two codecs, G.711 and G.729A, have been considered for each scenario, while no other  impairments have been considered (proper echo control assumed).

For the E-model calculations, all parameters which are not explicitly mentioned have been set to their default values as per Table 2/G.107.

The scenario in Figure II.1 describes a connection between two VoIP terminals via a packet-based network with a total network delay of 100 ms. The network delay is composed of the fixed delay and the value of the delay variation (jitter). It should be noted that, in this example, the packet-based network performs better than the maximum values set out in ITU-T Rec. Y.1541.

Figure II.1/G.108 – VoIP terminals with G.711 and WAN according to Y.1541 Class 0

For the E-model calculations, three different cases of terminal delay have been investigated:

•	VoIP terminal send delay = 20 ms, VoIP terminal receive delay = 30 ms

total delay = Ts + Twan + Tr  = (20 + 100 + 30) ms = 150 ms

R = 90: Users very satisfied

•	VoIP terminal send delay = 35 ms, VoIP terminal receive delay = 65 ms

total delay = Ts + Twan + Tr = (35 + 100 + 65) ms = 200 ms

R = 86: Users satisfied

•	VoIP terminal send delay = 50 ms, VoIP terminal receive delay = 100 ms

total delay = Ts + Twan + Tr = (50 + 100 + 100) ms = 250 ms

R = 79: Some users dissatisfied

The quality as perceived by the user in these example calculations is identical for both sides, because the example is symmetrically composed.

The scenario in Figure II.2 describes a connection between two VoIP islands via the PSTN with a delay in the PSTN of 125 ms. The delays in the LANs are 5 ms each which are composed of the fixed delay and the value of the delay variation (jitter).

Figure II.2/G.108 – VoIP islands with G.711 and PSTN

For the E-model calculations, three different cases of terminal delay (and gateway delay) have been investigated:

•	VoIP terminal send delay = 20 ms, VoIP terminal receive delay = 30 ms

VoIP gateway send delay = 20 ms, VoIP gateway receive delay = 30 ms

total delay = Ts + Tlan + Tr + Tpstn +  Ts + Tlan + Tr = (20 + 5 + 30 + 125 + 20 + 5 + 30) ms = 235 ms

R = 81: Users satisfied

•	VoIP terminal send delay = 35 ms, VoIP terminal receive delay = 65 ms

VoIP gateway send delay = 35 ms, VoIP gateway receive delay = 65 ms

total delay = Ts + Tlan + Tr + Tpstn +  Ts + Tlan + Tr = (35 + 5 + 65 + 125 + 35 + 5 + 65) ms = 335 ms

R = 69: Many users dissatisfied

•	VoIP terminal send delay = 50 ms, VoIP terminal receive delay = 100 ms

VoIP gateway send delay = 50 ms, VoIP gateway receive delay = 100 ms

total delay = Ts + Tlan + Tr + Tpstn +  Ts + Tlan + Tr = (50 + 5 + 100 + 125 + 50 + 5 + 100) ms = 435 ms

R = 59: Nearly all users dissatisfied

The quality as perceived by the user in these example calculations is identical for both sides, because the example is symmetrically composed.

The scenario in Figure II.3 describes a connection between one VoIP island and an analogue terminal connected to the PSTN with a delay in the PSTN of 25 ms. The delay in the LAN is 5 ms which is composed of the fixed delay and the value of the delay variation (jitter). In addition, the VoIP gateway provides an echo canceller according to ITU-T Rec. G.168 with sufficient tail delay covering the echo path via the local office.

Figure II.3/G.108 – VoIP terminal with G.711 and PSTN with analogue terminal

For the E-model calculations, three different cases of terminal delay (and gateway delay) have been investigated:

•	VoIP terminal send delay = 20 ms, VoIP terminal receive delay = 30 ms

VoIP gateway send delay = 20 ms, VoIP gateway receive delay = 30 ms

total delay = Ts + Tlan + Tr + Tpstn = (20 + 5 + 30 + 25) ms = 80 ms

R = 91: Users very satisfied

•	VoIP terminal send delay = 35 ms, VoIP terminal receive delay = 65 ms

VoIP gateway send delay = 35 ms, VoIP gateway receive delay = 65 ms

total delay = Ts + Tlan + Tr + Tpstn = (35 + 5 + 65 + 25) ms = 130 ms

R = 90: Users very satisfied

•	VoIP terminal send delay = 50 ms, VoIP terminal receive delay = 100 ms

VoIP gateway send delay = 50 ms, VoIP gateway receive delay = 100 ms

total delay = Ts + Tlan + Tr + Tpstn = (50 + 5 + 100 + 25) ms = 180 ms

R = 88: Users satisfied

The quality as perceived by the user in these example calculations is identical for both sides, because the example is symmetric in its behaviour.

The scenario in Figure II.4 describes a connection between two VoIP terminals via a packet-based network with a total network delay of 100 ms. The network delay is composed of the fixed delay and the value of the delay variation (jitter). It should be noted that, in this example, the packet-based network performs better than the maximum values set out in ITU-T Rec. Y.1541. In addition, low bit-rate coding as per ITU-T Rec. G.729A is deployed; thus, Ie = 11.

Figure II.4/G.108 – VoIP terminals with G.729A and WAN according to Y.1541 Class 0

For the E-model calculations, three different cases of terminal delay have been investigated:

•	VoIP terminal send delay = 20 ms, VoIP terminal receive delay = 30 ms

total delay = Ts + Twan + Tr = (20 + 100 + 30) ms = 150 ms

R = 79: Some users dissatisfied

•	VoIP terminal send delay = 35 ms, VoIP terminal receive delay = 65 ms

total delay = Ts + Twan + Tr = (35 + 100 + 65) ms = 200 ms

R = 75: Some users dissatisfied

•	VoIP terminal send delay = 50 ms, VoIP terminal receive delay = 100 ms

total delay = Ts + Twan + Tr = (50 + 100 + 100) ms = 250 ms

R = 68: Many users dissatisfied

The quality as perceived by the user in these example calculations is identical for both sides, because the example is symmetrically composed.

The scenario in Figure II.5 describes a connection between two VoIP islands via the PSTN with a delay in the PSTN of 125 ms. The delays in the LANs are 5 ms each which are composed of the fixed delay and the value of the delay variation (jitter). In addition, low bit-rate coding as per ITU-T Rec. G.729A is deployed in both VoIP islands; thus, Ie = 2  11 = 22.

Figure II.5/G.108 – VoIP islands, each with G.729A, and PSTN

For the E-model calculations, three different cases of terminal delay (and gateway delay) have been investigated:

•	VoIP terminal send delay = 20 ms, VoIP terminal receive delay = 30 ms

VoIP gateway send delay = 20 ms, VoIP gateway receive delay = 30 ms

total delay = Ts + Tlan + Tr + Tpstn +  Ts + Tlan + Tr = (20 + 5 + 30 + 125 + 20 + 5 + 30) ms = 235 ms

R = 59: Nearly all users dissatisfied

•	VoIP terminal send delay = 35 ms, VoIP terminal receive delay = 65 ms

VoIP gateway send delay = 35 ms, VoIP gateway receive delay = 65 ms

total delay = Ts + Tlan + Tr + Tpstn +  Ts + Tlan + Tr = (35 + 5 + 65 + 125 + 35 + 5 + 65) ms = 335 ms

R = 47: Not recommended

•	VoIP terminal send delay = 50 ms, VoIP terminal receive delay = 100 ms

VoIP gateway send delay = 50 ms, VoIP gateway receive delay = 100 ms

total delay = Ts + Tlan + Tr + Tpstn +  Ts + Tlan + Tr = (50 + 5 + 100 + 125 + 50 + 5 + 100) ms = 435 ms

R = 37: Not recommended

The quality as perceived by the user in these example calculations is identical for both sides, because the example is symmetrically composed.

The scenario in Figure II.6 describes a connection between one VoIP island and an analogue terminal connected to the PSTN with a delay in the PSTN of 25 ms. The delay in the LAN is 5 ms which is composed of the fixed delay and the value of the delay variation (jitter). In addition, the VoIP gateway provides an echo canceller according to ITU-T Rec. G.168 with sufficient tail delay covering the echo path via the local office. Furthermore, low bit-rate coding as per ITU-T Rec. G.729A is deployed; thus, Ie = 11.

Figure II.6/G.108 – VoIP terminal with G.729A and PSTN with analogue terminal

For the E-model calculations, three different cases of terminal delay (and gateway delay) have been investigated:

•	VoIP terminal send delay = 20 ms, VoIP terminal receive delay = 30 ms

VoIP gateway send delay = 20 ms, VoIP gateway receive delay = 30 ms

total delay = Ts + Tlan + Tr + Tpstn = (20 + 5 + 30 + 25) ms = 80 ms

R = 80: Users satisfied

•	VoIP terminal send delay = 35 ms, VoIP terminal receive delay = 65 ms

VoIP gateway send delay = 35 ms, VoIP gateway receive delay = 65 ms

total delay = Ts + Tlan + Tr + Tpstn = (35 + 5 + 65 + 25) ms = 130 ms

R = 79: Some users dissatisfied

•	VoIP terminal send delay = 50 ms, VoIP terminal receive delay = 100 ms

VoIP gateway send delay = 50 ms, VoIP gateway receive delay = 100 ms

total delay = Ts + Tlan + Tr + Tpstn = (50 + 5 + 100 + 25) ms = 180 ms

R = 77: Some users dissatisfied

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |