|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.180

TELECOMMUNICATION	(03/93)
STANDARDIZATION  SECTOR
OF  ITU

GENERAL  CHARACTERISTICS  OF  INTERNATIONAL  
TELEPHONE  CONNECTIONS  AND  INTERNATIONAL  
TELEPHONE  CIRCUITS

CHARACTERISTICS  OF  N  +  M  TYPE  DIRECT  
TRANSMISSION  RESTORATION  SYSTEMS  
FOR  USE  ON  DIGITAL  AND  ANALOGUE  
SECTIONS,  LINKS  OR  EQUIPMENT

ITU-T  Recommendation  G.180

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation G.180 was revised by the ITU-T Study Group XV (1988-1993) and was approved by the WTSC (Helsinki, March 1-12, 1993).

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

In order not to delay publication of this Recommendation, no change has been made in the text to references containing the acronyms “CCITT, CCIR or IFRB” or their associated entities such as Plenary Assembly, Secretariat, etc. Future editions of this Recommendation will contain the proper terminology related to the new ITU structure.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation G.180     (03/93)

Page

1	General		1

2	Object of Recommendation		1

3	RSE specifications		2

3.1	Interfaces		3

3.2	Operational aspects		3

4	RSCE specifications		5

4.1	Interfaces		5

4.2	Operational aspects		6

Annex A		7

A.1	Maximum permissible intrinsic jitter at output ports of regenerative restoration switching equipment		7

Annex B		9

B.1	Jitter transfer characteristics recommended for regenerative restoration switching equipment		9

Appendix I  –  Examples of alarms and status criteria for digital systems		11

Appendix II  –  Examples for monitoring and self-test procedures for digital system		12

Recommendation G.180

Recommendation G.180     (03/93)

CHARACTERISTICS  OF  N  +  M  TYPE  DIRECT  TRANSMISSION  
RESTORATION  SYSTEMS  FOR  USE  ON  DIGITAL  AND
ANALOGUE  SECTIONS,  LINKS  OR  EQUIPMENT

(Melbourne, 1988; amended at Helsinki, 1993)

# 1	General

Transmission restoration functionsXE "Transmission restoration functions"§ are often implemented in the modern telecommunication networks to improve the availability and quality of service, by minimizing the effects or potential effects of a transmission failure, and to make the maintenance operations easier.

The terminology and general principles of transmission restoration are described in Recommendation M.495. The functional organization for automatic transmission restorationXE "automatic transmission restoration"§ is described in Recommendation M.496.

# 2	Object of Recommendation

This Recommendation specifies the characteristics of equipment for N + M type direct transmission restoration systems (protection link switching)XE "N + M type direct transmission restoration systems (protection link switching)"§ for digital and analogue sections, links and equipment (see Recommendations G.701 and G.211). The general arrangement of a system for N + M direct transmission restoration is shown in Figure 1. This Recommendation refers to the equipment labelled as RSE (restoration switching equipmentXE "restoration switching equipment"§) and RSCE (restoration switching control equipmentXE "restoration switching control equipment"§).

This Recommendation is intended also to cover the case where the signals at the interfaces T belong to different hierarchical levels. In this case, each access at one side can be a group of accesses as indicated in the example of Figure 2. The left part of this figure refers to the particular case where the restored path is not on a complete link but just through a multiplex equipment.

NOTE – The equipment specified in this Recommendation can possibly be used also for N + M automatic or semi-automatic transmission rerouting (protection network switchingXE "protection network switching"§) but generally this type of restoration function is implemented by different equipment, often incorporating also other functions (such as, for example, automatic digital distribution frames).

Three types of direct transmission restorationXE "direct transmission restoration"§ systems are considered by this Recommendation:

The first one should permit routing of any one of N normal links on to any one of M restoration links.

The second type should permit the interconnection of any of the N accesses to any one of the N + M links.

The third one should permit routing of any one of N normal links to a single restoration link (in many cases this type can be considered just a special case of the first type).

For all the types two options exist:

a)	to switch the two directions of transmission independently; and

b)	to switch the two directions of transmission simultaneously.

This Recommendation does not cover the restoration systems fully embedded in transmission systems and the 1 + 1 systems where the switching occurs at the receive end only (see Recommendation G.181).

The hierarchical levels at interfaces T are those specified in Recommendation G.702 (hierarchy levels 1 and up). For RSE to be used on analogue transmission systems the frequency bands at interface T are those specified in Recommendation G.211.

FIGURE 1/G.180...[D01] =     20   CM   dimin.  à  19 cm

# 3	RSE specifications

Two types of RSE are considered by this Recommendation: “regenerative” and “non-regenerative”.

The first type, where the digital signal undergoes a complete process of retiming and reshaping, is for digital systems only. It makes the RSE a digital equipment, and it is sometimes considered to be advantageous, e.g. from the station cabling point of view.

FIGURE 2/G.180...[D02] =    11  CM

The second type, where the output signal is proportional to the input signal (except for minor distortion) is considered to be useful for digital systems under some circumstances, e.g. from a reliability and cost point of view, and it is the only one usable for analogue systems.

## 3.1	Interfaces

### 3.1.1	Transmission path interfaces (T)

The relevant parameters and the recommended values are listed in the uppermost part of Table 1 for the non-regenerative RSE, to be used for digital systems. The uppermost part of Table 2 covers the regenerative RSE. The uppermost part of Table 3 is for the non-regenerative RSE to be used for analogue systems.

### 3.1.2	Control interfaces

The only control interface of the RSE is X. This interface is not at present specified by the CCITT. However in the future, it may be specified as a Q interface (see Recommendation G.773).

If the interface X is not standardized, the separation between the RSE and RSCE (and consequently between clauses 3 and 4) will be somewhat arbitrary.

## 3.2	Operational aspects

### 3.2.1	Transfer of the switched signals

The relevant parameters and the recommended values are listed in the lower part of Table 1 for the non-regenerative RSE to be used for digital systems. The lower part of Table 2 covers the regenerative RSE. The lower part of Table 3 is for the non-regenerative RSE to be used for analogue systems.

TABLE  1/G.180

Recommended values for the interface and transfer characteristics of
non-regenerative RSE for use on digital systems

|  | Nominal impedance | As stated in Rec. G.703 |
| --- | --- | --- |
| Interface | Return loss at the input port (with the output port terminated on the nominal impedance) | 6 dB above the values stated in Rec.G.703 |
|  | Accepted levels | The output levels considered in Rec. G.703 |
| Transfer | Transfer function between the input and the output of the RSE (terminated on the nominal impedances) (see Note 2 of Table 2) | < 10% of the interconnecting pair loss and phase distortion allowed in Rec. G.703 for the relevant hierarchical level, or the complement thereof, plus or minus 0.5 dB flat loss. It is assumed that the loss vs frequency distortion approximates to the µ § law |
|  | Crosstalk attenuation | ³ 40 dB from any channel ³ 30 dB multi-channel interference evaluated on a voltage-sum basis. These values apply up to a frequency value equal to the nominal bit-rate |

TABLE  2/G.180

Recommended values for the interface and transfer characteristics of regenerative RSE

| Interface (Note 2) | Connecting pairs Test impedance and return loss at the input ports Pulse shape and levels Tolerable input jitter | As stated in Rec. G.703 |
| --- | --- | --- |
|  | Intrinsic output jitter | As stated in Annex A |
|  | Jitter transfer | As stated in Annex B |
| Transfer (Note 2) | Error performance | 99.99% error-free seconds (Note 1) |
|  | Others | The paths across each switch shall maintain bit sequence independence and integrity |
| - NOTES - 1 Evaluated under maximum loading condition and excluding any external source of interference. - 2 The nominal bit rate and tolerance are as stated in Recommendation G.703. |  |  |

TABLE  3/G.180

Recommended values for the interface and transfer characteristics of non-regenerative RSE
for use on analogue systems

|  | Nominal relative levels in switching points |  |
| --- | --- | --- |
|  | Nominal impedance at the input/output ports | As stated in Recommendation G.233 |
| Interface | Return loss at the input/output ports |  |
|  | Longitudinal balance conversion loss | (Note) ³ 46 dB |
|  | Insertion loss | £ 0.3 dB |
|  | Frequency response as stated in Recommendation G.211 | ± 0.2 dB |
| Transfer | Noise referred to the voice-frequency channel bandwith | £ 20 pW0 for a pair of RSEs loaded by the signal of maximum power averaged over one minute interval |
|  | Intelligible crosstalk ratio between ports within the same RSE | ³ 90 dB |
| NOTE – For groups only. |  |  |

### 3.2.2	Response

For RSE providing M restoration paths to N normal paths (M = 1 included) it is recommended that in response to a RSCE command the RSE should apply the incoming interface signal belonging to a given normal link to the input port of a given restoration link. The signal should not be removed from the input port of the concerned normal link, except that it may be replaced by a test signal.

For RSE providing N + M link to N accesses it is recommended that in response to a RSCE command, the RSE should apply the incoming interface signal belonging to a given access from 1 to N to a given link from 1 to N + M.

The time required for the above response is the restoration transfer time, as defined in Recommendation M.495.

### 3.2.3	Other operational aspects

A recognized failure of the RSCE or its disconnection from the RSE at interface X (when applicable) should either:

a)	Cause the RSE to route all the signals on the N normal links. After the failure of the RSCE is cleared or the RSCE is reconnected to the RSE, normal restoration operations will resume.

b)	Not alter the state of the RSE. The cross-connection pattern of the RSE should be available by interrogation from the RSCE to enable it to update, when the failure is cleared or it is reconnected to the RSE, its own record on the cross-connection pattern.

For the restoration systems of the second type (as defined alternative in b) only holds. For the systems of the first and third types both alternatives are applicable.

# 4	RSCE specifications

## 4.1	Interfaces

Interfaces Y, Z of the RSCE (see Figure 1) are at present non standardized. However they may be standardized in the future.

## 4.2	Operational aspects

### 4.2.1	Responses

A switching to a restoration linkXE "restoration link"§ should be initiated under a request coming from interfaces Y, Z, Q (and X where the faults are detected within the RSE) or on command from the local manual control.

When decided in the RSCE the allocation of a restoration link can optionally take place according to defined priority rules based on:

–	defined priority for each normal link;

–	request type (low or high priority request).

Otherwise the allocation should be specified by the information coming from interfaces Z, Q or local manual control.

For the restoration systems providing M restoration links (M = 1 included) on N normal links, when a successful restoration request clears, traffic should be returned to the pertinent normal link and the pertinent restoration link should be released.

It should be possible from interfaces Z, Q and under local manual control to lock in a working link (e.g. during system maintenance).

The time required for the above response is the sum of the waiting time and the restoration procedure time, as defined in Recommendation M.495.

### 4.2.2	Alarm and status criteria

The alarm and status criteria provided by the RSCE are not standardized. Examples are reported in Appendix I.

### 4.2.3	Monitoring and self-test procedures

The monitoring and self-test procedures of the RSCE are not standardized. Examples are reported in Appendix II.

Annex  A

(This annex provides information which is supplementary to Table 2 and
does not form an integral part of this Recommendation)

## A.1	Maximum permissible intrinsic jitter at output ports of regenerative restoration switching equipmentXE "Maximum permissible intrinsic jitter at output ports of regenerative restoration switching equipment"§

NOTE – Values for bit rates of the 1544 kbit/s digital hierarchy are not covered by this Recommendation.

### A.1.1	For asynchronous space matrix RSE

|  | Parameter value |  |  |
| --- | --- | --- | --- |
|  |  | Measurement filter bandwith |  |
| Digital rate (kbit/s) | Maximum value Unit interval peak-peak | Bandpass filter having a lower cut-off frequency f1 and an upper cut-off frequency f4 |  |
|  |  | f1 | f4 |
| 132 048 | 0.133 | 320 Hz | 3100 kHz |
| 138 448 | 0.133 | 320 Hz | 3400 kHz |
| 134 368 | 0.075 | 100 Hz | 3800 kHz |
| 139 264 | 0.053 | 200 Hz | 3500 kHz |

### A.1.2	For synchronous RSE using central timing

|  | Parameter value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Maximum value |  | Measurement filter bandwidth |  |  |
| Digital rate (kbit/s) | Unit interval peak-peak |  | Bandpass filter having a lower cut-off frequency f1 or f3 and an upper cut-off frequency f4 |  |  |
|  | B1 (f1 ¸ f4 filter) | B2 (f3 ¸ f4 filter) | f1 | f3 | f4 |
| 132 048 | 0.25 | 0.05 | 120 Hz | 18 kHz (700 Hz) | 3100 kHz |
| 138 448 | 0.25 | 0.05 | 120 Hz | 3 kHz (80 kHz) | 3400 kHz |
| 134 368 | 0.35 | 0.05 | 100 Hz | 10 kHz | 3800 kHz |
| 139 264 | 0.35 | 0.05 | 200 Hz | 10 kHz | 3500 kHz |
| - NOTES - 1 UI Unit interval - for 132 048 kbit/s 1 UI 488,18 ns - for 138 448 kbit/s 1 UI 118,18 ns - for 134 368 kbit/s 1 UI 129.18 ns - for 139 264 kbit/s 1 UI 117.18 ns. - 2 These figures shall be met for any valid signal in the absence of input jitter. The measurement shall be implemented using equipment designed in accordance with Recommendation O.171. - 3 Clause 2/G.823 indicates the measurement method. - 4 The frequency values in parentheses only apply to certain national interfaces. |  |  |  |  |  |

Annex  B

(This annex provides information which is supplementary to Table 2 and
does not form an integral part of this Recommendation)

## B.1	Jitter transfer characteristics recommended for regenerative restoration switching equipment

NOTE – Values for bit rates of the 1544 kbit/s digital hierarchy are not covered by this Recommendation.

FIGURE...[D03] =   4.5  CM

### B.1.1	For asynchronous space matrix RSE

|  | Parameter value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Digital rate (kbit/s) | x (dB) (Note 5) | –y (dB) | f0 (Hz) | f5 (kHz) | f6 (kHz) | f7 (kHz) | Test signal (pseudorandom as Rec. O.151) |
| 132 048 | 0.5 | –8.4 (–9.5) | (Note 1) | 36 (1.4) | 100 (4.4) | 3100 | 215 – 1 |
| 138 448 | 0.5 | –9.5 (–7.5) | (Note 1) | 6 (160) | 19 (400) | 3400 | 215 – 1 |
| 134 368 | 0.5 | –9.5 | (Note 1) | 20 | 64 | 3800 | 223 – 1 |
| 139 264 | 0.5 | –9.5 | (Note 1) | 20 | 64 | 3500 | 223 – 1 |
| NOTES 1 The frequency f0 should be as low as possible (e.g. 10 Hz) taking into account the limitations of measuring equipment. 2 The measuring method should be selective with a bandwith sufficiently small referred to the relevant measuring frequency, but not wider than 40 Hz. 3 In the f6 to f7 frequency range the jitter gain should be less than –y dB, with the exception of spurious responses, which should be suppressed below –6 dB. 4 The frequency values shown in parentheses only apply to certain national interfaces. 5 A value of 0.2 dB has been suggested as technically possible for this type of equipment. This may be useful where large numbers of RSE are employed in the network. |  |  |  |  |  |  |  |

### B.1.2	For synchronous RSE using central timing

|  | Parameter value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Digital rate (kbit/s) | x (dB) | –y (dB) | f0 (Hz) | f5 (Hz) | f6 (Hz) | f7 (kHz) | Test signal (pseudorandom as Rec. O.151) |
| 132 048 | 0.5 | 19.5 | (Note 1) | 140 | 1400 | 3100 | 215 – 1 |
| 138 448 | 0.5 | 19.5 | (Note 1) | 100 | 1000 | 3400 | 215 – 1 |
| 134 368 | 0.5 | 19.5 | (Note 1) | 300 | 3000 | 3800 | 223 – 1 |
| 139 264 | 0.5 | 19.5 | (Note 1) | 900 | 9000 | 3500 | 223 – 1 |
| - NOTES - 1 The frequency f0 should be as low as possible (e.g. 10 Hz) taking into account the limitations of measuring equipment. - 2 The measuring method should be selective with a bandwidth sufficiently small referred to the relevant measuring frequency, but not wider than 40 Hz. |  |  |  |  |  |  |  |

Appendix  I

Examples of alarms and status criteria for digital systems
(Both refer to a N + 1 system)

(This appendix does not form an integral part of this Recommendation)

| - Provided by STC PLC | - Provided by AT&T Network Systems International |
| --- | --- |
| - Alarms |  |
| - The system should include: | - Separate alarm criteria shall be issued at the occurrence of the following faulty conditions: |
| - a) System fail. | - a) Loss of signal at the traffic input port, transmit side. |
| - b) Protection failure. | - b) Loss of signal at the traffic output port, receive side. |
| - c) Manual switch in operation | - c) Automatic lock-in (see Note). |
| - d) System software self check in operation. | - d) Switch failure. |
| - e) Control system failure. | - e) Protection failure. |
| - f) System software failure. | - f) Control system failure. |
| - g) Communication failure. | - g) Communication failure. |
| - h) Stand-by channel failure. | - h) Stand-by channel failure. |
| - i) Power supply failure. | - i) Power supply unit failure. |
| - j) Card removal. | - j) Loss of power supply. |
|  | - k) Switch exerciser failure. |
|  | - Separate status criteria shall be issued, on the occurrence of the following situations: |
|  | - a) Switch operated. |
|  | - b) Switch locked. |
|  | - c) Switch request pending. |
|  | - d) Switch in manual mode. |
|  | - The protective switching control equipment shall make available to the remote control and maintenance centre alarm and status information corresponding to the criteria shown above. |
| - NOTE – This system is required to automatically lock in the normal or the protection channel if an excesive number of switching operations are made in a given period. |  |

Appendix  II

Examples for monitoring and self-test procedures for digital system
(Both refer to a N + 1 system)

(This appendix does not form an integral part of this Recommendation)

| - Provided by STC PLC | - Standby channel monitoring - The system should include means of monitoring the standby channel continuously for proper operation. - Self-check - The system should include self-check facilities as follows: - a) Communication channel. - b) Background-checking of the memory, coaxial relay drive buffer and other hardware. - c) Correct programme execution. |
| --- | --- |
| - Provided by AT&T Network Systems International | - Standby channel monitoring - The standby channel shall be monitored continuously for proper operation. - Switch exerciser - The protective switching system shall provide a switch exerciser meeting the following requirements: - The exerciser shall test the complete switch-over procedure up to but excluding the last transfer switch in the direction of transmission. - The switching system shall drop the exerciser routine and serve switch requests from failed or deteriorated channels. - A facility for including the last switch in the exercise routine may be provided. This feature shall have the capability of being disabled. |