|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.224

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

INTERNATIONAL  ANALOGUE  CARRIER  SYSTEMS

GENERAL  CHARACTERISTICS  COMMON  TO  ALL ANALOGUE  CARRIER-TRANSMISSION  SYSTEMS

MAXIMUM  PERMISSIBLE  VALUE  FOR  THE  ABSOLUTE  POWER  LEVEL  (POWER  REFERRED  TO  ONE  MILLIWATT)  OF  
A  SIGNALLING  PULSE

ITU-T  Recommendation  G.224

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.224 was published in Fascicle III.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.224

Fascicle III.2 - Rec. G.224

MAXIMUM PERMISSIBLE VALUE FOR THE ABSOLUTE POWER LEVEL (POWER REFERRED TO ONE MILLIWATT) OF A SIGNALLING PULSE

The CCITT recommends that, for crosstalk reasons, the absolute power level of each component of a short duration signal should not exceed the values given in Table 1/G.224.

TABLE 1/G.224

Maximum permissible value, at a zero relative level point

| Signalling frequency (Hz) | Maximum permissible power for a signal at a zero relative level point (microwatts) | Corresponding absolute power level Decibels referred to 1mW (dBm0) |
| --- | --- | --- |
| 800 1200 1600 2000 2400 2800 3200 | 750 500 400 300 250 150 150 | -1 -3 -4 -5 -6 -8 -8 |

Note 1 - If the signals are made up of two different frequency components transmitted simultaneously, the maximum permissible values for the absolute power levels are 3 dB below the above values.

Note 2 - The values given in this table result from a compromise between the characteristics of various channel filters now in existence.

Reference

[1]	CCITT Recommendation Maximum permissible value for the absolute power level of a signalling pulse, Vol. VI, Rec. Q.16.

This Recommendation applies both to national and to international signalling systems.