|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.227

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

INTERNATIONAL  ANALOGUE  CARRIER  SYSTEMS

GENERAL  CHARACTERISTICS  COMMON  TO  ALL ANALOGUE  CARRIER-TRANSMISSION  SYSTEMS

CONVENTIONAL  TELEPHONE  SIGNAL

ITU-T  Recommendation  G.227

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.227 was published in Fascicle III.2 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.227

Fascicle III.2 - Rec. G.227

CONVENTIONAL TELEPHONE SIGNAL

(Geneva, 1964; amended at Mar del Plata, 1968)

# 1	Principle

For the calculation or measurement of crosstalk noise between adjacent channels and, generally speaking, when it is desired to simulate the speech currents transmitted by a telephone channel1) , the CCITT recommends that a conventional telephone signal be used, the main characteristic of which is a shaping network as a function of the frequency.

This network is defined by the following transfer coefficient as a function of the frequency:

FIGURE 1/G.227

The response curve of the network is shown in Figure 2/G.227, and an example of the design is given in Figure 3/G.227 with relevant values.

FIGURE 2/G.227

Relative response curve of the shaping network of the conventional telephone signal generator

FIGURE 3/G.227

Shaping network of the conventional telephone signal generator

2	Example of network design

The network is made up of three bridged T sections with a constant characteristic impedance equal to R0 ohms.

Figure 3/G.227 represents the network and indicates the values of the various components normalized to R0.

A tolerance of ± 1% can be allowed on the value of each component.

Note - If q1, q2, q3 are the "composite" transfer coefficients of sections 1, 2 and 3 respectively, we have:

The minimum composite loss2) of the complete network lies in the vicinity of 600 Hz and equals a0 ~ 2.9 dB for this example.

The curve in Figure 2/G.227 represents, as a function of frequency, the composite loss2) of the network in Figure 3/G.227 relative to the minimum loss a0.

# 3	Signal at the network input

The network may be energized either by a uniform-spectrum random noise signal or by a closely spaced harmonic series. In the latter case, the following precautions are necessary:

1)	Spacing of the harmonics should not exceed 50 Hz.

2)	The measuring instrument must have an adequate integrating time with respect to the fundamental period of the harmonic series. Types of CCITT instruments in general use, such as the psophometer, are believed to be satisfactory in this respect.

3)	The peak/r.m.s. ratio of the signal should not exceed 3.5. This requirement may be achieved, in the case of a particular generator, by means of an associated phase-changing network.

4)	The energizing signals (uniform-spectrum random noise and harmonic series) could lead to different results for subjective, e.g. aural assessments at the receiving end, and such measurements should not, therefore, involve the use of the conventional telephone signal generator. That apparatus would be used solely for objective measurements, in which a psophometer served as measuring instrument.

Reference

[1]	CCITT - Question 5/C, Annex 2, Green Book, Vol. III, ITU, Geneva, 1973.

1)	Care is needed in applying this conventional signal to simulate speech loading, since the statistics of a Gaussian noise signal and of real speech are different. A speech-simulating generator for loading purposes is given in [1].

2)	Composite loss equals the insertion loss in this particular case since the source and the load impedances are equal.