| µ § | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.612

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TRANSMISSION  MEDIA  CHARACTERISTICS

CHARACTERISTICS  OF  SYMMETRIC  CABLE  PAIRS  DESIGNED  FOR  THE  TRANSMISSION  OF  SYSTEMS  WITH  BIT  RATES  OF  
THE  ORDER  OF  6 TO 34 Mbit/s

ITU-T  Recommendation  G.612

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.612 was published in Fascicle III.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.612

Fascicle III.3 - Rec. G.612

CHARACTERISTICS OF SYMMETRIC CABLE PAIRS DESIGNED FOR THE TRANSMISSION OF

SYSTEMS WITH BIT RATES OF THE ORDER OF 6 TO 34 Mbit/s

(Geneva, 1976; amended at Geneva, 1980)

# 1	Preamble

This Recommendation relates to symmetric pair cables which have been developed for the transmission of signals with bit rates of the order of 6 to 34 Mbit/s, but they are not ruled out for the transmission of lower or higher bit rates, subject to the use of an appropriate regeneration section; in most cases they can also be used for baseband transmission of videophone or television signals.

These cables fall into two categories, according to whether or not the cable is intended for use in both directions of transmission in the same cable.

# 2	Parameters to be measured

Those parameters which, for digital system transmission, have to be measured by a particular method or at frequencies different from those defined in Recommendation G.611, are: characteristic impedance, attenuation coefficient, and far-end crosstalk between pairs on the same direction of transmission. If the cable is intended for use with both directions of transmission within the same cable, it is also necessary to measure the near-end crosstalk between pairs intended for different directions of transmission.

2.1	Characteristic impedance

The characteristic impedance may be measured:

-	either in the sinusoidal mode, when the measured pair will be terminated by an impedance constantly equal to that measured by the bridge, except when the length is sufficient for the measurement result to be independent of the termination impedance;

-	or by a pulse echo meter1) , when the impedance of the pair being measured is compensated by an adjustable balancing network graduated to show the impedance value. The pair being measured is terminated by an identical network.

2.2	Attenuation coefficient

The attenuation per km of the pairs is derived from that value to be obtained on an elementary cable section, allowance being made for the tolerance accepted on the length of these sections.

Note - In the case of looped measurement, a check should be carried out to ensure that the near-end crosstalk attenuation between the ends of the circuit being measured is sufficient.

2.3	Crosstalk

Crosstalk may be specified either in sinusoidal mode, at a frequency near the timing half-frequency of the system concerned, or in digital mode2) .

2.3.1	Far-end crosstalk measurement

The far-end crosstalk measurements are carried out on pairs used in the same direction of transmission at a frequency above about 100 kHz; if this frequency is not the timing half-frequency of the system, the value to be specified will be corrected to the factor 20 log10 f 3) .

2.3.2	Near-end crosstalk measurements

If it is intended to transmit in both directions on the same cable, these measurements are conducted on a prototype length, either in sinusoidal mode or digital mode, between pairs used for opposite directions of transmission.

# 3	Description of pairs and cables

Administrations which decide to use symmetrical pairs to transmit digital signals with a bit rate of the order of 6 to 34 Mbit/s should, wherever possible, choose one of the types of cable described in §§ 3.1 and 3.2 below.

3.1	Cable designed for use with one cable for each direction of transmission

3.1.1	The basic characteristics of the pairs are given in Table 1/G.612.

3.1.2	The characteristics of cables constructed with these pairs are given in Table 2/G.612.

3.2	Cables designed for transmission in both directions in the same cable

Tables 3/G.612 and 4/G.612 indicate the characteristics of the pairs which make up cable pairs and quad cables respectively.

All these cables consist of bundles protected by one or more copper or aluminium screens, the pairs in each bundle being used for the same direction of transmission. For this reason, near-end crosstalk values relate only to pairs in different bundles.

Note 1 - To make the presentation of Tables 3/G.612 and 4/G.612 uniform, the values of characteristic impedance are given at 1 MHz (real part of Z1). The ratio between impedance Z1 = X1 - jY1 at 1 MHz and impedance Zf = Xf - jYf at f MHz is

.

The difference between the value of the real part of the impedance at 1 MHz and its value at 4 MHz is between 
2 and 3 W. At 1 MHz, the imaginary part of the impedance is between 4 and 6 W; for frequencies above about 0.3 MHz, it varies in the inverse ratio to the square root of the frequency.

Note 2 - For the same reason as in Note 1 above, the attenuation value is given at 1 MHz. At a frequency f MHz (f > 1), attenuation af is related to attenuation a1 at 1 MHz by the ratio  .

Note 3 - The value of far-end crosstalk is reduced to a length of 1000 m by a correction of 10 logl0 L if the cable length L being measured is different from 1000 m. The crosstalk values indicated are the minimum limit values for the specification of systems. Where either of the above conditions is not fulfilled, the values are shown between brackets.

TABLE 1/G.612

| Pair characteristics | Type I cable |
| --- | --- |
| Diameter of conductors (mm) Average mutual capacitance of pairs (nF/km) Characteristic impedance (W)a) Attenuation coefficient at 24° C (dB/km)a) | 0.64 24.2 178 13.5 |

a)	The attenuation and impedance measurement frequency is 3150 kHz.

TABLE 2/G.612

|  | Set 1a) | Set 2a) |
| --- | --- | --- |
| Nominal characteristic impedance Z0 (W) (desired average at 3150 kHz) | 178 |  |
| Attenuation and crosstalk Attenuation at 3150 kHz to 24° C (dB/km) pair minimum pair maximum Far-end crosstalk (FEXT) loss at 3150 kHz dB for a 300 m (1000 feet length) pair minimum power sum minimum pair-to-pair (0.1% point) | 11.8 14.35 37.5 40.5 | 11.8 14.6 39.0 40.5 |
| DC resistance at 24°C (W /km) maximum conductor desired average | 56.8 54.5 |  |
| Cable average mutual capacitance (nF/km) maximum minimum desired average r.m.s standard deviation (s) of pairs within a cable (%) | 25.4 23.0 24.2 £ 7 |  |
| Capacitance unbalance to ground (pF/km) maximum pair cable average | £ 443 £ 164 |  |
| DC dielectric strength between conductors for ARPAP b) sheath core and inner aluminium to shield core to inner aluminium and shield | ³ 1 500 V (applied for 1 s) ³ 20 000 V (applied for 3 s) ³ 5 000 V (applied for 3 s) |  |

a)	Two sets of values for attenuation and far-end crosstalk are given. The cable may meet either one of these sets, thus allowing a cable with lower loss to meet a less stringent crosstalk requirement.

b)	Aluminium-resin-polythene-aluminium-polythene.

TABLE 3/G.612

Cable pairs

| Characteristics |  | Cable type |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | I | II | III | IV | V |
| Nominal characteristic impedance Z0 at 1 MHz (W) |  | 160 | 160 | 140 | 120 | 145 |
| far-end crosstalk (minimum values referred to 1000m) (dB) | 1 MHz 4 MHz 17 MHz | 43a) | 43a) | 40 | 56 44 31 | 64 52 40 |
| Near-end crosstalk from 1 to 17 MHz (minimum values, dB) |  | 119 | 119 | 98 | 116 | 125 |
| Nominal attenuation coefficient at 1 MHzb) (dB/km at 10° C) |  | 7.0 | 9.3 | 10.5 | 9.5 | 5.2 |
| Nominal capacity (nF/km) |  | 28.5 | 28.5 | 31.5 | 38 | 30 |
| Diameter of conductors (mm) |  | 0.8 | 0.6 | 0.65 | 0.9 | 1.2 |

a)	Far-end crosstalk measurements on elementary cable sections for pairs of this type are made in the digital mode only (see Supplement No. 19). The maximum value specified is 30 mV.

b)	The real values should make it possible to meet the conditions required for an elementary cable section (Type I: 56 ± 2 dB at 4.2 MHz and 10°C for 4 km; Type II: 56 ± 2 dB at 4.2 MHz and 10 °C for 3 km; Type III: below 55 dB at 3.15 MHz for 2.8 km).

TABLE 4/G.612

Quad cables

| Characteristics |  |  | Cable Type |  |
| --- | --- | --- | --- | --- |
|  |  |  | I | II |
| Nominal characteristic impedance Z0 at 1 MHz (W) |  |  | 165 | 120 |
| Far-end crosstalk | Different quads | 1 MHz 4 MHz 13 MHz 17 MHz | 46 34 31 | 56 44 31 |
| (minimum values referred to 1000 m) (dB) | Same quad | 1 MHz 4 MHz 13 MHz 17 MHz | (45) (25)}a) (21) | 46 34 c) |
| Near-end crosstalk, from 1 to 17 MHz (minimum values, dB) |  |  | 125b) | 116 |
| Nominal attenuation coefficient at 1 MHz (dB/km at 10°C) |  |  | 8.8 | 9.5 |
| Nominal capacity (nF/km) |  |  | 28 | 38 |
| Diameter of conductors (mm) |  |  | 0.65 | 0.9 |

a)	For 34 Mbit/s transmission over each pair of a star quad, a balancing method is applied to the elementary cable section of 2 km by means of systematic crossings every 500 m, which improves the far-end crosstalk values by at least 15 dB. Hence the values given in this box correspond to 500 m of cable.

b)	The value must be above 130 dB in 99% of cases.

c)	The transmission of 34 Mbit/s over each pair of a star quad is studied.

1)	This method is similar to the one used for coaxial pairs, but with a symmetrical measuring head and networks. The pulse duration is equal to 100 ns; the echo is not corrected.

2)	An example of a digital technique is given in Supplement No. 19.

3)	For symmetrical pair star-quad cables the correction law 20 logl0 f is used for pairs of the same quad only up to a certain characteristic frequency, above this frequency the law 40 log10 f must be used.