|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.613

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TRANSMISSION  MEDIA  CHARACTERISTICS

_________________________________________

CHARACTERISTICS  OF  SYMMETRIC  CABLE  PAIRS  USABLE  WHOLLY  FOR  THE  TRANSMISSION  OF  DIGITAL  SYSTEMS  WITH  A  BIT  RATE  OF  UP  TO  2 Mbits

ITU-T  Recommendation  G.613

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.613 was published in Fascicle III.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.613

Fascicle III.3 - Rec. G.613

CHARACTERISTICS OF SYMMETRIC CABLE PAIRS

USABLE WHOLLY FOR THE TRANSMISSION OF DIGITAL SYSTEMS

WITH A BIT RATE OF UP TO 2 Mbits

(Malaga-Torremolinos, 1984)

# 1	Preamble

This Recommendation deals with cables designed for the transmission of standard digital systems (Recommendations of the G.900 series), although these cables can also be used to transmit digital signals with a lower bit rate and voice frequency signals. The cables described carry signals in both transmission directions simultaneously. The provisions of this Recommendation apply to cables designed to allow for digital operation of all the cable circuits. However, some of the provisions may be used to assess the possibility of (partial or full) digital operation of existing cables.

# 2	Parameters to be measured

2.1	Direct current resistance

The following formula is used to correct the value Rt of direct current resistance measured at t°C for 20 °C:

R20 = Rt /(l + 0.004 (t - 20))

2.2	Capacitance per unit length

This is measured at 800 Hz or 1000Hz.

2.3	Attenuation coefficient

The value of the attenuation coefficient is obtained either by direct measurement of the attenuation or by calculation on the basis of the mutual capacitance and direct current resistance of the pair. The attenuation coefficient is measured at one frequency only, f0, near the timing half-frequency.

| System | Recommendation | f0 |
| --- | --- | --- |
| 1544 kbit/s | G.951 | 772 kHz |
| 2048 kbit/s | G.952 | 1 MHz |

For cables with polyolefin insulation, the value of the attenuation coefficient at frequency f (for values of f above with a few hundred kHz) can be related toa0 by the equation

The value of the attenuation coefficient measured at t°C is corrected for 20 °C by the equation:

a20 = at /(1 + 0.002 (t - 20))

2.4	Characteristic impedance

2.4.1	Echometric measurement

When a pulse echometer is used, the impedance of the pair measured must be compensated by a calibrated balancing network which can be set in steps of about 0.5 W. Pulse duration will be equal to or less than 500 ns. With this method, which is both fast and simple, the value of the end impedance of the pair measured is read off directly on the scale of the balancing network.

2.4.2	Sinusoidal measurement

In this case, the pair tested will be terminated across an impedance, which is constantly equal to that measured by the bridge, unless it is long enough for the result of the measurement to be independent of end impedance (as for elementary cable sections).

2.5	Crosstalk

Crosstalk can be measured sinusoidally or digitally. The assignment of pairs to the direction of transmission depends on the structure and type of manufacture of the cable.

2.5.1	Sinusoidal measurement

2.5.1.1	Far-end crosstalk

The measurements are made between pairs assigned to the same direction of transmission, at frequency f0. If the frequency at which measurement is carried out is not the timing half-frequency, the value is corrected using the 20 logl0 f law. When the measurement is carried out on a pair of length, L, which is different from the specified reference length L0, the measured value is corrected using µ § when the value is expressed in mV or 10 log10 µ § when the value is expressed in dB.

2.5.1.2	Near-end crosstalk

The measurements are made between pairs assigned to transmission in opposite directions at a frequency near the system's timing half-frequency.

2.5.2	Digital measurement

By means of digital measurement, it is possible to estimate the total noise on an elementary section, taking account of both near-end and far-end crosstalk. This estimate can be made on the basis of separate near-end and far-end crosstalk measurements on either factory lengths or elementary sections.1) These measurements can be made either in factory conditions or on installed cables.

2.5.2.1	Far-end crosstalk

The measurements are carried out between pairs assigned to the same direction of transmission. When the measurement is carried out on a pair of length, L, which is different from the specified reference length L0, the measured value is corrected using µ § when the value is expressed in mV or 10 log10 (L / L0) when the value is expressed in dB.

2.5.2.2	Near-end crosstalk

The measurements are made between pairs assigned to transmission in opposite directions.

# 3	Circuit characteristics

These are given in Table 1/G.613.

# 4	Characteristics of connected cable sections

These are given in Table 2/G.613.

TABLE 1/G.613

Circuit characteristics *

| Characteristics |  |  | Type of cable |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Type I | Type II | Type II bis | Type III **** | f) |
| Operational bit rate (kbit/s) |  |  | 2048 | 2048 | 2048 | 2048 |  |
| Repeaters gain ** |  |  | 34 dB |  |  |  |  |
| Elements constituting the cable |  |  | star quad | pairs | pairs | pairs |  |
| Nominal conductor diameter (mm) |  |  | 0.8 | 0.7 | 1 | 0.6 |  |
| Nominal impedance *** at f0 MHz (W) |  | 1 MHz | 100 | 130 | 130 |  |  |
|  |  | 772 kHz |  |  |  |  |  |
| Nominal attenuation coefficient at f0 and at |  | 1 MHz | 16 | 11.5 b) | 8.5 b) | 15.5 |  |
| 20° C *** (dB/km) |  | 772 kHz |  |  |  |  |  |
| Crosstalk in digital operation |  | a) | c) | - | - | - |  |
| Total noise voltage (maximum value) |  | a) |  |  |  |  |  |
| Minimum near-end crosstalk (mV) |  | a) | - | 60 d, g) | 60 d, g) |  |  |
|  |  | a) |  |  |  |  |  |
| Minimum far-end crosstalk (mV) |  | a) | - | 45 e, g) | 45 e, g) |  |  |
|  |  | a) |  |  |  |  |  |
|  | Near-end (dB) | 1 MHz |  |  |  | 78 ± 3h) |  |
| Sinusoidal crosstalk |  | 772 kHz |  |  |  |  |  |
|  | Far-end (dB) | 1 MHz |  |  |  | 64 ± 3h) |  |
|  |  | 772 kHz |  |  |  |  |  |
| Nominal direct current resistance at 20°C (W/km) |  |  | 68.6 | 94.1 b) | 46.1 b) | 63 |  |
| Nominal mutual capacitance (nF/km) |  |  | 50 | 39 | 39 | 44 |  |

Notes of Table 1/G.613

*	At the present stage the values are given for information.

**	Reference value for the numerical data of the cable in question.

***	A standard deviation or margins will be given at a later stage.

****	Cable with diametral screen separating the pairs assigned to the two directions of transmission.

a)	To be specified.

b)	Maximum value.

c)	The specification value for factory controls is calculated to ensure compliance with the characteristics of connected cable.

d)	Between pairs of different groups.

e)	Between pairs belonging to one and the same group.

f)	Other columns will contain the data supplied by administrations.

g)	Values given in dB.

h)	The value given here depends on the content of the cable. It is the rounded-down mean of a standard deviation of the total production and is therefore not a specification for individual cable lengths.

TABLE 2/G.613

Characteristics of connected cable sections *

| Characteristics |  |  | Type of cable |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Type I |  | Type II | Type II bis | Type III | a) |
| Operational bit rate (kbit/s) |  |  | 2048 |  | 2048 | 2048 |  |  |
| Nominal impedance at f0 MHz (W) |  | 1 MHz | 100 | 130 |  | 130 |  |  |
|  |  | 772 kHz |  |  |  |  |  |  |
| Nominal attenuation coefficient at f0 and at |  | 1 MHz | 16 | 11.5 |  | 8.5 |  |  |
| 20°C (dB/km) |  | 772 KHz |  |  |  |  |  |  |
| Crosstalk in digital operation |  | b) | 40 mV |  |  |  |  |  |
| Total noise voltage (maximum value) |  | b) |  |  |  |  |  |  |
| Minimum near-end crostalk (mV) |  | b) |  |  |  |  |  |  |
|  |  | b) |  |  |  |  |  |  |
| Minimum far-end crosstalk (mV) |  | b) |  |  |  |  |  |  |
|  |  | b) |  |  |  |  |  |  |
|  | Near-end (dB) | 1 MHz |  |  |  |  |  |  |
| Sinusoidal crosstalk |  | 772 MHz |  |  |  |  |  |  |
|  | Far-end (dB) | 1 MHz |  |  |  |  |  |  |
|  |  | 772 MHz |  |  |  |  |  |  |

*	At the present stage the values are given for information.

a)	Other columns will contain the data supplied by Administrations.

b)	To be specified.

1)	One advantage of digital measurements is that it is possible to make a direct overall measurement of the total noise on an elementary section if enough generators are available.