|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.614

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TRANSMISSION  MEDIA  CHARACTERISTICS

CHARACTERISTICS  OF  SYMMETRIC  PAIR  STAR-QUAD  CABLES  DESIGNED  EARLIER  FOR  ANALOGUE  TRANSMISSION  SYSTEMS  AND  BEING  USED  NOW  FOR  DIGITAL  SYSTEM  TRANSMISSION  AT  BIT  RATES
OF 6  TO  34 Mbit/s

ITU-T  Recommendation  G.614

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.614 was published in Fascicle III.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.614

Fascicle III.3 - Rec. G.614

CHARACTERISTICS OF SYMMETRIC PAIR STAR-QUAD CABLES

DESIGNED EARLIER FOR ANALOGUE TRANSMISSION SYSTEMS

AND BEING USED NOW FOR DIGITAL SYSTEM TRANSMISSION

AT BIT RATES OF 6 TO 34 Mbit/s

(Melbourne, 1988)

# 1	Introduction

This Recommendation relates to symmetric pair star-quad cables which have been designed earlier and used to provide 60 or 120 carrier telephone channels of analogue transmission systems on each quad pair. Further, after reconstruction of the line, these cables are used for digital system transmission at bit rates of 6 to 34 Mbit/s. The cables concerned have no screened pairs and quads.

For digital transmission systems with a bit rate of 8 Mbit/s both one-cable and two-cable operations may be used. For systems with a bit rate of 34 Mbit/s two-cable operation is used only.

For digital transmission systems both several, or all cable pairs may be used.

# 2	Parameters to be measured

All parameters specified in Recommendation G.612, namely characteristic impedance, attenuation coefficient, far-end crosstalk between pairs on the same direction of transmission, and near-end crosstalk between pairs of two different cables intended for different directions of transmission are to be measured. If the cable is intended for use with both directions of transmission it is also necessary to measure the near-end crosstalk between pairs intended for different directions of transmission.

2.1	Characteristics impedance

The characteristics impedance is measured according to § 2.1 of Recommendation G.612.

2.2	Attenuation coefficient

The attenuation coefficient is measured according to § 2.2 of Recommendation G.612.

2.3	Crosstalk

The crosstalk is specified in sinusoidal mode at a frequency near the timing half-frequency of the digital system and/or at other frequencies. Digital mode of measuring may be used also.

2.3.1	Measurement of far-end crosstalk between pairs of different quads

The measurement of the far-end crosstalk is carried out on pairs used in the same direction of transmission at a frequency above about 0.1 MHz when a length of cable is L. If the frequency of measurements differs from the timing half-frequency of the digital transmission system the value to be measured will be corrected to the factor 20 log10 f . The values are corrected to the length of 1000 m by the factor 10 log10 L.

2.3.2	Measurement of far-end crosstalk between pairs of the same quad

This measurement is carried out at a cable length equal to maximum permissible length of regenerator section of digital transmission system with bit rates of 6 to 34 Mbit/s at a frequency above about 1.0 MHz (measurement is carried out for each rate of digital transmission system separately) with systematic component of crosstalk in the same quad compensated. The compensation of systematic crosstalk component is carried out by one of the approximately equivalent transposition patterns (see Figure 1/G.614). When regenerator sections are of less length these methods of falling the elementary cable sections into separate parts and of transposition in quad provide the greater values of the far-end crosstalk between pairs than those values when measurements are carried out at, a maximum length of regenerator section.

Note 1 - Transposition pattern No. 1 was proposed by the Netherlands Administration in 1978 (see COM XV-135, period 1977-1980).

Note 2 - Transposition pattern No. 2 has been proposed by the German Democratic Republic Administration.

Note 3 - Transposition patterns Nos. 3, 4, 5 and 6 are proposed by the USSR Administration.

FIGURE 1/G.614

2.3.3	Measurement of near-end crosstalk between pairs of the same or different cables intended for different directions of transmission

This measurement is carried out either between pairs of the same cable (when one-cable operation is used), or between pairs of two different cables intended for different directions of transmission (when two-cable operation is used). The measurements are carried out both in sinusoidal and digital modes.

# 3	Cable specification

Administrations which decided to use cables designed earlier and used for analogue carrier systems with up to 120 channels in digital operation at bit rates 6 to 34 Mbit/s are recommended to choose cables with characteristics given in Tables 1/G.614 and 2/G.614.

3.1	Tables used for digital transmission systems with bit rates of 6 to 8 Mbit/s in one-cable operation

See Table 1/G.614.

TABLE 1/G.614

| Characteristics | Requirements |  |  |
| --- | --- | --- | --- |
| Types of cable | I (Note 1) | II (Note 1) | III (Note 1) |
| Operational bit rate C, kbit/s Line code Modulation rate, kbaud Tolerate attenuation of regenerator section at a frequency of C/2 when pairs of cable are of maximum use and directions of transmission are set in different quads (maximum permissible value), dB Diameter of copper conductor, mm Previous cable operating range Type of insulation Number of star quads Characteristic impedance at 1 MHz, ohms Nominal capacity, nF/km Attenuation coefficient, dB/km at 10°C - at 1 MHz -at a frequency C/2 Near-end crosstalk at a frequency of C/2, dB -mean value -minimum value Far-end crosstalk between pairs of different quads (minimum value referred to 1,000 m), dB - at 1 MHz - at a frequency of C/2 Far-end crosstalk between pairs of the same quad (minimum value at regenerator section of maximum length), dB - at 1 MHz - at a frequency of C/2 | 8448 HDB-3 8448 23 1.2 HF P1 4 165 24.5 4.8 10.6 48 34 54 42 60 43 | 8448 HDB-3 8448 23 1.2 HF P1 7 (Note 2) 165 24.5 4.5 9.7 50 34 54 42 60 43 | 8448 HDB-3 8448 45 (Note 3) 1.3 AF, HF P1, P 3, 4, 8 170 21.0 3.7 8.0 50 44 60 48 60 48 |

Note 1 - These characteristics relate to cables with aluminium covering.

Note 2 - Central quad not used for digital system transmission.

Note 3 - Regenerators of the transmission direction B-A installed in midpoint of the section of the opposite direction A-B.

HF	High-frequency

AF	Audio-frequency

Pl	String polysterene

P	Paper

3.2	Cables used for digital transmission systems with bit rates of 6 to 34.368 Mbit/s in two-cable operation

See Table 2/G.614.

TABLE 2/G.614

| Characteristics | Requirements |  |  |
| --- | --- | --- | --- |
| Type of cable | I (Note 1) | II (Note 1) | III (Note 1) |
| Operational bit rate C, kbit/s Line code Modulation rate, kbaud Attenuation of regenerator section at a frequency of C/2 when all pairs of cable are used (maximum permissible value), dB Diameter of copper conductor, mm Number of star quads Characteristic impedance at 1 MHz, ohms Nominal capacity, nF/km Attenuation coefficient, dB/km at 10°C - at 1 MHz - at a frequency C/2 Far-end crosstalk between pair of different quads (minimum value referred to 1,000m), dB - at 1 MHz - at 4 MHz - at 12 MHz - at 17 MHz Far-end crosstalk between pairs of the same quad (minimum value at a regenerator section of maximum length), dB - at 1 MHz - at 4 MHz - at 12 MHz - at 17 MHz | 8448 HDB-3 8448 70 1.2 4 165 24.5 4.8 10.6 54 42 - - 42 30 - - | 34 368 5B6B 41 242 85 1.2 4 165 24.5 4.8 24.0 51 42 32 30 - 33 (Note 2) 17 (Note 2) 13 (Note 2) | 34 368 5B6B 41 242 85 1.3 3, 4, 8 170 21.0 3.7 17.0 60 48 30 26 60 (Note 3) 48 (Note 3) 27 (Note 3) 17 (Note 3) |

Note 1 - These characteristics relate to cables with aluminium covering.

Note 2 - These values are obtained by means of transposition pattern No. 5 (see Figure 1/G.614) for four cable lengths (0.825 km).

Note 3 - These values are obtained by means of transposition pattern No. 2 (see Figure 1/G.614).