|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.622

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

TRANSMISSION  MEDIA  CHARACTERISTICS

CHARACTERISTICS  OF  1.2/4.4 mm  COAXIAL  CABLE  PAIRS

ITU-T  Recommendation  G.622

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.622 was published in Fascicle III.3 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.622

Fascicle III.3 - Rec. G.622

CHARACTERISTICS OF 1.2/4.4 mm COAXIAL CABLE PAIRS

(former Recommendation G.342; further amended)

The following Recommendation describes the 1.2/4.4 mm coaxial pair recommended by the CCITT for the international service. The use of this pair is defined in Tables 1 and 2 given in the introduction to Subsection 6.2. When the possibility of television or digital transmission has been envisaged, it is expressly mentioned in each provision.

# 1	Characteristics of the pair

1.1	Electrical characteristics of the coaxial pair

1.1.1	Characteristic impedance

The nominal real part of the characteristic impedance is 75 W at 1 MHz.

The tolerance is ± 1.5 W for telephony or ± 1 W for pairs that may be used for television transmissions.

For information, the impedance values in Table 1/G.622 were obtained at various frequencies on coaxial pairs manufactured by different processes.

TABLE 1/G.622

Means real part of the characteristic impedance measured at various frequencies

| Frequency (MHz) | 0.06 | 0.1 | 0.2 | 0.5 | 1 | 1.3 | 4.5 | 12 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Impedance (W) | 79.8 | 78.9 | 77.4 | 75.8 | 75 | 74.8 | 74 | 73.6 | 73.5 |

1.1.2	Attenuation coefficient

The nominal value of the attenuation coefficient of the pair, at 12 MHz and at 10°C, is 18.0 ± 0.4 dB/km.

Table 2/G.622 shows the general trend of the variation of the attenuation coefficient as a function of frequency for all pairs which conform to the present Recommendation.

TABLE 2/G.622

Nominal values of the attenuation coefficient at various frequencies

| Frequency (MHz) | 0.06 | 0.1 | 0.3 | 0.5 | 1 | 1.3 | 4.5 | 12 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attenuation coefficient (dB/km) | 1.5 | 1.8 | 2.9 | 3.7 | 5.3 | 6.0 | 11 | 18 | 22 |

The following equation, in which a is expressed in dB/km and f in MHz, gives an approximation of the attenuation coefficient from 2 MHz onwards:

µ §.

Note - By way of information, Annex A shows the values measured or specified in various countries, with the corresponding deviations or tolerances. In any case, amplifier design must be based on the values measured on the type of cable which will actually be used.

1.1.3	Attenuation distortion

The attenuation distortion required in particular for digital transmission is checked by calculating the ratio

µ § between attenuation values a f 1 and a f 2 measured at two frequencies f1 and f2.

One of the following three limits should be observed:1)

The attenuation distortion is checked in the factory on a small percentage of factory lengths.

1.2	Mechanical construction of the coaxial pair

The nominal dimensions are the following:

-	diameter of solid copper centre conductor: 1.2 mm;

-	inner diameter of outer conductor: 4.4 mm.

The cylindrical outer conductor is obtained using a copper tape with a thickness of 0.15 or 0.18 mm.

# 2	Cable specification

2.1	Characteristic impedance

To check that the value given in § 1.1.1 above is met, pulse measurements can be made. The real part of impedance at 1 MHz is to be taken as meaning the resistive component of the impedance at 1 MHz of the network with the best balance against the coaxial pair measured.

2.2	Impedance regularity

Routine control measurements of impedance regularity are carried out by means of pulse echometers from one or both ends of the factory lengths. The echo curve should be plotted with correction in amplitude and if possible in amplitude and phase. If the equivalent resistance error is measured, it must be corrected. However, for routine measurements, correction may be dispensed with if the test length is so short that the correction is small.

Table 3/G.622 shows the various values to be obtained according to the purpose for which the cable is intended.

TABLE 3/G.622

Echometric measurement of factory lengths

| Type of system |  |  |  | Analogue |  | Digital |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range or bit rate |  |  |  | 0.06-6 MHz | 0.3-20 MHz | Medium bit rate (6-34 Mbit/s) | High bit rate (140 Mbit/s) |
| Maximum pulse duration |  |  |  | 100 ns | 50 ns | 50 ns | 10 ns |
| General | Maximum |  | 100% | 45 dB | 48 dB | 48 dB | 48 dB |
| provisions | peak |  | 95% | 50 dB | 50dB | 50dB | 49dB |
| Additional | A | Mean of 3 maximum peaks |  | 48dB | 51dB | 51dB | 47 dB |
| optional provisionsa) | B | Equivalent resistance error |  | 1.2 W | 1.6 W | 1.6 W | 2.5 W |

a) It is enough to check that one of the two conditions A or B is fulfilled.

Note 1 - For 0.06-1.3 MHz analogue systems, the provisions are the same as for 0.06-6 MHz analogue systems.

Note 2 - To detect systematic irregularities, return wave attenuation measurements should be carried out on a small proportion of factory lengths. The limits to be observed are set out in Table 4/G.622.

Note 3 - The percentage figures given in the table relate to all the pairs of a batch of cables submitted for control or delivered at the same time.

TABLE 4/G.622

Return wave attenuation on irregularities

| Type of system |  | Digital |  |
| --- | --- | --- | --- |
| Frequency range or bit rate |  | Medium bit rate (6-34 Mbit/s) | High bit rate (140 Mbit/s) |
| Percentage of lengths concerned |  | about 5% | about 5% |
| Frequency band explored |  | 1-40 MHz | 20-100 MHz |
| Minimum measured value | 100% | 20 dB | 20 dB |
|  | 95% | 23 dB | 23 dB |

2.3	Attenuation coefficient

The attenuation of pairs should be such as to allow compliance with the provision of § 3.3 below2).

If reference is made to the length measured along a generatrix of the cable sheath, the attenuation coefficient should be multiplied by the take-up factor, the values of which for different numbers of pairs contained in the cable are given as an indication in Table 5/G.622.

TABLE 5/G.622

Take-up factor values

| Number of pairs in cable | Take-up factor last layer | Weighted take-up factor, entire cable |
| --- | --- | --- |
| 4 or 6 8 12-18 24 48 | 1.004 1.005 1.008 | 1.002 1.003 1.003 1.004 1.006 |

2.4	Crosstalk

The crosstalk between pairs should be such as to allow compliance with the provisions of § 3.4 below2).

2.5	Dielectric strength

The pair should withstand an a.c. voltage of 1000 V r.m.s. at 50 Hz (or a  d.c. voltage of 1500 V) applied for at least one minute between the centre and outer conductors.

If, in normal use, the outer conductors of the coaxial pair are not earthed, a dielectric strength test is made between the outer conductors and the earthed metallic sheath. The conductors of the auxiliary quads or pairs are connected to the outer conductors of the coaxial pairs or to the sheath, according to the kind of system used for these quads or pairs. Under these conditions, an a.c. voltage of 2000 V r.m.s. or more at 50 Hz will be applied for at least one minute (or a  d.c. voltage of 3000 V or more).

Note - The test voltages recommended take account of the normal safety margins applied in the various countries. Polythene insulation, however, might reasonably withstand considerably higher test voltages. In any case, some other dielectric might conceivably be used in the future.

2.6	Insulation resistance

The insulation resistance between the centre and outer conductors of the coaxial pair, measured with a perfectly steady voltage of between 100 and 500 V, should not be less than 5000 MW-km after electrification for one minute, at a temperature not lower than 15 °C. The measurement of the insulation resistance should be made after the dielectric strength test. This measurement should be made on each factory length.

# 3	Elementary cable section specification

3.1	End impedance

The conditions described in §§ 1.1.1 and 2.1 above are applicable.

3.2	Impedance regularity

Impedance regularity measurements are carried out from each end of the elementary cable section. Reference should be made to one of the columns in Table 6/G.622, according to the purpose for which the cable is intended.

3.3	Attenuation coefficient

At 1 MHz, the real attenuation coefficient must not differ from the nominal figure by more than ± 0.2 dB.

Attenuation measured on a cable at an average temperature of t°C is referred to 10 °C by the formula:

The coefficient kµ §, of the variation in attenuation with temperature is about 2 x 10-3 per °C at frequencies of 500 kHz or more. It increases slightly at lower frequencies (about 2.8 x 10-3 per °C at 60 kHz).

3.4	Crosstalk

The far-end crosstalk ratio between two coaxial pairs in a cable transmitting in the same direction at any frequency in the band actually transmitted must be not less than the values given in Table 7/G.622.

TABLE 6/G.622

Echometric measurement of elementary cable sectionsa)

| Type of system |  |  |  | Analogue |  | Digital |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range or bit rate |  |  |  | 0.06-6 MHz | 0.3-20 MHz | Medium bit rate (6-34 Mbit/s) | High bit rate (140 Mbit/s) |
| Maximum pulse duration |  |  |  | 200 ns | 100 ns | 100 ns | 50 ns |
| General | Maximum | 100% |  | 42 dB | 42 dB | 42 dB | 40 dB |
| provisions | peak | 95% |  | 46 dB | 46 dB | 46 dB | 44 dB |
| Additional |  | A | Mean of 3 maximum peaks. Uncorreted maximum | 45 dB 48 dB | 45 dB 48 dB | 45 dB 48 dB | 43 dB 46 dB |
| optional provisionsa) | Equivalent | B | Energy corrected (W × km -1/2) | 2 | 2.5 | 2.5 | 3.5 |
|  | resistance error | C | Uncorrected (W) | 1.8 | 2.0 | 2.0 | 2.5 |

a) It is enough to check that one of the three conditions A, B or C is fulfilled.

Note 1 - Notes 1 and 2 to Table 3/G.622 still hold good. However, for 0.06 to 1.3 MHz analogue systems, the provisions of column 0.06 to 6 MHz apply, but the pulse duration may attain 400 ns for elementary cable sections longer than 4 km.

Note 2 - Measurements using sine-wave signals on elementary cable sections are unnecessary unless there are serious grounds for believing that systematic irregularities may have been introduced during the laying or installation of the cable. In such cases, the measurement results should not be less than 20 dB.

TABLE 7/G.622

Minimum far-end crosstalk ratio between two 1.2/4.4 mm coaxial pairs

| Length of the section | Far-end crosstalk ratio (dB) |  |
| --- | --- | --- |
| (km) | Without phase inversion | With phase inversion at repeaters |
| 8 6 4 3 2 | 87 89 93 95 99 | - 80 - 83 - |

There is no need to specify a near-end crosstalk ratio when the former limits are chosen for the far-end crosstalk ratio.

When phase inversion is used, the near-end crosstalk ratio for pairs transmitting in opposite directions must be at least 84 dB for a section about 6 km long, and 87 dB for a section about 3 km long.

Note - These limits enable a far-end crosstalk ratio of 65 dB to be obtained on the worst homogenous 280-km section, assuming that for the frequencies in question only far-end crosstalk due to the cable is to be considered3). It is assumed that the variation in the minimum far-end crosstalk ratio as a function of the distance approximately follows a 
20 dB/decade law for distances below a limit distance L1 and a 10 dB/decade law for distances above L1. The values depend on a number of factors, mainly the system used, the type of cable and the considered frequency. A value of 30 km appears suitable in most cases, although values of L1 ranging from a few kilometers to 30 kilometers have been observed in practice, ensuring the consistency of the limits in Table 7/G.622 with the 65 dB limit on a 280 km section.

3.5	Dielectric strength

The pair must withstand a d.c. voltage of at least 1000 V applied during at least one minute between the inner and the outer conductors.

In addition, a test of dielectric strength between the coaxial pair and earth shall be made as described in § 2.5, using a d.c. voltage of at least 2000 V applied for one minute.

Note - The recommended test voltages take account of the normal safety margins applied in the various countries. Polythene insulation, however, might reasonably withstand considerably higher test voltages. In any case, some other dielectric might conceivably be used in the future.

3.6	Insulation resistance

The insulation resistance between the centre and outer conductors of the coaxial pair, measured with a perfectly steady voltage of between 100 and 500 V, should not be less than 5000 MW-km after electrification for one minute. The measurement of the insulation resistance should be made after the dielectric strength test. This measurement should be made on every elementary cable section.

ANNEX A

(to Recommendation G.622)

Examples of attenuation coefficient measured or specified in some countries

(Values given as an indication)

TABLE A-1/G.622

Values measured on a type of pair whose outer conductor is 0.15 mm thick

| Frequency (MHz) | 0.060 | 0.1 | 0.3 | 0.5 | 1 | 4 | 12 | 18 | 52 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Attenuation (dB/km) | 1.54 | 1.85 | 2.89 | 3.67 | 5.21 | 10.4 | 18.0 | 22.0 | 37.5 |
| Tolerance (dB/km) | ± 0.1 | ± 0.1 | ± 0.1 | ± 0.1 | ± 0.1 | ± 0.1 | ± 0.2 | ± 0.2 | ± 0.5 |
| Temperature coefficient | 0.0028 | 0.0026 | 0.0024 | 0.00225 | 0.0020 | 0.0020 | 0.0020 | 0.0020 | 0.0020 |

TABLE A-2/G.622

Values specified in certain countries for a type of pair whose outer conductor is 0.18 mm thick

| Frequency (MHz) | 60 | 100 | 200 | 300 | 500 | 700 | 1000 | 1300 | 4500 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Specific attenuation (dB/km) | 1.49 | 1.80 | 2.42 | 2.91 | 3.73 | 4.43 | 5.30 | 6.05 | 11.2 |
| Tolerance (dB/km) | ± 0.1 | ± 0.1 | a) | a) | a) | a) | ± 0.2 | ± 0.2 | ± 0.2 |

a) Not specified.

1)	These three conditions are equivalent. Accordingly, only one of them is to be used for checking attenuation distortion.

2)	At this stage of manufacture, attenuation and crosstalk measurements are merely prototype measurements.

3)	In practice it is possible to forget the influence of line equipment on intelligible crosstalk, but this is only true for low frequencies of the band (less than 300 kHz).