|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.691 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (01/2005) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Transmission media characteristics – Characteristics of optical components and subsystems |  |  |  |
|  | Optical interfaces for single channel STM-64 and other SDH systems with optical amplifiers Amendment 1: |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Amendment 1 to ITU-T Recommendation G.691

Optical interfaces for single channel STM-64 and other SDH systems with optical amplifiers

Amendment 1

# Introduction

This document contains the text of amendment 1 to Recommendation ITU-T G.691, "Optical interfaces for single channel STM-64 and other SDH systems with optical amplifiers" for consent.

This document supersedes the version of this amendment contained in D-95

Summary

This amendment contains modifications of the text of ITU-T Rec. G.691 (2003) to clarify the definition of side mode suppression ratio in clause 6.2.1.4 and remove any ambiguity in the text of the definition of receiver sensitivity contained in clause 6.4.1

# 1)	Clause 6.2.1.4

Modify clause 6.2.1.4 as follows:

#### 6.2.1.4	Side mode suppression ratio

The Side Mode Suppression Ratio (SMSR) is defined as the ratio of the largest peak of the total source spectrum to the second largest peak. The spectral resolution of the measurement shall be better (i.e., the optical filter bandwidth shall be less) than the maximum spectral width of the peak, as defined above. The second largest peak may be next to the main peak or far removed from it.

Note – within this definition spectral peaks that are separated from the largest peak by the clock frequency are not considered to be side modes.

The SMSR specification is intended to minimize the occurrence of BER degradations due to Mode Partition Noise (MPN). Since MPN is a transient effect with low probability, SMSR measurements on PRBS or continuous signals may underestimate the MPN. The SMSR specification is relevant only to SLM laser sources.

# 2)	Clause 6.4.1

Modify clause 6.4.1 as follows:

### 6.4.1	Sensitivity

Receiver sensitivity is defined as the minimum value of mean received power at point MPI-R to achieve a 1 × 10–12 BER. This must be met with a transmitter with worst-case values of transmitter eye mask, extinction ratio, optical return loss at point MPI-S, receiver connector degradations and measurement tolerances. The definition of receiver sensitivity under worst-case conditions is further discussed in Annex A.

The receiver sensitivity does not have to be met in the presence of dispersion or reflections from the optical path. These effects are specified separately in the allocation of maximum optical path penalty.

Note - the receiver sensitivity does not have to be met in the presence of transmitter jitter in excess of the appropriate jitter generation limit (e.g. G.783 for SDH optical tributary signals).

Ageing effects are not specified separately since they are typically negotiated between a network provider and an equipment manufacturer.

Typical margins between a beginning-of-life, nominal temperature receiver and its end-of-life, worst-case counterpart are desired to be in the 2 to 4 dB range. The receiver sensitivities specified in Tables 3 to 5 are worst-case, end-of-life values.

_____________