|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.694.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (06/2002) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Transmission media characteristics – Characteristics of optical components and subsystems |  |  |  |
|  | Spectral grids for WDM applications: DWDM frequency grid |  |  |  |
|  | ITU-T Recommendation G.694.1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| General | G.600–G.609 |
| Symmetric cable pairs | G.610–G.619 |
| Land coaxial cable pairs | G.620–G.629 |
| Submarine cables | G.630–G.649 |
| Optical fibre cables | G.650–G.659 |
| Characteristics of optical components and subsystems | G.660–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.694.1 Spectral grids for WDM applications: DWDM frequency grid

Summary This Recommendation provides a frequency grid for dense wavelength division multiplexing (DWDM) applications. The frequency grid, anchored to 193.1 THz, supports a variety of channel spacings ranging from 12.5 GHz to 100 GHz and wider.

Source ITU-T Recommendation G.694.1 was prepared by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 13 June 2002.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

3.1	Terms defined in this Recommendation		1

3.1.1	Frequency Grid		1

3.2	Terms defined in other Recommendations		1

4	Abbreviations and acronyms		1

5	Dense WDM and its applications		1

6	Nominal central frequencies for dense WDM systems		2

ITU-T Recommendation G.694.1

Spectral grids for WDM applications: DWDM frequency grid

# 1	Scope

The purpose of this Recommendation is to provide the definition of a frequency grid to support dense wavelength division multiplexing (DWDM) applications.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T Recommendation G.671 (2002), Transmission characteristics of optical components and subsystems.

# 3	Definitions

## 3.1	Terms defined in this Recommendation

This Recommendation defines the following terms:

### 3.1.1	Frequency Grid

A frequency grid is a reference set of frequencies used to denote allowed nominal central frequencies that may be used for defining applications.

## 3.2	Terms defined in other Recommendations

This Recommendation uses terms defined in ITU-T Rec. G.671:

–	Coarse Wavelength Division Multiplexing (CWDM).

–	Dense Wavelength Division Multiplexing (DWDM).

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations:

CWDM		Coarse Wavelength Division Multiplexing

DWDM		Dense Wavelength Division Multiplexing

WDM		Wavelength Division Multiplexing

# 5	Dense WDM and its applications

Dense Wavelength Division Multiplexing (DWDM), a WDM technology, is characterised by narrower channel spacing than Coarse WDM (CWDM) as defined in ITU-T Rec. G.671. In general the transmitters employed in DWDM applications require a control mechanism to enable them to meet the application's frequency stability requirements, in contrast to CWDM transmitters which are generally uncontrolled in this respect.

The frequency grid defined by this Recommendation supports a variety of channel spacings ranging from 12.5 GHz to 100 GHz and wider (integer multiples of 100 GHz). Uneven channel spacings are also allowed.

The current steps in channel spacing have historically evolved by sub-dividing the initial 100 GHz grid by successive factors of two.

# 6	Nominal central frequencies for dense WDM systems

For channel spacings of 12.5 GHz on a fibre, the allowed channel frequencies (in THz) are defined by:

193.1 + n × 0.0125 where n is a positive or negative integer including 0

For channel spacings of 25 GHz on a fibre, the allowed channel frequencies (in THz) are defined by:

193.1 + n × 0.025 where n is a positive or negative integer including 0

For channel spacings of 50 GHz on a fibre, the allowed channel frequencies (in THz) are defined by:

193.1 + n × 0.05 where n is a positive or negative integer including 0

For channel spacings of 100 GHz or more on a fibre, the allowed channel frequencies (in THz) are defined by:

193.1 + n × 0.1 where n is a positive or negative integer including 0

Table 1 illustrates some nominal central frequencies within the C and L bands based on the 12.5 GHz minimum channel spacing anchored to the 193.1 THz reference. Table 1 also illustrates the 25, 50 and 100 GHz grid frequencies within the same region. The endpoints shown are illustrative, not normative.

Note that the value of "c" (speed of light in vacuum) that should be used for converting between frequency and wavelength is 2.99792458  108 m/s.

| Table 1/G.694.1 – Example nominal central frequencies of the DWDM grid |  |  |  |  |
| --- | --- | --- | --- | --- |
| Nominal central frequencies (THz) for spacings of |  |  |  | Approximate nominal central wavelengths (nm) |
| 12.5 GHz | 25 GHz | 50 GHz | 100 GHz and above |  |
| • | • | • | • | • |
| • | • | • | • | • |
| • | • | • | • | • |
| 195.9375 | – | – | – | 1530.04 |
| 195.9250 | 195.925 | – | – | 1530.14 |
| 195.9125 | – | – | – | 1530.24 |
| 195.9000 | 195.900 | 195.90 | 195.9 | 1530.33 |
| 195.8875 | – | – | – | 1530.43 |
| 195.8750 | 195.875 | – | – | 1530.53 |
| 195.8625 | – | – | – | 1530.63 |
| 195.8500 | 195.850 | 195.85 | – | 1530.72 |
| 195.8375 | – | – | – | 1530.82 |
| 195.8250 | 195.825 | – | – | 1530.92 |
| 195.8125 | – | – | – | 1531.02 |
| 195.8000 | 195.800 | 195.80 | 195.8 | 1531.12 |
| 195.7875 | – | – | – | 1531.21 |
| 195.7750 | 195.775 | – | – | 1531.31 |
| 195.7625 | – | – | – | 1531.41 |
| 195.7500 | 195.750 | 195.75 | – | 1531.51 |
| 195.7375 | – | – | – | 1531.60 |
| 195.7250 | 195.725 | – | – | 1531.70 |
| 195.7125 | – | – | – | 1531.80 |
| 195.7000 | 195.700 | 195.70 | 195.7 | 1531.90 |
| 195.6875 | – | – | – | 1532.00 |
| 195.6750 | 195.675 | – | – | 1532.09 |
| 195.6625 | – | – | – | 1532.19 |
| • | • | • | • | • |
| • | • | • | • | • |
| • | • | • | • | • |
| • | • | • | • | • |
| • | • | • | • | • |
|  |  |  |  |  |
| 193.2375 | – | – | – | 1551.42 |
| 193.2250 | 193.225 | – | – | 1551.52 |
| 193.2125 | – | – | – | 1551.62 |
| 193.2000 | 193.200 | 193.20 | 193.2 | 1551.72 |
| 193.1875 | – | – | – | 1551.82 |
| 193.1750 | 193.175 | – | – | 1551.92 |
| 193.1625 | – | – | – | 1552.02 |
| 193.1500 | 193.150 | 193.15 | – | 1552.12 |
| 193.1375 | – | – | – | 1552.22 |
| 193.1250 | 193.125 | – | – | 1552.32 |
| 193.1125 | – | – | – | 1552.42 |
| 193.1000 | 193.100 | 193.10 | 193.1 | 1552.52 |
| 193.0875 | – | – | – | 1552.62 |
| 193.0750 | 193.075 | – | – | 1552.73 |
| 193.0625 | – | – | – | 1552.83 |
| 193.0500 | 193.050 | 193.05 | – | 1552.93 |
| 193.0375 | – | – | – | 1553.03 |
| 193.0250 | 193.025 | – | – | 1553.13 |
| 193.0125 | – | – | – | 1553.23 |
| 193.0000 | 193.000 | 193.00 | 193.0 | 1553.33 |
| 192.9875 | – | – | – | 1553.43 |
| 192.9750 | 192.975 | – | – | 1553.53 |
| 192.9625 | – | – | – | 1553.63 |
|  |  |  |  |  |
|  |  |  |  |  |
|  |  |  |  |  |
|  |  |  |  |  |
|  |  |  |  |  |
|  |  |  |  |  |
| 184.7750 | 184.775 | – | – | 1622.47 |
| 184.7625 | – | – | – | 1622.58 |
| 184.7500 | 184.750 | 184.75 | – | 1622.69 |
| 184.7375 | – | – | – | 1622.80 |
| 184.7250 | 184.725 | – | – | 1622.91 |
| 184.7125 | – | – | – | 1623.02 |
| 184.7000 | 184.700 | 184.70 | 184.7 | 1623.13 |
| 184.6875 | – | – | – | 1623.24 |
| 184.6750 | 184.675 | – | – | 1623.35 |
| 184.6625 | – | – | – | 1623.46 |
| 184.6500 | 184.650 | 184.65 | – | 1623.57 |
| 184.6375 | – | – | – | 1623.68 |
| 184.6250 | 184.625 | – | – | 1623.79 |
| 184.6125 | – | – | – | 1623.90 |
| 184.6000 | 184.600 | 184.60 | 184.6 | 1624.01 |
| 184.5875 | – | – | – | 1624.12 |
| 184.5750 | 184.575 | – | – | 1624.23 |
| 184.5625 | – | – | – | 1624.34 |
| 184.5500 | 184.550 | 184.55 | – | 1624.45 |
| 184.5375 | – | – | – | 1624.56 |
| 184.5250 | 184.525 | – | – | 1624.67 |
| 184.5125 | – | – | – | 1624.78 |
| 184.5000 | 184.500 | 184.50 | 184.5 | 1624.89 |
|  |  |  |  |  |
|  |  |  |  |  |
|  |  |  |  |  |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |