|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.695 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (01/2005) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Transmission media characteristics – Characteristics of optical components and subsystems |  |  |  |
|  | Optical interfaces for Coarse Wavelength Division Multiplexing applications |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.695

Optical interfaces for Coarse Wavelength Division Multiplexing applications

Summary

This Recommendation provides optical parameter values for physical layer interfaces of coarse wavelength division multiplexing (CWDM) applications with up to 16 channels and up to 2.5 Gbit/s. Applications are defined using two different methods, one using multichannel interface parameters and the other using single channel interface parameters. Both unidirectional and bidirectional applications are specified.

# 1	Scope

This recommendation applies to optical interfaces for Coarse Wavelength Division Multiplexing (CWDM) optical line systems for network applications using single-mode optical fibres.  This recommendation defines and provides values for optical interface parameters of physical point-to-point and ring CWDM system applications. Their principal purpose is to enable transversely (multi-vendor) compatible interfaces.

Applications are defined using two different methods, one using multichannel interface parameters and the other using single channel interface parameters. Both unidirectional and bidirectional applications are specified.

This Recommendation describes optical line systems that include the following features:

Maximum number of channels: up to 16

Bit-rate of signal channel: up to NRZ 2.5G

The CWDM wavelength grid is provided in G.694.2.

Specifications are organised according to application codes.

In the future, applications enabling full transverse compatibility at both the multichannel and single channel interface points may be included.

# 2	References

## 2.1	Normative references

- The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

	ITU-T G.652 (2003), Characteristics of a single-mode optical fibre cable.

	ITU-T G.653 (2003), Characteristics of a dispersion-shifted single-mode optical fibre cable.

	ITU-T G.655 (2003), Characteristics of a non-zero dispersion-shifted single-mode optical fibre cable.

	ITU-T G.664 (2003), Optical safety procedures and requirements for optical transport systems.

	ITU-T G.671 (2002), Transmission characteristics of optical components and subsystems.

	ITU-T G.691 (2003), Optical interfaces for single-channel STM-64, STM-256 and other SDH systems with optical amplifiers.

	ITU-T G.692 (1998), Optical interfaces for multichannel systems with optical amplifiers.

- ITU-T G.694.2 (2003), Spectral Grids for WDM Applications: CWDM Wavelength Grid.

	ITU-T G.709/Y.1331 (2001), Interfaces for the optical transport network (OTN).

- ITU-T G.957 (1999), Optical interfaces for equipments and systems relating to the synchronous digital hierarchy.

- ITU-T G.959.1 (2003), Optical transport network physical layer interfaces.

	IEC 60825-1, Safety of laser products - Part 1: Equipment classification, requirements and user's guide.

	IEC 60825-2, Safety of laser products - Part 2: Safety of optical fibre communication systems.

## 2.2	Informative references

The following ITU-T Recommendations contain provisions which, through reference in this text, constitute other relevant information:

	Supplement 39 of the G.600 series Recommendations (2003), Optical system design and engineering considerations

# 3	Terms and definitions

## 3.1	Definitions

## 3.2	Terms defined in other Recommendations

This Recommendation uses terms defined in G.671:

	Coarse Wavelength Division Multiplexing (CWDM)

	Optical wavelength multiplexer / demultiplexer

	Channel insertion loss

	Channel spacing

	Differential group delay

	Reflectance

This Recommendation uses terms defined in G.694.2

	Wavelength grid

This Recommendation uses the following term defined in ITU-T G.709:

	Completely standardized OTUk (OTUk)

This Recommendation uses the following terms defined in ITU-T G.872:

	Inter-domain interface (IrDI)

- 3R regeneration

This Recommendation uses the following term defined in ITU-T G.957:

- Joint engineering

- Receiver sensitivity

- Transverse compatibility

This Recommendation uses terms defined in G.959.1

- Minimum equivalent sensitivity

- Optical tributary signal class NRZ 1.25G

- Optical tributary signal class NRZ 2.5G

# 4	Abbreviations

- This Recommendation uses the following abbreviations:

3R	(Regeneration) Re-amplification, reshaping, retiming

ASE	Amplified spontaneous emission

BER	Bit error ratio

DGD	Differential group delay

EX	Extinction ratio

ffs	For further study

IrDI	Inter-domain interface

MPI	Main path interface

MPI-RM	Multichannel MPI reference point at the CWDM network element aggregate input

MPI-SM	Multichannel MPI reference point at the CWDM network element aggregate output

NA	Not applicable

NE	Network element

NRZ	Non-return to zero

OA	Optical amplifier

OADM	Optical add-drop multiplexer

OD	Optical demultiplexer

OM	Optical multiplexer

ONE	Optical network element

OTUk	Completely standardized optical channel transport unit - k

PMD	Polarization Mode Dispersion

RPR	Link reference point at the CWDM network element aggregate input

RPS	Link reference point at the CWDM network element aggregate output

RS	Single channel reference point at the CWDM network element tributary output

SS	Single channel reference point at the CWDM network element tributary input

WDM	Wavelength division multiplexing

# 5	Classification of optical interfaces

## 5.1	Applications

This Recommendation provides the physical layer parameters and values for CWDM multichannel and single channel interfaces in physical point-to-point and ring applications. CWDM systems can realise cost-effective applications, through a combination of uncooled single mode lasers, relaxed laser wavelength selection tolerances and wide pass-band filters. CWDM systems can be used in transport networks for a variety of clients, services, and protocols.

The specification method used in this Recommendation is categorised into two types.

The first one is a “black-box” approach which means that it is not intended to restrict or specify the internal elements and/or the connections between the elements within the black-box. There are, however, functional requirements for the black-box, the most important being the inclusion of 3R regeneration. This approach enables transverse compatibility at the multichannel points.

The second type is a “black-link” approach which means that optical interface parameters for only (single channel) optical tributary signals are specified. Additional informative descriptions are provided for the fibre link parameters of the multichannel section such as maximum attenuation, chromatic dispersion, and polarisation mode dispersion.  This approach enables transverse compatibility at the single channel point using a direct wavelength-multiplexing configuration. However, it does not enable transverse compatibility at the multichannel points.  In this approach, the OM and OD are treated as a single set of optical devices and OADMs can be included.

This Recommendation considers non-amplified multichannel interfaces only, however in the future, amplified interfaces may be considered.

### 5.2	Reference points

### 5.2.1	Unidirectional Applications

Figure 5-1 shows a set of reference points for multichannel connection (MPI-SM and MPI-RM) only, for the use of the “black-box” approach.  Here the CWDM network element includes an OM and transmitters, or an OD and receivers.

Figure 5-1/G.695 – “Black-box” approach

Figure 5-2 shows a set of reference points for the linear “black-link” approach, for single-channel connection (SS and RS) between transmitters (Tx) and receivers (Rx).   Here the CWDM network elements include an OM and an OD, which are used as a pair with the opposing element and may also include one or more OADMs.

Figure 5-2/G.695 – Linear “black-link” approach

Figure 5-3 shows a corresponding set of reference points for the ring “black-link” approach, for single-channel connection (SS and RS) between transmitters (Tx) and receivers (Rx). Here the CWDM network elements include two or more OADMs connected in a ring.

Figure 5-3/G.695 – Ring “black-link” approach

These reference models do not include any optical amplifiers in the CWDM system.  However in the future, applications including optical amplifiers may be introduced.

The reference points in Figures 5-1, 5-2 and 5-3 are defined as follows:

- SS is a single-channel reference point at the CWDM network element tributary input;

- RS is a single-channel reference point at the CWDM network element tributary output;

- MPI-SM is a multichannel reference point at the CWDM network element aggregate output;

- MPI-RM is a multichannel reference point at the CWDM network element aggregate input;

- RPS is a link reference point at the CWDM network element aggregate output;

- RPR is a link reference point at the CWDM network element aggregate input.

Here, single-channel reference points SS and RS are applied to systems for the (linear or ring) “black-link” approach where every path from SS to its corresponding RS must comply with the parameter values of the application codes in Table 8-11.  Multichannel reference points MPI-SM and MPI-RM are applied to systems for the “black-box” approach.  Link reference points RPS and RPR are applied only to systems for the “black-link” approach.

Note that MPI-SM and MPI-RM are defined to provide normative specifications for optical interfaces.  On the other hand, RPS and RPR are only defined to provide information for fibre link and not to provide signal characteristics at these points.

### 5.2.2	Bidirectional Applications

Figure 5-4 shows a set of reference points for multichannel connection (MPI-SM and MPI-RM) only, for the use of the “black-box” approach for single-fibre bidirectional applications.  Here the CWDM network element includes an OM/OD, transmitters and receivers.

Figure 5-4/G.695 “Black Box” Approach for Bidirectional Applications

Figure 5-5 shows a set of reference points for the single-fibre bidirectional linear “black-link” approach, for single-channel connection (SS and RS) between transmitters (Tx) and receivers (Rx).  Here the CWDM network elements include an OM/OD, which is used as a pair with the opposing element and may also include one or more OADMs.

Figure 5-5/G.695 Linear “black Link” Approach for Bidirectional Applications

Figure 5-6 shows a corresponding set of reference points for the single-fibre bidirectional ring “black-link” approach, for single-channel connection (SS and RS) between transmitters (Tx) and receivers (Rx). Here the CWDM network elements include two or more OADMs connected in a ring.

Figure 5-6/G.695 – Ring “black-link” approach for bidirectional applications

The reference points in Figures 5-4, 5-5 and 5-6 are as defined in clause 5.2.1.

### 5.3	Nomenclature

The application code identifies the network, implementation, and architectural characteristics of an application.

The application code notation is constructed as follows:

CnWx-ytz

Where,

C is the indicator of CWDM applications.

n is the maximum number of channels supported by the application code.

W is a letter indicating the span distance such as:

- S indicating short-haul;

- L indicating long-haul.

x is the maximum number of spans allowed within the application code.

y indicates the highest class of optical tributary signal supported:

-	0 indicating NRZ 1.25G;

- 1 indicating NRZ 2.5G.

t is a placeholder letter indicating the configuration supported by the application code. In the current version of this Recommendation, the only value used is:

- D indicating that the application does not contain any optical amplifiers.

z indicates the fibre types, as follows;

- 2 indicating G.652 fibre;

- 3 indicating G.653 fibre;

- 5 indicating G.655 fibre.

A bidirectional system is indicated by the addition of the letter B at the front of the application code.  For CWDM application codes this will be:

B-CnWx-ytz

A system using the “black-link” approach is indicated by the addition of the letter S at the front of the application code.  For CWDM application codes this will be:

S-CnWx-ytz

## 5.4	Multichannel interfaces at the reference points MPI-SM and MPI-RM

The multichannel interfaces described in clauses 5.4.1 and 5.4.2 are intended to enable transverse compatibility. These interfaces may operate on G.652, G.653 or G.655 fibre, simultaneously transporting up to 16 channels, using either NRZ 1.25G or NRZ 2.5G optical tributary signals, depending on the particular application code.

Further requirements related to transverse compatibility can be found in clause 6.

Tables 5-1 to 5-5 summarise the multichannel application codes, which are structured according to the nomenclature in clause 5.3.

Table 5-1/G.695 – Classification of 4-channel unidirectional multichannel interfaces

| Application | Short-haul (S) |  |  | Long-haul (L) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of fibre | G.652 | G.653 | G.655 | G.652 | G.653 | G.655 |
| Optical tributary signal class NRZ 1.25G | - | - | - | - | - | - |
| Target Distance for class NRZ 1.25G (km) a) | - | - | - | - | - | - |
| Optical tributary signal class NRZ 2.5G | C4S1-1D2 | C4S1-1D3 | C4S1-1D5 | C4L1-1D2 | C4L1-1D3 | C4L1-1D5 |
| Target Distance for class NRZ 2.5G (km) a) | 37 | 37 | 37 | 69 | 72 | 72 |
| a) – These target distances are for classification and not for specification. |  |  |  |  |  |  |

Table 5-2/G.695 – Classification of 4-channel bidirectional multichannel interfaces

| Application | Short-haul (S) | Long-haul (L) |  |
| --- | --- | --- | --- |
| Type of fibre | G.652 | G.652 | G.653 |
| Optical tributary signal class NRZ 1.25G | - | B-C4L1-0D2 | B-C4L1-0D3 |
| Target Distance for class NRZ 1.25G (km) a) | - | 90 | 90 |
| Optical tributary signal class NRZ 2.5G | - | B-C4L1-1D2 | B-C4L1-1D3 |
| Target Distance for class NRZ 2.5G (km) a) | - | 80 | 83 |
| a) – These target distances are for classification and not for specification. |  |  |  |

Table 5-3/G.695 – Classification of 8-channel multichannel interfaces

| Application | Short-haul (S) | Long-haul (L) |  |
| --- | --- | --- | --- |
| Type of fibre | G.652 | G.652 | G.653 |
| Optical tributary signal class NRZ 1.25G | - | B-C8L1-0D2 | B-C8L1-0D3 |
| Target Distance for class NRZ 1.25G (km) a) | - | 64 | 64 |
| Optical tributary signal class NRZ 2.5G | C8S1-1D2 B-C8S1-1D2 | C8L1-1D2 B-C8L1-1D2 | B-C8L1-1D3 |
| Target Distance for class NRZ 2.5G (km) a) | 27 | 55 | 58 |
| a) – These target distances are for classification and not for specification. |  |  |  |

Table 5-4/G.695 – Classification of 12-channel multichannel interfaces

| Application | Short-haul (S) | Long-haul (L) |  |
| --- | --- | --- | --- |
| Type of fibre | G.652 | G.652 | G.653 |
| Optical tributary signal class NRZ 1.25G | - | B-C12L1-0D2 | - |
| Target Distance for class NRZ 1.25G (km) a) | - | 42 | - |
| Optical tributary signal class NRZ 2.5G | - | B-C12L1-1D2 | - |
| Target Distance for class NRZ 2.5G (km) a) | - | 38 | - |
| a) – These target distances are for classification and not for specification. |  |  |  |

Table 5-5/G.695 – Classification of 16-channel multichannel interfaces

| Application | Short-haul (S) | Long-haul (L) |  |
| --- | --- | --- | --- |
| Type of fibre | G.652 | G.652 | G.653 |
| Optical tributary signal class NRZ 1.25G | - | - | - |
| Target Distance for class NRZ 1.25G (km) a) | - | - | - |
| Optical tributary signal class NRZ 2.5G | C16S1-1D2 B-C16S1-1D2 | C16L1-1D2 B-C16L1-1D2 | - |
| Target Distance for class NRZ 2.5G (km) a) | 20 | 42 | - |
| a) – These target distances are for classification and not for specification. |  |  |  |

### 5.4.1	Non-amplified multichannel interfaces

The non-amplified multichannel interfaces in this Recommendation are specified in Tables 8-1 to 8-10.

### 5.4.2	Amplified multichannel interfaces

Amplified multichannel interfaces may be introduced into this Recommendation in the future.

## 5.5	Single-channel interfaces at the reference points SS and RS

The single-channel interfaces described in clause 5.5.1 are intended to enable transverse compatibility at the single-channel interfaces at either end of the CWDM link as shown in Figures 5-2, 5-3, 5-5 and 5-6.

Further requirements related to transverse compatibility can be found in clause 6.

Table 5-6 summarises the single channel application codes, which are structured according to the nomenclature in clause 5.3. Expected distances for a variety of CWDM network element insertion loss values are provided in Appendix II and information concerning black-links containing OADMs is given in Appendix III.

Table 5-6/G.695 – Classification of multichannel systems with single channel interfaces

| Application | Short-haul (S) | Long-haul (L) |
| --- | --- | --- |
| Type of fibre | G.652, G.653, G.655 | G.652, G.653, G.655 |
| Optical tributary signal class NRZ 2.5G | S-C8S1-1D2, S-C8S1-1D3, S-C8S1-1D5 | S-C8L1-1D2, S-C8L1-1D3, S-C8L1-1D5 |

### 5.5.1	Non-amplified multichannel systems with single-channel interfaces

The non-amplified multichannel systems with single-channel interfaces in this Recommendation are specified in Table 8-11.

### 5.5.2	Amplified multichannel systems with single-channel interfaces

Amplified multichannel systems with single-channel interfaces may be introduced into this Recommendation in the future.

# Transverse compatibility

This Recommendation specifies parameters in order to enable transverse (i.e. multivendor) compatibility at multi-channel reference points MPI-SM and MPI-RM of the “black-box” approach CWDM network elements (NEs), and at single channel reference points SS and RS of the “black-link” approach CWDM NEs.

The multi-channel reference points MPI-SM and MPI-RM are intended to interconnect two aggregate interfaces of CWDM NEs, which may be from two different vendors.

The single channel reference points SS and RS are intended to make multiple tributary interfaces of CWDM NEs transversely compatible. In this case, multiple tributary signal transmitters (Tx i) and receivers (Rx i) may be from many different vendors. Note that CWDM NEs (OM and OD) for the “black-link” approach are from a single vendor, and considered as a single set of optical devices.

Transverse (multivendor) compatibility is enabled for:

- all multi-channel reference points MPI-SM and MPI-RM of “black-box” approach CWDM NEs having exactly the same application code.

Interconnection of aggregate interfaces with different application codes is a matter of joint engineering. Care must be taken particularly with respect to critical parameters that must be matched, e.g., MPI-SM output power, MPI-RM input power, etc.

- all single channel reference points SS and RS of “black-link” approach CWDM NEs having exactly the same application code.

Co-existence of tributary interfaces with different application codes is a matter of joint engineering. Care must be taken particularly with respect to critical parameters that must be consistent, e.g., SS output power and RS input power, SS bit rate/line coding and RS bit rate/line coding, etc.

# 7	Parameter definitions

The parameters in Tables 7-1 and 7-2 are defined at the interface points, and the definitions are provided in the clauses below.

Table 7-1/G.695: Physical layer parameters and values for CWDM applications using the “black-box” approach.

| Parameter | Units | Defined in |
| --- | --- | --- |
| General information |  |  |
| Maximum number of channels | - | 7.1.1 |
| Bit rate/line coding of optical tributary signals | - | 7.1.2 |
| Maximum bit error ratio | - | 7.1.3 |
| Fibre type | - | 7.1.4 |
| Interface at point MPI-SM |  |  |
| Maximum mean channel output power | dBm | 7.2.1 |
| Minimum mean channel output power | dBm | 7.2.1 |
| Maximum mean total output power | dBm | 7.2.2 |
| Central wavelength | nm | 7.2.3 |
| Channel spacing | nm | 7.2.4 |
| Maximum central wavelength deviation | nm | 7.2.5 |
| Minimum channel extinction ratio | dB | 7.2.6 |
| Eye mask | - | 7.2.7 |
| Optical path from point MPI-SM to MPI-RM |  |  |
| Maximum attenuation | dB | 7.3.1 |
| Minimum attenuation | dB | 7.3.2 |
| Maximum chromatic dispersion | ps/nm | 7.3.3 |
| Minimum optical return loss at MPI-SM | dB | 7.3.4 |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 7.3.5 |
| Maximum differential group delay | ps | 7.3.6 |
| Interface at point MPI-RM |  |  |
| Maximum mean channel input power | dBm | 7.4.1 |
| Minimum mean channel input power | dBm | 7.4.2 |
| Maximum mean total input power | dBm | 7.4.3 |
| Maximum optical path penalty | dB | 7.4.4 |
| Minimum equivalent sensitivity | dBm | 7.4.7 |
| Maximum reflectance of optical network element | dB | 7.4.5 |

Table 7-2/G.695:  Physical layer parameters and values for CWDM applications using the “black-link” approach.

| Parameter | Units | Defined in |
| --- | --- | --- |
| General information |  |  |
| Maximum number of channels | - | 7.1.1 |
| Bit rate/line coding of optical tributary signals | - | 7.1.2 |
| Maximum bit error ratio | - | 7.1.3 |
| Fibre type | - | 7.1.4 |
| Interface at point SS |  |  |
| Maximum mean channel output power | dBm | 7.2.1 |
| Minimum mean channel output power | dBm | 7.2.1 |
| Central wavelength | nm | 7.2.3 |
| Channel spacing | nm | 7.2.4 |
| Maximum central wavelength deviation | nm | 7.2.5 |
| Minimum channel extinction ratio | dB | 7.2.6 |
| Eye mask | - | 7.2.7 |
| Optical path from point SS to RS |  |  |
| Maximum channel insertion loss | dB | 7.5.1 |
| Minimum channel insertion loss | dB | 7.5.1 |
| Maximum chromatic dispersion | ps/nm | 7.3.3 |
| Minimum optical return loss at SS | dB | 7.3.4 |
| Maximum discrete reflectance between SS and RS | dB | 7.3.5 |
| Maximum differential group delay | ps | 7.3.6 |
| Maximum inter-channel crosstalk at Rs | dB | 7.5.2 |
| Maximum interferometric crosstalk at Rs | dB | 7.5.3 |
| Interface at point RS |  |  |
| Maximum mean channel input power | dBm | 7.4.1 |
| Receiver sensitivity | dBm | 7.4.6 |
| Maximum optical path penalty | dB | 7.4.4 |
| Maximum reflectance of receiver | dB | 7.4.5 |

### 7.1	General information

#### 7.1.1	Maximum number of channels

The maximum number of Optical Channels that may be simultaneously present at an interface.

For bidirectional applications, the maximum number of channels is expressed in the form n/2 + n/2 where n is the maximum number of channels supported by the application code and n/2 is the number of channels in each direction.

#### 7.1.2	Bit rate/line coding of optical tributary signals

Optical tributary signal class NRZ 1.25G applies to continuous digital signals with non-return to zero line coding, from nominally 622 Mbit/s to nominally 1.25 Gbit/s. Optical tributary signal class NRZ 2.5G applies to continuous digital signals with non-return to zero line coding, from nominally 622 Mbit/s to nominally 2.67 Gbit/s.

#### 7.1.3	Maximum bit error ratio

The parameters are specified relative to an optical section design objective of a Bit Error Ratio (BER) not worse than the value specified by the application code. This value applies to each Optical Channel under the extreme case of optical path attenuation and dispersion conditions in each application. The possible effect on the definition of this parameter due to the presence of forward error correction (e.g. in an OTUk) has not been considered in the present version of this Recommendation.

#### 7.1.4	Fibre type

Single mode optical fibre types are chosen from those defined in G.652, G.653, and G.655.

### 7.2	Interface at point MPI-SM or SS

#### 7.2.1	Maximum and minimum mean channel output power

The mean launched power of each Optical Channel at reference point MPI-SM or SS is the average power of a pseudo-random data sequence coupled into the fibre or the CWDM link. It is given as a range (maximum and minimum) to allow for some cost optimisation and to cover allowances for operation under the standard operating conditions, connector degradations, measurement tolerances, and aging effects.

#### 7.2.2	Maximum mean total output power

The maximum value of the mean launched optical power at point MPI-SM.

NOTE – Optical safety aspects have been considered in determining the values given in this Recommendation, since it is desirable to avoid the need for Automatic Power Reduction (APR), Automatic Power Shutdown (APSD), or Automatic Laser Shutdown (ALS) procedures, for cost reasons.

#### 7.2.3	Central wavelength

The nominal single channel wavelengths on which the digital coded information of the particular optical channels are modulated by use of the NRZ line code (as defined in G.957 and G.691)

The central wavelengths are based on the wavelength grid given in G.694.2. The allowed central wavelengths for the multichannel CWDM network element are specified in Tables 8-1 to 8-11.

Note that the value of "c" (speed of light in vacuum) that should be used for converting between frequency and wavelength is 2.99792458  108 m/s.

#### 7.2.4	Channel spacing

The nominal difference in wavelength between two adjacent channels. All possible tolerances of actual wavelengths are considered in subclause 7.2.5.

#### 7.2.5	Maximum central wavelength deviation

The difference between the nominal central wavelength and the actual central wavelength. The central wavelength deviation is determined by mainly two factors. First, the laser manufacturer is allowed a wavelength variation around the nominal wavelength in order to achieve a higher yield and/or relax fabrication tolerances. Second, the use of uncooled lasers will cause the wavelength to change with temperature within the specified temperature range of the laser.

Also included in the central wavelength deviation are all the processes that affect the instantaneous value of the source central wavelength over a measurement interval appropriate to the channel bit-rate. These processes include source chirp, information bandwidth, broadening due to self-phase modulation, and effects due to aging.

Maximum central wavelength deviation in CWDM point-to-point systems is provided in Tables 8-1 to 8-11.

#### 7.2.6	Minimum channel extinction ratio

The extinction ratio (EX) is defined in G.693 for a single channel parameter, as:

EX = 10log10(A/B)

In the above definition of EX, A is the average optical power level at the centre of a logical "1" and B is the average optical power level at the centre of a logical "0". The convention adopted for optical logic levels is:

-       emission of light for a logical "1";

-       no emission for a logical "0".

For multichannel interfaces, two alternative methods can be used for the verification of this parameter as in G.959.1:

-	Method A can be used when single channel reference points are accessible at the transmit end of the link for verification. For this method the procedures described in ITU-T G.957 and G.691 are used. The configuration for this method is contained in Annex A/G.959.1;

-	Method B employs a reference optical bandpass filter to isolate the individual transmitted signal. The characteristics of the reference optical bandpass filter are contained in Annex B/G.959.1.

#### 7.2.7	Eye mask

The definition and limits for this parameter are found in G.691.  This definition can be directly applied to single channel interfaces of the “black-link” approach.  In the case of the multichannel interfaces of the “black-box” approach, two alternative methods can be used as in G.959.1:

-	Method A can be used when single channel reference points are accessible at the transmit end of the link for verification.  For this method the procedures described in G.957 and G.691 are used.  The configuration for this method is contained in Annex A/G.959.1;

-	Method B employs a reference optical bandpass filter to isolate the individual transmitted signals, followed by a reference receiver.  The characteristics of the reference optical bandpass filter and the reference receiver are contained in Annex B/G.959.1.

### 7.3	Common optical path parameters (single span) from point MPI-SM to MPI-RM, or from SS to RS

#### 7.3.1	Maximum attenuation

The maximum path attenuation, for all wavelengths used by the application, where the system in question operates under end-of-life conditions at a BER of 10-12 (or as given by the application code), under worst-case transmit-side signal and dispersion. The definition of effects included in the maximum attenuation is given in clause 6.3.1/G.691.

The target distances for each application are based on the set of assumed maximum attenuation coefficients found in Appendix I. The values given represent installed fibre loss (including splices and cable margin). It should be noted that this method gives a theoretical value. Connector and splice losses as well as losses due to bending or optical monitoring, which can be present in practical implementations, may lead to other distances.

#### 7.3.2	Minimum attenuation

The minimum path attenuation that allows the system in question, operating under worst case transmit conditions to achieve a BER no worse than 10-12 (or as given by the application code).

#### 7.3.3	Maximum chromatic dispersion

This parameter defines the maximum value of the optical path chromatic dispersion that the system shall be able to tolerate. This is considered a worst-case dispersion value. The worst-case approach on this parameter is intended to give some margins on a sensitive parameter, as well as making it possible to stretch the transmission distances for low-loss fibre plants.

The allowed optical path penalty considers all deterministic effects due to chromatic dispersion as well as the penalty due to the maximum differential group delay.

#### 7.3.4	Minimum optical return loss at MPI-SM or SS

Reflections are caused by refractive index discontinuities along the optical path. If not controlled, they can degrade system performance through their disturbing effect on the operation of the optical source, or through multiple reflections which lead to interferometric noise at the receiver. Reflections from the optical path are controlled by specifying the:

- minimum optical return loss of the cable plant at the source reference point (i.e., MPI-SM, SS), including any connectors; and

- maximum discrete reflectance between source reference points (i.e., MPI-SM, SS) and receive reference points (i.e., MPI-RM, RS).

Reflectance denotes the reflection from any single discrete reflection point, whereas the optical return loss is the ratio of the incident optical power to the total returned optical power from the entire fibre including both discrete reflections and distributed backscattering such as Rayleigh scattering.

Measurement methods for reflections are described in Appendix I/G.957. For the purpose of reflectance and return loss measurements, points SS and RS are assumed to coincide with the endface of each connector plug. It is recognized that this does not include the actual reflection performance of the respective connectors in the operational system. These reflections are assumed to have the nominal value of reflection for the specific type of connectors used.

#### 7.3.5	Maximum discrete reflectance between MPI-SM and MPI-RM or between SS and RS

Optical reflectance is defined to be the ratio of the reflected optical power present at a point, to the optical power incident to that point. Control of reflections is discussed extensively in G.957. The maximum number of connectors or other discrete reflection points which may be included in the optical path (e.g. for distribution frames, or WDM components) must be such as to allow the specified overall optical return loss to be achieved. If this cannot be done using connectors meeting the maximum discrete reflections cited in the tables of clause 8, then connectors having better reflection performance must be employed. Alternatively, the number of connectors must be reduced. It also may be necessary to limit the number of connectors or to use connectors having improved reflectance performance in order to avoid unacceptable impairments due to multiple reflections.

In the tables of clause 8, the value of maximum discrete reflectance between source reference points and receive reference points is intended to minimize the effects of multiple reflections (e.g. interferometric noise). The value for maximum receiver reflectance is chosen to ensure acceptable penalties due to multiple reflections for all likely system configurations involving multiple connectors, etc. Systems employing fewer or higher performance connectors produce fewer multiple reflections and consequently are able to tolerate receivers exhibiting higher reflectance.

#### 7.3.6	Maximum differential group delay

Differential group delay (DGD) is the time difference between the fractions of a pulse that are transmitted in the two principal states of polarization of an optical signal. For distances greater than several kilometres, and assuming random (strong) polarization mode coupling, DGD in a fibre can be statistically modelled as having a Maxwellian distribution.

In this Recommendation, the maximum differential group delay is defined to be the value of DGD that the system must tolerate with a maximum sensitivity degradation of 1 dB.

Due to the statistical nature of Polarization Mode Dispersion (PMD), the relationship between maximum DGD and mean DGD can only be defined probabilistically. The probability of the instantaneous DGD exceeding any given value can be inferred from its Maxwellian statistics. Therefore, if we know the maximum DGD that the system can tolerate, we can derive the equivalent mean DGD by dividing by the ratio of maximum to mean that corresponds to an acceptable probability. Some example ratios are given below in Table 7-4.

Table 7-4/G.695 - DGD means and probabilities

| Ratio of maximum to mean | Probability of exceeding maximum |
| --- | --- |
| 3.0 | 4.2  105 |
| 3.5 | 7.7  107 |
| 4.0 | 7.4  109 |

### 7.4	Interface at point MPI-RM or RS

#### 7.4.1	Maximum mean channel input power

The maximum acceptable value of the average received channel power at point MPI-RM or RS to achieve the specified maximum BER of the application code.

#### 7.4.2	Minimum mean channel input power

The minimum acceptable value of the average received channel power at point MPI-RM or RS. The minimum mean channel input power is the minimum mean channel output power minus the maximum attenuation of the application.

NOTE 1 – The minimum mean channel input power at MPI-RM must be higher than the minimum equivalent sensitivity by the value of the maximum optical path penalty.

#### 7.4.3	Maximum mean total input power

The maximum acceptable total input power at point MPI-RM.

#### 7.4.4	Maximum optical path penalty

The path penalty is the apparent reduction of receiver sensitivity (or equivalent sensitivity in the case of the “black-box” approach) due to distortion of the signal waveform during its transmission over the path. It is manifested as a shift of the system's BER-curves towards higher input power levels. This corresponds to a positive path penalty. Negative path penalties may exist under some circumstances, but should be small. (A negative path penalty indicates that a less than perfect transmitter eye has been partially improved by the path dependent distortions.) Ideally, the BER-curves should be translated only, but shape variations are not uncommon, and may indicate the emergence of BER-floors. Since the path penalty is a change in the receiver's sensitivity, it is measured at a BER-level of 1012.

In the “black-box” approach (where minimum channel input power is specified) the maximum optical path penalty is equal to the difference between the minimum mean channel input power at MPI-RM and the minimum equivalent sensitivity.

For the applications defined in this Recommendation the path penalties are limited to a maximum of 1.5 dB for short haul systems and 2.5 dB for long haul systems. These limits are higher than in other Recommendations due to the additional penalty caused by optical crosstalk.

In the future, systems employing dispersion accommodation techniques based on pre-distortion of the signal at the transmitter may be introduced. In this case, the path penalty in the above sense can only be defined between points with undistorted signals. These points, however, do not coincide with the main path interfaces, and may thus not even be accessible. The definition of path penalty for this case is for further study.

The average value of the random dispersion penalties due to PMD is included in the allowed path penalty. In this respect, the transmitter/receiver combination is required to tolerate an actual DGD of 0.3 bit period with a maximum sensitivity degradation of 1 dB (with 50% of optical power in each principal state of polarization). For a well-designed receiver, this corresponds to a penalty of 0.1-0.2 dB for a DGD of 0.1 bit period. The actual DGD that may be encountered in operation is a randomly varying fibre/cable property, and cannot be specified in this Recommendation. This subject is further discussed in Appendix I/G.691.

Note that a signal to noise ratio reduction due to optical amplification is not considered a path penalty.

For applications using the “black-link” approach, path penalty includes crosstalk penalty.

For multichannel interfaces, two alternative methods can be used for the verification of this parameter:

-	Method A can be used when single channel reference points are accessible at the receive end of the link for verification. For this method the procedures described in ITU-T G.957 and G.691 are used. The configuration for this method is contained in Annex A/G.959.1.

-	Method B employs a reference optical bandpass filter to isolate the individual transmitted signals, followed by a reference receiver. The characteristics of the reference optical bandpass filter and the reference receiver are contained in Annex B/G.959.1.

NOTE – The optical path penalty observed in the reference receiver may not be exactly the same as actually experienced in the receiving equipment, depending on the design implementation.

#### 7.4.5	Maximum reflectance of CWDM network element or receiver

Reflections from the equipment back into the cable plant, or from the receiver back into the CWDM link, are specified by the maximum permissible reflectance of equipment or the receiver measured at reference point MPI-RM or at RS, respectively. Optical reflectance is defined in G.671.

#### 7.4.6	Receiver sensitivity

Receiver sensitivity is defined as the minimum value of average received power at point RS to achieve a 10–12 BER. This must be met with a transmitter with worst-case values of transmitter eye mask, extinction ratio, optical return loss at point SS, receiver connector degradations and measurement tolerances. The receiver sensitivity does not have to be met in the presence of dispersion, reflections from the optical path, or optical crosstalk; these effects are specified separately in the allocation of maximum optical path penalty.

NOTE – The receiver sensitivity does not have to be met in the presence of transmitter jitter in excess of the appropriate jitter generation limit (e.g. G.8251 for OTN Optical tributary signals).

Aging effects are not specified separately since they are typically a matter between a network operator and an equipment manufacturer.

#### 7.4.7	Minimum equivalent sensitivity

This is the minimum sensitivity that would be required of a receiver placed at MPI-RM to achieve the specified maximum BER of the application code if all except one of the channels were to be removed (with an ideal loss-less filter) at point MPI-RM. This would have to be met with a transmitter with worst-case values of transmitter eye mask, extinction ratio, optical return loss at point MPI-SM, connector degradations, transmit-side crosstalk, optical amplifier noise, and measurement tolerances. This sensitivity would not have to be met in the presence of dispersion, nonlinearity, reflections from the optical path or crosstalk; these effects are specified separately in the allocation of maximum optical path penalty.

NOTE 1 – The minimum mean channel input power at MPI-RM must be higher than the minimum equivalent sensitivity by the value of the maximum optical path penalty.

NOTE 2 – The receiver sensitivity does not have to be met in the presence of transmitter jitter in excess of the appropriate jitter generation limit (e.g. G.8251 for OTN Optical tributary signals).

Aging effects are not specified separately. Worst-case, end-of-life values are specified.

### 7.5	Additional parameters for the optical path from SS to RS

#### 7.5.1	Minimum and maximum channel insertion loss

#### Channel insertion loss is defined in G.671.

#### 7.5.2 	Maximum inter-channel crosstalk

This parameter places a requirement on the isolation of a link conforming to the "black-link" approach such that under the worst case operating conditions the inter-channel crosstalk at any reference point RS is less than the maximum inter-channel crosstalk value.

Inter-channel crosstalk is defined as the ratio of total power in all of the disturbing channels to that in the wanted channel, where the wanted and disturbing channels are at different wavelengths.

Specifically, the isolation of the link shall be greater than the amount required to ensure that when any channel is operating at the minimum mean output power at point SS and all of the others are at the maximum mean output power, then the inter-channel crosstalk at the corresponding point RS is less than the maximum inter-channel crosstalk value.

#### 7.5.3 	Maximum interferometric crosstalk

This parameter places a requirement on the isolation of a link conforming to the "black-link" approach such that under the worst case operating conditions the interferometric crosstalk at any reference point RS is less than the maximum interferometric crosstalk value.

Interferometric crosstalk is defined as the ratio of the disturbing power to the wanted power within a single channel, where the disturbing power is the power (not including ASE) within the optical channel that would remain if the wanted signal were removed from the link while leaving all of the other link conditions the same.

Specifically, the isolation of the link shall be greater than the amount required to ensure that when any channel is operating at the minimum mean output power at point SS and all of the others are at the maximum mean output power, then the interferometric crosstalk at the corresponding point RS is less than the maximum interferometric crosstalk value.

# 8	Parameter values

The physical layer parameters and values for multichannel interfaces, for “black-box” systems, are given in Tables 8-1 to 8-10.

The physical layer parameters and values for single channel interfaces, for “black-link” systems, are given in table 8-11.

Table 8-1/G.695 – Physical layer parameters and values for multichannel interfaces for 4 channel short haul black box applications

| Parameter | Units | C4S1-1D2 | C4S1-1D3 | C4S1-1D5 |
| --- | --- | --- | --- | --- |
| General information |  |  |  |  |
| Maximum number of channels | - | 4 |  |  |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |  |  |
| Maximum bit error ratio | - | 10-12 |  |  |
| Fibre type | - | G.652 | G.653 | G.655 |
| Interface at point MPI-SM |  |  |  |  |
| Maximum mean channel output power | dBm | +4.5 |  |  |
| Minimum mean channel output power | dBm |  |  |  |
| Maximum mean total output power | dBm | +10.5 |  |  |
| Central wavelength | nm | 1531 + 20m m=0 to 3 |  |  |
| Channel spacing | nm | 20 |  |  |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |  |  |
| Minimum channel extinction ratio | dB | 8.2 |  |  |
| Eye mask | - | STM-16 per G.957 |  |  |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |
| Maximum attenuation | dB | 10.5 |  |  |
| Minimum attenuation | dB | 4 |  |  |
| Maximum chromatic dispersion | ps/nm | 1000 | 200 | 500 |
| Minimum optical return loss at MPI-SM | dB | 24 |  |  |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 |  |  |
| Maximum differential group delay | ps | 120 |  |  |
| Interface at point MPI-RM |  |  |  |  |
| Maximum mean channel input power | dBm |  |  |  |
| Minimum mean channel input power | dBm | .5 |  |  |
| Maximum mean total input power | dBm |  |  |  |
| Maximum optical path penalty | dB | 1.5 |  |  |
| Minimum equivalent sensitivity | dBm | 15 |  |  |
| Maximum reflectance of optical network element | dB | 27 |  |  |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |

Table 8-2/G.695 – Physical layer parameters and values for multichannel interfaces for 4 channel long haul black box applications

| Parameter | Units | C4L1-1D2 | C4L1-1D3 | C4L1-1D5 |
| --- | --- | --- | --- | --- |
| General information |  |  |  |  |
| Maximum number of channels | - | 4 |  |  |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |  |  |
| Maximum bit error ratio | - | 10-12 |  |  |
| Fibre type | - | G.652 | G.653 | G.655 |
| Interface at point MPI-SM |  |  |  |  |
| Maximum mean channel output power | dBm | +4.5 |  |  |
| Minimum mean channel output power | dBm |  |  |  |
| Maximum mean total output power | dBm | +10.5 |  |  |
| Central wavelength | nm | 1531 + 20m m=0 to 3 |  |  |
| Channel spacing | nm | 20 |  |  |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |  |  |
| Minimum channel extinction ratio | dB | 8.2 |  |  |
| Eye mask | - | STM-16 per G.957 |  |  |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |
| Maximum attenuation | dB | 19.5 | 20.5 | 20.5 |
| Minimum attenuation | dB | 13 |  |  |
| Maximum chromatic dispersion | ps/nm | 1700 | 350 | 1000 |
| Minimum optical return loss at MPI-SM | dB | 24 |  |  |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 |  |  |
| Maximum differential group delay | ps | 120 |  |  |
| Interface at point MPI-RM |  |  |  |  |
| Maximum mean channel input power | dBm |  |  |  |
| Minimum mean channel input power | dBm | .5 | .5 |  |
| Maximum mean total input power | dBm | .5 |  |  |
| Maximum optical path penalty | dB | 2.5 | 1.5 | 1.5 |
| Minimum equivalent sensitivity | dBm | 5 |  |  |
| Maximum reflectance of optical network element | dB | 27 |  |  |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |

Table 8-3/G.695 – Physical layer parameters and values for multichannel interfaces for 4 channel bidirectional long haul black box applications

| Parameter | Units | B-C4L1-0D2 | B-C4L1-0D3 | B-C4L1-1D2 | B-C4L1-1D3 |
| --- | --- | --- | --- | --- | --- |
| General information |  |  |  |  |  |
| Maximum number of channels | - | 2+2 | 2+2 | 2+2 | 2+2 |
| Bit rate/line coding of optical tributary signals | - | NRZ 1.25G | NRZ 1.25G | NRZ 2.5G | NRZ 2.5G |
| Maximum bit error ratio | - | 10-12 | 10-12 | 10-12 | 10-12 |
| Fibre type | - | G.652 | G.653 | G.652 | G.653 |
| Interface at point MPI-SM |  |  |  |  |  |
| Maximum mean channel output power | dBm | +5 b) | +5 b) | +5 b) | +5 b) |
| Minimum mean channel output power | dBm | 0 b) | 0 b) | 0 b) | +0 b) |
| Maximum mean total output power | dBm | +8 | +8 | +8 | +8 |
| Central wavelength | nm | 1531 + 20m m=0 to 3 | 1531 + 20m m=0 to 3 | 1531 + 20m m=0 to 3 | 1531 + 20m m=0 to 3 |
| Channel spacing | nm | 20 | 20 | 20 | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 | ±6.5 | ±6.5 | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 | 8.2 | 8.2 | 8.2 |
| Eye mask | - | STM-4 per G.957 | STM-4 per G.957 | STM-16 per G.957 | STM-16 per G.957 |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |  |
| Maximum attenuation | dB | 25.5 | 25.5 | 22.5 | 23.5 |
| Minimum attenuation | dB | 12 | 12 | 12 | 12 |
| Maximum chromatic dispersion | ps/nm | 2400 | - | 2000 | - |
| Minimum optical return loss at MPI-SM | dB | 24 | 24 | 24 | 24 |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 | 27 | 27 | 27 |
| Maximum differential group delay | ps | 120 | 120 | 120 | 120 |
| Interface at point MPI-RM |  |  |  |  |  |
| Maximum mean channel input power | dBm |  |  |  |  |
| Minimum mean channel input power | dBm | 25.5 | 25.5 | 22.5 | 23.5 |
| Maximum mean total input power | dBm |  |  |  |  |
| Maximum optical path penalty | dB | 1.5 | 1.5 | 2.5 | 1.5 |
| Minimum equivalent sensitivity | dBm | 27 | 27 | 25 | 25 |
| Maximum reflectance of optical network element | dB | 27 | 27 | 27 | 27 |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. NOTE b) – The transmit power levels for these 2+2 channel bidirectional applications are higher than for other applications in this Recommendation so that a target distance of 80 km is achievable for NRZ 2.5G. |  |  |  |  |  |

Table 8-4/G.695 – Physical layer parameters and values for multichannel interfaces for 8 channel black box applications

| Parameter | Units | C8S1-1D2 | B-C8S1-1D2 | C8L1-1D2 |
| --- | --- | --- | --- | --- |
| General information |  |  |  |  |
| Maximum number of channels | - | 8 | 4+4 | 8 |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G | NRZ 2.5G | NRZ 2.5G |
| Maximum bit error ratio | - | 1012 | 1012 | 1012 |
| Fibre type | - | G.652 | G.652 | G.652 |
| Interface at point MPI-SM |  |  |  |  |
| Maximum mean channel output power | dBm | +4 | +4 | +4 |
| Minimum mean channel output power | dBm | 3.5 | 3.5 | 3.5 |
| Maximum mean total output power | dBm | +13 | +10 | +13 |
| Central wavelength | nm | 1471 + 20m m=0 to 7 | 1471 + 20m m=0 to 7 | 1471 + 20m m=0 to 7 |
| Channel spacing | nm | 20 | 20 | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 | ±6.5 | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 | 8.2 | 8.2 |
| Eye mask | - | STM-16 per G.957 | STM-16 per G.957 | STM-16 per G.957 |
| Optical path (single span) from point MPI-SM to MPI-RM, |  |  |  |  |
| Maximum attenuation | dB | 9 | 9 | 18 |
| Minimum attenuation | dB | 3 | 3 | 12 |
| Maximum chromatic dispersion | ps/nm | 800 | 800 | 1600 |
| Minimum optical return loss at MPI-SM | dB | 24 | 24 | 24 |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 | 27 | 27 |
| Maximum differential group delay | ps | 120 | 120 | 120 |
| Interface at point MPI-RM |  |  |  |  |
| Maximum mean channel input power | dBm | +1 | +1 | 8 |
| Minimum mean channel input power | dBm | 12.5 | 12.5 | 21.5 |
| Maximum mean total input power | dBm | +10 | +7 | +1 |
| Maximum optical path penalty | dB | 1.5 | 1.5 | 2.5 |
| Minimum equivalent sensitivity | dBm | 14 | 14 | 24 |
| Maximum reflectance of optical network element | dB | 27 | 27 | 27 |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |

Table 8-5/G.695 – Physical layer parameters and values for multichannel interfaces for 8 channel bidirectional long haul black box applications

| Parameter | Units | B-C8L1-0D2 | B-C8L1-0D3 | B-C8L1-1D2 | B-C8L1-1D3 |
| --- | --- | --- | --- | --- | --- |
| General information |  |  |  |  |  |
| Maximum number of channels | - | 4+4 | 4+4 | 4+4 | 4+4 |
| Bit rate/line coding | - | NRZ 1.25G | NRZ 1.25G | NRZ 2.5G | NRZ 2.5G |
| Maximum bit error ratio | - | 10-12 | 10-12 | 10-12 | 10-12 |
| Fibre type | - | G.652 | G.653 | G.652 | G.653 |
| Interface at point MPI-SM |  |  |  |  |  |
| Maximum mean channel output power | dBm | +4 | +4 | +4 | +4 |
| Minimum mean channel output power | dBm | 3.5 | 3.5 | 3.5 | 3.5 |
| Maximum mean total output power | dBm | +10 | +10 | +10 | +10 |
| Central wavelength | nm | 1471 + 20m m=0 to 7 | 1471 + 20m m=0 to 7 | 1471 + 20m m=0 to 7 | 1471 + 20m m=0 to 7 |
| Channel spacing | nm | 20 | 20 | 20 | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 | ±6.5 | ±6.5 | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 | 8.2 | 8.2 | 8.2 |
| Eye mask | - | STM-4 per G.957 | STM-4 per G.957 | STM-16 per G.957 | STM-16 per G.957 |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |  |
| Maximum attenuation | dB | 21 | 21 | 18 | 19 |
| Minimum attenuation | dB | 12 | 12 | 12 | 12 |
| Maximum chromatic dispersion | ps/nm | 2000 | - | 1600 | - |
| Minimum optical return loss at MPI-SM | dB | 24 | 24 | 24 | 24 |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 | 27 | 27 | 27 |
| Maximum differential group delay | ps | 120 | 120 | 120 | 120 |
| Interface at point MPI-RM |  |  |  |  |  |
| Maximum mean channel input power | dBm |  |  |  |  |
| Minimum mean channel input power | dBm |  |  | 21.5 | 22.5 |
| Maximum mean total input power | dBm | 2 | 2 | 2 | 2 |
| Maximum optical path penalty | dB | 1.5 | 1.5 | 2.5 | 1.5 |
| Minimum equivalent sensitivity | dBm | 26 | 26 | 24 | 24 |
| Maximum reflectance of optical network element | dB | 27 | 27 | 27 | 27 |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |  |

Table 8-6/G.695 – Physical layer parameters and values for multichannel interfaces for 12 channel bidirectional long haul black box applications

| Parameter | Units | B-C12L1-0D2 |  | B-C12L1-1D2 |  |
| --- | --- | --- | --- | --- | --- |
| Wavelength Block | nm | 1291-1351 | 1471-1611 | 1291-1351 | 1471-1611 |
| General information |  |  |  |  |  |
| Maximum number of channels | - | 6+6 |  | 6+6 |  |
| Bit rate/line coding of optical tributary signals | - | NRZ 1.25G |  | NRZ 2.5G |  |
| Maximum bit error ratio | - | 1012 |  | 1012 |  |
| Fibre type | - | G.652 |  | G.652 |  |
| Interface at point MPI-SM |  |  |  |  |  |
| Maximum mean channel output power | dBm | +3.5 |  | +3.5 |  |
| Minimum mean channel output power | dBm | 4 |  | 4 |  |
| Maximum mean total output power | dBm | +10.7 |  | +10.7 |  |
| Central wavelength | nm | 1291 + 20m, m=0 to 3 | 1471 + 20m, m=0 to 7 | 1291 + 20m, m=0 to 3 | 1471 + 20m, m=0 to 7 |
| Channel spacing | nm | 20 |  | 20 |  |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |  | ±6.5 |  |
| Minimum channel extinction ratio | dB | 8.2 |  | 8.2 |  |
| Eye mask | - | STM-4 per G.957 |  | STM-16 per G.957 |  |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |  |
| Maximum attenuation | dB | 20 | 14.7 | 18 | 13.3 |
| Minimum attenuation | dB | 11 | 7 | 11 | 7 |
| Maximum chromatic dispersion | ps/nm | 310 | 1200 | 280 | 1100 |
| Minimum optical return loss at MPI-SM | dB | 24 |  | 24 |  |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 |  | 27 |  |
| Maximum differential group delay | ps | 120 |  | 120 |  |
| Interface at point MPI-RM |  |  |  |  |  |
| Maximum mean channel input power | dBm | 7.5 | .5 | 7.5 | .5 |
| Minimum mean channel input power | dBm | 24 | 20.7 | 22 | 19.3 |
| Maximum mean total input power | dBm | +2.3 |  | +2.3 |  |
| Maximum optical path penalty | dB | 1 | 1 | 1 | 1.5 |
| Minimum equivalent sensitivity | dBm | 25 | 21.7 | 23 | 20.8 |
| Maximum reflectance of optical network element | dB | 27 |  | 27 |  |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |  |

Table 8-7/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application C16S1-1D2

| Parameter | Units | C16S1-1D2 |
| --- | --- | --- |
| Wavelength Block | nm | ffs |
| General information |  |  |
| Maximum number of channels | - | 16 |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |
| Maximum bit error ratio | - | 1012 |
| Fibre type | - | G.652.C or G.652.D |
| Interface at point MPI-SM |  |  |
| Maximum mean channel output power | dBm | ffs |
| Minimum mean channel output power | dBm | ffs |
| Maximum mean total output power | dBm | ffs |
| Central wavelength | nm | ffs |
| Channel spacing | nm | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 |
| Eye mask | - | STM-16 per G.957 |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |
| Maximum attenuation | dB | ffs |
| Minimum attenuation | dB | ffs |
| Maximum chromatic dispersion | ps/nm | ffs |
| Minimum optical return loss at MPI-SM | dB | ffs |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | ffs |
| Maximum differential group delay | ps | 120 |
| Interface at point MPI-RM |  |  |
| Maximum mean channel input power | dBm | ffs |
| Minimum mean channel input power | dBm | ffs |
| Maximum mean total input power | dBm | ffs |
| Maximum optical path penalty | dB | ffs |
| Minimum equivalent sensitivity | dBm | ffs |
| Maximum reflectance of optical network element | dB | ffs |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |

Table 8-8/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application C16L1-1D2

| Parameter | Units | C16L1-1D2 |
| --- | --- | --- |
| Wavelength Block | nm | ffs |
| General information |  |  |
| Maximum number of channels | - | 16 |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |
| Maximum bit error ratio | - | 1012 |
| Fibre type | - | G.652.C or G.652.D |
| Interface at point MPI-SM |  |  |
| Maximum mean channel output power | dBm | ffs |
| Minimum mean channel output power | dBm | ffs |
| Maximum mean total output power | dBm | ffs |
| Central wavelength | nm | ffs |
| Channel spacing | nm | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 |
| Eye mask | - | STM-16 per G.957 |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |
| Maximum attenuation | dB | ffs |
| Minimum attenuation | dB | ffs |
| Maximum chromatic dispersion | ps/nm | ffs |
| Minimum optical return loss at MPI-SM | dB | ffs |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | ffs |
| Maximum differential group delay | ps | 120 |
| Interface at point MPI-RM |  |  |
| Maximum mean channel input power | dBm | ffs |
| Minimum mean channel input power | dBm | ffs |
| Maximum mean total input power | dBm | ffs |
| Maximum optical path penalty | dB | ffs |
| Minimum equivalent sensitivity | dBm | ffs |
| Maximum reflectance of optical network element | dB | ffs |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |

Table 8-9/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application B-C16S1-1D2

| Parameter | Units | B-C16S1-1D2 |
| --- | --- | --- |
| Wavelength Block | nm | ffs |
| General information |  |  |
| Maximum number of channels | - | 8+8 |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |
| Maximum bit error ratio | - | 1012 |
| Fibre type | - | G.652.C or G.652.D |
| Interface at point MPI-SM |  |  |
| Maximum mean channel output power | dBm | ffs |
| Minimum mean channel output power | dBm | ffs |
| Maximum mean total output power | dBm | ffs |
| Central wavelength | nm | ffs |
| Channel spacing | nm | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 |
| Eye mask | - | STM-16 per G.957 |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |
| Maximum attenuation | dB | ffs |
| Minimum attenuation | dB | ffs |
| Maximum chromatic dispersion | ps/nm | ffs |
| Minimum optical return loss at MPI-SM | dB | ffs |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | ffs |
| Maximum differential group delay | ps | 120 |
| Interface at point MPI-RM |  |  |
| Maximum mean channel input power | dBm | ffs |
| Minimum mean channel input power | dBm | ffs |
| Maximum mean total input power | dBm | ffs |
| Maximum optical path penalty | dB | ffs |
| Minimum equivalent sensitivity | dBm | ffs |
| Maximum reflectance of optical network element | dB | ffs |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |

Table 8-10/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application B-C16L1-1D2

| Parameter | Units | B-C16L1-1D2 |
| --- | --- | --- |
| Wavelength Block | nm | ffs |
| General information |  |  |
| Maximum number of channels | - | 8+8 |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |
| Maximum bit error ratio | - | 1012 |
| Fibre type | - | G.652.C or G.652.D |
| Interface at point MPI-SM |  |  |
| Maximum mean channel output power | dBm | ffs |
| Minimum mean channel output power | dBm | ffs |
| Maximum mean total output power | dBm | ffs |
| Central wavelength | nm | ffs |
| Channel spacing | nm | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 |
| Eye mask | - | STM-16 per G.957 |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |
| Maximum attenuation | dB | ffs |
| Minimum attenuation | dB | ffs |
| Maximum chromatic dispersion | ps/nm | ffs |
| Minimum optical return loss at MPI-SM | dB | ffs |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | ffs |
| Maximum differential group delay | ps | 120 |
| Interface at point MPI-RM |  |  |
| Maximum mean channel input power | dBm | ffs |
| Minimum mean channel input power | dBm | ffs |
| Maximum mean total input power | dBm | ffs |
| Maximum optical path penalty | dB | ffs |
| Minimum equivalent sensitivity | dBm | ffs |
| Maximum reflectance of optical network element | dB | ffs |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |

Table 8-11/G.695 – Physical layer parameters and values for multichannel systems with single channel interfaces for 8 channel black link applications

| Parameter | Units | S-C8S1-1D2 S-C8S1-1D3 S-C8S1-1D5 | S-C8L1-1D2 S-C8L1-1D3 S-C8L1-1D5 |
| --- | --- | --- | --- |
| General information |  |  |  |
| Maximum number of channels | - | 8 | 8 b) |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G | NRZ 2.5G |
| Maximum bit error ratio | - | 1012 | 1012 |
| Fibre type | - | G.652. G.653, G.655 | G.652. G.653, G.655 |
| Interface at point SS |  |  |  |
| Maximum mean channel output power | dBm | +5 | +5 |
| Minimum mean channel output power | dBm | 0 | 0 |
| Central wavelength | nm | 1471 + 20m m=0 to 7 | 1471 + 20m m=0 to 7 |
| Channel spacing | nm | 20 | 20 |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 | ±6.5 |
| Minimum channel extinction ratio | dB | 8.2 | 8.2 |
| Eye mask | - | STM-16 per G.957 | STM-16 per G.957 |
| Optical path from point SS to RS, |  |  |  |
| Maximum channel insertion loss | dB | 16.5 | 25.5 |
| Minimum channel insertion loss | dB | 5 | 14 |
| Maximum chromatic dispersion | ps/nm | 1000 | 1600 |
| Minimum optical return loss at SS | dB | 24 | 24 |
| Maximum discrete reflectance between SS and RS | dB | 27 | 27 |
| Maximum differential group delay | ps | 120 | 120 |
| Maximum inter-channel crosstalk | dB | 20 | 20 |
| Maximum interferometric crosstalk | dB | 45 | 45 |
| Interface at point RS |  |  |  |
| Maximum mean channel input power | dBm | 0 | 9 |
| Minimum receiver sensitivity | dBm | 18 | 28 |
| Maximum optical path penalty | dB | 1.5 | 2.5 |
| Maximum reflectance of receiver | dB | 27 | 27 |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. NOTE b) – The 1471 nm channel may not be usable with older G.655 fibre that has a maximum cable cut-off wavelength specified as 1480 nm. |  |  |  |

# 9	Optical safety considerations

See ITU-T Rec. G.664, IEC 60825-1 and IEC 60825-2 for optical safety considerations.

NOTE  Accessible emission limits for wavelengths above and below 1400 nm differ. Therefore, appropriate consideration must be given to how wavelengths in each of these regions contribute to the hazard level classification for CWDM applications.

# Appendix I

Wavelength dependence of attenuation and chromatic dispersion

# I.1	Attenuation

The attenuation coefficient of an installed optical fibre cable is wavelength dependent, the value at a particular wavelength depending on the characteristics of the un-cabled fibre together with additional losses caused by connectors, splicing and bending.

Table I.1 contains the assumed minimum and maximum attenuation coefficient values for each CWDM wavelength. They are also depicted graphically in Figures I.1 and I.2. These values were obtained by combining measurements of the attenuation coefficient of underground and buried optical fibre cables at 1550 nm and 1625 nm with full spectrum measurements of un-cabled fibres and with the limits specified in G.652.

Table I.1/G.695 – Assumed attenuation coefficient values

| Nominal central wavelength (nm) | G.652.A&B cable |  | G.652.C&D cable |  |
| --- | --- | --- | --- | --- |
|  | Minimum Attenuation Coefficient (dB/km) | Maximum Attenuation Coefficient (dB/km) | Minimum Attenuation Coefficient (dB/km) | Maximum Attenuation Coefficient (dB/km) |
| 1271 | 0.392 | 0.473 | 0.385 | 0.470 |
| 1291 | 0.370 | 0.447 | 0.365 | 0.441 |
| 1311 | 0.348 | 0.423 | 0.352 | 0.423 |
| 1331 | 0.331 | 0.425 | 0.340 | 0.411 |
| 1351 | 0.320 | 0.476 | 0.329 | 0.399 |
| 1371 |  |  | 0.316 | 0.386 |
| 1391 |  |  | 0.301 | 0.372 |
| 1411 |  |  | 0.285 | 0.357 |
| 1431 | 0.263 | 0.438 | 0.269 | 0.341 |
| 1451 | 0.250 | 0.368 | 0.254 | 0.326 |
| 1471 | 0.238 | 0.327 | 0.240 | 0.312 |
| 1491 | 0.229 | 0.303 | 0.229 | 0.300 |
| 1511 | 0.221 | 0.290 | 0.220 | 0.290 |
| 1531 | 0.215 | 0.283 | 0.213 | 0.283 |
| 1551 | 0.211 | 0.278 | 0.209 | 0.277 |
| 1571 | 0.208 | 0.276 | 0.208 | 0.273 |
| 1591 | 0.208 | 0.278 | 0.208 | 0.275 |
| 1611 | 0.208 | 0.289 | 0.212 | 0.283 |

NOTE – These coefficient values include an allowance for maximum central wavelength deviation.

Figure I.1/G.695 – Assumed attenuation coefficient values for G.652.A&B cable

Figure I.2/G.695 – Assumed attenuation coefficient values for G.652.C&D cable

These attenuation coefficient values are based on the spectral results of a limited number of fibres, together with an assumption of 0.275 dB/km at 1550 nm for the maximum attenuation coefficients and 0.210 dB/km at 1550 nm for the minimum attenuation coefficients. Actual installed cable attenuation is statistical in nature and these values should not be taken as specification limits on individual fibres, cable sections, or splices. In actual installed optical fibre cables, the attenuation coefficient values will differ from those shown in Table I.1 and Figure I.1 depending on factors such as connector loss, splicing loss, bending loss, or loss due to optical monitoring.

# I.2	Chromatic Dispersion

As the chromatic dispersion coefficient is wavelength dependent, some applications will be dispersion limited by the longest wavelength that is being used. Table I.2 contains the assumed maximum chromatic dispersion coefficient values at the longest central wavelength (plus the maximum allowed central wavelength deviation) for each block of wavelengths used in this Recommendation. These values of chromatic dispersion have been calculated using Supplement 39 of the G.600 and G.900 series Recommendations equations 10.7a and 10.7b, with 1 sigma.

Table I.2/G.695 – Assumed chromatic dispersion coefficient values

| Wavelength Block (nm) | Dispersion Coefficient (ps/nm/km) |
| --- | --- |
| 1291-1351 | 5.7 |
| 1311-1371 | 6.8 |
| 1391-1451 | 11.5 |
| 1531-1591 | 19.9 |
| 1471-1611 | 21.1 |

# Appendix II

Optical path from point RPS to RPR

Table 8-11 recommends physical layer parameter values for the optical path from point SS to point RS for Black-Link applications. The optical path from point SS to RS includes the path from RPS to RPR and a number of other network elements (NEs). In the case of linear black-link applications the NEs include an OM, an OD and (optionally) one or more OADMs. For ring black-link applications the NEs include all of the OADMs that are traversed by the path from SS to RS being considered. The total insertion loss and the total chromatic dispersion of the CWDM NEs and the optical path from RPS to RPR must not exceed the values specified for the optical path from point SS to RS.

Apportionment of (wavelength dependent) insertion loss to the CWDM NEs and to the optical path from RPS to RPR will depend on the characteristics of the NEs and the optical path from RPS to RPR. The assumed maximum attenuation coefficient values in Appendix I can be used to evaluate an assumed maximum channel insertion loss of each channel for the optical path from RPS to RPR and therefore expected distances for high loss fibre. Similarly for low loss fibres, the minimum attenuation coefficient values in Appendix I can be used. In some cases, the expected distance will be dispersion limited.

Tables II.1 and II.2 contain informative parameter values for the optical path from RPS to RPR for various CWDM NE insertion loss values.

Table II.1/G.695 – Parameters and values for optical path from RPS to RPR for application codes S-C8S1-1D2, -1D3 and -1D5

| Parameter | Units | Total CWDM network element insertion loss |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 7.5 dB | 6.5 dB | 5.5 dB | 4.5 dB | 3.5 dB |
| Optical path from point RPS to RPR |  |  |  |  |  |  |
| Maximum attenuation | dB | 9 | 10 | 11 | 12 | 13 |
| Minimum attenuation | dB | 0 | 0 | 0 | 0.5 | 1.5 |
| Maximum chromatic dispersion | ps/nm | 1000 | 1000 | 1000 | 1000 | 1000 |
| Maximum differential group delay | ps | 120 | 120 | 120 | 120 | 120 |
| Expected distance for high loss fibre a) | km | 27 | 30 | 33 | 36 | 39 |
| Expected distance for low loss fibre a) | km | 38 | 42 | 46 | 50 b) | 55 b) |

NOTE a) – In actual installed optical fibre cables, the expected distance may differ from the values shown depending on variations in factors such as connector loss, splicing loss, bending loss.

NOTE b) – For application code S-C8S1-1D2 which uses G.652 fibre, the expected distance is dispersion limited to approximately 47 km.

Table II.2/G.695 – Parameters and values for optical path from RPS to RPR for application codes S-C8L1-1D2, -1D3 and -1D5

| Parameter | Units | Total CWDM network element insertion loss |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 7.5 dB | 6.5 dB | 5.5 dB | 4.5 dB | 3.5 dB |
| Optical path from point RPS to RPR |  |  |  |  |  |  |
| Maximum attenuation | dB | 18 | 19 | 20 | 21 | 22 |
| Minimum attenuation | dB | 6.5 | 7.5 | 8.5 | 9.5 | 10.5 |
| Maximum chromatic dispersion | ps/nm | 1600 | 1600 | 1600 | 1600 | 1600 |
| Maximum differential group delay | ps | 120 | 120 | 120 | 120 | 120 |
| Expected distance for high loss fibre a) | km | 55 | 58 | 61 | 64 | 67 |
| Expected distance for low loss fibre a) | km | 75 | 79 a) | 84 b) | 88 b) | 92 b) |

NOTE a) – In actual installed optical fibre cables, the expected distance may differ from the values shown depending on variations in factors such as connector loss, splicing loss, bending loss.

NOTE b) – For application code S-C8L1-1D2 which uses G.652 fibre, the expected distance is dispersion limited to approximately 75 km.

# Appendix III

Black-links containing OADMs

## III.1	Number of OADMs in a black-link

The number of OADMs that can be used in a linear black-link or on a black-link ring depends on OM, OADM, OD, fibre and connector losses. The total loss from SS to RS must exceed the minimum channel insertion loss and must not exceed the maximum channel insertion loss for the application code being used for the path from SS to RS. Therefore we have:

ILmin  ILtotal  ILmax

Where:

ILmin	minimum channel insertion loss for the application code

ILmax	maximum channel insertion loss for the application code

And:

ILtotal=ILOM+NOADMILOADM+ILOD+NconILcon+α⋅L

Where:

ILOM	OM insertion loss or OADM add loss at point SS for the wavelength being used from SS to RS

NOADM	number of express OADMs

ILOADM	express OADM insertion loss for the wavelength being used from SS to RS

ILOD	OD insertion loss or OADM drop loss at point RS for the wavelength being used from SS to RS

Ncon	number of connectors between SS and RS

ILcon	connector insertion loss

	attenuation coefficient of the fibre, in dB/km, for the wavelength being used from SS to RS

L	total length of fibre between SS and RS

An express OADM is one through which the wavelength of interest passes without being added or dropped. The maximum number of express OADMs in a path between SS and RS is therefore given by:

NOADM=ILmax−ILOM−ILOD−NconILcon−α⋅LILOADM

The evaluation of the maximum number of OADMs must be done for each SS to RS path in the network so that the maximum number of OADMs is not exceeded for any SS to RS path. This is quite simple for networks where all of the paths share a common hub (see Figure III.1), but becomes more complicated as the path topology becomes more complex (see Figure III.2).

Figure III.1/G.695 – Simple example of linear black-link topology

Figure III.2/G.695 – Complex example of ring black-link topology

For some paths, it may be necessary to add some optical attenuation on the black-link side of the SS or RS interface so that the minimum channel insertion loss requirement of the black-link between SS and RS is met, without affecting the loss for other paths.

## III.2	Mixed Application Codes

It is possible to use a mixture of different, but compatible, application codes on the same black-link. For example, low loss paths may use S-C8S1-1D2 whilst high loss paths may use S-C8L1-1D2.

## III.3	Protection

CWDM black-link rings offer the possibility of protected optical paths. Protection may be implemented by several means, including:

- Client-level protection, with the CWDM black-link ring providing two physically diverse optical paths between a pair of clients. These two optical paths may or may not use the same CWDM wavelength.

- Integrated protection, with the OADMs providing a single client interface and with protection switching within the OADM.

The characteristics of systems that provide protection switching are outside the scope of this Recommendation, however, the application codes in this Recommendation can be used in these systems as long as the optical path between SS and RS always complies with the requirements of the relevant application code.

# Appendix IV

Parameter Values for 16 Channel Applications

This appendix provides initial parameter values for 16 channel applications. It is anticipated that as the technology evolves these values may need to be revised.

Table IV.1/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application C16S1-1D2

| Parameter | Units | C16S1-1D2 |  |  |
| --- | --- | --- | --- | --- |
| Wavelength Block | nm | 1311-1371 | 1391-1451 | 1471-1611 |
| General information |  |  |  |  |
| Maximum number of channels | - | 16 |  |  |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |  |  |
| Maximum bit error ratio | - | 1012 |  |  |
| Fibre type | - | G.652.C or G.652.D |  |  |
| Interface at point MPI-SM |  |  |  |  |
| Maximum mean channel output power | dBm | +3.5 | +2.5 | +1 |
| Minimum mean channel output power | dBm | 4 | 5 | 6.5 |
| Maximum mean total output power | dBm | +14.2 |  |  |
| Central wavelength | nm | 1311 + 20m, m=0 to 3 | 1391 + 20m, m=0 to 3 | 1471 + 20m, m=0 to 7 |
| Channel spacing | nm | 20 |  |  |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |  |  |
| Minimum channel extinction ratio | dB | 8.2 |  |  |
| Eye mask | - | STM-16 per G.957 |  |  |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |
| Maximum attenuation | dB | 8.5 | 7.5 | 6.5 |
| Minimum attenuation | dB | 3.5 | 2.5 | 0.5 |
| Maximum chromatic dispersion | ps/nm | 160 | 300 | 510 |
| Minimum optical return loss at MPI-SM | dB | 24 |  |  |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 |  |  |
| Maximum differential group delay | ps | 120 |  |  |
| Interface at point MPI-RM |  |  |  |  |
| Maximum mean channel input power | dBm | 0 | 0 | +0.5 |
| Minimum mean channel input power | dBm | 12.5 | 12.5 | 13 |
| Maximum mean total input power | dBm | +12.3 |  |  |
| Maximum optical path penalty | dB | 1 | 1 | 1 |
| Minimum equivalent sensitivity | dBm | 13.5 | 13.5 | 14 |
| Maximum reflectance of optical network element | dB | 27 |  |  |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |

Table IV.2/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application C16L1-1D2

| Parameter | Units | C16L1-1D2 |  |  |
| --- | --- | --- | --- | --- |
| Wavelength Block | nm | 1311-1371 | 1391-1451 | 1471-1611 |
| General information |  |  |  |  |
| Maximum number of channels | - | 16 |  |  |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |  |  |
| Maximum bit error ratio | - | 1012 |  |  |
| Fibre type | - | G.652.C or G.652.D |  |  |
| Interface at point MPI-SM |  |  |  |  |
| Maximum mean channel output power | dBm | +3.5 | +1.5 | 0.5 |
| Minimum mean channel output power | dBm | 4 | 6 | 8 |
| Maximum mean total output power | dBm | +13.4 |  |  |
| Central wavelength | nm | 1311 + 20m, m=0 to 3 | 1391 + 20m, m=0 to 3 | 1471 + 20m, m=0 to 7 |
| Channel spacing | nm | 20 |  |  |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |  |  |
| Minimum channel extinction ratio | dB | 8.2 |  |  |
| Eye mask | - | STM-16 per G.957 |  |  |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |
| Maximum attenuation | dB | 18 | 15.8 | 13.3 |
| Minimum attenuation | dB | 11 | 9 | 7 |
| Maximum chromatic dispersion | ps/nm | 350 | 650 | 1100 |
| Minimum optical return loss at MPI-SM | dB | 24 |  |  |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 |  |  |
| Maximum differential group delay | ps | 120 |  |  |
| Interface at point MPI-RM |  |  |  |  |
| Maximum mean channel input power | dBm | 7.5 |  |  |
| Minimum mean channel input power | dBm | 22 | 21.8 | 21.3 |
| Maximum mean total input power | dBm | +4.5 |  |  |
| Maximum optical path penalty | dB | 1 | 1.5 | 2 |
| Minimum equivalent sensitivity | dBm | 23 | 23.3 | 23.3 |
| Maximum reflectance of optical network element | dB | 27 |  |  |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |

Table IV.3/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application B-C16S1-1D2

| Parameter | Units | B-C16S1-1D2 |  |  |
| --- | --- | --- | --- | --- |
| Wavelength Block | nm | 1311-1371 | 1391-1451 | 1471-1611 |
| General information |  |  |  |  |
| Maximum number of channels | - | 8+8 |  |  |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |  |  |
| Maximum bit error ratio | - | 1012 |  |  |
| Fibre type | - | G.652.C or G.652.D |  |  |
| Interface at point MPI-SM |  |  |  |  |
| Maximum mean channel output power | dBm | +3.5 | +2 | +1 |
| Minimum mean channel output power | dBm | 4 | 5 | 6.5 |
| Maximum mean total output power | dBm | +11.8 |  |  |
| Central wavelength | nm | 1311 + 20m, m=0 to 3 | 1391 + 20m, m=0 to 3 | 1471 + 20m, m=0 to 7 |
| Channel spacing | nm | 20 |  |  |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |  |  |
| Minimum channel extinction ratio | dB | 8.2 |  |  |
| Eye mask | - | STM-16 per G.957 |  |  |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |
| Maximum attenuation | dB | 8.5 | 7.5 | 6.3 |
| Minimum attenuation | dB | 3.5 | 2.5 | 0.5 |
| Maximum chromatic dispersion | ps/nm | 160 | 300 | 510 |
| Minimum optical return loss at MPI-SM | dB | 24 |  |  |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 |  |  |
| Maximum differential group delay | ps | 120 |  |  |
| Interface at point MPI-RM |  |  |  |  |
| Maximum mean channel input power | dBm | 0 | 0.5 | +0.5 |
| Minimum mean channel input power | dBm | 12.5 | 12.5 | 12.8 |
| Maximum mean total input power | dBm | +9.5 |  |  |
| Maximum optical path penalty | dB | 1 | 1 | 1 |
| Minimum equivalent sensitivity | dBm | 13.5 | 13.5 | 13.8 |
| Maximum reflectance of optical network element | dB | 27 |  |  |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |

Table IV-4/G.695 – Physical layer parameters and values for multichannel interfaces for 16 channel black box application B-C16L1-1D2

| Parameter | Units | B-C16L1-1D2 |  |  |
| --- | --- | --- | --- | --- |
| Wavelength Block | nm | 1311-1371 | 1391-1451 | 1471-1611 |
| General information |  |  |  |  |
| Maximum number of channels | - | 8+8 |  |  |
| Bit rate/line coding of optical tributary signals | - | NRZ 2.5G |  |  |
| Maximum bit error ratio | - | 1012 |  |  |
| Fibre type | - | G.652.C or G.652.D |  |  |
| Interface at point MPI-SM |  |  |  |  |
| Maximum mean channel output power | dBm | +3.5 | +1.5 | 0.5 |
| Minimum mean channel output power | dBm | 4 | 6 | 8 |
| Maximum mean total output power | dBm | +11.6 |  |  |
| Central wavelength | nm | 1311 + 20m, m=0 to 3 | 1391 + 20m, m=0 to 3 | 1471 + 20m, m=0 to 7 |
| Channel spacing | nm | 20 |  |  |
| Maximum central wavelength deviation (Note a) | nm | ±6.5 |  |  |
| Minimum channel extinction ratio | dB | 8.2 |  |  |
| Eye mask | - | STM-16 per G.957 |  |  |
| Optical path (single span)from point MPI-SM to MPI-RM |  |  |  |  |
| Maximum attenuation | dB | 18 | 15.8 | 13.3 |
| Minimum attenuation | dB | 11 | 9 | 7 |
| Maximum chromatic dispersion | ps/nm | 350 | 650 | 1100 |
| Minimum optical return loss at MPI-SM | dB | 24 |  |  |
| Maximum discrete reflectance between MPI-SM and MPI-RM | dB | 27 |  |  |
| Maximum differential group delay | ps | 120 |  |  |
| Interface at point MPI-RM |  |  |  |  |
| Maximum mean channel input power | dBm | 7.5 |  |  |
| Minimum mean channel input power | dBm | 22 | 21.8 | 21.3 |
| Maximum mean total input power | dBm | +1.5 |  |  |
| Maximum optical path penalty | dB | 1 | 1.5 | 2 |
| Minimum equivalent sensitivity | dBm | 23 | 23.3 | 23.3 |
| Maximum reflectance of optical network element | dB | 27 |  |  |
| NOTE a) – A system with ±7 nm maximum central wavelength deviation which is compliant with all other G.695 parameter values of the relevant application code is transversely compatible for any applications covered by that code, except that it does not provide transverse compatibility with a ±6.5 nm system without joint engineering. |  |  |  |  |

____________________