|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.701

TELECOMMUNICATION	(03/93)
STANDARDIZATION  SECTOR
OF  ITU

GENERAL  ASPECTS  OF  DIGITAL
TRANSMISSION  SYSTEMS

VOCABULARY  OF  DIGITAL  TRANSMISSION
AND  MULTIPLEXING,  AND  PULSE
CODE  MODULATION  (PCM)  TERMS

ITU-T  Recommendation  G.701

(Previously “CCITT  Recommendation”)

# FOREWORD

The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the International Telecommunication Union. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, established the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

ITU-T Recommendation G.701 was revised by the ITU-T Study Group XV (1988-1993) and was approved by the WTSC (Helsinki, March 1-12, 1993).

___________________

NOTES

1	As a consequence of a reform process within the International Telecommunication Union (ITU), the CCITT ceased to exist as of 28 February 1993. In its place, the ITU Telecommunication Standardization Sector (ITU-T) was created as of 1 March 1993. Similarly, in this reform process, the CCIR and the IFRB have been replaced by the Radiocommunication Sector.

In order not to delay publication of this Recommendation, no change has been made in the text to references containing the acronyms “CCITT, CCIR or IFRB” or their associated entities such as Plenary Assembly, Secretariat, etc. Future editions of this Recommendation will contain the proper terminology related to the new ITU structure.

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1994

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation G.701     (03/93)

Page

1	Introduction		1

2	Vocabulary of digital transmission and multiplexing and pulse code modulation terms (PCM)		1

2.1	General		1

2.2	Digital signals		3

2.3	Digital transmission		7

2.4	Digital multiplexing		11

2.5	Frame alignment		17

2.6	Timing		18

2.7	Synchronization		21

2.8	Pulse code modulation		23

2.9	Codes		30

Annex A  –  Alphabetical list of terms defined in this Recommendation		32

Reference		35

Appendix I  –  List of abbreviations used in G-, H- and J-Series Recommendations		35

Recommendation G.701

Recommendation G.701     (03/93)

VOCABULARY  OF  DIGITAL  TRANSMISSION  AND  MULTIPLEXING,
AND  PULSE  CODE  MODULATION  (PCM)  TERMSXE "VOCABULARY  OF  DIGITAL  TRANSMISSION  AND  MULTIPLEXING,
AND  PULSE  CODE  MODULATION  (PCM)  TERMS"§

(Melbourne, 1988; revised at Helsinki, 1993)

# 1	Introduction

This Recommendation provides a vocabulary of terms and definitions that are appropriate to digital and pulse code modulation multiplexing and transmission systems.

A small number of the terms in the Recommendation are duplicated in Recommendation I.112. References to these definitions are given in parenthesis as an aid to ensuring consistency between the two Recommendations in the event of future amendments.

According to the conventions applied in this Recommendation any term in common usage, but whose use is deprecated in the sense defined, is shown after the recommended term as in the following example: “2026 controlled slip [slip]”.

Where a truncated term is widely used in an understood context the complete term is quoted following the colloquial form, for example: “1007 circuit, telecommunication circuit”.

Furthermore, any term which is in general use in addition to the principal term is shown after the principal term as in the following example: “6002 timing recovery (timing extraction)”.

In the interest of standardization in the drafting of documents the following abbreviations are recommended:

kbit/s;

Mbit/s;

Gbit/s.

To avoid misinterpretation of the use of the point (.) and the comma (,) in different languages to separate the whole and decimal parts, it is recommended that the use of decimals should be avoided wherever possible. For example, “2048 kbit/s” is preferred to “2.048 Mbit/s” or “2,048 Mbit/s”.

Annex A to this Recommendation contains an alphabetical list of all of the terms defined in this Recommendation.

Appendix I to this Recommendation contains a list of abbreviations used in G-, H- and J-Series Recommendations.

# 2	Vocabulary of digital transmission and multiplexing and pulse code modulation terms (PCM)

## 2.1	General

For the purposes of this Recommendation, the following definitions apply.

#### 1001	signal [102]

F:  signal
	S:  señal

A physical phenomenon one or more of whose characteristics may vary to represent information.

#### 1002	analogue signal [103]

F:  signal analogique
	S:  señal analógica

A signal one of whose characteristic quantities follows continuously the variations of another physical quantity representing information.

#### 1003	discretely-timed signal [104]

F:  signal (temporel) discret
	S:  señal discretamente temporizada

A signal composed of successive elements in time, each element having one or more characteristics which can convey information, for example, its duration, its waveform and its amplitude.

#### 1004	transmission [106]

F:  transmission
	S:  transmisión

The action of conveying signals from one point to one or more other points.

NOTES

1	Transmission can be effected directly or indirectly, with or without intermediate storage.

2	The use of the English word “transmission” in the sense of “emission” is deprecated.

#### 1005	channel, transmission channel [108]

F:  voie, voie de transmission
	S:  canal, canal de transmisión

A means of unidirectional transmission of signals between two points.

NOTES

1	Several channels may share a common path; for example each channel may be allocated a particular frequency band or a particular time slot.

2	The term may be qualified by the nature of the transmitted signals, by the bandwidth, by the digit rate, or by an arbitrary designation.

3	See also Recommendation I.112, Term 414, access channel.

#### 1006	telecommunication [110]

F:  télécommunication
	S:  telecomunicación

Any transmission and/or emission and reception of signals representing signs, writing, images and sounds or intelligence of any nature by wire, radio, optical or other electromagnetic systems.

#### 1007	circuit, telecommunication circuit [111]

F:  circuit, circuit de télécommunications
	S:  circuito, circuito de telecomunicación

A combination of two transmission channels permitting bidirectional transmission of signals between two points, to support a single communication.

NOTES

1	If the telecommunication is by nature unidirectional (for example: long distance television transmission), the term “circuit” is sometimes used to designate the single channel providing the facility.

2	In a telecommunication network, the use of the term “circuit” is generally limited to a telecommunication circuit directly connecting two switching devices or exchanges, together with associated terminating equipment.

3	A telecommunication circuit may permit transmission in both directions simultaneously (duplex), or not simultaneously (simplex).

4	A telecommunication circuit that is used for transmission in one direction only is sometimes referred to as a unidirectional telecommunication circuit. A telecommunication circuit that is used for transmission in both directions (whether simultaneously or not) is sometimes referred to as a bidirectional telecommunication circuit.

#### 1008	interface [408]

F:  jonction (interface)
	S:  interfaz

The common boundary between two associated systems.

#### 1009	congestion

F:  encombrement/engorgement
	S:  congestión

Congestion is the conditions that exists in a network if the capacity needed for the instantaneous traffic exceeds the bandwidth available in the network.

## 2.2	Digital signals

#### 2001	digit

F:  élément numérique
	S:  dígito

A member selected from a finite set.

NOTES

1	In digital transmission, a digit may be represented by a signal element, being characterized by the dynamic nature, discrete condition and discrete timing of the element, for example it may be represented as a pulse of specified amplitude and duration.

2	In equipment used in digital transmission, a digit may be represented by a stored condition being characterized by a specified physical condition, for example it may be represented as a binary magnetic condition of a ferrite core.

3	The context of the use of the term should be such as to indicate the radix of notation. (The meaning of “digit” in Notes 1, 2 and 3 translates into French as “élément numérique”.)

4	In telephone subscriber numbering, a digit is any of the numbers 1, 2, 3 . . . 9 or 0 forming the elements of a telephone number (Recommendation Q.10 [1]). (This meaning of “digit” translates into French as “chiffre”.)

#### 2002	binary figure

F:  chiffre binaire
	S:  cifra binaria

One of the two figures (that is, 0 or 1) used in the representation of numbers in binary notation.

#### 2003	binary digit (bit)

F:  élément binaire (bit)
	S:  dígito binario (bit)

A member selected from a binary set.

NOTES

1	Bit is an abbreviation for binary digit.

2	In the interest of clarity, it is recommended that the term “bit” should not be used in two-condition start-stop modulation instead of “unit element”.

#### 2004	octet

F:  octet
	S:  octeto

A group of eight binary digits or eight signal elements representing binary digits operated upon as an entity.

#### 2005	code word [character signal]

F:  mot de code [signal de caractère]
	S:  palabra de código [señal de carácter]

A set of signal elements representing the quantized value of a sample in PCM.

NOTE – In PCM, the term “PCM word” may be used in this sense.

#### 2006	digital signal [105]

F:  signal numérique
	S:  señal digital

A discretely timed signal in which information is represented by a number of well-defined discrete values that one of its characteristic quantities may take in time.

NOTE – The term may be qualified to indicate the digit rate, for example: “140 Mbit/s digital signal”.

#### 2007	signal element

F:  élément de signal
	S:  elemento de señal

A part of a digital signal, characterized by its discrete timing and its discrete value, and used to represent a digit.

#### 2008	digit position

F:  position d'un élément de signal, position d'un élément numérique

S:  posición de dígito

The position in time or space into which a representation of a digit may be placed.

#### 2009	n-ary digital signal

F:  signal numérique n-aire
	S:  señal digital n-aria

A digital signal in which each signal element has one of n permitted discrete values.

#### 2010	redundant digital signal

F:  signal numérique redondant
	S:  señal digital redundante

The signal that is produced by encoding a given signal in accordance with a redundant line code.

#### 2011	redundant n-ary signal

F:  signal n-aire redondant
	S:  señal n-aria redundante

A digital signal whose elements can assume n discrete states where the average equivalent binary content per signal element is less than log2 n.

NOTE – The relative redundancy R, of an n-ary digital signal, is given by:

where rd is the symbol rate of the n-ary signal and re is the equivalent bit rate.

This may also be expressed in terms of the number of binary digits which can be transmitted by an element of a particular line code. Examples are:

AMI (37% redundant), 1 binary digit per element;

4B3T (16% redundant), 1.33 binary digit per element.

#### 2012	pseudo n-ary signal

F:  signal pseudo n-aire
	S:  señal seudo n-aria

A redundant n-ary digital signal that is derived from a m-ary digital signal without change of the line digit rate.

NOTE – An alternate mark inversion signal is an example of a pseudo-ternary signal, i.e. n = 3, m = 2.

#### 2013	digit rate

F:  débit numérique
	S:  velocidad digital

The number of digits per unit time.

NOTES

1	An appropriate adjective may precede the word “digit”, for example, binary digit rate.

2	In the interests of clarity it is recommended that this term should not be used to express the symbol rate on the line.

#### 2014	line digit rate [symbol rate]

F:  débit numérique en ligne [débit de symboles]
	S:  velocidad digital de linea [velocidad de símbolos]

The number of signal elements of the line signal transmitted per unit time.

NOTES

1	The baud is usually used to quantify this, one baud being equal to one single element per second.

2	Modulation rate is the term used in telegraphy and data communication; it is the reciprocal of the duration of the unit interval.

#### 2015	equivalent binary content

F:  contenu binaire équivalent
	S:  contenido binario equivalente

The number of binary digits strictly necessary to convey the same information as a defined number of signal elements in a given digital signal.

#### 2016	equivalent bit rate

F:  débit binaire équivalent
	S:  velocidad binaria equivalente

The value of the bit rate strictly necessary to convey the same information in the same time as a given digital signal at a given digit rate.

#### 2017	significant instant, significant instant of a digital signal

F:  instant significatif, instant significatif d'un signal numérique
	S:  instante significativo, instante significativo de una señal digital

The instant at which a signal element commences in a discretely-timed signal.

#### 2018	unit interval

F:  intervalle unitaire
	S:  intervalo unitario (o intervalo unidad)

The nominal difference in time between consecutive significant instants of an isochronous signal.

#### 2019	decision instant, decision instant of a digital signal

F:  instant de décision, instant de décision d'un signal numérique
	S:  instante de decisión, instante de decisión de una señal digital

The instant at which a decision is taken as to the probable value of signal element of a received digital signal.

#### 2020	decision circuit

F:  circuit de décision
	S:  circuito de decisión

A circuit that decides the probable value of a signal element of a received digital signal.

#### 2021	regeneration

F:  régénération
	S:  regeneración

The process of receiving and reconstructing a digital signal so that the amplitudes, waveforms and timing of its signal elements are constrained within specified limits.

#### 2022	regenerator

F:  régénérateur
	S:  regenerador

A device that performs regeneration.

#### 2023	regenerative repeater

F:  répéteur régénérateur
	S:  repetidor regenerativo

A repeater that regenerates digital signals.

NOTES

1	A regenerative repeater may operate in one or both directions of transmission, and the term may be qualified by “unidirectional” or “bidirectional” as appropriate.

2	Repeater is defined in Recommendation G.601.

#### 2024	jitter

F:  gigue
	S:  fluctuación de fase

Short-term non-cumulative variations of the significant instants of a digital signal from their ideal positions in time.

#### 2025	wander

F:  dérapage
	S:  fluctuación lenta de fase

Long-term non-cumulative variations of the significant instants of a digital signal from their ideal positions in time.

#### 2026	controlled slip [slip]

F:  glissement commandé [saut]
	S:  deslizamiento controlado [deslizamiento]

The irretrievable loss or gain of a set of consecutive digit positions in a digital signal, in which both the magnitude and instant of that loss or gain are controlled, to enable the signal to accord with a rate different from its own.

NOTE – Where appropriate the term may be qualified, for example: controlled octet slip, controlled frame slip.

#### 2027	uncontrolled slip

F:  glissement non commandé
	S:  deslizamiento incontrolado

The loss or gain of a digit position or a set of consecutive digit positions in a digital signal resulting from an aberration of the timing processes associated with transmission or switching of a digital signal, and in which either the magnitude or the instant of that loss or gain is not controlled.

#### 2028	scrambler

F:  embrouilleur
	S:  aleatorizador

A device that converts a digital signal into a pseudo-random digital signal having the same meaning and the same digit rate.

#### 2029	descrambler

F:  désembrouilleur
	S:  desaleatorizador

A device that performs the complementary operation to that of a scrambler.

#### 2030	error, digital error

F:  erreur, erreur numérique
	S:  error, error digital

An inconsistency between a digit in a transmitted digital signal and the corresponding digit in the received digital signal.

#### 2031	error ratio [error rate]

F:  taux d'erreur [rapport d'erreur]
	S:  tasa de errores [proporción de errores]

The ratio of the number of digital errors received in a specified period to the total number of digits received in the same period.

NOTES

1	Numerical values of error ratio should be expressed in the form

n  ×  10–p

where p is a positive integer.

2	Error ratio may be qualified, for example by the term “bit” or “block”.

#### 2032	error multiplication

F:  multiplication d'erreurs
	S:  multiplicación de errores

The property of an apparatus whereby a single digital error in the input signal presented to it results in more than one digital error in the output signal.

NOTE – Line code converters and descramblers are examples of apparatus that may cause error multiplication.

#### 2033	error multiplication factor

F:  facteur de multiplication d'erreurs
	S:  factor de multiplicación de errores

The ratio of the number of digital errors in the output signal to the number of digital errors in the input signal.

NOTE – The error multiplication factor may be expressed as either an average or maximum value.

#### 2034	error spread

F:  étalement d'erreurs [répartition des erreurs]
	S:  dispersión de errores

The number of consecutive digits of the output signal over which digital errors are distributed when a single digital error in the input signal causes error multiplication.

## 2.3	Digital transmission

#### 3001	digital transmission [107]

F:  transmission numérique
	S:  transmisión digital

The transmission of digital signals by means of a channel or channels that may assume in time any one of a defined set of discrete states.

#### 3002	digital channel, digital transmission channel [109]

F:  voie numérique, voie de transmission numérique
	S:  canal digital, canal de transmisión digital

The means of unidirectional digital transmission of digital signals between two points.

#### 3003	digital circuit, digital telecommunication circuit [112]

F:  circuit numérique, circuit numérique de télécommunications
	S:  circuito digital, circuito de telecomunicación digital

A combination of two digital transmission channels permitting bidirectional digital transmission in both directions between two points, to support a single communication.

NOTES

1	If the telecommunication is by nature unidirectional (for example, long-distance television transmission), the term “digital circuit” is sometimes used to designate the single digital channel providing the facility.

2	In a telecommunication network, use of the term “digital circuit” is generally limited to a digital telecommunication circuit directly connecting two switching devices or exchanges, together with associated terminating equipment.

3	A digital telecommunication circuit may permit transmission in both directions simultaneously (duplex), or not simultaneously (simplex).

4	A digital telecommunication circuit that is used for transmission in one direction only is sometimes referred to as a unidirectional digital telecommunication circuit. A digital telecommunication circuit that is used for transmission in both directions (whether simultaneously or not) is sometimes referred to as a bidirectional digital telecommunication circuit.

#### 3004	digital connection [310]

F:  connexion numérique
	S:  conexión digital

A concatenation of digital transmission channels or digital telecommunication circuits, switching and other functional units set up to provide for the transfer of digital signals between two or more points in a telecommunication network, to support a single communication.

#### 3005	digital link, digital transmission link [digital path] [302]

F:  liaison numérique, liaison de transmission numérique [conduit numérique]
	S:  enlace digital, enlace de transmisión digital [trayecto digital]

The whole of the means of digital transmission of a digital signal of specified rate between two digital distribution frames (or equivalent).

NOTES

1	A digital link comprises one or more digital sections and may include multiplexing and/or demultiplexing, but not switching.

2	The term may be qualified to indicate the transmission medium used, for example, “digital satellite link”.

3	The term always applies to the combination of “go” and “return” directions of transmission, unless stated otherwise.

4	The term “digital path” is sometimes used to describe one or more digital links connected in tandem, especially between equipments at which the signals of the specified rate originate and terminate.

#### 3006	digital distribution frame

F:  répartiteur numérique
	S:  repartidor digital

A structure that provides flexibility of semipermanent interconnection of digital channels or digital circuits.

NOTE – Digital sections and digital links normally terminate at digital distribution frames.

#### 3007	digital section1)

F:  section numérique
	S:  sección digital

The whole of the means of digital transmission of a digital signal of specified rate between two consecutive digital distribution frames or equivalent.

NOTES

1	A digital section forms either a part or the whole of a digital link, and includes terminating equipments at both ends, but excludes multiplexers.

2	Where appropriate, the digital rate or multiplex order should qualify the title.

3	The definition applies to the combination of “go” and “return” directions of transmission, unless stated otherwise.

#### 3008	section termination

F:  extrémité de section
	S:  extremo de sección

A connectional interface selected to be the boundary between a physical transmission medium and its associated equipment.

NOTE – This point will usually be the connectors at the input and output of an equipment.

#### 3009	elementary cable section [repeater section]

F:  section élémentaire de câble [section (élémentaire) d'amplification]
	S:  sección elemental de cable [sección con amplificación]

The whole of the physical transmission medium between the section termination at the output of one equipment and the section termination at the input of the following equipment.

NOTES

1	An elementary cable section usually consists of several factory lengths of cable connected together and any associated accessories (such as flexible cables) necessary to connect it to the section terminals.

2	Examples of the physical transmission media are a coaxial or symmetric pair, and optical fibre.

#### 3010	elementary repeater section

F:  section élémentaire amplifiée
	S:  sección elemental de repetición

In a given direction of transmission, an elementary cable section together with the immediately following analogue repeater.

#### 3011	elementary regenerator section [regenerator section]

F:  section élémentaire régénérée [section de régénération]
	S:  sección elemental de regeneración [sección de regeneración]

In a given direction of transmission, an elementary cable section together with the immediately following regenerative repeater.

#### 3012	digital line section1)

F:  section de ligne numérique
	S:  sección de línea digital

A digital section implemented on a single type of manufactured transmission medium, such as symmetric pair, coaxial pair, or optical fibre.

NOTE – A digital line section includes line terminating equipments at both ends, and regenerative repeaters if needed, but excludes multiplexers.

#### 3013	digital line link [digital line path]2)

F:  liaison de ligne numérique [conduit de ligne numérique]
	S:  enlace de línea digital [trayecto de línea digital]

A digital link that comprises a digital line section or a number of tandem-connected digital line sections.

#### 3014	digital transmission system

F:  système de transmission numérique
	S:  sistema de transmisión digital

A specific means of providing a digital section.

#### 3015	digital line system

F:  système de ligne numérique
	S:  sistema de línea digital

A digital transmission system that provides a digital line section.

#### 3016	digital radio section2)

F:  section radioélectrique numérique
	S:  sección radiodigital

A digital section implemented on a radio-relay system.

#### 3017	digital radio link [digital radio path]2)

F:  liaison radioélectrique numérique [conduit radioélectrique numérique]
	S:  enlace radiodigital [trayecto radiodigital]

A digital link that comprises a digital radio section or a number of tandem-connected digital radio sections.

#### 3018	digital radio system

F:  système radioélectrique numérique
	S:  sistema radiodigital

A digital transmission system that provides a digital radio section.

#### 3019	bit sequence independence

F:  indépendance de la séquence des bits
	S:  independencia de la secuencia de bits

The property of a binary transmission channel, telecommunication circuit or connection, that permits all sequences of binary signal elements to be conveyed over it at its specified bit rate, without change to the value of any signal elements.

NOTE – Practical transmission systems that are not completely bit sequence independent may be described as quasi bit sequence independent. In such cases the limitations should be clearly stated.

#### 3020	digit sequence integrity

F:  intégrité de la suite des éléments numériques
	S:  integridad de la secuencia de dígitos

The property of a digital transmission channel, telecommunication circuit or connection, that permits a digital signal to be conveyed over it without change to the order of any signal elements.

#### 3021	octet sequence integrity

F:  intégrité de la suite des octets
	S:  integridad de la secuencia de octetos

The property of a digital transmission channel, telecommunication circuit or connection that permits a digital signal to be conveyed over it without change to the order of any octets.

#### 3022	transparency, digital transparency

F:  transparence, transparence numérique
	S:  transparencia, transparencia digital

The property of a digital transmission channel, telecommunication circuit or connection, that permits any digital signal to be conveyed over it without change to the value or order of any signal elements.

NOTE – The digital transmission channel, telecommunication circuit or connection concerned may introduce delay, and may contain reversible code conversion functions.

#### 3023	alarm indication signal (AIS)

F:  signal d'indication d'alarme (AIS)
	S:  señal de indicación de alarma (AIS)

A signal that replaces the normal traffic signal when a maintenance alarm indication has been activated.

#### 3024	upstream failure indication

F:  indication de défaillance en amont
	S:  indicación de fallo atrás

An indication provided by a digital multiplexer, line section or a radio section, that a signal applied at its input port is outside its prescribed maintenance limit.

#### 3025	service digits [housekeeping digits]

F:  éléments numériques de service
	S:  dígitos de servicio

Digits that are added to a digital signal at the sending end of a digital link, normally at regular intervals and removed at the receiving end of that link and used to provide ancillary facilities.

## 2.4	Digital multiplexing

#### 4001	highway (American: bus)

F:  bus (jonction multiplex interne) [canal]
	S:  arteria (bus)

A common path within an apparatus or station over which pass signals from a number of channels identified by time division.

#### 4002	channel gate

F:  porte de canal
	S:  puerta de canal

A device for connecting a channel to a highway, or a highway to a channel, at specified times.

#### 4003	digital multiplex hierarchy

F:  hiérarchie de multiplexage numérique
	S:  jerarquía de los múlitplex digitales

A series of digital multiplexers graded according to capability so that multiplexing at one level combines a defined number of digital signals, each having the digit rate prescribed for a lower order, into a digital signal having a prescribed digit rate which is then available for further combination with other digital signals of the same rate in a digital multiplexer of the next higher order.

FIGURE 1/G.701...[D01] =  21  CM    PAGE PLEINE  A L'ITALIENNE

#### 4004	primary digital group

F:  groupe numérique primaire
	S:  grupo digital primario

An assembly, by digital multiplexing, of digital signals occupying a specified number of channel time-slots to form a composite signal having a digit rate of 2048 kbit/s or 1544 kbit/s.

NOTE – Normally each channel time-slot has eight digit time-slots and an effective digit rate of 64 kbit/s.

#### 4005	primary PCM group [primary block] (American: digroup)

F:  groupe primaire MIC [bloc primaire]
	S:  grupo primario MIC [bloque primario]

An assembly, by digital multiplexing, of PCM signals occupying a specified number of channel time-slots to form a composite signal having a digit rate of 1544 kbit/s or 2048 kbit/s, in both directions of transmission.

NOTE – The following conventions could be useful:

Primary group m – a basic group of PCM signals derived from 1544 kbit/s PCM multiplex equipment.

Primary group A – a basic group of PCM signals derived from 2048 kbit/s PCM multiplex equipment.

#### 4006	n-ary digital group

F:  groupe numérique n-aire
	S:  grupo digital n-ario

A number of primary digital or PCM groups assembled by digital multiplexing to form a composite signal of specified digit rate, in both directions of transmission.

NOTES

1	A secondary digital group may comprise four primary digital or PCM groups to form a composite signal having a digit rate of 8448 or 6312 kbit/s.

2	A tertiary digital group may comprise four 8448 kbit/s secondary digital groups or five or seven 6312 kbit/s secondary digital groups to form a composite signal having a digit rate of 34 368, 32 064 or 44 736 kbit/s.

3	A quaternary digital group may comprise four 34 368 kbit/s tertiary digital groups to form a composite signal having a digit rate of 139 264 kbit/s.

#### 4007	frame

F:  trame
	S:  trama

A cyclic set of consecutive time slots in which the relative position of each time slot can be identified.

#### 4008	multiframe

F:  multitrame
	S:  multitrama

A cyclic set of consecutive frames in which the relative position of each frame can be identified.

#### 4009	subframe

F:  sous-trame, secteur de trame
	S:  subtrama

A sequence of noncontiguous time-slots within a frame, each occurring at n times the frame repetition rate where n is an integer >1.

#### 4010	parallel to serial converter (American: serializer) [dynamicizer]

F:  convertisseur parallèle/série
	S:  convertidor paralelo/serie

A device that converts a group of signal elements, all of which are presented simultaneously, into a corresponding sequence of consecutive signal elements.

#### 4011	serial to parallel converter (American: deserializer) [staticizer]

F:  convertisseur série/parallèle
	S:  convertidor serie/paralelo

A device that converts a sequence of consecutive signal elements into a corresponding group of signal elements all of which are presented simultaneously.

#### 4012	time-division multiplexing

F:  multiplexage temporel [multiplexage par répartition dans le temps]
	S:  multiplexación por división en el tiempo

Multiplexing in which several signals are interleaved in time for transmission over a common channel.

#### 4013	digital multiplexing

F:  multiplexage numérique
	S:  multiplexación digital

A form of time division multiplexing applied to digital channels which convey digital signals.

#### 4014	digital multiplexer

F:  multiplexeur numérique
	S:  multiplexor digital

Equipment that combines by time-division multiplexing several digital signals into a single composite digital signal.

#### 4015	digital demultiplexing

F:  démultiplexage numérique
	S:  demultiplexación digital

The separation of a composite digital signal into its component digital signals.

#### 4016	digital demultiplexer

F:  démultiplexeur numérique
	S:  demultiplexor digital

Equipment that separates a composite digital signal into its component digital signals.

#### 4017	digital multiplex equipment

F:  équipement de multiplexage numérique
	S:  equipo múltiplex digital

The combination of a digital multiplexer and a digital demultiplexer at the same location, operating in opposite directions of transmission.

#### 4018	PCM multiplex equipment

F:  équipement de multiplexage MIC
	S:  equipo múltiplex MIC

Equipment that derives a single digital signal at a defined digit rate from several voice frequency channels by a combination of pulse code modulation and time division multiplexing, and that also carries out the complementary functions in the opposite direction of transmission.

#### 4019	digital block

F:  bloc numérique
	S:  bloque digital

The combination of a digital link and associated digital multiplex equipments.

NOTE – The bit rate of the digital link should form part of the title.

#### 4020	transmultiplexer

F:  transmultiplexeur
	S:  transmultiplexor

An equipment that transforms a frequency-division multiplexed signal (such as group or supergroup) into a corresponding time-division multiplexed signal that has the same structure as if it had been derived from PCM multiplex equipment, and that also carries out the complementary function in the opposite direction of transmission.

#### 4021	digital filling [digital padding]

F:  remplissage numérique
	S:  relleno digital [complementación digital]

The addition of signal elements at regular intervals to a digital signal to change the digit rate from its original value to a predetermined higher value.

NOTE – The added digits are not normally used to transmit information.

#### 4022	justification [stuffing, pulse stuffing]

F:  justification
	S:  justificación [relleno de impulsos]

The process of changing the digit rate of a digital signal in a controlled manner so that it can accord with a digit rate different from its own inherent rate, usually without loss of information.

#### 4023	positive justification [positive stuffing, positive pulse stuffing]

F:  justification positive
	S:  justificación positiva [relleno positivo de impulsos]

A method of justification in which the digit time-slots used to convey a digital signal have a digit rate that is always higher than the digit rate of that original signal.

NOTES

1	Positive justification is usually achieved by the provision of a fixed number of digit time-slots (justifiable digit time-slots) per frame in the resultant signal which may be used to transmit either information from the original signal, or no information, according to the relative digit rates of the resultant signal and the original signal.

2	Information which indicates whether the justifiable digit time-slots contain information digits or justifying digits is conveyed by means of the justification service digits.

#### 4024	negative justification [negative stuffing, negative pulse stuffing]

F:  justification négative
	S:  justificación negativa [relleno negativo, relleno negativo de impulsos]

A method of justification in which the digit time-slots used to convey a digital signal have a digit rate that is always lower than the digit rate of that original signal.

NOTES

1	The deleted digits are conveyed by separate means.

2	Information which facilitates the recovery of the deleted digits is conveyed by means of the justification service digits.

#### 4025	positive/zero/negative justification [positive/zero/negative stuffing, positive/zero/negative pulse stuffing]

F:  justification positive/nulle/négative
	S:  justificación positiva/nula/negativa [relleno positivo/nulo/negativo de impulsos]

A method of justification in which the digit time-slots used to convey a digital signal have a digit rate that may be higher than, the same as, or lower than the digit rate of the original signal.

NOTES

1	Justifiable digit time-slots are provided in accordance with Note 1 of 4023 above.

2	Separate means of transmitting deleted digits are provided in accordance with Note 2 of 4024 above.

3	Information which facilitates the recovery of the original digits, which are conveyed by means of the justification service digits.

4	Usually the digit time-slots used to convey a digital signal have the same nominal digit rate as the original signal.

#### 4026	justifiable digit time-slot [stuffable digit time-slot]

F:  créneau temporel élémentaire justifiable
	S:  intervalo de tiempo de dígito justificable [intervalo de tiempo de dígito rellenable]

A digit time-slot that is provided for the purpose of justification and which may contain either an information digit or a justifying digit.

#### 4027	justifying digit [stuffing digit]

F:  élément numérique de justification
	S:  dígito de justificación [dígito de relleno]

A digit inserted in a justifiable digit time-slot when that time slot is not required for an information digit.

#### 4028	justification service digit [stuffing service digit]

F:  élément numérique de service de justification
	S:  dígito de servicio de justificación [dígito de servicio de relleno]

A digit that transmits information concerning the status of a justifiable digit time-slot.

#### 4029	justification rate [stuffing rate]

F:  débit de justification
	S:  velocidad de justificación [velocidad de relleno]

The rate at which justifying digits are inserted, or at which information digits are transmitted by other means.

#### 4030	nominal justification rate [nominal stuffing rate]

F:  débit nominal de justification
	S:  velocidad nominal de justificación [velocidad nominal de relleno]

The justification rate that occurs when the digit rates of both the original signal and the justified signal are at their nominal values.

#### 4031	maximum justification rate [maximum stuffing rate]

F:  débit maximal de justification
	S:  velocidad máxima de justificación [velocidad máxima de relleno]

The maximum possible justification rate that can be accommodated by a justification process.

NOTE – In practice the tolerance limits of the original signal and of the system used to convey the justified signal might be such that the maximum justification rate is never realized.

#### 4032	justification ratio [stuffing ratio]

F:  taux de justification
	S:  relación de justificación [relación de relleno]

The ratio of the actual justification rate to the maximum justification rate.

#### 4033	nominal justification ratio [nominal stuffing ratio]

F:  taux nominal de justification
	S:  relación nominal de justificación [relación nominal de relleno]

The ratio of the nominal justification rate to the maximum justification rate.

#### 4034	packetized circuit multiplication equipment (PCME)

F:  équipement de multiplication de circuits par mise en paquets
	S:  equipo de multiplicación de circuito por paquetización

A packetized circuit multiplication equipment is a general class of equipment that compresses and integrates voice, voice-band data, digital data, signalling, image, facsimile and network control into packets of common formats.

#### 4035	packetized circuit multiplication system

F:  système de multiplication de circuits par mise en paquets
	S:  sistema de multiplicación de circuito por paquetización

A packetized circuit multiplication system is a telecommunications network comprising of two or more PCME nodes.

## 2.5	Frame alignment

#### 5001	frame alignment3)

F:  verrouillage de trame
	S:  alineación de trama

The state in which the frame of the receiving equipment is synchronized with that of the received signal.

#### 5002	frame alignment signal3)

F:  signal de verrouillage de trame
	S:  señal de alineación de trama

The distinctive signal inserted in every frame or once in every n frames, always occupying the same relative position within the frame, and used to establish and maintain frame alignment.

#### 5003	bunched frame alignment signal3)

F:  signal de verrouillage de trame concentré
	S:  señal de alineación de trama concentrada

A frame alignment signal whose signal elements occupy consecutive digit time slots.

#### 5004	distributed frame alignment signal3)

F:  signal de verrouillage de trame réparti [signal de verrouillage de trame distribué]
	S:  señal de alineación de trama distribuida

A frame alignment signal whose signal elements occupy non-consecutive digit time slots.

#### 5005	frame alignment recovery time3)

F:  temps de reprise du verrouillage de trame
	S:  tiempo de recuperación de la alineación de trama

The time that elapses between a valid frame alignment signal being available at the receive terminal equipment and frame alignment being established.

NOTE – The frame alignment recovery time includes the time required for replicated verification of the validity of the frame alignment signal.

#### 5006	out-of-frame alignment time3)

F:  durée de perte du verrouillage de trame
	S:  duración de la pérdida de la alineación de trama

The time during which frame alignment is effectively lost.

NOTE – That time includes the time to detect loss of frame alignment and the frame alignment recovery time.

## 2.6	Timing

#### 6001	timing signal

F:  signal de rythme
	S:  señal de temporización

A cyclic signal used to control the timing of operations.

#### 6002	timing recovery [timing extraction]

F:  récupération du rythme
	S:  recuperación de la temporización [extracción de la temporización]

The derivation of a timing signal from a received signal.

#### 6003	retiming

F:  réajustement du rythme
	S:  reajuste de la temporización

Adjustment of the intervals between the significant instants of a digital signal, by reference to a timing signal.

#### 6004	time-slot

F:  créneau temporel [intervalle de temps]
	S:  intervalo de tiempo [sector de tiempo, celda de tiempo]

Any cyclic time interval that can be recognized and defined uniquely.

#### 6005	digit time-slot

F:  créneau temporel élémentaire [intervalle de temps élémentaire]
	S:  intervalo de tiempo de dígito

A time slot allocated to a single digit.

#### 6006	channel time-slot

F:  créneau temporel de voie
	S:  intervalo de tiempo de canal

A time slot occupying a specific position in a frame and allocated to a particular time-derived channel.

NOTES

1	Where appropriate a description may be added, for example “telephone channel time-slot”.

2	In addition to its main function of transmitting a character signal, a channel time-slot may also be used for in-slot signalling or for transmitting other information.

#### 6007	signalling time-slot

F:  créneau temporel de signalisation
	S:  intervalo de tiempo de señalización

A time slot occupying a specific position in a frame and allocated to the transmission of signalling.

#### 6008	frame alignment time-slot

F:  créneau temporel de verrouillage de trame
	S:  intervalo de tiempo de alineación de trama

A time slot occupying the same relative position in every frame and used to transmit the frame alignment signal.

#### 6009	clock

F:  horloge
	S:  reloj

Equipment that provides a timing signal.

NOTE – Where replicated sources are used for security reasons, the assembly of these is regarded as single clock.

#### 6010	reference clock

F:  horloge de référence
	S:  reloj de referencia

A clock of very high stability and accuracy that may be completely autonomous and whose frequency serves as a basis of comparison for the frequency of other clocks.

#### 6011	master clock

F:  horloge maîtresse
	S:  reloj maestro

A clock that is used to control the frequency of other clocks.

#### 6012	time interval error

F:  dérive temporelle
	S:  error de intervalo de tiempo

The total difference over a specified interval of time in the significant instants of a digital signal from their ideal positions in time.

#### 6013	relative time interval error

F:  dérive temporelle relative
	S:  error de intervalo de tiempo relativo

The total difference over a specified interval of time in the corresponding significant instants of two digital signals.

#### 6014	isochronous

F:  isochrone
	S:  isócrono

The essential characteristic of a time-scale or a signal such that the time intervals between consecutive significant instants either have the same duration or durations that are integral multiples of the shortest duration.

NOTE – In practice, variations in the time intervals are constrained within specified limits.

#### 6015	anisochronous

F:  anisochrone
	S:  anisócrono

The essential characteristic of a time-scale or a signal such that the time intervals between consecutive significant instants do not necessarily have the same duration or durations that are integral multiples of the shortest duration.

#### 6016	synchronous [mesochronous]

F:  synchrone [mésochrone]
	S:  síncrono [mesócrono]

The essential characteristic of time-scales or signals such that their corresponding significant instants occur at precisely the same average rate.

NOTE – The timing relationship between corresponding significant instants usually varies between specified limits.

#### 6017	homochronous

F:  homochrone
	S:  homócrono

The essential characteristic of time-scales or signals such that their corresponding significant instants have a constant, but uncontrolled, time relationship with each other.

#### 6018	non-synchronous [asynchronous/heterochronous]

F:  non synchrone [asynchrone/hétérochrone]
	S:  no-síncrono [asíncrono/heterócrono]

The essential characteristic of time-scales or signals such that their corresponding significant instants do not necessarily occur at the same average rate.

#### 6019	plesiochronous

F:  plésiochrone
	S:  plesiócrono

The essential characteristic of time-scales or signals such that their corresponding significant instants occur at nominally the same rate, any variation in rate being constrained within specified limits.

NOTES

1	Two signals having the same nominal digit rate, but not stemming from the same clock or homochronous clocks, are usually plesiochronous.

2	There is no limit to the time relationship between corresponding significant instants.

#### 6020	heterochronous

F:  hétérochrone
	S:  heterócrono

The essential characteristic of time-scales or signals such that their corresponding significant instants occur at different nominal rates.

NOTES

1	Two signals having different nominal digit rates, and not stemming from the same clock or from homochronous clocks are usually heterochronous.

2	Terms 6014 to 6020 are based on the following Greek roots:

iso	=  equal

homo	=  same

plesio	=  near

hetero	=  different.

#### 6021	codirectional interface

F:  interface codirectionnelle
	S:  interfaz codireccional

An interface across which the signals to be transferred and their associated timing signals are transmitted in the same direction.

#### 6022	centralized-clock interface

F:  interface à horloge centralisée
	S:  interfaz de reloj centralizado

An interface across which, for both directions of transmission of the signals to be transferred, the associated timing signals of both the exchange terminal on the line side and the exchange terminal on the service side are supplied from a centralized clock.

NOTE – The timing of the centralized clock may be derived from a nominated incoming line signal.

#### 6023	contradirectional interface

F:  interface contradirectionnelle
	S:  interfaz contradireccional

An interface across which the timing signals associated with both directions of transmission of the signals to be transferred, are directed towards the same side of the interface.

## 2.7	Synchronization

#### 7001	synchronization

F:  synchronisation
	S:  sincronización

The process of adjusting the corresponding significant instants of signals to make them synchronous.

#### 7002	timing information

F:  information de rythme
	S:  información de temporización

Information contained in a signal relating to the timing of another signal.

#### 7003	synchronization information

F:  information de synchronisation
	S:  información de sincronización

Information that indicates the relationship between the timing of two or more signals.

#### 7004	clock control signal

F:  signal de commande d'horloge
	S:  señal de control de reloj

A signal that directly controls the phase or frequency of a clock.

#### 7005	synchronization node

F:  noeud de synchronisation
	S:  nodo de sincronización

A point in a synchronized network at which synchronization information is derived, sent or received.

#### 7006	synchronization link

F:  liaison de synchronisation
	S:  enlace de sincronización

A link between two synchronization nodes over which synchronization information is transmitted.

#### 7007	synchronization network

F:  réseau de synchronisation
	S:  red de sincronización

An arrangement of synchronization nodes and synchronization links provided in order to synchronize the clocks at, or connected to, those nodes.

#### 7008	single-ended synchronization

F:  synchronisation locale [synchronisation unilatérale]
	S:  sincronización uniterminal

A method of synchronizing a specified synchronization node with respect to another synchronization node in which synchronization information at the specified node is derived from the phase difference between the local clock and the incoming digital signal from the other node.

#### 7009	double-ended synchronization

F:  synchronisation locale et distante [synchronisation bilatérale]
	S:  sincronización biterminal

A method of synchronizing a specified synchronization node with respect to another synchronization node in which synchronization information at the specified node is derived by comparing the phase difference between the local clock and the incoming digital signal from the other node, with the phase difference at the other node between its local clock and the digital signal incoming from the specified node.

#### 7010	unilateral control

F:  synchronisation unilatérale [commande unilatérale]
	S:  control unilateral

Control between two synchronization nodes such that the frequency of the clock of only one of these nodes is influenced by timing information derived from the clock of the other node.

#### 7011	bilateral control

F:  synchronisation bilatérale [commande bilatérale]
	S:  control bilateral

Control between two synchronization nodes such that the frequency of the clock of each of these nodes is influenced by timing information derived from the clock of the other node.

#### 7012	analogue control

F:  synchronisation analogique [mode analogique]
	S:  control analógico

A method of controlling clocks in which the clock control signal is a continuous (monotonic) function of the phase difference between clocks, at least over a limited range.

#### 7013	linear analogue control

F:  synchronisation analogique linéaire [mode analogique linéaire]
	S:  control analógico lineal

Analogue control in which the clock control signal is proportional to the phase difference between clocks, at least over a limited range.

#### 7014	amplitude quantized control

F:  synchronisation quantifiée [mode à quantification d'amplitude]
	S:  control por cuantificación de amplitud

A method of controlling clocks in which the clock control signal is a quantized function of the phase difference between clocks.

NOTE – In practice this implies that the working range of phase errors is divided into a finite number of subranges and that a unique signal is derived for each subrange whenever the error falls within a subrange.

#### 7015	time quantized control

F:  synchronisation échantillonnée [mode à quantification temporelle]
	S:  control por cuantificación temporal

A method of controlling clocks in which each clock control signal is derived or utilized only at a number of discrete instants, which may or may not be equally separated in time.

#### 7016	synchronized network [synchronous network]

F:  réseau synchronisé [réseau synchrone]
	S:  red sincronizada [red síncrona]

A network in which the corresponding significant instants of nominated signals are adjusted to make them synchronous.

#### 7017	non-synchronized network

F:  réseau non synchronisé
	S:  red no sincronizada

A network in which signals need not be synchronous.

#### 7018	mutually synchronized network

F:  réseau à synchronisation mutuelle
	S:  red mutuamente sincronizada

A synchronized network in which each clock exerts a degree of control on all others.

#### 7019	democratic network, democratic mutually synchronized network

F:  réseau démocratique, réseau à synchronisation mutuelle démocratique
	S:  red democrática, red democrática mutuamente sincronizada

A mutually synchronized network in which all clocks are of equal status and exert equal amounts of control on the others; the network operating frequency (digit rate) being the mean of the natural (uncontrolled) frequencies of all the clocks.

#### 7020	hierarchic network, hierarchic synchronized network

F:  réseau hiérarchisé, réseau à synchronisation hiérarchisée
	S:  red jerárquica, red con sincronización jerárquica

A synchronized network in which each clock is assigned a particular status which determines the degree of control it exerts over the other clocks.

#### 7021	hierarchic mutually synchronized network

F:  réseau hiérarchisé à synchronisation mutuelle
	S:  red jerárquica mutuamente sincronizada

A mutually synchronized network in which each clock is assigned a particular status which determines the degree of control it exerts over other clocks; the network operating frequency being a weighted mean of the natural frequencies of all the clocks.

#### 7022	monarchic network, monarchic synchronized network [despotic network, despotic synchronized network]

F:  réseau despotique, réseau à synchronisation despotique
	S:  red despótica, red con sincronización despótica [red monárquica, red con sincronización monárquica]

A synchronized network in which a single clock exerts control over all the other clocks.

#### 7023	oligarchic network, oligarchic synchronized network

F:  réseau oligarchique, réseau à synchronisation oligarchique
	S:   red oligárquica, red con sincronización oligárquica

A synchronized network in which a few selected clocks are mutually synchronized and exert control over all the other clocks.

## 2.8	Pulse code modulation

#### 8001	pulse code modulation (PCM)

F:  modulation par impulsions et codage (MIC)
	S:  modulación por impulsos codificados (MIC)

A process in which a signal is sampled, and each sample is quantized independently of other samples and converted by encoding to a digital signal.

#### 8002	differential pulse code modulation (DPCM)

F:  modulation par impulsions et codage différentiel (MICD)
	S:  modulación por impulsos codificados diferencial (MICD)

A process in which a signal is sampled, and the difference between each sample of this signal and its estimated value is quantized and converted by encoding to a digital signal.

NOTE – The estimated values of the signal are calculated by a predictor from the quantized difference signal.

#### 8003	delta modulation

F:  modulation delta
	S:  modulación delta

A form of differential pulse code modulation in which only the sign of the difference between each sample and its predicted value is detected and encoded by a single bit.

#### 8004	adaptive differential pulse code modulation (ADPCM)

F:  modulation par impulsions et codage différentiel adaptatif (MICDA)
	S:  modulación por impulsos codificados diferencial adaptativa (MICDA)

ADPCM algorithmes are compression algorithms that achieve bit rate reduction through the use of adaptive prediction and adaptive quantization.

#### 8005	predictor

F:  prédicteur
	S:  predictor

A device that provides an estimated value of a sampled signal derived from previous samples of the same signal or from a quantized version of those samples.

#### 8006	adaptive predictor

F:  prédicteur adaptatif
	S:  predictor adaptativo

A predictor whose estimating function is made variable according to the short term spectral characteristics of the sampled signal.

For ADPCM in particular, an adaptive predictor is a time-varying process that computer an estimate of the input signal from the quantized difference signal.

#### 8007	sample

F:  échantillon
	S:  muestra

A representative value of a signal at a chosen instant, derived from a portion of that signal.

#### 8008	sampling

F:  échantillonnage
	S:  muestreo

The process of taking samples of a signal, usually at equal time intervals.

#### 8009	sampling rate

F:  fréquence d'échantillonnage [taux d'échantillonnage]
	S:  velocidad de muestreo [frecuencia de muestreo]

The number of samples taken of a signal per unit time.

#### 8010	working range

F:  plage de fonctionnement [gamme de fonctionnement]
	S:  gama de funcionamiento

The range of values of an input signal over which an equipment is designed to operate with a specified performance. (See Figure 2.)

#### 8011	quantizing

F:  quantification
	S:  cuantificación

A process in which a continuous range of values is divided into a number of adjacent intervals, and any value within a given interval is represented by a single predetermined value within the interval. (See Figure 2.)

#### 8012	adaptive quantizing

F:  quantification adaptative
	S:  cuantificación adaptativa

Quantizing in which some parameters are made variable according to the short-term statistical characteristics of the quantized signal.

#### 8013	uniform quantizing

F:  quantification uniforme
	S:  cuantificación uniforme

Quantizing in which all the quantizing intervals lying entirely with the working range are equal. (See Figure 2.)

#### 8014	non-uniform quantizing

F:  quantification non uniforme
	S:  cuantificación no uniforme

Quantizing in which not all the quantizing intervals lying entirely with the working range are equal. (See Figure 2.)

#### 8015	quantizing interval

F:  intervalle de quantification
	S:  intervalo de cuantificación

One of the intervals used in quantizing. (See Figure 2.)

#### 8016	decision value

F:  valeur de décision [amplitude de décision]
	S:  valor de decisión

A value defining the boundary between adjacent quantizing intervals. (See Figures 2 and 4.)

#### 8017	virtual decision value

F:  valeur virtuelle de décision [amplitude virtuelle de décision]
	S:  valor virtual de decisión

Each of the two defined values, that provide conventional bounds for the working range in quantizing. (See Figure 2.)

NOTE – These values are taken to represent hypothetical outer bounds for the two extreme quantizing intervals of the quantizing law.

FIGURE 2/G.701...[D02] =   15.5  CM

#### 8018	quantized value

F:  valeur quantifiée [amplitude quantifiée]
	S:  valor cuantificado

The single discrete value used to represent any value in a particular quantizing interval. (See Figure 2.)

#### 8019	load capacity [overload point]

F:  capacité de charge [point de surcharge]
	S:  capacidad de carga [punto de sobrecarga]

The level of a sinusoidal signal whose positive and negative peaks coincide with the virtual decision values.

#### 8020	peak limiting, peak limiting in quantizing

F:  écrêtage
	S:  limitación de cresta (en cuantificación)

The effect whereby any value to be quantized lying outside the working range is replaced by the nearest quantized value. (See Figure 2.)

#### 8021	quantizing distortion

F:  distorsion de quantification
	S:  distorsión de cuantificación

The distortion resulting from the process of quantizing samples within the working range.

#### 8022	quantizing distortion power

F:  puissance de distorsion de quantification
	S:  potencia de la distorsión de cuantificación

The power of the distortion component of the output signal resulting from the process of quantizing.

#### 8023	encoding [coding, coding in PCM]

F:  codage
	S:  codificación [codificación en MIC]

The generation of a code word to represent a quantized value.

#### 8024	encoder [coder]

F:  codeur
	S:  codificador

A device that performs encoding.

#### 8025	uniform encoding

F:  codage uniforme
	S:  codificación uniforme

The generation of code words to represent uniformly quantized values.

#### 8026	non-uniform encoding

F:  codage non uniforme
	S:  codificación no uniforme

The generation of code words to represent non-uniformly quantized values. (See Figure 3.)

#### 8027	encoding law

F:  loi de quantification [loi de codage]
	S:  ley de codificación

The law defining the relative values of the quantizing intervals used in quantizing and encoding. (See Figure 4.)

#### 8028	segmented encoding law

F:  loi de quantification à segments [loi de codage à segments]
	S:  ley de codificación por segmentos

An encoding law in which an approximation to a smooth law is obtained by a number of linear segments. [See diagrams a) and b) of Figure 3.]

#### 8029	decoding

F:  décodage
	S:  decodificación

The generation of reconstructed samples.

#### 8030	decoder

F:  décodeur
	S:  decodificador

A device that performs decoding.

FIGURE 3/G.701...[D03] =  11   CM

FIGURE 4/G.701...[D04] =  9   CM

#### 8031	reconstructed sample

F:  échantillon reconstitué
	S:  muestra reconstruida

The signal generated at the output of a decoder when a specified digital signal representing a quantized value is applied to its input.

#### 8032	codec

F:  codec
	S:  códec

A combination of an encoder and a decoder operating in opposite directions of transmission in the same equipment.

NOTE – When used to describe an equipment the function of the equipment should qualify the title, for example: supergroup codec, hypergroup codec.

#### 8033	digilogue channel

F:  voie digilogue
	S:  canal digi-analógico

A channel in which information is represented by a digital signal at one end and the same information is represented by the corresponding analogue signal at the other end.

NOTE – The term may be qualified by “A to D” or “D to A” to indicate whether encoding or decoding is being performed.

#### 8034	digilogue circuit

F:  circuit digilogue
	S:  circuito digi-analógico

A circuit in which transmission is provided in one direction by an A to D digilogue channel and in the other direction by a D to A digilogue channel.

NOTE – Because the digital interface is inherently 4-wire, the term may be qualified by “2-W” or “4-W” to indicate whether the analogue interface is 2-wire or 4-wire.

#### 8035	asynchronous tandem encoding

F:  codage asynchrone en cascade
	S:  codificación asíncrona en tándem

An asynchronous tandem encoding is a transcoding of the form ADPCM-PCM-analog signal-PCM-ADPCM.

#### 8036	build-out delay

F:  délai de reconstitution
	S:  retardo de reconstitución

Build-out delay is the maximum variable transmission and processing delay that is permitted in a wideband network.

#### 8037	check sequence (CS)

F:  séquence de contrôle
	S:  secuencia de verificación

The check sequence is a sequence of bits that offers a cyclic redundancy check (CRC) to allow the detection of errors.

#### 8038	embedded adaptive differential pulse code modulation (embedded ADPCM)

F:  modulation par impulsions et codage différentiel adaptatif imbriqué (ADPCM imbriqué)
	S:  modulación por impulsos codificados diferencial adaptiva incorporada (MICDA incorporada)

Embedded ADPCM algorithms are ADPCM algorithms that quantize the difference between the input and the estimated signal into core bits and enhancement bits.

#### 8039	frame abort

F:  abandon de trames
	S:  aborto de trama

The frame abort is the event that occurs when a link layer entity ignores a frame that it is currently receiving.

#### 8040	frame check sequence (FCS)

F:  séquence de contrôle de la trame
	S:  secuencia de verificación de la trama

The frame check sequence is a cyclic redundancy check sequence that is derived over an entire frame (excluding flags).

#### 8041	synchronous coding adjustment

F:  ajustement du codage synchrone
	S:  ajuste de la codificación síncrona

Synchronous coding adjustment is a method that prevents the accumulation of distortion on synchronous tandem encodings in the absence of transmission errors and when the integrity of the bit stream is preserved.

#### 8042	synchronous tandem encoding

F:  codage synchrone en cascade
	S:  codificación síncrona en tándem

A synchronous tandem encoding is a transcoding of the form ADPCM-PCM-ADPCM in the absence of transmission errors and when the integrity of the bit stream is preserved.

## 2.9	Codes

#### 9001	pulse code

F:  code d'impulsions (code de modulation d'impulsions)
	S:  código de impulsos

A set of rules giving the equivalence between each quantized value and its corresponding code word.

#### 9002	line code

F:  code en ligne
	S:  código de línea, código en línea

A code chosen to suit the characteristics of a channel, that defines the equivalence between sets of digits presented for transmission and the corresponding sequence of signal elements transmitted over that channel.

#### 9003	redundant line code

F:  code en ligne redondant
	S:  código de línea redundante

A line code that uses more encoded signal elements than strictly necessary to represent groups of digits of the original signal.

#### 9004	alternate mark inversion code (AMI code)

F:  code bipolaire [code bipolaire alternant, code bipolaire strict]
	S:  código de inversión de marcas alternada (código AMI) [código bipolar]

A line code that employs a ternary signal to convey binary digits, in which successive binary ones are represented by signal elements that are normally of alternating, positive and negative polarity but equal in amplitude, and in which binary zeros are represented by signal elements that have zero amplitude.

#### 9005	modified alternate mark inversion code

F:  code bipolaire alternant modifié
	S:  código de inversión de marcas alternada modificado

A line code that is based on an alternate mark inversion code, in which alternate mark inversion violations occur in accordance with a defined set of rules.

#### 9006	alternate mark inversion signal [bipolar signal]

F:  signal bipolaire [signal bipolaire alternant]
	S:  señal de inversión de marcas alternada [señal bipolar]

The encoded signal produced by alternate mark inversion code.

#### 9007	alternate mark inversion violation [bipolar violation]

F:  violation de bipolarité
	S:  violación de inversión de marcas alternada [violación bipolar]

A non-zero signal element in an alternate mark inversion signal that has the same polarity as the previous non-zero signal element.

#### 9008	disparity

F:  disparité
	S:  disparidad

The algebraic sum of the values of the departure from notional zero level of one or more consecutive signal elements forming a defined group.

#### 9009	digital sum

F:  somme numérique
	S:  suma digital

The algebraic sum of the disparities of a sequence of consecutive defined groups of signal elements.

#### 9010	digital sum variation

F:  variation de la somme numérique
	S:  variación de la suma digital

The difference between the maximum possible digital sum and the minimum possible digital sum of a specified number of groups of signal elements.

#### 9011	balanced code

F:  code à somme bornée
	S:  código equilibrado

A code that generates only groups of signal elements that have zero digital sum.

#### 9012	paired-disparity code [alternate code, alternating code]

F:  code à disparité compensée
	S:  código con disparidad compensada [código alternado, código alternante]

A code in which some or all of the digits in the original signal are represented by two assemblies of digits, of opposite disparity, which are used in a sequence to minimize the digital sum of a longer sequence of digits.

NOTE – An alternate mark inversion signal is an example of a paired-disparity code.

#### 9013	PCM binary code

F:  code binaire MIC
	S:  código binario MIC

A pulse code in which all the quantized values are identified by binary numbers taken in order.

NOTE – This term should not be used for line transmission.

#### 9014	symmetrical binary code

F:  code binaire symétrique
	S:  código binario simétrico

A pulse code in which the sign of the quantized value is represented by one digit, and in which the remaining digits constitute a binary number representing the magnitude.

NOTES

1	In a particular symmetrical binary code, the order of the digits and the use made of the symbols 0 and 1 in the various digit positions must be specified.

2	This term should not be used for line transmission.

#### 9015	code conversion

F:  transcodage
	S:  conversión de código

The conversion of digital signals in one code to the corresponding signals in a different code.

Annex  A

Alphabetical list of terms defined in this Recommendation

(This annex forms an integral part of this Recommendation)

8004	adaptive differential pulse code modulation (ADPCM)

8006	adaptive predictor

8012	adaptive quantizing

3023	alarm indication signal (AIS)

9012	[alternate code]

9004	alternate mark inversion code (AMI code)

9012	[alternating code]

9006	alternate mark inversion signal

9007	alternate mark inversion violation

7014	amplitude quantized control

7012	analogue control

1002	analogue signal

6015	anisochronous

6018	[asynchronous/heterochronous]

8035	asynchronous tandem encoding

9011	balanced code

7011	bilateral control

2003	binary digit

2002	binary figure

9006	[bipolar signal]

9007	[bipolar violation]

3019	bit sequence independence

8036	build-out delay

5003	bunched frame alignment signal

4001	(bus: American)

6022	centralized-clock interface

1005	channel, transmission channel

4002	channel gate

6006	channel time-slot

2005	[character signal]

8037	check sequence (CS)

1007	circuit, telecommunication circuit

6009	clock

7004	clock control signal

2005	code word

8032	codec

8024	[coder]

8023	[coding, coding in PCM]

6021	codirectional interface

1009	congestion

6023	contradirectional interface

2026	controlled slip

2020	decision circuit

2019	decision instant, decision instant of a digital signal

8016	decision value

8030	decoder

8029	decoding

8003	delta modulation

7019	democratic network, democratic mutually synchronized network

2029	descrambler

4011	(deserializer: American)

7022	[despotic network, despotic synchronized network]

8002	differential pulse code modulation (DPCM)

8033	digilogue channel

8034	digilogue circuit

2001	digit

2008	digit position

2013	digit rate

3020	digit sequence integrity

6005	digit time-slot

4019	digital block

3002	digital channel, digital transmission channel

3003	digital circuit, digital telecommunication circuit

3004	digital connection

4016	digital demultiplexer

4015	digital demultiplexing

3006	digital distribution frame

2030	digital error

4021	digital filling

3013	digital line link

3013	[digital line path]

3012	digital line section

3015	digital line system

3005	digital link, digital transmission link

4014	digital multiplexer

4017	digital multiplex equipment

4003	digital multiplex hierarchy

4013	digital multiplexing

4021	[digital padding]

3005	[digital path]

3017	digital radio link

3017	[digital radio path]

3016	digital radio section

3018	digital radio system

3007	digital section

2006	digital signal

9009	digital sum

9010	digital sum variation

3003	digital telecommunication circuit

3001	digital transmission

3002	digital transmission channel

3005	digital transmission link

3014	digital transmission system

3022	digital transparency

4005	(digroup: American)

1003	discretely-timed signal

9008	disparity

5004	distributed frame alignment signal

7009	double-ended synchronization

4010	[dynamicizer]

3009	elementary cable section

3011	elementary regenerator section

3010	elementary repeater section

8038	embedded adaptive differential pulse code modulation (embedded ADPCM)

8024	encoder

8023	encoding

8027	encoding law

2015	equivalent binary content

2016	equivalent bit rate

2030	error, digital error

2032	error multiplication

2033	error multiplication factor

2031	[error rate]

2031	error ratio

2034	error spread

4007	frame

8039	frame abort

5001	frame alignment

5002	frame alignment signal

5005	frame alignment recovery time

6008	frame alignment time-slot

8040	frame check sequence

6020	heterochronous

7021	hierarchic mutually synchronized network

7020	hierarchic network, hierarchic synchronized network

4001	highway

6017	homochronous

3025	[housekeeping digits]

1008	interface

6014	isochronous

2024	jitter

4026	justifiable digit time-slot

4022	justification

4029	justification rate

4032	justification ratio

4028	justification service digit

4027	justifying digit

9002	line code

2014	line digit rate

7013	linear analogue control

8019	load capacity

6011	master clock

4031	maximum justification rate

4031	[maximum stuffing rate]

6016	[mesochronous]

9005	modified alternate mark inversion code

7022	monarchic network, monarchic synchronized network

4008	multiframe

7018	mutually synchronized network

4006	n-ary digital group

2009	n-ary digital signal

4024	negative justification

4024	[negative stuffing, negative pulse stuffing]

4030	nominal justification rate

4033	nominal justification ratio

4030	[nominal stuffing rate]

4033	[nominal stuffing ratio]

7017	non-synchronized network

6018	non-synchronous

8026	non-uniform encoding

8014	non-uniform quantizing

2004	octet

3021	octet sequence integrity

7023	oligarchic network, oligarchic synchronized network

5006	out-of-frame alignment time

8019	[overload point]

4034	packetized circuit multiplication equipment (PCME)

4035	packetized circuit multiplication system

9012	paired-disparity code

4010	parallel to serial converter

4018	PCM multiplex equipment

8020	peak limiting, peak limiting in quantizing

6019	plesiochronous

4023	positive justification

4023	[positive stuffing, positive pulse stuffing]

4025	positive/zero/negative justification

4025	[positive/zero/negative stuffing]

4025	positive/zero/negative pulse stuffing

8005	predictor

4005	[primary block]

4004	primary digital group

4005	primary PCM group

2012	psuedo n-ary signal

9001	pulse code

8001	pulse code modulation (PCM)

4022	[pulse stuffing]

8018	quantized value

8011	quantizing

8021	quantizing distortion

8022	quantizing distortion power

8015	quantizing interval

8031	reconstructed sample

2010	redundant digital signal

9003	redundant line code

2011	redundant n-ary signal

6010	reference clock

2021	regeneration

2023	regenerative repeater

2022	regenerator

3011	[regenerator section]

3009	[repeater section]

6013	relative time interval error

6003	retiming

8007	sample

8008	sampling

8009	sampling rate

2028	scrambler

3008	section termination

8028	segmented encoding law

4011	serial to parallel converter

4010	(serializer: American)

3025	service digits

1001	signal

2007	signal element

6007	signalling time-slot

2017	significant instant, significant instant of a digital signal

7008	single-ended synchronization

2026	[slip]

4011	[staticizer]

4026	[stuffable digit time-slot]

4022	[stuffing]

4027	[stuffing digit]

4029	[stuffing rate]

4032	[stuffing ratio]

4028	[stuffing service digit]

4009	subframe

2014	[symbol rate]

9014	symmetrical binary code

7001	synchronization

7003	synchronization information

7006	synchronization link

7005	synchronization node

7007	synchronization network

6016	synchronous

7016	synchronized network

8041	synchronous coding adjustment

7016	[synchronous network]

8042	synchronous tandem encoding

1006	telecommunication

1007	telecommunication circuit

4012	time-division multiplexing

6012	time interval error

7015	time quantized control

6004	time-slot

6002	[timing extraction]

7002	timing information

6001	timing signal

6002	timing recovery

1004	transmission

1005	tranmission channel

4020	transmultiplexer

3022	transparency, digital transparency

2027	uncontrolled slip

8025	uniform encoding

8013	uniform quantizing

7010	unilateral control

2018	unit interval

3024	upstream failure indication

8017	virtual decision value

2025	wander

8010	working range

## Reference

[1]	CCITT Recommendation Definitions relating to national and international numbering plans, Vol. VI, Rec. Q.10.

Appendix  I

List of abbreviations used in G-, H- and J-Series Recommendations

(This appendix does not form an integral part of this Recommendation)

The following abbreviations are used in G-, H- and J-Series Recommendations.

AAL	ATM adaptation layer

ACSE	Application control service element

A/D	Analog/Digital conversion

ADPCM	Adaptive differential pulse code modulation

AIS	Alarm indication signal

AMI	Alternate mark inversion

ASN.1	Abstract syntax notation one

ATM	Asynchronous transfer mode

AV	Audio visual

BAS	Bit rate allocation signal

B-ISDN	Broadband aspects of integrated services digital

BER	Bit error ratio (or rate)

BLER	Block error ratio

B8ZS	Bipolar eight zero substitution

CBO	Continuous bit-stream oriented (services)

CBR	Constant bit rate

CMI	Coded mark inversion

CMIP	Common management interface protocol

CMIS	Common management interface service

CRC	Cyclic redundancy check

DCME	Digital circuit multiplication equipment

DCMS	Digital circuit multiplication system

DCT	Discrete cosine transformation

DEMUX	Demultiplexer

DFB	Distributed feed-back (laser)

DFSM	Dispersion flattened single mode

DLC	Dynamic load control

DLCI	Data link connection identifier

DSI	Digital speech interpolation

DSP	Digital signal processor

DTMF	Dual tone multi-frequency

DXC	Digital cross-connect

E/O	Electro-optical conversion

F-Bit	Framing bit

FSK	Frequency shift keying

FTAM	File transfert, access and management (OSI application layer service)

HDLC	High-level data link control (protocol)

HEC	Header error control

HLC	Higher layer compatibility

HRX	Hypothetical reference connection

ISC	International switching centre

ISDN	Integrated services digital network

LAP	Link access protocols (e.g. LAPB, LAPD)

LED	Light emitting diode

LLC	Lower layer compatibility

LME	Layer management entity

LSB	Least significant bit

MOS	Mean opinion score

MSB	Most significant bit

MUX	Multiplexer

NRZ	Non-return to zero

O&M	Operations and maintenance

OMC	Operations and maintenance centre

OSF	Operations systems functions

OSI	Open system interconnection

PCM	Pulse code modulation

PDH	Plesiochronous digital hierarchy

PON	Passive optical network

qdu	Quantization distortion unit

QWL	Quantum well laser

ROSE	Remote operations service element

SDH	Synchronous digital hierarchy

TDM	Time division multiplexing

TDMA	Time division multiple access

TMN	Telecommunication management network

TS	Time slot

UNI	User network interface

VBR	Variable bit rate

WDM	Wavelength division multiplexing

WSF	Work station function

WTDM	Wavelenghth time division multiplexing

1)	Figure 1 gives examples of digital sections, digital links, digital line sections, etc.

2)	Figure 1 gives examples of digital sections, digital links, digital line sections, etc.

3)	Similar definitions are applicable to multiframe alignment.