|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.702

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

GENERAL  ASPECTS  OF  DIGITAL  TRANSMISSION  SYSTEMS

TERMINAL  EQUIPMENTS

DIGITAL  HIERARCHY  BIT  RATES

ITU-T  Recommendation  G.702

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.702 was published in Fascicle III.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.702

Fascicle III.4 - Rec. G.702

DIGITAL HIERARCHY BIT RATES

(Malaga-Torremolinos, 1984; amended at Melbourne, 1988)

The CCITT,

considering

(a)	that digital hierarchy bit rates are those bit rates which are or will be used as the basis for higher digital hierarchy levels if such levels exist (see Recommendation G.701, definition 4003);

(b)	that the specification of hierarchical bit rates is necessary to prevent the proliferation of interface standards used in digital networks;

(c)	that international interconnections of digital network components are preferably carried out at hierarchical bit rates;

(d)	that when determining hierarchical bit rates a number of factors relating to services, transmission media and networking need to be taken into account, for example:

-	characteristics of and suitable coding method for analogue source signals;

-	bit rates of digital source signals;

-	use of available transmission media;

-	compatibility with analogue multiplex systems;

-	modularity and flexibility in assembling and routing groups of source signals,

recommends

that the following bit rates should be used as hierarchical bit rates in digital networks:

| Digital Hierarchy Level | Hierarchical bit rates (kbit/s) for networks with the digital hierarchybased on a first level bit rate of |
| --- | --- |

|  | 1544 kbit/s | 2048 kbit/s |
| --- | --- | --- |

| 1 2 | 64 1544 6312 | 64 2 048 8 448 |
| --- | --- | --- |

| 3 4 | 32 064 97 728 | 44 736 | 34 368 139 264 |
| --- | --- | --- | --- |

Figure 1/G.702 and Figure 2/G.702 show the recommended hierarchical bit rates only, including references to related Recommendations on network interfaces, multiplex equipments, digital sections/systems.

Recommendations on the following may also exist, but are not identified in Figure 1/G.702 and Figure 2/G.702:

-	multiplex equipments operating between non-adjacent levels of the digital hierarchy;

-	network interfaces, multiplex equipments and digital sections/systems at non-hierarchical bit rates.

Annex A to this Recommendation provides guidelines on usable bits available for services.

ANNEX A

(to Recommendation G.702)

Usable bit rates available for services

In the case of access to the ISDN for broadband services, bit rates up to the primary hierarchy level are specified in the I.200 series of Recommendations.

In general, with regard to bit rates available for transport of service signals, the following guidelines apply:

A.1	For the hierarchies based on the 1544 kbit/s primary rate, the principle has been established that some bits in the frame should be reserved, in particular to perform quality control of the digital paths when several digital sections in tandem are involved. Table A-1/G.702 provides the necessary information.

TABLE A-1/G.702

Bit rates available for services and frame structures required at hierarchical interfaces

| Hierarchy bit rate (kbit/s) | Frame structure as per Recommendations | Reserved bits | Bit rate available for services (kbit/s) |
| --- | --- | --- | --- |

| 1 544 | G. 704 | F a) | 1 536 |
| --- | --- | --- | --- |

| 6 312 b) | Not applicable | None | 6 312 |
| --- | --- | --- | --- |

| 6 312 c) | G.704 | F bits and bits in time slots 97 and 98 | 6 144 |
| --- | --- | --- | --- |

| 44 736 | G.752 | Mj d) F0, F11, F12 | 44 407 d) |
| --- | --- | --- | --- |

a)	The use of the F-bit for purposes additional to framing is under study.

b)	In networks with asynchronous operation.

c)	In networks with synchronous operation.

d)	In some applications, the C-bits (Cj1 Cj2 and Cj3) may also need to be reserved. In this case, the bit rate available for services becomes 44 209 kbit/s.

A.2	In case of networks using 2048 kbit/s based hierarchy there is in principle no basic restriction on the use of full capacity of the digital path. However, it is recognized that compatibility with recommended frame structures at the various levels of the 2 Mbit/s hierarchy (e.g. the use of the same frame alignment pattern) could be a preferred solution since it offers the following advantages:

-	use of the same framing devices for switched and non-switched applications;

-	end-to-end quality control performed in a unique way by the network when the maintenance entity that terminates the service (e.g. the encoding device) does not belong to the network;

-	possibility of performing additional network management functions that could be required, depending on the applications.

The preference for compatibility with recommended frame structures could be reconsidered for the applications where significant restrictions on the efficient use of the digital path capability can be identified.