|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.7041-Y.1303 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (10/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – General |  |  |  |
|  | Amendment 1 to ITU-T Rec. G.7041/Y.1303 |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Amendment 1 to Recommendation G.7041/Y.1303

Add new references:

- ITU-T Recommendation G.8021/Y.1341 (2004), Characteristics of Ethernet Transport Network Equipment Functional Blocks

- ITU-T Recommendation G.8040/Y.1340 (2003), GFP Frame Mapping into Plesiochronous Digital Hierarchy (PDH)

Change the title of clause 6 to read:

6.	Aspects common to both Frame-mapped and Transparent-mapped modes of GFP

Modify Table6-3 as follows in order to assign a UPI code for a direct MPLS mapping into GFP-F:

Table 6-3/G.7041/Y.1303 – User payload identifiers for GFP client frames

| PTI = 000 |  |
| --- | --- |
| Type bits <7:0> | GFP frame payload area |
| 0000 0000 1111 1111 | Reserved and not available |
| 0000 0001 | Frame-Mapped Ethernet |
| 0000 0010 | Frame-Mapped PPP |
| 0000 0011 | Transparent Fibre Channel |
| 0000 0100 | Transparent FICON |
| 0000 0101 | Transparent ESCON |
| 0000 0110 | Transparent Gb Ethernet |
| 0000 0111 | Reserved for future |
| 0000 1000 | Frame-Mapped Multiple Access Protocol over SDH (MAPOS) |
| 0000 1001 | Transparent DVB ASI |
| 0000 1010 | Framed-Mapped IEEE 802.17 Resilient Packet Ring |
| 0000 1011 | Frame-Mapped Fibre Channel FC-BBW |
| 0000 1100 | Asynchronous Transparent Fibre Channel |
| 0000 1101 | Frame Mapped MPLS (direct mapping) |
| 0000 1101 0000 1110 through 1110 1111 | Reserved for future standardization |
| 1111 0000 through 1111 1110 | Reserved for proprietary use (Note) |
| NOTE – The use of proprietary code values is described in Annex A/G.806. |  |

Modify Table6-4 as follows in order to reserve UPI codes for CMFs:

Table 6-4/G.7041/Y.1303 - GFP Client Management frame User Payload Identifier

| PTI = 100 |  |
| --- | --- |
| UPI value | Usage |
| 0000 0000 1111 1111 | Reserved |
| 0000 0001 | Client Signal Fail (Loss of Client Signal) |
| 0000 0010 | Client Signal Fail (Loss of Character Synchronization) |
| 0000 0011 through 1101 1111 | Reserved for future use |
| 1110 0000 through 1111 1110 | Reserved for proprietary use (Note 1) |
| Note 1: The use of proprietary code values is described in Annex A/G.806 |  |

Add new subclause 7.4.1:

7.4.1	Client-specific Signal Fail aspects

When frame-mapped GFP source adaptation process detects a client signal failure at ingress, the preferred action is to output the appropriate Client Signal Fail AIS if available.

In the case where no client signal AIS is available it is possible to generate a CMF[csf] at the GFP-F source adaptation process it may send a “Client Signal Fail” indication as described in 6.3.3. Other implementation-dependent indications of a failed client signal (e.g. loss-of-clock from an interface between integrated circuits) may be encoded as Client Signal Fail.

Note:  For further details of processing this signal and consequent action, refer to Recommendations G.8021 and G.806.

Add new subclause 7.6:

7.6	Direct mapping of unicast MPLS into GFP-F frames

The unicast MPLS PDU frame contains one or more MPLS-specific label stack entries (as specified in RFC 3032) and a MPLS payload information field. All octets in the unicast MPLS PDU are placed in the Payload Information field of a GFP-F frame. Both octet-alignment and bit identification within octets are maintained within the GFP-F PDU. This direct mapping of unicast MPLS into GFP is intended to be the default mapping when unicast MPLS client signals are directly carried over a transport network.

The GFP Payload FCS is required and is computed as specified in 6.1.2.2.1.1 and inserted in the pFCS field. The PFI field is set to 1.

This relationship between unicast MPLS PDU and GFP-F frame is illustrated in Figure 7-y.

Figure 7-y/G.7041/Y.1303 - Unicast MPLS and GFP Frame Relationships

Note 1 - This mapping allows only for the transport of MPLS encapsulated information. Treatment of MPLS control plane traffic which can be transported using IP without MPLS encapsulation is out of the scope of this sub-clause.

Note 2 - The mapping of multicast MPLS PDU frames is for further study.

Place the existing contents of Appendix III under the heading

“III.1  Worked Example for a GFP-F frame”

Insert a new section at the end of Appendix III containing the following:

III.2  Worked Example for a GFP-T Superblock CRC Calculation

This section provides a worked example calculation of the CRC-16 for a GFP-T superblock.  For this example, the first octet of the superblock (octet 1,1) contains the value 80hex (i.e., a 1 in the MSB position), and all other octets in the superblock, including the L-bit octet, contain all 0s.  The resulting CRC-16 value will be 1001 1010 1010 0010 (9AA2 hex) in bits CRC1-CRC16, respectively.

_____________________