|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.7042/Y.1305 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Corrigendum 1 (08/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – General SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS Internet protocol aspects – Transport |  |  |  |
|  | Corrigendum 1 to the Recommendation G.7042/Y.1305 |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Corrigendum 1 to the Recommendation G.7042/Y.1305

Summary

This document contains a corrigendum both normative text and Appendix I to the ITU-T recommendation G.7042/Y.1305 (02/04).

# 4	Abbreviations

MSU	Member signal unavailable

MSU_L	Member signal unavailable, LCAS enabled criteria

# LCAS for virtual concatenation

## 6.1	Methodology

LCAS in the virtual concatenation source and sink adaptation functions provides a control mechanism to hitless increase or decrease the capacity of a VCG link to meet the bandwidth needs of the application. It also provides the capability of temporarily removing member links that have experienced a failure. The LCAS assumes that in cases of capacity initiation, increase or decrease, the construction or destruction of the end-to-end path of each individual member is the responsibility of the Network and Element Management Systems. A VCG capacity increase or decrease can be initiated at either end. However, the initiation of a VCG capacity decrease at the Sk may result in temporary loss of data, see section 6.5.

### 6.2.1	MultiFrame Indicator (MFI) field

At the So side the MFI is equal for all members of the VCG. At the Sk side the MFI shall be used to realign the payload for all the members in the group. The MFI is used to determine the differential delay between members of the same VCG.

### 6.2.2	Sequence Indicator (SQ) field

The SQ of a member of the VCG sending IDLE in the control field shall be set to the highest possible value.

### 6.2.4	Group Identification (GID) bit

Used for identification of the VCG. The GID bit of all members of the same VCG has the same value in the control packets with the same MFI.

- 6.2.5.1	CRC Multiplication/division process

The bits of the control packet can be regarded the coefficients of a polynomial where the first bit of the control packet to be transmitted is the most significant bit.  A particular CRC-n block is the remainder after multiplication of all bits in a control packet by xn and then division (modulo 2) by the application specific generator polynomial. The remainder is a polynomial of at most degree (n-1).

### 6.2.6	Member Status (MST) field

At initiation of a VCG sink all members shall report MST FAIL. A transition to MST=OK occurs when a control packet is received for that member with a control field of ADD (or NORM or EOS after it has been added, or DNU after recovery from a network failure). All unused MST and members that have a control field of IDLE, shall be set to FAIL.

### 6.2.7	Re-Sequence Acknowledge (RS-Ack) bit

The expiration of the time-out is equivalent to the detection of a toggled RS-Ack bit at So (refer to SDL protocol description, shown in figures A.1 and A.4, for details).

NOTE - No new change in the VCG should be executed, i.e. no member should be added or removed from the VCG, until the RS-Ack is received or the RS-Ack time-out has expired for the currently active change request.

## 6.3	VCG capacity increase (Addition of member(s))

When a member is added it shall always be assigned a sequence number one larger than the currently highest sequence number that has EOS or DNU in the CTRL code. When multiple members are added, they must each use a unique sequence number so there will be a unique MST response for each additional member.

In case more than one member (e.g. x) is being added, and MST OK is being simultaneously received for those members, then the allocation of sequence indicators is arbitrary provided they are the next x sequence numbers after the currently highest sequence number (which has CTRL code EOS or DNU). The newly added members will have CTRL code NORM or EOS.

## 6.4	VCG Capacity Decrease: Member(s) temporary removed  by the LCAS procedure (due to failure)

### 6.4.1 Temporary removal of a member

When a member sending a NORM or EOS experiences a failure in the Network this is detected at the Sk (MSU_L, TSD) and the Sk will send MST = FAIL for that particular member. The reporting of the MST = FAIL can be delayed by a Hold-Off time to limit the number of switch actions in case of nested protection mechanisms. Upon detection of the MST=FAIL the  So will either replace the NORM condition by a DNU condition, or replace the EOS condition with an DNU condition. The active member with the highest sequence number will send EOS in the CTRL field.

#### 6.4.1.1	Temporary removal of member payload

There are two reasons for a temporary  removal of a members payload:

- In case of a received MSU_L the final step for temporary removal of a member is to remove that particular member from the VCG. At the Sk side the removal shall start immediately after detection of the MSU_L defect. At the So side the last container frame that contains payload of the removed member shall be the container frame containing the last bit(s) of the control packet containing the first DNU control field. The following container frames will contain all ZEROes in the payload area. Upon reception at the Sk of the DNU control field, the payload of this particular member shall not be used to reconstruct the original VCG payload.

- In case of a received TSD the final step for temporary removal of a member is to remove that particular member from the VCG. At the Sk side the payload area of that particular member will continue to be used for the reconstruction of the original VCG payload. The bit errors in the payload area of the member have to be handled by the server to client adaptation function at the sink side of the VCG. At the So side the last container frame that contains payload of the removed member shall be the container frame containing the last bit(s) of the control packet containing the first DNU code in the control field. The following container frames will contain all ZEROes in the payload area. Upon reception at the Sk of DNU in the control field the payload area of that particular member is removed from the VCG.

### 6.4.2 Reinstatement of temporarily removed member

When the defect causing the temporary removal is cleared this is detected at the Sk. The Sk will send MST=OK for that particular member. The reporting of the MST = OK can be delayed by a Wait-To-Restore time to avoid unwanted effects due to intermittent defects. Upon detection of the MST=OK the So will either replace the DNU condition by an NORM condition, or replace the DNU condition with an EOS condition and the preceding member, that was sending CTRL code EOS,will send NORM in the CTRL field.

#### 6.4.2.1 Reinstatement of temporarily removed members payload

The final step after recovering from a temporary removal is to start using the payload area of that member again. The first container frame to contain payload data for the member shall be the container frame immediately following the container frame that contained the last bit(s) of the control packet containing the first CTRL code NORM or EOS in the control field for that member.

## 6.5	VCG Capacity Decrease: Removal (permanent) of member(s)

When members are deleted, the sequence numbers shall be renumbered. If the permanently removed member contained the highest sequence number of that group, the active member containing the next highest sequence number shall change its control field to EOS in its control packet coinciding with the permanently removed member's control packet with the IDLE control field. If the permanently removed member contained the highest sequence number of that group and sends DNU in the control field, the sequence numbering and control fields of the other members in the group will not change. If the permanent removal of a member occurs somewhere other than at the highest end of the sequence, then the other members with sequence numbers between the newly permanently removed member and the highest sequence number shall update their sequence indicators in their control packets coinciding with the control packet changing the status of the permanently removed member.

Note that if permanent removal of members is initiated at the sink end first and the removed members were not the ones receiving signals with the highest SQ numbers, some of the remaining sink end members will receive SQ numbers higher than the new provisioned size at the sink end (until the members are removed at the source, too); this is not a fault condition.

Note, if a permanent removal of an active member is initiated at the Sk, this will result in a hit to the reconstructed data. The duration of this hit will be from the time the member is removed (starts sending MST = FAIL) until the DNU would have been received from the So.

Annex A

## A.1	LCAS Protocol

2)	the parameter XP, which indicates the number of provisioned members in the virtual concatenated group. Each completed ADD[i] command will increment XP by 1, each completed REMOVE[i] command will decrement XP by 1. Furthermore, the relationship 0  XP  XM holds;

2)	OK: The incoming signal for this member experiences no failure condition (i.e. MSU_L) or has received and acknowledged a request for addition of this member. When the incoming signal is degraded (i.e. TSD) the member remains in the OK state.

To avoid possible misalignment between So and Sk regarding the sequence numbers and the corresponding received far-end statuses, the number of provisioned members XP in the VCG is only changed under management command.

The sequence number received just before an MSU_L will be used for the reporting of the member status, but the payload will not be used to reconstruct the original signal. If the failed member is removed (by manager action) there will be a renumbering of the remaining sequence numbers. Replacement of a failed member (in the state DNU) because the failure in the Network cannot be repaired has to be performed via a REMOVE – ADD sequence.

## A.2	State diagram of member(i) in the Virtual Concatenated Group

Figure A.1/G.7042/Y.1305 – Source side state diagram

| Note 1 | The SQ of the removed member i (0  i  XP) shall be set to the highest possible value and the SQ of members with numbers i+1, … (XP-1) will be renumbered to i, …(XP-2) |
| --- | --- |

Figure A.2/G.7042/Y.1305 – Sink side state diagram

| Note 1 | As per clause 6.2.2, no specific SQ is available at the sink after receiving a control word of “IDLE” for a member. MST=FAIL is generated here according to the general rule in Annex B/G.806 for members without validated SQ. |
| --- | --- |
| Note 2 | This check verifies whether the SQ received for the present member is unique compared to those of the members in the OK state. If the received SQ is unique (i.e. not in use by any member among those in the OK state), the “y” branch is followed. Otherwise the “n” branch is followed. |
| Note 3 | For a particular member(i), "hold off" and "wait to restore" procedures are never simultaneously active. |

## A.3	Procedures state diagrams

### A.3.1	RS-Ack procedure

Figure A.4/G.7042/Y.1305.- RS-Ack procedure

### A.3.2	WTR procedure

Figure A.5/G.7042/Y.1305 –WTR procedure.

### A.3.3	HO procedure

Figure A.6/G.7042/Y.1305 – HO procedure.

Appendix I

- I.3 Provisioning

When a new container(s) is provisioned to be a member of the group it must be allocated the following:

SQ	= Set to a value larger than the currently highest sequence number that has EOS in the CTRL code. The SQ shall not be interpreted while CTRL = IDLE (not yet in service). It is recommended to set SQ to the highest possible value. Because this highest value is technology dependent, it is not possible to indicate a precise value. In the following examples the value (max) is used to indicate this highest value.

GID 	= The group ID for that virtually concatenated group.

- I.4.1.1 Add: (ADD) Multiple After last member.

| Note |  | Member n |  |  | member a (new) |  |  | Member a+1 (new) |  |  | RS-Ack |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTRL | SQ | MST | CTRL | SQ | MST | CTRL | SQ | MST |  |
| 1 | Initial Condition | EOS | n-1 | OK | IDLE | (max) | FAIL | IDLE | (max) | FAIL | 0 |
| 2 | NMS issues Add command to So and Sk LCASC | EOS | n-1 | OK | IDLE | (max) | FAIL | IDLE | (max) | FAIL | 0 |
| 3 | So (a) sends CTRL = ADD and SQ = n; So (a+1) sends CTRL = ADD and SQ =n+1 | EOS | n-1 | OK | ADD | n | FAIL | ADD | n+1 | FAIL | 0 |
| 4 | Sk (a+1) sends MS=OK to So | EOS | n-1 | OK | ADD | n | FAIL | ADD | n+1 | OK | 0 |
| 5 | So (n-1) sends CTRL = NORM; So (a+1) sends CTRL = EOS and SQ = n | NORM | n-1 | OK | ADD | n+1 | FAIL | EOS | n | OK | 0 |
| 6 | RS-Ack bit inverted due to change in sequence | NORM | n-1 | OK | ADD | n+1 | FAIL | EOS | n | OK | 1 |
| 7 | Sk (a) sends MST=OK to So | NORM | n-1 | OK | ADD | n+1 | OK | EOS | n | OK | 1 |
| 8 | So (a) sends CTRL = EOS; So (a+1) sends CTRL = NORM | NORM | n-1 | OK | EOS | n+1 | OK | NORM | n | OK | 1 |
| 9 | RS-Ack bit inverted due to change in sequence | NORM | n-1 | OK | EOS | n+1 | OK | NORM | n | OK | 0 |

- I.4.2.1 Decrease: (REMOVE) Planned Multiple NOT including last. member.

Figure I-2/G.7042: Planned removal of members 4 and 5 out of 6

| Note |  | member 4 |  |  | member 5 |  |  | Member 6 |  |  | RS-Ack |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTRL | SQ | MST | CTRL | SQ | MST | CTRL | SQ | MST |  |
| 1 | Initial Condition | NORM | 3 | OK | NORM | 4 | OK | EOS | 5 | OK | 0 |
| 2 | NMS issues Decrease command to So LCASC | NORM | 3 | OK | NORM | 4 | OK | EOS | 5 | OK | 0 |
| 3 | So (3) sends CTRL = IDLE, SQ = (max) So (4) sends CTRL = IDLE, SQ = (max) So (5) sends SQ = 3 | IDLE | )max) | OK | IDLE | (max) | OK | EOS | 3 | OK | 0 |
| 4 | Sk (un-wanted) sends MST = FAIL to So | IDLE | (max) | FAIL | IDLE | (max) | OK | EOS | 3 | OK | 1 |
| 5 | Sk (un-wanted) sends MST = FAIL to So | IDLE | (max) | FAIL | IDLE | (max) | FAIL | EOS | 3 | OK | 1 |
| 6 | RS-Ack bit inverted due to change in sequence | IDLE | (max) | FAIL | IDLE | (max) | FAIL | EOS | 3 | OK | 1 |
| 7 | NMS issues Decrease command to Sk LCASC | IDLE | (max) | FAIL | IDLE | (max) | FAIL | EOS | 3 | OK | 1 |

All un-wanted member are re-allocated an SQ greater than the SQ of the member sending the EOS control field, i.e. the highest possible value (max).

|  | VC | A | B | C | D | E | F | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Before | SQ | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
|  |  |  |  | U | U |  |  | U |
| After | SQ | 0 | 1 | (max) | (max) | 2 | 3 | (max) |

- I.4.2.2 Decrease: (REMOVE) Planned Single Last member.

Figure I-3/G.7042: Planned decrease single (last) member

| Note |  | Member n-1 |  |  | Member n |  |  | RS-Ack |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CTRL | SQ | MST | CTRL | SQ | MST |  |
| 1 | Initial Condition | NORM | n-2 | OK | EOS | n-1 | OK | 0 |
| 2 | NMS issues Decrease command to So LCASC | NORM | n-2 | OK | EOS | n-1 | OK | 0 |
| 3 | So (un-wanted) sends CTRL = IDLE, SQ = (max), So (n-2) sends CTRL = EOS | EOS | n-2 | OK | IDLE | (max) | OK | 0 |
| 4 | RS-Ack bit inverted, due to a change in the sequence | EOS | n-2 | OK | IDLE | (max) | FAIL | 1 |
| 5 | At the same time Sk (un-wanted) sends MST=FAIL | EOS | n-2 | OK | IDLE | (max) | FAIL | 1 |
| 6 | NMS issues Decrease command to Sk LCASC | EOS | n-2 | OK | IDLE | (max) | FAIL | 1 |

- I.4.3.1 Decrease (DNU) Due to fault Single Last member.

Text referring to Note 4 of the table above:

- As soon as an MSU_L is detected the Sk will immediately begin re-assembly of the concatenated group using only the NORM and EOS members. For a time (propagation time from Sk to So + re-action time of the So + propagation time from So to Sk) the re-assembled data will be erroneous because it is sent on all members as per pre-fault.

- If a TSD is detected the Sk continues to use the payload of this member. The bit errors in the payload area of the member have to be handled by the server to client adaptation function at the sink side of the VCG. For a time (propagation time from Sk to So + re-action time of the So + propagation time from So to Sk) the re-assembled data will be erroneous because it is sent on all members as per pre-fault.

Text referring to Note 5 of the table above:

However the So will stop sending data on the erroneous members (since they will have been reported back as MST = FAIL and consequently set the failed member to DNU), and send data only on the remaining NORM and EOS members.

- In case of MSU_L: from the time the CTRL = DNU would have arrived at the Sk until the CTRL = NORM is received again the bandwidth of the VCG is reduced.

- In case of TSD: from the time the CTRL = DNU arrives at the Sk the bandwidth of the VCG is reduced

Note1: – If the failed channel is subsequently deleted through a planned decrease prior to the fault clearing, the Sk will not be able to see the change in the failed member’s control packet.  As a result, RS-Ack will be not be inverted by this planned decrease and the So has to rely on the RS-Ack timeout to continue processing MST. The bandwidth of the VCG is not affected.

- I.4.3.2 Decrease: (DNU) Due to fault NOT last member.

Text referring to Note 4 of the table above:

- As soon as an MSU_L is detected the Sk will immediately begin re-assembly of the concatenated group using only the NORM and EOS members. For a time (propagation time from Sk to So + re-action time of the So + propagation time from So to Sk) the re-assembled data will be erroneous because it is sent on all members as per pre-fault.

- If a TSD is detected the Sk continues to use the payload of this member. The bit errors in the payload area of the member have to be handled by the server to client adaptation function at sink side of the VCG. For a time (propagation time from Sk to So + re-action time of the So + propagation time from So to Sk) the re-assembled data will be erroneous because it is sent on all members as per pre-fault.

Text referring to Note 5 of the table above:

However the source will stop sending data on the erroneous members (since they will have been reported back as MST = FAIL and consequently set the failed member to DNU), and send data only on the remaining NORM and EOS members.

- In case of MSU_L: from the time the CTRL = DNU would have arrived at the Sk until the CTRL=NORM is received again the bandwidth of the VCG is reduced.

- In case of TSD: from the time the CTRL = DNU arrives at the Sk the bandwidth of the VCG is reduced.

___________________