|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.7043/Y.1343 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (07/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – General SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS Internet protocol aspects – Transport |  |  |  |
|  | Virtual concatenation of plesiochronous digital hierarchy (PDH) signals |  |  |  |
|  | ITU-T Recommendation G.7043/Y.1343 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE – GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| General | G.7000–G.7099 |
| Coding of analogue signals by pulse code modulation | G.7100–G.7199 |
| Coding of analogue signals by methods other than PCM | G.7200–G.7299 |
| Principal characteristics of primary multiplex equipment | G.7300–G.7399 |
| Principal characteristics of second order multiplex equipment | G.7400–G.7499 |
| Principal characteristics of higher order multiplex equipment | G.7500–G.7599 |
| Principal characteristics of transcoder and digital multiplication equipment | G.7600–G.7699 |
| Operations, administration and maintenance features of transmission equipment | G.7700–G.7799 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.7800–G.7899 |
| Other terminal equipment | G.7900–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.7043/Y.1343 Virtual concatenation of plesiochronous digital hierarchy (PDH) signals

Summary This Recommendation specifies the virtual concatenation of 1544, 2048, 34 368 and 44 736 kbit/s PDH signals. This Recommendation makes use of the frame structures defined in ITU-T Recs G.704 and G.832.

Source ITU-T Recommendation G.7043/Y.1343 was approved on 22 July 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2004

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		1

5	Conventions		2

6	Specification of the virtually concatenated interfaces		2

6.1	Interface at N × 1544 kbit/s with virtual concatenation of a clear channel payload		2

6.2	Interface at N × 2048 kbit/s with virtual concatenation of a clear channel payload		4

6.3	Interface at N × 34 368 kbit/s with virtual concatenation of a clear channel payload		5

6.4	Interface at N × 44 736 kbit/s with virtual concatenation of a clear channel payload		7

Introduction

The basic PDH signal frame structures for 1544, 2048 and 44 736 kbit/s signals are defined in ITU-T Rec. G.704 and the octet structured 34 368 kbit/s is defined in ITU-T Rec. G.832. This Recommendation enhances the functions of these frame structures to support the virtual concatenation of these frame structures to form larger transport entities (containers). Only frame structures of the same type can be virtually concatenated together. This virtual concatenation supports the Link Capacity Adjustment Scheme (LCAS) specified in ITU-T Rec. G.7042/Y.1305.

ITU-T Recommendation G.7043/Y.1343

Virtual concatenation of plesiochronous digital hierarchy (PDH) signals

# 1	Scope

This Recommendation enhances the frame structures of PDH signals of 1544, 2048 and 44 736 kbit/s, as defined in ITU-T Rec. G.704, and the octet structured 34 368 kbit/s, as defined in ITU-T Rec. G.832 to support virtual concatenation. Specifically, frame structures of the same type may be virtually concatenated together to form larger transport entities (containers). This virtual concatenation supports the Link Capacity Adjustment Scheme (LCAS) specified in ITU-T Rec. G.7042/Y.1305.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

–	ITU-T Recommendation G.702 (1988), Digital hierarchy bit rates.

–	ITU-T Recommendation G.704 (1998), Synchronous frame structures used at 1544, 6312, 2048, 8448 and 44 736 kbit/s hierarchical levels.

–	ITU-T Recommendation G.707/Y.1322 (2003), Network node interface for the synchronous digital hierarchy (SDH).

–	ITU-T Recommendation G.832 (1998), Transport of SDH elements on PDH networks – Frame and multiplexing structures.

–	ITU-T Recommendation G.7041/Y.1303 (2003), Generic framing procedure (GFP).

–	ITU-T Recommendation G.7042/Y.1305 (2004), Link capacity adjustment scheme (LCAS) for virtual concatenated signals.

–	ITU-T Recommendation G.8040/Y.1340 (2004), GFP frame mapping into plesiochronous digital hierarchy (PDH).

# 3	Definitions

This Recommendation defines the following term:

3.1	nibble: A group of four bits.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

CTRL		Control field sent from source to sink

DNU		Do Not Use

EOS		End of Sequence

GFP		Generic Framing Procedure

GID		Group Identification

LCAS		Link Capacity Adjustment Scheme

MFI		MultiFrame Indicator

MST		Member Status

NORM		Normal Operating Mode

PDH		Plesiochronous Digital Hierarchy

RS-Ack		Re-sequence Acknowledge

SQ		Sequence Indicator

VCG		Virtual Concatenation Group

VLI		Virtual concatenation and LCAS Information

# 5	Conventions

The order of transmission of information in all the diagrams in this Recommendation is first from left to right and then from top to bottom. Within each byte or nibble the most significant bit is transmitted first. The most significant bit (bit 1) is illustrated at the left in all the diagrams.

# 6	Specification of the virtually concatenated interfaces

## 6.1	Interface at N × 1544 kbit/s with virtual concatenation of a clear channel payload

### 6.1.1	Frame and multiframe format

The multiframe structure for the 24-frame multiframe as described in ITU-T Rec. G.704 shall be used. The first octet following the first framing bit of the multiframe is used to carry the concatenation overhead for each 1544 kbit/s signal, as illustrated in Figure 6-1. This octet is reserved for all values of N (N = 1…16).

Figure 6-1/G.7043/Y.1343 – Concatenation overhead location for 
the N × 1544 kbit/s signal

### 6.1.2	Concatenation of N 1544 kbit/s signals

The concatenation overhead octet allows the virtual concatenation of N 1544 kbit/s signals to form a single channel referred to as a virtual concatenation group (VCG). The payload container bandwidth of the resulting VCG is N × [1536 – (64/24)] kbit/s  N × 1533 kbit/s. Client signals are mapped into the N 1544 kbit/s signal members of the VCG on an octet-wise, round robin basis. The round robin sequence follows the ascending order of the per-member sequence numbers that are communicated in each member's concatenation overhead octet. For example, if a data packet's octet 1 is mapped into the 1544 kbit/s signal with sequence number 0, the next packet octet is mapped into the 1544 kbit/s signal with sequence number 1, etc. Up to sixteen 1544 kbit/s signals can be virtually concatenated into a single VCG.

In general, the virtual concatenation and associated Link Capacity Adjustment Scheme (LCAS) functionality and definitions are the same as those specified for SDH virtual containers in ITU-T Rec. G.707/Y.1322. The areas in which they differ are specified in this Recommendation.

#### 6.1.2.1	Concatenation overhead octet definition

The concatenation overhead octet is time-multiplexed to carry the overhead required for virtual concatenation. This time multiplexing takes the form of a 16-nibble VLI that is transmitted one nibble per concatenation overhead octet. The concatenation overhead octet contains one nibble for the VLI and one nibble for the MFI1. The contents and format of the concatenation overhead octet and VLI are shown in Figure 6-2.

| Concatenation overhead octet definition |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| Control packet |  |  |  | MFI1 |  |  |  |
| MST (bits 1-4) |  |  |  | 1 | 0 | 0 | 0 |
| MST (bits 5-8) |  |  |  | 1 | 0 | 0 | 1 |
| 0 | 0 | 0 | RS-ACK | 1 | 0 | 1 | 0 |
| Reserved (0000) |  |  |  | 1 | 0 | 1 | 1 |
| Reserved (0000) |  |  |  | 1 | 1 | 0 | 0 |
| Reserved (0000) |  |  |  | 1 | 1 | 0 | 1 |
| Reserved (0000) |  |  |  | 1 | 1 | 1 | 0 |
| SQ bits 1-4 |  |  |  | 1 | 1 | 1 | 1 |
| MFI2 MSBs (bits 1-4) |  |  |  | 0 | 0 | 0 | 0 |
| MFI2 LSBs (bits 5-8) |  |  |  | 0 | 0 | 0 | 1 |
| CTRL |  |  |  | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | GID | 0 | 0 | 1 | 1 |
| Reserved (0000) |  |  |  | 0 | 1 | 0 | 0 |
| Reserved (0000) |  |  |  | 0 | 1 | 0 | 1 |
| C1 | C2 | C3 | C4 | 0 | 1 | 1 | 0 |
| C5 | C6 | C7 | C8 | 0 | 1 | 1 | 1 |

Figure 6-2/G.7043/Y.1343 – Virtual concatenation control packet format 
for N × 1544/2048 kbit/s signals

The definitions of the fields within the control packet are given in ITU-T Rec. G.7042/Y.1305. MFI1 is the least significant four bits of the concatenation multiframe count and is incremented once per concatenation overhead octet with its LSB located in bit 8.

The Cn bits are the CRC check over the control packet, with C1 being the MSB of the CRC. CRC generator polynomial is x8 + x2 + x + 1, and the calculation method is specified in ITU-T Rec. G.7042/Y.1305.

The member status (MST) information is time-multiplexed across multiple control packets as determined by the MFI2 LSB value. This time-sharing forms a member status multiframe that is specified in Figure 6-3.

| MFI2 bit 8 (LSB) value | VLI octet |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bit 1 | Bit 2 | Bit 3 | Bit 4 |  |
| 0 | 0 | 1 | 2 | 3 | Member numbers |
|  | 4 | 5 | 6 | 7 |  |
| 1 | 8 | 9 | 10 | 11 |  |
|  | 12 | 13 | 14 | 15 |  |

Figure 6-3/G.7043/Y.1343 – Member status bit assignments for 
N × 1544/2048 kbit/s signals

#### 6.1.2.2	Differential delay detection

All N 1544 kbit/s member signals that constitute a VCG are aligned at their transmission from the VCG source with respect to their 1544 kbit/s clock frequency, PDH signal frame and multiframe, MFI1 and MFI2. The VCG sink can determine the differential delay that the different members encountered in the network by comparing their respective MFI1 and MFI2 values, and performing the proper realignment. Note that the four MSBs of MFI2 are not used for differential delay compensation. The maximum differential delay that can be detected is (256)(24)(125s)/2 = 384 ms.

## 6.2	Interface at N × 2048 kbit/s with virtual concatenation of a clear channel payload

### 6.2.1	Frame and multiframe format

The basic frame and 16-frame multiframe structure at 2048 kbit/s as described in ITU-T Rec. G.704 shall be used. The time slot 1 of the first frame of the multiframe is used to carry the concatenation overhead for each 2048 kbit/s signal, as illustrated in Figure 6-4. This octet is reserved for all values of N (N = 1…16).

Figure 6-4/G.7043/Y.1343 – Concatenation overhead location for
the N × 2048 kbit/s signal

### 6.2.2	Concatenation of N 2048 kbit/s signals

The concatenation overhead octet allows the virtual concatenation of N 2048 kbit/s signals to form a single channel referred to as a virtual concatenation group. The payload container bandwidth of the resulting VCG is N × [1984 – (64/16)] kbit/s = N × 1980 kbit/s. Client signals are mapped into the N 2048 kbit/s signal members of the VCG on an octet-wise, round robin basis. The round robin sequence follows the ascending order of the per-member sequence numbers that are communicated in each member's concatenation overhead octet. For example, if a data packet's octet 1 is mapped into the 2048 kbit/s signal with sequence number 0, the next packet octet is mapped into the 2048 kbit/s signal with sequence number 1, etc. Up to sixteen 2048 kbit/s signals can be virtually concatenated into a single VCG.

In general, the virtual concatenation and associated link capacity adjustment scheme functionality and definitions are the same as those specified for SDH virtual containers in ITU-T Rec. G.707/Y.1322. The areas in which they differ are specified in this Recommendation.

#### 6.2.2.1	Concatenation overhead octet definition

The concatenation octet definitions for the 2048 kbit/s signal are the same as those specified in 6.1.2.1.

#### 6.2.2.2	Differential delay detection

The differential delay compensation for the 2048 kbit/s signal is the same as specified in 6.1.2.2 except that the clock frequency is 2048 kbit/s. Note that the four MSBs of MFI2 are not used for differential delay compensation. The maximum differential delay that can be detected is (256)(16)(125s)/2 = 256 ms.

## 6.3	Interface at N × 34 368 kbit/s with virtual concatenation of a clear channel payload

### 6.3.1	Frame and multiframe format

The multiframe structure for the 24-frame multiframe as described in ITU-T Rec. G.832 shall be used with the addition of a concatenation overhead byte. Specifically, the first octet following the FA2 byte is used to carry the concatenation overhead for each 34 368 kbit/s signal, as illustrated in Figure 6-5. This octet is reserved for all values of N (N = 1…8).

Figure 6-5/G.7043/Y.1343 – Concatenation overhead location for 
the N × 34 368 kbit/s signal

### 6.3.2	Concatenation of N 34 368 kbit/s signals

The concatenation overhead octet allows the virtual concatenation of N 34 368 kbit/s signals to form a single channel referred to as a virtual concatenation group. The payload container bandwidth of the resulting VCG is N × [(529/537) × 34 368] kbit/s ≈ N × 33 856 kbit/s. Client signals are mapped into the N 34 368 kbit/s signal members of the VCG on an octet-wise, round robin basis. The round robin sequence follows the ascending order of the per-member sequence numbers that are communicated in each member's concatenation overhead octet. For example, if data packet's octet 1 is mapped into the 34 368 kbit/s signal with sequence number 0, the next packet octet is mapped into the 34 368 kbit/s signal with sequence number 1, etc. Up to eight 34 368 kbit/s signals can be virtually concatenated into a single VCG.

In general, the virtual concatenation and associated link capacity adjustment scheme functionality and definitions are the same as those specified for SDH virtual containers in ITU-T Rec. G.707/Y.1322. The areas in which they differ are specified in this Recommendation.

#### 6.3.2.1	Concatenation overhead octet definition

The concatenation overhead octet is time-multiplexed to carry the overhead required for virtual concatenation. This time multiplexing takes the form of a 16-nibble VLI that is transmitted one nibble per control octet. The control octet contains one nibble for the VLI and one nibble for the MFI1. The contents and format of the concatenation overhead octet and VLI are shown in Figure 6-6.

| Concatenation overhead octet definition |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit 1 | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| Control packet |  |  |  | MFI1 |  |  |  |
| MST (bits 1-4) |  |  |  | 1 | 0 | 0 | 0 |
| MST (bits 5-8) |  |  |  | 1 | 0 | 0 | 1 |
| 0 | 0 | 0 | RS-ACK | 1 | 0 | 1 | 0 |
| Reserved (0000) |  |  |  | 1 | 0 | 1 | 1 |
| Reserved (0000) |  |  |  | 1 | 1 | 0 | 0 |
| Reserved (0000) |  |  |  | 1 | 1 | 0 | 1 |
| Reserved (0000) |  |  |  | 1 | 1 | 1 | 0 |
| 0 | SQ bits 1-3 |  |  | 1 | 1 | 1 | 1 |
| MFI2 MSBs (bits 1-4) |  |  |  | 0 | 0 | 0 | 0 |
| MFI2 LSBs (bits 5-8) |  |  |  | 0 | 0 | 0 | 1 |
| CTRL |  |  |  | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | GID | 0 | 0 | 1 | 1 |
| Reserved (0000) |  |  |  | 0 | 1 | 0 | 0 |
| Reserved (0000) |  |  |  | 0 | 1 | 0 | 1 |
| C1 | C2 | C3 | C4 | 0 | 1 | 1 | 0 |
| C5 | C6 | C7 | C8 | 0 | 1 | 1 | 1 |

Figure 6-6/G.7043/Y.1343 – Virtual concatenation control packet format 
for N × 34 368 and N × 44 736 kbit/s signals

The definitions of the fields within the control packet are given in ITU-T Rec. G.7042/Y.1305.

MFI1 is the least significant four bits of the concatenation multiframe count and is incremented once per concatenation overhead octet with its LSB located in bit 8. MFI2 is the most significant eight bits of the 12-bit concatenation frame count.

The Cn bits are the CRC check over the control packet, with C1 being the MSB of the CRC. CRC generator polynomial is x8 + x2 + x + 1, and the calculation method is specified in ITU-T Rec. G.7042/Y.1305.

The member status information is carried in each control packet as specified in Figure 6-7.

| Frame number (MFI1) | VLI octet |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bit 1 | Bit 2 | Bit 3 | Bit 4 |  |
| 0 | 0 | 1 | 2 | 3 | Member numbers |
| 1 | 4 | 5 | 6 | 7 |  |

Figure 6-7/G.7043/Y.1343 – Member status bit assignments for 
N × 34 368 and N × 44 736 kbit/s signals

#### 6.3.2.2	Differential delay detection

All N 34 368 kbit/s member signals that constitute a VCG are aligned at their transmission from the VCG source with respect to their 34 368 kbit/s clock frequency, PDH signal frame and multiframe, MFI1 and MFI2. The VCG sink can determine the differential delay that the different members encountered in the network by comparing their respective MFI1 and MFI2 values, and performing the proper realignment. The maximum differential delay that can be detected is [(536)(8)(212)]/2/34368000 = 255 ms.

## 6.4	Interface at N × 44 736 kbit/s with virtual concatenation of a clear channel payload

### 6.4.1	Multiframe format for N × 44 736 kbit/s

The multiframe structure for 44 736 kbit/s signal described in ITU-T Rec. G.704 shall be used. For the case of an N × 44 736 kbit/s signal, the first octet (two nibbles) following the first framing bit (X1) of the multiframe is used to carry the concatenation overhead in each constituent 44 736 kbit/s signal, as illustrated in Figure 6-8. This octet is reserved for all values of N (N = 1…8).

Figure 6-8/G.7043/Y.1343 – 44 736 kbit/s signal multiframe with reserved 
concatenation overhead octet for N × 44 736 kbit/s mappings

### 6.4.2	Concatenation of N 44 736 kbit/s signals

The concatenation overhead octet allows the virtual concatenation of N 44 736 kbit/s signals to form a single channel referred to as a virtual concatenation group. The payload container bandwidth of the resulting VCG is N × (44 736)[(7)(680 – 8) – 8]/[(7)(680)] kbit/s ≈ N × 44 134 kbit/s. Client data signals are mapped into the N 44 736 kbit/s signal members of the VCG on a nibble-wise, round robin basis. The round robin sequence follows the ascending order of the per-member sequence numbers that are communicated in each member's concatenation overhead octet. For example, if the most significant nibble of GFP frame octet 1 is mapped into the 44 736 kbit/s signal with sequence number 0, the least significant nibble of GFP frame octet 1 is mapped into the 44 736 kbit/s signal with sequence number 1, the most significant nibble of GFP frame octet 2 is mapped into the 44 736 kbit/s signal with sequence number 2, etc. Up to eight 44 736 kbit/s signals can be virtually concatenated into a single VCG.

The virtual concatenation and associated link capacity adjustment scheme functionality and definitions are the same as those specified for SDH virtual containers in ITU-T Rec. G.707/Y.1322.

#### 6.4.2.1	Concatenation overhead octet definition

The concatenation overhead octet is time-multiplexed to carry the overhead required for virtual concatenation. This time multiplexing takes the form of consecutive 16-octet control packets. The contents and format of the control packets are shown in Figure 6-6.

The definitions of the fields within the control packet are given in ITU-T Rec. G.7042/Y.1305. MFI1 is the least significant four bits of the concatenation multiframe count and is incremented once per concatenation overhead octet with its LSB located in bit 8. MFI2 is the most significant eight bits of the 12-bit concatenation frame count.

The Cn bits are the CRC check over the control packet, with C1 being the MSB of the CRC. CRC generator polynomial is x8 + x2 + x + 1, and the calculation method is specified in ITU-T Rec. G.7042/Y.1305.

The member status information is carried in each control packet as specified in Figure 6-7.

#### 6.4.2.2	Differential delay detection

All N 44 736 kbit/s member signals that constitute a VCG are aligned at their transmission from the VCG source with respect to their 44 736 kbit/s clock frequency, signal frame and multiframe, and their MFI1 and MFI2 frame count values. The VCG sink can determine the differential delay that the different members encountered in the network by comparing their respective MFI1 and MFI2 values, and performing the proper realignment. The maximum differential delay that can be detected is [(7)(680)(212)]/2/44736000 = 217 ms.

ITU-T Y-SERIES  RECOMMENDATIONS

GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS

| GLOBAL INFORMATION INFRASTRUCTURE |  |
| --- | --- |
| General | Y.100–Y.199 |
| Services, applications and middleware | Y.200–Y.299 |
| Network aspects | Y.300–Y.399 |
| Interfaces and protocols | Y.400–Y.499 |
| Numbering, addressing and naming | Y.500–Y.599 |
| Operation, administration and maintenance | Y.600–Y.699 |
| Security | Y.700–Y.799 |
| Performances | Y.800–Y.899 |
| INTERNET PROTOCOL ASPECTS |  |
| General | Y.1000–Y.1099 |
| Services and applications | Y.1100–Y.1199 |
| Architecture, access, network capabilities and resource management | Y.1200–Y.1299 |
| Transport | Y.1300–Y.1399 |
| Interworking | Y.1400–Y.1499 |
| Quality of service and network performance | Y.1500–Y.1599 |
| Signalling | Y.1600–Y.1699 |
| Operation, administration and maintenance | Y.1700–Y.1799 |
| Charging | Y.1800–Y.1899 |
| NEXT GENERATION NETWORKS |  |
| Frameworks and functional architecture models | Y.2000–Y.2099 |
| Quality of Service and performance | Y.2100–Y.2199 |
| Service aspects: Service capabilities and service architecture | Y.2200–Y.2249 |
| Service aspects: Interoperability of services and networks in NGN | Y.2250–Y.2299 |
| Numbering, naming and addressing | Y.2300–Y.2399 |
| Network management | Y.2400–Y.2499 |
| Network control architectures and protocols | Y.2500–Y.2599 |
| Security | Y.2700–Y.2799 |
| Generalized mobility | Y.2800–Y.2899 |

For further details, please refer to the list of ITU-T Recommendations.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |