|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.7043/Y.1343 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (01/2005) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – General |  |  |  |
|  | Virtual concatenation of PDH signals Amendment 1 |  |  |  |
|  | CAUTION ! PREPUBLISHED RECOMMENDATION This prepublication is an unedited version of a recently approved Recommendation. It will be replaced by the published version after editing. Therefore, there will be differences between this prepublication and the published version. |  |  |  |

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU [had/had not] received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

Amendment 1 to ITU-T Recommendation G.7043/Y.1343

Virtual concatenation of PDH signals

Summary

This amendment changes the specification of frame formats from nibbles to octets.

Change the following text in clause 6.4:

## 6.4	Interface at Nx44736 kbit/s with virtual concatenation of a clear channel payload

### 6.4.1	Multiframe format for N x 44736 kbit/s

The multiframe structure 44736 kbit/s signal described in ITU-T Rec. G.704 shall be used. For the case of an Nx44736 kbit/s signal, the first octet following the first framing bit (X1) of the multiframe is used to carry the concatenation overhead in each constituent 44736 kbit/s signal, as illustrated in Figure 6-8. This octet is reserved for all values of N (N=1…8).

### 6.4.2	Concatenation of N 44736 kbit/s signals

The concatenation overhead octet allows the virtual concatenation of N 44736 kbit/s signals to form a single channel referred to as a virtually concatenated group (VCG). The payload container bandwidth of the resulting VCG is N x (44736)[(7)(680 - 8) - 8]/[(7)(680)] kbit/s ≈ N x 44134 kbit/s. Client data signals are mapped into the N 44736 kbit/s signal members of the VCG on a octet-wise, round robin basis. The round robin sequence follows the ascending order of the per-member sequence numbers that are communicated in each member’s concatenation overhead octet. For example, if data packet’s octet 1 is mapped into the 44736 kbit/s signal with sequence number 0, the next packet octet is mapped into the 44736 kbit/s signal with sequence number 1, etc. Up to eight 44736 kbit/s signals can be virtually concatenated into a single VCG.

Note: The bits F1, F2, F3 and F4 will be located in the middle of a data packet’s octet

__________