|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.707/Y.1322 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (08/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – General SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS Internet protocol aspects – Transport |  |  |  |
|  | Network node interface for the synchronous digital hierarchy (SDH) Amendment 1 |  |  |  |
|  | ITU-T Recommendation G.707/Y.1322 (2003) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.707/Y.1322 Network node interface for the synchronous digital hierarchy (SDH) Amendment 1

Source Amendment 1 to ITU-T Recommendation G.707/Y.1322 (2003) was approved on 22 August 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1)	Recommended VC-11 interconnecting rule		1

2)	Add a new Annex G for SHDSL based TU-12 transport		1

ITU-T Recommendation G.707/Y.1322

Network node interface for the synchronous digital hierarchy (SDH)

Amendment 1

# 1)	Recommended VC-11 interconnecting rule

Clause 10.1.5  Mapping into VC-11

Replace:

NOTE – Refer to clause 9/G.803 for recommended selection criteria on the choice of primary rate mapping.

With:

Different structures are defined for the transport of 1544 kbit/s and 64 kbit/s client signals. To support 1544 kbit/s transport across SDH and PDH networks, the rule for interconnecting VC-11 mappings will be to use the 1544 kbit/s asynchronous mapping unless otherwise mutually agreed by the operators providing the transport. This SDH interconnection rule does not modify the mapping recommendations in ITU-T Rec. G.803. Refer to clause 9/G.803 for additional information on selection criteria and the choice of primary rate mappings.

# 2)	Add a new Annex G for SHDSL based TU-12 transport

Add to clause 2, References

–	ITU-T Recommendation G.991.2 (2003), Single-pair high-speed digital subscriber line (SHDSL) transceivers.

Add to clause 3, Terms and definitions

3.19	dSTM-12NMi interface: An SDH transmission interface which transports one or more TU-12, with SHDSL-based Section overhead. dSTM-12NMi interfaces are defined for SHDSL transport technologies. The number (N) of TU-12 in dSTM-12NMi interfaces provided by this Recommendation is limited to N  1 to 9 inclusive. The number (M) of SHDSL wire pairs over which the dSTM-12NMi signal is transported is limited to M = 1 to 4 inclusive. The number (i) represents the presence or absence of an (M × i × 8) kbit/s DCC in the dSTM-12NMi signal; it is limited to i = 0,..,7 (single-pair mode), i = 0,..,4 (2-pair mode), i = 0,..,3 (3-pair mode)  and i = 0, 1, 2 (4-pair mode). Not all combinations of N and M are allowed. Refer to Table G.1.

Add to clause 4, Acronyms and abbreviations

dSTM		SHDSL based Synchronous Transport Module

Add new Annex G:

Annex G

Mapping of N × TU-12 in M virtual concatenated SHDSL pairs (dSTM-12NMi)

Clause E.14/G.991.2 specifies the mapping of N × TU-12 (N = 1...9) into M (M = 1...4) virtual concatenated SHDSL wire pairs with an optional (M × i × 8) kbit/s DCC.

This annex specifies the associated additional multiplexing route and the SDH related signal names.

## G.1	Multiplex structure

Figure G.1 shows, within the general SDH multiplexing scheme (including the one defined in Rec. G.708 for sub-STM-0), the additional multiplexing route provided by E.14/G.991.2. I.e., the multiplexing routes of specific dSTM-12NMi (of order N  1...9, M = 1...4 and i = 0,..,7 (single-pair mode), i = 0,..,4 (2-pair mode), i = 0,..,3 (3-pair mode)  and i = 0, 1, 2 (4-pair mode)) via a Tributary Unit Group d12N (TUG-d12N).

## G.2	Mapping overview

Table G.1 provides an overview of the defined set of dSTM-12NMi signals. This table is based on Table E.41/G.991.2.

Table G.1/G.707/Y.1322 – Assignment of dSTM-12NMi names to N × TU-12/VC-12 connections over M-Pair SHDSL

| Number (N) of TU-12 / VC-12 connections | Aggregate payload bit rate [kbit/s] | 1-Pair SHDSL Size 1 × ks bits of each payload sub-block with ks = i + n × 8 [bits] M = 1 | 2-Pair SHDSL Size 2 × ks bits of each payload sub-block with ks = i + n × 8 [bits] M = 2 | 3-Pair SHDSL Size 3 × ks bits of each payload sub-block with ks = i + n × 8 [bits] M = 3 | 4-Pair SHDSL Size 4 × ks bits of each payload sub-block with ks = i + n × 8 [bits] M = 4 |
| --- | --- | --- | --- | --- | --- |
| 1 | 2304 + M × i × 8 | n = 36; i = 0,..,7 dSTM-12110 to dSTM-12117 | n = 18; i = 0,..,4 dSTM-12120 to dSTM-12124 | n = 12; i = 0,..,3 dSTM-12130 to dSTM-12133 | n = 9; i = 0,1,2 dSTM-12140 to dSTM-12142 |
| 2 | 4608 + M × i × 8 | n = 72; i = 0,..,7 dSTM-12210 to dSTM-12217 | n = 36; i = 0,..,4 dSTM-12220 to dSTM-12224 | n = 24; i = 0,..,3 dSTM-12230 to dSTM-12233 | n = 18; i = 0,1,2 dSTM-12240 to dSTM-12242 |
| 3 | 6912 + M × i × 8 | – | n = 54; i = 0,..,4 dSTM-12320 to dSTM-12324 | n = 36; i = 0,..,3 dSTM-12330 to dSTM-12333 | n = 27; i = 0,1,2 dSTM-12340 to dSTM-12342 |
| 4 | 9216 + M × i × 8 | – | n = 72; i = 0,..,4 dSTM-12420 to dSTM-12424 | n = 48; i = 0,..,3 dSTM-12430 to dSTM-12433 | n = 36; i = 0,1,2 dSTM-12440 to dSTM-12442 |
| 5 | 11 520 + M × i × 8 | – | – | n = 60; i = 0,..,3 dSTM-12530 to dSTM-12533 | n = 45; i = 0,1,2 dSTM-12540 to dSTM-12542 |
| 6 | 13 824 + M × i × 8 | – | – | n = 72; i = 0,..,3 dSTM-12630 to dSTM-12633 | n = 54; i = 0,1,2 dSTM-12640 to dSTM-12642 |
| 7 | 16 128 + M × i × 8 | – | – | n = 84; i = 0,..,3 dSTM-12730 to dSTM-12733 | n = 63; i = 0,1,2 dSTM-12740 to dSTM-12742 |
| 8 | 18 432 + M × i × 8 | – | – | – | n = 72; i = 0,1,2 dSTM-12840 to dSTM-12842 |
| 9 | 20 736 + M × i × 8 | – | – | – | n = 81; i = 0,1,2 dSTM-12940 to dSTM-12942 |
|  |  | If no data communication channel is used i = 0. If management, signalling, control and maintenance functions are to be transmitted over the Z-bits, i × 8 kbit/s per wire-pair are additionally required with i = 1,..,7 (1-pair), i = 1,..,4 (2-pair), i = 1, 2, 3 (3-pair) and i = 1, 2 (4-pair). |  |  |  |

ITU-T Y-SERIES  RECOMMENDATIONS

GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS

| GLOBAL INFORMATION INFRASTRUCTURE |  |
| --- | --- |
| General | Y.100–Y.199 |
| Services, applications and middleware | Y.200–Y.299 |
| Network aspects | Y.300–Y.399 |
| Interfaces and protocols | Y.400–Y.499 |
| Numbering, addressing and naming | Y.500–Y.599 |
| Operation, administration and maintenance | Y.600–Y.699 |
| Security | Y.700–Y.799 |
| Performances | Y.800–Y.899 |
| INTERNET PROTOCOL ASPECTS |  |
| General | Y.1000–Y.1099 |
| Services and applications | Y.1100–Y.1199 |
| Architecture, access, network capabilities and resource management | Y.1200–Y.1299 |
| Transport | Y.1300–Y.1399 |
| Interworking | Y.1400–Y.1499 |
| Quality of service and network performance | Y.1500–Y.1599 |
| Signalling | Y.1600–Y.1699 |
| Operation, administration and maintenance | Y.1700–Y.1799 |
| Charging | Y.1800–Y.1899 |
| NEXT GENERATION NETWORKS |  |
| Frameworks and functional architecture models | Y.2000–Y.2099 |
| Quality of Service and performance | Y.2100–Y.2199 |
| Service aspects: Service capabilities and service architecture | Y.2200–Y.2249 |
| Service aspects: Interoperability of services and networks in NGN | Y.2250–Y.2299 |
| Numbering, naming and addressing | Y.2300–Y.2399 |
| Network management | Y.2400–Y.2499 |
| Network control architectures and protocols | Y.2500–Y.2599 |
| Security | Y.2700–Y.2799 |
| Generalized mobility | Y.2800–Y.2899 |

For further details, please refer to the list of ITU-T Recommendations.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |