|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.722.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Annex B (11/2000) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of analogue signals by methods other than PCM |  |  |  |
|  | Coding at 24 and 32 kbit/s for hands-free operation in systems with low frame loss Annex B: Floating-point implementation for G.722.1 |  |  |  |
|  | ITU-T Recommendation G.722.1 – Annex B |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.722.1 Coding at 24 and 32 kbit/s for hands-free operation in systems with low frame loss Annex B Floating-point implementation for G.722.1

Summary G.722.1 implements a fixed-point algorithm. This annex contains the description of a floating-point extension to G.722.1, which will allow for easier implementation of the standard in general-purpose processors. This annex includes an electronic attachment containing the reference code and the test vectors for ITU-T G.722.1/Annex B floating-point algorithm implementation verification.

Source Annex B to ITU-T Recommendation G.722.1 was prepared by ITU-T Study Group 16 (2001-2004) and approved under the WTSA Resolution 1 procedure on 17 November 2000.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

Annex B  Floating-point implementation for G.722.1		1

B.1	Introduction		1

B.2	Algorithmic description		1

B.3	ANSI C code		1

Electronic attachment: Reference code and test vectors

ITU-T Recommendation G.722.1

Coding at 24 and 32 kbit/s for hands-free operation in systems 
with low frame loss

Annex B

Floating-point implementation for G.722.1

## B.1	Introduction

This annex provides a description of the floating-point arithmetic implementation for ITU-T Rec. G.722.1.

## B.2	Algorithmic description

This floating-point version of ITU-T Rec. G.722.1 has the same basic algorithmic steps as the fixed-point version.

## B.3	ANSI C code

ANSI-compliant C code simulating the floating-point version of ITU-T Rec. G.722.1 is available as an attachment to this annex. The algorithmic description given by the C code shall take precedence over the texts contained in the main body of ITU-T Rec. G.722.1 or this annex.

The files in Table B.1 comprise the floating-point source C code.

Table B.1/G.722.1 – List of software files specific to G.722.1 
floating-point source code

| File Name | Description |
| --- | --- |
| common.c | routines used by encoder and decoder |
| dct4.c | forward and inverse DCT |
| decode.c | main program for decoder |
| decoder.c | routines for decoder |
| encode.c | main program for encoder |
| encoder.c | routines for encoder |
| rmlt_coefs_to_samples.c | inverse MLT |
| samples_to_rmlt_coefs.c | MLT |
| defs.h | parameter definitions |
| huff_defs.h | definitions for Huffman coding |
| huff_tables.h | declaration of Huffman tables |

Once the stand-alone (floating-point) program is compiled into the encoder file, encode, and the decoder file, decode, then the command line format for using the coder is as follows:

encode   bit-stream-type  input-audio-file  output-bit-stream-file   bit-rate

decode   bit-stream-type  input-bit-stream-file  output-audio-file   bit-rate

where:

bit-stream-type = 0		specifies using the compacted bit stream;

bit-stream-type =1		specifies using the ITU-T Rec. G.192 bit stream format for test purposes;

input-audio-file	name of 16-bit PCM audio file to read samples from;

output-audio-file	name of 16-bit PCM audio file to save decoded output;

input-bit-stream-file	name of file to read the input bit stream from;

output-bit-stream-file	name of file to save the encoded bit stream output;

bit-rate = 24 000 or 32 000   for 24 kbit/s and 32 kbit/s operation respectively.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |