|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.722.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Corrigendum 1 (11/2000) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of analogue signals by methods other than PCM |  |  |  |
|  | Coding at 24 and 32 kbit/s for hands-free operation in systems with low frame loss Corrigendum 1 |  |  |  |
|  | ITU-T Recommendation G.722.1 – Corrigendum 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.722.1 Coding at 24 and 32 kbit/s for hands-free operation in systems with low frame loss Corrigendum 1

Summary This corrigendum corrects three changes necessary to the existing C code (Release 1.1) that is supplied with ITU-T Rec. G.722.1. In each case, it corrects an error that was introduced when the original C code (known as release code3.003 at the time of Determination) was converted to use basic operators. The corrected code will be labelled as Release 1.2. The complete release 1.2 code is available in a zipped form with ITU-T Rec. G.722.1 (09/99) on ITU-T website.

Source Corrigendum 1 to ITU-T Recommendation G.722.1 was prepared by ITU-T Study Group 16 (2001-2004) and approved under the WTSA Resolution 1 procedure on 17 November 2000.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

ITU-T Recommendation G.722.1

Coding at 24 and 32 kbit/s for hands-free operation 
in systems with low frame loss

Corrigendum 1

Introduction

This corrigendum corrects three changes necessary to the existing C code (Release 1.1) that is supplied with ITU-T Rec. G.722.1. In each case, it corrects an error that was introduced when the original C code (know in as release code3.003 at the time of Determination) was converted to use basic operators. The corrected code will be labelled as Release 1.2.

The complete release 1.2 code is available in a zipped form with ITU-T Rec. G.722.1 (09/99) on the ITU-T website.

Correction #1

The code in question concerned the decoder error checking to test if the regions had been properly received and decoded. The code in question is only used when the decoder is unaware that it is processing an errored frame, i.e. that the receiving system has not informed the G.722.1 decoder the frame is errored.

The erroneous code is located in file decoder.c, in the function test_4_frame_errors(). The relevant code from Release 1.1 is shown in Figure 1, and the proposed corrected code is also shown in Figure 1.

/* the next two lines of comments were modified in release 1.2 * to correct the description of the range of * absolute_region_power_index[] to be tested in the next * 9 lines of code. */ /* if ((absolute_region_power_index[region] > 33 31) ¦¦ (absolute_region_power_index[region] < -6 -8) */ acca = L_add(absolute_region_power_index[region], ESF_ADJUSTMENT_TO_RMS_INDEX); accb = L_sub(acca,31); acca = L_add(acca,8); test(); /* the next line was modifed in release 1.2 to * correct miss typed code and error checking. */ if ((accb > 0) ¦¦ (acca < 0)) if ((accb > 0) ¦¦ (accb > 0)) { frame_error_flag ¦= 4; logic16(); } Figure 1. Code extracted from Release 1.1. The incorrect comments and code which are to be deleted from Release 1.1 are shown with a strike-through, and the new text is shown in bold.

Correction #2

The function compute_region_powers() in file encoder.c contains an error when testing the range of the regions powers. Figure 2 shows the relevant section of code and the necessary correction.

/* The MLT is currently scaled too low by the factor ENCODER_SCALE_FACTOR(=18318)/32768 * (1./sqrt(160). This is the ninth power of 1 over the square root of 2. So later we will add ESF_ADJUSTMENT_TO_RMS_INDEX (now 9) to drp_code_bits[0]. */ /* drp_code_bits[0] can range from 1 to 31. 0 will be used only as an escape sequence. */ temp1 = sub(1,ESF_ADJUSTMENT_TO_RMS_INDEX); temp2 = sub(absolute_region_power_index[0],temp1); test(); if (temp2 < 0) { absolute_region_power_index[0] = temp1; move16(); } temp1 = sub(31,ESF_ADJUSTMENT_TO_RMS_INDEX); /** next line was corrected in Release 1.2 *******/ temp2 = sub(absolute_region_power_index[0], 31 temp1); test(); if (temp2 > 0) { absolute_region_power_index[0] = temp1; move16(); } Figure 2. Code extracted from Release 1.1. The incorrect comments and code which are to be deleted from Release 1.1 are shown with a strike-through, and the new text is shown in bold.

Correction #3

In file encoder.c, in function vector_huffman(), the variable inv_of_step_size_times_std_dev is declared as a word16. In the function vector_huffman() it is set to the product of two 16-bit variables, therefore is susceptible to overflow. In the original release of code it was a 32-bit quantity. In order to correct this using the basic operators, it is necessary to adjust several lines of code to accommodate this change. Figure 3 shows the changes to function vector_huffman(); it is also necessary change the contents of the table variable int_dead_zone[] in the file tables.c, add a new table, int_dead_zone_low_bits[] – and declare it in file tables.h as extern Word16 int_dead_zone_low_bits[NUM_CATEGORIES]. Figure 4 shows the new table and the modifications to int_dead_zone[].

Word16 mytemp; Word16 myacca; /* initialize variables */ vec_dim = vector_dimension[category]; move16(); num_vecs = number_of_vectors[category]; move16(); kmax = max_bin[category]; move16(); kmax_plus_one = add(kmax,1); move16(); current_word = 0L; move16(); current_word_bits_free = 32; move16(); number_of_region_bits = 0; move16(); /* set up table pointers */ bitcount_table_ptr = (Word16 *)table_of_bitcount_tables[category]; code_table_ptr = (UWord16 *) table_of_code_tables[category]; /* compute inverse of step size * standard deviation */ acca = L_mult(step_size_inverse_table[category], standard_deviation_inverse_table[power_index]); acca = L_shr(acca,1); acca = L_add(acca,4096); acca = L_shr(acca,13); mytemp = acca & 0x3; acca = L_shr(acca,2); inv_of_step_size_times_std_dev = extract_l(acca); for (n=0; n<num_vecs; n++) { index = 0; move16(); signs_index = 0; move16(); number_of_non_zero = 0; move16(); for (j=0; j<vec_dim; j++) { k = abs_s(*raw_mlt_ptr); acca = L_mult(k,inv_of_step_size_times_std_dev); acca = L_shr(acca,1);

myacca = L_mult(k,mytemp); myacca = L_shr(myacca,1); myacca = L_add(myacca,int_dead_zone_low_bits[category]); myacca = L_shr(myacca,2); acca = L_add(acca,int_dead_zone[category]); acca = L_add(acca,myacca); acca = L_shr(acca, 15 13); k = extract_l(acca); Figure 3. Code extracted from function vector_huffman() of Release 1.1. The incorrect comments and code which are to be deleted from Release 1.1 are shown with a strike-through, and the new text is shown in bold.

Word16 int_dead_zone_low_bits[NUM_CATEGORIES]= { 2, 1, 0, 0, 3, 2, 0, 0 }; Word16 int_dead_zone[NUM_CATEGORIES]= { 9830,10813,11796,12780,13763,14746,16384,16384 2457, 2703, 2949, 3195, 3440, 3686, 4096, 4096 }; Figure 4. New table, int_dead_zone_low_bits [], to be entered into the file tables.c, and the new values for int_dead_zone[].

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |