|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.722.2 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Annex A (01/2002) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of analogue signals by methods other than PCM |  |  |  |
|  | Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB) Annex A: Comfort noise aspects |  |  |  |
|  | ITU-T Recommendation G.722.2 – Annex A |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| General | G.6000–G.6099 |
| Symmetric cable pairs | G.6100–G.6199 |
| Land coaxial cable pairs | G.6200–G.6299 |
| Submarine cables | G.6300–G.6499 |
| Optical fibre cables | G.6500–G.6599 |
| Characteristics of optical components and subsystems | G.6600–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| General | G.7000–G.7099 |
| Coding of analogue signals by pulse code modulation | G.7100–G.7199 |
| Coding of analogue signals by methods other than PCM | G.7200–G.7299 |
| Principal characteristics of primary multiplex equipment | G.7300–G.7399 |
| Principal characteristics of second order multiplex equipment | G.7400–G.7499 |
| Principal characteristics of higher order multiplex equipment | G.7500–G.7599 |
| Principal characteristics of transcoder and digital multiplication equipment | G.7600–G.7699 |
| Operations, administration and maintenance features of transmission equipment | G.7700–G.7799 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.7800–G.7899 |
| Other terminal equipment | G.7900–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |
| General aspects | G.8000–G.8099 |
| Design objectives for digital networks | G.8100–G.8199 |
| Quality and availability targets | G.8200–G.8299 |
| Network capabilities and functions | G.8300–G.8399 |
| SDH network characteristics | G.8400–G.8499 |
| Management of transport network | G.8500–G.8599 |
| SDH radio and satellite systems integration | G.8600–G.8699 |
| Optical transport networks | G.8700–G.8799 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.722.2 Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB) Annex A Comfort noise aspects

Summary This annex details the operation of the background acoustic noise evaluation, noise parameter encoding/decoding and comfort noise generation for the AMR Wideband (AMR-WB) speech codec during Source Controlled Rate (SCR) operation. The comfort noise operations described here were also adopted by 3GPP in 3GPP specification TS 26.192.

Source Annex A to ITU-T Recommendation G.722.2 was prepared by ITU-T Study Group 16 (2001-2004) and approved under the WTSA Resolution 1 procedure on 13 January 2002.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

A.1	Scope		1

A.2	Definitions, symbols and abbreviations		1

A.2.1	Definitions		1

A.2.2	Symbols		1

A.2.3	Abbreviations		2

A.3	General		2

A.4	Functions on the transmit (TX) side		2

A.4.1	ISF evaluation		3

A.4.2	Frame energy calculation		4

A.4.3	Analysis of the variation and stationarity of the background noise		5

A.4.4	Modification of the speech encoding algorithm during SID frame generation		5

A.4.5	SID-frame encoding		6

A.5	Functions on the receive (RX) side		6

A.5.1	Averaging and decoding of the LP and energy parameters		6

A.5.2	Comfort noise generation and updating		7

A.6	Computational details and bit allocation		7

ITU-T Recommendation G.722.2

Wideband coding of speech at around 16 kbit/s using Adaptive
Multi-Rate Wideband (AMR-WB)

Annex A

Comfort noise aspects

## A.1	Scope

This annex details the operation of the background acoustic noise evaluation, noise parameter encoding/decoding and comfort noise generation for the AMR Wideband (AMR-WB) speech codec during Source Controlled Rate (SCR) operation.

Implementation of this annex is necessary for interoperability with 3GPP systems, but its use is not limited to mobile applications.

The user should note however that the C-code implementation of this annex is available as part of the C-code in Annex C/G.722.2. In case of discrepancy between the requirements described in this annex and the fixed point computational description of these requirements contained in Annex C/G.722.2, the description in Annex C/G.722.2 shall prevail.

## A.2	Definitions, symbols and abbreviations

### A.2.1	Definitions

This annex defines the following terms:

A.2.1.1	frame: Time interval of 20 ms corresponding to the time segmentation of the adaptive multi-rate wideband speech transcoder, also used as a short term for "traffic frame".

A.2.1.2	SID frames: Special Comfort Noise frames. It may convey information on the acoustic background noise or inform the decoder that it should start generating background noise.

A.2.1.3	speech frame: Traffic frame that cannot be classified as a SID frame.

A.2.1.4	VAD flag: Voice Activity Detection flag.

A.2.1.5	TX_TYPE: Classification of the transmitted traffic frame (defined in Annex B/G.722.2).

A.2.1.6	RX_TYPE: Classification of the received traffic frame (defined in Annex B/G.722.2).

Other definitions of terms used in this annex can be found in the main body of ITU-T Rec. G.722.2 and in Annex B/G.722.2. The overall operation of SCR is described in Annex B/G.722.2.

### A.2.2	Symbols

This annex uses the following symbols. Boldface symbols are used for vector variables:

fT = [f1f2 ... f16]	Unquantized ISF vector

fT = f1f2...f16	Quantized ISF vector

f (m)	Unquantized ISF vector of frame m

f(m)	Quantized ISF vector of frame m

f mean	Averaged ISF parameter vector

enlog	Logarithmic frame energy

enlogmean	Averaged logarithmic frame energy

e	ISF parameter prediction residual

ê	Quantized ISF parameter prediction residual

n=abx(n)	= x(a) + x(a + 1) + ... + x(b – 1) + x(b)

### A.2.3	Abbreviations

This annex uses the following abbreviations:

AMR	Adaptive Multi-Rate

AMR-WB	Adaptive Multi-Rate Wideband

CN	Comfort Noise

ISF	Immittance Spectral Frequency

ISP	Immittance Spectral Pair

LP	Linear Prediction

RSS	Radio Subsystem

RX	Receive

SCR	Source Controlled Rate (operation) (aka source discontinuous transmission)

SID	Silence Insertion Descriptor

TX	Transmit

UE	User Equipment

VAD	Voice Activity Detector

## A.3	General

A basic problem when using SCR is that the background acoustic noise, which is transmitted together with the speech, would disappear when the transmission is cut, resulting in discontinuities of the background noise. Since the SCR switching can take place rapidly, it has been found that this effect can be very annoying for the listener – especially in a car environment with high background noise levels. In bad cases, the speech may be hardly intelligible.

This annex specifies the way to overcome this problem by generating on the receive (RX) side synthetic noise similar to the transmit (TX) side background noise. The comfort noise parameters are estimated on the TX side and transmitted to the RX side at a regular rate when speech is not present. This allows the comfort noise to adapt to the changes of the noise on the TX side.

## A.4	Functions on the transmit (TX) side

The comfort noise evaluation algorithm uses the following parameters of the AMR-WB speech encoder, defined in the main body of ITU-T Rec. G.722.2:

–	the unquantized Linear Prediction (LP) parameters, using the Immittance Spectral Pair (ISP) representation, where the unquantized Immittance Spectral Frequency (ISF) vector is given by  f T = [f1f2 ... f16];

The algorithm computes the following parameters to assist in comfort noise generation:

–	the weighted averaged ISF parameter vector f mean (weighted average of the ISF parameters of the eight most recent frames);

–	the averaged logarithmic frame energy enlogmean (average of the logarithmic energy of the eight most recent frames).

These parameters give information on the level (enlogmean) and the spectrum (f mean) of the background noise.

The evaluated comfort noise parameters (f mean and enlogmean) are encoded into a special frame, called a Silence Insertion Descriptor (SID) frame, for transmission to the RX side.

A hangover logic is used to enhance the quality of the silence descriptor frames. A hangover of seven frames is added to the VAD flag so that the coder waits with the switch from active to inactive mode for a period of seven frames; during that time the decoder can compute a silence descriptor frame from the quantized ISFs and the logarithmic frame energy of the decoded speech signal. Therefore, no comfort noise description is transmitted in the first SID frame after active speech. If the background noise contains transients which will cause the coder to switch to active mode and then back to inactive mode in a very short time period, no hangover is used. Instead the previously used comfort noise frames are used for comfort noise generation.

The first SID frame also serves to initiate the comfort noise generation on the receive side, as a first SID frame is always sent at the end of a speech burst, i.e. before the transmission is terminated.

The scheduling of SID or speech frames on the network path is described in Annex B/G.722.2.

### A.4.1	ISF evaluation

The comfort noise parameters to be encoded into a SID frame are calculated over N = 8 consecutive frames marked with VAD = 0, as follows:

Prior to averaging the ISF parameters over the CN averaging period, a median replacement is performed on the set of ISF parameters to be averaged, to remove the parameters which are not characteristic of the background noise on the transmit side. First, the spectral distances from each of the ISF parameter vectors f(i) to the other ISF parameter vectors f(j), i=0,...,7, j=0,...,7, ij, within the CN averaging period are approximated according to the equation:

ΔRij=k=116fik−fjk2	(A-1)

where fi(k) is the kth ISF parameter of the ISF parameter vector f(i) at frame i.

To find the spectral distance ΔSi of the ISF parameter vector f(i) to the ISF parameter vectors f(j) of all the other frames j=0,...,7, ji, within the CN averaging period, the sum of the spectral distances ΔRij  is computed as follows:

ΔSi=j=0,j≠i7ΔRij	(A-2)

for all i=0,...,7, ij.

The ISF parameter vector f(i) with the smallest spectral distance ΔSi of all the ISF parameter vectors within the CN averaging period is considered as the median ISF parameter vector fmed of the averaging period, and its spectral distance is denoted as ΔSmed. The median ISF parameter vector is considered to contain the best representation of the short-term spectral detail of the background noise of all the ISF parameter vectors within the averaging period. If there are ISF parameter vectors f(j) within the CN averaging period with:

ΔSjΔSmed>THmed	(A-3)

where THmed = 2.25 is the median replacement threshold, then at most two of these ISF parameter vectors (the ISF parameter vectors causing THmed to be exceeded the most) are replaced by the median ISF parameter vector prior to computing the averaged ISF parameter vector f mean.

The set of ISF parameter vectors obtained as a result of the median replacement are denoted as f'(n – i), where n is the index of the current frame, and i is the averaging period index (i=0,...,7).

When the median replacement is performed at the end of the hangover period (first CN update), all of the ISF parameter vectors f(n – i) of the 7 previous frames (the hangover period, i=1,...,7) have quantized values, while the ISF parameter vector f(n) at the most recent frame n has unquantized values. In the subsequent CN updates, the ISF parameter vectors of the CN averaging period in the frames overlapping with the hangover period have quantized values, while the parameter vectors of the more recent frames of the CN averaging period have unquantized values. When the period of the eight most recent frames is non-overlapping with the hangover period, the median replacement of ISF parameters is performed using only unquantized parameter values.

The averaged ISF parameter vector f mean(n) at frame n shall be computed according to the equation:

fmeann = 18 i=07f'n − i ,	(A-4)

where f'(n – i) is the ISF parameter vector of one of the eight most recent frames (i=0,...,7) after performing the median replacement, i is the averaging period index, and n is the frame index.

The averaged ISF parameter vector f mean(n) at frame n is quantized using the comfort noise ISF quantization tables The mean removed ISF vector to be quantized is obtained according to the following equation:

r(n) = f mean(n) –f	(A-5)

where f mean(n) is the averaged ISF parameter vector at frame n, f is the constant mean ISF vector, r(n) is the computed ISF mean removed vector at frame n, and n is the frame index.

### A.4.2	Frame energy calculation

The frame energy is computed for each frame marked with VAD = 0 according to the equation:

enlogi = 12 log21Nn=0N−1s2n	(A-6)

where s(n) is the high-pass-filtered input speech signal of the current frame i. The energy is also adjusted according to the signalled speech modes capabilities, as to provide high quality transitions from Comfort Noise to Speech.

The averaged logarithmic energy is computed by:

enlogmeani = 18n=07enlog(i−n)	(A-7)

The averaged logarithmic energy is quantized using a 6-bit arithmetic quantizer. The 6-bits for the energy index are transmitted in the SID frame (see bit allocation in Table A.1).

Table A.1/G.722.2 – Source encoder output parameters in order of occurrence
and bit allocation for comfort noise encoding

| Bits (MSB-LSB) | Description |
| --- | --- |
| s1-s6 | Index of 1st ISF subvector |
| s7-s12 | Index of 2st ISF subvector |
| s13-s18 | Index of 3nd ISF subvector |
| s19-s23 | Index of 4th ISF subvector |
| s24-s28 | Index of 5th ISF subvector |
| s29-s34 | Index of logarithmic frame energy |
| s35 | Dithering flag |

## A.4.3	Analysis of the variation and stationarity of the background noise

The encoder first determines how stationary background noise is. Dithering is employed for non-stationary background noise. The information about whether to use dithering or not is transmitted to the decoder using a binary information (CNdith – flag).

The binary value for the CNdith – flag is found by using the spectral distance ΔSi of the spectral parameter vector f(i) to the spectral parameter vectors f(j) of all the other frames j=0,..., ldtx-1, ji within the CN averaging period (ldtx). The computation of the spectral distance is described in A.4.1. A sum of spectral distances Ds=i=07ΔSiis then computed. If DS is small, CNdith – flag is set to 0. Otherwise, CNdith – flag is set to 1. Additionally, variation of energy between frames is studied. The sum of absolute deviation of enlog(i) from the average enlog is computed. If the sum is large, CNdith – flag is set to 1, even if the flag was earlier set to 0.

### A.4.4	Modification of the speech encoding algorithm during SID frame generation

When the TX_TYPE is not equal to SPEECH, the speech encoding algorithm is modified in the following way:

–	The non-averaged LP parameters which are used to derive the filter coefficients of the filters H(z) and W(z) of the speech encoder are not quantized;

–	The open loop pitch lag search is performed, but the closed loop pitch lag search is inactivated. The adaptive codebook memory is set to zero.

–	No fixed codebook search is made.

–	The memory of weighting filter W(z) is set to zero, i.e. the memory of W(z) is not updated.

–	The ordinary LP parameter quantization algorithm is inactive. The averaged ISF parameter vector f mean is calculated each time a new SID frame is to be sent. This parameter vector is encoded into the SID frame as defined in A.4.1.

–	The ordinary gain quantization algorithm is inactive.

–	The predictor memories of the ordinary LP parameter quantization algorithm is initialized when TX_TYPE is not SPEECH, so that the quantizers start from known initial states when the speech activity begins again.

In the 23.85 kbit/s mode, when the TX_TYPE is equal to SPEECH and VAD is OFF, the speech encoding algorithm is modified in the following way:

–	The generation of high-band gain gHB is changed by adapting it during non-active speech period towards estimated gain in order to ensure smooth transition of high-band gain. gHB is then:

gHB=hangDTX7gHB+(1−hangDTX7)gest	(A-8)

where hangDTX is DTX counter.

### A.4.5	SID-frame encoding

The encoding of the comfort noise bits in a SID frame is described in Annex E/G.722.2 where the indication of the first SID frame is also described. The bit allocation and sequence of the bits from comfort noise encoding is shown in Table A.1.

## A.5	Functions on the receive (RX) side

The situations in which comfort noise shall be generated on the receive side are defined in Annex B/G.722.2. In general, the comfort noise generation is started or updated whenever a valid SID frame is received.

### A.5.1	Averaging and decoding of the LP and energy parameters

When speech frames are received by the decoder the LP and the energy parameters of the last seven speech frames shall be kept in memory. The decoder counts the number of frames elapsed since the last SID frame was updated and passed to the RSS by the encoder. Based on this count, the decoder determines whether or not there is a hangover period at the end of the speech burst (defined in Annex B/G.722.2). The interpolation factor is also adapted to the SID update rate.

As soon as a SID frame is received, comfort noise is generated at the decoder end. The first SID frame parameters are not received but computed from the parameters stored during the hangover period. If no hangover period is detected, the parameters from the previous SID update are used.

The averaging procedure for obtaining the comfort noise parameters for the first SID frame is as follows:

–	When a speech frame is received, the ISF vector is decoded and stored in memory; moreover the logarithmic frame energy of the decoded signal is also stored in memory.

–	The averaged values of the quantized ISF vectors and the averaged logarithmic frame energy of the decoded frames are computed and used for comfort noise generation.

The averaged value of the ISF vector for the first SID frame is given by:

fmeani=18n=07fi−n	(A-9)

where fi−n, n > 0 is the quantized ISF vector of one of the frames of the hangover period and where fi−0 = fi−1. The averaged logarithmic frame energy for the first SID frame is given by:

enlogmeani=18n=07enlogi−n	(A-10)

where ênlog(i – n), n > 0 is the logarithmic vector of one of the frames of the hangover period computed for the decoded frames and where ênlog(i – 0) = ênlog(i – 1).

For ordinary SID frames, the ISF vector and logarithmic frame energy are computed by table lookup. The ISF vector is given by the sum of the decoded reference vector and the constant mean ISF vector.

During comfort noise generation the spectrum and energy of the comfort noise is determined by interpolation between old and new SID frames.

When dithering is used, the ISF vector f is modified by:

f(i) = f(i) + rand(–L(i),L(i))             i = 1,...,16	(A-11)

where L(i) = 100 + 0.8i Hz and rand(L(i),L(i)) is random function generating values between L(i) and L(i). A minimum gap of 175 Hz is ensured between elements of f.

Dithering insertion for energy parameter is similar to spectral dithering and can be computed as follows:

enlogmean=enlogmean+rand(−L,L)	(A-12)

where L = 75 and enlogmean is the energy value used for scaling the energy of the comfort noise excitation.

### A.5.2	Comfort noise generation and updating

The comfort noise generation procedure uses the Adaptive Multi-Rate Wideband (AMR-WB) speech decoder algorithm defined in the main body of ITU-T Rec. G.722.2.

When comfort noise is to be generated, the various encoded parameters are set as follows:

In each subframe, the pulse positions and signs of the excitation are locally generated using uniformly distributed pseudo-random numbers. The excitation pulses take values between +2047 and –2048 when comfort noise is generated. The fixed codebook comfort noise excitation generation algorithm works as follows:

for (i = 0; i < 64; i++)	u[i] = shr(random(),4)

where:

u[0..63]	excitation buffer;

random()	generates a random integer value, uniformly distributed between –32 768 and +32 767;

The excitation gain is computed from the logarithmic frame energy parameter by converting it to the linear domain.

The adaptive codebook gain values in each subframe are set to 0; also, the memory of the adaptive codebook is set to zero.

The pitch delay values in each subframe are set to 64.

The LP filter parameters used are those received in the SID frame.

The predictor memory of the ordinary LP parameter algorithm is initialized when RX_TYPE is not SPEECH , so that the quantizer start from given initial states when the speech activity begins again. With these parameters, the speech decoder now performs the standard operations described in the main body of ITU-T Rec. G.722.2 and synthesizes comfort noise. During CN generation, the high-band generation is performed using estimated high-band gain like in 8.85, 12.65, 14.25, 15.85, 18.25, 19.85 or 23.05 kbit/s modes during active speech.

Updating of the comfort noise parameters (energy and LP filter parameters) occurs each time a valid SID frame is received, as described in Annex B/G.722.2.

When updating the comfort noise, the parameters above should be interpolated over the SID update period to obtain smooth transitions.

## A.6	Computational details and bit allocation

A bit exact computational description of comfort noise encoding and generation in form of an ANSI-C source code is found in Annex C/G.722.2.

The detailed bit allocation and the sequence of bits in the comfort noise encoding is shown in Table A.1.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |