|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.722.2 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Annex F (11/2002) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Coding of analogue signals by methods other than PCM |  |  |  |
|  | Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB) Annex F: AMR-WB usage in H.245 |  |  |  |
|  | ITU-T Recommendation G.722.2 – Annex F |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.722.2 Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB) Annex F AMR-WB usage in H.245

Summary This annex specifies the packet format and capability signalling for G.722.2 Adaptive Multi-Rate Wideband (AMR-WB) for usage with H.245 specification. Both the format and the capability parameters are fully compatible with the corresponding AMR-WB RTP definitions to allow seamless interoperability.

Source Annex F to ITU-T Recommendation G.722.2 was prepared by ITU-T Study Group 16 (2001-2004) and approved under the WTSA Resolution 1 procedure on 29 November 2002.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2003

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

F.1	Scope		1

F.2	References		1

F.3	Introduction		1

F.4	ITU-T Rec. G.722.2 (Adaptive Multi-Rate Wideband) capability definitions for H.245		1

ITU-T Recommendation G.722.2

Wideband coding of speech at around 16 kbit/s using
Adaptive Multi-Rate Wideband (AMR-WB)

Annex F

AMR-WB usage in H.245

## F.1	Scope

This annex specifies the packet format and capability signalling for G.722.2 Adaptive Multi-Rate Wideband (AMR-WB) [3] for usage with H.245 specification. Both the format and the capability parameters are fully compatible with the corresponding AMR-WB RTP definitions to allow seamless interoperability.

## F.2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[1]	SJOBERG (J.) et al., Real-Time Transport Protocol (RTP) Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs, RFC 3267, June 2002.

[2]	ITU-T Recommendation H.245 (2003), Control protocol for multimedia communication.

[3]	ITU-T Recommendation G.722.2 (2002), Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB).

## F.3	Introduction

The packet format and corresponding capability signalling for the Adaptive Multi-Rate Wideband (AMR-WB) speech codec within Real-Time Transport Protocol (RTP) is specified in [1]. This packet format is not merely a way to encapsulate AMR-WB speech frames into the payload of the RTP packet but also introduces a number of optional packetization features for enhanced robustness against transmission errors. Furthermore, also a set of parameters for out-of-band signalling of packetizer capabilities is specified.

This annex specifies the usage of the packet format defined in [1] and provides corresponding capability definitions for usage with H.245. This approach enables one-to-one mapping between H.245 and a system using parameters specified in [1] (e.g. MIME signalling) thus providing full interoperability.

## F.4	ITU-T Rec. G.722.2 (Adaptive Multi-Rate Wideband) capability definitions for H.245

Table F.1 defines the capability identifier for G.722.2 (AMR-WB) capabilities. Tables F.2 to F.10 define the associated capability parameters. The details of the packetization of speech frames of AMR-WB into the octet structure and mode signalling are specified in [1]. Because of the request mode procedure provided by H.245 [2], the in-band mode request mechanism specified in [1] shall not be used, and the CMR field of the packet shall always be set to value 15 indicating "no request".

Table F.1/G.722.2  G.722.2 Capability identifier

| Capability name: | G.722.2 (AMR-WB) audio capability |
| --- | --- |
| Capability class: | Audio codec |
| Capability identifier type: | Standard |
| Capability identifier value: | {itu-t (0) recommendation (0) g (7) 7222 generic-capabilities (1) 0} |
| maxBitRate: | Shall be 239 |
| NonCollapsingRaw: | This field shall not be included |
| transport: | This field shall not be included |

Table F.2/G.722.2  G.722.2 Capability parameter – maxAl-sduFrames

| Parameter name: | maxAl-sduFrames |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. It specifies what is the maximum number of audio frames per AL-SDU. |
| Parameter identifier value: | 0 |
| Parameter status: | Shall be present for capability exchange and logical channel signalling. Shall not be present for mode request. |
| Parameter type: | UnsignedMin |
| Supersedes: |  |

Table F.3/G.722.2  G.722.2 Capability parameter – bitRate

| Parameter name: | bitRate |
| --- | --- |
| Parameter description: | This is a nonCollapsing GenericParameter. It specifies the AMR bit-rate. This parameter shall be used in mode requests and in mode-set definitions (see Table F.5). 0 = 6.60; 1 = 8.85; 2 = 12.65; 3 = 14.25; 4 = 15.85; 5 =18.25; 6 = 19.85; 7 = 23.05; 8 = 23.85 |
| Parameter identifier value: | 1 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | – |

Table F.4/G.722.2 – G.722.2 Capability parameter – octet align

| Parameter name: | octetAlign |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. It specifies whether the bandwidth efficient or octet-aligned mode of operation is used. If parameter is present and the value is TRUE, octet-aligned mode is used, otherwise the bandwidth efficient mode is used. |
| Parameter identifier value: | 2 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: |  |

Table F.5/G.722.2 – G.722.2 Capability parameter – mode-set

| Parameter name: | modeSet |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. If present, it specifies the set of modes available for current session. If not present, all modes are available. |
| Parameter identifier value: | 3 |
| Parameter status: | Optional |
| Parameter type: | genericParameter. Parameter values (SET OF unsignedMin) indicate the G.722.2 modes (see Table F.3) supported for current session. |
| Supersedes: |  |

Table F.6/G.722.2 – G.722.2 Capability parameter – mode change period

| Parameter name: | modeChangePeriod |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. If present, it specifies the interval N (as number of frames) at which the mode changes are allowed. The initial phase is arbitrary, but the mode changes must be separated by the multiple of N frames. If not present, mode changes are allowed at any time. |
| Parameter identifier value: | 4 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: | – |

Table F.7/G.722.2 – G.722.2 Capability parameter – mode change neighbour

| Parameter name: | modeChangeNeighbour |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. If present and the value is TRUE, it specifies that the mode may be changed to a neighbouring mode within the specified mode-set. If not present or the value is FALSE, mode changes are allowed to any mode within the specified mode-set. |
| Parameter identifier value: | 5 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | – |

Table F.8/G.722.2 – G.722.2 Capability parameter – crc

| Parameter name: | crc |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. If present and the value is TRUE, it specifies that frame CRCs are included in the payload packet. If not present or the value is FALSE, CRCs are not included. Note that enabling CRC also implies usage of octet-aligned mode of operation. |
| Parameter identifier value: | 6 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | – |

Table F.9/G.722.2 – G.722.2 Capability parameter – robust sorting

| Parameter name: | robustSorting |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. If present and the value is TRUE, it specifies that the robust sorting is applied on the frames carried in the payload. If not present or the value is FALSE, simple payload sorting is used. Note that enabling robust sorting also implies usage of octet-aligned mode of operation. |
| Parameter identifier value: | 7 |
| Parameter status: | Optional |
| Parameter type: | Logical |
| Supersedes: | – |

Table F.10/G.722.2  G.722.2 Capability parameter – interleaving

| Parameter name: | interleaving |
| --- | --- |
| Parameter description: | This is a collapsing GenericParameter. If present, it specifies that frame level interleaving shall be applied for the payloads of this session, and the value specifies the maximum number of frames in an interleaving group. If not present, frames in the payload are not interleaved. Note that enabling interleaving also implies usage of octet-aligned mode of operation. |
| Parameter identifier value: | 8 |
| Parameter status: | Optional |
| Parameter type: | UnsignedMin |
| Supersedes: |  |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |