|  | # COVERING NOTE |
| --- | --- |

## general secretariat international telecommunication union

Geneva,  29 July 2003

### ITU – TELECOMMUNICATION
STANDARDIZATION  SECTOR

Subject:	Amendment 1 (07/2003) to

ITU-T  Recommendation G.722.2 Appendix I (01/2002), Wideband coding of speech at around 16 kbit/s using Adaptive Multi-Rate Wideband (AMR-WB) – Appendix I: Error concealment of erroneous or lost frames

# 1)	In clause I.5.2.3.4.2, correct the last but one line as follows (change "second" into "third"):

Tmax−2is second third largest value in Tbuffer

# 2)	Add a new clause I.5.2.5 as follows:

#### I.5.2.5	High-band gain (for 23.85 kbit/s mode)

When RX_FRAMETYPE = SPEECH_BAD or RX_FRAMETYPE = SPEECH_LOST, the received high-band energy parameter of the frame is not used and the estimation for the high-band gain is used instead. This means that in case of bad/lost speech frames, the high-band reconstruction operates in the same way for all the modes.