|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.723.1 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | Annex C (11/96) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA

Digital transmission systems – Terminal equipments – Coding of analogue signals by methods other than PCM

Dual rate speech coder for multimedia communications transmitting at 5.3 and 6.3 kbit/s

Annex C: Scalable channel coding scheme for wireless applications

ITU-T  Recommendation  G.723.1  –  Annex C

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600-G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| General aspects | G.800–G.809 |
| Design objectives for digital networks | G.810–G.819 |
| Quality and availability targets | G.820–G.829 |
| Network capabilities and functions | G.830–G.839 |
| SDH network characteristics | G.840–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| General | G.900–G.909 |
| Parameters for optical fibre cable systems | G.910–G.919 |
| Digital sections at hierarchical bit rates based on a bit rate of 2048 kbit/s | G.920–G.929 |
| Digital line transmission systems on cable at non-hierarchical bit rates | G.930–G.939 |
| Digital line systems provided by FDM transmission bearers | G.940–G.949 |
| Digital line systems | G.950–G.959 |
| Digital section and digital transmission systems for customer access to ISDN | G.960–G.969 |
| Optical fibre submarine cable systems | G.970–G.979 |
| Optical line systems for local and access networks | G.980–G.999 |

ITU-T RECOMMENDATION G.723.1 – Annex C scalable channel coding scheme for wireless applications

Source Annex C to ITU-T Recommendation G.723.1, was prepared by ITU-T Study Group 15 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 8th of November 1996.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

  ITU  1997

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

C.1	Introduction		1

C.1.1	Scope		1

C.1.2	Bit-rates		1

C.1.3	Delay		1

C.1.4	Channel codec description		1

C.1.5	Interface to control H.245		1

C.2	Channel encoder		2

C.2.1	Channel encoder adaptation for active speech modes		2

C.2.2	Bit sensitivity classes for active speech modes		6

C.2.3	Bit sensitivity classes for SID frames		8

C.2.4	CRC encoder		9

C.2.5	Convolutional encoder		9

C.3	Channel decoder		13

C.3.1	Decoding of configuration bits		13

C.3.2	Speech decoder adaptation for active speech modes		13

C.3.3	Error analysis		14

C.3.4	Speech parameter format		14

C.3.5	Error concealment		16

C.3.6	Performance of error analysis flags		17

C.4	Fixed point C source code		17

Recommendation G.723.1 – Annex C

SCALABLE CHANNEL CODING SCHEME FOR WIRELESS APPLICATIONS

(Geneva, 1996)

## C.1	Introduction

### C.1.1	Scope

This Annex specifies a channel coding scheme which can be used with the triple rate speech codec G.723.1. The channel codec is scalable in bit-rate and is designed for mobile multimedia applications as a part of the overall H.324 family of standards. With this Annex, G.723.1 can be adapted to any wireline or wireless transmission system. Transmission system dependent functionalities like interleaving or burst formatting are not subject of this Annex.

### C.1.2	Bit-rates

A wide range of channel codec bit-rates is supported ranging from 0.7 kbit/s up to 14.3 kbit/s. The channel codec supports all three operational modes of the G.723.1 codec, namely high rate, low rate and discontinuous transmission modes.

The applied channel codec bit-rates depend on the transmission system and the application and are therefore not specified in this Annex.

### C.1.3	Delay

No additional delay is introduced by the channel encoder and decoder described in this Annex.

### C.1.4	Channel codec description

The channel codec is based on punctured convolutional codes. The possibility to generate multiple rates from a given mother code is the basis for the scalability feature. Based on the subjective importance of each class of information bits, the available channel codec bit-rate is allocated optimally to the bit classes. This allocation is based on an algorithm which has to be known by encoder and decoder. Each time the system control signals either a change in the G.723.1 rate or in the available channel codec bit-rate, this algorithm is executed to adapt the channel codec to the new speech service configuration.

If only a very low channel codec bit-rate is available, the subjectively most sensitive bits are protected first. When increasing the channel codec bit-rate, the additional bits are used first to protect more information bits and second to increase the protection of the already protected classes.

A special configuration of the channel codec enables a robust G.723.1 transmission without the use of convolutional codes but with only an error detection code for the subjectively most sensitive bits. The channel codec bit-rate for this configuration is 667 bits per second.

Prior to the application of the channel encoding functions, the speech parameters are partly modified in a channel adaptation layer to improve their robustness against transmission errors.

Two parameters are sufficient to configure and control the channel codec:

1)	G.723.1 configuration bit, signalling the operational mode of the speech encoder;

2)	Channel codec configuration bits, signalling the channel codec bit-rate to apply.

### C.1.5	Interface to control H.245

The channel codec bit-rate available for the speech service is provided by the control H.245 using one channel codec configuration bits. Both, channel codec configuration bit and G.723.1

configuration bits are protected and transmitted together with the protected information bitstream to the decoder. Prior to the decoding of the information bitstream, the configuration bits are decoded and channel and speech decoder are configured accordingly.

## C.2	Channel encoder

### C.2.1	Channel encoder adaptation for active speech modes

This clause describes the transformation of the original G.723.1 speech parameters to a modified bitstream with increased robustness to transmission errors.

#### C.2.1.1	Gain index decompression

The format of the speech parameters is specified in Tables 5 and 6 of Recommendation G.723.1. The 12-bit overall gain index (GAINx_By) of each subframe is a compressed index of two individual gain indices. These indices shall be decompressed as it is described in the speech decoding part (subclauses 2.17 and 2.18 of Recommendation G.723.1). This results in one 8-bit (AGAINx_By) and one 5-bit (FGAINx_By) gain index per subframe, requiring one additional bit compared to the compressed index. The overhead introduced by this decompression operation shall be exploited in the channel decoder for error detection and error concealment purposes.

#### C.2.1.2	LPC reordering

The 24-bit LPC index (LPC_Bx, x=0,23) consists of three 8 bit sub-vectors as described in 2.6 of Recommendation G.723.1:

e0=LPCB7,LPCB6,LPCB5,LPCB4,LPCB3,LPCB1,LPCB0

e1=LPCB15,LPCB14,LPCB13,LPCB12,LPCB11,LPCB10,LPCB9,LPCB8

e2=LPCB23,LPCB22,LPCB21,LPCB20,LPCB19,LPCB18,LPCB17,LPCB16

The sub-vectors shall be reordered applying the mapping:

emR=ReorderTabmemm=0,1,2

The LPC reordering tables are given in Table C.1a for the first sub-vector, C.1b for the second and C.1c for the third sub-vector. The entry (i,j) to the table is determined by: . The output obtained is the transmitted index of the sub-vector.

Table C.1a/G.723.1

Reordering table for m=0 (ReorderTab0[e0])

|  | i=0 | i=1 | i=2 | i=3 | i=4 | i=5 | i=6 | i=7 | i=8 | i=9 | i=10 | i=11 | i=12 | i=13 | i=14 | i=15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| j=0 | 82 | 91 | 190 | 191 | 189 | 36 | 187 | 32 | 38 | 39 | 185 | 112 | 166 | 175 | 116 | 120 |
| j=1 | 34 | 35 | 122 | 41 | 40 | 43 | 42 | 56 | 57 | 90 | 168 | 85 | 74 | 170 | 234 | 174 |
| j=2 | 169 | 172 | 178 | 182 | 184 | 179 | 181 | 180 | 37 | 186 | 44 | 33 | 159 | 183 | 188 | 155 |
| j=3 | 253 | 252 | 147 | 154 | 246 | 165 | 218 | 139 | 63 | 160 | 157 | 146 | 145 | 144 | 177 | 176 |
| j=4 | 244 | 131 | 148 | 129 | 128 | 161 | 219 | 135 | 34 | 203 | 200 | 206 | 207 | 204 | 205 | 254 |
| j=5 | 212 | 222 | 213 | 220 | 221 | 141 | 216 | 88 | 38 | 137 | 136 | 217 | 133 | 132 | 201 | 197 |
| j=6 | 196 | 76 | 77 | 243 | 192 | 195 | 193 | 119 | 18 | 108 | 121 | 249 | 247 | 245 | 130 | 240 |
| j=7 | 241 | 235 | 69 | 68 | 210 | 226 | 227 | 224 | 25 | 79 | 211 | 250 | 251 | 255 | 198 | 242 |
| j=8 | 194 | 127 | 45 | 46 | 117 | 125 | 124 | 2 | 67 | 3 | 101 | 72 | 0 | 13 | 97 | 65 |
| j=9 | 66 | 98 | 96 | 158 | 248 | 228 | 229 | 93 | 31 | 109 | 100 | 110 | 111 | 75 | 73 | 115 |
| j=10 | 114 | 23 | 94 | 95 | 6 | 22 | 7 | 230 | 5 | 4 | 99 | 81 | 20 | 21 | 83 | 113 |
| j=11 | 102 | 14 | 103 | 15 | 12 | 16 | 52 | 17 | 19 | 1 | 29 | 8 | 28 | 30 | 31 | 11 |
| j=12 | 9 | 126 | 24 | 18 | 26 | 25 | 27 | 86 | 87 | 64 | 71 | 70 | 153 | 152 | 173 | 162 |
| j=13 | 167 | 164 | 89 | 80 | 238 | 208 | 232 | 233 | 15 | 236 | 239 | 92 | 202 | 209 | 199 | 143 |
| j=14 | 142 | 140 | 223 | 84 | 78 | 171 | 151 | 156 | 37 | 150 | 149 | 214 | 106 | 107 | 10 | 53 |
| j=15 | 47 | 54 | 55 | 104 | 105 | 123 | 48 | 58 | 59 | 49 | 50 | 51 | 62 | 60 | 61 | 63 |

Table C.1b/G.723.1

Reordering table for m=1 (ReorderTab1[e1])

|  | i=0 | i=1 | i=2 | i=3 | i=4 | i=5 | i=6 | i=7 | i=8 | i=9 | i=10 | i=11 | i=12 | i=13 | i=14 | i=15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| j=0 | 122 | 150 | 201 | 213 | 214 | 212 | 246 | 244 | 136 | 135 | 130 | 134 | 140 | 216 | 218 | 146 |
| j=1 | 209 | 208 | 110 | 153 | 156 | 158 | 152 | 144 | 147 | 145 | 199 | 197 | 193 | 204 | 205 | 249 |
| j=2 | 248 | 127 | 185 | 172 | 174 | 173 | 179 | 169 | 168 | 181 | 182 | 177 | 183 | 161 | 178 | 167 |
| j=3 | 163 | 162 | 160 | 165 | 232 | 234 | 235 | 233 | 237 | 236 | 238 | 239 | 78 | 155 | 154 | 75 |
| j=4 | 74 | 69 | 68 | 71 | 70 | 73 | 72 | 139 | 123 | 114 | 59 | 58 | 113 | 170 | 137 | 141 |
| j=5 | 186 | 133 | 151 | 132 | 148 | 159 | 196 | 149 | 187 | 189 | 191 | 157 | 198 | 190 | 121 | 126 |
| j=6 | 125 | 120 | 57 | 112 | 171 | 63 | 45 | 62 | 47 | 46 | 99 | 109 | 98 | 103 | 124 | 102 |
| j=7 | 107 | 106 | 129 | 128 | 131 | 89 | 118 | 105 | 104 | 95 | 91 | 90 | 94 | 86 | 83 | 82 |
| j=8 | 138 | 119 | 117 | 88 | 116 | 92 | 37 | 36 | 38 | 32 | 96 | 87 | 81 | 39 | 93 | 49 |
| j=9 | 48 | 23 | 22 | 21 | 20 | 29 | 8 | 56 | 55 | 44 | 175 | 60 | 61 | 53 | 43 | 143 |
| j=10 | 115 | 176 | 180 | 142 | 166 | 184 | 188 | 207 | 206 | 231 | 252 | 195 | 194 | 219 | 217 | 221 |
| j=11 | 223 | 211 | 215 | 210 | 192 | 200 | 164 | 226 | 202 | 203 | 230 | 227 | 229 | 253 | 255 | 224 |
| j=12 | 225 | 243 | 228 | 220 | 240 | 241 | 242 | 222 | 245 | 251 | 250 | 247 | 254 | 111 | 108 | 77 |
| j=13 | 76 | 79 | 101 | 100 | 30 | 15 | 97 | 52 | 54 | 41 | 50 | 51 | 40 | 33 | 42 | 35 |
| j=14 | 34 | 13 | 12 | 14 | 85 | 84 | 25 | 31 | 27 | 80 | 17 | 67 | 66 | 16 | 9 | 28 |
| j=15 | 24 | 65 | 11 | 10 | 26 | 64 | 19 | 18 | 4 | 1 | 0 | 5 | 7 | 6 | 3 | 2 |

Table C.1c/G.723.1

Reordering table for m=2 (ReorderTab2[e2])

|  | i=0 | i=1 | i=2 | i=3 | i=4 | i=5 | i=6 | i=7 | i=8 | i=9 | i=10 | I=11 | i=12 | i=13 | i=14 | i=15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| j=0 | 243 | 154 | 190 | 170 | 189 | 169 | 140 | 188 | 171 | 168 | 166 | 164 | 165 | 216 | 146 | 132 |
| j=1 | 242 | 234 | 158 | 144 | 252 | 147 | 148 | 13 | 74 | 75 | 19 | 77 | 76 | 79 | 78 | 70 |
| j=2 | 71 | 198 | 199 | 72 | 12 | 73 | 220 | 150 | 151 | 131 | 204 | 133 | 205 | 230 | 229 | 225 |
| j=3 | 200 | 201 | 226 | 227 | 196 | 192 | 197 | 195 | 202 | 203 | 136 | 143 | 142 | 175 | 207 | 206 |
| j=4 | 137 | 138 | 139 | 174 | 173 | 172 | 240 | 228 | 209 | 224 | 238 | 239 | 128 | 130 | 145 | 250 |
| j=5 | 236 | 237 | 255 | 248 | 253 | 232 | 153 | 179 | 181 | 180 | 235 | 233 | 152 | 156 | 184 | 178 |
| j=6 | 185 | 162 | 183 | 182 | 163 | 160 | 161 | 167 | 157 | 159 | 155 | 134 | 186 | 191 | 135 | 187 |
| j=7 | 141 | 254 | 177 | 251 | 176 | 33 | 244 | 245 | 61 | 60 | 34 | 35 | 32 | 249 | 126 | 41 |
| j=8 | 62 | 59 | 58 | 63 | 57 | 56 | 42 | 43 | 25 | 29 | 40 | 125 | 120 | 24 | 26 | 127 |
| j=9 | 123 | 124 | 122 | 48 | 121 | 46 | 47 | 105 | 104 | 109 | 108 | 28 | 116 | 112 | 113 | 247 |
| j=10 | 246 | 36 | 54 | 149 | 55 | 4 | 52 | 53 | 49 | 37 | 39 | 51 | 50 | 107 | 211 | 215 |
| j=11 | 20 | 21 | 17 | 3 | 5 | 223 | 222 | 0 | 231 | 129 | 218 | 9 | 217 | 219 | 194 | 221 |
| j=12 | 193 | 210 | 38 | 2 | 1 | 212 | 14 | 15 | 241 | 44 | 208 | 8 | 45 | 11 | 10 | 110 |
| j=13 | 68 | 69 | 103 | 100 | 213 | 214 | 16 | 106 | 18 | 111 | 22 | 23 | 6 | 7 | 64 | 65 |
| j=14 | 27 | 118 | 117 | 102 | 31 | 30 | 97 | 114 | 115 | 96 | 88 | 89 | 90 | 91 | 99 | 94 |
| j=15 | 80 | 82 | 83 | 119 | 98 | 101 | 92 | 95 | 66 | 67 | 84 | 93 | 85 | 81 | 86 | 87 |

#### C.2.1.3	Channel adapted bitstream format

The speech parameter format of the high and low bit-rate modes after gain decompression and LPC reordering is given in Tables C.2a and C.2b (UB means "unused bit").

Table C.2a/G.723.1

Format of decompressed high rate codec speech parameters

| Channel Encoder Octets | PARx_By |
| --- | --- |
| 1 | R_LPC_B5...R_LPC_B0, VAD, RATE |
| 2 | R_LPC_B13...R_LPC_B6 |
| 3 | R_LPC_B21...R_LPC_B14 |
| 4 | ACL0_B5...ACL0_B0, R_LPC_B23, R_LPC_B22 |
| 5 | ACL2_B4...ACL2_B0, ACL1_B1, ACL1_B0, ACL0_B6 |
| 6 | AGAIN0_B3...AGAIN0_B0, ACL3_B1, ACL3_B0, ACL2_B6, ACL2_B5 |
| 7 | AGAIN1_B3...AGAIN1_B0, AGAIN0_B7...AGAIN0_B4 |
| 8 | AGAIN2_B3...AGAIN2_B0, AGAIN1_B7...AGAIN1_B4 |
| 9 | AGAIN3_B3...AGAIN3_B0, AGAIN2_B7...AGAIN2_B4 |
| 10 | FGAIN0_B3...FGAIN0_B0, AGAIN3_B7...AGAIN3_B4 |
| 11 | FGAIN2_B1, FGAIN2_B0, FGAIN1_B4...FGAIN1_B0, FGAIN0_B4 |
| 12 | FGAIN3_B4...FGAIN3_B0, FGAIN2_B4...FGAIN2_B2 |
| 13 | MSBPOS_B3...MSBPOS_B0, GRID3_B0, GRID2_B0, GRID1_B0, GRID0_B0 |
| 14 | MSBPOS_B11...MSBPOS_B4 |
| 15 | POS0_B6...POS0_B0, MSBPOS_B12 |
| 16 | POS0_B14...POS0_B7 |
| 17 | POS1_B6...POS1_B0, POS0_B15 |
| 18 | POS2_B0, POS1_B13...POS1_B7 |
| 19 | POS2_B8...POS2_B1 |
| 20 | POS3_B0, POS2_B15...POS2_B9 |
| 21 | POS3_B8...POS3_B1 |
| 22 | PSIG0_B2...PSIG0_B0, POS3_B13...POS3_B9 |
| 23 | PSIG1_B4...PSIG1_B0, PSIG0_B5...PSIG0_B3 |
| 24 | PSIG3_B1, PSIG3_B0, PSIG2_B5...PSIG2_B0 |
| 25 | UB,UB,UB,UB,UB,PSIG3_B4...PSIG3_B2 |

Table C.2b/G.723.1

Format of decompressed low rate codec speech parameters

| Channel Encoder Octets | PARx_By |
| --- | --- |
| 1 | R_LPC_B5...R_LPC_B0, VAD, RATE |
| 2 | R_LPC_B13...R_LPC_B6 |
| 3 | R_LPC_B21...R_LPC_B14 |
| 4 | ACL0_B5...ACL0_B0, R_LPC_B23, R_LPC_B22 |
| 5 | ACL2_B4...ACL2_B0, ACL1_B1, ACL1_B0, ACL0_B6 |
| 6 | AGAIN0_B3...AGAIN0_B0, ACL3_B1, ACL3_B0, ACL2_B6, ACL2_B5 |
| 7 | AGAIN1_B3...AGAIN1_B0 , AGAIN0_B7...AGAIN0_B4 |
| 8 | AGAIN2_B3...AGAIN2_B0 , AGAIN1_B7...AGAIN1_B4 |
| 9 | AGAIN3_B3...AGAIN3_B0 , AGAIN2_B7...AGAIN2_B4 |
| 10 | FGAIN0_B3...FGAIN0_B0 , AGAIN3_B7...AGAIN3_B4 |
| 11 | FGAIN2_B1 , FGAIN2_B0 , FGAIN1_B4...FGAIN1_B0 , FGAIN0_B4 |
| 12 | FGAIN3_B4...FGAIN3_B0 , FGAIN2_B4...FGAIN2_B2 |
| 13 | POS0_B3...POS0_B0 , GRID3_B0, GRID2_B0, GRID1_B0, GRID0_B0 |
| 14 | POS0_B11...POS0_B8 , POS0_B7...POS0_B4 |
| 15 | POS1_B7...POS1_B4 , POS1_B3...POS1_B0, |
| 16 | POS2_B3...POS2_B0 , POS1_B11...POS1_B8 |
| 17 | POS2_B11...POS2_B8 , POS2_B7...POS2_B4 |
| 18 | POS3_B7...POS3_B4 , POS3_B3...POS3_B0 |
| 19 | PSIG0_B3...PSIG0_B0 , POS3_B11...POS3_B8 |
| 20 | PSIG2_B3...PSIG2_B0 , PSIG1_B3...PSIG1_B0 |
| 21 | UB . UB , UB , UB , PSIG3_B3...PSIG3_B0 |

### C.2.2	Bit sensitivity classes for active speech modes

The bits of the decompressed speech parameters specified in Tables C.2a and C.2b are rearranged according to Table C.3a for the high bit-rate mode and Table C.3b for the low bit-rate mode.

The numbers indicate the position of the corresponding information bit in the ordered bitstream. The subjectively most sensitive bit is copied to position "0" of the ordered bitstream.

The ordered bitstream is denoted with:

i(0), i(0), … i(92)

for the high rate codec, and:

i(0), i(1), ...i(162)

for the low rate codec.

Table C.3a/G.723.1

High rate codec bit ordering table

| Octet | Bit position in ordered bitstream |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 175 | 180 | 189 | 190 | 191 | 192 | VAD | RATE |
| 2 | 98 | 73 | 107 | 154 | 167 | 168 | 169 | 170 |
| 3 | 30 | 17 | 16 | 31 | 48 | 55 | 49 | 71 |
| 4 | 6 | 4 | 0 | 2 | 11 | 26 | 10 | 14 |
| 5 | 5 | 1 | 3 | 12 | 27 | 24 | 60 | 8 |
| 6 | 44 | 66 | 62 | 82 | 25 | 61 | 9 | 7 |
| 7 | 45 | 67 | 63 | 83 | 78 | 36 | 50 | 40 |
| 8 | 46 | 68 | 64 | 84 | 79 | 37 | 51 | 41 |
| 9 | 47 | 69 | 65 | 85 | 80 | 38 | 52 | 42 |
| 10 | 56 | 99 | 159 | 185 | 81 | 39 | 53 | 43 |
| 11 | 161 | 187 | 20 | 57 | 100 | 160 | 186 | 19 |
| 12 | 22 | 59 | 102 | 162 | 188 | 21 | 58 | 101 |
| 13 | 35 | 54 | 70 | 72 | 179 | 178 | 177 | 176 |
| 14 | 13 | 15 | 23 | 28 | 29 | 32 | 33 | 34 |
| 15 | 128 | 132 | 146 | 155 | 163 | 171 | 181 | 18 |
| 16 | 76 | 86 | 90 | 94 | 103 | 108 | 112 | 116 |
| 17 | 129 | 133 | 147 | 156 | 164 | 172 | 182 | 74 |
| 18 | 183 | 87 | 91 | 95 | 104 | 109 | 113 | 117 |
| 19 | 114 | 118 | 130 | 134 | 148 | 157 | 165 | 173 |
| 20 | 184 | 75 | 77 | 88 | 92 | 96 | 105 | 110 |
| 21 | 115 | 119 | 131 | 135 | 149 | 158 | 166 | 174 |
| 22 | 136 | 124 | 120 | 89 | 93 | 97 | 106 | 111 |
| 23 | 151 | 141 | 137 | 125 | 121 | 144 | 150 | 140 |
| 24 | 127 | 123 | 145 | 152 | 142 | 138 | 126 | 122 |
| 25 | UB | UB | UB | UB | UB | 153 | 143 | 139 |

Table C.3b/G.723.1

Low rate codec bit ordering table

| Octet | Bit position in ordered bitstream |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 152 | 153 | 158 | 159 | 160 | 161 | VAD | RATE |
| 2 | 69 | 64 | 70 | 91 | 145 | 140 | 147 | 146 |
| 3 | 24 | 15 | 14 | 25 | 46 | 50 | 47 | 63 |
| 4 | 4 | 6 | 0 | 2 | 11 | 18 | 10 | 13 |
| 5 | 7 | 1 | 3 | 12 | 19 | 16 | 48 | 8 |
| 6 | 42 | 59 | 55 | 65 | 17 | 49 | 9 | 5 |
| 7 | 43 | 60 | 56 | 66 | 26 | 30 | 34 | 38 |
| 8 | 44 | 61 | 57 | 67 | 27 | 31 | 35 | 39 |
| 9 | 45 | 62 | 58 | 68 | 28 | 32 | 36 | 40 |
| 10 | 51 | 87 | 141 | 154 | 29 | 33 | 37 | 41 |
| 11 | 143 | 156 | 21 | 52 | 88 | 142 | 155 | 20 |
| 12 | 23 | 54 | 90 | 144 | 157 | 22 | 53 | 89 |
| 13 | 100 | 128 | 96 | 104 | 151 | 150 | 149 | 148 |
| 14 | 132 | 112 | 116 | 136 | 108 | 120 | 124 | 92 |
| 15 | 109 | 121 | 125 | 93 | 101 | 129 | 97 | 105 |
| 16 | 102 | 130 | 98 | 106 | 133 | 113 | 117 | 137 |
| 17 | 134 | 114 | 118 | 138 | 110 | 122 | 126 | 94 |
| 18 | 111 | 123 | 127 | 95 | 103 | 131 | 99 | 107 |
| 19 | 83 | 79 | 75 | 71 | 135 | 115 | 119 | 139 |
| 20 | 85 | 81 | 77 | 73 | 84 | 80 | 76 | 72 |
| 21 | UB | UB | UB | UB | 86 | 82 | 78 | 74 |

### C.2.3	Bit sensitivity classes for SID frames

The format of the SID frame type is specified in Annex A of Recommendation G.723.1. The bits of this frame are rearranged according to Table C.3c. For SID frames neither a gain decompression nor a LPC reordering is performed. The numbers indicate the position of the corresponding information bit in the ordered bitstream. The subjectively most sensitive bit is copied to position "0" of the ordered bitstream.

The ordered bitstream is denoted with:

i(0), i(1), ...i(29)

Table C.3c/G.723.1

Bit ordering table for SID frames

| Octet | Bit position in ordered bitstream |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 21 | 22 | 23 | 24 | 25 | 26 | VAD | RATE |
| 2 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 19 |
| 3 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| 4 | 0 | 1 | 2 | 29 | 28 | 27 | 3 | 4 |

### C.2.4	CRC encoder

The subjectively most sensitive bits are protected by five parity bits used for error detection in the channel decoder.

The first  bits of the ordered bitstream are within the CRC window:

Low rate codec:   = 34

High rate codec:  = 44

SID frames:         = 30

The following generator polynomial for the cyclic code shall be applied:

g(D)=D5+D2+1.

The five parity bits p(0) - p(4) are copied right after the  ordered information bits.

The bitstream after CRC encoding shall therefore have the following format:

Low rate codec:		bs(n) = i(0),i(1), ...i(33),p(0),p(1),p(2),p(3),p(4),i(34),i(35), ... i(161); n=0 ... 166

High rate codec:		bs(n) = i(0),i(1), ...i(43),p(0),p(1),p(2),p(3),p(4),i(44),i(45), ... i(192); n=0 ... 197

SID frame:			bs(n) = i(0),i(1), ...i(29),p(0),p(1),p(2)p(3),p(4); n=0 ... 34

### C.2.5	Convolutional encoder

The information bits bs(n) shall be encoded with punctured convolutional codes defined by the mother polynomials:

gCC0=D4+D+1gCC1=D4+D3+D2+1gCC2=D4+D2+D+1

Puncturing of the mother code with a puncturing period of 12 generates 24 intermediate rates:

1236≤r≤1213

The puncturing matrices are given in Table C.4.

Table C.4/G.723.1

Puncturing tables (all values in hexadecimal representation)

| Rate r | 12/13 | 12/14 | 12/15 | 12/16 | 12/17 | 12/18 | 12/19 | 12/20 | 12/21 | 12/22 | 12/23 | 12/24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (0) | D6F | D7F | D7F | D7F | DFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF |
| (1) | 690 | 690 | 691 | 695 | 695 | 695 | 69D | 6DD | 6DF | 7DF | 7FF | FFF |
| (2) | 000 | 000 | 000 | 000 | 000 | 000 | 000 | 000 | 000 | 000 | 000 | 000 |

| Rate r | 12/25 | 12/26 | 12/27 | 12/28 | 12/29 | 12/30 | 12/31 | 12/32 | 12/33 | 12/34 | 12/35 | 12/36 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (0) | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF |
| (1) | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF | FFF |
| (2) | 001 | 009 | 109 | 309 | 329 | 729 | 72D | 72F | 7AF | 7BF | 7FF | FFF |

#### C.2.5.1	Allocation of channel codec bit-rate

In this clause the resource allocation algorithm is described which shall be applied for determining the degree of protection for each information bit. This algorithm has to be executed in the channel encoder and channel decoder each time at least one of the following events occur:

•	Reset.

•	Change of the channel codec bit-rate allocated for G.723.1.

•	Change of the G.723.1 source codec bit-rate.

The following rate allocation procedure shall be executed:

•	The ordered bitstream shall be divided in five-bit classes c[i], i=0...4, where c[0] contains the most sensitive and c[4] the least sensitive bits.

•	A weighting factor w[i], i=0...4 shall be assigned to each class, controlling the rate allocation algorithm.

•	Class 0 contains c[0]-5 information bits and the five parity check code bits which are copied to the end of that class.

•	If no channel bits () are available the channel bitstream shall contain the decompressed G.723.1 bitstream and the five parity check bits.

•	The maximum number of channel codec bits per frame is achieved, when all classes have the protection rate r = 1/3

Bmax=2⋅i=0i=4ci+4

•	If:

B≤c0+8

channel codec bits are available per frame these shall be equally distributed over the bits of class 0, applying the formula:

r0=12INT12∗1+Bc0+4

INT	Next lower integer value

•	If  channel codec bits per frame are available, in a first pass the intermediate rates r'[i], i=0...4 of the bit classes shall be determined with the following formula:

r'i=12MIN36,NINT12∗1+B∗wici

The final rate is determined by applying the constraint:

ri=1           if  r'i>1218r'i         otherwise

•	If the above algorithm allocates less bits than available, in the second pass bits from the first unprotected class shall be shifted to the last protected class until all available bits are allocated.

•	If more bits than available have been allocated, in the second pass bits from the last protected class shall be shifted to the first unprotected class until the maximum allowed channel codec bit-rate is achieved.

•	In the original configuration class 3 contains the tail bits. If this class receives no protection applying the above formula, the tail bits shall be attached to the last protected class.

##### C.2.5.1.1	Control parameter settings for active speech modes

The following settings of c[i] and w[i] shall be chosen (Table C.5a):

Table C.5a/G.723.1

Settings of control parameters for rate allocation (active speech frames)

|  | G.723.1 – 6.3 kbit/s |  |  |  |  | G.723.1 – 5.3 kbit/s |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i | 0 | 1 | 2 | 3 | 4 | 0 | 1 | 2 | 3 | 4 |
| c[i] | 44+5crc | 44 | 46 | 47+4tail | 12 | 34+5crc | 40 | 40 | 40+4tail | 8 |
| w[i] | 0.26 | 0.29 | 0.24 | 0.21 | 0.00 | 0.24 | 0.31 | 0.24 | 0.21 | 0.00 |

#### C.2.5.1.2	Control parameter settings for SID frames

The following settings of c[i] and w[i] shall be chosen (Table C.5b):

Table C.5b/G.723.1

Settings of control parameters for rate allocation (SID frames)

|  | G.723.1 – SID |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| i | 0 | 1 | 2 | 3 | 4 |
| c[i] | 30+5crc+4tail | 0 | 0 | 0 | 0 |
| w[i] | 1.0 | 0.00 | 0.00 | 0.00 | 0.00 |

#### C.2.5.2	End of trellis adaptation

To take the lower error probability of the last protected bits into account, the bits

bs(Nlast-n);    n = 0 ... 9;

shall be exchanged with the bits:

bs(Nlast-19+n);   n = 0 ... 9.

Nlast is the last protected information bit in the bitstream.

This operation shall only be performed if more than 19 information bits are protected in the classes 1 to 3.

#### C.2.5.3	Encoding of the information bitstream

Each bit bs(j) which receives protection () shall be encoded applying the following steps:

•	Encoding of information bit bs(j) of class i with the three generator polynomials gCCn. This results in three intermediate channel bits u'(n) at the output (n=0,1,2).

•	Puncturing of u'(n) with Pri (n) in the following way:

If bit p of Pri (n) equals 0 then u'(n) is discarded otherwise it is inserted in the channel bitstream u(m) and m is incremented. p is a modulo 12 counter which is incremented with the information bit counter j.

After encoding of the protected bits the channel bitstream is contained in array u(m), m = 0 ..Mp−1.

The unprotected bits shall be copied to the end of the protected channel bitstream, starting at location m = Mp and ending at location m =MAll−1.

#### C.2.5.4	Encoding of configuration bits

Three channel codec configuration bits shall be added to the two source codec configuration bits as shown in Table C.6. With this it is possible to add to each of the G.723.1 source codec modes (high rate, low rate, SID) one of 8 different channel codec bit-rates. The choice of the channel codec bit-rates depends on the transmission system and is therefore not subject of this Annex.

Table C.6/G.723.1

G.723.1 and channel codec configuration bits

| Octet | Configuration Bits |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | UB | UB | UB | UB | UB | ccc | VAD | RATE |

A (13,5) block code with the generator polynomial:

gchD=D10+D8+D5+D2+D+1

shall be applied for error protection of the configuration bits.

The 13 encoded configuration bits (ucb[n], n=0 ... 12) shall be transmitted prior to the encoded information bits.

Table C.7 shows the format of the packed channel bitstream (depending on the value of , there may be unused bits "UB" in the last octet).

Table C.7/G.723.1

Channel bitstream packing

| Transmitted Octet | Channel Bit |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ucb[7] | ucb[6] | ucb[5] | ucb[4] | ucb[3] | ucb[2] | ucb[1] | ucb[0] |
| 2 | u[2] | u[1] | u[0] | ucb[12] | ucb[11] | ucb[10] | ucb[9] | ucb[8] |
| 3 | u[10] | u[9] | u[8] | u[7] | u[6] | u[5] | u[4] | u[3] |
| /8+2 | ... | ... | u[] | u[-1] | u[-2] | ... | ... | ... |
| ... | ... | ... | ... | ... | ... | ... | ... | ... |
| /8+2 | UB | UB | UB | u[-1] | u[-2] | u[-3] | u[-4] | u[-5] |

The bit-rate of the protected bitstream is MAll+13·100030 bits per second.

## C.3	Channel decoder

### C.3.1	Decoding of configuration bits

The first 13 bits shall be decoded to obtain the speech and channel codec configurations. The rate allocation algorithm described in C.2.5.1 is executed after reset or after a change in one of the configuration bits.

### C.3.2	Speech decoder adaptation for active speech modes

The speech parameter format of the channel codec shall be adapted to the speech parameter format of G.723.1.

#### C.3.2.1	Gain index compression

If the speech decoder operates either in high or low bit-rate mode the gain compression operation as described in 2.17 and 2.18 of Recommendation G.723.1 shall be performed to generate the bitstream format required by the speech decoder. One 8-bit (AGAINx_By) and one 5-bit (FGAINx_By) gain index per subframe are compressed to form one 12-bit compressed gain index. If forbidden indices occur either in the decoded adaptive codebook gain index or in the decoded fixed excitation gain index the forbidden index, indication flag (FII) shall be set and the indices shall be replaced by suitable valid values. The FII flag shall also be set, if a forbidden lag index is decoded (subclause 2.18 of Recommendation G.723.1). The decoded lag shall then be replaced by a suitable valid value.

#### C.3.2.2	LPC ordering

The three LPC sub-vectors shall be ordered applying the inverse tables of C.2.1.2.

em=ReorderTabminvemRm=0,1,2

### C.3.3	Error analysis

Two flags indicating the reliability of the received G.723.1 speech parameters shall be generated in the channel decoder:

1)	Bad Frame Indication (BFI) which signals uncorrected errors within the bits of the CRC window ().

2)	Erroneous Frame Indication (EFI) which signals uncorrected errors outside the bits covered by the CRC code. This flag shall only be evaluated if information bits outside the CRC window are protected.

These flags shall be made available to the speech decoder together with the speech parameters and the FII flag. In the case of received SID frames, only the BFI flag shall be determined.

### C.3.4	Speech parameter format

The speech parameters shall be transmitted from the channel decoder to the speech decoder in the format specified in Tables 5 and 6 of Recommendation G.723.1 with the error flags appended to the end. This format is shown in Table C.8a for the high bit-rate mode, Table C.8b for the low bit-rate mode and in Table C.8c for SID frames.

Table C.8a/G.723.1

Decoder bitstream packing for high rate mode

| Speech Decoder Octets | PARx_By, .... |
| --- | --- |
| 1 | LPC_B5...LPC_B0, VAD, RATE |
| 2 | LPC_B13...LPC_B6 |
| 3 | LPC_B21...LPC_B14 |
| 4 | ACL0_B5...ACL0_B0, LPC_B23, LPC_B22 |
| 5 | ACL2_B4...ACL2_B0, ACL1_B1, ACL1_B0, ACL0_B6 |
| 6 | GAIN0_B3...GAIN0_B0, ACL3_B1, ACL3_B0, ACL2_B6, ACL2_B5 |
| 7 | GAIN0_B11...GAIN0_B4 |
| 8 | GAIN1_B7...GAIN1_B0 |
| 9 | GAIN2_B3...GAIN2_B0, GAIN1_B11...GAIN1_B8 |
| 10 | GAIN2_B11...GAIN2_B4 |
| 11 | GAIN3_B7...GAIN3_B0 |
| 12 | GRID3_B0, GRID2_B0, GRID1_B0, GRID0_B0, GAIN3_B11...GAIN3_B8 |
| 13 | MSBPOS_B6...MSBPOS_B0, UB |
| 14 | POS0_B1, POS0_B0, MSBPOS_B12...MSBPOS_B7 |
| 15 | POS0_B9...POS0_B2 |
| 16 | POS1_B2, POS1_B0, POS0_B15...POS0_B10 |
| 17 | POS1_B10...POS1_B3 |
| 18 | POS2_B3...POS2_B0, POS1_B13...POS1_B11 |
| 19 | POS2_B11...POS2_B4 |
| 20 | POS3_B3...POS3_B0, POS2_B15...POS2_B12 |
| 21 | POS3_B11...POS3_B4 |
| 22 | PSIG0_B5...PSIG0_B0, POS3_B13, POS3_B12 |
| 23 | PSIG2_B2..PSIG2_B0, PSIG1_B4...PSIG1_B0 |
| 24 | PSIG3_B4...PSIG3_B0, PSIG2_B5...PSIG2_B3 |
| 25 | UB , UB , UB , UB , UB , FII , EFI , BFI |

Table C.8b/G.723.1

Decoder bitstream packing for low rate mode

| Speech Decoder Octets | PARx_By, .... |
| --- | --- |
| 1 | LPC_B5...LPC_B0, VAD, RATE |
| 2 | LPC_B13...LPC_B6 |
| 3 | LPC_B21...LPC_B14 |
| 4 | ACL0_B5...ACL0_B0, LPC_B23, LPC_B22 |
| 5 | ACL2_B4...ACL2_B0, ACL1_B1, ACL1_B0, ACL0_B6 |
| 6 | GAIN0_B3...GAIN0_B0, ACL3_B1, ACL3_B0, ACL2_B6, ACL2_B5 |
| 7 | GAIN0_B11...GAIN0_B4 |
| 8 | GAIN1_B7...GAIN1_B0 |
| 9 | GAIN2_B3...GAIN2_B0, GAIN1_B11...GAIN1_B8 |
| 10 | GAIN2_B11...GAIN2_B4 |
| 11 | GAIN3_B7...GAIN3_B0 |
| 12 | GRID3_B0, GRID2_B0, GRID1_B0, GRID0_B0, GAIN3_B11...GAIN3_B8 |
| 13 | POS0_B7...POS0_B0 |
| 14 | POS1_B3...POS1_B0, POS0_B11...POS0_B8 |
| 15 | POS1_B11...POS1_B4 |
| 16 | POS2_B7...POS2_B0 |
| 17 | POS3_B3...POS3_B0, POS2_B11...POS2_B8 |
| 18 | POS3_B11...POS3_B4 |
| 19 | PSIG1_B3...PSIG1_B0, PSIG0_B3...PSIG0_B0 |
| 20 | PSIG3_B3...PSIG3_B0, PSIG2_B3...PSIG2_B0 |
| 21 | UB , UB , UB , UB , UB , FII , EFI , BFI |

Table C.8c/G.723.1

Decoder bitstream packing for SID frames

| Speech Decoder Octets | PARx_By, .... |
| --- | --- |
| 1 | LPC_B5...LPC_B0, VAD, RATE |
| 2 | LPC_B13...LPC_B6 |
| 3 | LPC_B21...LPC_B14 |
| 4 | GAIN_B5 ... GAIN_B0, LPC_B23, LPC_B22 |
| 5 | UB , UB , UB , UB , UB , UB , UB , BFI |

### C.3.5	Error concealment

The flags produced by the channel decoder and by the gain compression operation shall be used in the speech decoder for error concealment purposes.

### C.3.6	Performance of error analysis flags

This clause specifies the minimum required error analysis performance in terms of bad frame detection probability () and erroneous frame detection probability (). The requirements contained in Table C.9 shall apply to all transmission channels.

Table C.9/G.723.1

Detection performance of BFI and EFI flags

|  | G.723.1 – 6.3 kbit/s | G.723.1 – 5.3 kbit/s | G.723.1 – SID |
| --- | --- | --- | --- |
|  | 0.99 | 0.99 | 0.99 |
|  | 0.65 | 0.65 | --- |

## C.4	Fixed point C source code

All details of the channel encoder algorithm are described in the form of a fixed point ANSI-C source code. In the case of any discrepancy between the above description and the C source code, the latter has to be adopted as reference. This C source code is a part of the code distributed by the ITU-T as Recommendation G.723.1.

Test vectors are included to check the bit-exactness of an implementation compared to the C source code.

ITU-T  RECOMMENDATIONS  SERIES

Series A	Organization of the work of the ITU-T

Series B	Means of expression

Series C	General telecommunication statistics

Series D	General tariff principles

Series E	Telephone network and ISDN

Series F	Non-telephone telecommunication services

Series G	Transmission systems and media

Series H	Transmission of non-telephone signals

Series I	Integrated services digital network

Series J	Transmission of sound-programme and television signals

Series K	Protection against interference

Series L	Construction, installation and protection of cables and other elements of outside plant

Series M	Maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits

Series N	Maintenance: international sound-programme and television transmission circuits

Series O	Specifications of measuring equipment

Series P	Telephone transmission quality

Series Q	Switching and signalling

Series R	Telegraph transmission

Series S	Telegraph services terminal equipment

Series T	Terminal equipment and protocols for telematic services

Series U	Telegraph switching

Series V	Data communication over the telephone network

Series X	Data networks and open system communication

Series Z	Programming languages