|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.726  –  Annex  A

TELECOMMUNICATION	(11/94)
STANDARDIZATION  SECTOR
OF  ITU

GENERAL  ASPECTS  OF  DIGITAL
TRANSMISSION  SYSTEMS

40,  32,  24,  16  kbit/s  ADAPTIVE  
DIFFERENTIAL  PULSE  CODE  
MODULATION  (ADPCM)

ANNEX  A:  EXTENSIONS  OF  
RECOMMENDATION  G.726  FOR  
USE  WITH  UNIFORM-QUANTIZED  
INPUT  AND  OUTPUT

ITU-T  Recommendation  G.726  –  ANNEX  A

(Previously “CCITT  Recommendation”)

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation G.726 – Annex A was prepared by ITU-T Study Group 15 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 1st of november 1994.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1995

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation G.726  –  Annex  A     (11/94)

Page

Annex  A  –  Extensions of recommendation G.726 on 40, 32, 24, 16 kbit/s adaptive differential pulse code modulation for use with uniform-quantized input and output		1

A.1	General		1

A.2	Changes to the ADPCM algorithm		1

A.3	Computational details of changes to Recommendation G.726 for uniform-quantized PCM		4

Recommendation G.726

Recommendation G.726  –  Annex  A     (11/94)

40,  32,  24,  16  kbit/s  ADAPTIVE  DIFFERENTIAL  PULSE  
CODE  MODULATION  (ADPCM)1)

(Geneva, 1990)

Annex  A

Extensions of Recommendation G.726 on 40, 32, 24, 16 kbit/s
adaptive differential pulse code modulation for use 
with uniform-quantized input and outputXE "Extensions of Recommendation G.726 on 40, 32, 24, 16 kbit/s
adaptive differential pulse code modulation for use 
with uniform-quantized input and output"§

(Geneva, 1994)

(This annex forms an integral part of this Recommendation)

## A.1	General

This annex extends the Adaptive Differential Pulse Code ModulationXE "Adaptive Differential Pulse Code Modulation"§ (ADPCM) algorithm specified in Recommendation G.726 to allow use of a uniform PCM interface at input and output.

The principal application for this extension is at the analogue end points of digital transmission links, especially telephone terminal equipment.

This annex defines a 14-bit uniform-quantized interfaceXE "14-bit uniform-quantized interface"§ for Recommendation G.726. Use with other data widths requires scaling the data such that the most significant bit of the 14-bit uniform PCM corresponds to the most significant bit of the target data width. For example, a 13-bit input value must be left shifted by one bit to provide the input, SL. Likewise SO, the output value, must be right shifted by one bit to provide a 13-bit output.

Subclause A.2 describes the changes in principles to use uniform-quantized PCM. Subclause A.3 provides the changed computational details from Recommendation G.726. Appendix I/G.726 describes the digital test sequences.

NOTE – Uniform-quantized PCM is defined by reference to Recommendation G.701 and to 3.6/G.711.

## A.2	Changes to the ADPCM algorithm

The core ADPCM algorithmXE "core ADPCM algorithm"§ is unchanged, but the PCM conversion and formatting blocks EXPAND and COMPRESS as well as the Synchronous Coding Adjustment shall not be used. Because there was an implicit limiting function in the COMPRESS block, a new block LIMO has been introduced to explicitly limit the output to the range –8192 to +8191, inclusive.

The input to the encoder is no longer s(k), but sl(k), the uniform PCM signal. The output of the decoder is no longer sd(k), but so(k), the 14-bit limited version of sr(k), the reconstructed signal.

The Synchronous Coding AdjustmentXE "Synchronous Coding Adjustment"§ was introduced into Recommendation G.726 to compensate for cumulative distortion caused by multiple transcodings as a signal propagates through successive PCM and ADPCM links. However, it is not possible to use this technique with uniform-quantized PCM. Analysis has shown that cumulative distortion is not possible at 16, 24 or 32 kbit/s. However, because cumulative distortion is possible at 40 kbit/s, a 40 kbit/s decoder as described in this annex, shall be used only at the analogue endpoint of a digital transmission chain.

The 14-bit uniform-PCM output value shall not be re-compressed to A-law or m-law PCM as this bypasses the Synchronous Coding Adjustment of Recommendation G.726.

When a uniform PCM interfaceXE "uniform PCM interface"§ is implemented, references to Figure 1/G.726 shall be replaced with references to Figure A.1.

FIGURE A.1/G.726...[D01] = 14 CM

When a uniform PCM interface is implemented, Figure A.2 shall be used instead of Figure 2/G.726.

FIGURE A.2/G.726...[D02] = 10 CM

When a uniform PCM interface is implemented, Figure A.3 shall be used instead of Figure 3/G.726.

FIGURE A.3/G.726...[D03] = 10.5 CM

## A.3	Computational details of changes to Recommendation G.726 for uniform-quantized PCMXE "Computational details of changes to Recommendation G.726 for uniform-quantized PCM"§

### A.3.1	Input and output signals

When a uniform PCM interface is implemented, Table A.1 shall be used instead of Table 1/G.726.

TABLE A.1/G.726

Input and output signals

|  | Name | Number of bits | Description |
| --- | --- | --- | --- |
| Encoder |  |  |  |
| Input | SL | 14 | Uniform PCM input word |
| Input | R (optional) | 1 | Reset |
| Output | I | 5 | 40 kbit/s ADPCM word |
| Output | I | 4 | 32 kbit/s ADPCM word |
| Output | I | 3 | 24 kbit/s ADPCM word |
| Output | I | 2 | 16 kbit/s ADPCM word |
| Decoder |  |  |  |
| Input | I | 5 | 40 kbit/s ADPCM word |
| Input | I | 4 | 32 kbit/s ADPCM word |
| Input | I | 3 | 24 kbit/s ADPCM word |
| Input | I | 2 | 16 kbit/s ADPCM word |
| Input | R (optional) | 1 | Reset |
| Output | SO | 14 | Uniform PCM output word |

### A.3.2	Description of internal processing variables

For uniform PCM implementations, the internal processing variables shall be the same as in Table 6/G.726, with the exception of the variables DLNX, DLX, DSX, DX, SL, SLX and SP, which shall not be used.

### A.3.3	Changes to input PCM conversion and difference signal computation

When a uniform PCM interface is implemented, Figure A.4 shall be used instead of Figure 4/G.726.

When a uniform PCM interface is implemented, the EXPAND block shall not be used.

FIGURE A.4/G.726...[D04] = 3.5 CM

### A.3.4	Output PCM format conversion and synchronous coding adjustment deletedXE "Output PCM format conversion and synchronous coding adjustment deleted"§

When a uniform PCM interface is implemented the blocks COMPRESS, EXPAND and SYNC (see 4.2.8/G.726) shall not be used. Figure 11/G.726 shall also not be used.

In addition, 4.2.8/G.726 shall be replaced by the following subclause:

### A.3.5	Output limiting (decoder only)

See Figure A.5.

FIGURE A.5/G.726...[D05] = 3.5 CM

_______________________________________________________________________________________________

LIMO (decoder only)

Input:			SR
Output:			SO
Function:		Limit output to 14-bit two’s compliment value

|	8191,		SR > 8191 and SR < 32 768

SO =		|	SR & 16383,	SR < 8192 or SR > 57 344

|	57 344,		SR > 32767 and SR < 57 344

_______________________________________________________________________________________________

1	)	This Recommendation completely replaces the text of Recommendations G.721 and G.723 published in Volume III.4 of the Blue Book. It should be noted that systems designed in accordance with the present Recommendation will be compatible with systems designed in accordance with Blue Book version.