|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.728 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | Annex I (05/99) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Terminal equipments – Coding of analogue signals by methods other than PCM

Coding of speech at 16 kbit/s using low-delay code excited linear prediction

Annex I: Frame or packet loss concealment for the LD-CELP decoder

ITU-T  Recommendation  G.728  –  Annex I

(Previously  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.728 Coding of speech at 16 kbit/s using low-delay code excited linear prediction Annex I Frame or packet loss concealment for the LD-CELP decoder

Summary This annex defines an extension for robust operation of the G.728 algorithm at 9.6, 12.8 and 16 kbit/s in the presence of frame erasure or packet loss in mobile or packet environments.

Source Annex I to ITU-T Recommendation G.728 was prepared by ITU-T Study Group 16 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on 27 May 1999.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation the term recognized operating agency (ROA) includes any individual, company, corporation or governmental organization that operates a public correspondence service. The terms Administration, ROA and public correspondence are defined in the Constitution of the ITU (Geneva, 1992).

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

Annex I – Frame or packet loss concealment for the LD-CELP decoder		1

I.1	Scope		1

I.2	Normative references		1

I.3	Introduction		1

I.4	Principles of operation		2

I.4.1	Excitation extrapolation		2

I.4.2	LPC filter		3

I.4.3	Backward adaptation of LPC and gain predictors		4

I.4.4	Post-filter		5

I.4.5	Limitation of gain growth after frame erasure		5

I.5	Pseudo-code for frame erasure concealment		6

I.5.1	New main program loop for decoder		6

I.5.2	Block 31SF – Set flags and scaling factor for frame erasure		8

I.5.3	Block 31FE – Excitation signal extrapolation		9

I.5.4	Block 31E – Excitation signal update		10

I.5.5	Block 43FE – Hybrid windowing module		11

I.5.6	Block 47AF – Log-gain limiter after frame erasure		12

I.5.7	Block 49FE – Hybrid window module for synthesis filter		12

I.5.8	Block 51FE – Bandwidth expansion module for frame erasures		15

I.5.9	Block 97FE – Update GSTATE during frame erasures		16

I.5.10	Block 98AF – Log-gain limiter after frame erasure		17

I.6	Additional coder parameters and variables		17

Appendix I.I – Values used for scaling ETPAST during frame erasures		19

Recommendation G.728

Coding of speech at 16 kbit/s using low-delay code
excited linear prediction

Annex I

Frame or packet loss concealment for the LD-CELP decoder

(Geneva, 1999)

## I.1	Scope

This annex is concerned with how to conceal the loss of bit-stream information due to frame or packet loss in the communications channel. During normal operation, the decoder performs in a manner identical with Recommendation G.728 (main body) or G.728 Annex G at 16 kbit/s or with G.728 Annex H when operating at 12.8 or 9.6 kbit/s. The modification proposed in this annex only involves changing the decoder during times when the bit-stream is unavailable. It is presumed that this loss of the bit stream is indicated to the decoder by some external means. This annex is not essential for normal operation of G.728.

## I.2	Normative references

–	CCITT Recommendation G.728 (1992), Coding of speech at 16 kbit/s using low-delay code excited linear prediction.

–	ITU-T Recommendation G.728 Annex G (1994), 16 kbit/s fixed point specification.

–	ITU-T Recommendation G.728 Annex H (1997), Variable bit rate LD-CELP operation mainly for DCME at rates less than 16 kbit/s.

## I.3	Introduction

This annex describes the modifications of the G.728 decoder to make it suitable for handling frame erasures in the received bit stream. These modifications include excitation extrapolation, bandwidth expansion for the LPC filter, continuation of part of backward adaptation using the extrapolated excitation, continuation of post-filtering and post-filter adapter using the extrapolated excitation, and limiting the rate of increase for the backward-adaptive gain after frame erasures. Such decoder changes maintain the bit-stream compatibility to Recommendation G.728 and the additional complexity is negligible.

To maintain the bit-stream compatibility to Recommendation G.728, all the modifications are confined to the decoder. They take effect only during erased frames and the first few "good" frames after frame erasures. The proposed system otherwise operates exactly the same way as in G.728 during all other "good" frames.

This annex assumes that the readers are already familiar with the Recommendation G.728 specification document. The G.728 algorithm will not be discussed here. Only modifications to G.728 will be described. To avoid confusion, from here on, we will reserve the word frame for the block size associated with frame erasure (typically a multiple of 2.5 ms). We will use adaptation cycle to denote the 2.5 ms block for the adaptation of G.728 filter parameters.

Based on the input from ITU-R (formerly CCIR), with a frame erasure channel a received frame of bits can be considered to be either totally good (zero bit error), or totally bad (bit error rate within the frame approaches 50%). It is also assumed that an error detection code has been used so that the

receiver can reliably determine which frames are good and bad, and make such information available to the G.728 speech decoder.

This annex is divided as follows. Subclause I.4, "Principles of operation", contains subclauses which describe the changes in operation of the decoder during frame erasure concealment. Subclause I.5, "Pseudo-code for frame erasure concealment", contains all of the pseudo-code needed for the decoder modifications. For completeness, pseudo-code is given for the floating-point as well as fixed-point specifications. Finally, subclause I.6 contains tables of the additional coder parameters and variables that have been added for frame erasure concealment operations.

## I.4	Principles of operation

The modifications to Recommendation G.728 for erased frames include extrapolation of the excitation and the LPC filter coefficients, continuation of certain backward adaptation operations for LPC and gain predictors, and limiting the rate of growth for the gain after frame erasures. These are described separately in the following subclauses.

### I.4.1	Excitation extrapolation

First refer to Figure 3/G.728. We do not extrapolate the output of the excitation VQ codebook (Block 29). Instead, we extrapolate the output of the gain scaling unit (Block 31), or the gain-scaled excitation, which is denoted by the ET() array in the pseudo-code in 5.14/G.728. In the original G.728 decoder, we keep only 5 samples of ET() in the memory. To be able to extrapolate ET(), now we need to maintain an additional array ETPAST() which stores the present and past ET() vectors.

Refer to the pseudo-code in 5.14/G.728. In the G.728 decoder, the pitch period extraction module (Block 82) gives us a pitch period KP. The pitch predictor tap calculator (Block 83) calculates the optimal tap weight of a single-tap pitch predictor for the decoded speech. This tap weight PTAP is used as a rough indicator of whether the current adaptation cycle is voiced or not. If PTAP is greater than PPFTH, where PPFTH  0.6, then the current adaptation cycle is considered to be voiced, and the long-term post-filter is applied; otherwise, the long-term post-filter is disabled. Since we already have the pitch period and a voicing indicator available in the original G.728 decoder, we can directly use them in the excitation extrapolation without having to recompute them again.

At the beginning of the first erased frame in an erasure, we use the value of the voicing indicator PTAP obtained in the last adaptation cycle of the last good frame to determine whether the speech is voiced or unvoiced. During erasures we extrapolate the excitation differently for voiced and unvoiced frames. For extrapolation purposes we use a voicing threshold, VTH  PPFTH/1.4, that is lower than PPFTH.

If PTAP > VTH, we treat the last frame as voiced. Then, the extrapolation of ET() is done by periodically repeating a scaled-down version of the last KP samples of the ETPAST() array. Alternatively, this operation can be explained in the following way: Consider a 5-sample rectangular window which has unit sample values and is located at the position corresponding to the current vector of ETPAST(). Now, slide this rectangular window back in time by KP samples, extract the corresponding 5 samples of ETPAST(), and scale them down by multiplying by a scaling factor. The result is the extrapolated ET() vector that we desire. This extrapolated ET() vector is then copied to the proper location of the ETPAST() array to update the current vector of ETPAST(). This process is repeated for all vectors within the erased frame. During erased frames this operation produces an ET() sequence with periodic shapes but decaying magnitudes. The period is KP samples. Such periodic extrapolation of the ETPAST() array is continued until a good frame is received. The scaling factor, FESCALE, is 0.8 for the first 20 ms of an erasure and then decreases by 0.2 for each additional 10 ms. These scaling factors are stored in VOICEDFEGAIN(). If the erasure is longer than 50 ms, FESCALE is set to 0 to avoid introducing long, unnatural, periodic artifacts in the output speech signal.

If PTAP  VTH, then we treat the last adaptation cycle as "unvoiced". Strictly speaking, it is anything other than "voiced", as it could include silence, transition, etc. In this case, we do not perform periodic extrapolation because we do not want to introduce artificial periodicity that is not in the original speech signal. We avoid periodicity by randomly repeating 5 consecutive samples from the last 140 samples in ETPAST(). For each vector in a bad frame, we use a random number generator to generate a random integer between 5 and 140. This random number determines how many samples the sliding rectangular window should slide back in time to extract the corresponding 5 samples of ETPAST() as the extrapolated ET(). Of course, if desired, such a sequence of random numbers can be precomputed and stored.

To insure that the level of the signal generated during an unvoiced frame erasure resembles its recent history, we match the average magnitude in each extrapolated excitation vector to the average excitation magnitude that occurred in the last 5 ms (40 samples) before the erasure. This is done in the following way: At the beginning of the first erased frame, if we determine the speech is unvoiced we calculate the average magnitude of the last 8 vectors of ETPAST(). Let AVMAG be the resulting average magnitude. Then, when we extrapolate each excitation vector, we first calculate the magnitude (MAG) of the vector [which is 5 consecutive samples randomly selected from the last 140 samples of ETPAST()]. Next, we divide AVMAG by MAG to obtain a scaling factor. This scaling factor is then used to multiply each of the 5 samples in the current extrapolated excitation vector. This operation ensures that each of the extrapolated and gain-scaled excitation vector has an average magnitude of AVMAG. To avoid artifacts caused by very long erasures AVMAG is attenuated by 20% for every 10 ms the erasure lasts beyond 20 ms. The attenuation factors are stored in the array UNVOICEDFEGAIN().

### I.4.2	LPC filter

During erased frames, we extrapolate the gain-scaled excitation ET() as described above, but we still need an LPC synthesis filter to produce the output speech. We use a somewhat "softened" version of the last good set of LPC coefficients in the last good frame. Such "softening" is achieved by bandwidth expansion, as is done in the bandwidth expansion module of Block 51 (5.6/G.728), except that the bandwidth expansion factor FAC (5.1/G.728) is 0.97 rather than 253/256  0.9883. In the first erased frame following a good frame, this bandwidth expansion operation is performed at the 3rd vector of the 1st adaptation cycle, just when the LPC coefficients would have normally been updated were it not for the frame erasure.

Let ai be the ith LPC coefficients used in the last adaptation cycle of the last good frame. Then, the new set of softened LPC coefficients is obtained as a´i  (0.97)i ai, i  1, 2, …, 50. This new set of {a´i} is used during the first 10 ms of the erasure (if it lasts that long), even though there may be subsequent adaptation cycles in the same frame. If the erasure extend beyond 10 ms, this set of {a´i} is further bandwidth expanded by a factor of 0.97. In other words if the erasure lasts more than 10 ms, the LPC coefficients are updated again at the third vector in the 5th adaptation cycle and the ith LPC coefficient is a"i  (0.97)i a´i  (0.97)2i ai. Again, this set of {a"i} is used for the entire next 10 ms of the erasure. Similarly, in the erasure extends to (k * 10 ms), the LPC coefficients are bandwidth expanded version of the last good set of LPC coefficients, where the bandwidth expansion factor is (0.97)k. The farther away from the last good frame, the more bandwidth expansion we apply, and this continues until the next good frame is received. Then, next time there is a bad frame again, the process starts from a bandwidth expansion factor of 0.97 again.

In the first frame of an erasure we do something special to get the most recent LPC coefficients for bandwidth expansion. Specifically, if we directly perform bandwidth expansion on the A() array produced in the last good frame, that A() array corresponds to the autocorrelation function calculated based on the synthesized speech before the last good frame. Therefore, it is obsolete for the first erased frame. To get a more recent LPC predictor coefficient array A() for bandwidth expansion, the hybrid window and Durbin's recursion for the 50th-order LPC analysis are performed as usual in the

first erased frame. The resulting A() array is then bandwidth expanded by a factor of 0.97. The resulting LPC coefficients are more up to date because they are based on the synthesized speech up to the last vector of the last good frame. If the erasure extends beyond 10 ms, the special handling stops and the bandwidth expansion is based on the last set of A() array in the memory, which had been bandwidth expanded before as described in the last paragraph.

Note that no such bandwidth expansion of the gain predictor is necessary during erased frames. Since we already extrapolated the gain-scaled excitation ET(), we directly feed this extrapolated ET() to the LPC synthesis filter to produce synthesized speech. Therefore, during erased frames there is no need to produce the backward-adapted excitation gain or to update the gain predictor coefficients.

### I.4.3	Backward adaptation of LPC and gain predictors

During erased frames, we do not need the outputs of the backward synthesis filter adapter and backward vector gain adapter (Blocks 33 and 30 in Figure 3/G.728). However, we still want to continue vital operations of the backward adaptation process in these two blocks to reduce discontinuities in the internal states when the next good frame comes. By "vital operations", we mean those parts of the backward adaptation operations that will affect the outputs of Blocks 30 and 33 in the future frames.

Refer to Block 30 (backward vector gain adapter), which is identical to Block 20 (5.7/G.728 and Figure 6/G.728). For Block 30 (backward vector gain adapter), the vital operations include Blocks 67, 39, 40, 41, 42 (which together compute the offset-removed logarithmic gain), and parts of Block 43 (hybrid window module) and Block 46 (log-gain linear predictor). Blocks 67 and 39 through 42 use the extrapolated ET() to compute the offset-removed log-gain, whose value is used to update GTMP(), which is the input to Block 43, and GSTATE(), which is the filter memory of the log-gain linear predictor. Inside Block 43 (hybrid window module), we need to use GTMP() to update the SBLG() buffer, and we also need to update REXPLG(), the recursive component of the autocorrelation function. These operations mentioned above maintain the internal states that will affect the future outputs of Block 30. Any other operations, such as Levinson-Durbin recursion (Block 44), bandwidth expansion (Block 45), inverse logarithm calculator (Block 48), etc., need not be performed during frame erasure, since their outputs are discarded anyway.

Note that since the gain predictor coefficients GP() are not updated during a frame erasure, the first vector after the frame erasure will still use the old GP() calculated in the last good frame before the frame erasure. This is because the GP() array is not updated until the second vector of a G.728 adaptation cycle. We do not attempt to make the GP() array more up to date for the first vector, because it will unnecessarily complicate the algorithm flow control. Also, the gain clamping described below in I.4.5 will guard against a large gain excursion for this first vector. Therefore, the use of a slightly obsolete gain predictor for only one vector is not likely to make a perceptually noticeable difference in speech quality.

Next, refer to Block 33 (decoder backward synthesis filter adapter in 5.14/G.728), which is identical to Block 23 (5.6/G.728 and Figure 5/G.728). For Block 33, the vital operations include:

1)	updating the SB() buffer in the hybrid windowing module (Block 49) using the synthesized speech (which is obtained by passing extrapolated ET() through bandwidth expanded version of the last good LPC filter); and

2)	computing REXP() in the normal manner using SB().

Most other parts of Block 33 need not be done during erased frames. This is because the LPC coefficients are updated not by the outputs of Blocks 50 and 51 but by a bandwidth expanded version of the LPC coefficients in the last good frame. However, an additional part of Blocks 49 and a small part of Block 50 still need to be performed for the post-filter. This will be discussed in I.4.4.

As just explained, many computationally intensive operations in Blocks 30 and 33 need not be done during erased frames. We can use the saved computation (or DSP processor time) to perform excitation extrapolation and bandwidth expansion of the last good LPC filter, both of which take very little computation to do. This is the reason why the modifications to the G.728 decoder do not increase the overall computational complexity.

### I.4.4	Post-filter

For flow control and program simplicity, the operation of the post-filter (5.14/G.728) and post-filter adapter (Blocks 34 and 35 in Figure 3/G.728) is the same in both good and erased frames. This has several implications.

First, during erased frames the post-filter adapter still needs the 10th-order LPC predictor coefficients and the first reflection coefficient from the backward synthesis filter adapter (Block 33 in Figure 3/G.728). Hence, in Block 49 (hybrid windowing module, Figure 5/G.728), we still need to calculate the values of RTMP(1) through RTMP(11); we do not need to calculate RTMP(12) through RTMP(51), however. Similarly, in Block 50, we still need to perform Durbin's recursion from order 1 to order 10, although we do not need to continue from order 11 to order 50. Note that during erased frames the 10th-order LPC coefficients and the first reflection coefficient are actually obtained from the synthesized speech which is ultimately driven by the extrapolated excitation ET().

Second, during erased frames the post-filter updates all its coefficients based on the speech synthesized in the erased frames and then uses the updated coefficients to filter such synthesized speech to produce the final output speech. We do not attempt to "freeze" the post-filter coefficients or use a bandwidth expanded version of the post-filter in the last good frame as we did for the LPC filter. We intentionally let the post-filter "float" with the speech synthesized from the extrapolated excitation. This ensures that the spectral peaks and valleys of the post-filter frequency response match those of the synthesized speech all the time. This avoids the potential cancellation or de-emphasis of the spectral peaks of the synthesized speech by the spectral valleys in the post-filter frequency response due to a mismatch of the two.

While the post-filter continues to update the pitch period KP and the pitch predictor tap PTAP during an erasure, we do not use the updated values to change the voiced/unvoiced decision VOICED and stored pitch period FEDELAY while an erasure is in progress. These values are set once at the beginning of an erasure and do not change for the duration of the erasure.

### I.4.5	Limitation of gain growth after frame erasure

It was observed that after a very long frame erasure (up to 100 ms), or when a frame erasure was preceded by a low-amplitude region and followed by a high-amplitude region in the speech, the decoded speech after the frame erasure occasionally had a big "pop", or gain overshoot. This was because the log-gain buffer SBG() in the hybrid window contained very low level gains at the end of the frame erasure, and these low gains were followed by much higher gains in the first few good frames after the frame erasure. Thus, the LPC analysis produced a gain predictor which attempted to quickly catch up with the abrupt jump in the gain level. The predicted (i.e. backward adapted) gain therefore grew too fast and too much, causing a gain overshoot and the resulting "pop".

This problem is avoided by limiting the rate of growth of the backward adapted gain for the first few good frames after a frame erasure. After a frame erasure we limit the gain growth to no more than FEGAINMAX  2 dB per 5-sample vector. If the increase in the predicted log-gain is more than 2 dB/vector we clip it to 2 dB above the last log-gain. The duration of this "gain clamping" depends on the length of the frame erasure. If the frame erasure is AFTERFEMAX  16 (40 ms) or less, the duration of gain clamping is the length of the frame erasure. If the frame erasure is longer than 40 ms long, the gain clamping is limited to the first 40 ms after the erasure. The two constants FEGAINMAX and AFTERFEMAX are tunable coder parameters.

## I.5	Pseudo-code for frame erasure concealment

First, the pseudo-code for the decoder main program is given below. Only the block execution sequence is shown and no low-level detail of parameter passing is described. In the subclauses which follow, the pseudo-codes for the new routines are given.

### I.5.1	New main program loop for decoder

The main program loop for the decoder for either fixed-point or floating-point code only differs in regard to the backward vector gain adapter, which was changed in the fixed-point specification. Since these changes are relatively small in the main loop, we only give one version of the pseudo-code here. The C preprocessor constructs #ifdef, #else, and #endif, will be used to specify which segment belongs to the fixed-point specification and which belongs to the floating-point specification. The pseudo-code between a #ifdef FIXPT and the next #else is for fixed-point specification, while the pseudo-code between that #else and the next #endif is for floating-point specification.

Initialize all decoder variables to their initial values.

ILLCOND=.FALSE.

ILLCONDG=.FALSE.

FERROR=.FALSE.  | TRUE if current frame is erased

FESIZE=4        | Number of 2.5 msec adaptation cycles in a frame

| erasure(FE)

| e.g.: 1 for 2.5 msec FEs, 4 for 10 msec FEs

| FEs may be any multiple of 2.5 msec. Set by user.

ICOUNT=4        | Vector counter, 1,2,3,4,1,2,.., .625 ms each

ADCOUNT=FESIZE  | Adaptation cycle counter, 2.5 msec each

| Counts from 1,2,...,FESIZE,1,2,...,FESIZE,...;

FECOUNT=0       | Number of consecutively erased 2.5 msec adaptation

| cycles

AFTERFE=0       | Number of 2.5 msec adaptations cycles after a FE to

| limit the gain. Counts down.

OGAINDB=-32     | Last predicted gain in dB

VEC_LOOP:

If ICOUNT = 4, do the next 14 lines

ICOUNT = 0	| reset vector counter

| If the last adaptation cycle was not erased, and the

| gain was clamped decrease the number of adaptation cycles

| left to clamp the gain.

If FERROR = .FALSE. and AFTERFE > 0, do AFTERFE = AFTERFE - 1

| Check if the next frame is erased

If ADCOUNT = FESIZE, do the next 7 lines

ADCOUNT = 0	| reset adaptation cycle counter

read FERROR	| read in new ferror

| At the first good frame after an erasure set AFTERFE

| so the gain will be clamped for the next few good frames.

If FERROR = .FALSE. and FECOUNT > 0, do the next 4 lines

AFTERFE = AFTERFE + FECOUNT	|add the length of the FE to AFTERFE

If AFTERFE > AFTERFEMAX, do the next line

AFTERFE = AFTERFEMAX	| clamp gain for 40 msec max

FECOUNT = 0	| reset FECOUNT

If FERROR = .TRUE., do the next 2 lines

FECOUNT = FECOUNT + 1	| Update FECOUNT

| At the start of a FE, and every 10 msec thereafter,

| call block31SF to initialize the FE flags and update the

| excitation FE gains. e.g. when FECOUNT is equal to 1, 5, 9, 13, ...

If (FECOUNT & 3) = 1, do block31SF

ADCOUNT = ADCOUNT + 1	| update adaptation cycle counter

ICOUNT = ICOUNT + 1	| update vector counter

Get ICHAN of the current vector from the input buffer.

Obtain the shape index IS and gain index IG from ICHAN

| check whether to update LPC coeff.

If ICOUNT = 3, do the next 2 lines

If FERROR = .FALSE. and ILLCOND = .FALSE., do block 51.

If FERROR = .TRUE. and (FECOUNT & 3) = 1, do block 51FE

If ICOUNT = 2 and ILLCONDG=.FALSE. and FERROR = .FALSE., do block 45.

do block 46	| GSTATE(1:9) shifted down 1 position

#ifdef FIXPT

If FERROR = .FALSE., do the next 3 lines

do blocks 98AF, 99 and 48	| get backward-adapted gain

OGAINDB = output of block 98AF	| for limiting gain growth after FE

do blocks 29 and 31	| scale selected excitation codevector

#else

If FERROR = .FALSE., do the next 3 lines

do blocks 47AF and 48	| get backward-adapted gain

OGAINDB = output of block 47AF	| for limiting gain growth after FE

do blocks 29 and 31	| scale selected excitation codevector

#endif

If FERROR = .TRUE., do the next line

do block 31FE	| get extrapolated excitation

do block 31E	| update ETPAST()

do block 32	| get ST()

If ICOUNT = 1, do block 85	| update short-term postfilter coeff.

do block 81	| 10th-order LPC inverse filtering

If ICOUNT = 3, do the next 3 lines

do block 82	| pitch period extraction

do block 83	| compute pitch predictor tap

do block 84	| update long-term postfilter coeff.

do block 71	| long-term postfilter

do block 72	| short-term postfilter

do blocks 73 and 74	| calculate sums of absolute values

do block 75	| ratio of sums of absolute values

do block 76	| low-pass filter of scaling factor

do block 77	| gain control of postfilter output

#ifdef FIXPT

If FERROR = .FALSE., do the next 2 lines

do blocks 93, 94, 96, and 97	| update log-gain

GSTATE(1) = output of block 97	| update gain predictor memory

#else

If FERROR = .FALSE., do the next 2 lines

do blocks 39, 40 and 42	| update log-gain

GSTATE(1) = output of block 42	| update gain predictor memory

#endif

If FERROR = .TRUE., do the next 3 lines

do block 97FE	| compute log gain of extrapolated ET()

OGAINDB = output of 97FE	| save for limiting gain growth after FE

GSTATE(1) = output of block 97FE

| then update gain predictor memory

I=(ICOUNT-1)*IDIM	| I=starting address of STTMP()

copy ST(1:5) to STTMP(I+1:I+5)	| update STTMP()

NLSSTTMP(ICOUNT)=NLSST	| update NLSSTTMP()

| end of once-per-vector processing

| start once-per-cycle processing

If ICOUNT = 4, do the next 6 lines

do block 49FE	| output ill-condition flag = ILLCOND

do block 50, order 1 to 10	| output pred coef= ATMP() with NLSATMP

| output ill-condition flag = 	ILLCONDP

NLSAPF=NLSATMP	| save the 10th-order predictor for

copy ATMP(2:11) to APF(2:11)	| postfilter use later

If FERROR = .FALSE., do the next line

continue block 50,	| continue to finish block 50

order 11 to 50	| output pred coef= ATMP() with

| NLSATMP output ill-condition flag = ILLCOND

If ICOUNT = 1, do the next 7 lines

GTMP(1)=GSTATE(4)	| update GTMP() in one shot

GTMP(2)=GSTATE(3)

GTMP(3)=GSTATE(2)

GTMP(4)=GSTATE(1)

do block 43FE	| output ill-condition flag = ILLCONDG

If FERROR = .FALSE., do the next line

do block 44	| output pred. coeff. = GPTMP()

| output ill-condition flag = ILLCONDG

| end of once-per-frame processing

Go to VEC_LOOP

### I.5.2	Block 31SF – Set flags and scaling factor for frame erasure

This block is new. It is called at the beginning of a frame erasure, and every 10 ms thereafter during an erasure. At the beginning of an erasure it sets the voiced/unvoiced flag based on the value of PTAP. If the last good frame was voiced, it saves the pitch period KP in FEDELAY for periodic extrapolation. If unvoiced, it calculates the sum of magnitudes of the last 40 samples of ETPAST() in the last good frame, and then divides the result by 8. These values are computed once at the beginning of an erasure and are used for the whole erasure. If the erasure lasts more than 10 ms Block 31SF is called again (every 10 ms) and the excitation extrapolation scaling factor FESCALE is attenuated based on the length of the erasure and whether it is voiced or unvoiced. With voiced speech if the erasure lasts 60 ms or more, FESCALE is set to 0 to stop "beeping" artifacts. If the erasure is unvoiced FESCALE is set to 0 at 70 ms. The floating-point code is given first:

VOICEDFEGAIN(0:4) = .8, .8, .6, .4, .2

UNVOICEDFEGAIN(0:5) = 1., 1., .8, .6, .4, .2

N10MSEC = FECOUNT >> 2

If N10MSEC == 0, do the following indented lines

If PTAP > VTH, do the next 2 indented lines

FEDELAY = KP

VOICED = .TRUE.

Otherwise, do the following indented lines

VOICED = .FALSE.

AVMAG = 0.

For I = -39, -38, ..., 0, do the next line

AVMAG = AVMAG + |ETPAST(I)|	| sum of ETPAST() magnitudes

AVMAG = 0.125*AVMAG	| divide by 8

If VOICED = .TRUE., do the following indented lines

If N10MSEC < 5, do FESCALE = VOICEDFEGAIN(N10MSEC)

Otherwise, do FESCALE = 0.

Otherwise, do the following indented lines	| not voiced

If N10MSEC < 6, do FESCALE = AVMAG * UNVOICEDFEGAIN(N10MSEC)

Otherwise, do FESCALE = 0.

The fixed-point code is the following:

| Constants below are in Q15

VOICEDFEGAIN(0:4) = 26214, 26214, 19661, 13107, 6554

UNVOICEDFEGAIN(0:5) = 32767, 32767, 26214, 19661, 13107, 6554

N10MSEC = FECOUNT >> 2

If N10MSEC == 0, do the following indented lines

If PTAP > VTH, do the next 2 indented lines

FEDELAY = KP

VOICED = .TRUE.

Otherwise, do the following indented lines

VOICED = .FALSE.

AA0 = 0

For I = -39, -38, ..., 0, do the next line

AA0 = AA0 + |ETPAST(I)|	| sum of 40 ETPAST magnitudes

Call VSCALE(AA0,1,1,30,AA0,NLS)	| normalize numerator

AVMAG = RND(AA0)

NLSAVMAG = NLS - 16 + 3	| "+3" is for "divided by 8"

If VOICED = .TRUE., do the following indented lines

If N10MSEC < 5, do FESCALE = VOICEDFEGAIN(N10MSEC)

Otherwise, do FESCALE = 0

Otherwise, do the following indented lines	| not voiced

If N10MSEC < 6, do the following indented lines

FESCALE = UNVOICEDFEGAIN(N10MSEC)	| scaling factor in Q15

AA0 = FESCALE * AVMAG	| scale AVMAG by FESCALE

Call VSCALE(AA0,1,1,30,AA0,NLSFESCALE)	| normalize FESCALE

FESCALE = RND(AA0)	| save result in FESCALE

NLSFESCALE = NLSFESCALE + NLSAVMAG - 1	| NLS for FESCALE

| -1 is from (Q15 - 16)

Otherwise, do FESCALE = 0

### I.5.3	Block 31FE – Excitation signal extrapolation

This block is new and is used only during erased frames. It extrapolates the excitation signal in two possible ways, depending on whether the last good frame is considered voiced or not. For voiced speech the signal is extrapolated periodically with a decayed value. The scaling factor FESCALE starts at 0.8 and decreases every 10 ms during the erasure in Block 31SF.

For anything else (unvoiced, silence, etc.), a random segment from the previous 140 samples is used. This random segment is scaled so that it has the same average magnitude as the previous 40 samples. To calculate the average magnitudes of the 5-sample random segment and of the previous 40 samples, the sums of magnitudes should have been divided by 5 and 40, respectively. However, all we need is the ratio of the two average magnitudes. Thus, in practice, we calculate MAG as the sum of magnitudes of the 5-sample random segment, and AVMAG as the sum of magnitudes of the previous 40 samples divided by 8. The ratio of the two resulting numbers will still be the correct ratio we want. The value of AVMAG was calculated in Block 31SF above during the first adaptation cycle of the first erased frame. AVMAG is attenuated in Block 31SF based on the duration of the erasure and placed in FESCALE. Note that since interoperability with the G.728 encoder is not an issue during frame erasures there is no need to have a pre-defined sequence of random numbers. The numbers also do not have to be truly random. Any sequence of integers between 5 and 140 can be used here, as long as the sequence is not periodic or close to periodic. Therefore, no pseudo-code is given below for the random number generator.

We begin with the floating-point code.

|  Voiced case

If VOICED = .TRUE., do the next 2 indented lines

For I = 1, 2, ..., IDIM, do the next line

ET(I) = FESCALE * ETPAST(I-FEDELAY)	| KP is pitch period

| from 82

| Unvoiced case

If VOICED = .FALSE., do the following indented lines

set FEDELAY = a random number between 5 and 140

MAG = 0.

For I = 1, 2,  ..., IDIM, do the next 2 lines

ET(I) = ETPAST(I-FEDELAY)

MAG = MAG + |ET(I)|	| sum of ET() magnitudes

If MAG = 0 do MAG = 1.	| avoid divide by zero

RATIO = FESCALE / MAG

For I = 1, 2,  ..., IDIM, do the next line

ET(I) = RATIO*ET(I)	| scale to have same magnitude

For the fixed-point pseudo-code, ET and ETPAST are using different representations. As specified in Annex G/G.728, ET is represented in 15-bit block floating-point with one leading zero. On the other hand, to simplify the operation of this block, we use a fixed Q2 format to represent the long ETPAST array. There is no point to use block floating-point for ETPAST, because even the numerical error of the fixed Q2 representation is much smaller than the difference between the extrapolated excitation and the true excitation in the encoder. In the extremely rare case of ETPAST overflow, the value of ETPAST is clipped at the saturation level of 32767 or 32768 to avoid

"wrap-around." The following pseudo-code first writes the extrapolated excitation vector (in Q2) into ETPAST(1) through ETPAST(5), then converts the extrapolated 5 ETPAST samples into 15-bit block floating-point ET array.

If VOICED = .TRUE., do the next 4 lines	| Voiced case

TEMP = FESCALE

NLSTEMP = 15	| FESCALE is Q15

For I = 1, 2,  ..., IDIM, do the next line

ETPAST(I) = ETPAST(I-FEDELAY)| FEDELAY is pitch period

If VOICED = .FALSE., do the next 13 lines	| Unvoiced case

set FEDELAY = a random number between 5 and 140

AA1 = 0

For I = 1, 2,  ..., IDIM, do the next 2 lines

ETPAST(I) = ETPAST(I-FEDELAY)

AA1 = AA1 + |ETPAST(I)|	| sum of ETPAST(1:5) magnitudes

If AA1 = 0 or FESCALE = 0, do the next 2 lines

TEMP = 0

NLSTEMP = 15

Otherwise, do the next 4 lines

Call VSCALE(AA1,1,1,30,AA1,NLSDEN)	| normalize

| denominator

DEN = RND(AA1)

NLSDEN = NLSDEN - 16

Call DIVIDE(FESCALE,NLSFESCALE,DEN,NLSDEN,TEMP,NLSTEMP)

For I = 1, 2,  ..., IDIM, do the next 5 lines

AA0 = TEMP * ETPAST(I)	| AA0 in Q(2+NLSTEMP) format

AA0 = AA0 >> NLSTEMP	| shift AA0 to make Q2

if AA0 > 32767, set AA0 = 32767	| clip if necessary

if AA0 < -32768, set AA0 = -32768

ETPAST(I) = AA0 	| scaled ETPAST in Q2

Call VSCALE(ETPAST,IDIM,IDIM,13,ET,NLS)	| convert to ET in 15-bit

| BFL

NLSET = NLS + 2	| update NLSET

### I.5.4	Block 31E – Excitation signal update

This block updates the excitation signal buffer. Regardless of whether there is a frame erasure currently being concealed, this block must be completed so that the buffer will be ready when it is needed. We begin with the floating-point code.

For I = -KPMAX+1, -KPMAX+2, ..., -IDIM, do the next line

ETPAST(I) = ETPAST(I+IDIM)	| shift ETPAST by IDIM samples

For I = 1, 2, ..., IDIM, do the next line

ETPAST(I-IDIM) = ET(I)	| copy ET to ETPAST

For the fixed-point pseudo-code, ETPAST is stored in Q2 format, and ET is stored in 15-bit block floating-point with Q format NLSET. Therefore, a format conversion is needed when copying ET to ETPAST. Here is the fixed-point code.

For I = -KPMAX+1, -KPMAX+2, ..., -IDIM, do the next line

ETPAST(I) = ETPAST(I+IDIM)	| shift Q2 ETPAST by 5 samples

NRS = NLSET - 2	| # of right shifts to make Q2

For I = 1, 2, ..., IDIM, do the next 6 lines

AA0 = ET(I)	| get 15-bit mantissa of ET

if NRS > 0, AA0 >> NRS	| perform the shifting

if NRS < 0, AA0 << -NRS

if AA0 > 32767, set AA0 = 32767	| clip if necessary

if AA0 < -32768, set AA0 = -32768

ETPAST(I-IDIM) = AA0	| ETPAST is now Q2

### I.5.5	Block 43FE – Hybrid windowing module

In this subclause, both the floating-point and fixed-point pseudo-code for Block 43FE are given. This code has been modified for the case of frame erasure concealment and should be substituted for the original Block 43 described in 5.7/G.728. The only modification in the floating-point code involves only one more line of pseudo-code which skips the final autocorrelation function calculation in the event of a frame erasure. First, the floating-point pseudo-code is presented.

N1=LPCLG+NUPDATE	| compute some constants (can be

N2=LPCLG+NONRLG	| precomputed and stored in memory)

N3=LPCLG+NUPDATE+NONRLG

For N=1,2,...,N2, do the next line

SBLG(N)=SBLG(N+NUPDATE)	| shift the old signal buffer;

For N=1,2,...,NUPDATE, do the next line

SBLG(N2+N)=GTMP(N)	| shift in the new signal;

| SBLG(N3) is the newest sample

K=1

For N=N3,N3-1,...,3,2,1, do the next 2 lines

WS(N)=SBLG(N)*WNRLG(K)	| multiply the window function

K=K+1

For I=1,2,...,LPCLG+1, do the next 4 lines

TMP=0.

For N=LPCLG+1,LPCLG+2,...,N1, do the next line

TMP=TMP+WS(N)*WS(N+1-I)

REXPLG(I)=(3/4)*REXPLG(I)+TMP	| update the recursive

| component

If FERROR = .TRUE., skip all the lines below

For I=1,2,...,LPCLG+1, do the next 3 lines

R(I)=REXPLG(I)

For N=N1+1,N1+2,...,N3, do the next line

R(I)=R(I)+WS(N)*WS(N+1-I)	| add the non-recursive

| component

R(1)=R(1)*WNCF	| white noise correction

Now we give the fixed-point version of the same module. The only modification for the fixed-point code actually occurs in the module HWMCOREFE, which is substituted for HWMCORE. This fixed-point pseudo-code replaces the original Block 43 fixed-point code in G.3.14/G.728. The subroutine HWMCOREFE can be found with Block 49FE in I.5.7.

N1=LPCLG+NUPDATE (=10+4)	| compute some constants (can be

N2=LPCLG+NONRLG (=10+20)	| precomputed and stored in memory)

N3=LPCLG+NUPDATE+NONRLG (=10+4+20)

For N=1,2,...,N2, do the next line

SBLG(N)=SBLG(N+NUPDATE)	| shift the old signal buffer

For N=1,2,..,NUPDATE, do the next line

SBLG(N2+N)=GTMP(N)	| SBLG(N3) is the newest sample

| all SBLG are Q9 and represented

| in 16 bits precision

Call FINDNLS(SBLG,N3,N3,14,NLS)	| find the amount of left shifts

NLSTMP=NLS-1	| needed in the next loop for 2

| bits of headroom later

K=1

For N=34,33,...,1, do the next 5 lines

P=SBLG(N)*WNRLG(K)	| WNRLG is Q15

If NLSTMP = -1, set AA0=P >> 1

If NLSTMP > -1, set AA0=P << NLSTMP

WS(N)=RND(AA0)	| WS(N) is 14 bits or less

K=K+1

NLSATTLG = 14

Call HWMCOREFE(LPCLG,N1,N3,NLSATTLG,WS,NLSTMP,REXPLG,NLSREXPLG,R,ILLCOND)

### I.5.6	Block 47AF – Log-gain limiter after frame erasure

This new block should replace Block 47 in 5.7/G.728. This is the floating-point pseudo-code.

If GAIN < 0., set GAIN = 0.    | Correspond to linear gain 1.

If GAIN > 60., set GAIN = 60.  | Correspond to linear gain 1000.

TMP = GAIN - OGAINDB

If AFTERFE > 0 and TMP > FEGAINMAX, do the next line

GAIN = OGAINDB + FEGAINMAX

### I.5.7	Block 49FE – Hybrid window module for synthesis filter

This is the block for the hybrid window module for the synthesis filter as modified for the case of frame erasures. This block should be substituted for the previous Block 49 described in 5.6/G.728. We begin with the floating-point pseudo-code for the hybrid windowing module.

N1=LPC+NFRSZ	| compute some constants (can be

N2=LPC+NONR	| precomputed and stored in memory)

N3=LPC+NFRSZ+NONR

For N=1,2,...,N2, do the next line

SB(N)=SB(N+NFRSZ)	| shift the old signal buffer;

For N=1,2,...,NFRSZ, do the next line

SB(N2+N)=STTMP(N)	| shift in the new signal;

| SB(N3) is the newest sample

K=1

For N=N3,N3-1,...,3,2,1, do the next 2 lines

WS(N)=SB(N)*WNR(K)	| multiply the window function

K=K+1

For I=1,2,...,LPC+1, do the next 4 lines

TMP=0.

For N=LPC+1,LPC+2,...,N1, do the next line

TMP=TMP+WS(N)*WS(N+1-I)

REXP(I)=(3/4)*REXP(I)+TMP	| update the recursive component

If FERROR = .TRUE., do the next 4 lines

For I=1,2,...,11, do the next 3 lines

RTMP(I)=REXP(I)

For N=N1+1,N1+2,...,N3, do the next line

RTMP(I)=RTMP(I)+WS(N)*WS(N+1-I)	| add non-recursive

| component

If FERROR = .FALSE., do the next 4 lines

For I=1,2,...,LPC+1, do the next 3 lines

RTMP(I)=REXP(I)

For N=N1+1,N1+2,...,N3, do the next line

RTMP(I)=RTMP(I)+WS(N)*WS(N+1-I)	| add non-recursive

| component

RTMP(1)=RTMP(1)*WNCF	| white noise correction

The fixed-point pseudo-code for the hybrid windowing module (Block 49FE) is modified in a similar fashion. This time the change is actually found in HWMCOREFE. This fixed-point pseudo-code replaces the original Block 49 fixed-point code in G.3.17/G.728.

N1=LPC+NFRSZ(=70)	| compute some constants (can be

N2=LPC+NONR(=85)	| precomputed and stored in memory)

N3=LPC+NFRSZ+NONR(=105)

N4=N3/IDIM(=21)

N5=NFRSZ/IDIM(=4)

N6=N4-N5(=17)

For N=1,2,...,N2, do the next line

SB(N)=SB(N+NFRSZ)	| shift old part of buffer SB

For N=1,2,...,N6, do the next line

NLSSB(N)=NLSSB(N+N5)	| shift old NLSSB

For N=1,2,...,NFRSZ, do the next line

SB(N2+N)=STTMP(N)	| shift in new part of SB

For N=1,2,...,N5, do the next line

NLSSB(N6+N)=NLSSTTMP(N)	| shift in new NLSSB

| Now find the minimum NLSSB - this determines NLSTMP

NLSTMP=Min{NLSSB(1),NLSSB(2),...,NLSSB(N4)}

K=1	| now multiply SB by the

N=N3	| hybrid window function

For J=1,2,...,N4, do the next 8 lines

NRSH=NLSSB(J)-NLSTMP-1	| -1 to compensate for Q15 mult

For M=1,2,...,IDIM, do the next 5 lines

P=SB(K)*WNR(N)	| WNR is Q15 multiplication

If NRSH = -1, set AA0=P << 1

If NRSH > -1, set AA0=P >> NRSH

WS(K)=RND(AA0)	| round upper word & store in WS

N=N-1

K=K+1

NLSATT50 = 14

Call HWMCOREFE(LPC,N1,N3,NLSATT50,WS,NLSTMP,REXP,NLSREXP,RTMP,ILLCOND)

The fixed-point code for HWMCOREFE has been modified so that in the event of a frame erasure, only the values of RTMP(1), ..., RTMP(11) are calculated, although REXP(1), ..., REXP(51) are updated.

SUBROUTINE HWMCOREFE(LPO,N1,N3,NLSATT,WS,NLSTMP,

RREC,NLSRREC,R,ILLCOND)

NLSAA0=2*NLSTMP

AA0=0	| compute recursive part of RREC(1)

For N=LPO+1,...,N1, do the next 2 lines

P=WS(N)*WS(N)	| WS has 2 bits of headroom

AA0=AA0+P	| AA0 will have 5 bits of headroom

| for energy calculation

| Case 1: NLSRREC > NLSAA0

If NLSRREC > NLSAA0, do the next 22 lines

AA0=AA0 >> 1

IR=NLSRREC-NLSAA0+1

AA1=RREC(1) << NLSATT	| this can be done by multiplication

AA1=-AA1+(RREC(1) << 16)	| scale RREC by attenuation factor

AA1=AA1 >> IR	| align AA0 & AA1

AA0=AA0+AA1

Call VSCALE(AA0,1,1,30,AA0,NLSRE)	| Find NLS for RREC

RREC(1)=RND(AA0)	| upper 16 bits of AA1 saved

NLSRREC=NLSAA0-1+NLSRE

For I=1,2,...,LPO, do the next 11 lines

AA0=0	| compute recursive part of RREC(I+1)

For N=LPO+1,...,N1, do the next 2 lines

P=WS(N)*WS(N-I)

AA0=AA0+P

AA0=AA0 >> 1

AA1=RREC(I+1) << NLSATT	| scale RREC by 3/4 or 1/2

AA1=-AA1+(RREC(I+1) << 16)	|

AA1=AA1 >> IR

AA0=AA0+AA1

AA0=AA0 << NLSRE

RREC(I+1)=RND(AA0)	| upper 16 bits of AA0 saved

Go to FIN_RECUR

| Case 2: NLSRREC = NLSAA0

If NLSRREC = NLSAA0, do the next 21 lines

AA1=RREC(1) << NLSATT	| scale RREC by 3/4 or 1/2

AA1=-AA1+(RREC(1) << 16)	|

AA0=AA0 >> 1

AA1=AA1 >> 1

AA0=AA0+AA1

Call VSCALE(AA0,1,1,30,AA0,NLSRE)	| Find NLS for RREC

RREC(1)=RND(AA0)	| upper 16 bits of AA1 saved

NLSRREC=NLSRREC-1+NLSRE

For I=1,2,...,LPO, do the next 11 lines

AA0=0	| compute recursive part of RREC(I+1)

For N=LPO+1,...,N1, do the next 2 lines

P=WS(N)*WS(N-I)

AA0=AA0+P

AA0=AA0 >> 1

AA1=RREC(I+1) << NLSATT	| scale RREC by 3/4 or 1/2

AA1=-AA1+(RREC(I+1) << 16)	|

AA1=AA1 >> 1

AA0=AA0+AA1

AA0=AA0 << NLSRE

RREC(I+1)=RND(AA0)	| upper 16 bits of AA0 saved

Go to FIN_RECUR

| Case 3: NLSRREC < NLSAA0

If NLSRREC < NLSAA0, do the next 21 lines

IR=NLSAA0-NLSRREC+1

AA0=AA0 >> IR

AA1=RREC(1) << NLSATT	| scale RREC by 3/4 or 1/2

AA1=-AA1+(RREC(1) << 16)	|

AA1=AA1 >> 1

AA0=AA0+AA1

Call VSCALE(AA0,1,1,30,AA0,NLSRE)

RREC(1)=RND(AA0)	| upper 16 bits of AA1 saved

NLSRREC=NLSRREC-1+NLSRE

For I=1,2,...,LPO, do the next 11 lines

AA0=0	| compute recursive part of RREC(I+1)

For N=LPO+1,...,N1, do the next 2 lines

P=WS(N)*WS(N-I)

AA0=AA0+P

AA0=AA0 >> IR

AA1=RREC(I+1) << NLSATT	| scale RREC by 3/4 or 1/2

AA1=-AA1+(RREC(I+1) << 16)	|

AA1=AA1 >> 1

AA0=AA0+AA1

AA0=AA0 << NLSRE

RREC(I+1)=RND(AA0)	| upper 16 bits of AA0 saved

FIN_RECUR:	| when you reach this point the

| recursive component has been computed

AA0=0	| compute non-recursive part of R(1)

For N=N1+1,...,N3, do the next 2 lines

P=WS(N)*WS(N)

AA0=AA0+P

LPFE = LPO

If FERROR .=. TRUE, set LPFE = 10

| Case 1: NLSRREC > NLSAA0

If NLSRREC > NLSAA0, do the next 21 lines

IR=NLSRREC-NLSAA0+1

AA1=RREC(1) << 16

AA1=AA1 >> IR

AA0=AA0 >> 1

AA1=AA0+AA1

AA0=AA1 >> 8	| apply white noise correction factor

AA1=AA1+AA0

Call VSCALE(AA1,1,1,30,AA1,NLSRR)

R(1)=RND(AA1)	| upper 16 bits of AA1 saved

For I=1,2,...,LPFE, do the next 10 lines

AA0=0	| compute non-recursive part of R(I+1)

For N=N1+1,...,N3, do the next 2 lines

P=WS(N)*WS(N-I)

AA0=AA0+P

AA0=AA0 >> 1

AA1=RREC(I+1) << 16

AA1=AA1 >> IR

AA1=AA0+AA1

AA1=AA1 << NLSRR

R(I+1)=RND(AA1)	| save upper 16 bits

Go to END

| Case 2: NLSRREC = NLSAA0

If NLSRREC = NLSAA0, do the next 18 lines

AA0=AA0 >> 1

AA1=RREC(1) << 15	| this can be done by multiplication

AA1=AA0+AA1

AA0=AA1 >> 8	| apply white noise correction factor

AA1=AA1+AA0

Call VSCALE(AA1,1,1,30,AA1,NLSRR)

R(1)=RND(AA1)	| upper 16 bits of AA1 saved

For I=1,2,...,LPFE, do the next 9 lines

AA0=0	| compute non-recursive part of R(I+1)

For N=N1+1,...,N3, do the next 2 lines

P=WS(N)*WS(N-I)

AA0=AA0+P

AA0=AA0 >> 1

AA1=RREC(I+1) << 15

AA1=AA0+AA1

AA1=AA1 << NLSRR

R(I+1)=RND(AA1)	| save upper 16 bits

Go to END

| Case 3: NLSRREC < NLSAA0

If NLSRREC < NLSAA0, do the next 18 lines

IR=NLSAA0-NLSRREC+1

AA0=AA0 >> IR

AA1=RREC(1) << 15	| this can be done by multiplication

AA1=AA0+AA1

AA0=AA1 >> 8	| apply white noise correction factor

AA1=AA1+AA0

Call VSCALE(AA1,1,1,30,AA1,NLSRR)

R(1)=RND(AA1)	| upper 16 bits of AA1 saved

For I=1,2,...,LPFE, do the next 9 lines

AA0=0	| compute non-recursive part of R(I+1)

For N=N1+1,...,N3, do the next 2 lines

P=WS(N)*WS(N-I)

AA0=AA0+P

AA0=AA0 >> IR

AA1=RREC(I+1) << 15

AA1=AA0+AA1

AA1=AA1 << NLSRR

R(I+1)=RND(AA1)	| save upper 16 bits

END:	| one last job, check for ill-conditioning

ILLCOND=.FALSE.

If AA1 = 0, set ILLCOND=.TRUE.	| AA1 still contains 32 bit R(LPFE+1)

### I.5.8	Block 51FE – Bandwidth expansion module for frame erasures

This is the floating-point pseudo-code for Block 51FE, the bandwidth expansion module for the LPC synthesis filter for frame erasure conditions. This module is similar to Block 51 in 5.6/G.728, except that the bandwidth broadening coefficients are different. Also, the input values are the A(I) array.

If FECOUNT = 1 and ILLCOND = .FALSE., do the next 2 lines

For I=2,3,...,LPC+1, do the next line

A(I)=FACVFE(I) * ATMP(I)              | scale coeff.

Otherwise, do the next 2 lines

For I=2,3,...,LPC+1, do the next line

A(I)=FACVFE(I) * A(I)                 | scale coeff.

The tables for FACVFE are given in Q14 format. The values for the input A array are always in Q14 format. The final values of A can always be represented in Q14 format. Thus, a shift is necessary after the multiplication. This is the fixed-point pseudo-code for Block 51.

If FECOUNT = 1 and ILLCOND = .FALSE., do the next 4 lines

For I=2,3,4,...,LPC+1, do the next 3 lines

AA0 = FACVFE(I) * ATMP(I)	| AA0 is Q28

AA0 = AA0 << 2	| make AA0 Q30

A(I) = RND(AA0)	| round to high word and get Q14

Otherwise, do the next 4 lines

For I=2,3,4,...,LPC+1, do the next 3 lines

AA0 = FACVFE(I) * A(I)	| AA0 is Q28

AA0 = AA0 << 2	| make AA0 Q30

A(I) = RND(AA0)	| round to high word and get Q14

### I.5.9	Block 97FE – Update GSTATE during frame erasures

We begin with the floating-point version of this new block. This is the same code as Blocks 67, 39, 40 and 42 described in 5.7/G.728.

ETRMS = 0.

For K = 1,...,IDIM, do the next line	|  compute energy

ETRMS = ETRMS + ET(K)*ET(K)	|  of excitation

ETRMS = ETRMS * DIMINV

If ETRMS < 1., set ETRMS = 1.	|  check lower limit

ETRMS = 10.* log10 (ETRMS)	|  compute dB value

GSTATE(1) = ETRMS - GOFF	| subtract dB gain offset

The fixed-point version of this new block performs the equivalent operations. The log10 function was eliminated from the original fixed-point specification of G.728. Now it must be re-introduced in order to perform the above calculation. Because we are not trying to exactly match the output of the floating-point specification, we do not need the greatest possible precision for the output of log10. Here is the fixed-point code. Also, because bit exact compatibility is not required for frame erasures, we have not included a bit exact description of the LIN2DB function for fixed-point implementation. This LIN2DB convert a scalar floating-point representation of ETRMS to the dB value represented in Q9. Any fixed-point implementation of LIN2DB with reasonable accuracy can be used here. An example of such a LIN2DB fixed-point code is included here.

AA0 = 0

For K = 1,...,IDIM, do the next line	|  compute energy of excitation

AA0 = AA0 + ET(K)*ET(K)	|  ET is Q2, so AA0 is Q4

Call VSCALE(AA0,1,1,30,AA0,NLS)	|  scale AA0

ETRMS = AA0 >> 16	|  take high word of AA0

NLSETRMS = (4 + NLS) - 16	|  NLS of ETRMS

AA0 = ETRMS * DIMINV	|  DIMINV = 0.2 in Q16

Call VSCALE(AA0,1,1,30,AA0,NLS)	|  scale AA0

ETRMS = AA0 >> 16	|  take high word of AA0

NLSETRMS = NLSETRMS + NLS	|  NLS of ETRMS

If NLSETRMS > 14, do the next 2 lines	|  if ETRMS is Q15 or

|  smaller (<1)

ETRMS = 16384	|  set ETRMS to 1 in Q16

NLSETRMS = 14

Call LIN2DB(ETRMS,NLSETRMS,GSTATE(1))	| convert linear to dB (fixed

| Q9)

| gain offset subtraction is done

| inside LIN2DB

The constants needed for the LIN2DB fixed-point code is listed below.

c(0)			  3 400		0.4150374993 in Q13

c(1)			 15 758		1.9235933879 in Q13

c(2)			10 505		1.2823955919 in Q13

c(3)			   9 338		1.1399071928 in Q13

c(4)			 9 338		1.1399071928 in Q13

c(5)			   9 961		1.2159010057 in Q13

FAC1			 24 660		3.0102999566 in Q13

GOFF			16 384		32 in Q9

THRQTR		 24 576		0.75 in Q15

The LIN2DB fixed-point code is given below. It calculates a truncated Taylor series expansion of log2 function around 0.75. It assumes the input X is a positive normalized 16-bit mantissa with NLSX as the Q format. It performs the necessary scaling to get the dB value and subtracts the gain offset GOFF from the resulting 32-bit number before converting to the Q9 fixed Q format output.

LIN2DB(X, NLSX, RET)

AA0 = x	| normalized .5 <= x <= 1, Q15

AA0 = AA0 - THRQTR	| subtract .75, now -.25 <= x .25, Q15

AA0 = AA0 << 1	| convert to Q16

AA1 = 0

For I=5,4...,1 do the next 3 lines

AA1 = AA1 + C(I)	| Q13

P = AA1 * AA0	| Q13 * Q16 = Q29

AA1 = P >> 16	| Q29 -> Q13

AA1 = AA1 + C(0)	| Q13

AA1 = AA1 >> 3	| convert to Q10

AA0 = 15 - NLSX	| exponent in Q0

AA0 = AA0 << 10	| convert exponent to Q10

AA1 = AA0 + AA1	| Add exp to log of mantissa, AA1 now log2(x)

AA1 = AA1 * FAC1	| Q10 * Q13 = Q23, convert to 10*log10(x)

AA1 = AA1 >> 14	| Q23 -> Q9

AA1 = AA1 - GOFF	| Subtract GOFF

RET = AA1	| return Q9 value

### I.5.10	Block 98AF – Log-gain limiter after frame erasure

This new block should replace Block 98 in G.3.16/G.728. This is the fixed-point pseudo-code for Block 98AF.

If LOGGAIN > 14336, set LOGGAIN=14336

If LOGGAIN < -16384, set LOGGAIN=-16384

TMP = LOGGAIN - OGAINDB

If AFTERFE > 0 and TMP > FEGAINMAX, do the next line

LOGGAIN = OGAINDB + FEGAINMAX

## I.6	Additional coder parameters and variables

This clause contains a table with the new variables that have been added to the G.728 decoder for frame erasure concealment. Under the format column, we list the fixed-point format. For a floating-point implementation, all block floating-point or fixed Q format variables can be represented in floating-point.

Table I.1/G.728 – Additional coder parameters needed for frame erasure concealment

| Name | Floating- point value | Fixed- point value | Q format | Description |
| --- | --- | --- | --- | --- |
| AFTERFEMAX | 16 | 16 | Q0 | Maximum 2.5 ms frames for gain clipping |
| DIMINV | 0.2 | 13 107 | Q16 | Inverse of vector dimension |
| FEGAINMAX | 2 | 1 024 | Q9 | Maximum gain DB increase after frame erasure |
| VTH | 0.4285714 | 7 022 | Q14 | Voicing threshold for frame erasures |
| FESCALE | 0.8 | 26 214 | Q15 | ETPAST scaling factor for frame erasures |

Table I.2/G.728 – Additional LD-CELP internal processing variables
needed for frame erasure concealment

| Name | Array index range | Fixed-point format | Description |
| --- | --- | --- | --- |
| MAG | 1 | a) | Average magnitude of current excitation vector |
| ADCOUNT | 1 | Integer | G.728 Adaptation cycle counter (2.5 ms) |
| AFTERFE | 1 | Integer | Counter for gain clipping after FE (2.5 ms) |
| AVMAG | 1 | SFL | Average magnitude of past 8 excitation vectors |
| ETPAST | 139 to IDIM | Q2 | Past excitation (initially zero) |
| ETRMS | 1 | SFL | Energy in dB of current vector |
| FECOUNT | 1 | Integer | Length of current frame erasure (2.5 ms) |
| FEDELAY | 1 | Integer | Pitch (KP) stored at beginning of FE |
| FERROR | 1 | Logical | Frame erasure indicator |
| FESCALE | 1 | Integer | ETPAST scaling factor for frame erasures |
| FESIZE | 1 | Integer | Number of 2.5 ms Adaptation cycles in a FE |
| NLSFESCALE | 1 | Integer | NLS for FESCALE |
| NLSAVMAG | 1 | Integer | NLS for AVGMAG |
| NLSETRMS | 1 | Integer | NLS for ETRMS |
| OGAINDB | 1 | Q9 | Old prediction gain in dB |
| VOICED | 1 | Logical | Voiced/Unvoiced flag |
| a) MAG is not used in the fixed-point pseudo-code. |  |  |  |

Appendix I.i

Values used for scaling ETPAST during frame erasures

Tables I.I.1 and I.I.2 give the integer and floating-point values used to scale ETPAST for both voiced (VOICEDFEGAIN) and non-voiced (UNVOICEDFEGAIN) signals during frame erasures. For the fixed-point these are in Q15 format.

Table I.I.1/G.728 – Values used to scale ETPAST with 
voiced signals (VOICEDFEGAIN)

| VOICEDFEGAIN index | Floating-point value | Fixed-point value Q15 |
| --- | --- | --- |
| 0 | 0.8 | 26 214 |
| 1 | 0.8 | 26 214 |
| 2 | 0.6 | 19 661 |
| 3 | 0.4 | 13 107 |
| 4 | 0.2 | 6 554 |

Table I.I.2/G.728 – Values used to scale ETPAST with 
non-voiced signals (UNVOICEDFEGAIN)

| VOICEDFEGAIN index | Floating-point value | Fixed-point value Q15 |
| --- | --- | --- |
| 0 | 1.0 | 32 767 |
| 1 | 1.0 | 32 767 |
| 2 | 0.8 | 26 214 |
| 3 | 0.6 | 19 661 |
| 4 | 0.4 | 13 107 |
| 5 | 0.2 | 6 554 |

Values used for bandwidth expansion of LPC predictor during frame erasures

The following table gives the integer values for the bandwidth expansion coefficient array FACVFE used to soften the LPC predictor during frame erasures. These values are in Q14 format. To obtain the equivalent floating-point values, divide the integer values by 16384. Starting at FACVFE(1) and ending at FACVFE(51), this table should be read from left to right, and then from top to bottom.

| 16384 | 15892 | 15416 | 14953 | 14505 |
| --- | --- | --- | --- | --- |
| 14069 | 13647 | 13238 | 12841 | 12456 |
| 12082 | 11719 | 11368 | 11027 | 10696 |
| 10375 | 10064 | 9762 | 9469 | 9185 |
| 8910 | 8642 | 8383 | 8131 | 7888 |
| 7651 | 7421 | 7199 | 6983 | 6773 |
| 6570 | 6373 | 6182 | 5996 | 5816 |
| 5642 | 5473 | 5309 | 5149 | 4995 |
| 4845 | 4700 | 4559 | 4422 | 4289 |
| 4161 | 4036 | 3915 | 3797 | 3683 |
| 3573 |  |  |  |  |

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |