|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.729

TELECOMMUNICATION	(03/96)
STANDARDIZATION  SECTOR
OF  ITU

GENERAL  ASPECTS  OF  DIGITAL  TRANSMISSION
SYSTEMS

CODING  OF  SPEECH  AT  8  kbit/s
USING  CONJUGATE-STRUCTURE
ALGEBRAIC-CODE-EXCITED
LINEAR-PREDICTION  (CS-ACELP)

ITU-T  Recommendation  G.729

(Previously “CCITT  Recommendation”)

FOREWORD

The ITU-T (Telecommunication Standardization Sector) is a permanent organ of the International Telecommunication Union (ITU). The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1 (Helsinki, March 1-12, 1993).

ITU-T Recommendation G.729 was prepared by ITU-T Study Group 15 (1993-1996) and was approved under the WTSC Resolution No. 1 procedure on the 19th of March 1996.

___________________

NOTE

In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1996

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Recommendation G.729     (03/96)

Page

1	Introduction		1

2	General description of the coder		1

2.1	Encoder		2

2.2	Decoder		3

2.3	Delay		4

2.4	Speech coder description		4

2.5	Notational conventions		4

3	Functional description of the encoder		7

3.1	Pre-processing		7

3.2	Linear prediction analysis and quantization		7

3.3	Perceptual weighting		14

3.4	Open-loop pitch analysis		15

3.5	Computation of the impulse response		16

3.6	Computation of the target signal		16

3.7	Adaptive-codebook search		17

3.8	Fixed codebook – Structure and search		19

3.9	Quantization of the gains		22

3.10	Memory update		24

4	Functional description of the decoder		25

4.1	Parameter decoding procedure		25

4.2	Post-processing		28

4.3	Encoder and decoder initialization		30

4.4	Concealment of frame erasures		30

5	Bit-exact description of the CS-ACELP coder		32

5.1	Use of the simulation software		32

5.2	Organization of the simulation software		32

Recommendation G.729

Recommendation G.729     (03/96)

CODING  OF  SPEECH  AT  8  kbit/s  USING  CONJUGATE-STRUCTURE
ALGEBRAIC-CODE-EXCITED  LINEAR-PREDICTION  (CS-ACELP)

(Geneva, 1996)

# 1	Introduction

This Recommendation contains the description of an algorithm for the coding of speech signals at 8 kbit/s using Conjugate-Structure Algebraic-Code-Excited Linear-Prediction (CS-ACELP).

This coder is designed to operate with a digital signal obtained by first performing telephone bandwidth filtering (Recommendation G.712) of the analogue input signal, then sampling it at 8000 Hz, followed by conversion to 16-bit linear PCM for the input to the encoder. The output of the decoder should be converted back to an analogue signal by similar means. Other input/output characteristics, such as those specified by Recommendation G.711 for 64 kbit/s PCM data, should be converted to 16-bit linear PCM before encoding, or from 16-bit linear PCM to the appropriate format after decoding. The bitstream from the encoder to the decoder is defined within this Recommendation.

This Recommendation is organized as follows: Clause 2 gives a general outline of the CS-ACELP algorithm. In clauses 3 and 4, the CS-ACELP encoder and decoder principles are discussed, respectively. Clause 5 describes the software that defines this coder in 16 bit fixed-point arithmetic.

# 2	General description of the coder

The CS-ACELP coder is based on the Code-Excited Linear-Prediction (CELP) coding model. The coder operates on speech frames of 10 ms corresponding to 80 samples at a sampling rate of 8000 samples per second. For every 10 ms frame, the speech signal is analysed to extract the parameters of the CELP model (linear-prediction filter coefficients, adaptive and fixed-codebook indices and gains). These parameters are encoded and transmitted. The bit allocation of the coder parameters is shown in Table 1. At the decoder, these parameters are used to retrieve the excitation and synthesis filter parameters. The speech is reconstructed by filtering this excitation through the short-term synthesis filter, as is shown in Figure 1. The short-term synthesis filter is based on a 10th order Linear Prediction (LP) filter. The long-term, or pitch synthesis filter is implemented using the so-called adaptive-codebook approach. After computing the reconstructed speech, it is further enhanced by a postfilter.

TABLE  1/G.729

Bit allocation of the 8 kbit/s CS-ACELP algorithm (10 ms frame)XE "Bit allocation of the 8 kbit/s CS-ACELP algorithm (10 ms frame)"§

| Parameter | Codeword | Subframe 1 | Subframe 2 | Total per frame |
| --- | --- | --- | --- | --- |
| Line spectrum pairs | L0, L1, L2, L3 |  |  | - 18 |
| - Adaptive-codebook delay | P1, P2 | - 8 | - 5 | - 13 |
| - Pitch-delay parity | P0 | - 1 |  | - 1 |
| - Fixed-codebook index | C1, C2 | - 13 | - 13 | - 26 |
| - Fixed-codebook sign | S1, S2 | - 4 | - 4 | - 8 |
| - Codebook gains (stage 1) | GA1, GA2 | - 3 | - 3 | - 6 |
| - Codebook gains (stage 2) | GB1, GB2 | - 4 | - 4 | - 8 |
| - Total |  |  |  | - 80 |

FIGURE 1/G.729...[D01] = 5 CM

## 2.1	Encoder

The encoding principle is shown in Figure 2. The input signal is high-pass filtered and scaled in the pre-processing block. The pre-processed signal serves as the input signal for all subsequent analysis. LP analysis is done once per 10 ms frame to compute the LP filter coefficients. These coefficients are converted to Line Spectrum Pairs (LSP) and quantized using predictive two-stage Vector Quantization (VQ) with 18 bits. The excitation signal is chosen by using an analysis-by-synthesis search procedure in which the error between the original and reconstructed speech is minimized according to a perceptually weighted distortion measure. This is done by filtering the error signal with a perceptual weighting filter, whose coefficients are derived from the unquantized LP filter. The amount of perceptual weighting is made adaptive to improve the performance for input signals with a flat frequency-response.

The excitation parameters (fixed and adaptive-codebook parameters) are determined per subframe of 5 ms (40 samples) each. The quantized and unquantized LP filter coefficients are used for the second subframe, while in the first subframe interpolated LP filter coefficients are used (both quantized and unquantized). An open-loop pitch delay is estimated once per 10 ms frame based on the perceptually weighted speech signal. Then the following operations are repeated for each subframe. The target signal x(n) is computed by filtering the LP residual through the weighted synthesis filter W(z)/Â(z). The initial states of these filters are updated by filtering the error between LP residual and excitation. This is equivalent to the common approach of subtracting the zero-input response of the weighted synthesis filter from the weighted speech signal. The impulse response h(n) of the weighted synthesis filter is computed. Closed-loop pitch analysis is then done (to find the adaptive-codebook delay and gain), using the target x(n) and impulse response h(n), by searching around the value of the open-loop pitch delay. A fractional pitch delay with 1/3 resolution is used. The pitch delay is encoded with 8 bits in the first subframe and differentially encoded with 5 bits in the second subframe. The target signal x(n) is updated by subtracting the (filtered) adaptive-codebook contribution, and this new target, x¢(n), is used in the fixed-codebook search to find the optimum excitation. An algebraic codebook with 17 bits is used for the fixed-codebook excitation. The gains of the adaptive and fixed-codebook contributions are vector quantized with 7 bits, (with MA prediction applied to the fixed-codebook gain). Finally, the filter memories are updated using the determined excitation signal.

FIGURE 2/G.729...[D02] = 16 CM

## 2.2	Decoder

The decoder principle is shown in Figure 3. First, the parameter’s indices are extracted from the received bitstream. These indices are decoded to obtain the coder parameters corresponding to a 10 ms speech frame. These parameters are the LSP coefficients, the two fractional pitch delays, the two fixed-codebook vectors, and the two sets of adaptive and fixed-codebook gains. The LSP coefficients are interpolated and converted to LP filter coefficients for each subframe. Then, for each 5 ms subframe the following steps are done:

·	the excitation is constructed by adding the adaptive and fixed-codebook vectors scaled by their respective gains;

·	the speech is reconstructed by filtering the excitation through the LP synthesis filter;

·	the reconstructed speech signal is passed through a post-processing stage, which includes an adaptive postfilter based on the long-term and short-term synthesis filters, followed by a high-pass filter and scaling operation.

FIGURE 3/G.729...[D03] = 7 CM

## 2.3	Delay

This coder encodes speech and other audio signals with 10 ms frames. In addition, there is a look-ahead of 5 ms, resulting in a total algorithmic delay of 15 ms. All additional delays in a practical implementation of this coder are due to:

·	processing time needed for encoding and decoding operations;

·	transmission time on the communication link;

·	multiplexing delay when combining audio data with other data.

## 2.4	Speech coder description

The description of the speech coding algorithm of this Recommendation is made in terms of bit-exact, fixed-point mathematical operations. The ANSI C code indicated in clause 5, which constitutes an integral part of this Recommendation, reflects this bit-exact, fixed-point descriptive approach. The mathematical descriptions of the encoder (clause 3), and decoder (clause 4), can be implemented in several other fashions, possibly leading to a codec implementation not complying with this Recommendation. Therefore, the algorithm description of the ANSI C code of clause 5 shall take precedence over the mathematical descriptions of clauses 3 and 4 whenever discrepancies are found. A non-exhaustive set of test signals, which can be used with ANSI C code, are available from the ITU.

## 2.5	Notational conventions

Throughout this Recommendation, it is tried to maintain the following notational conventions:

·	Codebooks are denoted by caligraphic characters (e.g. C).

·	Time signals are denoted by their symbol and a sample index between parenthesis [e.g. s(n)]. The symbol n is used as sample index.

·	Superscript indices between parenthesis (e.g. g(m) are used to indicate time-dependency of variables. The variable m refers, depending on the context, to either a frame or subframe index, and the variable n to a sample index.

·	Recursion indices are identified by a superscript between square brackets (e.g. E[k]).

·	Subscripts indices identify a particular element in a coefficient array.

·	The symbol ^ identifies a quantized version of a parameter (e.g.  ).

·	Parameter ranges are given between square brackets, and include the boundaries (e.g. [0.6, 0.9]).

·	The function log denotes a logarithm with base 10.

·	The function int denotes truncation to its integer value.

·	The decimal floating-point numbers used are rounded versions of the values used in the 16 bit fixed-point ANSI C implementation.

Table 2 lists the most relevant symbols used throughout this Recommendation. A glossary of the most relevant signals is given in Table 3. Table 4 summarizes relevant variables and their dimension. Constant parameters are listed in Table 5. The acronyms used in this Recommendation are summarized in Table 6.

TABLE  2/G.729

Glossary of most relevant symbols

| Name | Reference | Description |
| --- | --- | --- |
| 1/Â(z) | Equation (2) | LP synthesis filter |
| Hh1(z) | Equation (1) | Input high-pass filter |
| Hp(z) | Equation (78) | Long-term postfilter |
| Hf(z) | Equation (84) | Short-term postfilter |
| Ht(z) | Equation (86) | Tilt-compensation filter |
| Hh2(z) | Equation (91) | Output high-pass filter |
| P(z) | Equation (46) | Pre-filter for fixed codebook |
| W(z) | Equation (27) | Weighting filter |

TABLE  3/G.729

Glossary of most relevant signals

| Name | - Reference | Description |
| --- | --- | --- |
| c(n) | - 3.8 | Fixed-codebook contribution |
| d(n) | - 3.8.1 | Correlation between target signal and h(n) |
| ew(n) | - 3.10 | Error signal |
| h(n) | - 3.5 | Impulse response of weighting and synthesis filters |
| r(n) | - 3.6 | Residual signal |
| s(n) | - 3.1 | Pre-processed speech signal |
|  | - 4.1.6 | Reconstructed speech signal |
| s¢(n) | - 3.2.1 | Windowed speech signal |
| sf(n) | - 4.2 | Postfiltered output |
| sf¢(n) | - 4.2 | Gain-scaled postfiltered output |
| sw(n) | - 3.6 | Weighted speech signal |
| x(n) | - 3.6 | Target signal |
| x¢(n) | - 3.8.1 | Second target signal |
| u(n) | - 3.10 | Excitation to LP synthesis filter |
| v(n) | - 3.7.1 | Adaptive-codebook contribution |
| y(n) | - 3.7.3 | Convolution v(n) * h(n) |
| z(n) | - 3.9 | Convolution c(n) * h(n) |

TABLE  4/G.729

Glossary of most relevant variables

| Name | - Size | Description |
| --- | --- | --- |
| gp | - 1 | Adaptive-codebook gain |
| gc | - 1 | Fixed-codebook gain |
| gl | - 1 | Gain term for long-term postfilter |
| gf | - 1 | Gain term for short-term postfilter |
| gt | - 1 | Gain term for tilt postfilter |
| G | - 1 | Gain for gain normalization |
| Top | - 1 | Open-loop pitch delay |
| ai | - 11 | LP coefficients (a0 = 1.0) |
| ki | - 10 | Reflection coefficients |
|  | - 1 | Reflection coefficient for tilt postfilter |
| oi | - 2 | LAR coefficients |
| wi | - 10 | LSF normalized frequencies |
|  | - 40 | MA predictor for LSF quantization |
| qi | - 10 | LSP coefficients |
| r(k) | - 11 | Auto-correlation coefficients |
| r¢(k) | - 11 | Modified auto-correlation coefficients |
| wi | - 10 | LSP weighting coefficients |
|  | - 10 | LSP quantizer output |

TABLE  5/G.729

Glossary of most relevant constants

| Name | Value | Description |
| --- | --- | --- |
| fs | 8000 | Sampling frequency |
| f0 | 60 | Bandwidth expansion |
| g1 | 0.94/0.98 | Weight factor perceptual weighting filter |
| g2 | 0.60/[0.4 - 0.7] | Weight factor perceptual weighting filter |
| gn | 0.55 | Weight factor postfilter |
| gd | 0.70 | Weight factor postfilter |
| gp | 0.50 | Weight factor pitch postfilter |
| gt | 0.90/0.2 | Weight factor tilt postfilter |
| C | Table 7 | Fixed (algebraic) codebook |
| L0 | 3.2.4 | Moving-average predictor codebook |
| L1 | 3.2.4 | First stage LSP codebook |
| L2 | 3.2.4 | Second stage LSP codebook (low part) |
| L3 | 3.2.4 | Second stage LSP codebook (high part) |
| GA | 3.9 | Gain codebook (first stage) |
| GB | 3.9 | Gain codebook (second stage) |
| wlag | Equation (6) | Correlation lag window |
| wlp | Equation (3) | LP analysis window |

TABLE  6/G.729

Glossary of acronyms

| Acronym | Description |
| --- | --- |

| CELP | Code-Excited Linear-Prediction |
| --- | --- |

| CS-ACELP | Conjugate-Structure Algebraic-CELP |
| --- | --- |

| MA | Moving Average |
| --- | --- |

| MSB | Most Significant Bit |
| --- | --- |

| MSE | Mean-Squared Error |
| --- | --- |

| LAR | Log Area Ratio |
| --- | --- |

| LP | Linear Prediction |
| --- | --- |

| LSP | Line Spectral Pair |
| --- | --- |

| LSF | Line Spectral Frequency |
| --- | --- |

| VQ | Vector quantization |
| --- | --- |

# 3	Functional description of the encoder

In this clause the different functions of the encoder represented in the blocks of Figure 2 are described. A detailed signal flow is shown in Figure 4.

## 3.1	Pre-processing

As stated in clause 2, the input to the speech encoder is assumed to be a 16 bit PCM signal. Two pre-processing functions are applied before the encoding process:

1)	signal scaling; and

2)	high-pass filtering.

The scaling consists of dividing the input by a factor 2 to reduce the possibility of overflows in the fixed-point implementation. The high-pass filter serves as a precaution against undesired low-frequency components. A second order pole/zero filter with a cut-off frequency of 140 Hz is used. Both the scaling and high-pass filtering are combined by dividing the coefficients at the numerator of this filter by 2. The resulting filter is given by:

(1)

The input signal filtered through Hh1(z) is referred to as s(n), and will be used in all subsequent coder operations.

## 3.2	Linear prediction analysis and quantization

The short-term analysis and synthesis filters are based on 10th order Linear Prediction (LP) filters.

The LP synthesis filter is defined as:

(2)

where âi, i = 1,...,10, are the (quantized) Linear Prediction (LP) coefficients. Short-term prediction, or linear prediction analysis is performed once per speech frame using the autocorrelation method with a 30 ms asymmetric window. Every 80 samples (10 ms), the autocorrelation coefficients of windowed speech are computed and converted to the LP coefficients using the Levinson algorithm. Then the LP coefficients are transformed to the LSP domain for quantization and interpolation purposes. The interpolated quantized and unquantized filters are converted back to the LP filter coefficients (to construct the synthesis and weighting filters for each subframe).

FIGURE 4/G.729...[D04] = PAGE PLAINE

### 3.2.1	Windowing and autocorrelation computation

The LP analysis window consists of two parts: the first part is half a Hamming window and the second part is a quarter of a cosine function cycle. The window is given by:

(3)

There is a 5 ms lookahead in the LP analysis which means that 40 samples are needed from the future speech frame. This translates into an extra algorithmic delay of 5 ms at the encoder stage. The LP analysis window applies to 120 samples from past speech frames, 80 samples from the present speech frame, and 40 samples from the future frame. The windowing procedure is illustrated in Figure 5.

FIGURE 5/G.729...[D05] = 5 CM

The different shading patterns identify corresponding excitation and LP analysis windows.

The windowed speech:

(4)

is used to compute the autocorrelation coefficients:

(5)

To avoid arithmetic problems for low-level input signals the value of r(0) has a lower boundary of r(0) = 1.0. A 60 Hz bandwidth expansion is applied, by multiplying the autocorrelation coefficients with:

(6)

where f0 = 60 Hz is the bandwidth expansion and fs = 8000 Hz is the sampling frequency. Furthermore, r(0) is multiplied by a white-noise correction factor 1.0001, which is equivalent to adding a noise floor at -40 dB. The modified autocorrelation coefficients are given by:

(7)

### 3.2.2	Levinson-Durbin algorithmXE "Levinson-Durbin algorithm"§

The modified autocorrelation coefficients r¢(k) are used to obtain the LP filter coefficients, ai, i = 1,...,10, by solving the set of equations:

(8)

The set of equations in (8) is solved using the Levinson-Durbin algorithm. This algorithm uses the following recursion:

The final solution is given as aj = , j = 0, ...,10, with a0 = 1.0.

### 3.2.3	LP to LSP conversionXE "LP to LSP conversion"§

The LP filter coefficients ai, i = 0,...10 are converted to Line Spectral Pair (LSP) coefficients for quantization and interpolation purposes. For a 10th order LP filter, the LSP coefficients are defined as the roots of the sum and difference polynomials:

(9)

and:

(10)

respectively. The polynomial  is symmetric, and  is antisymmetric. It can be proven that all roots of these polynomials are on the unit circle and they alternate each other.  has a root z = -1 (w = p) and  has a root z = 1 (w = 0). These two roots are eliminated by defining the new polynomials:

(11)

and:

(12)

Each polynomial has five conjugate roots on the unit circle (e±jwi), and they can be written as:

(13)

and:

(14)

where qi = cos(wi). The coefficients wi are the Line Spectral Frequencies (LSF) and they satisfy the ordering property 0 < wi < w2 < ... < w10 < p. The coefficients qi are referred to as the LSP coefficients in the cosine domain.

Since both polynomials F1(z) and F2(z) are symmetric only the first five coefficients of each polynominal need to be computed. The coefficients of these polynomials are found by the recursive relations:

(15)

where f1(0) = f2(0) = 1.0. The LSP coefficients are found by evaluating the polynomials F1(z) and F2(z) at 60 points equally spaced between 0 and p and checking for sign changes. A sign change signifies the existence of a root and the sign change interval is then divided four times to allow better tracking of the root. The Chebyshev polynomials are used to evaluate F1(z) and F2(z). In this method the roots are found directly in the cosine domain. The polynomials F1(z) or F2(z), evaluated at z = ejw, can be written as:

(16)

with:

(17)

where Tm(x) = cos(mw) is the mth order Chebyshev polynomial, and f(i), i = 1,...,5, are the coefficients of either F1(z) or F2(z), computed using Equation (15). The polynomial C(x) is evaluated at a certain value of x = cos(w) using the recursive relation:

with initial values b5 = 1 and b6 = 0.

### 3.2.4	Quantization of the LSP coefficientsXE "Quantization of the LSP coefficients"§

The LSP coefficients qi are quantized using the LSF representation wi in the normalized frequency domain [0, p]; that is:

(18)

A switched 4th order MA prediction is used to predict the LSF coefficients of the current frame. The difference between the computed and predicted coefficients is quantized using a two-stage vector quantizer. The first stage is a 10-dimensional VQ using codebook L1 with 128 entries (7 bits). The second stage is a 10 bit VQ which has been implemented as a split VQ using two 5-dimensional codebooks, L2 and L3 containing 32 entries (5 bits) each.

To explain the quantization process, it is convenient to first describe the decoding process. Each coefficient is obtained from the sum of two codebooks:

(19)

where L1, L2 and L3 are the codebook indices. To avoid sharp resonances in the quantized LP synthesis filter, the coefficients  are arranged such that adjacent coefficients have a minimum distance of J. The rearrangement routine is shown below:

This rearrangement process is done twice. First with a value of J = 0.0012, then with a value of J = 0.0006. After this rearrangement process, the quantized LSF coefficients  for the current frame m, are obtained from the weighted sum of previous quantizer outputs , and the current quantizer output :

(20)

where  are the coefficients of the switched MA predictor. Which MA predictor to use is defined by a separate bit L0. At start-up the initial values of  are given by  for all k < 0.

After computing , the corresponding filter is checked for stability. This is done as follows:

1)	order the coefficient  in increasing value;

2)	if  < 0,005 then  = 0.005;

3)	if ;

4)	if .

The procedure for encoding the LSF parameters can be outlined as follows. For each of the two MA predictors the best approximation to the current LSF coefficients has to be found. The best approximation is defined as the one that minimizes the weighted mean-squared error:

(21)

The weights wi are made adaptive as a function of the unquantized LSF coefficients,

(22)

In addition, the weights w5 and w6 are multiplied by 1.2 each.

The vector to be quantized for the current frame m is obtained from

(23)

The first codebook L1 is searched and the entry L1 that minimizes the (unweighted) mean-squared error is selected. This is followed by a search of the second codebook L2, which defines the lower part of the second stage. For each possible candidate, the partial vector , i = 1,...,5, is reconstructed using Equation (20), and rearranged to guarantee a minimum distance of 0.0012. The weighted MSE of Equation (21) is computed, and the vector L2 which results in the lowest error is selected. Using the selected first stage vector L1 and the lower part of the second stage L2, the higher part of the second stage is searched from codebook L3. Again the rearrangement procedure is used to guarantee a minimum distance of 0.0012. The vector L3 that minimizes the weighted MSE is selected. The resulting vector , i = 1,...,10 is rearranged to guarantee a minimum distance of 0.0006. This process is done for each of the two MA predictors defined by L0, and the MA predictor L0 that produces the lowest weighted MSE is selected. As was explained at the beginning of this clause, the resulting vector  is rearranged twice and a stability check is applied to produce the quantized LSF coefficients .

### 3.2.5	Interpolation of the LSP coefficientsXE "Interpolation of the LSP coefficients"§

The quantized (and unquantized) LP coefficients are used for the second subframe. For the first subframe, the quantized (and unquantized) LP coefficients are obtained by linear interpolation of the corresponding parameters in the adjacent subframes. The interpolation is done on the LSP coefficients in the cosine domain. Let  be the LSP coefficients computed for the current 10 ms frame, and  the LSP coefficients computed in the previous 10 ms frame. The (unquantized) interpolated LSP coefficients in each of the two subframes are given by:

(24)

The same interpolation procedure is used for the interpolation of the quantized LSP coefficients by substituting qi by  in Equation (24).

### 3.2.6	LSP to LP conversionXE "LSP to LP conversion"§

Once the LSP coefficients are quantized and interpolated, they are converted back to the LP coefficients ai. This conversion is done as follows. The coefficients of F1(z) and F2(z) are found by expanding Equations (13) and (14) knowing the quantized and interpolated LSP coefficients. The coefficients f1(i), i = 1,...,5, are computed from qi using the recursive relation:

with initial values f1(0) = 1 and f1(-1) = 0. The coefficients f2(i) are computed similarly by replacing q2i-1 by q2i.

Once the coefficients f1(i) and f2(i) are found, F1(z) and F2(z) are multiplied by 1 + z-1 and 1 - z-1, respectively, to obtain  and ; that is:

(25)

Finally the LP coefficients are computed from  and  by:

(26)

This is directly derived from the relation , and because  and  are symmetric and antisymmetric polynomials, respectively.

## 3.3	Perceptual weightingXE "Perceptual weighting"§

The perceptual weighting filter is based on the unquantized LP filter coefficients ai, and is given by:

(27)

The values of g1 and g2 determine the frequency response of the filter W(z). By proper adjustment of these variables it is possible to make the weighting more effective. This is done by making g1 and g2 a function of the spectral shape of the input signal. This adaptation is done once per 10 ms frame, but an interpolation procedure for each first subframe is used to smooth this adaptation process. The spectral shape is obtained from a 2nd order linear prediction filter, obtained as a by-product from the Levinson-Durbin recursion (3.2.2). The reflection coefficients ki are converted to Log Area Ratio (LAR) coefficients oi by:

(28)

The LAR coefficients corresponding to the current 10 ms frame are used for the second subframe. The LAR coefficients for the first subframe are obtained through linear interpolation with the LAR parameters from the previous frame. The interpolated LAR coefficients in each of the two subframes are given by:

(29)

The spectral envelope is characterized as being either flat (flat = 1) or tilted (flat = 0). For each subframe this characterization is obtained by applying a threshold function to the LAR coefficients. To avoid rapid changes, a hysteresis is used by taking into account the value of flat in the previous subframe m - 1,

(30)

If the interpolated spectrum for a subframe is classified as flat (flat(m) = 1), the weight factors are set to g1 = 0.94 and g2 = 0.6. If the spectrum is classified as tilted (flat(m) = 0), the value of g1 is set to 0.98, and the value of g2 is adapted to the strength of the resonances in the LP synthesis filter, but is bounded between 0.4 and 0.7. If a strong resonance is present, the value of g2 is set closer to the upper bound. This adaptation is achieved by a criterion based on the minimum distance between two successive LSP coefficients for the current subframe. The minimum distance is given by:

(31)

The value of g2 is computed using the linear relationship:

(32)

The weighted speech signal in a subframe is given by:

(33)

The weighted speech signal sw(n) is used to find an estimation of the pitch delay in the speech frame.

## 3.4	Open-loop pitch analysisXE "Open-loop pitch analysis"§

To reduce the complexity of the search for the best adaptive-codebook delay, the search range is limited around a candidate delay Top, obtained from an open-loop pitch analysis. This open-loop pitch analysis is done once per frame (10 ms). The open-loop pitch estimation uses the weighted speech signal sw(n) of Equation (33), and is done as follows: In the first step, three maxima of the correlation:

(34)

are found in the following three ranges:

The retained maxima R(ti), i = 1,...,3, are normalized through:

(35)

The winner among the three normalized correlations is selected by favouring the delays with the values in the lower range. This is done by weighting the normalized correlations corresponding to the longer delays. The best open-loop delay Top is determined as follows:

This procedure of dividing the delay range into three sections and favouring the smaller values is used to avoid choosing pitch multiples.

## 3.5	Computation of the impulse responseXE "Computation of the impulse response"§

The impulse response h(n) of the weighted synthesis filter W(z)/Â(z) is needed for the search of adaptive and fixed codebooks. The impulse response h(n) is computed for each subframe by filtering a signal consisting of the coefficients of the filter A(z/g1) extended by zeros through the two filters 1/Â(z) and 1/A(z/g2).

## 3.6	Computation of the target signalXE "Computation of the target signal"§

The target signal x(n) for the adaptive-codebook search is usually computed by subtracting the zero-input response of the weighted synthesis filter W(z)/Â(z) = A(z/g1)/[Â(z)A(z/g2)] from the weighted speech signal sw(n) of Equation (33). This is done on a subframe basis.

An equivalent procedure for computing the target signal, which is used in this Recommendation, is the filtering of the LP residual signal r(n) through the combination of synthesis filter 1/Â(z) and the weighting filter A(z/g1)/A(z/g2). After determining the excitation for the subframe, the initial states of these filters are updated by filtering the difference between the residual and excitation signals. The memory update of these filters is explained in 3.10.

The residual signal r(n), which is needed for finding the target vector is also used in the adaptive-codebook search to extend the past excitation buffer. This simplifies the adaptive-codebook search procedure for delays less than the subframe size of 40 as will be explained in the next subclause. The LP residual is given by:

(36)

## 3.7	Adaptive-codebook searchXE "Adaptive-codebook search"§

The adaptive-codebook parameters (or pitch parameters) are the delay and gain. In the adaptive-codebook approach for implementing the pitch filter, the excitation is repeated for delays less than the subframe length. In the search stage, the excitation is extended by the LP residual to simplify the closed-loop search. The adaptive-codebook search is done every (5 ms) subframe. In the first subframe, a fractional pitch delay T1 is used with a resolution of 1/3 in the range of [   ] and integers only in the range [85, 143]. For the second subframe, a delay T2 with a resolution of 1/3 is always used in the range , where int(T1) is the integer part of the fractional pitch delay T1 of the first subframe. This range is adapted for the cases where T1 straddles the boundaries of the delay range.

For each subframe the optimal delay is determined using closed-loop analysis that minimizes the weighted mean-squared error. In the first subframe the delay T1 is found by searching a small range (six samples) of delay values around the open-loop delay Top (see 3.4). The search boundaries tmin and tmax are defined by:

For the second subframe, closed-loop pitch analysis is done around the pitch selected in the first subframe to find the optimal delay T2. The search boundaries are between  and , where tmin and tmax are derived from T1 as follows:

The closed-loop pitch search minimizes the mean-squared weighted error between the original and reconstructed speech. This is achieved by maximizing the term:

(37)

where x(n) is the target signal and yk(n) is the past filtered excitation at delay k [past excitation convolved with h(n)]. Note that the search range is limited around a preselected value, which is the open-loop pitch Top for the first subframe, and T1 for the second subframe.

The convolution yk(n) is computed for the delay tmin. For the other integer delays in the search range k = tmin + 1,...,tmax, it is updated using the recursive relation:

(38)

where u(n), n = -143,...,39, is the excitation buffer, and yk-1(-1) = 0. Note that in the search stage, the samples u(n), n = 0,...,39 are not known, and they are needed for pitch delays less than 40. To simplify the search, the LP residual is copied to u(n) to make the relation in Equation (38) valid for all delays.

For the determination of T2, and T1 if the optimum integer closed-loop delay is less than 85, the fractions around the optimum integer delay have to be tested. The fractional pitch search is done by interpolating the normalized correlation in Equation (37) and searching for its maximum. The interpolation is done using a FIR filter b12 based on a Hamming windowed sinc function with the sinc truncated at ± 11 and padded with zeros at ± 12 (b12(12) = 0). The filter has its cut-off frequency (-3 dB) at 3600 Hz in the oversampled domain. The interpolated values of R(k) for the fractions , , 0,  and   are obtained using the interpolation formula

(39)

where t = 0, 1, 2 corresponds to the fractions 0,  and , respectively. Note that it is necessary to compute the correlation terms in Equation (37) using a range tmin - 4, tmax + 4, to allow for the proper interpolation.

### 3.7.1	Generation of the adaptive-codebook vectorXE "Generation of the adaptive-codebook vector"§

Once the pitch delay has been determined, the adaptive-codebook vector v(n) is computed by interpolating the past excitation signal u(n) at the given integer delay k and fraction t:

n  =  0,...,39     t  =  0, 1, 2	(40)

The interpolation filter b30 is based on a Hamming windowed sinc functions truncated at ± 29 and padded with zeros at ± 30 [b30(30) = 0]. The filter has a cut-off frequency (-3 dB) at 3600 Hz in the oversampled domain.

### 3.7.2	Codeword computation for adaptive-codebook delaysXE "Codeword computation for adaptive-codebook delays"§

The pitch delay T1 is encoded with 8 bits in the first subframe and the relative delay in the second subframe is encoded with 5 bits. A fractional delay T is represented by its integer part int(T), and a fractional part frac/3, frac = -1,0,1. The pitch index P1 is now encoded as:

(41)

The value of the pitch delay T2 is encoded relative to the value of T1. Using the same interpretation as before, the fractional delay T2 represented by its integer part int(T2), and a fractional part frac/3, frac = -1,0,1, is encoded as:

(42)

where tmin is derived from T1 as in 3.7.

To make the coder more robust against random bit errors, a parity bit P0 is computed on the delay index P1 of the first subframe. The parity bit is generated through an XOR operation on the six most significant bits of P1. At the decoder this parity bit is recomputed and if the recomputed value does not agree with the transmitted value, an error concealment procedure is applied.

### 3.7.3	Computation of the adaptive-codebook gainXE "Computation of the adaptive-codebook gain"§

Once the adaptive-codebook delay is determined, the adaptive-codebook gain gp is computed as:

(43)

where x(n) is the target signal and y(n) is the filtered adaptive-codebook vector [zero-state response of W(z)/Â(z) to v(n)]. This vector is obtained by convolving v(n) with h(n):

(44)

## 3.8	Fixed codebook – Structure and search

The fixed codebook is based on an algebraic codebook structure using an Interleaved Single-Pulse Permutation (ISPP) design. In this codebook, each codebook vector contains four non zero pulses. Each pulse can have either the amplitudes +1 or -1, and can assume the positions given in Table 7.

TABLE  7/G.729

Structure of fixed codebook C XE "Structure of fixed codebook C "§

| Pulse | Sign | Positions |
| --- | --- | --- |
| i0 | s0: ± 1 | - m0: 0, 5, 10, 15, 20, 25, 30, 35 |
| - i1 | s1: ± 1 | - m1: 1, 6, 11, 16, 21, 26, 31, 36 |
| - i2 | s2: ± 1 | - m2: 2, 7, 12, 17, 22, 27, 32, 37 |
| - i3 | s3: ± 1 | - m3: 3, 8, 13, 18, 23, 28, 33, 38 - 4, 9, 14, 19, 24, 29, 34, 39 |

The codebook vector c(n) is constructed by taking a zero vector of dimension 40, and putting the four unit pulses at the found locations, multiplied with their corresponding sign:

(45)

where d(0) is a unit pulse. A special feature incorporated in the codebook is that the selected codebook vector is filtered through an adaptive pre-filter P(z) that enhances harmonic components to improve the quality of the reconstructed speech. Here the filter:

(46)

is used, where T is the integer component of the pitch delay of the current subframe, and b is a pitch gain. The value of b is made adaptive by using the quantized adaptive-codebook gain from the previous subframe, that is:

(47)

For delays less than 40, the codebook c(n) of Equation (45) is modified according to:

(48)

This modification is incorporated in the fixed-codebook search by modifying the impulse response h(n) according to:

(49)

### 3.8.1	Fixed-codebook search procedureXE "Fixed-codebook search procedure"§

The fixed codebook is searched by minimizing the mean-squared error between the weighted input speech sw(n) of Equation (33) and the weighted reconstructed speech. The target signal used in the closed-loop pitch search is updated by subtracting the adaptive-codebook contribution. That is:

(50)

where y(n) is the filtered adaptive-codebook vector of Equation (44) and gp the adaptive-codebook gain of Equation (43).

The matrix H is defined as the lower triangular Toepliz convolution matrix with diagonal h(0) and lower diagonal h(1),...,h(39). The matrix F = HtH contains the correlations of h(n), and the elements of this symmetric matrix are given by:

(51)

The correlation signal d(n) is obtained from the target signal x¢(n) and the impulse response h(n) by:

(52)

If ck is the kth fixed-codebook vector, then the codebook is search by maximizing the term:

(53)

where t denotes transpose.

The signal d(n) and the matrix F are computed before the codebook search. Note that only the elements actually needed are computed and an efficient storage procedure has been designed to speed up the search procedure.

The algebraic structure of the codebook C allows for a fast search procedure since the codebook vector ck contains only four non zero pulses. The correlation in the numerator of Equation (53) for a given vector ck is given by:

(54)

where mi is the position of the ith pulse and si is its amplitude. The energy in the denominator of Equation (53) is given by:

(55)

To simplify the search procedure, the pulse amplitudes are predetermined by quantizing the signal d(n). This is done by setting the amplitude of a pulse at a certain position equal to the sign of d(n) at the position. Before the codebook search, the following steps are done. First, the signal d(n) is decomposed into two parts: its absolute value |d(n)| and its sign sign [d(n)]. Second, the matrix F is modified by including the sign information; that is,

(56)

The main-diagonal elements of F are scaled to remove the factor 2 in Equation (55)

(57)

The correlation in Equation (54) is now given by:

(58)

and the energy in Equation (55) is given by:

(59)

A focused search approach is used to further simplify the search procedure. In this approach a precomputed threshold is tested before entering the last loop, and the loop is entered only if this threshold is exceeded. The maximum number of times the loop can be entered is fixed so that a low percentage of the codebook is searched. The threshold is computed based on the correlation C. The maximum absolute correlation and the average correlation due to the contribution of the first three pulses, max3 and av3, are found before the codebook search. The threshold is given by:

(60)

The fourth loop is entered only if the absolute correlation (due to three pulses) exceeds thr3, where 0 £ K3 < 1. The value of K3 controls the percentage of codebook search and it is set here to 0.4. Note that this results in a variable search time. To further control the search the number of times the last loop is entered (for the two subframes) cannot exceed a certain maximum, which is set here to 180 (the average worst case per subframe is 90 times).

### 3.8.2	Codeword computation of the fixed codebookXE "Codeword computation of the fixed codebook"§

The pulse positions of the pulses i0, i1 and i2, are encoded with 3 bits each, while the position of i3 is encoded with 4 bits. Each pulse amplitude is encoded with 1 bit. This gives a total of 17 bits for the 4 pulses. By defining s = 1 if the sign is positive and s = 0 if the sign is negative, the sign codeword is obtained from:

(61)

and the fixed-codebook codeword is obtained from:

(62)

where jx = 0 if m3 = 3,8,...,38, and jx = 1 if m3 = 4,9...,39.

## 3.9	Quantization of the gainsXE "Quantization of the gains"§

The adaptive-codebook gain (pitch gain) and the fixed-codebook gain are vector quantized using 7 bits. The gain codebook search is done by minimizing the mean-squared weighted error between original and reconstructed speech which is given by:

(63)

where x is the target vector (see 3.6), y is the filtered adaptive-codebook vector of Equation (44), and z is the fixed-codebook vector convolved with h(n),

(64)

### 3.9.1	Gain predictionXE "Gain prediction"§

The fixed-codebook gainXE "fixed-codebook gain"§ gc can be expressed as:

(65)

where  is a predicted gain based on previous fixed-codebook energies, and g is a correction factor.

The mean energy of the fixed-codebook contribution is given by:

(66)

After scaling the vector c(n) with the fixed-codebook gain gc, the energy of the scaled fixed codebook is given by 20 log gc + E. Let E(m) be the mean-removed energy (in dB) of the (scaled) fixed-codebook contribution at the subframe m, given by:

(67)

where  is the mean energy of the fixed-codebook excitation. The gain gc can be expressed as a function of E(m), E and  by:

(68)

The predicted gain  is found by predicting the log-energy of the current fixed-codebook contribution from the log-energy of previous fixed-codebook contributions. The 4th order MA prediction is done as follows. The predicted energy is given by:

(69)

where [b1 b2 b3 b4] = [0.68 0.58 0.34 0.19] are the MA prediction coefficients, and Û(m) is the quantized version of the prediction error U(m) at subframe m, defined by:

(70)

The predicted gainXE "predicted gain"§  is found by replacing E(m) by its predicted value in Equation (68).

(71)

The correction factor g is related to the gain-prediction error by:

(72)

### 3.9.2	Codebook search for gain quantizationXE "Codebook search for gain quantization"§

The adaptive-codebook gainXE "adaptive-codebook gain"§, gp, and the factor g are vector quantized using a two-stage conjugate structured codebook. The first stage consists of a 3 bit two-dimensional codebook GA, and the second stage consists of a 4 bit two-dimensional codebook GB. The first element in each codebook represents the quantized adaptive-codebook gain  , and the second element represents the quantized fixed-codebook gain correction factor . Given codebook indices GA and GB for GA and GB, respectively, the quantized adaptive-codebook gain is given by:

(73)

and the quantized fixed-codebook gain by:

(74)

This conjugate structure simplifies the codebook search, by applying a pre-selection process. The optimum pitch gain gp, and fixed-codebook gain, gc, are derived from Equation (63), and are used for the pre-selection. The codebook GA contains eight entries in which the second element (corresponding to gc) has in general larger values than the first element (corresponding to gp). This bias allows a pre-selection using the value of gc. In this pre-selection process, a cluster of four vectors whose second elements are close to gc are selected. Similarly, the codebook GB contains 16 entries in which each has a bias towards the first element (corresponding to gp). A cluster of eight vectors whose first elements are close to gp are selected. Hence for each codebook the best 50% candidate vectors are selected. This is followed by an exhaustive search over the remaining 4 ´ 8 = 32 possibilities, such that the combination of the two indices minimizes the weighted mean-squared error of Equation (63).

### 3.9.3	Codeword computation for gain quantizerXE "Codeword computation for gain quantizer"§

The codewords GA and GB for the gain quantizer are obtained from the indices corresponding to the best choice. To reduce the impact of single bit errors the codebook indices are mapped.

## 3.10	Memory updateXE "Memory update"§

An update of the states of the synthesis and weighting filters is needed to compute the target signal in the next subframe. After the two gains are quantized, the excitation signal, u(n), in the present subframe is obtained using:

(75)

where  and  are the quantized adaptive and fixed-codebook gains, respectively, v(n) is the adaptive-codebook vector (interpolated past excitation), and c(n) is the fixed-codebook vector including harmonic enhancement. The states of the filters can be updated by filtering the signal r(n)-u(n) (difference between residual and excitation) through the filters 1/Â(z) and A(z/g1)/A(z/g2) for the 40 sample subframe and saving the states of the filters. This would require three filter operations. A simpler approach, which requires only one filter operation, is as follows. The locally reconstructed speech  is computed by filtering the excitation signal through 1/Â(z). The output of the filter due to the input r(n) - u(n) is equivalent to e(n) = s(n) - . So the states of the synthesis filter 1/Â(z) are given by e(n), n = 30,...,39. Updating the states of the filter A(z/g1)/A(z/g2) can be done by filtering the error signal e(n) through this filter to find the perceptually weighted error ew(n). However, the signal ew(n) can be equivalently found by:

(76)

Since the signals x(n), y(n) and z(n) are available, the states of the weighting filter are updated by computing ew(n) as in Equation (76) for n = 30,...,39. This saves two filter operations.

# 4	Functional description of the decoder

The principle of the decoder was shown in clause 2 (Figure 3). First the parameters are decoded (LP coefficients, adaptive-codebook vector, fixed-codebook vector and gains). The transmitted parameters are listed in Table 8. These decoded parameters are used to compute the reconstructed speech signal as will be described in 4.1. This reconstructed signal is enhanced by a post-processing operation consisting of a postfilter, a high-pass filter and an upscaling (see 4.2). Subclause 4.4 describes the error concealment procedure used when either a parity error has occurred, or when the frame erasure flag has been set. A detailed signal flow diagram of the decoder is shown in Figure 6.

TABLE  8/G.729

Description of transmitted parameters indices – The bitstream ordering is reflected by the order
in the table – For each parameter the Most Significant Bit (MSB) is transmitted first

| Symbol | Description | - Bits |
| --- | --- | --- |
| - L0 | Switched MA predictor of LSP quantizer | - 1 |
| - L1 | First stage vector of quantizer | - 7 |
| - L2 | Second stage lower vector of LSP quantizer | - 5 |
| - L3 | Second stage higher vector of LSP quantizer | - 5 |
| - P1 | Pitch delay first subframe | - 8 |
| - P0 | Parity bit for pitch delay | - 1 |
| - C1 | Fixed codebook first subframe | - 13 |
| - S1 | Signs of fixed-codebook pulses 1st subframe | - 4 |
| - GA1 | Gain codebook (stage 1) 1st subframe | - 3 |
| - GB1 | Gain codebook (stage 2) 1st subframe | - 4 |
| - P2 | Pitch delay second subframe | - 5 |
| - C2 | Fixed codebook 2nd subframe | - 13 |
| - S2 | Signs of fixed-codebook pulses 2nd subframe | - 4 |
| - GA2 | Gain codebook (stage 1) 2nd subframe | - 3 |
| - GB2 | Gain codebook (stage 2) 2nd subframe | - 4 |

## 4.1	Parameter decoding procedure

The decoding process is done in the following order.

### 4.1.1	Decoding of LP filter parametersXE "Decoding of LP filter parameters"§

The received indices L0, L1, L2 and L3 of the LSP quantizer are used to reconstruct the quantized LSP coefficients using the procedure described in 3.2.4. The interpolation procedure described in 3.2.5 is used to obtain two sets of interpolated LSP coefficients (corresponding to two subframes). For each subframe, the interpolated LSP coefficients are converted to LP filter coefficients ai, which are used for synthesizing the reconstructed speech in the subframe.

The following steps are repeated for each subframe:

1)	decoding of the adaptive-codebook vector;

2)	decoding of the fixed-codebook vector;

3)	decoding of the adaptive and fixed-codebook gains;

4)	computation of the reconstructed speech.

FIGURE 6/G.729...[D06] = 16 cm
XE "Signal flow at the CS-ACELP decoder"§

### 4.1.2	Computation of the parity bitXE "Computation of the parity bit"§

Before the excitation is reconstructed, the parity bit is recomputed from the adaptive-codebook delay index P1 (see 3.7.2). If this bit is not identical to the transmitted parity bit P0, it is likely that bit errors occurred during transmission.

If a parity error occurs on P1, the delay value T1 is set to the integer part of the delay value T2 of the previous frame. The value T2 is derived with the procedure outlined in 4.1.3, using this new value of T1.

### 4.1.3	Decoding of the adaptive-codebook vectorXE "Decoding of the adaptive-codebook vector"§

If no parity error has occurred the received adaptive-codebook index P1 is used to find the integer and fractional parts of the pitch delay T1. The integer part int(T1) and fractional part frac of T1 are obtained from P1 as follows:

The integer and fractional part of T2 are obtained from P2 and tmin, where tmin is derived from T1 as follows:

Now T2 is decoded using:

The adaptive-codebook vector v(n) is found by interpolating the past excitation u(n) (at the pitch delay) using Equation (40).

### 4.1.4	Decoding of the fixed-codebook vectorXE "Decoding of the fixed-codebook vector"§

The received fixed-codebook index C is used to extract the positions of the excitation pulses. The pulse signs are obtained from S. This is done by reversing the process described in 3.8.2. Once the pulse positions and signs are decoded the fixed-codebook vector c(n) is constructed using Equation (45). If the integer part of the pitch delay T is less than the subframe size 40, c(n) is modified according to Equation (48).

### 4.1.5	Decoding of the adaptive and fixed-codebook gainsXE "Decoding of the adaptive and fixed-codebook gains"§

The received gain-codebook index gives the adaptive-codebook gain  and the fixed-codebook gain correction factor . This procedure is described in detail in 3.9. The estimated fixed-codebook gain  is found using Equation (71). The fixed-codebook vector is obtained from the product of the quantized gain correction factor with this predicted gain Equation (74). The adaptive-codebook gain is reconstructed using Equation (73).

### 4.1.6	Computing the reconstructed speechXE "Computing the reconstructed speech"§

The excitation u(n) [see Equation (75)] is input to the LP synthesis filter. The reconstructed speech for the subframe is given by:

(77)

where âi are the interpolated LP filter coefficients for the current subframe. The reconstructed speech  is then processed by the post processor described in the next subclause.

## 4.2	Post-processingXE "Post-processing"§

Post-processing consists of three functions: adaptive postfiltering, high-pass filtering and signal upscaling. The adaptive postfilter is the cascade of three filters: a long-term postfilter Hp(z), a short-term postfilter Hf (z) and a tilt compensation filter Ht(z), followed by an adaptive gain control procedure. The postfilter coefficients are updated every 5 ms subframe. The postfiltering process is organized as follows. First, the reconstructed speech  is inverse filtered through Â(z /gn) to produce the residual signal . This signal is used to compute the delay T and gain gt of the long-term postfilter Hp(z). The signal  is then filtered through the long-term postfilter Hp(z) and the synthesis filter 1/[gf Â(z /gd)]. Finally, the output signal of the synthesis filter 1/[gf Â(z /gd)] is passed through the tilt compensation filter Ht(z) to generate the postfiltered reconstructed speech signal sf(n). Adaptive gain control is then applied to sf(n) to match the energy of . The resulting signal sf¢(n) is high-pass filtered and scaled to produce the output signal of the decoder.

### 4.2.1	Long-term postfilterXE "Long-term postfilter"§

The long-term postfilter is given by:

(78)

where T is the pitch delay, and gl is the gain coefficient. Note that gl is bounded by 1, and it is set to zero if the long-term prediction gain is less than 3 dB. The factor gp controls the amount of long-term postfiltering and has the value of gp = 0.5. The long-term delay and gain are computed from the residual signal  obtained by filtering the speech  through Â(z/gn), which is the numerator of the short-term postfilter (see 4.2.2).

(79)

The long-term delay is computed using a two-pass procedure. The first pass selects the best integer T0 in the range [int(T1) - 1, int(T1) +1], where int(T1) is the integer part of the (transmitted) pitch delay T1 in the first subframe. The best integer delay is the one that maximizes the correlation.

(80)

The second pass chooses the best fractional delay T with resolution 1/8 around T0. This is done by finding the delay with the highest pseudo-normalized correlation

(81)

where  is the residual signal at delay k. Once the optimal delay T is found, the corresponding correlation R¢(T) is normalized with the square-root of the energy of . The squared value of this normalized correlation is used to determine if the long-term postfilter should be disabled. This is done by setting gl = 0 if:

(82)

Otherwise the value of gl is computed from:

(83)

The non-integer delayed signal  is first computed using an interpolation filter of length 33. After the selection of T,  is recomputed with a longer interpolation filter of length 129. The new signal replaces the previous one only if the longer filter increases the value of R¢(T).

### 4.2.2	Short-term postfilterXE "Short-term postfilter"§

The short-term postfilter is given by:

(84)

where Â(z) is the received quantized LP inverse filter (LP analysis is not done at the decoder) and the factors gn and gd control the amount of short-term postfiltering, and are set to gn = 0.55, and gd = 0.7. The gain term gf is calculated on the truncated impulse response hf (n) of the filter Â(z /gn)/Â(z /gd) and is given by:

(85)

### 4.2.3	Tilt compensationXE "Tilt compensation"§

The filter Ht(z) compensates for the tilt in the short-term postfilter Hf (z) and is given by:

(86)

where  is a tilt factor  being the first reflection coefficient calculated from hf (n) with

(87)

The gain term gt = 1 – || compensates for the decreasing effect of gf in Hf(z). Furthermore, it has been shown that the product filter Hf (z)Ht(z) has generally no gain. Two values for gt are used depending on the sign of . If  is negative, gt = 0.9, and if  is positive, gt = 0.2.

### 4.2.4	Adaptive gain controlXE "Adaptive gain control"§l

Adaptive gain control is used to compensate for gain differences between the reconstructed speech signal  and the postfiltered signal sf(n). The gain scaling factor G for the present subframe is computed by:

(88)

The gain-scaled postfiltered signal sf ¢(n) is given by:

(89)

where g(n) is updated on a sample-by-sample basis and given by:

(90)

The initial value of g(-1) = 1.0 is used. Then for each new subframe, g(-1) is set equal to g(39) of the previous subframe.

### 4.2.5	High-pass filtering and upscalingXE "High-pass filtering and up-scaling"§

A high-pass filter with a cut-off frequency of 100 Hz is applied to the reconstructed postfiltered speech sf ¢(n). The filter is given by:

(91)

The filtered signal is multiplied by a factor 2 to restore the input signal level.

## 4.3	Encoder and decoder initialization

All static encoder and decoder variables should be initialized to 0, except the variables listed in Table 9.

TABLE  9/G.729

Description of parameters with non-zero initialization

| Variable | Reference | Initial value |
| --- | --- | --- |
| b | 3.8 | 0.8 |
| g(-1) | 4.2.4 | 1.0 |
|  | 3.2.4 | ip/11 |
| qi | 3.2.4 | arccos(ip/11) |
|  | 3.9.1 | -14 |

## 4.4	Concealment of frame erasuresXE "Concealment of frame erasures"§

An error concealment procedure has been incorporated in the decoder to reduce the degradation in the reconstructed speech because of frame erasures in the bitstream. This error concealment process is functional when the frame of coder parameters (corresponding to a 10 ms frame) has been identified as being erased. The mechanism for detecting frame erasures is not defined in the Recommendation, and will depend on the application.

The concealment strategy has to reconstruct the current frame, based on previously received information. The method replaces the missing excitation signal with one of similar characteristics, while gradually decaying its energy. This is done by using a voicing classifier based on the long-term prediction gain, which is computed as part of the long-term postfilter analysis. The long-term postfilter (see 4.2.1) finds the long-term predictor for which the prediction gain is more than 3 dB. This is done by setting a threshold of 0.5 on the squared normalized correlation of (Equation 82). For the error concealment process, a 10 ms frame is declared periodic if at least one 5 ms subframe has a long-term prediction gain of more than 3 dB. Otherwise the frame is declared non-periodic. An erased frame inherits its class from the preceding (reconstructed) speech frame. Note that the voicing classification is continuously updated based on this reconstructed speech signal.

The specific steps taken for an erased frame are:

1)	repetition of the synthesis filter parameters;

2)	attenuation of adaptive and fixed-codebook gains;

3)	attenuation of the memory of the gain predictor;

4)	generation of the replacement excitation.

### 4.4.1	Repetition of synthesis filter parametersXE "Repetition of synthesis filter parameters"§

The synthesis filter in an erased frame uses the LP parameters of the last good frame. The memory of the MA LSF predictor contains the values of the received codewords . Since the codeword is not available for the current frame m, it is computed from the repeated LSF parameters  and the predictor memory using:

(92)

where the MA predictor coefficients  are those of the last received good frame.

### 4.4.2	Attenuation of adaptive and fixed-codebook gainsXE "Attenuation of adaptive and fixed-codebook gains"§

The fixed-codebook gain is based on an attenuated version of the previous fixed-codebook gain and is given by:

(93)

where m is the subframe index. The adaptive-codebook gain is based on an attenuated version of the previous adaptive-codebook gain and is given by:

(94)

### 4.4.3	Attenuation of the memory of the gain predictorXE "Attenuation of the memory of the gain predictor"§

As was described in 3.9 the gain predictor uses the energy of previously selected fixed-codebook vectors c(n). To avoid transitional effects at the decoder, once good frames are received, the memory of the gain predictor is updated with an attenuated version of the codebook energy. The value of Û(m) for the current subframe m is set to the averaged quantized gain prediction-error, attenuated by 4 dB:

(95)

### 4.4.4	Generation of the replacement excitationXE "Generation of the replacement excitation"§

The excitation used depends on the periodicity classification. If the last reconstructed frame was classified as periodic, the current frame is considered to be periodic as well. In that case only the adaptive codebook is used, and the fixed-codebook contribution is set to zero. The pitch delay is based on the integer part of the pitch delay in the previous frame, and is repeated for each successive frame. To avoid excessive periodicity the delay is increased by one for each next subframe but bounded by 143. The adaptive-codebook gain is based on an attenuated value according to Equation (94).

If the last reconstructed frame was classified as non-periodic, the current frame is considered to be non-periodic as well, and the adaptive-codebook contribution is set to zero. The fixed-codebook contribution is generated by randomly selecting a codebook index and sign index. The random generator is based on the function

(96)

with the initial seed value of 21845. The fixed-codebook index is derived from the 13 least significant bits of the next random number. The fixed-codebook sign is derived from the 4 least significant bits of the next random number. The fixed-codebook gain is attenuated according to Equation (93).

# 5	Bit-exact description of the CS-ACELP coderXE "Bit-exact description of the CS-ACELP coder"§

ANSI C code simulating the CS-ACELP coder in 16 bit fixed-point is available from ITU-T. The following subclauses summarize the use of this simulation code, and how the software is organized.

## 5.1	Use of the simulation software

The C code consists of two main programs coder.c, which simulates the encoder, and decoder.c, which simulates the decoder. The encoder is run as follows:

coder inputfile bitstreamfileXE "coder inputfile bitstreamfile"§

The input file and output file are sampled data files containing 16 bit PCM signals. The decoder is run as follows:

decoder bitstreamfile outputfileXE "decoder bitstreamfile outputfile"§

The mapping table of the encoded bitstream is contained in the simulation software.

## 5.2	Organization of the simulation software

In the fixed-point ANSI C simulation, only two types of fixed-point data are used as is shown in Table 10. To facilitate the implementation of the simulation code, loop indices, Boolean values and flags use the type Flag, which would be either 16 bits or 32 bits depending on the target platform.

TABLE  10/G.729

Data types used in ANSI C simulationXE "Data types used in ANSI C simulation"§

| Type | Maximal value | Minimal value | Description |
| --- | --- | --- | --- |
| Word16 | 0x7fff | 0x8000 | Signed 2’s complement 16-bit word |
| Word32 | 0x7fffffffL | 0x80000000L | Signed 2’s complement 32-bit word |

All the computations are done using a predefined set of basic operators. The description of these operators is given in Table 11. The tables used by the simulation coder are summarized in Table 12. These main programmes use a library of routines that are summarized in Tables 13, 14 and 15.

TABLE  11/G.729

Basic operations used in ANSI C simulationXE "Basic operations used in ANSI C simulation"§

| Operation | Description |
| --- | --- |
| Word16 sature(Word32 L_var1) | Limit to 16 bits |
| Word16 add(Word16 var1, Word16 var2) | Short addition |
| Word16 sub(Word16 var1, Word16 var2) | Short subtraction |
| Word16 abs_s(Word16 var1) | Short absolute value |
| Word16 sh1(Word16 var1, Word16 var2) | Short shift left |
| Word16 shr(Word16 var1, Word16 var2) | Short shift right |
| Word16 mult(Word16 var1, Word16 var2) | Short multiplication |
| Word32 L_mult(Word16 var1, Word16 var2) | Long multiplication |
| Word16 negate(Word16 var1) | Short negate |
| Word16 extract_h(Word32 L_var1) | Extract high |
| Word16 extract_1(Word32 L_var1) | Extract low |
| Word16 round(Word32 L_var1) | Round |
| Word32 L_mac(Word32 L_var3, Word16 var1, Word16 var2) | Multiply and accumulate |
| Word32 L_msu(Word32 L_var3, Word16 var1, Word16 var2) | Multiply and subtract |
| Word32 L_add(Word32 L_var1, Word32 L_var2) | Long addition |
| Word32 L_sub(Word32 L_var1, Word32 L_var2) | Long subtraction |
| Word32 L_negate(Word32 L_var1) | Long negate |
| Word16 mult_r(Word16 var1, Word16 var2) | Multiplication with rounding |
| Word32 L_sh1(Word32 L_var1, Word16 var2) | Long shift left |
| Word32 L_shr(Word32 L_var1, Word16 var2) | Long shift right |
| Word16 shr_r(Word16 var1, Word16 var2) | Shift right with rounding |
| Word16 mac_r(Word32 L_var3, Word16 var1, Word16 var2) | Mac with rounding |
| Word16 msu_r(Word32 L_var3, Word16 var1, Word16 var2) | Msu with rounding |
| Word32 L_deposit_h(Word16 var1) | 16-bit var1 into MSB part |
| Word32 L_deposit_l(Word16 var1) | 16-bit var1 into LSB part |
| Word32 L_shr_r(Word32 L_var1, Word16 var2) | Long shift right with round |
| Word32 L_abs(Word32 L_var1) | Long absolute value |
| Word16 norm_s(Word16 var1) | Short norm |
| Word16 div_s(Word16 var1, Word16 var2) | Short division |
| Word16 norm_1(Word32 L_var1) | Long norm |

TABLE  12/G.729

Summary of tables fond in tab. ld8.c

| Table name | Size | Description |
| --- | --- | --- |
| tab_hup_s | 28 | Upsampling filter for postfilter |
| tab_hup_1 | 112 | Upsampling filter for postfilter |
| inter_3 | 13 | FIR filter for interpolating the correlation |
| inter_3 | 31 | FIR filter for interpolating past excitation |
| lspcb1 | 128 ´ 10 | LSP quantizer (first stage) |
| lspcb2 | 32 ´ 10 | LSP quantizer (second stage) |
| fg | 2 ´ 4 ´ 10 | MA predictors in LSP VQ |
| fg_sum | 2 ´ 10 | Used in LSP VQ |
| fg_sum_inv | 2 ´ 10 | Used in LSP VQ |
| gbk1 | 8 ´ 2 | Codebook GA in gain VQ |
| gbk2 | 16 ´ 2 | Codebook GB in gain VQ |
| map1 | 8 | Used in gain VQ |
| imap1 | 8 | Used in gain VQ |
| map2 | 16 | Used in gain VQ |
| ima21 | 16 | Used in gain VQ |
| window | 240 | LP analysis window |
| lag_h | 10 | Lag window for bandwidth expansion (high part) |
| lag_1 | 10 | Lag window for bandwidth expansion (low part) |
| grid | 61 | Grid points in LP to LSP conversion |
| tabsqr | 49 | Lookup table in inverse square root computation |
| tablog | 33 | Lookup table in base 2 logarithm computation |
| table | 65 | Lookup table in LSF to LSP conversion and vice versa |
| slope | 64 | Line slopes in LSP to LSF conversion |
| tabpow | 33 | Lookup table in 2x computation |

TABLE  13/G.729

Summary of encoder specific routines

| Filename | Description |
| --- | --- |

| acelp_co.c | Search fixed codebook |
| --- | --- |

| cod_1d8k.c | Encoder routine |
| --- | --- |

| lpc.c | LP analysis |
| --- | --- |

| pitch.c | Pitch search |
| --- | --- |

| pre_proc.c | Pre-processing (HP filtering and scaling) |
| --- | --- |

| pwf.c | Computation of perceptual weighting coefficients |
| --- | --- |

| qua_gain.c | Gain quantizer |
| --- | --- |

| qua_1sp.c | LSP quantizer |
| --- | --- |

TABLE  14/G.729

Summary of decoder specific routines

| Filename | Description |
| --- | --- |
| de_acelp.c | Decode algebraic codebook |
| dec_gain.c | Decode gains |
| dec_lag3.c | Decode adaptive-codebook index |
| dec_ld8k.c | Decoder routine |
| lspdec.c | LSP decoding routing |
| post_pro.c | Post processing (HP filtering and scaling) |
| pst.c | Postfilter routines |

TABLE  15/G.729

Summary of general routines

| Filename | Description |
| --- | --- |
| basicop2.c | Basic operators |
| oper_32b.c | Extended basic operators |
| bits.c | Bit manipulation routines |
| dspfunc.c | Mathematical functions |
| filter.c | Filter functions |
| gainpred.c | Gain predictor |
| lpcfunc.c | Miscellaneous routines related to LP filter |
| lspgetq.c | LSP quantizer |
| p_parity.c | Compute pitch parity |
| pred_lt3.c | Generation of adaptive codebook |
| util.c | Utility functions |