|  | COVERING NOTE |
| --- | --- |

general secretariat international telecommunication union

Subject:	Corrigendum 2 (02/2000)

Geneva, 22 April 2002

Recommendation ITU-T G.729 Annex B

TELECOMMUNICATION
	STANDARDIZATION  SECTOR
	OF  ITU

ITU-T Recommendation G.729 Annex B (10/1996)

Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear-prediction (CS-ACELP) – Annex B: A silence compression scheme for G.729 optimized for terminals conforming to Recommendation V.70

This correction should be introduced in the ANSI-C source code of G.729 Annex B version 1.3 delivered to ITU-T in 1997. The software included in ITU-T Rec. G.729 Annex B has already been modified accordingly (version 1.4).

In file dec_sid.c, replace the line 32 Static Word16 lspSid[M];  by

Static Word16 lspSid[M] = {31441,  27566,  21458,  13612,   4663,  -4663, -13612, -21458, -27566, -31441};

and delete the line  46 in routine Init_Dec_cng in file dec_sid.c : Copy( freq_prev_reset, lspSid, M);