|  | COVERING NOTE |
| --- | --- |

general secretariat international telecommunication union

Subject:	Corrigendum 1 (02/2000)

Geneva, 22 April 2002

Recommendation ITU-T G.729 Annex E

TELECOMMUNICATION
	STANDARDIZATION  SECTOR
	OF  ITU

ITU-T Recommendation G.729 Annex E (09/1998)

Coding of speech at 8 kbit/s using conjugate-structure algebraic-code-excited linear-prediction (CS-ACELP) – Annex E: 11.8 kbit/s CS-ACELP speech coding algorithm

These corrections should be introduced in the ANSI-C source code of G.729 annex E version 1.2 delivered to ITU-T in September 1998. The software included in ITU-T Rec. G.729 Annex E has already been modified accordingly (version 1.3).

	Delete line 224 in decodere.c if (ga_harm != 0).

	In routine pst_ltpe (in file pste.c), the instruction at line 267 :       gain_plt = MIN_GPLT;

should be replaced by :

if (gamma_harm == GAMMA_HARM)   gain_plt = MIN_GPLT;

else {

num_gltp = mult_r(den_gltp, gamma_harm);

num_gltp = shr(num_gltp, 1);

den_gltp = shr(den_gltp, 1);

temp = add(den_gltp, num_gltp);

gain_plt = div_s(den_gltp, temp);

}