|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.745

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

GENERAL  ASPECTS  OF  DIGITAL  TRANSMISSION  SYSTEMS

TERMINAL  EQUIPMENTS

SECOND  ORDER  DIGITAL  MULTIPLEX  EQUIPMENT  OPERATING  AT  8448 kbit/s 
AND  USING  POSITIVE/ZERO/NEGATIVE JUSTIFICATION

ITU-T  Recommendation  G.745

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.745 was published in Fascicle III.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.745

Fascicle III.4 - Rec. G.745

SECOND ORDER DIGITAL MULTIPLEX EQUIPMENT OPERATING AT

8448 kbit/s AND USING POSITIVE/ZERO/NEGATIVE JUSTIFICATION

(Geneva, 1976; amended at Geneva, 1980 and at Melbourne, 1988)

# 1	General

The second order digital multiplex equipment using positive/zero/negative justification, considered below, is intended for use on digital paths between countries using 2048 kbit/s primary multiplex equipments, such as the PCM multiplex equipment described in Recommendation G.732 or any identical equipment.

# 2	Bit rate

The nominal bit rate should be 8448 kbit/s. The tolerance on that rate should be ± 30 parts per million (ppm).

# 3	Frame structure

Table 1/G.745 gives:

-	the tributary bit rate and the number of tributaries;

-	the number of bits per frame;

-	the bit numbering scheme;

-	the bit assignment;

-	the bunched frame alignment signal.

# 4	Loss and recovery of frame alignment and consequent action

Loss of frame alignment should be assumed to have taken place when five consecutive frame alignment signals have been incorrectly received in their predicted positions.

Recovery of frame alignment should take place in the case of receiving without errors at least two consecutive frame signals in their predicted positions.

As soon as frame alignment has been lost and until it has been recovered, a definite pattern should be sent to all tributaries from the output of the demultiplexer. The equivalent binary content of this pattern, called the Alarm Indication Signal (AIS), at 2048 kbit/s is a continuous stream of binary 1s.

# 5	Multiplexing method

Cyclic bit interleaving in the tributary numbering order and positive/zero/negative justification with two-command control are recommended.

The justification control signal should be distributed and use the Cjn-bits (n = 1, 2, 3, see Table 1/G.745). Correction of one error in command is possible.

Positive justification should be indicated by the signal 111, transmitted in each of two consecutive frames; negative justification should be indicated by the signal 000 transmitted in each of two consecutive frames, and no justification by the signal 111 in one frame followed by 000 in the next frame. Bits 5, 6, 7 and 8 in Set IV (see Table 1/G.745) are used for negative justification of tributaries 1, 2, 3 and 4 respectively, and bits 9 to 12 for positive justification of the same tributaries.

Besides, when information from tributaries 1, 2, 3 and 4 is not transmitted, bits 5, 6, 7 and 8 in Set IV are available for transmitting information concerning the type of justification (positive or negative) in frames containing commands of positive justification control and intermediate amount of jitter in frames containing commands of negative justification.

Table 1/G.745 gives the maximum justification rate per tributary.

# 6	Jitter

The amount of jitter that should be tolerated at the input of the multiplexer and the demultiplexer should be according to Rec. G.823, § 3.1.1. The amount of jitter at the output of the multiplexer and the demultiplexer should be studied and specified.

# 7	Digital interface

The digital interfaces at 2048 kbit/s and 8448 kbit/s should be in accordance with Recommendation G.703.

# 8	Timing signal

It might be desirable to be able to derive the multiplexer timing signal from an external source as well as from an internal one.

# 9	Service digits

Some spare bits per frame are available for service functions (bits from 5 to 8 in Set II and bit 8 in Set III) for national and international use. Bits 5, 6, 7 and 8 in Set II are available for a digital service channel between two terminals (using 32 kbit/s Adaptive Delta Modulation) and bit 8 in Set III is available for ringing up a digital service channel. Utilization of other spare bits is under study.

TABLE 1/G.745

8448-kbit/s digital multiplexing frame structure using positive/zero/negative justification

| Tributary bit rate (kbit/s) | 2048 |
| --- | --- |
| Number of tributaries | 4 |
| Frame structure | Bit number |
| Frame alignment signal (11100110) Bits from tributaries Justification control bits (Cj1) (see Note) Bits for service functions Bits from tributaries Justification control bits (Cj2) (see Note) Spare bits Bits from tributaries Justification control bits (Cj3) (see Note) Bits from tributaries available for negative justification Bits from tributaries available for positive justification Bits from tributaries | Set I 1 to 8 9 to 264 Set II 1 to 4 5 to 8 9 to 264 Set III 1 to 4 5 to 8 9 to 264 Set IV 1 to 4 5 to 8 9 to 12 12 to 264 |
| Frame length Frame duration Bits per tributary Maximum justification rate per tributary | 1056 bits 125 ms 256 bits 8 kbit/s |

Note - Cjn indicates nth justification control bit of the jth tributary.

# 10	Fault conditions and consequent actions

10.1	The digital multiplex equipment should detect the following fault conditions:

10.1.1	Failure of power supply.

10.1.2	Loss of incoming signal at 2048 kbit/s at the input of the multiplexer.

Note - When using separate circuits for the digital signal and the timing signal, loss of either or both should constitute loss of the incoming signal.

10.1.3	Loss of the incoming signal at 8448 kbit/s at the input of the demultiplexer.

Note 1 - The detection of this fault condition is required only when it does not result in an indication of loss of frame alignment.

Note 2 - When using separate circuits for the digital signal and the timing signal, loss of either or both should constitute loss of the incoming signal.

10.1.4	Loss of frame alignment.

10.1.5	Alarm indication received from the remote multiplex equipment at the 8448 kbit/s input of the demultiplexer (see § 10.2.2).

10.2	Consequent actions

After detection of a fault condition appropriate actions should be taken as specified in Table 2/G.745. The consequent actions are as follows:

10.2.1	Prompt maintenance alarm indication generated to designate that the performance is below acceptable standards and maintenance attention is required locally. When detecting the AIS at the 8448 kbit/s input of the demultiplexer, the prompt maintenance alarm indication associated with loss of frame alignment should be prohibited (see Note 1 below).

Note - The location and provision of any visual and/or audible alarm activated by this prompt maintenance alarm indication is left to the discretion of each Administration.

10.2.2	Alarm indication to the remote multiplex equipment generated by changing from the state 0 to the state 1 bit 7 of set III at the 8448 kbit/s output of the multiplexer.

10.2.3	AIS (see Note 2 below) applied to all the four 2048 kbit/s tributary outputs from the demultiplexer.

10.2.4	AIS (see Note 2 below) applied to the 8448 kbit/s output of the multiplexer.

10.2.5	AIS (see Note 2 below) applied to the time slots of the 8448 kbit/s signal at the multiplexer output corresponding to the relevant 2048 kbit/s tributary.

Note 1 - The bit rate of the AIS at the output of the corresponding demultiplexer should be as specified for the tributaries. The method of achieving this is under study.

Note 2 - The equivalent binary content of the AIS at 2048 kbit/s and 8448 kbit/s is a continuous stream of binary 1s.

TABLE 2/G.745

Fault conditions and consequent actions

|  |  | Consequent actions (see §10.2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AIS applied |  |  |
| Equipment part | Fault condition (see § 10.1) | Prompt maintenance alarm indication generated | Alarm indication to the remote multiplexer generated | To all the tributaries | To the composite signal | To the relevant time slots of the composite signal |
| Multiplexer and demultiplexer _____________ | Failure of power supply | Yes | Yes, if practicable | Yes, if practicable | Yes, if practicable |  |
| Multiplexer only | Loss of incoming signal on a tributary | Yes |  |  |  | Yes |
|  | Loss of incoming signal at 8448 kbit/s | Yes | Yes | Yes |  |  |
| Demultiplexer | Loss of frame alignment | Yes | Yes | Yes |  |  |
| only | AIS received from the remote multiplexer |  |  |  |  |  |

Note - A Yes in the table signifies that a certain action should be taken as a consequence of the relevant fault condition. An open space in the table signifies that the relevant action should not be taken as a consequence of the relevant fault condition, if this condition is the only one present. If more than one fault condition is simultaneously present the relevant action should be taken if, for at least one of the conditions, a Yes is defined in relation to this action.