|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.746

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

GENERAL  ASPECTS  OF  DIGITAL  TRANSMISSION  SYSTEMS

TERMINAL  EQUIPMENTS

CHARACTERISTICS  OF  SECOND  ORDER  PCM  MULTIPLEX  EQUIPMENT  OPERATING  AT  6312 kbit/s

ITU-T  Recommendation  G.746

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.746 was published in Fascicle III.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.746

Fascicle III.4 - Rec. G.746

CHARACTERISTICS OF SECOND ORDER PCM MULTIPLEX

EQUIPMENT OPERATING AT 6312 kbit/s

(Malaga-Torremolinos, 1984)

# 1	General characteristics

1.1	Fundamental characteristics

The encoding law used is the m-law as specified in Recommendation G.711. The sampling rate, load capacity and the code are also specified in that Recommendation.

The number of quantized values is 255. Two character signals are reserved for zero value (11111111 and 01111111).

In some networks the all 0 character signal (00000000) is eliminated to avoid loss of timing information to the digital line, resulting in 254 quantized values.

1.2	Bit rate

The nominal bit rate is 6312 kbit/s. The tolerance on this rate is ± 30 parts per million (ppm).

1.3	Timing signal

It should be possible to derive the transmitting timing signal of a PCM multiplex equipment from an internal source, from the incoming digital signal and also from an external source.

# 2	Frame structure

Refer to §§ 3.2.1 and 3.2.2 of Recommendation G.704 for frame structure and use of derived channel time slots.

# 3	Loss and recovery of frame alignment

The strategy for the loss and recovery of frame alignment should be according to Rec. G.706, § 3.1.

# 4	Fault conditions and consequent actions

4.1	Fault conditions

The PCM multiplex equipment should detect the following conditions:

4.1.1	Failure of power supply.

4.1.2	Loss of incoming signals at 6312 kbit/s.

4.1.3	Loss of frame alignment.

4.1.4	Alarm indication received from the remote PCM multiplex equipment.

4.2	Consequent actions

Further to the detection of a fault condition, appropriate actions should be taken as specified in Table 1/G.746. The consequent actions are as follows:

4.2.1	A service alarm indication should be generated to signify that the service provided by the PCM multiplex is no longer available. This indication should be forwarded to the switching and/or signalling equipment depending upon the arrangement provided.

TABLE 1/G.746

Fault conditions and consequent actions for the PCM multiplex equipment

|  |  | Consequent actions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Equipment part | Fault condition | Service alarm indication generated | Prompt maintenance alarm indication generated | Alarm indication to the remote end generated | Transmission suppressed at the analogue outputs |
| Multiplexer and demultiplexer | Failure of power supply | Yes | Yes | Yes (if practicable) | Optional |
|  | Loss of incoming signal at 6312 kbit/s | Yes | Yes | Yes | Yes |
| Demultiplexer only | Loss of frame alignment | Yes | Yes | Yes | Yes |
|  | Alarm indication received from the remote end | Optional | Yes |  | Optional |

Note 1- A Yes in the table signifies that a certain action should be taken as a consequence of the relevant fault condition. An open space in the table signifies that the relevant action should not be taken as a consequence of the relevant fault condition, if this condition is the only one present. If more than one fault condition is simultaneously present the relevant action should be taken if, for at least one of the conditions, a Yes is defined in relation to this action.

Note 2 - Indications of additional fault conditions, such as codec failure and excessive bit errors, are left to the discretion of individual Administrations.

4.2.2	The service alarm described in § 4.2.1 above should be used to automatically remove the associated circuits from service and to restore them to service when frame alignment has been recovered.

Note - The removal of the associated circuits described in § 4.2.2 above should be done in such a way that the circuits are not needlessly removed in the case of a brief isolated loss of frame alignment but are removed in the case of a permanent or intermittent loss of frame alignment.

It is also important to minimize the impact of signalling errors which may occur during periods of loss of frame alignment. These functions should be provided in the PCM multiplex equipment or in the switching/signalling equipment.

4.2.3	A prompt maintenance alarm indication should be generated to signify that performance is below acceptable standards and maintenance attention is required locally.

4.2.4	An alarm indication to the remote end should be generated by forcing bit a to the value 1.

4.2.5	Transmission should be suppressed at the analogue outputs.

4.2.6	Rapid indication of loss of frame alignment

An indication should be given to the Signalling System No. 6 equipment (digital version) when the PCM multiplex equipment (local end only) detects a loss of frame alignment. The average time to detect and give an indication of random bits in the frame alignment signal bit positions should not be greater than 3 ms. This indication will serve the same function as that provided by the data carrier failure alarm in the analogue version (see Recommendation Q.275 [1]).

5	Signalling

5.1	Signalling arrangement

Refer to § 3.2.3 of Recommendation G.704.

5.2	Loss of multiframe alignment in case of channel associated signalling

Loss of multiframe alignment is assumed to have taken place when loss of frame alignment occurs.

6	Interfaces

Analogue: Refer to Recommendations G.712, G.713 and G.714.

Digital: Refer to Recommendation G.703.

Reference

[1]	CCITT Recommendation Data channel failure detection, Vol. VI, Rec. Q.275.