|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.752

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

GENERAL  ASPECTS  OF  DIGITAL  TRANSMISSION  SYSTEMS

TERMINAL  EQUIPMENTS

CHARACTERISTICS  OF  DIGITAL  MULTIPLEX  EQUIPMENTS  BASED  ON  A  SECOND  ORDER  BIT  RATE  OF  6312 kbit/s  AND  USING  POSITIVE  JUSTIFICATION

ITU-T  Recommendation  G.752

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.752 was published in Fascicle III.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.752

Fascicle III.4 - Rec. G.752

CHARACTERISTICS OF DIGITAL MULTIPLEX EQUIPMENTS BASED

ON A SECOND ORDER BIT RATE OF 6312 kbit/s AND USING POSITIVE JUSTIFICATION

(Geneva, 1976; amended at Geneva, 1980)

The CCITT,

considering

(a)	that various third- and higher-order multiplex equipments exist due to the differing characteristics of networks and signal sources in those networks;

(b)	that, although studies will continue with the aim of reducing the differences between various systems, the existing situation cannot be changed in the near future;

recommends the following

(1)	when countries using 1544 kbit/s primary multiplex equipments, such as the PCM multiplex equipment according to Recommendation G.733 and second-order multiplex using 6312 kbit/s according to Recommendations G.743 and G.746, are planning digital paths requiring interconnection at higher bit rates they should, when practical, utilize third-order bit rates of either 32 064 kbit/s or 44 736 kbit/s. When countries using 32 064 kbit/s third-order multiplex equipments are planning digital paths requiring interconnection at higher bit rates, they should, when practical, utilize the fourth-order bit rate of 97 728 kbit/s.

For Figure 1/G.752 refer to Figure 1/G.702 for the basic multiplex arrangements recommended for Administrations using 1544 kbit/s primary multiplex equipment. The bit rates of terrestrial systems should accommodate multiples of 1544 kbit/s. Whenever practical, the bit rate should also accommodate a multiple of 6312 kbit/s, either 32 064 or 44 736 kbit/s, and 97 728 kbit/s;

(2)	the characteristics of the third-order multiplex equipments using positive justification is given in § 1, below;

(3)	the characteristics of the fourth-order multiplex equipments using positive justification is given in § 2 below.

# 1	Third-order digital multiplex equipment based on second-order bit rate of 6312 kbit/s and using positive justification

1.1	General

The third-order digital multiplex equipment using positive justification described below, is intended for use on digital paths and between countries using 1544 kbit/s and 6312 kbit/s primary and secondary multiplex equipments.

A bit rate of either 32 064 kbit/s or 44 736 kbit/s is recommended to allow for the efficient and economical coding of wideband signals in the networks of Administrations using primary systems according to Recommendations G.733 and G.743. For instance for a 300 voice-circuit mastergroup (Recommendation G.233 [1]) 32 064 kbit/s is appropriate, while for a 600 voice-circuit mastergroup 44 736 kbit/s coding is appropriate.

1.2	Third-order digital multiplex equipment operating at 32 064 kbit/s

1.2.1	Bit rate

The nominal bit rate should be 32 064 kbit/s. The tolerance on that rate should be ± 10 parts per million (ppm).

1.2.2	Frame structure

Table 1/G.752 gives:

-	the tributary bit rate and the number of tributaries;

-	the number of bits per frame;

-	the bit numbering scheme;

-	the bit assignment;

-	the frame alignment signal.

TABLE 1/G.752

32 064 kbit/s multiplexing frame structure

| Tributary bit rate (kbit/s) | 6312 |
| --- | --- |
| Number of tributaries | 5 |
| Frame structure | Bit number |
| Bits for frame alignment signal (see Note 1) Bits from tributaries Cj1 (j = 1 to 5) for justification control signal (see Note 2) Bits from tributaries Cj2 (j = 1 to 5) for justification control signal (see Note 2) Bits from tributaries Bits for frame alignment (see Note 2) Bits from tributaries Cj3 (j = 1 to 5) for justification control signal (see Note 2) Bits from tributaries Auxiliary bits Hn (n = 1, 2, 3, 4, 5) (see Note 4) Bits from tributaries | Set I 1 to 5 6 to 320 Set II 1 to 5 6 to 320 Set III 1 to 5 6 to 320 Set IV 1 to 5 6 to 320 Set V 1 to 5 6 to 320 Set VI 1 to 5 6 to 320 |
| Frame length Bits per tributary (including justification) Maximum justification rate per tributary Nominal justification ratio | 1 920 bits 378 bits 16 700 bits 0.036 |

Note 1 - The frame alignment signal is a 11010 pattern.

Note 2 - Cjn indicates the nth justification control bit of the jth tributary (j = 1 to 5).

Note 3 - The frame alignment signal is a 00101 pattern.

Note 4 - H5 is used for transmitting failure information from the receive end to the transmit end.

Note 5 - The bit available for the justification of each tributary is the first slot of the tributary in set VI.

1.2.3	Loss and recovery of frame alignment and consequent action

The frame alignment recovery time should not exceed 8 ms. The signal to be applied to the tributaries during the out-of-frame alignment time should be studied.

1.2.4	Multiplexing method

Cyclic bit interleaving in the tributary numbering order and positive justification is recommended.

The justification control signal should be distributed and the Cjn-bits should be used (n = 1, 2, 3, see Table 1/G.752).

Positive justification should be indicated by the signal 111, no justification by the signal 000. Majority decision is recommended.

Table 1/G.752 gives the maximum justification rate per tributary and the nominal justification ratio.

1.2.5	Jitter

1.2.5.1	Specifications at the input ports

The digital signal presented at the input ports shall be as defined in Recommendation G.703 modified by the transmission characteristic of the interconnecting cable. The input ports shall be able to tolerate a digital signal with these electrical characteristics but modified by sinusoidal jitter up to the limits specified by the amplitude frequency relationship in Figure 2/G.752. The equivalent binary content of the signal, with jitter modulation, applied to the inputs shall be a pseudo-random bit sequence of length 215 - 1.

Note - The signal with jitter modulation applied to the demultiplexer input shall contain the bits necessary for framing and justification in addition to information bits.

| Input | A1 (UI) | A2 (UI) | f1 (Hz) | f2 (kHz) | f3 (kHz) | f4 (kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 6312 kbit/s | 1 | 0.05 | 60 | 1.6 | 32 | 160 |
| 32 064 kbit/s (provisional) | 5 | 0.05 | 60 | 1.6 | 160 | 800 |

UI Unit interval

FIGURE 2/G.752

Lower limit of maximum tolerable input jitter

1.2.5.2	Multiplex signal output jitter

The jitter at the 32 064 kbit/s output of the multiplexer should not exceed 0.01 UI rms.

1.2.5.3	Demultiplexer output jitter with no multiplexer or demultiplexer input jitter

The peak-to-peak jitter at a tributary output of the demultiplexer with no jitter at the inputs should not exceed 0.2 UI.

1.2.5.4	Demultiplexer jitter transfer characteristic

A 6312 kbit/s signal, modulated by sinusoidal jitter, should be subject to a demultiplexer jitter transfer characteristic within the gain/frequency limits given in Figure 3/G.752.

Note - The frequency f0 should be as low as possible taking into account the limitations of measuring equipment.

FIGURE 3/G.752

Demultiplexer transfer characteristic

1.2.6	Digital interface

The digital interfaces at 6312 kbit/s and 32 064 kbit/s should be in accordance with Recommendation G.703.

1.2.7	Timing signal

If it is economically feasible, it may be desirable to be able to derive the multiplexer timing signal from an external source as well as from an internal source.

1.2.8	Service digits

The service digits are reserved for national use.

1.3	Third-order digital multiplex operating at 44 736 kbit/s

1.3.1	Bit rate

The nominal bit rate should be 44 736 kbit/s. The tolerance on that rate should be ± 20 parts per million (ppm).

1.3.2	Frame structure (see Table 2/G.752).

1.3.3	Loss and recovery of frame and multiframe alignment and consequent action

The frame alignment recovery time should not exceed 2.5 ms. The signal to be applied to the tributaries during the out-of-frame alignment time should be studied. Once frame alignment is established, multiframe alignment should be recovered in less than 250 ms.

1.3.4	Multiplexing method

Cyclic bit interleaving in the tributary numbering order and positive justification is recommended.

The justification control signal should be distributed and the Cjn-bits should be used (n = 1, 2, 3, see Table 2/G.752).

Positive justification should be indicated by the signal 111, no justification by the signal 000. Majority decision is recommended.

Table 2/G.752 gives the maximum justification rate per tributary and the nominal justification ratio.

TABLE 2/G.752

44 736-kbit/s multiplexing frame structure

| Tributary bit rate (kbit/s) | 6312 |
| --- | --- |
| Number of tributaries | 7 |
| Frame structure (see Note 1) | Bit number |
| Bit for multiframe alignment signal (Mj) (see Note 1) Bits from tributaries 1st bit for frame alignment signal (F11) (see Note 2) Bits from tributaries 1st bit for justification control signal (Cj1) Bits from tributaries 2nd bit for frame alignment signal (F0) Bits from tributaries 2nd bit for justification control signal (Cj2) Bits from tributaries 3rd bit for frame alignment signal (F0) Bits from tributaries 3rd bit for justification control signal (Cj3) Bits from tributaries 4th bit frame alignment signal (F12) Bits from tributaries (see Note 3) | Set I 1 2 to 85 Set II 1 2 to 85 Set III 1 2 to 85 Set IV 1 2 to 85 Set V 1 2 to 85 Set VI 1 2 to 85 Set VII 1 2 to 85 Set VIII 1 2 to 85 |
| Frame length Multiframe length Bits per tribury per multiframe (including justification) Maximum justification rate per tributary Nominal justification ratio | 680 bits 4760 bits 672 bits 9398 bit/s 0.390 |

Note 1 - This frame is repeated 7 times to form a multiframe with frames designated j = 1, 2, 3, 4, 5, 6, 7. The multiframe alignment signal is an XXPP010 pattern where X is a bit assigned to service function, and P is the parity bit for the preceding multiframe (i.e. from M1 to M7). P = 1 if the number of marks in all tributary bits in the preceding multiframe is odd, P = 0 if the number of marks in all tributary bits in the preceding multiframe is even. Note that the two X bits are identical in any multiframe, as are the two P bits.

Note 2 - The frame alignment signal is F0 = 0 and F11 = F12 = 1.

Note 3 - The bit available for the justification of tributary j is the first slot of tributary j following F12 in the jth frame.

1.3.5	Jitter

1.3.5.1	Specifications at the input ports

The digital signal presented at the input ports shall be as defined in Recommendation G.703 modified by the transmission characteristic of the interconnecting cable. The input ports shall be able to tolerate a digital signal with these electrical characteristics but modified by sinusoidal jitter up to the limits specified by the amplitude frequency relationship in Figure 4/G.752. The equivalent binary content of the signal, with jitter modulation, applied to the inputs shall be a pseudo-random bit sequence of length 215 - 1.

Note - The signal with jitter modulation applied to the demultiplexer input shall contain the bits necessary for framing and justification in addition to information bits.

| Input | A1 (UI) | A2 (UI) | f1 (Hz) | f2 (kHz) | f3 (kHz) | f4 (kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 6312 kbit/s | 2 | 0.05 | 10 | 0.6 | 24 | 120 |
| 44 736 kbit/s | 14 | 0.05 | 10 | 3.2 | 900 | 4500 |

UI Unit interval

FIGURE 4/G.752

Lower limit of maximum tolerable input jitter

1.3.5.2	Multiplex signal output jitter

The jitter at the 44 736 kbit/s multiplexer output should not exceed 0.01 UI rms.

1.3.5.3	Demultiplexer output jitter with no multiplexer or demultiplexer input jitter

With no jitter at the input to the multiplexer and demultiplexer, the jitter at the demultiplexer output should not exceed 1/5 UI peak-to-peak.

1.3.5.4	Demultiplexer jitter transfer characteristic

The gain of the jitter transfer characteristic should not exceed the limits given in Figure 5/G.752.

Note - The frequency f0 should be as low as possible taking into account the limitations of measuring equipment.

FIGURE 5/G.752

Demultiplexer transfer characteristic

1.3.6	Digital interfaces

The digital interfaces at 6312 kbit/s and 44 736 kbit/s should be in accordance with Recommendation G.703.

1.3.7	Timing signal

If it is economically feasible, it may be desirable to be able to derive the multiplexer timing signal from an external source as well as from an internal source.

1.3.8	Service digits

The service digits are reserved for national use.

# 2	Fourth-order multiplex equipment operating at 97 728 kbit/s

2.1	Bit rate

The nominal bit rate should be 97 728 kbit/s. The tolerance on that rate should be ± 10 parts per million (ppm).

2.2	Frame structure

Table 3/G.752 gives:

-	the tributary bit rate and the number of tributaries;

-	the number of bits per frame;

-	the bit numbering scheme;

-	the bit assignment;

-	the frame alignment signal.

2.3	Loss and recovery of frame alignment and consequent action

The mean frame alignment recovery time should not exceed 1 ms.

The signal to be applied to the tributaries during the out-of-frame alignment time should be all 1s pattern.

2.4	Multiplexing method

Cyclic bit interleaving in the tributary numbering order and positive justification is recommended.

The justification control signal should be distributed and the Cjn-bits should be used (n = 1, 2, 3, see Table 3/G.752).

Positive justification should be indicated by the signal 111, no justification by the signal 000. Majority decision is recommended.

Table 3/G.752 gives the maximum justification rate per tributary and the nominal justification ratio.

TABLE 3/G.752

97 728 kbit/s multiplexing frame structure

| Tributary bit rate (kbit/s) | 32 064 |
| --- | --- |
| Number of tributaries | 3 |
| Frame structure | Bit number |
| Frame alignment signal (110) Bits from tributaries Justification control bits Cj1 (see Note 1) Bits from tributaries For justification control bits Cj2 (see Note 1) Bits from tributaries Frame alignment signal (001) Bits from tributaries Justification control bits Cj3 (see Note 1) Bits from tributaries Auxiliary bits Hn (n = 1, 2, 3) (see Note 2) Bits from tributaries | Set I 1 to 3 4 to 192 Set II 1 to 3 4 to 192 Set III 1 to 3 4 to 192 Set IV 1 to 3 4 to 192 Set V 1 to 3 4 to 92 Set VI 1 to 3 4 to 192 |
| Frame length Bits per tributary per frame (including justification) Maximum justification rate per tribute Nominal justification ratio | 1 152 bits 378 bits 84 833 bits 0.035 |

Note 1 - Cjn indicates the nth justification bit of the jth tributary (j = 1, 2, 3).

Note 2 - This signal is H1 H2 H3 pattern where H1 is the parity bit for the preceding frame, and H2 is a bit reserved for national use, and H3 is a bit to transmit the failure information from the receiving end to the transmitting end.

Note 3 - The bit available for justification of tributary j is the first slot of tributary j following Hn.

2.5	Jitter

2.5.1	Specifications al the input ports

The digital signal presented at the input ports shall be as defined in Recommendation G.703 modified by the transmission characteristic of the interconnecting cable. The input ports shall be able to tolerate a digital signal with these electrical characteristics but modified by sinusoidal jitter up to the limits specified by the amplitude frequency relationship in Figure 6/G.752. The equivalent binary content of the signal, with jitter modulation, applied to the inputs shall be a pseudo-random bit sequence of length 215 - 1.

Note - The signal with jitter modulation applied to the demultiplexer input shall contain the bits necessary for framing and justification in addition to information bits.

| Input | A1 (UI) | A2 (UI) | f1 (Hz) | f2 (kHz) | f3 (kHz) | f4 (kHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 32 064 kbit/s | 2 | 0.2 | 10 | 0.8 | 8 | 400 |
| 97 728 kbit/s | 6 | 0.1 | 10 | 5.5 | 325 | 1000 |

UI Unit interval

FIGURE 6/G.752

Lower limit of maximum tolerable input jitter

(provisional)

2.5.2	Multiplex signal output jitter

The jitter at the 97 728 kbit/s output of the multiplexer should not exceed 0.01 UI rms.

2.5.3	Demultiplexer output jitter with no multiplexer or demultiplexer input jitter

The peak-to-peak jitter at a tributary output of the demultiplexer with no jitter at the inputs should not exceed 0.25 UI.

2.5.4	Demultiplexer jitter transfer characteristic

A 32 064 kbit/s signal, modulated by sinusoidal jitter, should be subject to a demultiplexer jitter transfer characteristic within the gain/frequency limits given in Figure 7/G.752.

Note - The frequency f0, should be as low as possible taking into account the limitations of measuring equipment.

FIGURE 7/G.752

Demultiplexer transfer characteristic

2.6	Digital interface

The digital interfaces at 32 064 kbit/s and 97 728 kbit/s should be in accordance with Recommendation G.703.

2.7	Service digits

The service digits are reserved for national use.

Reference

[1]	CCITT Recommendation Recommendations concerning translating equipments, Vol. III, Rec. G.233.