|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-T	G.755

TELECOMMUNICATION

STANDARDIZATION SECTOR

OF  ITU

GENERAL  ASPECTS  OF  DIGITAL  TRANSMISSION  SYSTEMS

TERMINAL  EQUIPMENTS

DIGITAL  MULTIPLEX  EQUIPMENT  OPERATING  AT  139 264 kbit/s  AND  MULTIPLEXING  THREE  TRIBUTARIES
AT  44 736 kbit/s

ITU-T  Recommendation  G.755

(Extract from the Blue Book)

# NOTES

1	ITU-T Recommendation G.755 was published in Fascicle III.4 of the Blue Book. This file is an extract from the Blue Book. While the presentation and layout of the text might be slightly different from the Blue Book version, the contents of the file are identical to the Blue Book version and copyright conditions remain unchanged (see below).

2	In this Recommendation, the expression “Administration” is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

ã  ITU  1988, 1993

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

Recommendation G.755

Fascicle III.4 - Rec. G.755

DIGITAL MULTIPLEX EQUIPMENT OPERATING AT 139 264 KBIT/S

AND MULTIPLEXING THREE TRIBUTARIES AT 44 736 KBIT/S

(Melbourne, 1988)

# 1	General

The digital multiplex equipment described in this Recommendation is intended for use between networks using different digital hierarchies as specified in Recommendations G.702 and G.802.

# 2	Bit rate

The bit rates of the tributary and multiplex signals should be 44 736 kbit/s ± 20 ppm and 139 264 kbit/s 
±15 ppm, respectively, as specified in Recommendation G.703.

# 3	Frame structure

Table 1/G.755 gives the recommended 139 264 kbit/s multiplexing frame structure.

# 4	Loss and recovery of frame alignment and consequent action

Loss of frame alignment should be assumed to have taken place when four consecutive frame alignment signals have been incorrectly received in their predicted positions.

When frame alignment is assumed to be lost, the frame alignment device should decide that such alignment has effectively been recovered when it detects the presence of three consecutive correct frame alignment signals.

The frame alignment device, having detected the appearance of a single correct frame alignment signal, should begin a new search for the frame alignment signal when it detects the absence of the frame alignment signal in one of the two following frames.

Note - As it is not strictly necessary to specify the detailed frame alignment strategy, any suitable frame alignment strategy may be used provided the performance achieved is at least as efficient in all respects as that obtained by the above frame alignment strategy.

5	Multiplexing and justification methods

Cyclic bit interleaving in the tributary numbering order and positive justification are recommended.

The justification control signal should be distributed and use the Cji -bits (j = 1, 2, 3; i = 1, 2, 3, 4, 5) (see Note 6 to Table 1/G.755).

Positive justification should be indicated by the justification control signal 11111 and no justification by the signal 00000. Majority decision is recommended.

Table 1/G.755 gives the maximum justification rate per tributary and the nominal justification ratio.

# 6	Jitter

6.1	Demultiplexer triburaty jitter transfer characteristic

The demultiplexer 44 736 kbit/s tributary jitter transfer characteristic should meet the gain/frequency limits given in Figure 1/G.755. The equivalent binary content of the test signal used should result in a tributary output signal of 1000.

TABLE 1/G.755

139 264 kbit/s multiplexing frame structure

| Nominal tributary bit rate (kbit/s) | 44 736 |
| --- | --- |
| Number of tributaries | 3 |
| Frame structure | Bit number |
| Frame alignment signal (111110100000) Bits from tributaries Justification control bits Cj1 (Note 1) Bits from tributaries Justification control bits Cj2 (Note 1) Bits from tributaries Justification control bits Cj3 (Note 1) Alarm indication to the remote multiplex equipment (Note 2) Parity bit (Notes 3, 4 and 5) Bits reserved for future use (Note 6) Bits from tributaries Justification control bits Cj4 (Note 1) Bits from tributaries Justification control bits Cj5 (Note 1) Bits from tributaries available for justification Bits from tributaries | Set I 1 to 12 13 to 159 Set II 1 to 3 4 to 159 Set III 1 to 3 4 to 159 Set IV 1 to 3 4 5 6 to 9 10 to 159 Set V 1 to 3 4 to 159 Set VI 1 to 3 4 to 6 7 to 159 |
| Frame length bits per tributary in a frame Maximum justification rate per tributary Nominal justification ratio | 954 bits 307 bits 146 kbit/s 0.545 |

Note 1 - Cji (j = 1, 2, 3; i = 1, 2, 3, 4, 5) indicates the ith justification control bit of the jth tributary.

Note 2 - See § 10.2.1.

Note 3 - The parity bit = 1 if the number of marks in all tributary bits including the bits in the justifiable time-slots in the preceding frame is odd; the parity bit = 0 if the number of marks in all tributary bits including the bits in the justifiable time-slots in the preceding frame is even.

Note 4 - It is recognized that existing multiplex equipment installed prior to adoption of this Recommendation does not insert the parity bit.

Note 5 - The implementation and the use of this parity bit procedure are for further study.

Note 6 - These bits should be set to 1 when not used.

Note 1 - This characteristic is usually measured between the high speed and low speed interfaces of the demultiplexer and the measurements are taken in unit intervals. It is then necessary to introduce a correction factor to account for the difference in the size of unit intervals.

Note 2 - In addition, the need to specify a muldex jitter transfer characteristic is for further study.

Note 3 - It is recognized that the existing multiplex equipment designed prior to the adoption of this Recommendation might need tributary test signals incorporating the 44 736 kbit/s frame structure defined in Recommendation G.572.

Note - The frequency f0 should be as low as possible, taking into account the limitations of measuring equipment In any case f0 should be no greater than 10 Hz. The selective measurement method should be used.

FIGURE 1/G.755

Demultiplexer tributary jitter transfer characteristic

6.2	Output jitter

6.2.1	Tributary output jitter

With no jitter applied to the input ports of the multiplexer and with the multiplexer directly connected to the demultiplexer, the peak-to-peak jitter at the tributary output port should not exceed 0.3 UI when measured over a one minute interval within the frequency range from f1 = 10 Hz to f4 = 400 Hz.

When measured with an instrument incorporating a bandpass filter having a lower cutoff frequency of f3 = 60 kHz, a roll-off of 20 dB/decade and an upper limit of f4 = 400 kHz, the peak-to-peak output jitter should not exceed 0.05 UI when it is measured over a one minute interval.

6.2.2	Multiplexer output jitter

The peak-to-peak jitter at the 139 264 kbit/s output port should not exceed 0.05 UI when it is measured over a one minute interval within the frequency range from 200 Hz to 3500 kHz.

6.3	Input jitter

6.3.1	Tributary input jitter

The 44 736 kbit/s input port should be capable of accommodating levels of input jitter up to the limits given in Figure 2/G.755.

Note - Current Recommendation G.703 does not refer to the jitter tolerated at the digital distribution frame at 44 736 kbit/s nor at the input port of equipment connected to this distribution frame.

| Peak-to-peak sinusoidal jitter amplitude |  | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- |
| A1 (UI) | A2 (UI) | f1 (Hz) | f2 (Hz) | f3 (kHz) | f4 (kHz) |
| 5.0 | 0.1 | 10 | 2.3 | 60 | 400 |

FIGURE 2/G.755

Lower limit of maximum tolerable sinusoidal input jitter

at 44 736 kbit/s

6.3.2	Demultiplexer input jitter

The 139 264 kbit/s input port should be capable of accommodating levels of input jitter up to the limits given in Recommendation G.823.

Note - The jitter accommodation requirement should be met when the jittered input signal is composed of the multiplexed tributary signals having any value of jitter allowed for the 44 736 kbit/s level.

# 7	Digital interfaces

The digital interfaces at 44 736 kbit/s and 139 264 kbit/s should be in accordance with Recommendation G.703.

# 8	Timing signal

If it is economically feasible, it may be desirable to be able to derive the multiplexing timing signal from an external source as well as from an internal source.

# 9	Service digits

Six bits per frame are available for service functions (see Table 1/G.755): bit 4 of Set IV is used to transmit an alarm indication to the remote multiplex equipment when specific fault conditions are detected in the multiplex equipment when specific fault conditions are detected in the multiplex equipment (see § 10 below); bit 5 of Set IV may be used for a parity check; bits 6 to 9 of Set IV are reserved for further use.

# 10	Fault conditions and consequent actions

10.1	Fault conditions

10.1.1	The digital multiplex equipment should detect the following fault conditions:

1)	failure of power supply;

2)	loss of an incoming 44 736 kbit/s tributary signal at a multiplexer input port;

3)	loss of an incoming 139 264 kbit/s multiplex signal at a demultiplexer input port;

Note - The detection of this fault condition is required only when it does not result in an indication of loss of frame alignment.

4)	loss of frame alignment signal at a demultiplexer input port;

5)	detection of an alarm indication received from the remote multiplex equipment at a demultiplexer input port;

6)	detection of alarm indication signal (AIS) at a demultiplexer input port.

Note 1 - The equivalent binary content of the AIS at 139 264 kbit/s should be a continuous stream of binary 1s (marks) as recommended in Recommendation M.20.

Note 2 - The strategy for detecting the presence of the AIS should be such that the AIS is detectable even in the presence of an error ratio of 1 × 10-3. However, a signal with all bits except the frame alignment signal in the state of 1 should not be mistaken as an AIS.

10.1.2	The need to monitor the degradation of the incoming 139 264 kbit/s signal for the purpose of end-to-end error performance monitoring of the 139 264 kbit/s digital block, as well as the procedure for detecting such degradation are for further study.

10.2	Consequent actions

Further to the detection of a fault condition, the appropriate actions should be taken as specified in Table 2/G.755.

Note 1 - The concept and definition of prompt maintenance alarm indication is given in Recommendation M.20.

Note 2 - When the alarm indication signal (AIS) is detected at the input of the demultiplexer, the prompt maintenance alarm indication associated with loss of frame alignment should be inhibited, while the rest of the consequent actions are in accordance with those associated in Table 2/G.755 with the fault condition.

10.2.1	Alarm indication to the remote multiplex equipment should be generated by changing bit 4 of Set IV (see Table 1/G.755) from the state 0 to the state 1.

10.2.2	AIS should be applied to the following as specified in Table 2/G.755:

-	all three 44 736 kbit/s tributary outputs from the demultiplexer;

-	139 264 kbit/s output of the multiplexer;

-	the time slots of the 139 264 kbit/s signal at the output of the multiplexer, corresponding to the relevant 44 736 kbit/s tributary.

Note - The equivalent binary content of the AIS at 44 736 kbit/s is a signal with a valid frame alignment signal, parity and justification control bits as defined in Table 2/G.752, with the tributary bits being set to a 1010 ... sequence, starting with a binary 1 after each frame alignment, multi-frame alignment and justification control bit, and with all justification control bits being set to binary 0.

TABLE 2/G.755

Fault conditions and consequent actions

|  |  | Consequent actions (see §10.2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | AIS applied |  |  |
| Equipment part | Fault condition (see § 10.1) | Prompt maintenance alarm indication generated | Alarm indication to the remote multiplex equipment generated | To all the tributaries | To the composite signal | To the relevant time slots of the composite signal |
| Multiplexer and demultiplexer | Failure of power supply | Yes |  | Yes, if practicable | Yes, if practicable |  |
| Multiplexer only | Loss of incoming signal on a tributary | Yes |  |  |  | Yes |
|  | Loss of incoming signal at 139 264 kbit/s | Yes | Yes | Yes |  |  |
| Demultiplexer | Loss of frame alignment | Yes | Yes | Yes |  |  |
| only | Alarm indication received from the remote multiplex equipment |  |  |  |  |  |

Note - A Yes in the table signifies that a certain action should be taken as a consequence of the relevant fault condition. An open space in the table signifies that the relevant action should not be taken as a consequence of the relevant fault condition, if this condition is the only one present. If more than one fault condition is simultaneously present the relevant action should be taken if, for at least one of the conditions, a Yes is defined in relation to this action.