|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.7713/Y.1704 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | Amendment 1 (06/2004) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment SERIES Y: GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS Internet protocol aspects – Operation, administration and maintenance |  |  |  |
|  | Distributed call and connection management (DCM) Amendment 1 |  |  |  |
|  | ITU-T Recommendation G.7713/Y.1704 (2001) – Amendment 1 |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |
| QUALITY OF SERVICE AND PERFORMANCE - GENERIC AND USER-RELATED ASPECTS | G.1000–G.1999 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.6000–G.6999 |
| DIGITAL TERMINAL EQUIPMENTS | G.7000–G.7999 |
| General | G.7000–G.7099 |
| Coding of analogue signals by pulse code modulation | G.7100–G.7199 |
| Coding of analogue signals by methods other than PCM | G.7200–G.7299 |
| Principal characteristics of primary multiplex equipment | G.7300–G.7399 |
| Principal characteristics of second order multiplex equipment | G.7400–G.7499 |
| Principal characteristics of higher order multiplex equipment | G.7500–G.7599 |
| Principal characteristics of transcoder and digital multiplication equipment | G.7600–G.7699 |
| Operations, administration and maintenance features of transmission equipment | G.7700–G.7799 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.7800–G.7899 |
| Other terminal equipment | G.7900–G.7999 |
| DIGITAL NETWORKS | G.8000–G.8999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.7713/Y.1704 Distributed call and connection management (DCM) Amendment 1

Summary This amendment contains additional material and changes to be incorporated into ITU-T Rec. G.7713/Y.1704. The additions include alignment of call and connection separation with ITU-T Rec. G.8080/Y.1304, separation of call and connection parameters, and terminology alignment with ITU-T Rec. G.8080/Y.1304. Throughout this amendment, text that directs what additions and modifications are to be made to ITU-T Rec. G.7713/Y.1704 is italicized. Also, the clause numbering is parallel to that of ITU-T Rec. G.7713/Y.1704 so that changes can be correlated between this amendment and the Recommendation. Where there is no text following a clause heading, there are no changes to the corresponding clause contents.

Source Amendment 1 to ITU-T Recommendation G.7713/Y.1704 (2001) was approved on 13 June 2004 by ITU-T Study Group 15 (2001-2004) under the ITU-T Recommendation A.8 procedure.

Keywords The following keywords are added: ASON, call control, connection management, signalling Some keywords are replaced as indicated below: Existing; Replacement. Exterior Network-Node Interface; External NNI. Interior Network-Node Interface; Internal NNI.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

Compliance with this Recommendation is voluntary. However, the Recommendation may contain certain mandatory provisions (to ensure e.g. interoperability or applicability) and compliance with the Recommendation is achieved when all of these mandatory provisions are met.  The words "shall" or some other obligatory language such as "must" and the negative equivalents are used to express requirements. The use of such words does not suggest that compliance with the Recommendation is required of any party.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2005

All rights reserved. No part of this publication may be reproduced, by any means whatsoever, without the prior written permission of ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and definitions		1

4	Abbreviations		2

5	Conventions		3

6	DCM requirements		4

6.1	Distributed Call and Connection Management Operations procedures		4

6.2	Signalling Network Resilience		7

6.3	DCM Signal Flow – Exception Handling		7

7	DCM Attributes List		7

7.1	UNI Attributes List		10

7.2	I-NNI Attributes List		12

7.3	E-NNI Attributes List		12

8	DCM Message Sets		13

8.1	UNI Messages		13

8.2	I-NNI Messages		13

8.3	E-NNI Messages		14

9	DCM State Diagrams		15

10	Management of the Call and Connection Controller Function		15

ITU-T Recommendation G.7713/Y.1704

Distributed call and connection management (DCM) 

Amendment 1

# 1	Scope

This amendment provides updated material pertaining to the requirements of the ASON distributed call and connection management as described in ITU-T Rec. G.7713/Y.1704.

# 2	References

New references to be added alphanumerically are as follows:

–	ITU-T Recommendation G.8080/Y.1304 Amendment 1 (2003), Architecture for the automatic switched optical network (ASON).

–	ITU-T Recommendation Q.1901 (2000), Bearer Independent Call Control protocol.

–	ITU-T Recommendation Q.2931 and Amendments, Digital Subscriber Signalling System No. 2 – User-network interface (UNI) layer 3 specification for basic call/connection control.

–	ITU-T Recommendation Q.2982 (1999), Broadband integrated services digital network (B-ISDN) – Digital subscriber signalling system No. 2 (DSS2) – Q.2931-based separated call control protocol.

# 3	Terms and definitions

Add the following definitions directly after the heading:

3.1	call segment: An association between two call control entities (as per ITU-T Rec. Q.2982, which is equivalent to G.8080/Y.1304 call controllers), using a telecommunication service to concatenate a call.

3.2	signalling controller: A signalling controller contains the functions of connection control and/or call control.

Add the following new definitions:

a)	under terms defined in ITU-T Rec. G.8080/Y.1304:

–	Access Group Container

–	Domain

b)	The following term is defined in ITU-T Rec. G.783:

–	TPmode/PortMode

Delete the following terms from list of terms defined in ITU-T Rec. G.8080/Y.1304:

Neighbour Discovery

Subnetwork termination point

Subnetwork termination point pool

Replace the following term:

| Existing | Replacement |
| --- | --- |
| Route Controller | Routing Controller |

# 4	Abbreviations

Add the following new abbreviations alphabetically:

ACC-n	A-end CC at domain n

AGC	Access Group Container

ARC	Alarm Reporting Control (from ITU-T Rec. M.3100)

CC-a	A-end Connection Controller

CC-z	Z-end Connection Controller

CCC	Calling/Called Party Call Controller

CoS	Class of Service

GoS	Grade of Service

LC	Link Connection (from ITU-T Rec. G.805)

NCC	Network Call Controller

NCC-n	NCC in domain n

SC-a	A-end user Signalling Controller

SC-z		Z-end user Signalling Controller

TCC-n		Transit CC in domain n

ZCC-n		Z-end CC at domain n

Replace abbreviations as indicated below:

| Existing | Replacement |
| --- | --- |
| ANSN-n A-end Network SubNetwork-domain n | ASN-n A-end SN in domain n |
| ARA A-end Requester Agent | CCC-a A-end CCC |
| ASC-n A-end Subnetwork Controller-domain n | ASC-n A-end Signalling Controller in domain n |
| AUSN A-end User SubNetwork | AGC-a A-end AGC |
| INSN-n Intermediate Network SubNetwork- domain n | TSN-n Transit SN in domain n |
| ISC-n Intermediate SubNetwork Controller- domain n | TSC-n Transit Signalling Controller in domain n |
| PNNI Private Network-to-Network Interface | PNNI Private NNI |
| ZNSN-n Z-end Network SubNetwork-domain n | ZSN-n Z-end SN in domain n |
| ZRA Z-end Requester Agent | CCC-z Z-end CCC |
| ZSC-n Z-end SubNetwork Controller-domain n | ZSC-n Z-end Signalling Controller in domain n |
| ZUSN Z-end User SubNetwork | AGC-z Z-end AGC |

Delete the following abbreviations:

RA	Requester Agent

ND	Neighbour Discovery

# 5	Conventions

Replace Figure 5-1 and its legend with the following new figure. Delete the NOTE following the original diagram.

Figure 5-1/G.7713/Y.1704 – Reference diagram for distributed connection management

Add the following new text after Figure 5-1 to explain the terms used:

Transport plane components in Figure 5-1 are the various subnetworks and access group containers (AGCs). These define the locality to which the control plane functions are associated. They are labelled in Figure 5-1 as AGC-a, ASN-1, TSN-1, ZSN-1, ASN-n, ZSN-n, and AGC-z.

Distributed call and connection management is also known as "signalling" and this Recommendation will also use this convention. Call-related functions at end users are known as Calling/Called Party Call Controllers, or CCCs. An originating CCC is a "CCC-a" and destination CCC is a "CCC_z". Call controllers associated with a subnetwork are Network Call Controllers (NCC) and for a particular domain n, is an "NCC-n".

Connection control for end users is identified as CC-a, and CC-z. Within a domain n, the A-end, transit, and Z-end connection controllers are known as ACC-n, TCC-n, and ZCC-n.

A signalling controller contains the functions of connection control and/or call control. For end users, this is denoted as SC-a and SC-z. Within domain n, the A-end, transit, and Z-end signalling controllers are known as ASC-n, TSC-n, and ZSC-n. Note that TSCs usually do not have call control as shown in Figure 5-1.

An address for signalling control is assigned to the signalling controller and is used by protocol controller to exchange information between call controllers or between connection controllers. The signalling controller address is a control address, and the signalling channel will be identified by two adjacent signalling controller names. The signalling channel is provided by DCN communication.

# 6	DCM requirements

Throughout this clause, abbreviations and terms require changing as indicated in the table in clause 4 of this amendment. This affects text and Figures 6-1, 6-4, 6-5, 6-6, 6-7, 6-8, 6-9, 6-10, 6-11, 6-12, 6-13, 6-14, 6-15, 6-16, 6-17, 6-18, 6-19, 6-20, 6-21, 6-22, 6-23, and 6-24.

## 6.1	Distributed Call and Connection Management Operations procedures

Replace the first bullet in this clause as follows:

| Existing | Replacement |
| --- | --- |
| Route Controller (RC): Route Controller provides route information as queried by the CC | Routing Controller (RC): Routing Controller provides route information as queried by the CC |

Prior to the paragraph in 6.1

"As the communication between the controllers are defined as an external interface in ITU-T Rec. G.8080/Y.1304, messages are defined in this Recommendation to help the exchange of information."

Insert the following new text and figure:

As described in ITU-T Rec. G.8080/Y.1304, the calling party call controller interacts with a called party call controller by means of one or more intermediate network call controllers (NCC). The NCC function is provided at the network edge (i.e., UNI reference point) and may also be provided at gateways between domains (i.e., E-NNI reference points). The functions performed by NCCs at the network edge are defined by the policies associated by interactions between users and network, and the functions performed by NCCs at domain boundaries are defined by the policies associated by the interactions between the domains. As such, an end-to-end call is considered to consist of multiple call segments, when the call traverses multiple domains. Each call segment could have one or more connections (LC or SNC) associated with it. This allows for flexibility in the choices of signalling, protection and recovery paradigms in different domains.

The number of connections associated with call segments may not be the same even in one end-to-end call. In Figure 6-Amd.1-1, the UNI call segment has one LC associated, the subnetwork call segment for domain 1 has 2 associated SNCs. This allows the network to have different policies in their domain.

Note that both calls and connections could be across intra-carrier E-NNI reference points. The concept of call segments and call/connection separation enables the following applications:

•	Domain-based protection. The number of SNCs could be different between domains.

•	Domain-based restoration.  SNC failure may not cause an LC to go down, and a rerouting procedure could be provided by network to restore the failed SNC (refer to ITU-T Rec. G.8080/Y.1304 Amendment 1).

Figure 6-Amd.1-1/G.7713/Y.1704 – Call segments and connections

The NCC at domain boundaries will also allow each domain to have independent functions, e.g., one domain could have 1+1 protection capability while another does not.

The NCC and CC at network edge and boundaries perform different functions.

The call controllers perform the following:

•	The NCC correlates the SNCs to the call.

•	The NCC works with the Calling/Called Party Call Controller at network edge to correlate LC(s) to the call.

•	The NCC works with its peer NCC at domain boundaries to correlate LC(s) to a call.

•	The NCC correlates the LC and SNCs that are associated with the same call.

The CC establishes the connections that are associated to each call segment.

### 6.1.1	Process for Call Request

#### 6.1.1.1	Setting Up a Call

Replace this clause and its Figure 6-4 with the following:

Figure 6-4 illustrates the setup of a call, and the associated signal flows between the relevant components.

Figure 6-4/G.7713/Y.1704 – Call setup request processing: Logical request progression

For a call request to set up a call, the steps include:

–	The Calling Party Call Controller (CCC-a) requests call setup. At the ingress NCC-1, processes are initiated to check the call request (this may include checking for authentication and integrity of the request, as well as constraints placed by policy decisions). The request is also sent to the intermediate Network Call Controllers. Processes included in the egress NCC (NCC-n associated with ZCC-n in Figure 6-4) may include verifying that the call request is accepted end-to-end (e.g., request for CCC-z call verification).

–	Upon successful checking, the Calling Party Call Controller (CCC-a) continues the call setup request by initiating a connection setup request to the CC. The process for connection setup request is described in 6.1.2. Note that, based on different protocol design decisions, initiation of connection setup request may occur in a different order as shown in Figure 6-4. The requirement is that a network connection is set up before the call is completed.

–	Upon successful indication by the connection setup request process (across all call segments), the call setup request is successfully completed, and transfer of user characteristic information may begin.

If the connection setup request process was unsuccessful, a call denied notification is sent to the user.

#### 6.1.1.2	Releasing a Call

Replace Figure 6-5 with the following:

Figure 6-5/G.7713/Y.1704 – Call release request processing: Logical request progression

Replace the 3 bullet paragraphs and their immediately preceding sentence with:

For a call release request originating from the calling party call controller as in Figure 6-5:

–	Check the call release request at the ingress Network Call Controller (ingress NCC-1). This may include checking for authentication and integrity of the request, as well as constraints placed by policy decisions.

–	Upon successful checking, the call release request continues by initiating a connection release request. The process for connection release request is described in 6.1.2. Note that, based on different protocol design decisions, initiation of connection release request may occur in different orders as shown in Figure 6-5. The requirement is that a connection is released before a call is released. If there are multiple connections associated with a call segment, all of them are released.

–	Upon indication by the connection release request process(es), the call release request is successfully completed.

Replace the NOTE with the following:

NOTE – Depending on the "characteristics" of the transport network (e.g., whether monitoring and trace is enabled), race conditions may occur between the call release request message and the connection release request. Based on this race condition between the signalling progression from CCC-a to CCC-z, and the transport signal (e.g., unequipped or OCI) progression from AGC-a to AGC-z, certain alarms may be raised at downstream subnetworks. To support such an environment, a mechanism is needed to allow for disabling/enabling of the monitoring/trace capabilities associated with the call prior to de-allocation of connections. For example, this may include initiation of ARC or TPmode/PortMode process prior to any initiation of connection release request. Defect reporting suppression may be needed to prevent triggering the protection/restoration process.

### 6.1.2	Process for Connection Request

## 6.2	Signalling Network Resilience

### 6.2.1	User signalling defect

### 6.2.2	Network signalling defect

## 6.3	DCM Signal Flow – Exception Handling

### 6.3.1	Setup Connection

#### 6.3.1.1	A-end CCC UNI defect (request message)

Title is changed from "ARA UNI defect (request message)".

#### 6.3.1.2	A-end CCC UNI defect (response message)

Title is changed from  "ARA UNI defect (response message)".

#### 6.3.1.3	Intra-domain and Inter-domain defects

### 6.3.1.4		Z-end CCC UNI defect

Title is changed from "ZRA UNI defect".

### 6.3.2	Existing Calls

### 6.3.3	Call Release

#### 6.3.3.1	A-end CCC or Z-end CCC initiated call release defect (request message)

Title is changed from "ARA or ZRA initiated call release defect (request message)".

#### 6.3.3.2	A-end CCC or Z-end CCC initiated call release defect (response message)

Title is changed from "ARA or ZRA initiated call release defect (response message)".

# 7	DCM Attributes List

Replace the first paragraph of this clause with the following:

The DCM attributes list may be separated into attributes associated with the call and attributes associated with the connection. Tables 7-1, 7-2 and 7-3 summarize a list of attributes that are considered for UNI, I-NNI and E-NNI signalling processing.

•	UNI signalling processing includes call attributes, and also connection attributes for setting up Link Connection(s) on user-to-network access links.

•	I-NNI signalling processing includes connection attributes. Call attributes must be exchanged between Call Controllers (e.g., ASC-n to ZSC-n in Figure 5-1). Many mechanisms used to achieve this are not part of this architecture. I-NNI signalling might be used to exchange call attributes by piggybacking them on connection-related messages but, if so, they do not form part of I-NNI processing.

•	E-NNI signalling processing includes call attributes, and also connection attributes for setting up Link Connection(s) on network-to-network access links.

All attributes represent the logical information that is exchanged across the respective interfaces to support the CCC/NCC, CC, and LRM. Note that protocol design decisions may result in aggregation (or segmentation) of some of this logical information; however, the functions supported by the information shall be present.

Replace Table 7-1 with the following:

Table 7-1/G.7713/Y.1704 – UNI Attributes List

|  | Attributes | Scope | Call vs Connection |
| --- | --- | --- | --- |
| Identity attributes | Calling UNI Transport Resource Address | End-to-end | call |
|  | Called UNI Transport Resource Address | End-to-end | call |
|  | Initiating CC/CallC name | Local | connection |
|  | Terminating CC/CallC name | Local | connection |
|  | Connection name | Local | connection |
|  | Call name | End-to-end | call |
| Service attributes | Calling AGC SNP ID | Local | connection |
|  | Calling AGC SNPP ID | Local | connection |
|  | Called AGC SNP ID | Local at remote end | connection |
|  | Called AGC SNPP ID | Local at remote end | connection |
|  | Directionality | Local | call/connection |
| Policy attributes | CoS | End-to-end (Note) | call |
|  | GoS | End-to-end (Note) | call |
|  | Security | Local | call/connection |
| NOTE – Although CoS and GoS are end-to-end in scope, their values may be changed as they cross domains. However, the policy associated with the requested service should be met. |  |  |  |

Replace Table 7-2 with the following:

| Table 7-2/G.7713/Y.1704 – I-NNI Attributes list |  |  |  |
| --- | --- | --- | --- |
|  | Attributes | Scope | Call vs Connection |
| Identity attributes | Calling UNI Transport Resource Address | Carry transparently | call |
|  | Called UNI Transport Resource Address | Carry transparently | call |
|  | Initiating CC name | Local | connection |
|  | Terminating CC name | Local | connection |
|  | Connection name | Global in one domain | connection |
|  | Call name | End-to-end | call |
| Service attributes | SNP ID | Local | connection |
|  | SNPP ID | Local | connection |
|  | Called AGC SNP ID | Carry transparently | connection |
|  | Called AGC SNPP ID | Carry transparently | connection |
|  | Directionality | Global in one domain | Call/connection |
| Policy attributes | CoS | Carry transparently | call |
|  | GoS | Carry transparently | call |
|  | Connection CoS | Global in one domain | connection |
|  | Connection GoS | Global in one domain | connection |
|  | Explicit resource list | Global in one domain | connection |
|  | Recovery | Global in one domain | connection |

Replace Table 7-3 with the following:

| Table 7-3/G.7713/Y.1704 – E-NNI Attributes List |  |  |  |
| --- | --- | --- | --- |
|  | Attributes | Scope | Call vs Connection |
| Identity attributes | Calling UNI Transport Resource Address | End-to-end or Carry transparently | call |
|  | Called UNI Transport Resource Address | End-to-end or Carry transparently | call |
|  | Initiating CC/CallC name | Local | connection |
|  | Terminating CC/CallC name | Local | connection |
|  | Connection name | Local | connection |
|  | Call name | End-to-end | call |
| Service attributes | SNP ID | Local | connection |
|  | SNPP ID | Local | connection |
|  | Called AGC SNP ID | Carry transparently | connection |
|  | Called AGC SNPP ID | Carry transparently | connection |
|  | Directionality | Local | Call/connection |
| Policy attributes | CoS | End-to-end | call |
|  | GoS | End-to-end | call |
|  | Security | Local | call/connection |
|  | Explicit resource list | Local | connection |
|  | Recovery | Local | connection |

## 7.1	UNI Attributes List

### 7.1.1	Identity Attributes

#### 7.1.1.1	Calling UNI Transport Resource Address

Title changed from "A-end user name".

Replace the text as follows:

This attribute is the G.8080/Y.1304 UNI Transport Resource Address used to reach the A-end Call Controller. The value of this attribute has to be globally unique and assigned by the service provider. For example, a user name may be an NSAP address assigned by service provider #1, while another user name may be an IPv6 address assigned by service provider #2. As the user name provides a globally unique identification of the users, different formats may co-exist.

#### 7.1.1.2	Called UNI Transport Resource Address

Title changed from "Z-end user name".

Replace the text as follows:

This attribute is the G.8080/Y.1304 UNI Transport Resource Address used to reach the Z-end Call Controller. Characteristics are the same as that specified for the Calling UNI Transport Resource Address.

#### 7.1.1.3	Initiating CC/CallC name

#### 7.1.1.4	Terminating CC/CallC name

#### 7.1.1.5	Connection name

#### 7.1.1.6	Call name

### 7.1.2	Service Attributes

#### 7.1.2.1	SNP ID

Add the following text at the end of existing paragraph:

The SNP ID in signalling messages include:

•	Calling AGC SNP ID – This ID is used to setup the LC on the calling AGC to network element access links.

•	Called AGC SNP ID – This ID is used to setup the LC on the called AGC to network element access links.

•	SNP ID – This ID is used to setup a LC on network element to network element access links.

#### 7.1.2.2	SNPP ID

Add the following text at the end of existing paragraph:

The SNPP ID in signalling messages include:

•	Calling AGC SNPP ID – This ID is used to setup the LC on the calling AGC to network element access links.

•	Called AGC SNPP ID – This ID is used to setup the LC on the called AGC to network element access links.

•	SNPP ID – This ID is used to setup a LC on the network element to network element access links.

#### 7.1.2.3	Directionality

### 7.1.3	Policy Attributes

#### 7.1.3.1	CoS and Connection CoS

Title changed from "Class of Service".

Add the following before the last sentence:

This call CoS attribute is part of the Service Level Agreement (SLA) of the calling party to specify the CoS. A translation function that translates the call CoS to a domain-specific CoS (connection CoS) value is required.

The connection CoS might be different from domain-to-domain, but the connection CoS in each domain has to meet the SLA to support end-to-end CoS requests.

#### 7.1.3.2	GoS and Connection GoS

Title changed from "Grade of Service".

Add the following before the last sentence (Example list …):

This call GoS attribute is part of the Service Level Agreement (SLA) of the calling party to specify the GoS. At the domain boundary a translation function that translates call GoS to domain-specific GoS (connection GoS) value is required.

The connection GoS will be different from domain-to-domain, but the connection GoS in each domain has to meet the SLA to support end-to-end GoS requests.

#### 7.1.3.3	Security

### 7.1.4	Status Attributes

## 7.2	I-NNI Attributes List

### 7.2.1	Identity Attributes

#### 7.2.1.1	Calling UNI Transport Resource Address

Title changed from "A-end user name".

#### 7.2.1.2	Called UNI Transport Resource Address

Title changed from "Z-end user name".

#### 7.2.1.3	Initiating CC/CallC name

#### 7.2.1.4	Terminating CC/CallC name

#### 7.2.1.5	Connection name

#### 7.2.1.6	Call name

### 7.2.2	Service Attributes

### 7.2.3	Policy Attributes

#### 7.2.3.1	CoS and Connection CoS

Title changed from "CoS".

#### 7.2.3.2	GoS and Connection GoS

Title changed from "GoS".

#### 7.2.3.3	Explicit resource list

#### 7.2.3.4	Recovery

### 7.2.4	Status Attributes

## 7.3	E-NNI Attributes List

### 7.3.1	Identity Attributes

#### 7.3.1.1	Calling UNI Transport Resource Address

Title changed from "A-end user name".

#### 7.3.1.2	Called UNI Transport Resource Address

Title changed from "Z-end user name".

#### 7.3.1.3	Initiating CC/CallC name

#### 7.3.1.4	Terminating CC/CallC name

#### 7.3.1.5	Connection name

#### 7.3.1.6	Call name

### 7.3.2	Service Attributes

### 7.3.3	Policy Attributes

### 7.3.4	Status Attributes

# 8	DCM Message Sets

## 8.1	UNI Messages

### 8.1.1	Call Setup

#### 8.1.1.1	Request: Setup Call

Replace Table 8-4 with the following:

| Table 8-4/G.7713/Y.1704 – UNI Call Setup Request Message |  |
| --- | --- |
| User sent attributes | Network sent attributes |
| Calling UNI Transport Resource Address | Calling UNI Transport Resource Address |
| Called UNI Transport Resource Address | Called UNI Transport Resource Address |
| Initiating CallC name | Initiating CC/CCC name |
| Terminating CallC name | Terminating CC/CCC name |
| Calling AGC SNP ID | Calling AGC SNP ID |
| Calling AGC SNPP ID | Calling AGC SNPP ID |
| Called AGC SNP ID | Called AGC SNP ID |
| Called AGC SNPP ID | Called AGC SNPP ID |
| Directionality | Directionality |
| CoS | CoS |
| GoS | GoS |
| Security | Security |
| connName | connName |
|  | callName |

#### 8.1.1.2	Indication: Setup Call

#### 8.1.1.3	Confirm: Setup Call

### 8.1.2	Call Release

### 8.1.3	Call Query

### 8.1.4	Notification

## 8.2	I-NNI Messages

### 8.2.1	Connection Setup

#### 8.2.1.1	Request: Setup Connection

Replace Table 8-14 with the following:

Table 8-14/G.7713/Y.1704 – I-NNI Connection Setup Request Message

Attributes

Calling UNI Transport Resource Address

Called UNI Transport Resource Address

Initiating CC name

Terminating CC name

connName

callName

Local SNP ID

Local SNPP ID

Called AGC SNP ID

Called AGC SNPP ID

Directionality

CoS

GoS

Connection CoS

Connection GoS

Explicit resource list

Recovery

#### 8.2.1.2	Indication: Setup Connection

#### 8.2.1.3	Confirm: Setup Connection

### 8.2.2	Connection Release

### 8.2.3	Connection Query

### 8.2.4	Notification

## 8.3	E-NNI Messages

### 8.3.1	Connection Setup

#### 8.3.1.1	Request: Setup Connection

Replace Table 8-24 with the following:

Table 8-24/G.7713/Y.1704 – E-NNI Connection
Setup Request Message

Attributes

Calling UNI Transport Resource Address

Called UNI Transport Resource Address

Initiating CC/NCC name

Terminating CC/NCC name

connName

callName

Local SNP ID

Local SNPP ID

Called AGC SNP ID

Called AGC SNPP ID

Directionality

CoS

GoS

Explicit resource list

Recovery

#### 8.3.1.2	Indication: Setup Connection

#### 8.3.1.3	Confirm: Setup Connection

### 8.3.2	Connection Release

### 8.3.3	Connection Query

### 8.3.4	Notification

# 9	DCM State Diagrams

No changes other than replacement of terms and abbreviations as indicated in clause 4 of this amendment.

# 10	Management of the Call and Connection Controller Function

No changes other than replacement of terms and abbreviations as indicated in clause 4 of this amendment.

ITU-T Y-SERIES  RECOMMENDATIONS

GLOBAL INFORMATION INFRASTRUCTURE, INTERNET PROTOCOL ASPECTS AND NEXT GENERATION NETWORKS

| GLOBAL INFORMATION INFRASTRUCTURE |  |
| --- | --- |
| General | Y.100–Y.199 |
| Services, applications and middleware | Y.200–Y.299 |
| Network aspects | Y.300–Y.399 |
| Interfaces and protocols | Y.400–Y.499 |
| Numbering, addressing and naming | Y.500–Y.599 |
| Operation, administration and maintenance | Y.600–Y.699 |
| Security | Y.700–Y.799 |
| Performances | Y.800–Y.899 |
| INTERNET PROTOCOL ASPECTS |  |
| General | Y.1000–Y.1099 |
| Services and applications | Y.1100–Y.1199 |
| Architecture, access, network capabilities and resource management | Y.1200–Y.1299 |
| Transport | Y.1300–Y.1399 |
| Interworking | Y.1400–Y.1499 |
| Quality of service and network performance | Y.1500–Y.1599 |
| Signalling | Y.1600–Y.1699 |
| Operation, administration and maintenance | Y.1700–Y.1799 |
| Charging | Y.1800–Y.1899 |
| NEXT GENERATION NETWORKS |  |
| Frameworks and functional architecture models | Y.2000–Y.2099 |
| Quality of Service and performance | Y.2100–Y.2199 |
| Service aspects: Service capabilities and service architecture | Y.2200–Y.2249 |
| Service aspects: Interoperability of services and networks in NGN | Y.2250–Y.2299 |
| Numbering, naming and addressing | Y.2300–Y.2399 |
| Network management | Y.2400–Y.2499 |
| Network control architectures and protocols | Y.2500–Y.2599 |
| Security | Y.2700–Y.2799 |
| Generalized mobility | Y.2800–Y.2899 |

For further details, please refer to the list of ITU-T Recommendations.

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure, Internet protocol aspects and Next Generation Networks |
| Series Z | Languages and general software aspects for telecommunication systems |