|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH) – Management information model for the network element view |  |  |  |
|  | ITU-T Recommendation G.774 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774 Synchronous digital hierarchy (SDH)  Management information model for the network element view

- Summary This Recommendation provides an information model for the synchronous digital hierarchy (SDH). This model describes the managed object classes and their properties that are useful to describe information exchanged across interfaces defined in ITU-T M.3010 telecommunications management network (TMN) architecture. This Recommendation specialises the generic object classes of ITU-T M.3100 to provide management information specifically for the SDH.

Document history. Issue; Notes. 2001; First revision incorporated the changes documented in the G.774 Corrigendum 1 (1996).. 09/1992; Initial version of the Recommendation..

- Source ITU-T Recommendation G.774 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

 ITU 2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and Definition		3

4	Abbreviations		3

5	SDH information model		4

5.1	Overview		4

5.2	Requirements		5

6	Object classes		5

6.1	Administrative Unit 3 Object Classes		6

6.2	Administrative Unit 4 Object Classes		7

6.3	Administrative Unit Group Object Classes		8

6.4	Electrical SPI trail termination point object classes		9

6.5	Indirect Adaptor Object Classes		10

6.6	Multiplex Section Connection Termination Point Object Classes		10

6.7	Multiplex Section Data Communications Channel Object Classes		11

6.8	Multiplex Section Orderwire Object Classes		11

6.9	Multiplex Section Trail Termination Point Object Classes		12

6.10	Optical SDH Physical Interface Trail Termination Point Object Classes		13

6.11	Regenerator Section Connection Termination Point Object Classes		14

6.12	Regenerator Section Data Communications Channel Object Classes		15

6.13	Regenerator Section Orderwire Object Classes		15

6.14	Regenerator Section Trail Termination Point Object Classes		16

6.15	Regenerator Section User Channel Object Classes		17

6.16	SDH Network Element Object Class		18

6.17	Tributary Unit 11 Object Classes		18

6.18	Tributary Unit 12 Object Classes		19

6.19	Tributary Unit 2 Object Classes		20

6.20	Tributary Unit 3 Object Classes		20

6.21	Tributary Unit Group 2 Object Classes		21

6.22	Tributary Unit Group 3 Object Classes		22

6.23	Virtual Container 11 Object Classes		23

6.24	Virtual Container 12 Object Classes		24

6.25	Virtual Container 2 Object Classes		24

6.26	Virtual Container 3 Object Classes		25

Page

6.27	Virtual Container 4 Object Classes		26

6.28	VC-n User Channel Object Classes		27

7	Packages		28

7.1	electricalSPIPackage		28

7.2	msCTPPackage		28

7.3	msTTPPackage		28

7.4	opticalSPIPackage		28

7.5	rsCTPPackage		29

7.6	rsTTPPackage		29

7.7	trailTraceSinkPackage		29

7.8	trailTraceSourcePackage		29

7.9	tu-nSinkPackage		29

7.10	vc11-2BidirectionalPackageR1		30

7.11	vc11-2SinkPackageR1		30

7.12	vc3-4BidirectionalPackageR1		30

7.13	vc3-4SinkPackageR1		30

7.14	vc3-4SourcePackageR1		31

8	Attributes		31

8.1	AU-3 Identification		31

8.2	AU-4 Identification		32

8.3	AUG Identification		32

8.4	C2 Signal Label Expected		32

8.5	C2 Signal Label Receive		32

8.6	C2 Signal Label Send		33

8.7	Electrical SDH Physical Interface Trail Termination Point Identification		33

8.8	Excessive Bit Error Ratio Maintenance Inhibit		33

8.9	J1 Path Trace Expected		33

8.10	J1 Path Trace Receive		34

8.11	J1 Path Trace Send		34

8.12	Multiplex Section Connection Termination Point Identification		34

8.13	MS Data Communications Channel CTP Identification		34

8.14	Multiplex Section Orderwire Identification		35

8.15	Multiplex Section Trail Termination Point Identification		35

8.16	Optical Reach		35

8.17	Optical SDH Physical Interface Trail Termination Point Identification		35

Page

8.18	Optical WaveLength		36

8.19	Pointer Sink Type		36

8.20	Pointer Source Type		36

8.21	Regenerator Section Connection Termination Point Identification		36

8.22	Regenerator Section Data Communications Channel CTP Identification		36

8.23	Regenerator Section Orderwire Identification		37

8.24	Regenerator Section Trail Termination Point Identification		37

8.25	Regenerator Section User Channel Identification		37

8.26	Signal Degrade Threshold		37

8.27	STM Level		38

8.28	Trail Trace Expected		38

8.29	Trail Trace Receive		38

8.30	Trail Trace Send		38

8.31	Tributary Unit 11 Connection Termination Point Identification		39

8.32	Tributary Unit 12 Connection Termination Point Identification		39

8.33	Tributary Unit 2 Connection Termination Point Identification		39

8.34	Tributary Unit 3 Connection Termination Point Identification		39

8.35	TUG-2 Identification		40

8.36	TUG-3 Identification		40

8.37	V5 Signal Label Expected		40

8.38	V5 Signal Label Receive		40

8.39	V5 Signal Label Send		41

8.40	Virtual Container 11 Trail Termination Point Identification		41

8.41	Virtual Container 12 Trail Termination Point Identification		41

8.42	Virtual Container 2 Trail Termination Point Identification		41

8.43	Virtual Container 3 Trail Termination Point Identification		42

8.44	Virtual Container 4 Trail Termination Point Identification		42

8.45	VC-n User Channel Identification		42

9	Name Bindings		42

9.1	au3CTP		44

9.2	au4CTP		45

9.3	aug		46

9.4	electricalSPITTP		47

9.5	msCTP		48

9.6	msDatacomCTP		49

9.7	msOrderwireCTP		50

Page

9.8	msTTP		51

9.9	opticalSPITTP		51

9.10	rsCTP		52

9.11	rsDatacomCTP		54

9.12	rsOrderwireCTP		55

9.13	rsTTP		56

9.14	rsUserChannelCTP		57

9.15	tu11CTP		58

9.16	tu12CTP		59

9.17	tu2CTP		60

9.18	tu3CTP		61

9.19	tug2		62

9.20	tug3		64

9.21	vc11TTP		64

9.22	vc12TTP		65

9.23	vc2TTP		65

9.24	vc3TTP		66

9.25	vc4TTP		67

9.26	vcnUserChannelCTP		67

10	Supporting ASN.1		69

11	Object relations		70

11.1	Syntax		70

11.1.1	Subordination rule templates		71

11.1.2	Constraint rule templates		72

11.2	Connectivity pointer constraints		73

11.3	Naming constraints		84

Annex A – Entity relationship diagrams		93

ITU-T Recommendation G.774

Synchronous digital hierarchy (SDH)  Management information model
for the network element view

# 1	Scope

This Recommendation provides an information model for the synchronous digital hierarchy (SDH). It identifies the telecommunications management network (TMN) object classes required for the management of SDH network elements. These objects are relevant to information exchanged across standardized interfaces defined in ITU-T M.3010 TMN architecture. The managed object classes in this Recommendation are specialized from the generic managed object classes defined in ITU-T M.3100 generic network information model.

This Recommendation applies to SDH network elements and those systems in the TMN that manage SDH network elements. Functional capabilities of SDH multiplex equipment are given in ITU-T G.783, and aspects of the management of SDH equipment are provided in ITU-T G.784. This Recommendation provides the management information required for use with the protocols specified in ITU-T G.784.

The new objects defined in this Recommendation supersede those defined in the previous version of the Recommendation (i.e. the 1992 version). For each object class, attribute, action, notification, parameter defined in this version of the Recommendation, it shall be indicated what the impacts are upon the existing ITU-T G.774 (1992).

Structure of this Recommendation

Clause 5 provides an overview of the SDH information model. Clauses 6 to 9 describe the information model using the notation mechanisms defined in ITU-T X.722 Guidelines for the definition of managed Objects. Section 10 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. The relationships between the SDH managed object classes contained in this Recommendation are defined in clause 11. Diagrams illustrating the construction of the SDH model are provided in Annex A.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy (SDH).

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774.1 (2001), Synchronous digital hierarchy (SDH)  Bidirectional performance monitoring for the network element view.

	ITU-T G.774.3 (2001), Synchronous digital hierarchy (SDH)  Management of multiplex-section protection for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section  fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunications management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology  Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology  Open Systems Interconnection  Systems management overview.

	ITU-T X.710 (1997), Information technology  Open Systems Interconnection  Common management information service.

	ITU-T X.711 (1997), Information technology  Open Systems Interconnection  Common management information protocol: Specification.

	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology  Open Systems Interconnection  Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology  Open Systems Interconnection  Systems management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology  Open Systems Interconnection  Systems management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology  Open Systems Interconnection  Systems management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.734 (1992), Information technology  Open Systems Interconnection  Systems management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.735 (1992), Information technology  Open Systems Interconnection  Systems management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

# 3	Terms and Definition

This Recommendation uses the terms and definitions defined in ITU-T G.783, G.784, and M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AIS	Alarm indication signal

ASN.1	Abstract syntax notation one

AU	Administrative unit

AUG	AU group

BER	Bit error ratio

CP	Connection point

CTP	Connection termination point

DS	Digital section

FERF	Far end receive failure

FO	Optical fibre

GDMO	Guidelines for the definition of managed objects

HPA	Higher order path adaptation

HPC	Higher order path connection

HPT	Higher order path termination

IA	Indirect adaptor

LOF	Loss of frame

LOP	Loss of pointer

LOS	Loss of signal

LPA	Lower order path adaptation

LPC	Lower order path connection

LPT	Lower order path termination

MS	Multiplex section

MSP	Multiplex section protection

MST	Multiplex section termination

NE	Network element

NNI	Network node interface

PDH	Plesiochronous digital hierarchy

POH	Path overhead

RDN	Relative distinguished name

RS	Regenerator section

RST	Regenerator section termination

SA	Section adaptation

SDH	Synchronous digital hierarchy

SOH	Section overhead

SPI	SDH physical interface

STM	Synchronous transfer mode

TMN	Telecommunications management network

TTP	Trail termination point

TU	Tributary unit

TUG	Tributary unit group

VC	Virtual container

# 5	SDH information model

## 5.1	Overview

The SDH information model is based on the Generic Network Information Model of ITU-T M.3100. The Generic Network Information Model includes a Termination Point fragment which serves as a structure for specialization of those object classes specific to the SDH network. It is these SDH specific object classes, along with the generic object classes in other fragments of the Generic Network Information Model (e.g. the cross-connection fragment and the equipment fragment), that are used to manage SDH network elements. The services used to manage the SDH resources represented by these object classes are provided in ITU-T M.3100 and other Recommendations.

The information exchanged at a management interface is modelled using design principles outlined in ITU-T X.720 Management Information Model. Resources are modelled as objects, and the management view of a resource is a managed object. Objects with similar attributes may be grouped into object classes. An object is characterized by its object class and object instance, and may possess multiple attribute types and associated values. The terms "managed object class" and "managed object instance" apply specifically to objects that are being managed. This Recommendation specifies the properties of the resource visible for management.

An object class may be a subclass of another class. A subclass inherits attribute types, packages and behaviours of its superclass, in addition to possessing its own specific attributes and properties. The SDH specific object classes are all derived from superclasses in the Generic Network Information Model ITU-T M.3100.

Object classes and attribute types are defined only for the purpose of communicating network management messages between systems, and need not be related to the structure of data within those systems. The object classes defined in this issue of the SDH information model can apply to multiple management functional areas (e.g. fault management and configuration management).

There are several different viewpoints of management information that may be defined for management purposes. The network element viewpoint is concerned with the information that is required to manage a network element. This refers to information required to manage the network element function and the physical aspects of the network element. This Recommendation addresses only the network element viewpoint of SDH management.

## 5.2	Requirements

To allow SDH equipment to be represented in a consistent manner across the interface, some of the conditional packages in ITU-T M.3100 are made mandatory in this Recommendation. The following conditional packages inherited from ITU-T M.3100 shall not be used when the SDH object classes defined in this Recommendation are instantiated: ttpInstancePackage, ctpInstancePackage, networkLevelPackage, characteristicInformationPackage, channelNumberPackage.

The SDH specific subclasses specified in this Recommendation shall be used to manage the specific transport resources of SDH network elements. Implementations shall conform to both the management information defined in clauses 6 to 10 and the requirements identified in this clause and clause 11.

In the context of this Recommendation, the various objects defined hereafter will be named using local distinguished naming.

# 6	Object classes

This clause provides replacement managed object class definitions for the existing Recommendation G.774 (1992). Any managed object class replaced by the one in this clause is considered to be deprecated. The reasons for the replacement of a managed object class are as follows:

a)	The replaced managed object class is faulty and must be fixed.

b)	The replaced managed object class includes an attribute, package, notification or action that has been re-registered in this Recommendation.

c)	The replaced managed object class inherits from a managed object class that has been re-registered in this Recommendation.

In each case where a class is replaced, the new class will be registered within this Recommendation. The textual label for the class will be revised to include the text "R1". For example, in the revision of the G.774 (1992) "au4CTPSink" managed object class, the revised label will become "au4CTPSinkR1".

Below is a table of classes deprecated from Recommendation G.774 (1992) and the new G.774 classes that replace them:

Deprecated G.774 (1992) Classes	Replacement G.774 Classes

au3CTPSink					au3CTPSinkR1

au3CTPBidirectional			au3CTPBidirectionalR1

au4CTPSink					au4CTPSinkR1

au4CTPBidirectional			au4CTPBidirectionalR1

tu11CTPSink					tu11CTPSinkR1

tu11CTPBidirectional			tu11CTPBidirectionalR1

tu12CTPSink					tu12CTPSinkR1

tu12CTPBidirectional			tu12CTPBidirectionalR1

tu2CTPSink					tu2CTPSinkR1

tu2CTPBidirectional			tu2CTPBidirectionalR1

tu3CTPSink					tu3CTPSinkR1

tu3CTPBidirectional			tu3CTPBidirectionalR1

vc11TTPBidirectional			vc11TTPBidirectionalR1

vc11TTPSink					vc11TTPSinkR1

vc12TTPBidirectional			vc12TTPBidirectionalR1

vc12TTPSink					vc12TTPSinkR1

vc2TTPBidirectional			vc2TTPBidirectionalR1

vc2TTPSink					vc2TTPSinkR1

vc3TTPBidirectional			vc3TTPBidirectionalR1

vc3TTPSink					vc3TTPSinkR1

vc3TTPSource					vc3TTPSourceR1

vc4TTPBidirectional			vc4TTPBidirectionalR1

vc4TTPSink					vc4TTPSinkR1

vc4TTPSource					vc4TTPSourceR1

New Object Classes to support trail trace management at regenerator section level:

rsTTPTrailTraceTrailTraceBidirectional

rsTTPTrailTraceSink

rsTTPTrailTraceSource

## 6.1	Administrative Unit 3 Object Classes

au3CTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointBidirectional,

au3CTPSinkR1,

au3CTPSource;

REGISTERED AS { g774ObjectClass 83 };

au3CTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100": connectionTerminationPointSink;

CHARACTERIZED BY
	"Recommendation M.3100": createDeleteNotificationsPackage,

"Recommendation M.3100": operationalStatePackage,

"Recommendation M.3100": stateChangeNotificationPackage,

"Recommendation M.3100": tmnCommunicationsAlarmInformationPackage,

au3CTPSinkR1Pkg PACKAGE

BEHAVIOUR

au3CTPSinkR1PkgBehaviour BEHAVIOUR

DEFINED AS

*This object class represents a termination point where an AU-3 Connection is terminated.

The AU-3 consists of a VC-3 plus an AU pointer which indicates the phase alignment of the VC-3 with respect to the STM-N frame.

A communicationsAlarm notification shall be issued if a loss of AU pointer is detected.

The probableCause parameter of the notification shall indicate LOP (Loss Of Pointer).

A communicationsAlarm notification shall be issued if an AU path alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).

A change in the operational state shall cause a state change notification *

;;

ATTRIBUTES

au3CTPId			GET,

pointerSinkType	GET;

;;

REGISTERED AS { g774ObjectClass 84 };

au3CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

au3CTPSourcePkg PACKAGE

BEHAVIOUR

au3CTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class represents a termination point where an AU-3 Connection is originated.

The AU-3 consists of a VC-3 plus an AU pointer which indicates the phase alignment of the VC-3 with respect to the STM-N frame.*

;;

ATTRIBUTES

au3CTPId				GET,

pointerSourceType		GET;

;;

REGISTERED AS { g774ObjectClass 3 };

## 6.2	Administrative Unit 4 Object Classes

au4CTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointBidirectional,

au4CTPSinkR1,

au4CTPSource;

REGISTERED AS { g774ObjectClass 85 };

au4CTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

au4CTPSinkR1Pkg PACKAGE

BEHAVIOUR

au4CTPSinkR1PkgBehaviour BEHAVIOUR

DEFINED AS

*This object class represents a termination point where an AU-4 Connection is terminated.

The AU-4 consists of a VC-4 plus an AU pointer which indicates the phase alignment of the VC-4 with respect to the STM-N frame.

A communicationsAlarm notification shall be issued if a loss of AU pointer is detected.

The probableCause parameter of the notification shall indicate LOP (Loss Of Pointer).

A communicationsAlarm notification shall be issued if an AU path alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).

A change in the operational state shall cause a state change notification *

;;

ATTRIBUTES

au4CTPId				GET,

pointerSinkType		GET;

;;

REGISTERED AS { g774ObjectClass 86 };

au4CTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

au4CTPSourcePkg PACKAGE

BEHAVIOUR

au4CTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class represents a termination point where an AU-4 Connection is originated.

The AU-4 consists of a VC-4 plus an AU pointer which indicates the phase alignment of the VC-4 with respect to the STM-N frame.*

;;

ATTRIBUTES

au4CTPId				GET,

pointerSourceType		GET;

;;

REGISTERED AS { g774ObjectClass 6 };

## 6.3	Administrative Unit Group Object Classes

augBidirectional MANAGED OBJECT CLASS

DERIVED FROM		indirectAdaptorBidirectional,

augSink,

augSource;

REGISTERED AS { g774ObjectClass 7 };

augSink MANAGED OBJECT CLASS

DERIVED FROM		indirectAdaptorSink;

CHARACTERIZED BY

augSinkPkg PACKAGE

BEHAVIOUR

augSinkPkgBehaviour BEHAVIOUR

DEFINED AS

*This object class is instantiated if AU-n Connection(s) are being terminated.

An AUG consists of a homogeneous, byte interleaved, assembly of either three AU-3s or one AU-4.

This object class represents the point at which the AU-3/4 pointer is derived, based on the phase of the VC-3/4 POH relative to the STM-N SOH. Also, the STM-N payload is byte-demultiplexed into its component AU Groups (AUGs).*

;;

ATTRIBUTES

augId										GET,

"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 8 };

augSource MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorSource;

CHARACTERIZED BY

augSourcePkg PACKAGE

BEHAVIOUR

augSourcePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class is instantiated if AU-n Connection(s) are being originated.

An AUG consists of a homogeneous, byte interleaved, assembly of either three AU-3s or one AU-4.

This object class represents the point at which the AU-3/4 pointer is generated to indicate the phase of the VC-3/4 POH relative to the STM-N SOH. Also, the AU Groups (AUGs) are byte-multiplexed to construct the complete STM-N frame.*

;;

ATTRIBUTES

augId			GET,

"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 9 };

## 6.4	Electrical SPI trail termination point object classes

electricalSPITTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointBidirectional,

electricalSPITTPSink,

electricalSPITTPSource;

REGISTERED AS { g774ObjectClass 10 };

electricalSPITTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

electricalSPIPackage,

electricalSPITTPSinkPkg PACKAGE

BEHAVIOUR

electricalSPITTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the point where the incoming electrical interface signal is converted into an internal logic level and the timing is recovered from the line signal.

A communicationsAlarm notification shall be issued if a loss of signal is detected.

The probableCause parameter of the notification shall indicate LOS (Loss Of signal).

The upstream connectivity pointer is NULL for an instance of this class when the upstream

termination point is not contained within the same network element.

*

;;;;

REGISTERED AS { g774ObjectClass 11 };

electricalSPITTPSource MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

electricalSPIPackage,

electricalSPITTPSourcePkg PACKAGE

BEHAVIOUR

electricalSPITTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the point at which an outgoing internal logic level STM-N signal is converted into a STM-N in station electrical interface signal.

The downstream connectivity pointer is NULL for an instance of this class when the downstream termination point is not contained within the same network element.

*

;;;;

REGISTERED AS { g774ObjectClass 12 };

## 6.5	Indirect Adaptor Object Classes

indirectAdaptorBidirectional MANAGED OBJECT CLASS

DERIVED FROM		indirectAdaptorSink,

indirectAdaptorSource;

REGISTERED AS { g774ObjectClass 13 };

indirectAdaptorSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation X.721":top;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

indirectAdaptorSinkPkg PACKAGE

BEHAVIOUR

indirectAdaptorSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class provides a naming mechanism which describes the multiplexing hierarchy of the SDH signal.*

;;;;

REGISTERED AS { g774ObjectClass 14 };

indirectAdaptorSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation X.721":top;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

indirectAdaptorSourcePkg PACKAGE

BEHAVIOUR

indirectAdaptorSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class provides a naming mechanism which describes the multiplexing hierarchy of the SDH signal.*

;;;;

REGISTERED AS { g774ObjectClass 15 };

## 6.6	Multiplex Section Connection Termination Point Object Classes

msCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointBidirectional,

msCTPSink,

msCTPSource;

REGISTERED AS { g774ObjectClass 16 };

msCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msCTPPackage,

msCTPSinkPkg PACKAGE

BEHAVIOUR

msCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates a multiplex section connection.*

;;;;

REGISTERED AS { g774ObjectClass 17 };

msCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msCTPPackage,

msCTPSourcePkg PACKAGE

BEHAVIOUR

msCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates a multiplex section connection.*

;;;;

REGISTERED AS { g774ObjectClass 18 };

## 6.7	Multiplex Section Data Communications Channel Object Classes

msDatacomCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": connectionTerminationPointBidirectional,

msDatacomCTPSink,

msDatacomCTPSource;

REGISTERED AS { g774ObjectClass 19 };

msDatacomCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msDatacomCTPSinkPkg PACKAGE

BEHAVIOUR

msDatacomCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates the D4-D12 bytes in the MSOH.*

;;

ATTRIBUTES

msDatacomCTPId			GET;

;;

REGISTERED AS { g774ObjectClass 20 };

msDatacomCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msDatacomCTPSourcePkg PACKAGE

BEHAVIOUR

msDatacomCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates the D4-D12 bytes in the MSOH.*

;;

ATTRIBUTES

msDatacomCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 21 };

## 6.8	Multiplex Section Orderwire Object Classes

msOrderwireCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": connectionTerminationPointBidirectional,

msOrderwireCTPSink,

msOrderwireCTPSource;

REGISTERED AS { g774ObjectClass 22 };

msOrderwireCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msOrderwireCTPSinkPkg PACKAGE

BEHAVIOUR

msOrderwireCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents a termination point where the E2 byte orderwire channel is terminated.*

;;

ATTRIBUTES

msOrderwireCTPId	GET;

;;

REGISTERED AS { g774ObjectClass 23 };

msOrderwireCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

msOrderwireCTPSourcePkg PACKAGE

BEHAVIOUR

msOrderwireCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents a termination point where the E2 byte orderwire channel is originated.*

;;

ATTRIBUTES

msOrderwireCTPId		GET;

;;

REGISTERED AS { g774ObjectClass 24 };

## 6.9	Multiplex Section Trail Termination Point Object Classes

msTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": trailTerminationPointBidirectional,

msTTPSink,

msTTPSource;

CHARACTERIZED BY

msTTPBidirectionalPkg PACKAGE

BEHAVIOUR

msTTPBidirectionalBehaviourPkg BEHAVIOUR

DEFINED AS

*When the excessiveBERMtceInhibit attribute is set to TRUE, MS-FERF is not inserted upstream upon detection of excessive BER.

A communicationsAlarm notification shall be issued if a far end receive failure is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure).*

;;;;

REGISTERED AS { g774ObjectClass 25 };

msTTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

msTTPPackage,

msTTPSinkPkg PACKAGE

BEHAVIOUR

msTTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates a multiplex section trail, i.e. the processing and removal of the multiplex section overhead from the incoming signal.

When the excessiveBERMtceInhibit attribute is set to TRUE, AIS is not inserted downstream upon detection of excessive BER.

A communicationsAlarm notification shall be issued if an excessive bit error rate is detected. The probableCause parameter of the notification shall indicate excessive BER.

A communicationsAlarm notification shall be issued if a degraded signal is detected. The probableCause parameter of the notification shall indicate signal degrade.

A communicationsAlarm notification shall be issued if an MS alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).*

;;

ATTRIBUTES

excessiveBERMtceInhibit		GET-REPLACE,

signalDegradeThreshold		GET-REPLACE;

;;

REGISTERED AS { g774ObjectClass 26 };

msTTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

msTTPPackage,

msTTPSourcePkg PACKAGE

BEHAVIOUR

msTTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates a multiplex section trail, i.e. the generation and addition of the multiplex section overhead to the outgoing signal.*

;;;;

REGISTERED AS { g774ObjectClass 27 };

## 6.10	Optical SDH Physical Interface Trail Termination Point Object Classes

opticalSPITTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointBidirectional,

opticalSPITTPSink,

opticalSPITTPSource;

REGISTERED AS { g774ObjectClass 28 };

opticalSPITTPSink MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

opticalSPIPackage,

opticalSPITTPSinkPkg PACKAGE

BEHAVIOUR

opticalSPITTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the point where the incoming optical interface signal is converted into an internal logic level and the timing is recovered from the line signal.

A communicationsAlarm notification shall be issued if a loss of signal is detected.

The probableCause parameter of the notification shall indicate LOS (Loss Of signal).

The upstream connectivity pointer is NULL for an instance of this class when the upstream termination point is not contained within the same network element.*

;;;;

REGISTERED AS { g774ObjectClass 29 };

opticalSPITTPSource MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

opticalSPIPackage,

opticalSPITTPSourcePkg PACKAGE

BEHAVIOUR

opticalSPITTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the point at which an outgoing internal logic level STM-N signal is converted into a STM-N in-station or inter-station optical interface signal.

A communicationsAlarm notification shall be issued if the transmit laser fails.

The probableCause parameter of the notification shall indicate TransmitFail.

The downstream connectivity pointer is NULL for an instance of this class when the downstream termination point is not contained within the same network element.*

;;;;

REGISTERED AS { g774ObjectClass 30 };

## 6.11	Regenerator Section Connection Termination Point Object Classes

rsCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": connectionTerminationPointBidirectional,

rsCTPSink,

rsCTPSource;

REGISTERED AS { g774ObjectClass 31 };

rsCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsCTPPackage,

rsCTPSinkPkg PACKAGE

BEHAVIOUR

rsCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates an regenerator section connection.*

;;;;

REGISTERED AS { g774ObjectClass 32 };

rsCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsCTPPackage,

rsCTPSourcePkg PACKAGE

BEHAVIOUR

rsCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates a regenerator section connection.*

;;;;

REGISTERED AS { g774ObjectClass 33 };

## 6.12	Regenerator Section Data Communications Channel Object Classes

rsDatacomCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": connectionTerminationPointBidirectional,

rsDatacomCTPSink,

rsDatacomCTPSource;

REGISTERED AS { g774ObjectClass 34 };

rsDatacomCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsDatacomCTPSinkPkg PACKAGE

BEHAVIOUR

rsDatacomCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates the D1-D3 bytes in the RSOH.*

;;

ATTRIBUTES

rsDatacomCTPId			GET;

;;

REGISTERED AS { g774ObjectClass 35 };

rsDatacomCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsDatacomCTPSourcePkg PACKAGE

BEHAVIOUR

rsDatacomCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates the D1-D3 bytes in the RSOH.*

;;

ATTRIBUTES

rsDatacomCTPId			GET;

;;

REGISTERED AS { g774ObjectClass 36 };

## 6.13	Regenerator Section Orderwire Object Classes

rsOrderwireCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": connectionTerminationPointBidirectional,

rsOrderwireCTPSink,

rsOrderwireCTPSource;

REGISTERED AS { g774ObjectClass 37 };

rsOrderwireCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsOrderwireCTPSinkPkg PACKAGE

BEHAVIOUR

rsOrderwireCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the point where the E1 byte orderwire channel is terminated.*

;;

ATTRIBUTES

rsOrderwireCTPId			GET;

;;

REGISTERED AS { g774ObjectClass 38 };

rsOrderwireCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsOrderwireCTPSourcePkg PACKAGE

BEHAVIOUR

rsOrderwireCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the point where the E1 byte orderwire channel is originated.*

;;

ATTRIBUTES

rsOrderwireCTPId			GET;

;;

REGISTERED AS { g774ObjectClass 39 };

## 6.14	Regenerator Section Trail Termination Point Object Classes

rsTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": trailTerminationPointBidirectional,

rsTTPSink,

rsTTPSource;

REGISTERED AS { g774ObjectClass 40 };

rsTTPTrailTraceBidirectional MANAGED OBJECT CLASS

DERIVED FROM		rsTTPBidirectional, -- This derivation is necessary for name binding purposes
rsTTPTrailTraceSink, 
rsTTPTrailTraceSource;

REGISTERED AS { g774ObjectClass 107 };

rsTTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

rsTTPPackage,

rsTTPSinkPkg PACKAGE

BEHAVIOUR

rsTTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the termination of the regenerator section trail, i.e. the processing and removal of the regenerator section overhead from the incoming signal and the descrambling of that signal.

A communicationsAlarm notification shall be issued if a loss of frame is detected. The probableCause parameter of the notification shall indicate LOF (Loss of Frame).*

;;;;

REGISTERED AS { g774ObjectClass 41 };

rsTTPTrailTraceSink MANAGED OBJECT CLASS

DERIVED FROM		rsTTPSink;

CHARACTERIZED BY

trailTraceSinkPackage,

rsTTPTrailTraceSinkPkg PACKAGE

BEHAVIOUR

rsTTPTrailTraceSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the termination of the regenerator section trail and supports trail trace management.*

;;;;

REGISTERED AS { g774ObjectClass 108 };

rsTTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

rsTTPPackage,

rsTTPSourcePkg PACKAGE

BEHAVIOUR

rsTTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the origination of the regenerator section trail, i.e. generates the regenerator section overhead for the outgoing signal, and scrambles that signal.*

;;;;

REGISTERED AS { g774ObjectClass 42 };

rsTTPTrailTraceSource MANAGED OBJECT CLASS

DERIVED FROM		rsTTPSource;

CHARACTERIZED BY

trailTraceSourcePackage,

rsTTPTrailTraceSourcePkg PACKAGE

BEHAVIOUR

rsTTPTrailTraceSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents the origination of the regenerator section trail and supports trail trace management.*

;;;;

REGISTERED AS { g774ObjectClass 109 };

## 6.15	Regenerator Section User Channel Object Classes

rsUserChannelCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": connectionTerminationPointBidirectional,

rsUserChannelCTPSink,

rsUserChannelCTPSource;

REGISTERED AS { g774ObjectClass 43 };

rsUserChannelCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsUserChannelCTPSinkPkg PACKAGE

BEHAVIOUR

rsUserChannelCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents a class of objects that terminates the F1 byte user channel.*

;;

ATTRIBUTES

rsUserChannelCTPId		GET;

;;

REGISTERED AS { g774ObjectClass 44 };

rsUserChannelCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

rsUserChannelCTPSourcePkg PACKAGE

BEHAVIOUR

rsUserChannelCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class represents a class of objects that originates the F1 byte user channel.*

;;

ATTRIBUTES

rsUserChannelCTPId		GET;

;;

REGISTERED AS { g774ObjectClass 45 };

## 6.16	SDH Network Element Object Class

sdhNE MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":managedElement;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

sdhNEPackage PACKAGE

BEHAVIOUR

sdhNEBehaviour BEHAVIOUR

DEFINED AS

*An SDH network element is a system which has no internal open-system interfaces.*

;;

;;

REGISTERED AS { g774ObjectClass 46 };

## 6.17	Tributary Unit 11 Object Classes

tu11CTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointBidirectional,

tu11CTPSinkR1,

tu11CTPSource;

REGISTERED AS { g774ObjectClass 87 };

tu11CTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu11CTPSinkR1Pkg PACKAGE

BEHAVIOUR

tu11CTPSinkR1BehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates a tu-11 connection.

A change in the operational state shall cause a state change notification *

;;

ATTRIBUTES

tu11CTPId	GET;

;;

REGISTERED AS { g774ObjectClass 88 };

tu11CTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu11CTPSourcePkg PACKAGE

BEHAVIOUR

tu11CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates a tu-11 connection.*

;;

ATTRIBUTES

tu11CTPId				GET,

pointerSourceType		GET;

;;

REGISTERED AS { g774ObjectClass 49 };

## 6.18	Tributary Unit 12 Object Classes

tu12CTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointBidirectional,

tu12CTPSinkR1,

tu12CTPSource;

REGISTERED AS { g774ObjectClass 89 };

tu12CTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu12CTPSinkR1Pkg PACKAGE

BEHAVIOUR

tu12CTPSinkR1BehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates a tu-12 connection.

A change in the operational state shall cause a state change notification *

;;

ATTRIBUTES

tu12CTPId GET;

;;

REGISTERED AS { g774ObjectClass 90 };

tu12CTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu12CTPSourcePkg PACKAGE

BEHAVIOUR

tu12CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates a tu-12 connection.*

;;

ATTRIBUTES

tu12CTPId			GET,

pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 52 };

## 6.19	Tributary Unit 2 Object Classes

tu2CTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointBidirectional,

tu2CTPSinkR1,

tu2CTPSource;

REGISTERED AS { g774ObjectClass 91 };

tu2CTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu2CTPSinkR1Pkg PACKAGE

BEHAVIOUR

tu2CTPSinkR1BehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates a tu-2 connection.

A change in the operational state shall cause a state change notification.*

;;

ATTRIBUTES

tu2CTPId		GET;

;;

REGISTERED AS { g774ObjectClass 92 };

tu2CTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu2CTPSourcePkg PACKAGE

BEHAVIOUR

tu2CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates a tu-2 connection.*

;;

ATTRIBUTES

tu2CTPId			GET,

pointerSourceType	GET;

;;

REGISTERED AS { g774ObjectClass 55 };

## 6.20	Tributary Unit 3 Object Classes

tu3CTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointBidirectional,

tu3CTPSinkR1,

tu3CTPSource;

REGISTERED AS { g774ObjectClass 93 };

tu3CTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":operationalStatePackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

tu-nSinkPackage,

tu3CTPSinkR1Pkg PACKAGE

BEHAVIOUR

tu3CTPSinkR1BehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates a tu-3 connection.

A change in the operational state shall cause a state change notification.*

;;

ATTRIBUTES

tu3CTPId			GET;

;;

REGISTERED AS { g774ObjectClass 94 };

tu3CTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

tu3CTPSourcePkg PACKAGE

BEHAVIOUR

tu3CTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates a tu-3 connection.*

;;

ATTRIBUTES

tu3CTPId				GET,

pointerSourceType		GET;

;;

REGISTERED AS { g774ObjectClass 58 };

## 6.21	Tributary Unit Group 2 Object Classes

tug2Bidirectional MANAGED OBJECT CLASS

DERIVED FROM	indirectAdaptorBidirectional,

tug2Sink,

tug2Source;

REGISTERED AS { g774ObjectClass 59 };

tug2Sink MANAGED OBJECT CLASS

DERIVED FROM		indirectAdaptorSink;

CHARACTERIZED BY

tug2SinkPkg PACKAGE

BEHAVIOUR

tug2SinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class is instantiated if TU-11, TU-12, TU-2 connection(s) are being terminated or originated.

A TUG-2 consists of a homogeneous or heterogeneous, assembly of four TU-11s, three TU-12s or one TU-2.

This object class represents the point at which the TU pointer is derived, based on the phase of the VC 11/12/2 POH relative to the VC-3/4 POH and the VC-3/4 signal is disassembled.*

;;

ATTRIBUTES

tug2Id			GET,

"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 60 };

tug2Source MANAGED OBJECT CLASS

DERIVED FROM	 	indirectAdaptorSource;

CHARACTERIZED BY

tug2SourcePkg PACKAGE

BEHAVIOUR

tug2SourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class is instantiated if TU-11, TU-12, TU-2 connection(s) are being terminated or originated.

A TUG-2 consists of a homogeneous or heterogeneous, assembly of four TU-11s, three TU-12s or one TU-2.

This object class represents the point at which the TU pointer is processed to indicate the phase of the VC 11/12/2 POH relative to the VC-3/4 POH and assembles the complete VC-3/4.*

;;

ATTRIBUTES

tug2Id			GET,

"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 61 };

## 6.22	Tributary Unit Group 3 Object Classes

tug3Bidirectional MANAGED OBJECT CLASS

DERIVED FROM		indirectAdaptorBidirectional,

tug3Sink,

tug3Source;

REGISTERED AS { g774ObjectClass 62 };

tug3Sink MANAGED OBJECT CLASS

DERIVED FROM		indirectAdaptorSink;

CHARACTERIZED BY

tug3SinkPkg PACKAGE

BEHAVIOUR

tug3SinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class is instantiated if TU-3 connection(s) are being terminated. Or if TU-11s, TU-12s, or TU-2s are being terminated from a VC-4.

A TUG-3 consists of a homogeneous assembly of seven TUG-2s or one TU-3.*

;;

ATTRIBUTES

tug3Id			GET,

"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 63 };

tug3Source MANAGED OBJECT CLASS

DERIVED FROM		indirectAdaptorSource;

CHARACTERIZED BY

tug3SourcePkg PACKAGE

BEHAVIOUR

tug3SourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class is instantiated if TU-3 connection(s) are being originated. Or TU-11s, TU-12s, or TU-2s are being combined to form a VC-4.

A TUG-3 consists of a homogeneous assembly of seven TUG-2s or one TU-3.*

;;

ATTRIBUTES

tug3Id			GET,

"Recommendation M.3100":supportableClientList	GET;

;;

REGISTERED AS { g774ObjectClass 64 };

## 6.23	Virtual Container 11 Object Classes

vc11TTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointBidirectional,

vc11TTPSinkR1,

vc11TTPSource;

CHARACTERIZED BY

vc11-2BidirectionalPackageR1;

REGISTERED AS { g774ObjectClass 95 };

vc11TTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc11-2SinkPackageR1,

vc11TTPSinkPkgR1 PACKAGE

BEHAVIOUR

vc11TTPSinkPkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class terminates a vc11 trail, i.e. the point at which the VC11-POH is extracted from the STM-N frame.*

;;

ATTRIBUTES

vc11TTPId			GET;

;;

REGISTERED AS { g774ObjectClass 96 };

vc11TTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc11TTPSourcePkg PACKAGE

BEHAVIOUR

vc11TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class originates a vc11 trail, i.e. the point at which the VC11-POH is added to the STM-N frame.*

;;

ATTRIBUTES

vc11TTPId				GET,

v5SignalLabelSend		GET;

;;

REGISTERED AS { g774ObjectClass 67 };

## 6.24	Virtual Container 12 Object Classes

vc12TTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointBidirectional,

vc12TTPSinkR1,

vc12TTPSource;

CHARACTERIZED BY

vc11-2BidirectionalPackageR1;

REGISTERED AS { g774ObjectClass 97 };

vc12TTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc11-2SinkPackageR1,

vc12TTPSinkPkgR1 PACKAGE

BEHAVIOUR

vc12TTPSinkPkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class terminates a vc12 trail, i.e. the point at which the VC12-POH is extracted from the STM-N frame.*

;;

ATTRIBUTES

vc12TTPId			GET;

;;

REGISTERED AS { g774ObjectClass 98 };

vc12TTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc12TTPSourcePkg PACKAGE

BEHAVIOUR

vc12TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class originates a vc12 trail, i.e. the point at which the VC12-POH is added to the STM-N frame.*

;;

ATTRIBUTES

vc12TTPId				GET,

v5SignalLabelSend		GET;

;;

REGISTERED AS { g774ObjectClass 70 };

## 6.25	Virtual Container 2 Object Classes

vc2TTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointBidirectional,

vc2TTPSinkR1,

vc2TTPSource;

CHARACTERIZED BY

vc11-2BidirectionalPackageR1;

REGISTERED AS { g774ObjectClass 99 };

vc2TTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY
	"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc11-2SinkPackageR1,

vc2TTPSinkPkgR1 PACKAGE

BEHAVIOUR

vc2TTPSinkPkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class terminates a vc2 trail, i.e. the point at which

the VC2-POH is extracted from the STM-N frame.*

;;

ATTRIBUTES

vc2TTPId			GET;

;;

REGISTERED AS { g774ObjectClass 100 };

vc2TTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc2TTPSourcePkg PACKAGE

BEHAVIOUR

vc2TTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class originates a vc2 trail, i.e. the point at which the VC2-POH is added to the STM-N frame.*

;;

ATTRIBUTES

vc2TTPId				GET,

v5SignalLabelSend		GET;

;;

REGISTERED AS { g774ObjectClass 73 };

## 6.26	Virtual Container 3 Object Classes

vc3TTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointBidirectional,

vc3TTPSinkR1,

vc3TTPSourceR1;

CHARACTERIZED BY

vc3-4BidirectionalPackageR1;

REGISTERED AS { g774ObjectClass 101 };

vc3TTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc3-4SinkPackageR1,

vc3TTPSinkPkgR1 PACKAGE

BEHAVIOUR

vc3TTPSinkPkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class terminates a vc3 trail, i.e. the point at which

the SDH VC-3 is terminated.*

;;

ATTRIBUTES

vc3TTPId			GET;

;;

REGISTERED AS { g774ObjectClass 102 };

vc3TTPSourceR1 MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc3-4SourcePackageR1,

vc3TTPSourcePkgR1 PACKAGE

BEHAVIOUR

vc3TTPSourcePkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class originates a vc3 trail, i.e. the point at which

the SDH VC-3 is originated.*

;;

ATTRIBUTES

vc3TTPId			GET;

;;

REGISTERED AS { g774ObjectClass 103 };

## 6.27	Virtual Container 4 Object Classes

vc4TTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation M.3100":trailTerminationPointBidirectional,

vc4TTPSinkR1,

vc4TTPSourceR1;
	CHARACTERIZED BY

vc3-4BidirectionalPackageR1;

REGISTERED AS { g774ObjectClass 104 };

vc4TTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM "Recommendation M.3100":trailTerminationPointSink;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

"Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

vc3-4SinkPackageR1,

vc4TTPSinkPkgR1 PACKAGE

BEHAVIOUR

vc4TTPSinkPkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class terminates a vc4 trail, i.e. the point at which

the SDH VC-4 is terminated.*

;;

ATTRIBUTES

vc4TTPId			GET;

;;

REGISTERED AS { g774ObjectClass 105 };

vc4TTPSourceR1 MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":trailTerminationPointSource;

CHARACTERIZED BY

"Recommendation X.721":administrativeStatePackage,

"Recommendation M.3100":createDeleteNotificationsPackage,

"Recommendation M.3100":stateChangeNotificationPackage,

vc3-4SourcePackageR1,

vc4TTPSourcePkgR1 PACKAGE

BEHAVIOUR

vc3-4TTPSourcePkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class originates a vc4 trail, i.e. the point at which

the SDH VC-4 is originated.*

;;

ATTRIBUTES

vc4TTPId			GET;

;;

REGISTERED AS { g774ObjectClass 106 };

## 6.28	VC-n User Channel Object Classes

vcnUserChannelCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100": connectionTerminationPointBidirectional,

vcnUserChannelCTPSink,

vcnUserChannelCTPSource;

REGISTERED AS { g774ObjectClass 80 };

vcnUserChannelCTPSink MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSink;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

vcnUserChannelCTPSinkPkg PACKAGE

BEHAVIOUR

vcnUserChannelCTPSinkBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class terminates the F2 byte user channel.*

;;

ATTRIBUTES

vcnUserChannelCTPId		GET;

;;

REGISTERED AS { g774ObjectClass 81 };

vcnUserChannelCTPSource MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation M.3100":connectionTerminationPointSource;

CHARACTERIZED BY

"Recommendation M.3100":createDeleteNotificationsPackage,

vcnUserChannelCTPSourcePkg PACKAGE

BEHAVIOUR

vcnUserChannelCTPSourceBehaviourPkg BEHAVIOUR

DEFINED AS

*This object class originates the F2 byte user channel.*

;;

ATTRIBUTES

vcnUserChannelCTPId		GET;

;;

REGISTERED AS { g774ObjectClass 82 };

# 7	Packages

This clause provides replacement package definitions for the existing Recommendation G.774 (1992). Any package replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a package are as follows:

1)	The replaced package is faulty and must be fixed.

2)	The replaced package includes an attribute, package, notification or action which has been re-registered in this Recommendation.

In each case where a package is replaced, the new package will be registered within this Recommendation. The textual label for the package will be revised to include the text "R1". For example, in the revision of the G.774 (1992) package "vc3-4SourcePackage", the revised label will become "vc3-4SourcePackageR1".

Below is a table of packages deprecated from Recommendation G.774 (1992) and the G.774 packages which replace them:

Deprecated G.774 (1992) Packages	Replacement G.774 Packages

vc11-2BidirectionalPackage		vc11-2BidirectionalPackageR1

vc11-2SinkPackage				vc11-2SinkPackageR1

vc3-4BidirectionalPackage		vc3-4BidirectionalPackageR1

vc3-4SinkPackage				vc3-4SinkPackageR1

vc3-4SourcePackage				vc3-4SourcePackageR1

New Packages to support trail trace management at regenerator section level:

trailTraceSinkPackage

trailTraceSourcePackage

## 7.1	electricalSPIPackage

electricalSPIPackage PACKAGE

ATTRIBUTES

electricalSPITTPId	GET,

stmLevel			GET;

;

## 7.2	msCTPPackage

msCTPPackage PACKAGE

ATTRIBUTES

msCTPId			GET,

stmLevel			GET;

;

## 7.3	msTTPPackage

msTTPPackage PACKAGE

ATTRIBUTES

msTTPId			GET,

stmLevel			GET;

;

## 7.4	opticalSPIPackage

opticalSPIPackage PACKAGE

ATTRIBUTES

opticalSPITTPId	GET,

opticalReach		GET,

opticalWavelength	GET,

stmLevel			GET;

;

## 7.5	rsCTPPackage

rsCTPPackage PACKAGE

ATTRIBUTES

rsCTPId			GET,

stmLevel			GET;

;

## 7.6	rsTTPPackage

rsTTPPackage PACKAGE

ATTRIBUTES

rsTTPId			GET,

stmLevel			GET;

;

## 7.7	trailTraceSinkPackage

trailTraceSinkPackage PACKAGE

BEHAVIOUR

trailTraceSinkPackageBehaviour BEHAVIOUR

DEFINED AS

*This Package supports trail trace management at the termination sink.*

;;

ATTRIBUTES

trailTraceExpected

REPLACE-WITH-DEFAULT

DEFAULT VALUE SDH.defaultNull

GET-REPLACE,

trailTraceReceive		GET;

;

## 7.8	trailTraceSourcePackage

trailTraceSourcePackage PACKAGE

BEHAVIOUR

trailTraceSourcePackageBehaviour BEHAVIOUR

DEFINED AS

*This Package supports trail trace management at the termination source.*

;;

ATTRIBUTES

trailTraceSend			GET-REPLACE;

;

## 7.9	tu-nSinkPackage

tu-nSinkPackage PACKAGE

BEHAVIOUR

tu-nSinkPackageBehaviour BEHAVIOUR

DEFINED AS

*A communicationsAlarm notification shall be issued if a loss of TU pointer is detected. The probableCause parameter of the notification shall indicate LOP (Loss of Pointer).

A communicationsAlarm notification shall be issued if an TU path alarm indication signal is detected. The probableCause parameter of the notification shall indicate AIS (Alarm Indication Signal).*

;;

ATTRIBUTES

pointerSinkType	GET;

;

## 7.10	vc11-2BidirectionalPackageR1

vc11-2BidirectionalPackageR1 PACKAGE

BEHAVIOUR

vc11-2BidirectionalPackageR1Behaviour BEHAVIOUR

DEFINED AS

*A communicationsAlarm notification shall be issued if a far end receive failure (V5 byte) is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure).*

;;;

## 7.11	vc11-2SinkPackageR1

vc11-2SinkPackageR1 PACKAGE

BEHAVIOUR

vc11-2SinkPackageR1Behaviour BEHAVIOUR

DEFINED AS

*A communicationsAlarm notification shall be issued if the signal label received (V5 Byte) does not match the signal label expected. The probableCause parameter of the notification shall indicate signal label mismatch.*

;;

ATTRIBUTES

v5SignalLabelExpected		GET,

v5SignalLabelReceive		GET;

;

## 7.12	vc3-4BidirectionalPackageR1

vc3-4BidirectionalPackageR1 PACKAGE

BEHAVIOUR

vc3-4BidirectionalPackageR1Behaviour BEHAVIOUR

DEFINED AS

*A communicationsAlarm notification shall be issued if a far end receive failure (G1 byte) is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure).*

;;;

## 7.13	vc3-4SinkPackageR1

vc3-4SinkPackageR1 PACKAGE

BEHAVIOUR

vc3-4SinkPackageR1Behaviour BEHAVIOUR

DEFINED AS

*A communicationsAlarm notification shall be issued if the signal label received (C2 Byte) does not match the signal label expected. The probableCause parameter of the notification shall indicate signal label mismatch.

A communicationsAlarm notification shall be issued if the path trace received (J1 Byte) does not match the path trace expected. The probableCause parameter of the notification shall indicate path trace mismatch.

A communicationsAlarm notification shall be issued if a loss of TU multiframe indicator (H4 Byte) is detected. The probableCause parameter of the notification shall indicate loss of TU multiframe.

This communicationsAlarm notification is only required for high order paths with payloads that require use of the multiframe indicator.

When 16 bytes are supported, the 16 bytes of the path trace shall be conveyed at the management interface in both ways. This is a local issue whether the NE recompute the CRC-7 under a replace operation.*

;;

ATTRIBUTES

j1PathTraceExpected

REPLACE-WITH-DEFAULT

DEFAULT VALUE SDH.defaultNull

GET-REPLACE,

"Recommendation G.774.05": j1PathTraceReceive		GET,

c2SignalLabelExpected		GET,

c2SignalLabelReceive		GET;

;

## 7.14	vc3-4SourcePackageR1

vc3-4SourcePackageR1 PACKAGE

BEHAVIOUR

vc3-4SourcePackageR1Behaviour BEHAVIOUR

DEFINED AS

*When 16 bytes are supported, the 16 bytes of the path trace shall be conveyed at the management interface.*

;;

ATTRIBUTES

"Recommendation G.774.5": j1PathTraceSend	GET-REPLACE,

c2SignalLabelSend	GET;

;

# 8	Attributes

This clause provides replacement attribute definitions for the existing Recommendation G.774 (1992). Any attribute replaced by one in this clause is considered to be deprecated. The reasons for the replacement of an attribute are as follows:

1)	The replaced attribute is faulty and must be fixed.

Below is a table of attributes deprecated from Recommendation G.774 (1992):

Deprecated G.774 (1992) Attributes

ferfState

New Attributes to support trail trace management at regenerator section level:

trailTraceExpected

trailTraceReceive

trailTraceSend

## 8.1	AU-3 Identification

au3CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR

au3CTPIdBehaviour BEHAVIOUR

DEFINED AS

*The au3CTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the AU3CTPBidirectional, AU3CTPSink, and AU3CTPSource managed object classes. This attribute specifies the timeslot of the au3CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 1 };

## 8.2	AU-4 Identification

au4CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR

au4CTPIdBehaviour BEHAVIOUR

DEFINED AS

*The au4CTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the AU4CTPBidirectional, AU4CTPSink, and AU4CTPSource managed object classes. This attribute specifies the timeslot of the au4CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 2 };

## 8.3	AUG Identification

augId ATTRIBUTE

WITH ATTRIBUTE SYNTAX			SDH.NameType;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR

augIdBehaviour BEHAVIOUR

DEFINED AS

*The augId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the AUG managed object class. This attribute specifies the timeslot of the aug within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 3 };

## 8.4	C2 Signal Label Expected

c2SignalLabelExpected ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.C2SignalLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR

c2SignalLabelExpectedBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the expected C2 VC Signal Label for an incoming VC-n.

See Recommendation G.707/Y.1322 for a list of valid values.*

;;

REGISTERED AS { g774Attribute 4 };

## 8.5	C2 Signal Label Receive

c2SignalLabelReceive ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.C2SignalLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR

c2SignalLabelReceiveBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the C2 VC Signal Label for an incoming VC-n.

See Recommendation G.707/Y.1322 for a list of valid values.*

;;

REGISTERED AS { g774Attribute 5 };

## 8.6	C2 Signal Label Send

c2SignalLabelSend ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.C2SignalLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR

c2SignalLabelSendBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the C2 VC Signal Label for an outgoing VC-n.

See Recommendation G.707/Y.1322 for a list of valid values.*

;;

REGISTERED AS { g774Attribute 6 };

## 8.7	Electrical SDH Physical Interface Trail Termination Point Identification

electricalSPITTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX 		SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

electricalSPITTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the electricalSPITTP object classes.

If the string choice of the syntax is used then matching on substrings is permitted. If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 7 };

## 8.8	Excessive Bit Error Ratio Maintenance Inhibit

excessiveBERMtceInhibit ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.Boolean;

MATCHES FOR EQUALITY;

BEHAVIOUR

excessiveBERMtceInhibitBehaviour BEHAVIOUR

DEFINED AS

*This attribute is set to TRUE to cause the inhibition of consequent maintenance signalling upon the detection of excessive bit error ratio.*

;;

REGISTERED AS { g774Attribute 8 };

## 8.9	J1 Path Trace Expected

j1PathTraceExpected ATTRIBUTE

WITH ATTRIBUTE SYNTAX 		SDH.PathTrace;

MATCHES FOR EQUALITY;

BEHAVIOUR

j1PathTraceExpectedBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used to specify the value of the expected J1 Byte VC Path Trace byte message for instances of the VC-n. If the value of this attribute is 	set to NULL then any Received Path Trace shall be considered to match.*

;;

REGISTERED AS { g774Attribute 10 };

## 8.10	J1 Path Trace Receive

j1PathTraceReceive ATTRIBUTE

WITH ATTRIBUTE SYNTAX		 SDH.PathTrace;

MATCHES FOR EQUALITY;

BEHAVIOUR

j1PathTraceReceiveBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used to indicate the value of the incoming J1 Byte VC Path Trace byte message for instances of the VC-n.*

;;

REGISTERED AS { g774Attribute 11 };

## 8.11	J1 Path Trace Send

j1PathTraceSend ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.PathTrace;

MATCHES FOR EQUALITY;

BEHAVIOUR

j1PathTraceSendBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used to indicate the value of the outgoing J1 VC Path Trace byte message for instances of the VC-n. The NULL choice is not supported.*

;;

REGISTERED AS { g774Attribute 12 };

## 8.12	Multiplex Section Connection Termination Point Identification

msCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

msCTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the msCTP object class. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 13 };

## 8.13	MS Data Communications Channel CTP Identification

msDatacomCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

msDatacomCTPIdBehaviour BEHAVIOUR

DEFINED AS

*The msDatacomCTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the msDatacomCTPBidirectional, msDatacomCTPSink, and msDatacomCTPSource managed object classes. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 14 };

## 8.14	Multiplex Section Orderwire Identification

msOrderwireCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

msOrderwireCTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the Multiplex Section Orderwire Channel object class. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 15 };

## 8.15	Multiplex Section Trail Termination Point Identification

msTTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

msTTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the msTTP object class.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 16 };

## 8.16	Optical Reach

opticalReach ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.OpticalReach;

MATCHES FOR EQUALITY;

BEHAVIOUR

opticalReachBehaviour BEHAVIOUR

DEFINED AS

*This attribute indicates the length the optical signal may travel before requiring termination or regeneration.*

;;

REGISTERED AS { g774Attribute 17 };

## 8.17	Optical SDH Physical Interface Trail Termination Point Identification

opticalSPITTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX 		SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

opticalSPITTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the opticalSPITTP object class.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 18 };

## 8.18	Optical WaveLength

opticalWavelength ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.OpticalWavelength;

MATCHES FOR EQUALITY;

BEHAVIOUR

opticalWavelengthBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the optical wavelength used by an opticalSPITTP object instance.*

;;

REGISTERED AS { g774Attribute 19 };

## 8.19	Pointer Sink Type

pointerSinkType ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.PointerSinkType;

MATCHES FOR EQUALITY;

BEHAVIOUR

pointerSinkTypeBehaviour BEHAVIOUR

DEFINED AS

*This attribute indicates the status of the incoming Pointer of a sink or bidirectional CTP.*

;;

REGISTERED AS { g774Attribute 20 };

## 8.20	Pointer Source Type

pointerSourceType ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.PointerSourceType;

MATCHES FOR EQUALITY;

BEHAVIOUR

pointerSourceTypeBehaviour BEHAVIOUR

DEFINED AS

*This attribute indicates the status of the outgoing Pointer of a source or bidirectional CTP.*

;;

REGISTERED AS { g774Attribute 21 };

## 8.21	Regenerator Section Connection Termination Point Identification

rsCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

rsCTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the rsCTP object classes. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 22 };

## 8.22	Regenerator Section Data Communications Channel CTP Identification

rsDatacomCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

rsDatacomCTPIdBehaviour BEHAVIOUR

DEFINED AS

*The rsDatacomCTPId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the rsDatacomCTPBidirectional, rsDatacomCTPSink, and rsDatacomCTPSource managed object classes. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 23 };

## 8.23	Regenerator Section Orderwire Identification

rsOrderwireCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

rsOrderwireCTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the Regenerator Section Orderwire Channel object classes. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 24 };

## 8.24	Regenerator Section Trail Termination Point Identification

rsTTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

rsTTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the rsTTP object classes.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 25 };

## 8.25	Regenerator Section User Channel Identification

rsUserChannelCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

rsUserChannelCTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the rsUserChannel object classes. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 26 };

## 8.26	Signal Degrade Threshold

signalDegradeThreshold ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.Integer;

MATCHES FOR EQUALITY;

BEHAVIOUR

signalDegradeThresholdBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the specific BER used to define the signal degrade threshold. The specific BER used is an equipment issue. This attribute represents the negative power of 10. So, for instance, if this attribute had a value of 5, then the BER threshold is 10 raised to the power of –5.*

;;

REGISTERED AS { g774Attribute 27 };

## 8.27	STM Level

stmLevel ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.Integer;

MATCHES FOR EQUALITY;

BEHAVIOUR

stmLevelBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the level, n, of the STM-n signal being received, transmitted, or received and transmitted for termination point object instance.*

;;

REGISTERED AS { g774Attribute 28 };

## 8.28	Trail Trace Expected

trailTraceExpected ATTRIBUTE

WITH ATTRIBUTE SYNTAX 		SDH.PathTrace;

MATCHES FOR EQUALITY;

BEHAVIOUR

trailTraceExpectedBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used to specify the value of the expected Trail Trace information.

If the value of this attribute is set to NULL then any Trail Trace shall be considered to match.*

;;

REGISTERED AS { g774Attribute 44 };

## 8.29	Trail Trace Receive

trailTraceReceive ATTRIBUTE

WITH ATTRIBUTE SYNTAX 		SDH.TrailTraceReceiveSend;

MATCHES FOR EQUALITY;

BEHAVIOUR

trailTraceReceiveBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used to indicate the value of the incoming Trail Trace information.*

;;

REGISTERED AS { g774Attribute 45 };

## 8.30	Trail Trace Send

trailTraceSend ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.TrailTraceReceiveSend;

MATCHES FOR EQUALITY;

BEHAVIOUR

trailTraceSendBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used to indicate the value of the outgoing Trail Trace information.*

;;

REGISTERED AS {g774Attribute 46};

## 8.31	Tributary Unit 11 Connection Termination Point Identification

tu11CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR

tu11CTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the tu11CTP object classes. This attribute specifies the timeslot of the TU-11 CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 29 };

## 8.32	Tributary Unit 12 Connection Termination Point Identification

tu12CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR

tu12CTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the tu12CTP object classes. This attribute specifies the timeslot of the TU-12 CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 30 };

## 8.33	Tributary Unit 2 Connection Termination Point Identification

tu2CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR

tu2CTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the tu2CTP object classes. This attribute specifies the timeslot of the TU-2 CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 31 };

## 8.34	Tributary Unit 3 Connection Termination Point Identification

tu3CTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR

tu3CTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the tu3CTP object classes. This attribute specifies the timeslot of the TU-3 CTP within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 32 };

## 8.35	TUG-2 Identification

tug2Id ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR	EQUALITY, ORDERING;

BEHAVIOUR

tug2IdBehaviour BEHAVIOUR

DEFINED AS

*The tug2Id attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the TUG-2 managed object class. This attribute specifies the timeslot of the TUG2 within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 33 };

## 8.36	TUG-3 Identification

tug3Id ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.NameType;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR

tug3IdBehaviour BEHAVIOUR

DEFINED AS

*The tug3Id attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the TUG-3 managed object class. This attribute specifies the timeslot of the TUG3 within its server TTP or IA. The value shall be the integer which represents the position of the timeslot in temporal order. The first timeslot shall be numbered one.*

;;

REGISTERED AS { g774Attribute 34 };

## 8.37	V5 Signal Label Expected

v5SignalLabelExpected ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.V5SignalLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR

v5SignalLabelExpectedBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the expected V5 VC Signal Label for an incoming VC-n.

See Recommendation G.707/Y.1322 for a list of valid values.*

;;

REGISTERED AS { g774Attribute 35 };

## 8.38	V5 Signal Label Receive

v5SignalLabelReceive ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.V5SignalLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR

v5SignalLabelReceiveBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the V5 VC Signal Label for an incoming VC-n. See Recommendation G.707/Y.1322 for a list of valid values.*

;;

REGISTERED AS { g774Attribute 36 };

## 8.39	V5 Signal Label Send

v5SignalLabelSend ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDH.V5SignalLabel;

MATCHES FOR EQUALITY;

BEHAVIOUR

v5SignalLabelSendBehaviour BEHAVIOUR

DEFINED AS

*This attribute specifies the V5 VC Signal Label for an outgoing VC-n.

See Recommendation G.707/Y.1322 for a list of valid values.*

;;

REGISTERED AS { g774Attribute 37 };

## 8.40	Virtual Container 11 Trail Termination Point Identification

vc11TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

vc11TTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the vc11TTP object classes.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 38 };

## 8.41	Virtual Container 12 Trail Termination Point Identification

vc12TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

vc12TTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the vc12TTP object classes.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 39 };

## 8.42	Virtual Container 2 Trail Termination Point Identification

vc2TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

vc2TTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the vc2TTP object classes.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 40 };

## 8.43	Virtual Container 3 Trail Termination Point Identification

vc3TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

vc3TTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the vc3TTP object classes.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 41 };

## 8.44	Virtual Container 4 Trail Termination Point Identification

vc4TTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDH.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR

vc4TTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the vc4TTP object classes.

If the string choice of the syntax is used then matching on substrings is permitted.

If the number choice for the syntax is used then matching on ordering is permitted.*

;;

REGISTERED AS { g774Attribute 42 };

## 8.45	VC-n User Channel Identification

vcnUserChannelCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDH.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR

vcnUserChannelCTPIdBehaviour BEHAVIOUR

DEFINED AS

*This attribute is used as an RDN for naming instances of the vcnUserChannel object classes. This attribute represents the first and only timeslot of this type. The value of this attribute shall be integer one.*

;;

REGISTERED AS { g774Attribute 43 };

# 9	Name Bindings

This clause provides replacement namebinding definitions for the existing Recommendation G.774 (1992). Any namebinding replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a namebinding are as follows:

1)	The replaced namebinding is faulty and must be fixed.

2)	The replaced namebinding refers to a superior managed object class which has been re-registered in this Recommendation.

3)	The replaced namebinding refers to a subordinate managed object class which has been re-registered in this Recommendation.

4)	The replaced namebinding refers to a naming attribute which has been re-registered in this Recommendation.

In each case where a namebinding is replaced, the new namebinding will be registered within this Recommendation. The textual label for the namebinding will be revised to include the text "R1". For example, in the revision of the G.774 (1992) namebinding "vcnUserChannelCTPSource-vc4TTPSource", the revised label will become "vcnUserChannelCTPSource-vc4TTPSourceR1" or in the case of the "vc4TTPSink-sdhNE" namebinding, the revised label becomes "vc4TTPSinkR1-sdhNE". Note the "R1" is placed immediately following the revised class which impacts the namebinding.

Below is a table of namebindings deprecated from Recommendation G.774 (1992) and the G.774 namebindings which replace them:

Deprecated G.774 (1992) Namebindings

vcnUserChannelCTPBidirectional-vc3TTPBidirectional

vcnUserChannelCTPSink-vc3TTPBidirectional

vcnUserChannelCTPSink-vc3TTPSink

vcnUserChannelCTPSource-vc3TTPBidirectional

vcnUserChannelCTPSource-vc3TTPSource

vcnUserChannelCTPBidirectional-vc4TTPBidirectional

vcnUserChannelCTPSink-vc4TTPBidirectional

vcnUserChannelCTPSink-vc4TTPSink

vcnUserChannelCTPSource-vc4TTPBidirectional

vcnUserChannelCTPSource-vc4TTPSource

au3CTPBidirectional-augBidirectional

au3CTPSink-augBidirectional

au3CTPSink-augSink

au4CTPBidirectional-augBidirectional

au4CTPSink-augBidirectional

au4CTPSink-augSink

tu11CTPBidirectional-tug2Bidirectional

tu11CTPSink-tug2Bidirectional

tu11CTPSink-tug2Sink

tu12CTPBidirectional-tug2Bidirectional

tu12CTPSink-tug2Bidirectional

tu12CTPSink-tug2Sink

tu2CTPBidirectional-tug2Bidirectional

tu2CTPSink-tug2Bidirectional

tu2CTPSink-tug2Sink

tu3CTPBidirectional-tug3Bidirectional

tu3CTPSink-tug3Bidirectional

tu3CTPSink-tug3Sink

tug2Bidirectional-vc3TTPBidirectional

tug2Sink-vc3TTPSink

tug2Source-vc3TTPSource

tug3Bidirectional-vc4TTPBidirectional

tug3Sink-vc4TTPSink

tug3Source-vc4TTPSource

vc11TTPBidirectional-sdhNE

vc11TTPSink-sdhNE

vc12TTPBidirectional-sdhNE

vc12TTPSink-sdhNE

vc2TTPBidirectional-sdhNE

vc2TTPSink-sdhNE

vc3TTPBidirectional-sdhNE

vc3TTPSink-sdhNE

vc3TTPSource-sdhNE

vc4TTPBidirectional-sdhNE

vc4TTPSink-sdhNE

vc4TTPSource-sdhNE

Replacement G.774 Namebindings

vcnUserChannelCTPBidirectional-vc3TTPBidirectionalR1

vcnUserChannelCTPSink-vc3TTPBidirectionalR1

vcnUserChannelCTPSink-vc3TTPSinkR1

vcnUserChannelCTPSource-vc3TTPBidirectionalR1

vcnUserChannelCTPSource-vc3TTPSourceR1

vcnUserChannelCTPBidirectional-vc4TTPBidirectionalR1

vcnUserChannelCTPSink-vc4TTPBidirectionalR1

vcnUserChannelCTPSink-vc4TTPSinkR1

vcnUserChannelCTPSource-vc4TTPBidirectionalR1

vcnUserChannelCTPSource-vc4TTPSourceR1

au3CTPBidirectionalR1-augBidirectional

au3CTPSinkR1-augBidirectional

au3CTPSinkR1-augSink

au4CTPBidirectionalR1-augBidirectional

au4CTPSink-augBidirectional

au4CTPSinkR1-augSink

tu11CTPBidirectionalR1-tug2Bidirectional

tu11CTPSinkR1-tug2Bidirectional

tu11CTPSinkR1-tug2Sink

tu12CTPBidirectionalR1-tug2Bidirectional

tu12CTPSinkR1-tug2Bidirectional

tu12CTPSinkR1-tug2Sink

tu2CTPBidirectionalR1-tug2Bidirectional

tu2CTPSinkR1-tug2Bidirectional

tu2CTPSinkR1-tug2Sink

tu3CTPBidirectionalR1-tug3Bidirectional

tu3CTPSinkR1-tug3Bidirectional

tu3CTPSinkR1-tug3Sink

tug2Bidirectional-vc3TTPBidirectionalR1

tug2Sink-vc3TTPSinkR1

tug2Source-vc3TTPSourceR1

tug3Bidirectional-vc4TTPBidirectionalR1

tug3Sink-vc4TTPSinkR1

tug3Source-vc4TTPSourceR1

vc11TTPBidirectionalR1-sdhNE

vc11TTPSinkR1-sdhNE

vc12TTPBidirectionalR1-sdhNE

vc12TTPSinkR1-sdhNE

vc2TTPBidirectionalR1-sdhNE

vc2TTPSinkR1-sdhNE

vc3TTPBidirectionalR1-sdhNE

vc3TTPSinkR1-sdhNE

vc3TTPSourceR1-sdhNE

vc4TTPBidirectionalR1-sdhNE

vc4TTPSinkR1-sdhNE

vc4TTPSourceR1-sdhNE

## 9.1	au3CTP

au3CTPBidirectionalR1-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE 	au3CTPId;

BEHAVIOUR

au3CTPBidirectionalR1-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 121 };

au3CTPSinkR1-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPSinkR1-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 122 };

au3CTPSinkR1-augSink NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	augSink;

WITH ATTRIBUTE 	au3CTPId;

BEHAVIOUR

au3CTPSinkR1-augSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 123 };

au3CTPSource-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSource;

NAMED BY SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPSource-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 4 };

au3CTPSource-augSource NAME BINDING

SUBORDINATE OBJECT CLASS	au3CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	augSource;

WITH ATTRIBUTE	au3CTPId;

BEHAVIOUR

au3CTPSource-augSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 5 };

## 9.2	au4CTP

au4CTPBidirectionalR1-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE 	au4CTPId;

BEHAVIOUR

au4CTPBidirectionalR1-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 124 };

au4CTPSinkR1-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE 	au4CTPId;

BEHAVIOUR

au4CTPSinkR1-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 125 };

au4CTPSinkR1-augSink NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS 	augSink;

WITH ATTRIBUTE 	au4CTPId;

BEHAVIOUR

au4CTPSinkR1-augSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 126 };

au4CTPSource-augBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	augBidirectional;

WITH ATTRIBUTE	au4CTPId;

BEHAVIOUR

au4CTPSource-augBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 9 };

au4CTPSource-augSource NAME BINDING

SUBORDINATE OBJECT CLASS	au4CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	augSource;

WITH ATTRIBUTE	au4CTPId;

BEHAVIOUR

au4CTPSource-augSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 10 };

## 9.3	aug

augBidirectional-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	augBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	augId;

BEHAVIOUR

augBidirectional-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 11 };

augSink-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	augSink;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPSink;

WITH ATTRIBUTE	augId;

BEHAVIOUR

augSink-msTTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 12 };

augSource-msTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	augSource;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPSource;

WITH ATTRIBUTE	augId;

BEHAVIOUR

augSource-msTTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 13 };

## 9.4	electricalSPITTP

electricalSPITTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	electricalSPITTPBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 14 };

electricalSPITTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	electricalSPITTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 15 };

electricalSPITTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	electricalSPITTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 16 };

msCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPBidirectional AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 17 };

## 9.5	msCTP

msCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 18 };

msCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPSink AND SUBCLASSES;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 19 };

msCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 20 };

msCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	msCTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE	msCTPId;

BEHAVIOUR

msCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 21 };

## 9.6	msDatacomCTP

msDatacomCTPBidirectional-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPBidirectional-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 22 };

msDatacomCTPSink-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSink-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 23 };

msDatacomCTPSink-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSink;

NAMED BY SUPERIOR OBJECT CLASS	msTTPSink;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSink-msTTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 24 };

msDatacomCTPSource-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSource-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 25 };

msDatacomCTPSource-msTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	msDatacomCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPSource;

WITH ATTRIBUTE	msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSource-msTTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 26 };

## 9.7	msOrderwireCTP

msOrderwireCTPBidirectional-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPBidirectional-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 27 };

msOrderwireCTPSink-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSink-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 28 };

msOrderwireCTPSink-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPSink;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSink-msTTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 29 };

msOrderwireCTPSource-msTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPBidirectional;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSource-msTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 30 };

msOrderwireCTPSource-msTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	msOrderwireCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	msTTPSource;

WITH ATTRIBUTE	msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSource-msTTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 31 };

## 9.8	msTTP

msTTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	msTTPBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 32 };

msTTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	msTTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 33 };

msTTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	msTTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 34 };

## 9.9	opticalSPITTP

opticalSPITTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	opticalSPITTPBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 35 };

opticalSPITTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	opticalSPITTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 36 };

opticalSPITTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	opticalSPITTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 37 };

rsCTPBidirectional-electricalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	electricalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPBidirectional-electricalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 38 };

## 9.10	rsCTP

rsCTPSink-electricalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	electricalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-electricalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 39 };

rsCTPSink-electricalSPITTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	electricalSPITTPSink;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-electricalSPITTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 40 };

rsCTPSource-electricalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	electricalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-electricalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 41 };

rsCTPSource-electricalSPITTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	electricalSPITTPSource;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-electricalSPITTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 42 };

rsCTPBidirectional-opticalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPBidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	opticalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPBidirectional-opticalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 43 };

rsCTPSink-opticalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	opticalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-opticalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 44 };

rsCTPSink-opticalSPITTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSink;

NAMED BY 	SUPERIOR OBJECT CLASS	opticalSPITTPSink;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSink-opticalSPITTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 45 };

rsCTPSource-opticalSPITTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	opticalSPITTPBidirectional;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-opticalSPITTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 46 };

rsCTPSource-opticalSPITTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsCTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	opticalSPITTPSource;

WITH ATTRIBUTE	rsCTPId;

BEHAVIOUR

rsCTPSource-opticalSPITTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 47 };

## 9.11	rsDatacomCTP

rsDatacomCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPBidirectional AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 48 };

rsDatacomCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 49 };

rsDatacomCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPSink AND SUBCLASSES;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 50 };

rsDatacomCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 51 };

rsDatacomCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsDatacomCTPSource AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS	rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE	rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 52 };

rsOrderwireCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPBidirectional AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 53 };

## 9.12	rsOrderwireCTP

rsOrderwireCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 54 };

rsOrderwireCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPSink AND SUBCLASSES;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 55 };

rsOrderwireCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 56 };

rsOrderwireCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsOrderwireCTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE	rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 57 };

## 9.13	rsTTP

rsTTPBidirectional-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 58 };

rsTTPSink-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	rsTTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 59 };

rsTTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	rsTTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 60 };

## 9.14	rsUserChannelCTP

rsUserChannelCTPBidirectional-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPBidirectional AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPBidirectional-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 61 };

rsUserChannelCTPSink-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSink-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 62 };

rsUserChannelCTPSink-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSink AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPSink AND SUBCLASSES;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSink-rsTTPSinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 63 };

rsUserChannelCTPSource-rsTTPBidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPBidirectional AND SUBCLASSES;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSource-rsTTPBidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 64 };

rsUserChannelCTPSource-rsTTPSource NAME BINDING

SUBORDINATE OBJECT CLASS	rsUserChannelCTPSource AND SUBCLASSES;

NAMED BY 	SUPERIOR OBJECT CLASS	rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE	rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSource-rsTTPSourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 65 };

## 9.15	tu11CTP

tu11CTPBidirectionalR1-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE 	tu11CTPId;

BEHAVIOUR

tu11CTPBidirectionalR1-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 127 };

tu11CTPSinkR1-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE 	tu11CTPId;

BEHAVIOUR
	tu11CTPSinkR1-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 128 };

tu11CTPSinkR1-tug2Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Sink;

WITH ATTRIBUTE 	tu11CTPId;

BEHAVIOUR

tu11CTPSinkR1-tug2SinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 129 };

tu11CTPSource-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu11CTPId;

BEHAVIOUR

tu11CTPSource-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 69 };

tu11CTPSource-tug2Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu11CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	tug2Source;

WITH ATTRIBUTE	tu11CTPId;

BEHAVIOUR

tu11CTPSource-tug2SourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 70 };

## 9.16	tu12CTP

tu12CTPBidirectionalR1-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE 	tu12CTPId;

BEHAVIOUR

tu12CTPBidirectionalR1-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 130 };

tu12CTPSinkR1-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSinkR1-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 131 };

tu12CTPSinkR1-tug2Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Sink;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSinkR1-tug2SinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 132 };

tu12CTPSource-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSource-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 74 };

tu12CTPSource-tug2Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu12CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	tug2Source;

WITH ATTRIBUTE	tu12CTPId;

BEHAVIOUR

tu12CTPSource-tug2SourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 75 };

## 9.17	tu2CTP

tu2CTPBidirectionalR1-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPBidirectionalR1-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 133 };

tu2CTPSinkR1-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE 	tu2CTPId;

BEHAVIOUR

tu2CTPSinkR1-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 134 };

tu2CTPSinkR1-tug2Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug2Sink;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPSinkR1-tug2SinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 135 };

tu2CTPSource-tug2Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	tug2Bidirectional;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPSource-tug2BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 79 };

tu2CTPSource-tug2Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu2CTPSource;

NAMED BY

SUPERIOR OBJECT CLASS	tug2Source;

WITH ATTRIBUTE	tu2CTPId;

BEHAVIOUR

tu2CTPSource-tug2SourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 80 };

## 9.18	tu3CTP

tu3CTPBidirectionalR1-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE 	tu3CTPId;

BEHAVIOUR

tu3CTPBidirectionalR1-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 136 };

tu3CTPSinkR1-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE 	tu3CTPId;

BEHAVIOUR

tu3CTPSinkR1-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 137 };

tu3CTPSinkR1-tug3Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	tug3Sink;

WITH ATTRIBUTE 	tu3CTPId;

BEHAVIOUR

tu3CTPSinkR1-tug3SinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 138 };

tu3CTPSource-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE	tu3CTPId;

BEHAVIOUR

tu3CTPSource-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 84 };

tu3CTPSource-tug3Source NAME BINDING

SUBORDINATE OBJECT CLASS	tu3CTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	tug3Source;

WITH ATTRIBUTE	tu3CTPId;

BEHAVIOUR

tu3CTPSource-tug3SourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 85 };

## 9.19	tug2

tug2Bidirectional-tug3Bidirectional NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Bidirectional;

NAMED BY 	SUPERIOR OBJECT CLASS	tug3Bidirectional;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Bidirectional-tug3BidirectionalBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 86 };

tug2Sink-tug3Sink NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Sink;

NAMED BY 	SUPERIOR OBJECT CLASS	tug3Sink;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Sink-tug3SinkBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 87 };

tug2Source-tug3Source NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Source;

NAMED BY 	SUPERIOR OBJECT CLASS	tug3Source;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Source-tug3SourceBehaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 88 };

tug2Bidirectional-vc3TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Bidirectional;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPBidirectionalR1;

WITH ATTRIBUTE	tug2Id;

BEHAVIOUR

tug2Bidirectional-vc3TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 139 };

tug2Sink-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Sink;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPSinkR1;

WITH ATTRIBUTE 	tug2Id;

BEHAVIOUR

tug2Sink-vc3TTPSinkR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 140 };

tug2Source-vc3TTPSourceR1 NAME BINDING

SUBORDINATE OBJECT CLASS	tug2Source;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPSourceR1;

WITH ATTRIBUTE 	tug2Id;

BEHAVIOUR

tug2Source-vc3TTPSourceR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 141 };

## 9.20	tug3

tug3Bidirectional-vc4TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	tug3Bidirectional;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPBidirectionalR1;

WITH ATTRIBUTE 	tug3Id;

BEHAVIOUR

tug3Bidirectional-vc4TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 142 };

tug3Sink-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS	tug3Sink;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPSinkR1;

WITH ATTRIBUTE 	tug3Id;

BEHAVIOUR

tug3Sink-vc4TTPSinkR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 143 };

tug3Source-vc4TTPSourceR1 NAME BINDING

SUBORDINATE OBJECT CLASS	tug3Source;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPSourceR1;

WITH ATTRIBUTE	tug3Id;

BEHAVIOUR

tug3Source-vc4TTPSourceR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 144 };

## 9.21	vc11TTP

vc11TTPBidirectionalR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc11TTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 145 };

vc11TTPSinkR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc11TTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 146 };

vc11TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc11TTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 97 };

## 9.22	vc12TTP

vc12TTPBidirectionalR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc12TTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 147 };

vc12TTPSinkR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc12TTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 148 };

vc12TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc12TTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 100 };

## 9.23	vc2TTP

vc2TTPBidirectionalR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc2TTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 149 };

vc2TTPSinkR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc2TTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 150 };

vc2TTPSource-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc2TTPSource;

NAMED BY 	SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE	vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 103 };

vc3TTPBidirectionalR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc3TTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 151 };

## 9.24	vc3TTP

vc3TTPSinkR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc3TTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 152 };

vc3TTPSourceR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc3TTPSourceR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 153 };

## 9.25	vc4TTP

vc4TTPBidirectionalR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc4TTPBidirectionalR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 154 };

vc4TTPSinkR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc4TTPSinkR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 155 };

vc4TTPSourceR1-sdhNE NAME BINDING

SUBORDINATE OBJECT CLASS	vc4TTPSourceR1;

NAMED BY SUPERIOR OBJECT CLASS	sdhNE;

WITH ATTRIBUTE 	vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774NameBinding 156 };

## 9.26	vcnUserChannelCTP

vcnUserChannelCTPBidirectional-vc3TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPBidirectional;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPBidirectionalR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPBidirectional-vc3TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 157 };

vcnUserChannelCTPSink-vc3TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPBidirectionalR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc3TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 158 };

vcnUserChannelCTPSink-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPSinkR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc3TTPSinkR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 159 };

vcnUserChannelCTPSource-vc3TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSource;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPBidirectionalR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc3TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 160 };

vcnUserChannelCTPSource-vc3TTPSourceR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSource;

NAMED BY SUPERIOR OBJECT CLASS	vc3TTPSourceR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc3TTPSourceR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 161 };

vcnUserChannelCTPBidirectional-vc4TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPBidirectional;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPBidirectionalR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPBidirectional-vc4TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 162 };

vcnUserChannelCTPSink-vc4TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPBidirectionalR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc4TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 163 };

vcnUserChannelCTPSink-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSink;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPSinkR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc4TTPSinkR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 164 };

vcnUserChannelCTPSource-vc4TTPBidirectionalR1 NAME BINDING

SUBORDINATE OBJECT CLASS	vcnUserChannelCTPSource;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPBidirectionalR1;

WITH ATTRIBUTE 	vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc4TTPBidirectionalR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 165 };

vcnUserChannelCTPSource-vc4TTPSourceR1 NAME BINDING

SUBORDINATE OBJECT CLASS		vcnUserChannelCTPSource;

NAMED BY SUPERIOR OBJECT CLASS	vc4TTPSourceR1;

WITH ATTRIBUTE 				vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc4TTPSourceR1Behaviour BEHAVIOUR

DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*

;;

REGISTERED AS { g774NameBinding 166 };

# 10	Supporting ASN.1

SDH {itu-t(0) recommendation(0) g(7) sdhm(774) informationModel(0) asn1Module(2) sdh(0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

NameType  -- Recommendation M.3100

FROM ASN1DefinedTypesModule {itu-t(0) recommendation(0) m(13) gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypeModule(0)}

;

g774 OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) g(7) sdhm(774) informationModel(0)}

g774ObjectClass OBJECT IDENTIFIER ::= {g774 managedObjectClass(3)}

g774Attribute OBJECT IDENTIFIER ::= {g774 attribute(7)}

g774NameBinding OBJECT IDENTIFIER ::= {g774 nameBinding(6)}

Boolean ::= BOOLEAN

C2SignalLabel ::= INTEGER (0..255)

defaultNull Null ::= NULL

Null ::= NULL

Integer ::= INTEGER

OpticalReach ::= ENUMERATED {

intraOffice(0),

shortHaul(1),

longHaul(2)

}

OpticalWavelength ::= ENUMERATED {

wl1310(0),

wl1550(1)

}

PathTrace ::= CHOICE {

null			NULL,

pathtrace		[1] GraphicString

}

-- Reffering to PointerSinkType. The ENUMERATED value of invalidPointer(2)

-- should be used when a LOP condition exists or if the pointer value is unknown.

PointerSinkType ::= ENUMERATED {

normalPointer(0),

concatenationIndication(1),

invalidPointer(2)

}

PointerSourceType ::= ENUMERATED {

normalPointer(0),

concatenationIndication(1)

}

TrailTraceReceiveSend	::=	GraphicString

V5SignalLabel ::= INTEGER (0..7)

END

# 11	Object relations

NOTE  The SUBORDINATION RULE and CONSTRAINT RULE templates are used in this Recommendation as an interim specification tool. A RELATIONSHIP template is currently under study; when it has been standardized, the rules specified in the SUBORDINATION RULE and CONSTRAINT RULE templates will be re-specified using the RELATIONSHIP template.

## 11.1	Syntax

<subordination-rule-label> SUBORDINATION RULE

SUPERIOR OBJECT CLASS <class-label> ;

NAMES SUBORDINATES <class-list> ;

ACCORDING TO RULE <subordination-rule> ;

;

<constraint-rule-label> CONSTRAINT RULE

OBJECT CLASS <class-label> [AND SUBCLASSES] ;

IS RELATED TO <class-list> ;

USING ATTRIBUTE <attribute-label> ;

<constraint-rule-set> ;

;

<constraint-rule-set> ::= <single-constraint-rule> | <named-type-constraint-rules>

<single-constraint-rule> ::= ACCORDING TO RULE <constraint-rule>

<named-type-constraint-rule> ::= CASE { <named-type-constraint-rule-list> }

<named-type-constraint-rule-list> ::= <named-type-constraint-rule-item> |

<named-type-constraint-rule-item> , <named-type-constraint-rule-list>

<named-type-constraint-rule-item> ::=

<named-type> ACCORDING TO RULE <constraint-rule>

<class-label> ::= label string as defined in ISO/IEC IS 10165-4

<attribute-label> ::= label string as defined in ISO/IEC IS 10165-4

<class-list> ::= <class-label> | <class-list> , <class-label>

<subordination-rule> ::= SET { <subordination-members> }

| CHOICE { <subordination-members> }

| SET SIZE ( <ordinality> ) OF <subordination-term>

<constraint-rule> ::= SET { <constraint-members> }

| SEQUENCE { <constraint-members> }

| CHOICE { <constraint-members> }

| SET SIZE ( <ordinality> ) OF <constraint-term>

| SEQUENCE SIZE ( <ordinality> ) OF <constraint-term>

<subordination-members> ::= <subordination-term>

| <subordination-term> , <subordination-members>

<constraint-members> ::=  <constraint-term>

| <constraint-term> , <constraint-members>

<subordination-term> ::= <class-label> | <subordination-rule>

<constraint-term> ::= <class-label> | <constraint-rule>

<ordinality> ::= <valueRange> | <valueList>

<valueRange> ::= <lowerValue> .. <upperValue>

<valueList>  ::= <itemValue> | <itemValue> , <valueList>

<itemValue>  ::= INTEGER

<lowerValue> ::= INTEGER

<upperValue> ::= INTEGER | N

### 11.1.1	Subordination rule templates

SUPERIOR OBJECT CLASS <class-label>;

indicates the class that is governing this subordination-rule. A class may govern through more than one subordination-rule several non-overlapping sets of subordinate classes in the NAMES SUBORDINATES clause.

NAMES SUBORDINATES <class-list>;

indicates the set of classes of subordinates that are governed by this subordination-rule. Any classes that are not in this list are not governed by this subordination-rule.

ACCORDING TO RULE <subordination-rule>

provides the rule

SET { <subordination-members> }

indicates that *all* of the subordination-members must be present.

CHOICE { <subordination-members> }

indicates that *any one* of the subordination-members must be present.

SET SIZE <ordinality> OF <subordination-term>

indicates the number of <subordination-term> that must be present.

### 11.1.2	Constraint rule templates

OBJECT CLASS <class-label>;

indicates the class with this attribute that is governed by this constraint-rule. A class may be governed by more than one constraint-rule with non-overlapping sets of related classes in the RELATES TO OBJECT CLASSES clause.

IS RELATED TO OBJECT CLASSES <class-list>;

indicates the set of classes of related instances that are governed by this constraint-rule. Any classes that are not in this list are not governed by this constraint-rule.

USING ATTRIBUTE <attribute-label>;

indicates the attribute that represents a relationship by means of a pointer (DN) to the related object instances.

<constraint-rule-set>;

there can be either a single rule, or a set of rules one for each of a set of named choices. In the latter case the CASE { ... } structure is used.

CASE { ... };

provides a distinct constraint-rule for each of the set of named choices in the attribute syntax.

ACCORDING TO RULE <constraint-rule>

provides the rule

SET { <constraint-members> }

indicates that *all* of the constraint-members must be present in any order.

SEQUENCE { <constraint-members> }

indicates that *all* of the constraint-members must be present in sequence.

CHOICE { <constraint-members> }

indicates that *any one* of the constraint-members must be present.

SET SIZE <ordinality> OF <constraint-term>

indicates the number of <constraint-term> that must be present in any order.

SEQUENCE SIZE <ordinality> OF <constraint-term>

indicates the number of <constraint-term> that must be present in sequence.

## 11.2	Connectivity pointer constraints

This clause defines the allowable values for the downstreamConnectivityPointer and upstreamConnectivityPointer attributes using the object classes defined in this Recommendation.

This clause provides replacement constraint rule definitions for the existing Recommendation G.774 (1992). Any constraint rule replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a constraint rule are as follows:

1)	The replaced constraint rule is faulty and must be fixed.

2)	The replaced constraint rule refers to a managed object class which has been re-registered in this Recommendation.

3)	The replaced constraint rule refers to an attribute which has been re-registered in this Recommendation.

In each case where a constraint is replaced, the new constraint will be registered within this Recommendation. The textual label for the constraint will be revised to include the text "R1". For example, in the revision of the G.774 (1992) constraint "downstreamConnectivityPointer-au3CTPSink", the revised label will become "downstreamConnectivityPointer-au3CTPSinkR1". Note the "R1" is placed immediately following the revised class which impacts the constraint. In the case where the class within the label has not changed but the constraint is still altered because the constraint refers to a class that has changed, then the "R1" is placed immediately following the "downstreamConnectivityPointer" text of the revised constraint label. For example, in the revision of the G.774 (1992) constraint "downstreamConnectivityPointer-au3CTPSource", the revised label will become "downstreamConnectivityPointerR1-au3CTPSource".

Below is a table of constraint rules deprecated from Recommendation G.774 (1992) and the G.774 constraint rules which replace them:

Deprecated G.774 (1992) Constraint Rules

downstreamConnectivityPointer-au3CTPSink

upstreamConnectivityPointer-au3CTPSource

downstreamConnectivityPointer-au4CTPSink

upstreamConnectivityPointer-au4CTPSource

downstreamConnectivityPointer-tu11CTPSink

upstreamConnectivityPointer-tu11CTPSource

downstreamConnectivityPointer-tu12CTPSink

upstreamConnectivityPointer-tu12CTPSource

downstreamConnectivityPointer-tu2CTPSink

upstreamConnectivityPointer-tu2CTPSource

downstreamConnectivityPointer-tu3CTPSink

upstreamConnectivityPointer-tu3CTPSource

upstreamConnectivityPointer-vc11TTPSink

downstreamConnectivityPointer-vc11TTPSource

upstreamConnectivityPointer-vc12TTPSink

downstreamConnectivityPointer-vc12TTPSource

upstreamConnectivityPointer-vc2TTPSink

downstreamConnectivityPointer-vc2TTPSource

upstreamConnectivityPointer-vc3TTPSink

downstreamConnectivityPointer-vc3TTPSource

upstreamConnectivityPointer-vc4TTPSink

downstreamConnectivityPointer-vc4TTPSource

Replacement G.774 Constraint Rules

downstreamConnectivityPointer-au3CTPSinkR1

upstreamConnectivityPointerR1-au3CTPSource

downstreamConnectivityPointer-au4CTPSinkR1

upstreamConnectivityPointerR1-au4CTPSource

downstreamConnectivityPointer-tu11CTPSinkR1

upstreamConnectivityPointerR1-tu11CTPSource

downstreamConnectivityPointer-tu12CTPSinkR1

upstreamConnectivityPointerR1-tu12CTPSource

downstreamConnectivityPointer-tu2CTPSinkR1

upstreamConnectivityPointerR1-tu2CTPSource

downstreamConnectivityPointer-tu3CTPSinkR1

upstreamConnectivityPointerR1-tu3CTPSource

upstreamConnectivityPointer-vc11TTPSinkR1

downstreamConnectivityPointerR1-vc11TTPSource

upstreamConnectivityPointer-vc12TTPSinkR1

downstreamConnectivityPointerR1-vc12TTPSource

upstreamConnectivityPointer-vc2TTPSinkR1

downstreamConnectivityPointerR1-vc2TTPSource

upstreamConnectivityPointer-vc3TTPSinkR1

downstreamConnectivityPointer-vc3TTPSourceR1

upstreamConnectivityPointer-vc4TTPSinkR1

downstreamConnectivityPointer-vc4TTPSourceR1

downstreamConnectivityPointer-au3CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

au3CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc3TTPSinkR1, vc3TTPBidirectionalR1,

au3CTPSource, au3CTPBidirectionalR1,

tu3CTPSource, tu3CTPBidirectionalR1,

vc4TTPSinkR1, vc4TTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSinkR1,vc3TTPBidirectionalR1,

au3CTPSource,

au3CTPBidirectionalR1,

tu3CTPSource,

tu3CTPBidirectionalR1,

vc4TTPSinkR1,vc4TTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SET SIZE(1..N) OF CHOICE {

vc3TTPSinkR1, vc3TTPBidirectionalR1,

tu3CTPSource,

tu3CTPBidirectionalR1,

au3CTPSource,

au3CTPBidirectionalR1 },

SET SIZE(1..N) OF CHOICE {

vc4TTPSinkR1, vc4TTPBidirectionalR1 }

}

};

;

upstreamConnectivityPointerR1-au3CTPSource CONSTRAINT RULE

OBJECT CLASS

au3CTPSource AND SUBCLASSES;

IS RELATED TO

vc3TTPSourceR1, vc3TTPBidirectionalR1,

au3CTPSinkR1, au3CTPBidirectionalR1,

tu3CTPSinkR1, tu3CTPBidirectionalR1,

vc4TTPSourceR1, vc4TTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE { vc3TTPSourceR1,vc3TTPBidirectionalR1,

au3CTPSinkR1,au3CTPBidirectionalR1,

tu3CTPSinkR1,tu3CTPBidirectionalR1,

vc4TTPSourceR1,vc4TTPBidirectionalR1 }

};

;

downstreamConnectivityPointer-au4CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

au4CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

au4CTPSource, au4CTPBidirectionalR1,

vc4TTPSinkR1, vc4TTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSinkR1,vc4TTPBidirectionalR1,

au4CTPSource,

au4CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc4TTPSinkR1, vc4TTPBidirectionalR1,

au4CTPSource,

au4CTPBidirectionalR1 }

};

;

upstreamConnectivityPointerR1-au4CTPSource CONSTRAINT RULE

OBJECT CLASS

au4CTPSource AND SUBCLASSES;

IS RELATED TO

au4CTPSinkR1, au4CTPBidirectionalR1,

vc4TTPSourceR1, vc4TTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSourceR1, vc4TTPBidirectionalR1,

au4CTPSinkR1, au4CTPBidirectionalR1 }

};

;

downstreamConnectivityPointer-msCTPSink CONSTRAINT RULE

OBJECT CLASS

msCTPSink AND SUBCLASSES;

IS RELATED TO

msTTPSink, msTTPBidirectional,

msCTPSource, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msTTPSink, msTTPBidirectional,

msCTPSource, msCTPBidirectional }

};

;

upstreamConnectivityPointer-msCTPSource CONSTRAINT RULE

OBJECT CLASS

msCTPSource AND SUBCLASSES;

IS RELATED TO

msTTPSource, msTTPBidirectional,

msCTPSink, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msTTPSource, msTTPBidirectional,

msCTPSink, msCTPBidirectional }

};

;

upstreamConnectivityPointer-msTTPSink CONSTRAINT RULE

OBJECT CLASS

msTTPSink AND SUBCLASSES;

IS RELATED TO

msCTPSink, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msCTPSink, msCTPBidirectional }

};

;

downstreamConnectivityPointer-msTTPSource CONSTRAINT RULE

OBJECT CLASS

msTTPSource AND SUBCLASSES;

IS RELATED TO

msCTPSource, msCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

msCTPSource, msCTPBidirectional }

};

;

downstreamConnectivityPointer-rsCTPSink CONSTRAINT RULE

OBJECT CLASS

rsCTPSink AND SUBCLASSES;

IS RELATED TO

rsTTPSink, rsTTPBidirectional,

rsTTPTrailTraceSink, rsTTPTrailTraceBidirectional,

rsCTPSource, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsTTPSink, rsTTPBidirectional,

rsTTPTrailTraceSink, rsTTPTrailTraceBidirectional,

rsCTPSource, rsCTPBidirectional }

};

;

upstreamConnectivityPointer-rsCTPSource CONSTRAINT RULE

OBJECT CLASS

rsCTPSource AND SUBCLASSES;

IS RELATED TO

rsTTPSource, rsTTPBidirectional,

rsTTPTrailTraceSource, rsTTPTrailTraceBidirectional,

rsCTPSink, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsTTPSource, rsTTPBidirectional,

rsTTPTrailTraceSource, rsTTPTrailTraceBidirectional,

rsCTPSink, rsCTPBidirectional }

};

;

upstreamConnectivityPointer-rsTTPSink CONSTRAINT RULE

OBJECT CLASS

rsTTPSink AND SUBCLASSES;

IS RELATED TO

rsCTPSink, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSink, rsCTPBidirectional }

};

;

downstreamConnectivityPointer-rsTTPSource CONSTRAINT RULE

OBJECT CLASS

rsTTPSource AND SUBCLASSES;

IS RELATED TO

rsCTPSource, rsCTPBidirectional;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSource, rsCTPBidirectional }

};

;

downstreamConnectivityPointer-tu11CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

tu11CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc11TTPSinkR1, vc11TTPBidirectionalR1,

tu11CTPSource, tu11CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSinkR1, vc11TTPBidirectionalR1,

tu11CTPSource,

tu11CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc11TTPSinkR1, vc11TTPBidirectionalR1,

tu11CTPSource,

tu11CTPBidirectionalR1 }

};

;

upstreamConnectivityPointerR1-tu11CTPSource CONSTRAINT RULE

OBJECT CLASS

tu11CTPSource AND SUBCLASSES;

IS RELATED TO

vc11TTPSource, vc11TTPBidirectionalR1,

tu11CTPSinkR1, tu11CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSource,

vc11TTPBidirectionalR1,

tu11CTPSinkR1, tu11CTPBidirectionalR1 }

};

;

downstreamConnectivityPointer-tu12CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

tu12CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc12TTPSinkR1, vc12TTPBidirectionalR1,

tu12CTPSource, tu12CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSinkR1, vc12TTPBidirectionalR1,

tu12CTPSource,

tu12CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc12TTPSinkR1, vc12TTPBidirectionalR1,

tu12CTPSource,

tu12CTPBidirectionalR1 }

};

;

upstreamConnectivityPointerR1-tu12CTPSource CONSTRAINT RULE

OBJECT CLASS

tu12CTPSource AND SUBCLASSES;

IS RELATED TO

vc12TTPSource, vc12TTPBidirectionalR1,

tu12CTPSinkR1, tu12CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSource,

vc12TTPBidirectionalR1,

tu12CTPSinkR1, tu12CTPBidirectionalR1 }

};

;

downstreamConnectivityPointer-tu2CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

tu2CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc2TTPSinkR1, vc2TTPBidirectionalR1,

tu2CTPSource, tu2CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSinkR1, vc2TTPBidirectionalR1,

tu2CTPSource,

tu2CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc2TTPSinkR1, vc2TTPBidirectionalR1,

tu2CTPSource,

tu2CTPBidirectionalR1 }

};

;

upstreamConnectivityPointerR1-tu2CTPSource CONSTRAINT RULE

OBJECT CLASS

tu2CTPSource AND SUBCLASSES;

IS RELATED TO

vc2TTPSource, vc2TTPBidirectionalR1,

tu2CTPSinkR1, tu2CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSource,

vc2TTPBidirectionalR1,

tu2CTPSinkR1, tu2CTPBidirectionalR1 }

};

;

downstreamConnectivityPointer-tu3CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

tu3CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc3TTPSinkR1, vc3TTPBidirectionalR1,

au3CTPSource, au3CTPBidirectionalR1,

tu3CTPSource, tu3CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSinkR1, vc3TTPBidirectionalR1,

au3CTPSource,

au3CTPBidirectionalR1,

tu3CTPSource,

tu3CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc3TTPSinkR1, vc3TTPBidirectionalR1,

au3CTPSource,

au3CTPBidirectionalR1,

tu3CTPSource,

tu3CTPBidirectionalR1 }

};

;

upstreamConnectivityPointerR1-tu3CTPSource CONSTRAINT RULE

OBJECT CLASS

tu3CTPSource AND SUBCLASSES;

IS RELATED TO

vc3TTPSourceR1, vc3TTPBidirectionalR1,

au3CTPSinkR1, au3CTPBidirectionalR1,

tu3CTPSinkR1, tu3CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSourceR1, vc3TTPBidirectionalR1,

au3CTPSinkR1, au3CTPBidirectionalR1,

tu3CTPSinkR1, tu3CTPBidirectionalR1 }

};

;

upstreamConnectivityPointer-vc11TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

vc11TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc11TTPSource, vc11TTPBidirectionalR1,

tu11CTPSinkR1, tu11CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSource,

vc11TTPBidirectionalR1,

tu11CTPSinkR1, tu11CTPBidirectionalR1 }

};

;

downstreamConnectivityPointerR1-vc11TTPSource CONSTRAINT RULE

OBJECT CLASS

vc11TTPSource AND SUBCLASSES;

IS RELATED TO

vc11TTPSinkR1, vc11TTPBidirectionalR1,

tu11CTPSource, tu11CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc11TTPSinkR1, vc11TTPBidirectionalR1,

tu11CTPSource,

tu11CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc11TTPSinkR1, vc11TTPBidirectionalR1,

tu11CTPSource,

tu11CTPBidirectionalR1 }

};

;

upstreamConnectivityPointer-vc12TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

vc12TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc12TTPSource, vc12TTPBidirectionalR1,

tu12CTPSinkR1, tu12CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSource,

vc12TTPBidirectionalR1,

tu12CTPSinkR1, tu12CTPBidirectionalR1 }

};

;

downstreamConnectivityPointerR1-vc12TTPSource CONSTRAINT RULE

OBJECT CLASS

vc12TTPSource AND SUBCLASSES;

IS RELATED TO

vc12TTPSinkR1, vc12TTPBidirectionalR1,

tu12CTPSource, tu12CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc12TTPSinkR1, vc12TTPBidirectionalR1,

tu12CTPSource,

tu12CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc12TTPSinkR1, vc12TTPBidirectionalR1,

tu12CTPSource,

tu12CTPBidirectionalR1 }

};

;

upstreamConnectivityPointer-vc2TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

vc2TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc2TTPSource, vc2TTPBidirectionalR1,

tu2CTPSinkR1, tu2CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSource,

vc2TTPBidirectionalR1,

tu2CTPSinkR1, tu2CTPBidirectionalR1 }

};

;

downstreamConnectivityPointerR1-vc2TTPSource CONSTRAINT RULE

OBJECT CLASS

vc2TTPSource AND SUBCLASSES;

IS RELATED TO

vc2TTPSinkR1, vc2TTPBidirectionalR1,

tu2CTPSource, tu2CTPBidirectionalR1;

USING ATTRIBUTE
		"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc2TTPSinkR1, vc2TTPBidirectionalR1,

tu2CTPSource,

tu2CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc2TTPSinkR1, vc2TTPBidirectionalR1,

tu2CTPSource,

tu2CTPBidirectionalR1 }

};

;

upstreamConnectivityPointer-vc3TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

vc3TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc3TTPSourceR1, vc3TTPBidirectionalR1,

au3CTPSinkR1, au3CTPBidirectionalR1,

tu3CTPSinkR1, tu3CTPBidirectionalR1;
	USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSourceR1, vc3TTPBidirectionalR1,

au3CTPSinkR1, au3CTPBidirectionalR1,

tu3CTPSinkR1, tu3CTPBidirectionalR1 }

};

;

downstreamConnectivityPointer-vc3TTPSourceR1 CONSTRAINT RULE

OBJECT CLASS

vc3TTPSourceR1 AND SUBCLASSES;

IS RELATED TO

vc3TTPSinkR1, vc3TTPBidirectionalR1,

au3CTPSource, au3CTPBidirectionalR1,

tu3CTPSource, tu3CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc3TTPSinkR1, vc3TTPBidirectionalR1,

au3CTPSource,

au3CTPBidirectionalR1,

tu3CTPSource,

tu3CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc3TTPSinkR1, vc3TTPBidirectionalR1,

au3CTPSource,

au3CTPBidirectionalR1,

tu3CTPSource,

tu3CTPBidirectionalR1 }

};

;

upstreamConnectivityPointer-vc4TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

vc4TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc4TTPSourceR1, vc4TTPBidirectionalR1,

au4CTPSinkR1, au4CTPBidirectionalR1,

au3CTPSinkR1, au3CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSourceR1, vc4TTPBidirectionalR1,

au4CTPSinkR1, au4CTPBidirectionalR1 },

concatenated ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SEQUENCE SIZE(3) OF au3CTPSinkR1,

SEQUENCE SIZE(3) OF au3CTPBidirectionalR1 }

};

;

downstreamConnectivityPointer-vc4TTPSourceR1 CONSTRAINT RULE

OBJECT CLASS

vc4TTPSourceR1 AND SUBCLASSES;

IS RELATED TO

vc4TTPSinkR1, vc4TTPBidirectionalR1,

au4CTPSource, au4CTPBidirectionalR1,

au3CTPSource, au3CTPBidirectionalR1;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

vc4TTPSinkR1, vc4TTPBidirectionalR1,

au4CTPSource,

au4CTPBidirectionalR1 },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc4TTPSinkR1, vc4TTPBidirectionalR1,

au4CTPSource,

au4CTPBidirectionalR1 },

concatenated ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SEQUENCE SIZE(3) OF

au3CTPSource,

SEQUENCE SIZE(3) OF au3CTPBidirectionalR1 },

broadcastConcatenated ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

SEQUENCE SIZE(3) OF

au3CTPSource,

SEQUENCE SIZE(3) OF au3CTPBidirectionalR1 }

};

;

## 11.3	Naming constraints

This clause defines the allowable combinations of subordinate object class instances that may be named by a superior object class instance, using the object classes contained in this Recommendation.

This clause provides replacement subordination rule definitions for the existing Recommendation G.774 (1992). Any subordination rule replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a subordination rule are as follows:

1)	The replaced subordination rule is faulty and must be fixed.

2)	The replaced subordination rule refers to a managed object class which has been re-registered in this Recommendation.

In each case where a subordination rule is replaced, the new subordination rule will be registered within this Recommendation. The textual label for the subordination rule will be revised to include the text "R1". For example, in the revision of the G.774 (1992) subordination rule "vc3TTPSinkSubordination", the revised label will become "vc3TTPSinkR1Subordination". Note the "R1" is placed immediately following the revised class which impacts the subordination rule. In the case where the class within the label has not changed but the subordination rule is still altered because the subordination rule refers to a class that has changed, then the "R1" is placed at the end of the revised subordination rule label. For example, in the revision of the G.774 (1992) subordination rule "tug3BidirectionalSubordination", the revised label will become "tug3BidirectionalSubordinationR1".

Below is a table of subordination rules deprecated from Recommendation G.774 (1992) and the G.774 subordination rules which replace them.

Deprecated G.774 (1992) Subordination Rules

augSinkSubordination

augBidirectionalSubordination

sdhNESubordination

tug2SinkSubordination

tug2BidirectionalSubordination

tug3SinkSubordination

tug3BidirectionalSubordination

vc3TTPSinkSubordination

vc3TTPSourceSubordination

vc3TTPBidirectionalSubordination

vc4TTPSinkSubordination

vc4TTPSourceSubordination

vc4TTPBidirectionalSubordination

Replacement G.774 Subordination Rules

augSinkSubordinationR1

augBidirectionalSubordinationR1

sdhNESubordinationR1

tug2SinkSubordinationR1

tug2BidirectionalSubordinationR1

tug3SinkSubordinationR1

tug3BidirectionalSubordinationR1

vc3TTPSinkR1Subordination

vc3TTPSourceSubordinationR1

vc3TTPBidirectionalR1Subordination

vc4TTPSinkR1Subordination

vc4TTPSourceR1Subordination

vc4TTPBidirectionalR1Subordination

augSinkSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

augSink;

NAMES SUBORDINATES

au3CTPSinkR1,

au4CTPSinkR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF au4CTPSinkR1,

SET SIZE(3) OF au3CTPSinkR1

};

;

augSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

augSource;

NAMES SUBORDINATES

au3CTPSource,

au4CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF au4CTPSource,

SET SIZE(3) OF au3CTPSource

};

;

augBidirectionalSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

augBidirectional;

NAMES SUBORDINATES

au3CTPSinkR1, au3CTPSource,

au3CTPBidirectionalR1,

au4CTPSinkR1, au4CTPSource,

au4CTPBidirectionalR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

au4CTPSinkR1, au4CTPSource,

au4CTPBidirectionalR1 },

SET SIZE(3) OF CHOICE {

au3CTPSinkR1, au3CTPSource,

au3CTPBidirectionalR1 }

};

;

electricalSPITTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

electricalSPITTPSink;

NAMES SUBORDINATES

rsCTPSink;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSink;

;

electricalSPITTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

electricalSPITTPSource;

NAMES SUBORDINATES

rsCTPSource;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSource;

;

electricalSPITTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

electricalSPITTPBidirectional;

NAMES SUBORDINATES

rsCTPSink, rsCTPSource, rsCTPBidirectional;

ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSink, rsCTPSource, rsCTPBidirectional };

;

opticalSPITTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

opticalSPITTPSink;

NAMES SUBORDINATES

rsCTPSink;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSink;

;

opticalSPITTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

opticalSPITTPSource;

NAMES SUBORDINATES

rsCTPSource;

ACCORDING TO RULE

SET SIZE(1) OF rsCTPSource;

;

opticalSPITTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

opticalSPITTPBidirectional;

NAMES SUBORDINATES

rsCTPSink, rsCTPSource, rsCTPBidirectional;

ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

rsCTPSink, rsCTPSource, rsCTPBidirectional };

;

msTTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

msTTPSink;

NAMES SUBORDINATES

augSink,

msDatacomCTPSink,

msOrderwireCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF augSink,

SET SIZE(0..1) OF msDatacomCTPSink,

SET SIZE(0..1) OF msOrderwireCTPSink

};

;

msTTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

msTTPSource;

NAMES SUBORDINATES

augSource,

msDatacomCTPSource,

msOrderwireCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF augSource,

SET SIZE(0..1) OF msDatacomCTPSource,

SET SIZE(0..1) OF msOrderwireCTPSource

};

;

msTTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

msTTPBidirectional;

NAMES SUBORDINATES

augBidirectional,

msDatacomCTPSink, msDatacomCTPSource,

msDatacomCTPBidirectional,

msOrderwireCTPSink, msOrderwireCTPSource,

msOrderwireCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF augBidirectional,

SET SIZE(0..1) OF CHOICE {

msDatacomCTPSink, msDatacomCTPSource,

msDatacomCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

msOrderwireCTPSink, msOrderwireCTPSource,

msOrderwireCTPBidirectional }

};

;

rsTTPSinkSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

rsTTPSink AND SUBCLASSES;

NAMES SUBORDINATES

msCTPSink,

rsDatacomCTPSink,

rsOrderwireCTPSink,

rsUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(1) OF msCTPSink,

SET SIZE(0..1) OF rsDatacomCTPSink,

SET SIZE(0..1) OF rsOrderwireCTPSink,

SET SIZE(0..1) OF rsUserChannelCTPSink

};

;

rsTTPSourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

rsTTPSource AND SUBCLASSES;

NAMES SUBORDINATES

msCTPSource,

rsDatacomCTPSource,

rsOrderwireCTPSource,

rsUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(1) OF msCTPSource,

SET SIZE(0..1) OF rsDatacomCTPSource,

SET SIZE(0..1) OF rsOrderwireCTPSource,

SET SIZE(0..1) OF rsUserChannelCTPSource

};

;

rsTTPBidirectionalSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

rsTTPBidirectional AND SUBCLASSES;

NAMES SUBORDINATES

msCTPSink, msCTPSource, msCTPBidirectional,

rsDatacomCTPSink, rsDatacomCTPSource,rsDatacomCTPBidirectional,

rsOrderwireCTPSink, rsOrderwireCTPSource,

rsOrderwireCTPBidirectional,

rsUserChannelCTPSink, rsUserChannelCTPSource,

rsUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(1) OF CHOICE {

msCTPSink, msCTPSource, msCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

rsDatacomCTPSink, rsDatacomCTPSource,

rsDatacomCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

rsOrderwireCTPSink, rsOrderwireCTPSource,

rsOrderwireCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

rsUserChannelCTPSink, rsUserChannelCTPSource,

rsUserChannelCTPBidirectional }

};

;

sdhNESubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

sdhNE;

NAMES SUBORDINATES

electricalSPITTPSink,

electricalSPITTPSource,

electricalSPITTPBidirectional,

msTTPSink,

msTTPSource,

msTTPBidirectional,

opticalSPITTPSink,

opticalSPITTPSource,

opticalSPITTPBidirectional,

rsTTPSink,

rsTTPSource,

rsTTPBidirectional,

rsTTPTrailTraceSink,

rsTTPTrailTraceSource,

rsTTPTrailTraceBidirectional,

vc11TTPSinkR1,

vc11TTPSource,

vc11TTPBidirectionalR1,

vc12TTPSinkR1,

vc12TTPSource,

vc12TTPBidirectionalR1,

vc2TTPSinkR1,

vc2TTPSource,

vc2TTPBidirectionalR1,

vc3TTPSinkR1, vc3TTPSourceR1, vc3TTPBidirectionalR1,

vc4TTPSinkR1, vc4TTPSourceR1, vc4TTPBidirectionalR1;

ACCORDING TO RULE

SET {

SET SIZE(0..N) OF

electricalSPITTPSink,

SET SIZE(0..N) OF

electricalSPITTPSource,

SET SIZE(0..N) OF

electricalSPITTPBidirectional,

SET SIZE(0..N) OF msTTPSink,

SET SIZE(0..N) OF msTTPSource,

SET SIZE(0..N) OF

msTTPBidirectional,

SET SIZE(0..N) OF

opticalSPITTPSink,

SET SIZE(0..N) OF

opticalSPITTPSource,

SET SIZE(0..N) OF

opticalSPITTPBidirectional,

SET SIZE(0..N) OF rsTTPSink,

SET SIZE(0..N) OF rsTTPSource,

SET SIZE(0..N) OF rsTTPBidirectional,

SET SIZE(0..N) OF rsTTPTrailTraceSink,

SET SIZE(0..N) OF rsTTPTrailTraceSource,

SET SIZE(0..N) OF rsTTPTrailTraceBidirectional,

SET SIZE(0..N) OF vc11TTPSinkR1,

SET SIZE(0..N) OF vc11TTPSource,

SET SIZE(0..N) OF vc11TTPBidirectionalR1,

SET SIZE(0..N) OF vc12TTPSinkR1,

SET SIZE(0..N) OF vc12TTPSource,

SET SIZE(0..N) OF vc12TTPBidirectionalR1,

SET SIZE(0..N) OF vc2TTPSinkR1,

SET SIZE(0..N) OF vc2TTPSource,

SET SIZE(0..N) OF vc2TTPBidirectionalR1,

SET SIZE(0..N) OF vc3TTPSinkR1,

SET SIZE(0..N) OF vc3TTPSourceR1,

SET SIZE(0..N) OF vc3TTPBidirectionalR1,

SET SIZE(0..N) OF vc4TTPSinkR1,

SET SIZE(0..N) OF vc4TTPSourceR1,

SET SIZE(0..N) OF vc4TTPBidirectionalR1

};

;

tug2SinkSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug2Sink;

NAMES SUBORDINATES

tu11CTPSinkR1,

tu12CTPSinkR1,

tu2CTPSinkR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu2CTPSinkR1,

SET SIZE(3) OF tu12CTPSinkR1,

SET SIZE(4) OF tu11CTPSinkR1

};

;

tug2SourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug2Source;

NAMES SUBORDINATES

tu11CTPSource,

tu12CTPSource,

tu2CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu2CTPSource,

SET SIZE(3) OF tu12CTPSource,

SET SIZE(4) OF tu11CTPSource

};

;

tug2BidirectionalSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug2Bidirectional;

NAMES SUBORDINATES

tu11CTPSinkR1,

tu11CTPSource,

tu11CTPBidirectionalR1,

tu12CTPSinkR1,

tu12CTPSource,

tu12CTPBidirectionalR1,

tu2CTPSinkR1,

tu2CTPSource,

tu2CTPBidirectionalR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

tu2CTPSinkR1,

tu2CTPSource,

tu2CTPBidirectionalR1 },

SET SIZE(3) OF CHOICE {

tu12CTPSinkR1,

tu12CTPSource,

tu12CTPBidirectionalR1 },

SET SIZE(4) OF CHOICE {

tu11CTPSinkR1,

tu11CTPSource,

tu11CTPBidirectionalR1 }

};

;

tug3SinkSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug3Sink;

NAMES SUBORDINATES

tug2Sink,

tu3CTPSinkR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu3CTPSinkR1,

SET SIZE(7) OF tug2Sink

};

;

tug3SourceSubordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug3Source;

NAMES SUBORDINATES

tug2Source,

tu3CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF tu3CTPSource,

SET SIZE(7) OF tug2Source

};

;

tug3BidirectionalSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

tug3Bidirectional;

NAMES SUBORDINATES

tug2Sink,

tug2Source,

tug2Bidirectional,

tu3CTPSinkR1,

tu3CTPSource,

tu3CTPBidirectionalR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

tu3CTPSinkR1,

tu3CTPSource,

tu3CTPBidirectionalR1 }

SET SIZE(7) OF CHOICE {

tug2Sink,

tug2Source,

tug2Bidirectional }

};

;

vc3TTPSinkR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc3TTPSink;

NAMES SUBORDINATES

tug2Sink,

vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(7) OF tug2Sink,

SET SIZE(1) OF vcnUserChannelCTPSink

};

;

vc3TTPSourceSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc3TTPSourceR1;

NAMES SUBORDINATES

tug2Source,

vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(7) OF tug2Source,

SET SIZE(1) OF vcnUserChannelCTPSource

};

;

vc3TTPBidirectionalR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc3TTPBidirectionalR1;

NAMES SUBORDINATES

tug2Bidirectional,

vcnUserChannelCTPSink,

vcnUserChannelCTPSource,

vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(7) OF tug2Bidirectional,

SET SIZE(1) OF CHOICE {

vcnUserChannelCTPSink,

vcnUserChannelCTPSource,

vcnUserChannelCTPBidirectional }

};

;

vc4TTPSinkR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc4TTPSinkR1;

NAMES SUBORDINATES

tug3Sink,

vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(3) OF tug3Sink,

SET SIZE(1) OF vcnUserChannelCTPSink

};

;

vc4TTPSourceR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc4TTPSourceR1;

NAMES SUBORDINATES

tug3Source,

vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(3) OF tug3Source,

SET SIZE(1) OF vcnUserChannelCTPSource

};

;

vc4TTPBidirectionalR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

vc4TTPBidirectionalR1;

NAMES SUBORDINATES

tug3Bidirectional,

vcnUserChannelCTPSink,

vcnUserChannelCTPSource,

vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(3) OF tug3Bidirectional,

SET SIZE(1) OF CHOICE {

vcnUserChannelCTPSink,

vcnUserChannelCTPSource,

vcnUserChannelCTPBidirectional }

};

;

ANNEX A

Entity relationship diagrams

Figure A.1 shows the inheritance hierarchy for the termination points, indirect adaptor and network element object classes of the SDH information model.

Figure A.2 shows the naming tree for the SDH information model.

Figure A.3 illustrates the naming, connectivity pointer and cross-connect relationships for the SDH information model.

Figures A.4 and A.5 are an example of how the managed objects are used to represent a SDH multiplexer and regenerator.

Figure A.1/G.774  Inheritance hierarchy

Figure A.2/G.774  Naming tree

Figure A.5/G.774  Example of information model (SDH regenerator)

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |