|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.1 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH)  Bidirectional performance monitoring for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.1 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.1 Synchronous digital hierarchy (SDH)  Bidirectional performance monitoring for the network element view

- Summary This Recommendation provides an information model for the Performance Monitoring of Synchronous Digital hierarchy (SDH) Network. This model describes the managed object classes and their properties for the performance monitoring function, as defined in ITU-T G.784 and as related to SDH Network Elements. These objects are useful to describe information exchanged across interfaces defined in ITU-T M.3010 Telecommunications Management Network (TMN) architecture for the management of the performance monitoring function.

Document history. Issue; Notes. 2001; First revision incorporated the changes documented in the G.774.1 Corrigendum 1 (1996). The rsCurrentData, rsCurrentDataTR, rsHistoryData, msAdaptationCurrentData, and msAdaptationHistoryData were moved to ITU-T G.774.6 for unidirectional performance monitoring. The current data and history data object classes for electrical SPI, optical SPI, ms, protection, and path termination TR were removed from the normative part of this Recommendation as the functionality were no longer required.. 11/1994; Initial version of the Recommendation..

- Source ITU-T Recommendation G.774.1 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and definitions		3

4	Abbreviations		3

5	Performance management model		4

5.1	Overview		4

5.2	Requirements		5

6	Managed Object Classes Definitions		5

6.1	SDH Current Data		5

6.2	Regenerator Section Current Data		6

6.3	Regenerator Section Current Data Threshold Reset		7

6.4	Path Termination Current Data		7

6.5	Multiplex Section Adaptation Current Data		7

6.6	Regenerator Section History Data		7

6.7	Path Termination History Data		8

6.8	Multiplex Section Adaptation History Data		8

7	Package definitions		8

7.1	Consecutive Severely Errored Second Current Data Package		8

7.2	Far End Consecutive Severely Errored Second Current Data Package		9

7.3	Far End Current Data Package		9

7.4	Far End History Data Package		9

7.5	History Package		10

7.6	Threshold Reset Package		10

7.7	Unavailable Second Current Data Package		10

7.8	Unavailable Second History Data Package		11

7.9	Unavailable Time Alarm Package		11

8	Attributes definitions		11

8.1	Consecutive Severely Errored Second Event		11

8.2	Errored Second		11

8.3	Far End Errored Second		12

8.4	Far End Background Block Error		12

8.5	Far End Consecutive Severely Errored Second Event		12

8.6	Number of Consecutive Severely Errored Second		12

Page

8.7	Background Block Error		12

8.8	Severely Errored Seconds		13

8.9	Far End Severely Errored Seconds		13

8.10	Unavailable Seconds		13

9	Actions		13

10	Notifications		13

11	Parameters		14

12	Name binding definitions		14

12.1	History Data – SDH Current Data		14

12.2	Path Termination Current Data – VC4 TTP SinkR1		14

12.3	Path Termination Current Data – VC3 TTP SinkR1		14

12.4	Path Termination Current Data – VC2 TTP SinkR1		15

12.5	Path Termination Current Data – VC12 TTP SinkR1		15

12.6	Path Termination Current Data – VC11 TTP SinkR1		15

13	Subordination rules		15

14	Pointer constraints		15

15	Supporting ASN.1 productions		16

Appendix I – Inheritance and naming diagram		17

Appendix II – Threshold Reset (TR) Behaviour		18

Appendix III – Managed object classes moved to ITU-T G.774.6		19

III.1	rsCurrentData		19

III.2	rsCurrentDataTR		19

III.3	rsHistoryData		19

III.4	msAdaptationCurrentData		19

III.5	msAdaptationHistoryData		19

Appendix IV – Functionality that is no longer required		19

IV.1	Managed object class definitions		19

IV.1.1	Electrical Source Synchronous Physical Interface Current Data		19

IV.1.2	Optical Source Synchronous Physical Interface Current Data		19

IV.1.3	Multiplex Section Current Data		20

IV.1.4	Multiplex Section Current Data Threshold Reset		21

IV.1.5	Protection Current Data		22

IV.1.6	Path Termination Current Data Threshold Reset		22

IV.1.7	Electrical Synchronous Physical Interface History Data		23

Page

IV.1.8	Optical Synchronous Physical Interface History Data		23

IV.1.9	Multiplex Section History Data		24

IV.1.10	Protection History Data		24

IV.2	Package definitions		25

IV.2.1	Laser Bias Current Data Package		25

IV.2.2	Laser Bias Tide Mark Package		25

IV.2.3	Laser Temperature Current Data Package		25

IV.2.4	Laser Temperature Tide Mark Package		25

IV.2.5	Transmit Power Level Current Data Package		26

IV.2.6	Transmit Power Level Tide Mark Package		26

IV.3	Attributes definitions		26

IV.3.1	Laser Bias		26

IV.3.2	Laser Bias Tide Mark Maximum		26

IV.3.3	Laser Bias Tide Mark Minimum		27

IV.3.4	Laser Temperature		27

IV.3.5	Laser Temperature Tide Mark Maximum		27

IV.3.6	Laser Temperature Tide Mark Minimum		27

IV.3.7	Protection Switch Count		27

IV.3.8	Protection Switch Duration		28

IV.3.9	Transmit Power Level		28

IV.3.10	Transmit Power Level Tide Mark Maximum		28

IV.3.11	Transmit Power Level Tide Mark Minimum		28

IV.4	Name binding definitions		28

IV.4.1	MS Current Data – MS TTP Sink		28

IV.4.2	MS Current Data Threshold Reset – MS TTP Sink		29

IV.4.3	MS Current Data – Protected TTP Sink		29

IV.4.4	MS Current Data Threshold Reset – Protected TTP Sink		29

IV.4.5	Protection Current Data – Protection Unit		29

IV.4.6	Path Termination Current Data Threshold Reset – VC4 TTP SinkR1		30

IV.4.7	Path Termination Current Data Threshold Reset – VC3 TTP SinkR1		30

IV.4.8	Path Termination Current Data Threshold Reset – VC2 TTP SinkR1		30

IV.4.9	Path Termination Current Data Threshold Reset – VC12 TTP SinkR1		30

IV.4.10	Path Termination Current Data Threshold Reset – VC11 TTP SinkR1		30

IV.4.11	Electrical Source SPI Current Data – Electrical SPITTP Source		31

IV.4.12	Optical Source SPI Current Data – Optical SPITTP Source		31

ITU-T Recommendation G.774.1

Synchronous digital hierarchy (SDH)  Bidirectional performance 
monitoring for the network element view

# 1	Scope

SDH Performance Monitoring Functions are used to monitor specified performance events of specified Termination Points managed objects and to report these performance data, as well as Quality of Service Alarms to its managing system according to a given schedule.

This Recommendation is for bidirectional performance management of SDH path only.

ITU-T M.2120 defines maintenance of transport network, ITU-T G.784 defines the management of SDH based network element. This Recommendation defines the object model based on ITU-T Q.822 according to the requirements described in ITU-T G.784 and ITU-T M.2120. This model uses generic mechanisms defined in ITU-T Q.822.

Structure of this Recommendation

Clause 5.1 provides an overview of the SDH performance monitoring information model. Clauses 6 to 15 describe the information model using the notation mechanisms defined in ITU-T X.722: Guidelines for the Definition of Managed Objects. Clause 15 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. Naming and Inheritance are illustrated in informative Appendix I. Diagrams illustrating the Threshold Reset behaviour are provided in informative Appendix II.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy (SDH).

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774 (2001), Synchronous digital hierarchy (SDH)   Management information model for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunication management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3 interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview.

	ITU-T X.710 (1997), Information technology – Open Systems Interconnection – Common management information service.

	ITU-T X.711 (1997), Information technology – Open Systems Interconnection – Common management information protocol: Specification.

	ITU-T  X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology – Open Systems Interconnection - Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997), and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology – Open Systems Interconnection – Systems management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology – Open Systems Interconnection – Systems management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology – Open Systems Interconnection – Systems management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.734 (1992), Information technology – Open Systems Interconnection – Systems management: Event report management function, plus Cor.1(1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.735 (1992), Information technology – Open Systems Interconnection – Systems management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.739 (1993), Information technology – Open Systems Interconnection – Systems Management: Metric objects and attributes.

# 3	Terms and definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, ITU-T G.784 and ITUT-T M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AIS	Alarm Indication Signal

BBE	Background Block Error

CSES	Consecutive Severely Errored Second

CTP	Connection Termination Point

EBER	Excessive Bit Error Ratio

ES	Errored Second

FEBBE	Far End Background Block Error

FEEB	Far End Errored Block

FEES	Far End Errored Second

FERF	Far End Receive Failure

FESES	Far End Severely Errored Second

ISO	International Organization for Standardization

ITU	International Telecommunications Union

LB	Laser Bias

LOF	Loss of Frame

LOS	Loss of Signal

LT	Laser Temperature

MS	Multiplex Section

NCSES	Number of Consecutive Severely Errored Second

NE	Network Element

OFS	Out of Frame Second

OOF	Out of Frame

OS	Operations System

OSI	Open Systems Interconnection

OSL	Optical Signal Level

PJC	Pointer Justification Count

PJE	Pointer Justification Event

Pkg	Package

PPI	Plesiochronous Physical Interface

PSC			Protection Switch Count

PSD	Protection Switch Duration

QS	Quality of Service

RAI	Remote Alarm Indication

RDN	Relative Distinguished Name

RS	Regenerator section

SDH	Synchronous Digital Hierarchy

SES	Severely Errored Second

SPI	Synchronous Physical Interface

STM-N	Synchronous Transport Module N

TMN	Telecommunications Management Network

TP	Termination Point

TR	Threshold Reset

TTP	Trail Termination Point

UAS	Unavailable Second

# 5	Performance management model

## 5.1	Overview

This clause provides Managed Objects required to support management of performance monitoring in SDH Network Elements.

This model defines subclasses of the generic currentData and historyData object classes from ITU-T Q.822, for each kind of monitoring point. For each type of monitoring point two subclasses of currentData are defined. One is defined for either a 15-minute count or 1-day count period, and provides implicit clearing of threshold crossing alarms at the end of each granularity period. The other subclass is only relevant to 15-minute counting and performs explicit clearing of threshold alarms (Threshold Reset) at the end of a clear 15-minute period (refer to 2.3.4.2/M.2120).

Starting and ending of unavailability period is reported by the instances of these subclasses that hold the 24-hour counts.

History information could either be collected as part of historyData instances or one of its subclasses or as an eventRecord or one of its subclasses contained in a log. Utilization of a LOG is not mandatory in this Recommendation.

The threshold reset (TR) mechanism used in this Recommendation corresponds to the reset threshold report (RTR) of ITU-T G.784.

## 5.2	Requirements

The SDH Performance Monitoring functions shall provide for:

–	the ability for a managing system to request the collection of the various Performance events relating to a given monitored entity for a given collection period;

–	the ability for a managing system to suspend/resume the performance data collection for a given monitored entity (or set of entities);

–	the ability for a managing system to instruct the NE to reset the performance monitoring counters for a given monitored entity (or set of entities);

–	the scheduling of performance collection activity within specified time periods, for a given monitored entity (or set of entities);

–	the ability for a managing system to request the performance monitoring counters for a given monitored entity (or set of entities);

–	the ability for a managed system to send event reports to a managing system to notify the results of the performance data collection, at the end of the collection period;

–	the ability for a managing system to instruct the NE to maintain Performance historical data for a specified duration, under specified conditions;

–	the ability for a managing system to instruct the NE to remove specific Performance historical data;

–	the ability for a managing system to instruct the NE to establish thresholding criteria for a given monitored entity (or set of entities);

–	the ability for a managed system to send Quality of Service Alarms upon threshold violation of a performance counter of a monitored entity.

# 6	Managed Object Classes Definitions

NOTE 1  See Appendix III for a list of the managed object classes that have been moved to ITU-T G.774.6 for unidirectional performance monitoring.

NOTE 2  See Appendix IV for the managed object classes that are no longer required.

## 6.1	SDH Current Data

sdhCurrentData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822" : currentData;

CHARACTERIZED BY

"Recommendation Q.822": zeroSuppressionPkg,

"Recommendation Q.822": thresholdPkg,

sdhCurrentDataPackage  PACKAGE

BEHAVIOUR	sdhCurrentDataBehaviour;

ATTRIBUTES

"Recommendation M.3100": currentProblemList	GET;;;

CONDITIONAL	PACKAGES

historyPackage	PRESENT IF

"an instance does not support flexible assignment of the history length",

unavailableTimeAlarmPackage PRESENT IF

"starting and ending of unavailable period has to be reported and the

granularity period is 24 hours";

REGISTERED AS {g774-01MObjectClass 1};

sdhCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"The sdhCurrentData class is used to define generic characteristic for SDH performance monitoring from which subclasses are defined in order to hold performance event counts for a specific monitoring point. Subclasses of this class are used in order to support performance monitoring of SDH paths at various layers as described in Recommendation G.803. The performance monitoring events ES, SES and BBE which are monitored by some of the subclasses of this subclass are defined in 4.1.1/G.826. The granularityPeriod attribute can only be assigned a value at creation time.

This class can only contain one reference to an instance of the thresholdData object class in the thresholdDataInstance attribute.

The 15' and 24 hours granularity period must be supported fully. If a threshold is reached or crossed then the currentProblemList attribute shall indicate it with the probable cause 'Threshold crossed'. Subclass of this class is used to monitor the near end of the path in case of bidirectional path the far end of the path shall be supported additionaly. In case of monitoring of a bidirectional path and Far End unavailability is monitored, an unavailable period starts if either the near end or the far end is in an unavailable condition. In case of monitoring of a unidirectional path only the near end is considered.

An unavailable condition starts when 10 consecutive severely errored seconds have been detected; these 10 seconds belong to the unavailable time. An unavailable condition ends when 10 consecutive seconds with no severely errored second are detected. These 10 seconds belong to the available time. The unavailable period entry/exit criteria are described in Annex A/G.826.

For threshold reset subclasses of this object class the following rules apply:

•	No more than one QOS alarm shall be generated until there has been a 15-minute rectangular fixed window with less error count than the low error count threshold and no unavailable period exists.

•	To provision the high and low threshold value, the counterThresholdAttributList attribute of the Q.822-ThresholdDataInstance is used.

In this attribute all the thresholds (high and low) for each necessary counter are stored in a list. This means for example, that the high threshold for ES and the low threshold for ES are individually stored in the same list. It is up to the network element to recognize which is the high and which is the low one.

If the unavailableTimeAlarmPackage is present and if an unavailable period starts, then a communication alarm shall be sent with a probable cause of 'Unavailable' and the presence of this unavailable condition is indicated by the currentProblemList attribute. If an unavailable period is ending, then a communication alarm shall be sent with a probable cause of 'Unavailable' and a severity of 'Cleared'. An available condition is indicated by the absence of the unavailable condition in the current problem list. The unavailable condition has no effect on the operationalState. Each subclass of this class shall define the performance attributes required to hold the mandatory or optional performance events. These performance event counts are inhibited during unavailable time. Attributes which are defined in a subclass of this class shall be included in history information using the historyData, or one of its subclass, unless it is explicitly specified in the subclass of this class that a particular attribute be not included. Each subclass of this class shall indicate which subclass of the history data is used for history retention. The following conditional packages are not used in this class: filterSuppressionPkg, observedManagedObjectPkg.

Concerning the subclasses of this class the following rule applies:

If a subclass of this class has a granularity period of 15 minutes, it should be either an instance with the threshold reset functionality or an instance without this functionality instantiated (per termination point), but not both.

";

## 6.2	Regenerator Section Current Data

This managed object class was moved to ITU-T G.774.6.

## 6.3	Regenerator Section Current Data Threshold Reset

This managed object class was moved to ITU-T G.774.6.

## 6.4	Path Termination Current Data

pathTerminationCurrentData  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentData;

CHARACTERIZED BY

pathTerminationCurrentDataPackage  PACKAGE

BEHAVIOUR	pathTerminationCurrentDataBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod REQUIRED VALUES

SDHPMASN1.SDHGranularityPeriod,

-- According to ITU-G.784 (1999), only the 24 h (one day) granularity period has to be supported.

bBE	REPLACE-WITH-DEFAULT	GET,

eS	REPLACE-WITH-DEFAULT	GET,

sES	REPLACE-WITH-DEFAULT	GET;;;

CONDITIONAL PACKAGES

cSESCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCSESCurrentDataPackage PRESENT IF

"an instance supports it" ,

uASCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCurrentDataPackage PRESENT IF

"monitoring of the far end is supported and the monitored point is Bidirectional";

REGISTERED AS {g774-01MObjectClass 9};

pathTerminationCurrentDataBehaviour  BEHAVIOUR

DEFINED AS

"Instances of the pathTerminationCurrentData managed object Class are used to hold the current register counts for a High Order Path and or Low Order Path during a collection period. An instance of this object class, for a monitored managed object instance, holds the current register counts of each performance events (BBE, ES, SES, FEBBE, FEES, FESES, UAS).

This managed object class uses the pathTerminationHistoryData managed object class for history retention.

A QOS alarm shall be sent as soon as a threshold is reached or crossed. At the end of the granularity period the QOS alarm is implictly cleared and, provided there are no other outstanding threshold crossing QOS alarms, 'Threshold crossing' removed from the currentProblemList (i.e. No Notification is Sent) and a new QOS alarm shall be sent if the threshold is reached or crossed again during the next granularity period. Only one threshold value per performance counter will be supported. ";

## 6.5	Multiplex Section Adaptation Current Data

This managed object class was moved to ITU-T G.774.6.

## 6.6	Regenerator Section History Data

This managed object class was moved to ITU-T G.774.6.

## 6.7	Path Termination History Data

pathTerminationHistoryData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822": historyData;

CHARACTERIZED BY

pathTerminationHistoryDataPackage  PACKAGE

BEHAVIOUR	pathTerminationHistoryDataBehaviour;

ATTRIBUTES

bBE	GET,

eS	GET,

sES	GET ;;;

CONDITIONAL PACKAGES

uASHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentData or contains the uASCurrentDataPackage",

farEndHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentData or pathTerminationCurrentDataTR instance contains the farEndCurrentDataPackage";

REGISTERED AS {g774-01MObjectClass 17};

pathTerminationHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"Instances of this class are used to store the observed events of a pathTerminationCurrentData or pathTerminationCurrentDataTR object at the end of an observation interval. An instance of this managed object is contained by a pathTerminationCurrentData or pathTerminationCurrentDataTR managed object instance.";

## 6.8	Multiplex Section Adaptation History Data

This managed object class was moved to ITU-T G.774.6.

# 7	Package definitions

## 7.1	Consecutive Severely Errored Second Current Data Package

cSESCurrentDataPackage  PACKAGE

BEHAVIOUR

cSESCurrentDataPackageBehaviour;

ATTRIBUTES

cSESEvent	GET,

nCSES	PERMITTED VALUES SDHPMASN1.NCSESRange	 GET-REPLACE;

REGISTERED AS {g774-01Package 1};

cSESCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to hold Consecutive Severely Errored Second (CSES) events. A CSES is detected each time x consecutive SES appear. The number of consecutive SES that generate a CSES is given by the nCSES attribute, in the range of 2 to 9. The cSES events are not detected during unavailable time. The CSES events are recorded in the cSESEvent attribute, this attribute contains the time at which the consecutive severely errored seconds started and the value of the nCSES attribute at the time the event has occured. The cSESEvent attribute shall at least support recording of 6 CSES events. When the cSESEvent attribute is full, a wrapping mechanism is used to discard the oldest CSES event. These attributes are not reset and are not stored in history data objects at the end of the granularity period. The cSESEvent attribute is initialized as an empty set when the corresponding object that holds this attribute is created. The CSES event is described in Recommendation G.784.";

## 7.2	Far End Consecutive Severely Errored Second Current Data Package

farEndCSESCurrentDataPackage  PACKAGE

BEHAVIOUR

farEndCSESCurrentDataPackageBehaviour;

ATTRIBUTES

fECSESEvent	GET,

nCSES		PERMITTED VALUES SDHPMASN1.NCSESRange	GET-REPLACE ;

REGISTERED AS {g774-01Package 2};

farEndCSESCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to hold Far End Consecutive Severely Errored Second (FECSES) events. A fECSES is detected each time x consecutive fESES appear. The number of consecutive fESES that generate a fECSES is given by the nCSES attribute, in the range of 2 to 9. The fECSES events are not detected during unavailable time. The fECSES events are recorded in the fECSESEvent attribute, this attribute contains the time at which the far end consecutive severely errored seconds started and the value of the nCSES attribute at the time the event has occured. The fECSESEvent attribute shall at least support recording of 6 fECSES events. When the fECSESEvent attribute is full, a wrapping mechanism is used to discard the oldest fECSES event . These attributes are not reset and are not stored in history data objects at the end of the granularity period. The fECSESEvent attribute is initialized as an empty set when the corresponding object that holds this attribute is created.";

## 7.3	Far End Current Data Package

farEndCurrentDataPackage  PACKAGE

BEHAVIOUR

farEndCurrentDataPackageBehaviour;

ATTRIBUTES

fEBBE	REPLACE-WITH-DEFAULT	GET,

fEES		REPLACE-WITH-DEFAULT	GET,

fESES		REPLACE-WITH-DEFAULT	GET;

REGISTERED AS {g774-01Package 3};

farEndCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to record the far end performance event counts.";

## 7.4	Far End History Data Package

farEndHistoryDataPackage  PACKAGE

BEHAVIOUR

farEndHistoryDataPackageBehaviour;

ATTRIBUTES

fEBBE	GET,

fEES		GET,

fESES		GET;

REGISTERED AS {g774-01Package 4};

farEndHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to record the corresponding current data attribute values at the end of the granularity period.";

## 7.5	History Package

historyPackage  PACKAGE

BEHAVIOUR

historyPackageBehaviour;

REGISTERED AS {g774-01Package 5};

historyPackageBehaviour  BEHAVIOUR

DEFINED AS

"At the end of each performance interval an instance of historyData or one of its subclass shall be created if history retention was not suppressed by other means such as zero suppression. The values of the historyData object or one of its subclass is the copy of the values of the corresponding attributes of the currentData or one of its subclass at the end of each performance interval. Once the new historyData instance is created or one of its subclass this instance shall be retained in the Network Element at least for 16 periods of 15 minutes for 15' performance interval and 1 period of 1 day for 1 day performance interval. The storing of history data is described in 5.3.2/G.784.";

## 7.6	Threshold Reset Package

thresholdResetPackage PACKAGE

BEHAVIOUR

thresholdResetPackageBehaviour;

REGISTERED AS {g774-01Package 14};

thresholdResetPackageBehaviour BEHAVIOUR

DEFINED AS

"The following rules apply to the thresholds mechanism:

–	ES thresholds:

Two thresholds are defined: The upper ES threshold and the low ES threshold – No more than one QOS alarm shall be generated until there has been a 15-minute rectangular fixed window with less ES than the low ES threshold and no unavailable period. At the end of the first 15-minute rectangular period with less ES than the low ES threshold and no unavailable period, if a threshold crossing has been previously generated, then a QOS alarm shall be sent which indicates the clearing of the low ES threshold, and the 'Threshold crossing' removed from the currentProblemList. If the upper ES threshold is reached or crossed, after a 15-minute rectangular fixed window with less ES than the low ES threshold, then a QOS alarm shall be sent.

–	SES threshold:

One threshold is defined – No more than one QOS alarm shall be generated until there has been a 15-minute rectangular fixed window with zero SES. At the end of the first 15-minute rectangular  period with zero SES, a QOS alarm shall be sent which indicates the clearing of a zero threshold. If after a 15-minute rectangular fixed window with zero SES the SES threshold is reached or crossed then a QOS alarm shall be sent.

–	BBE threshold: refer to the ES threshold.

For any of the above thresholds, A QOS clear will not be sent if the Performance Monitoring Data is suspect, as defined by the suspectIntervalFlag attribute. ";

## 7.7	Unavailable Second Current Data Package

uASCurrentDataPackage  PACKAGE

BEHAVIOUR

uASCurrentDataPackageBehaviour;

ATTRIBUTES

uAS REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-01Package 15};

uASCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the counter of one second intervals pertaining to an Unavailable Time.";

## 7.8	Unavailable Second History Data Package

uASHistoryDataPackage  PACKAGE

BEHAVIOUR

uASHistoryDataPackageBehaviour;

ATTRIBUTES

uAS	GET;

REGISTERED AS {g774-01Package 16};

uASHistoryDataPackageBehaviour  BEHAVIOUR

DEFINED AS

"This package is used to record the corresponding current data attribute values at the end of the granularity period.";

## 7.9	Unavailable Time Alarm Package

unavailableTimeAlarmPackage  PACKAGE

BEHAVIOUR

unavailableTimeAlarmPackageBehaviour;

NOTIFICATIONS

"Recommendation X.721 ": communicationsAlarm;

REGISTERED AS {g774-01Package 17};

unavailableTimeAlarmPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used when a Communication Alarm Notification with the probable cause. Unavailable is to be emitted to indicate the beginning of an unavailable time period. The end of an unavailable time period shall be indicated by the clearing of this alarm.";

# 8	Attributes definitions

## 8.1	Consecutive Severely Errored Second Event

cSESEvent  ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDHPMASN1.CSES;

BEHAVIOUR

cSESEventBehaviour BEHAVIOUR

DEFINED AS

"The value of the cSESEvent attribute represents the recording of at least 6 cSES events. A cSES event is generated each time x consecutive SES appear during the available time of the monitored resource."

;;

REGISTERED AS {g774-01Attribute 1};

## 8.2	Errored Second

eS  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

eSBeh BEHAVIOUR

DEFINED AS

"The value of the eS attribute represents the count of seconds with one or more errored blocks during the available time of the monitored resource during the corresponding granularity period. The ES performance event is described in 3.1.1/G.826."

;;

REGISTERED AS {g774-01Attribute 2};

## 8.3	Far End Errored Second

fEES  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

ffEESBeh BEHAVIOUR

DEFINED AS

"The value of the fEES  attribute represents the count of seconds with one or more far end errored blocks detected at the remote terminal during the available time of the monitored resource during the corresponding granularity period."

;;

REGISTERED AS {g774-01Attribute 3};

## 8.4	Far End Background Block Error

fEBBE  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

fEBBEBeh	BEHAVIOUR

DEFINED AS

"The value of the fEBBE attribute represents the count of errored blocks (Estimate Errored Block on Bip-n violation) not occuring as part of an fESES."

;;

REGISTERED AS {g774-01Attribute 4};

## 8.5	Far End Consecutive Severely Errored Second Event

fECSESEvent  ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDHPMASN1.CSES;

BEHAVIOUR

fECSESEventBehaviour BEHAVIOUR

DEFINED AS

"The value of the fECSESEvent attribute represents the recording of at least 6 fECSES events. A fECSES event is generated each time x consecutive fESES appear during the available time of the monitored resource during the corresponding granularity period."

;;

REGISTERED AS {g774-01Attribute 5};

## 8.6	Number of Consecutive Severely Errored Second

nCSES  ATTRIBUTE

WITH ATTRIBUTE SYNTAX  SDHPMASN1.NCSESRange;

BEHAVIOUR

nCSESBeh BEHAVIOUR

DEFINED AS

"The value of the nCSES attribute represents the number of consecutive (near or far end) SES which will lead to the generation of a (near or far end) cSES event. The nCSES is in the range 2 to 9."

;;

REGISTERED AS {g774-01Attribute 12};

## 8.7	Background Block Error

bBE  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

bBEBeh BEHAVIOUR

DEFINED AS

"The value of the BBE attribute represents the count of errored blocks (Estimate Errored Block on Bip-n violation) not occuring as part of an SES. The BBE performance event is described in 3.1.1/G.826."

;;

REGISTERED AS {g774-01Attribute 13};

## 8.8	Severely Errored Seconds

sES  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

sESBeh BEHAVIOUR

DEFINED AS

"The value of the sES attribute represents the count of one second periods containing greater than or equal to 30% of errored blocks, or at least one Severely Disturbed Period (SDP) that is one second containing one or more defects during the available time of the monitored resource during the corresponding granularity period. An SES is also counted as an ES. The SES performance event is described in Recommendation G.826."

;;

REGISTERED AS {g774-01Attribute 17};

## 8.9	Far End Severely Errored Seconds

fESES  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

fESESBeh BEHAVIOUR

DEFINED AS

"The value of the fESES attribute represents the count of one second periods containing greater than or equal to 30% of far end errored blocks as detected at the remote terminal (fEBC), or at least one Far End Severely Disturbed Period (SDP) that is one second containing one or more far end defects as detected at the remote terminal (FERF) during the available time of the monitored resource during the corresponding granularity period. An fESES is also counted as an fEES."

;;

REGISTERED AS {g774-01Attribute 18};

## 8.10	Unavailable Seconds

uAS  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

uASBeh BEHAVIOUR

DEFINED AS

"The value of the uAS attribute represents the count of one second intervals pertaining to an UnavailableTime. A period of unavailable time begins when the SES continues for a period of ten consecutive seconds. These seconds are considered to be unavailable time. A new period of available time begins with the first second of ten consecutive non-SES. The unavailable time entry/exit criteria is described in Annex A/G.826."

;;

REGISTERED AS {g774-01Attribute 22};

# 9	Actions

None.

# 10	Notifications

None.

# 11	Parameters

None.

# 12	Name binding definitions

## 12.1	History Data – SDH Current Data

historyData-sdhCurrentData  NAME BINDING

SUBORDINATE OBJECT CLASS  "Recommendation Q.822": historyData AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS sdhCurrentData AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation Q.822": historyDataId;

BEHAVIOUR

historyData-sdhCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"Instance of the historyData object class or one of its subclass is created at the end of the granularity period of an instance of the sdhCurrentData object or one of its subclass and is directly contained by that instance."

;;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 1};

## Path Termination Current Data – VC4 TTP SinkR1

pathTerminationCurrentData-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc4TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 23};

# 12.3	Path Termination Current Data – VC3 TTP SinkR1

pathTerminationCurrentData-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc3TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 24};

## 12.4	Path Termination Current Data – VC2 TTP SinkR1

pathTerminationCurrentData-vc2TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc2TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 25};

## 12.5	Path Termination Current Data – VC12 TTP SinkR1

pathTerminationCurrentData-vc12TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc12TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 26};

## 12.6	Path Termination Current Data – VC11 TTP SinkR1

pathTerminationCurrentData-vc11TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentData AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc11TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 27};

# 13	Subordination rules

None.

# 14	Pointer constraints

None.

# 15	Supporting ASN.1 productions

SDHPMASN1 { itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) pm(01) informationModel(0)

asn1Module(2) sdhpm (0) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

--  EXPORTS everything

IMPORTS

ProbableCause FROM Attribute-ASN1Module { joint-iso-itu-t ms(9) smi(3) part2(2) asn1Module(2) 1}

TimePeriod FROM MetricModule {joint-iso-itu-t ms(9) function(2) part11(11) asn1Module(2) 0};

sdhPM OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) pm(01) informationModel(0) }

g774-01MObjectClass OBJECT IDENTIFIER ::= {sdhPM managedObjectClass(3)}

g774-01Attribute OBJECT IDENTIFIER ::= {sdhPM attribute(7)}

g774-01NameBinding OBJECT IDENTIFIER ::= {sdhPM nameBinding(6)}

g774-01Package OBJECT IDENTIFIER ::= {sdhPM package(4)}

Integer ::= INTEGER

CSES ::= SET OF SEQUENCE {

eventTime	GeneralizedTime,

nCSES	NCSESRange

}

NCSESRange ::= Integer(2 .. 9)

SDHGranularityPeriod ::= TimePeriod (WITH COMPONENTS {minutes (15), days(1)})

SDHPVGranularityPeriod ::= TimePeriod (WITH COMPONENTS {minutes (15)})

END  --  end of SDHPMASN1

Appendix I

Inheritance and naming diagram

See Figures I.1 and I.2.

Figure I.1/G.774.1  Object name binding for bidirectional performance 
management fragment

Figure I.2/G.774.1  Inheritance tree for bidirectional performance management fragment

Appendix II

Threshold Reset (TR) Behaviour

Figure II.1 illustrates the Threshold Reset (TR) behaviour for three possible scenarios.

Figure II.1/G.774.1  Threshold Reset (TR) behaviour

Appendix III

Managed object classes moved to ITU-T G.774.6

The following managed object classes have been moved to ITU-T G.774.6 for unidirectional performance monitoring.

## III.1	rsCurrentData

## III.2	rsCurrentDataTR

## III.3	rsHistoryData

## III.4	msAdaptationCurrentData

## III.5	msAdaptationHistoryData

Appendix IV

Functionality that is no longer required

## IV.1	Managed object class definitions

### IV.1.1	Electrical Source Synchronous Physical Interface Current Data

electricalSourceSPICurrentData  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentData;

CHARACTERIZED BY

transmitPowerLevelCurrentDataPackage,

electricalSourceSPICurrentDataPackage  PACKAGE

BEHAVIOUR	electricalSourceSPICurrentDataBehaviour;;;

CONDITIONAL PACKAGES

transmitPowerLevelTideMarkPackage PRESENT IF

"an instance supports it";

REGISTERED AS {g774-01MObjectClass 4};

electricalSourceSPICurrentDataBehaviour BEHAVIOUR

DEFINED AS

"Instances of the electricalSPICurrentData managed object Class are used to hold the monitoring of the physical characteristics of the output of an electrical source (electricalSPITTP). These characteristics are handled by gauge attributes. The following performance event is defined: transmit Power Level.

When a Tide Mark Package is used ( in conjunction with its associated gauge), the corresponding Tide Mark attribute shall be reported in the history data at the end of the granularity period and the values of the tide marks shall be reset to the current values of the associated gauge at the end of the granularity period. The unavailableTimeAlarmPackage is not used in this class. This managed object class uses the electricalSPIHistoryData managed object class for history retention.";

### IV.1.2	Optical Source Synchronous Physical Interface Current Data

opticalSourceSPICurrentData  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentData;

CHARACTERIZED BY

opticalSourceSPICurrentDataPackage  PACKAGE

BEHAVIOUR	opticalSourceSPICurrentDataBehaviour;;;

CONDITIONAL PACKAGES

transmitPowerLevelCurrentDataPackage PRESENT IF

"an instance supports it",

transmitPowerLevelTideMarkPackage PRESENT IF

"an instance supports it and transmitPowerLevelCurrentDataPackage is present",

laserBiasCurrentDataPackage PRESENT IF

"an instance supports it",

laserBiasTideMarkPackage PRESENT IF

"an instance supports it and laserBiasCurrentDataPackage is present",

laserTemperatureCurrentDataPackage PRESENT IF

"an instance supports it",

laserTemperatureTideMarkPackage PRESENT IF

"an instance supports it and laserTemperatureCurrentDataPackage is present";

REGISTERED AS {g774-01MObjectClass 5};

opticalSourceSPICurrentDataBehaviour BEHAVIOUR

DEFINED AS

"Instances of the opticalSPICurrentData managed object Class are used to hold the monitoring of the physical characteristics of the output of an optical source (opticalSPITTP). These characteristics are handled by gauge attributes. The following performance primitives are observed:

OSL	Optical Signal Level

For the OSL performance primitive, the following performance events are defined:

transmit Power Level

LB	Laser Bias

For the LB performance primitive, the following performance events are defined:

laser Bias

LT	Laser Temperature

For the LT performance primitive, the following performance events are defined:

laserTemperature

These performance event counts are inhibited under certain failure or unavailable conditions as specified in the following list:

Laser  Shutdown

When a Tide Mark Package is used (in conjunction with its associated gauge), the corresponding Tide Mark attribute shall be reported in the history data at the end of the granularity period and the tide marks shall be reset to the current values of the associated gauge at the end of the granularity period. The unavailableTimeAlarmPackage is not used in this class. This managed object class uses the opticalSPIHistoryData managed object class for history retention.";

### IV.1.3	Multiplex Section Current Data

msCurrentData  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentData;

CHARACTERIZED BY

msCurrentDataPackage PACKAGE

BEHAVIOUR	msCurrentDataBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod REQUIRED VALUES

SDHPMASN1.SDHGranularityPeriod,

bBE	REPLACE-WITH-DEFAULT	GET,

eS	REPLACE-WITH-DEFAULT	GET,

sES	REPLACE-WITH-DEFAULT	GET;;;

CONDITIONAL PACKAGES

cSESCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCSESCurrentDataPackage PRESENT IF

"an instance supports it",

uASCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCurrentDataPackage PRESENT IF

"monitoring of the far end is supported and the monitored point is bidirectional.";

REGISTERED AS {g774-01MObjectClass 6};

msCurrentDataBehaviour BEHAVIOUR

DEFINED AS

*Instances of the msCurrentData managed object Class are used to hold the current register counts for a multiplex section trail termination point during a collection period. The following performance primitives are observed:

EB	Errored Block

For the EB performance primitive, the following performance events are defined:

BBE	Background Block Error

For the EB performance primitive and the following defect: MS-AIS, Excessive-Error, the following performance events are defined:

ES	Errored Second

SES	Severely Errored Second

This managed object class uses the msHistoryData managed object class for history retention.

A QOS alarm shall be sent as soon as a threshold is reached or crossed. At the end of the granularity period the QOS alarm is implictly cleared, providing there are no other outstanding threshold crossing QOS alarms, "Threshold crossing" removed from the currentProblemList (i.e. No Notification is Sent) and a new QOS alarm shall be sent if the threshold is reached or crossed again during the next granularity period. Only one threshold value per performance counter will be supported.*;

### IV.1.4	Multiplex Section Current Data Threshold Reset

msCurrentDataTR  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentData;

CHARACTERIZED BY

thresholdReset Package,

msCurrentDataTRPackage PACKAGE

BEHAVIOUR

msCurrentDataTRBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod PERMITTED VALUES

SDHPMASN1.SDHPVGranularityPeriod,

bBE	REPLACE-WITH-DEFAULT	GET,

eS	REPLACE-WITH-DEFAULT	GET,

sES	REPLACE-WITH-DEFAULT	GET;;;

CONDITIONAL PACKAGES

cSESCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCSESCurrentDataPackage PRESENT IF

"an instance supports it",

uASCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCurrentDataPackage PRESENT IF

"monitoring of the far end is supported and the monitored point is

bidirectional.";

REGISTERED AS {g774-01MObjectClass 7};

msCurrentDataTRBehaviour BEHAVIOUR

DEFINED AS

"Instances of the msCurrentDataTR managed object Class are used to hold the current register counts for a multiplex section trail termination point during a collection period.

The following performance primitives are observed:

EB	Errored Block

For the EB performance primitive, the following performance events are defined:

BBE	Background Block Error

For the EB performance primitive and the following defect: MS-AIS, Excessive-Error, the following performance events are defined:

ES	Errored Second

SES	Severely Errored Second

This managed object class uses the msHistoryData managed object class for history retention.";

### IV.1.5	Protection Current Data

NOTE – The use of this class is not meaningful for 1 + 1 non-revertive protection.

protectionCurrentData  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentData;

CHARACTERIZED BY

protectionCurrentDataPackage PACKAGE

BEHAVIOUR

protectionCurrentDataBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod REQUIRED VALUES

SDHPMASN1.SDHGranularityPeriod,

pSC	REPLACE-WITH-DEFAULT	GET,

pSD	REPLACE-WITH-DEFAULT	GET;;;

REGISTERED AS {g774-01MObjectClass 8};

protectionCurrentDataBehaviour BEHAVIOUR

DEFINED AS

"Instances of the protectionCurrentData managed object Class are used to hold the current register counts for a protection during a collection period. The following performance primitives are observed:

PSC	Protection Switch Count.

PSD	Protection Switch Duration.

This managed object class uses the protectionHistoryData managed object class for history retention.";

### IV.1.6	Path Termination Current Data Threshold Reset

pathTerminationCurrentDataTR  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentData;

CHARACTERIZED BY

thresholdResetPackage,

pathTerminationCurrentDataTRPackage  PACKAGE

BEHAVIOUR	pathTerminationCurrentDataTRBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod PERMITTED VALUES

SDHPMASN1.SDHPVGranularityPeriod,

bBE	REPLACE-WITH-DEFAULT	GET,

eS	REPLACE-WITH-DEFAULT	GET,

sES	REPLACE-WITH-DEFAULT	GET;;;

CONDITIONAL PACKAGES

cSESCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCSESCurrentDataPackage PRESENT IF

"an instance supports it",

uASCurrentDataPackage PRESENT IF

"an instance supports it",

farEndCurrentDataPackage PRESENT IF

"if monitoring of the far end is supported and the monitored point is Bidirectional";

REGISTERED AS {g774-01MObjectClass 10};

pathTerminationCurrentDataTRBehaviour BEHAVIOUR

DEFINED AS

"Instances of the pathTerminationCurrentDataTR managed object Class are used to hold the current register counts for a High Order Path and or Low Order Path during a collection period. An instance of this object class, for a monitored managed object instance, holds the current register counts of each performance events (BBE, ES, SES, FEBBE , FEES, FESES, UAS).

Near End Monitoring 
The following performance primitives are observed:

EB	Errored Block

For the EB performance primitive, the following performance events are defined:

BBE	Background  Block Error

For the EB performance primitive and the following defect: AU-AIS/TU-AIS, Path Trace Mismatch, Signal Label Mismatch, and Loss of TU Multiframe the following performance events are defined:

ES	Errored Second

SES	Severely Errored Second

Far End Monitoring

The following performance primitive are observed:

FEEB	Far End Errored Block

For the FEEB performance primitive, the following performance events are defined:

FEBBE	Far End Background Block Error

For the FEEB performance primitive and the following defect: Far End Remote Failure the following performance events are defined:

FEES	Far End Errored Second

FESES	Far End Severely Errored Second

This managed object class uses the pathTerminationHistoryData managed object class for history retention.";

### IV.1.7	Electrical Synchronous Physical Interface History Data

electricalSPIHistoryData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822": historyData;

CHARACTERIZED BY

transmitPowerLevelCurrentDataPackage,

electricalSPIHistoryDataPackage  PACKAGE

BEHAVIOUR	opticalSPIHistoryDataBehaviour;;;

CONDITIONAL PACKAGES

transmitPowerLevelTideMarkPackage PRESENT IF

"the containing electricalSPICurrentData instance contains this package";

REGISTERED AS {g774-01MObjectClass 13;

electricalSPIHistoryDataBehaviour  BEHAVIOUR

DEFINED AS

"Instances of this class are used to store the observed events of an electricalSourceSPICurrentData object at the end of an observation interval. An instance of this managed object is contained by an electricalSourceSPICurrentData managed object instance.";

### IV.1.8	Optical Synchronous Physical Interface History Data

opticalSPIHistoryData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822": historyData;

CHARACTERIZED BY

opticalSPIHistoryDataPackage  PACKAGE

BEHAVIOUR	opticalSPIHistoryDataBehaviour;;;

CONDITIONAL PACKAGES

transmitPowerLevelCurrentDataPackage PRESENT IF

"the containing opticalSourceSPICurrentData instance contains this package",

transmitPowerLevelTideMarkPackage PRESENT IF

"the containing opticalSourceSPICurrentData instance contains this package",

laserBiasCurrentDataPackage PRESENT IF

"the containing opticalSourceSPICurrentData instance contains this package",

laserBiasTideMarkPackage PRESENT IF

"the containing opticalSourceSPICurrentData instance contains this package",

laserTemperatureCurrentDataPackage PRESENT IF

"the containing opticalSourceSPICurrentData instance contains this package",

laserTemperatureTideMarkPackage PRESENT IF

"the containing opticalSourceSPICurrentData instance contains this package";

REGISTERED AS {g774-01MObjectClass 14};

opticalSPIHistoryDataBehaviour  BEHAVIOUR

DEFINED AS

"Instances of this class are used to store the observed events of an opticalSourceSPICurrentData object at the end of an observation interval. An instance of this managed object is contained by an opticalSourceSPICurrentData managed object instance.";

### IV.1.9	Multiplex Section History Data

msHistoryData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822": historyData;

CHARACTERIZED BY

msHistoryDataPackage PACKAGE

BEHAVIOUR

msHistoryDataBehaviour;

ATTRIBUTES

bBE	GET,

eS	GET,

sES	GET;;;

CONDITIONAL PACKAGES

uASHistoryDataPackage PRESENT IF

"the containing msCurrentData contains the uASCurrentDataPackage",

farEndHistoryDataPackage PRESENT IF

"the containing msCurrentData or msCurrentDataTR instance contains the farEndCurrentDataPackage";

REGISTERED AS {g774-01MObjectClass 15};

msHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"Instances of this class are used to store the observed events of an msCurrentData or msCurrentDataTR object at the end of an observation interval. An instance of this managed object is contained by an msCurrentData or msCurrentDataTR managed object instance.";

### IV.1.10	Protection History Data

protectionHistoryData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822": historyData;

CHARACTERIZED BY

protectionHistoryDataPackage  PACKAGE

BEHAVIOUR	protectionHistoryDataBehaviour;

ATTRIBUTES

pSC	GET,

pSD	GET;;;

REGISTERED AS {g774-01MObjectClass 16};

protectionHistoryDataBehaviour BEHAVIOUR

DEFINED AS

"Instances of this class are used to store the observed events of a protectionCurrentData object at the end of an observation interval. An instance of this managed object is contained by a protectionCurrentData or subclass managed object instance.";

## IV.2	Package definitions

### IV.2.1	Laser Bias Current Data Package

laserBiasCurrentDataPackage  PACKAGE

BEHAVIOUR

laserBiasCurrentDataPackageBehaviour;

ATTRIBUTES

laserBias	GET;

REGISTERED AS {g774-01Package 6};

laserBiasCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the gauge of percentage of laser bias of an SDH optical source.";

### IV.2.2	Laser Bias Tide Mark Package

laserBiasTideMarkPackage PACKAGE

BEHAVIOUR

laserBiasTideMarkPackageBehaviour;

ATTRIBUTES

laserBiasTideMarkMax	GET,

laserBiasTideMarkMin	GET;

REGISTERED AS {g774-01Package 7};

laserBiasTideMarkPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the minimum and maximum values reached by the laser bias gauge during an observation period.";

### IV.2.3	Laser Temperature Current Data Package

laserTemperatureCurrentDataPackage  PACKAGE

BEHAVIOUR

laserTemperatureCurrentDataPackageBehaviour;

ATTRIBUTES

laserTemperature	GET;

REGISTERED AS {g774-01Package 8};

laserTemperatureCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the gauge of laser temperature value of an SDH optical source.";

### IV.2.4	Laser Temperature Tide Mark Package

laserTemperatureTideMarkPackage PACKAGE

BEHAVIOUR

laserTemperatureTideMarkPackageBehaviour;

ATTRIBUTES

laserTemperatureTideMarkMax	GET,

laserTemperatureTideMarkMin	GET;

REGISTERED AS {g774-01Package 9};

laserTemperatureTideMarkPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the minimum and maximum values reached by the laser temperature gauge during an observation period.";

### IV.2.5	Transmit Power Level Current Data Package

transmitPowerLevelCurrentDataPackage  PACKAGE

BEHAVIOUR

transmitPowerLevelCurrentDataPackageBehaviour;

ATTRIBUTES

transmitPowerLevel	GET;

REGISTERED AS {g774-01Package 12};

transmitPowerLevelCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the gauge of transmit power level value of a physical source.";

### IV.2.6	Transmit Power Level Tide Mark Package

transmitPowerLevelTideMarkPackage  PACKAGE

BEHAVIOUR

transmitPowerLevelTideMarkPackageBehaviour;

ATTRIBUTES

transmitPowerLevelTideMarkMax	GET,

transmitPowerLevelTideMarkMin	GET;

REGISTERED AS {g774-01Package 13};

transmitPowerLevelTideMarkPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the minimum and maximum values reached by the transmit power level gauge during an observation period.";

## IV.3	Attributes definitions

### IV.3.1	Laser Bias

laserBias  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": gauge;

BEHAVIOUR

laserBiasBeh BEHAVIOUR

DEFINED AS

"The value of the laserBias attribute represents the percentage of the normalized value of laser bias current at a SDH optical SPI source or a SDH optical SPI bidirectional trail termination point."

;;

REGISTERED AS {g774-01Attribute 6};

### IV.3.2	Laser Bias Tide Mark Maximum

laserBiasTideMarkMax  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": tideMark;

BEHAVIOUR

laserBiasTideMarkMaxBeh BEHAVIOUR

DEFINED AS

"The laserBias TideMark Maximum attribute stores the maximum value reached by the laser bias during a granularity period."

;;

REGISTERED AS {g774-01Attribute 7};

### IV.3.3	Laser Bias Tide Mark Minimum

laserBiasTideMarkMin  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": tideMark;

BEHAVIOUR

laserBiasTideMarkMinBeh BEHAVIOUR

DEFINED AS

"The laserBias TideMark Minimum attribute stores the minimum value reached by the laser bias during a granularity period."

;;

REGISTERED AS {g774-01Attribute 8};

### IV.3.4	Laser Temperature

laserTemperature  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": gauge;

BEHAVIOUR

laserTemperatureBeh BEHAVIOUR

DEFINED AS

"The value of the laserTemperature attribute represents the laser temperature of a physical optical source."

;;

REGISTERED AS {g774-01Attribute 9};

### IV.3.5	Laser Temperature Tide Mark Maximum

laserTemperatureTideMarkMax  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": tideMark;

BEHAVIOUR

laserTemperatureTideMarkMaxBeh BEHAVIOUR

DEFINED AS

"The laserTemperature TideMark Maximum attribute stores the maximum value reached by the laser temperature during a granularity period."

;;

REGISTERED AS {g774-01Attribute 10};

### IV.3.6	Laser Temperature Tide Mark Minimum

laserTemperatureTideMarkMin  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": tideMark;

BEHAVIOUR

laserTemperatureTideMarkMinBeh BEHAVIOUR

DEFINED AS

"The laserTemperature TideMark Minimum attribute stores the minimum value reached by the laser temperature during a granularity period."

;;

REGISTERED AS {g774-01Attribute 11};

### IV.3.7	Protection Switch Count

pSC  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

pSCBeh BEHAVIOUR

DEFINED AS

"In the case of a protected unit, the value of the pSC attribute represents the count of switches to the protecting unit. In the case of a protecting unit, this attribute represents the count of switches from any protected unit to that protecting unit. Editor Note: This behaviour needs clarification in accordance with protection management."

;;

REGISTERED AS {g774-01Attribute 15};

### IV.3.8	Protection Switch Duration

pSD  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

pSDBeh BEHAVIOUR

DEFINED AS

"The value of the pSD attribute represents the count of seconds during which the service was switched from working to protection."

;;

REGISTERED AS {g774-01Attribute 16};

### IV.3.9	Transmit Power Level

transmitPowerLevel  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": gauge;

BEHAVIOUR

transmitPowerLevelBeh BEHAVIOUR

DEFINED AS

"The value of the transmitPowerLevel gauge attribute represents the value of the output signal level emitted by a physical (electrical or optical) source."

;;

REGISTERED AS {g774-01Attribute 19};

### IV.3.10	Transmit Power Level Tide Mark Maximum

transmitPowerLevelTideMarkMax  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": tideMark;

BEHAVIOUR

transmitPowerLevelTideMarkMaxBeh BEHAVIOUR

DEFINED AS

"The transmitPowerLevelTideMark Maximum attribute stores the maximum value reached by the transmit power level during an observation period."

;;

REGISTERED AS {g774-01Attribute 20};

### IV.3.11	Transmit Power Level Tide Mark Minimum

transmitPowerLevelTideMarkMin  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": tideMark;

BEHAVIOUR

transmitPowerLevelTideMarkMinBeh BEHAVIOUR

DEFINED AS

"The transmitPowerLevelTideMark Minimum attribute stores the minimum value reached by the transmit power level during an observation period."

;;

REGISTERED AS {g774-01Attribute 21};

## IV.4	Name binding definitions

### IV.4.1	MS Current Data – MS TTP Sink

msCurrentData-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS  msCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": msTTPSink AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 2};

### IV.4.2	MS Current Data Threshold Reset – MS TTP Sink

msCurrentDataTR-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS  msCurrentDataTR  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774": msTTPSink AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 3};

### IV.4.3	MS Current Data – Protected TTP Sink

msCurrentData-protectedTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS  msCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774.03": protectedTTPSink AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 4};

### IV.4.4	MS Current Data Threshold Reset – Protected TTP Sink

msCurrentDataTR-protectedTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS  msCurrentDataTR  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774.03": protectedTTPSink AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 5};

### IV.4.5	Protection Current Data – Protection Unit

protectionCurrentData-protectionUnit  NAME BINDING

SUBORDINATE OBJECT CLASS  protectionCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774.03": protectionUnit AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 6};

### IV.4.6	Path Termination Current Data Threshold Reset – VC4 TTP SinkR1

pathTerminationCurrentDataTR-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataTR  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc4TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 28};

### IV.4.7	Path Termination Current Data Threshold Reset – VC3 TTP SinkR1

pathTerminationCurrentDataTR-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataTR  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc3TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 29};

### IV.4.8	Path Termination Current Data Threshold Reset – VC2 TTP SinkR1

pathTerminationCurrentDataTR-vc2TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataTR  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc2TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 30};

### IV.4.9	Path Termination Current Data Threshold Reset – VC12 TTP SinkR1

pathTerminationCurrentDataTR-vc12TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataTR  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc12TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 31};

### IV.4.10	Path Termination Current Data Threshold Reset – VC11 TTP SinkR1

pathTerminationCurrentDataTR-vc11TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": vc11TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 32};

### IV.4.11	Electrical Source SPI Current Data – Electrical SPITTP Source

electricalSourceSPICurrentData-electricalSPITTPSource NAME BINDING

SUBORDINATE OBJECT CLASS  electricalSourceSPICurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": electricalSPITTPSource AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 19};

### IV.4.12	Optical Source SPI Current Data – Optical SPITTP Source

opticalSourceSPICurrentData-opticalSPITTPSource NAME BINDING

SUBORDINATE OBJECT CLASS  opticalSourceSPICurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": opticalSPITTPSource AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 20};

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |