|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.2 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH) – Configuration of the payload structure for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.2 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.2 Synchronous digital hierarchy (SDH) – Configuration of the payload structure for the network element view

- Summary This Recommendation provides an information model for the Payload Configuration Management of Synchronous Digital Hierarchy (SDH) Networks. This model describes the managed object classes and their properties for the Payload Configuration function as related to SDH Network Elements. These objects are useful to describe information exchanged across interfaces defined in ITU-T M.3010 Telecommunications Management Network (TMN) architecture for the management of the Payload Configuration function.

Document history. Issue; Notes. 2001; First revision incorporated the changes documented in the G.774.2 Corrigendum 1 (1996).. 11/1994; Initial version of the Recommendation..

- Source ITU-T Recommendation G.774.2 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and definitions		2

4	Abbreviations		3

5	Payload configuration information model		3

5.1	Overview		3

5.2	Requirements		4

6	Object classes		5

6.1	Indirect Adaptors		5

6.2	High Order Path Layer		7

6.3	Low Order Path Layer		9

7	Packages		11

8	Attributes		11

9	Actions		11

9.1	Define AUG Structure		11

9.2	Define VC4 Structure		12

9.3	Define VC3 Structure		13

9.4	Define Tug3 Structure		13

9.5	Define Tug2 Structure		14

9.6	Definition of the Client type		14

10	Notifications		15

11	Parameters		15

12	Name Bindings		15

13	Constraint rules		30

13.1	Constraint rules extended syntax		30

13.1.1	Constraint rules grammar		30

13.1.2	Constraint rule templates		31

13.2	Connectivity pointer constraints		31

14	Subordination rules		42

15	Supporting ASN.1 productions		51

Appendix I – Inheritance and Naming Diagrams		52

ITU-T Recommendation G.774.2

Synchronous digital hierarchy (SDH) – Configuration of the payload 
structure for the network element view

# 1	Scope

SDH Payload Configuration Functions are used to configure the various SDH adaptation functions.

Modification of the SDH payload structure is done by applying an action on relevant managed objects. These actions are included by subclassing of existing G.774 managed object classes.

The new objects defined in this Recommendation supersede those defined in ITU-T G.774.2 (1994).

Structure of this Recommendation

Clause 5.1 provides an overview of the SDH Payload Configuration information model. Clauses 6 to 12 describe the information model using the notation mechanisms defined in ITU-T X.722 Guidelines for the Definition of Managed Objects. Clauses 13 and 14 describe the connectivity pointer constraint rules and the subordination rules applicable to the termination point fragment. Clause 15 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. Naming and Inheritance are illustrated in informative Appendix I.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy (SDH).

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774 (2001), Synchronous digital hierarchy (SDH)  Management information model for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunications management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview.

	ITU-T X.710 (1997), Information technology – Open Systems Interconnection – Common management information service.

	ITU-T X.711 (1997), Information technology – Open Systems Interconnection – Common management information protocol: Specification.

	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology – Open Systems Interconnection  Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology – Open Systems Interconnection – Systems Management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology – Open Systems Interconnection – Systems Management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology – Open Systems Interconnection – Systems Management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.734 (1992), Information technology – Open Systems Interconnection – Systems Management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.735 (1992), Information technology – Open Systems Interconnection – Systems Management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

# 3	Terms and definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, G.784 and M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AU	Administrative Unit

AUG	Administrative Unit Group

Bid	Bidirectional

CTP	Connection Termination Point

GTP	Group Termination Point

Id	Identifier

MS	Multiplex Section

NE	Network Element

OS	Operations System

OSI	Open Systems Interconnection

PDH	Plesiochronous Digital Hierarchy

RS	Regenerator Section

SDH	Synchronous Digital Hierarchy

SPI	Synchronous Physical Interface

TMN	Telecommunications Management Network

TP	Termination Point

TTP	Trail Termination Point

TU	Tributary Unit

TUG	Tributary Unit Group

VC-n	Virtual Container n

# 5	Payload configuration information model

## 5.1	Overview

Modification of the SDH frame structure is done by applying an action on relevant managed objects. These actions are included by subclassing of G.774 managed object classes. Actions are applied on the following classes:

New Managed Object Classes	Action used

modifiableAugSink	defineAUGStructure

modifiableAugSource	defineAUGStructure

modifiableAugBidirectional	defineAUGStructure

modifiableTug3Sink	defineTug3Structure

modifiableTug3Source	defineTug3Structure

modifiableTug3Bidirectional	defineTug3Structure

modifiableTug2Sink	defineTug2Structure

modifiableTug2Source	defineTug2Structure

modifiableTug2Bidirectional	defineTug2Structure

modifiableVC4TTPSinkR1	defineVC4Structure

modifiableVC4TTPSourceR1	defineVC4Structure

modifiableVC4TTPBidirectionalR1	defineVC4Structure

modifiableVC3TTPSinkR1	defineVC3Structure

modifiableVC3TTPSourceR1	defineVC3Structure

modifiableVC3TTPBidirectionalR1	defineVC3Structure

modifiableVC2TTPSinkR1	defineClientType

modifiableVC2TTPSource	defineClientType

modifiableVC2TTPBidirectionalR1	defineClientType

modifiableVC12TTPSinkR1	defineClientType

modifiableVC12TTPSource	defineClientType

modifiableVC12TTPBidirectionalR1	defineClientType

modifiableVC11TTPSinkR1	defineClientType

modifiableVC11TTPSource	defineClientType

modifiableVC11TTPBidirectionalR1	defineClientType

Different actions are defined depending on the class of the managed object on which the action is applied.

The behaviour related to the action is specified along with the action definition.

For object classes defined in this Recommendation, the specialization is made from object classes defined in ITU-T G.774 according to the following scheme:

–	Sink from Sink;

–	Source from Source;

–	Bidirectional from Bidirectional.

The supportableClientList attribute is used to contain a list of managed object classes. Only instances of classes present in the list may be contained by an instance of the class which contains the supportableClientList attribute.

In a case where a TP or indirectAdaptor could only contain one type of client, the managed object classes from ITU-T G.774 should be used.

## 5.2	Requirements

The choice between instantiation of modifiable or non-modifiable indirect adaptor object classes is made according to the make-up and mode of operation of the network element.

The choice between instantiation of modifiable or non-modifiable trail termination point object classes is made according to the make-up and mode of operation of the network element or by direct OS operations.

All the structure below any Trail Termination Point (i.e. vc4TTP, vc3TTP, ...) is completely configured once they are created according to a predefined default configuration.

Reconfiguration of the multiplexing structure should be supported (i.e. changing a tug3 from a not multiplexed structure into 7 tug2).

Configuration or reconfiguration defines the complete subtrees between the Trail Termination Point of the server layer (e.g. vc4TTP) and the Connection Termination Points of its clients (e.g. tu3CTP, tu12CTP, ...).

In a case when a Trail Termination Point can be cross-connected, it can exist independently of any existing path that has to be terminated.

The management system may select if the created connection termination point (e.g. tu3CTP) can be flexibly assigned or not.

The concatenation of Nxtu2CTP or Nxau4CTP is done by means of GTP objects and by means of the addTpsToGTP and removeTpsFromGTP actions on the fabric managed object.

The management system should be able to select the type of mapping of the PDH payload inside virtual container.

Change of configuration of already cross-connected CTP which is contained directly or indirectly by the object on which the action is applied is not feasible. The involved connection termination points should be disconnected first using the disconnect action onto the fabric.

# 6	Object classes

Revisions that require re-registration

This clause provides replacement managed object class definitions for the existing ITU-T G.774.2 (1994). Any managed object class replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a managed object class are as follows:

1)	The replaced managed object class is faulty and must be fixed.

2)	The replaced managed object class includes an attribute, package, notification or action which has been re-registered in this or another Recommendation.

3)	The replaced managed object class inherits from a managed object class which has been re-registered in this or another Recommendation.

In each case where a class is replaced, the new class will be registered within this Recommendation. The textual label for the class will be revised to include the text "R1".  For example, in the revision of the G.774.2 (1994) managed object class "modifiableVC4TTPBidirectional", the revised label will become "modifiableVC4TTPBidirectionalR1".

Below is a table of classes deprecated from ITU-T G.774.2 (1994) and the G.774.2 classes which replace them:

Deprecated G.774.2 1994 Classes	Replacement G.774.2 Classes

modifiableVC4TTPBidirectional		modifiableVC4TTPBidirectionalR1

modifiableVC4TTPSink				modifiableVC4TTPSinkR1

modifiableVC4TTPSource				modifiableVC4TTPSourceR1

modifiableVC3TTPBidirectional		modifiableVC3TTPBidirectionalR1

modifiableVC3TTPSink				modifiableVC3TTPSinkR1

modifiableVC3TTPSource				modifiableVC3TTPSourceR1

modifiableVC2TTPBidirectional		modifiableVC2TTPBidirectionalR1

modifiableVC2TTPSink				modifiableVC2TTPSinkR1

modifiableVC12TTPBidirectional		modifiableVC12TTPBidirectionalR1

modifiableVC12TTPSink				modifiableVC12TTPSinkR1

modifiableVC11TTPBidirectional		modifiableVC11TTPBidirectionalR1

modifiableVC11TTPSink				modifiableVC11TTPSinkR1

## 6.1	Indirect Adaptors

modifiableAugBidirectional  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": augBidirectional;

CHARACTERIZED BY

modifiableAugBidPackage  PACKAGE

BEHAVIOUR

modifiableAugBidBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported *

;;

ACTIONS

defineAUGStructure;;;

REGISTERED AS { g774-02MObjectClass 1 };

modifiableAugSink  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": augSink;

CHARACTERIZED BY

modifiableAugSinkPackage  PACKAGE

BEHAVIOUR

modifiableAugSinkBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineAUGStructure;;;

REGISTERED AS { g774-02MObjectClass 2 };

modifiableAugSource  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": augSource;

CHARACTERIZED BY

modifiableAugSourcePackage  PACKAGE

BEHAVIOUR

modifiableAugSourceBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineAUGStructure;;;

REGISTERED AS { g774-02MObjectClass 3 };

modifiableTug3Bidirectional  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": tug3Bidirectional;

CHARACTERIZED BY

modifiableTug3BidPackage  PACKAGE

BEHAVIOUR

modifiableTug3BidBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineTug3Structure;;;

REGISTERED AS { g774-02MObjectClass 4 };

modifiableTug3Sink  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": tug3Sink;

CHARACTERIZED BY

modifiableTug3SinkPackage  PACKAGE

BEHAVIOUR

modifiableTug3SinkBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineTug3Structure;;;

REGISTERED AS { g774-02MObjectClass 5 };

modifiableTug3Source  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": tug3Source;

CHARACTERIZED BY

modifiableTug3SourcePackage  PACKAGE

BEHAVIOUR

modifiableTug3SourceBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineTug3Structure;;;

REGISTERED AS { g774-02MObjectClass 6 };

modifiableTug2Bidirectional  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": tug2Bidirectional;

CHARACTERIZED BY

modifiableTug2BidPackage  PACKAGE

BEHAVIOUR

modifiableTug2BidBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineTug2Structure;;;

REGISTERED AS { g774-02MObjectClass 7 };

modifiableTug2Sink  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": tug2Sink;

CHARACTERIZED BY

modifiableTug2SinkPackage  PACKAGE

BEHAVIOUR

modifiableTug2SinkBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineTug2Structure;;;

REGISTERED AS { g774-02MObjectClass 8 };

modifiableTug2Source  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": tug2Source;

CHARACTERIZED BY

modifiableTug2SourcePackage  PACKAGE

BEHAVIOUR

modifiableTug2SourceBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineTug2Structure;;;

REGISTERED AS { g774-02MObjectClass 9 };

## 6.2	High Order Path Layer

modifiableVC4TTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc4TTPBidirectionalR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC4TTPBidR1Package  PACKAGE

BEHAVIOUR

modifiableVC4TTPBidR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineVC4Structure;;;

REGISTERED AS { g774-02MObjectClass 25 };

modifiableVC4TTPSinkR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc4TTPSinkR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC4TTPSinkR1Package  PACKAGE

BEHAVIOUR

modifiableVC4TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineVC4Structure;;;

REGISTERED AS { g774-02MObjectClass 26 };

modifiableVC4TTPSourceR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc4TTPSourceR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC4TTPSourceR1Package  PACKAGE

BEHAVIOUR

modifiableVC4TTPSourceR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineVC4Structure;;;

REGISTERED AS { g774-02MObjectClass 27 };

modifiableVC3TTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc3TTPBidirectionalR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC3TTPBidR1Package  PACKAGE

BEHAVIOUR

modifiableVC3TTPBidR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineVC3Structure;;;

REGISTERED AS { g774-02MObjectClass 28 };

modifiableVC3TTPSinkR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc3TTPSinkR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC3TTPSinkR1Package  PACKAGE

BEHAVIOUR

modifiableVC3TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineVC3Structure;;;

REGISTERED AS { g774-02MObjectClass 29 };

modifiableVC3TTPSourceR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc3TTPSourceR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC3TTPSourceR1Package  PACKAGE

BEHAVIOUR

modifiableVC3TTPSourceR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineVC3Structure;;;

REGISTERED AS { g774-02MObjectClass 30 };

## 6.3	Low Order Path Layer

modifiableVC2TTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc2TTPBidirectionalR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC2TTPBidR1Package  PACKAGE

BEHAVIOUR

modifiableVC2TTPBidR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 31 };

modifiableVC2TTPSinkR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc2TTPSinkR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC2TTPSinkR1Package  PACKAGE

BEHAVIOUR

modifiableVC2TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 32 };

modifiableVC2TTPSource  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc2TTPSource;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC2TTPSourcePackage  PACKAGE

BEHAVIOUR

modifiableVC2TTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 18 };

modifiableVC12TTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc12TTPBidirectionalR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC12TTPBidR1Package  PACKAGE

BEHAVIOUR

modifiableVC12TTPBidR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 33 };

modifiableVC12TTPSinkR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc12TTPSinkR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC12TTPSinkR1Package  PACKAGE

BEHAVIOUR

modifiableVC12TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 34 };

modifiableVC12TTPSource  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc12TTPSource;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC12TTPSourcePackage  PACKAGE

BEHAVIOUR

modifiableVC12TTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 21 };

modifiableVC11TTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc11TTPBidirectionalR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC11TTPBidR1Package  PACKAGE

BEHAVIOUR

modifiableVC11TTPBidR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 35 };

modifiableVC11TTPSinkR1  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc11TTPSinkR1;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC11TTPSinkR1Package  PACKAGE

BEHAVIOUR

modifiableVC11TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 36 };

modifiableVC11TTPSource  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774": vc11TTPSource;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC11TTPSourcePackage  PACKAGE

BEHAVIOUR

modifiableVC11TTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  This CLASS shall be instantiated when change of the SDH frame structure by management operation is supported  *

;;

ACTIONS

defineClientType;;;

REGISTERED AS { g774-02MObjectClass 24 };

# 7	Packages

None.

# 8	Attributes

None.

# 9	Actions

## 9.1	Define AUG Structure

defineAUGStructure  ACTION

BEHAVIOUR    defineAUGStructureBehaviour;

MODE    CONFIRMED;

PARAMETERS    defineSDHStructureError;

WITH INFORMATION SYNTAX    SDHConfASN1.AUGStructureInfo;

REGISTERED AS { g774-02Action 1 };

defineAUGStructureBehaviour  BEHAVIOUR

DEFINED AS

*This action is used to select between a one au4 structured aug and a three au3 structured aug.

If the object(s) contained by the aug already matche(s) the "AUGStructureInfo" parameter nothing is raised. In all other cases objects contained by the aug are deleted and corresponding to the "AUGStructureInfo" parameter one au4CTP, or three au3CTP are created.

The corresponding connection termination point(s) are created with the crossConnectionPointerPackage package according to the "connectionInfo" parameter. If the "unknown" choice of the "connectionInfo" parameter is selected the choice is left to the network element according to its make-up and mode.

When applied on a sink managed object, sink connection termination points are created.

When applied on a source managed object, source connection termination points are created.

When applied on a bidirectional managed object, bidirectional connection termination points are created.

The action fails if:

–	change of configuration applies on an already existing cross-connected connection termination point contained directly or indirectly by the object on which the action is applied;

–	the multiplexing structure is not supported by the network element;

–	at least one created connection termination point is not cross-connectable and the crossConnectable choice of the "connectionInfo" parameter is selected.

NOTE – The definition of the connection termination point managed object classes which represent the adaptation function of the characteristic information of the client inside SDH virtual container is not already defined.*

;

## 9.2	Define VC4 Structure

defineVC4Structure  ACTION

BEHAVIOUR    defineVC4StructureBehaviour,

defineTug3StructureBehaviour,

defineTug2StructureBehaviour;

MODE    CONFIRMED;

PARAMETERS    defineSDHStructureError;

WITH INFORMATION SYNTAX    SDHConfASN1.VC4StructureInfo;

REGISTERED AS { g774-02Action 2 };

defineVC4StructureBehaviour  BEHAVIOUR

DEFINED AS

*If the action parameter is "notSubmultiplexed" and contains "noClient" all contained objects are deleted. If the action parameter is "notSubmultiplexed" and contains a value different from "noClient" a CTP which corresponds to the client of the VC4 is created and the existing contained object(s) are deleted.

If the action parameter is "threeTUG3", three tug3 are created if they do not already exist, contained objects are deleted. Each TUG3 is structured according to the "TUG3StructureInfo" parameter ranked according to the time sequence of the tug3.

If the action has succeeded then the C2 signal label is updated according to the new frame structure:

–	When applied on a sink VC4 the expected signal label is updated.

–	When applied on a source VC4 the sent signal label is updated.

–	When applied on a bidirectional VC4 both the expected and sent signal label are updated.

NOTE – The definition of the connection termination point managed object classes which represent the adaptation function of the characteristic information of the client inside SDH virtual container is not already defined.*

;

## 9.3	Define VC3 Structure

defineVC3Structure  ACTION

BEHAVIOUR    defineVC3StructureBehaviour,

defineTug2StructureBehaviour;

MODE    CONFIRMED;

PARAMETERS    defineSDHStructureError;

WITH INFORMATION SYNTAX    SDHConfASN1.VC3StructureInfo;

REGISTERED AS { g774-02Action 3 };

defineVC3StructureBehaviour  BEHAVIOUR

DEFINED AS

*If the action parameter is "notSubmultiplexed" and contains "noClient" all contained objects are deleted. If the action parameter is "notSubmultiplexed" and contains a value different from "noClient" a CTP which corresponds to the client of the VC3 is created and the existing contained objects are deleted.

If the action parameter is "sevenTUG2", seven tug2 are created if they do not already exist, contained CTP are deleted. Each TUG2 is structured according to the "TUG2StructureInfo" parameter ranked according to the time sequence of the tug2.

If the action has succeeded then the C2 signal label is updated according to the new frame structure:

–	When applied on a sink VC3 the expected signal label is updated.

–	When applied on a source VC3 the sent signal label is updated.

–	When applied on a bidirectional VC3 both the expected and sent signal label are updated.

NOTE – The definition of the connection termination point managed object classes which represent the adaptation function of the characteristic information of the client inside SDH virtual container is not already defined.*

;

## 9.4	Define Tug3 Structure

defineTug3Structure  ACTION

BEHAVIOUR    defineTug3StructureBehaviour,

defineTug2StructureBehaviour;

MODE    CONFIRMED;

PARAMETERS    defineSDHStructureError;

WITH INFORMATION SYNTAX    SDHConfASN1.TUG3StructureInfo;

REGISTERED AS  { g774-02Action 4 };

defineTug3StructureBehaviour  BEHAVIOUR

DEFINED AS

*If the tug3StructureInfo parameter is "oneTU3" a tu3CTP is created if it does not already exist and the existing contained object(s) are deleted.

If the tug3StructureInfo parameter is "sevenTUG2", seven tug2 are created if they do not already exist, contained CTP is deleted. Each TUG2 is structured according to the "TUG2StructureInfo" parameter ranked according to the time sequence of the tug2.

NOTE – The definition of the connection termination point managed object classes which represent the adaptation function of the characteristic information of the client inside SDH virtual container is not already defined.*

;

## 9.5	Define Tug2 Structure

defineTug2Structure  ACTION

BEHAVIOUR    defineTug2StructureBehaviour;

MODE    CONFIRMED;

PARAMETERS    defineSDHStructureError;

WITH INFORMATION SYNTAX    SDHConfASN1.TUG2StructureInfo;

REGISTERED AS { g774-02Action 5 };

defineTug2StructureBehaviour  BEHAVIOUR

DEFINED AS

*If the object(s) contained by the tug2 already matche(s) the tug2StructureInfo parameter nothing is raised. In all other cases objects contained by the tug2 are deleted and corresponding to the "tug2StructureInfo" parameter one tu2CTP, three tu12CTP or four tu11CTP are created.

The corresponding connection termination point(s) are created with the crossConnectionPointerPackage package according to the "connectionInfo" parameter. If the "unknown" choice of the "connectionInfo" parameter is selected the choice is left to the network element according to its make-up and mode.

When applied on a sink managed object, sink connection termination points are created.

When applied on a source managed object, source connection termination points are created.

When applied on a bidirectional managed object, bidirectional connection termination points are created.

The action fails if:

–	change of configuration applies on an already existing cross-connected connection termination point contained directly or indirectly by the object on which the action is applied;

–	the multiplexing structure is not supported by the network element;

–	at least one created connection termination point is not cross-connectable and the crossConnectable choice of the "connectionInfo" parameter is selected.*

;

## 9.6	Definition of the Client type

defineClientType  ACTION

BEHAVIOUR    defineClientTypeBehaviour;

MODE    CONFIRMED;

PARAMETERS    defineSDHStructureError;

WITH INFORMATION SYNTAX    SDHConfASN1.DefineClientTypeInfo;

REGISTERED AS { g774-02Action 6 };

defineClientTypeBehaviour  BEHAVIOUR

DEFINED AS

*This action is used to choose the type of the adaptation function of payload inside the SDH low order Virtual Container.

If the action parameter is "noClient" the contained object is deleted. If the action parameter is different from "noClient" a CTP which corresponds to the client of the TTP is created and the existing contained object is deleted.

When applied on a sink TTP, a sink connection termination point is created.

When applied on a source TTP, a source connection termination point is created.

When applied on a bidirectional TTP, a bidirectional connection termination point is created.

The action fails if:

–	the client type is not supported by the network element.

If the action has succeeded then the signal label V5 (bits 5-7) is updated according to the new frame structure:

–	When applied on a sink VC the expected signal label is updated.

–	When applied on a source VC the sent signal label is updated.

–	When applied on a bidirectional VC both the expected and sent signal label are updated.

NOTE – The definition of the connection termination point managed object classes which represent the adaptation function of the characteristic information of the client inside SDH virtual container is not already defined.*

;

# 10	Notifications

None.

# 11	Parameters

defineSDHStructureError  PARAMETER

CONTEXT    SPECIFIC-ERROR;

WITH SYNTAX    SDHConfASN1.DefineSDHStructureError;

REGISTERED AS { g774-02Parameter 1 };

# 12	Name Bindings

Revisions that require re-registration

This clause provides replacement namebinding definitions for the existing ITU-T G.774.2 (1994). Any namebinding replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a namebinding are as follows:

1)	The replaced namebinding is faulty and must be fixed.

2)	The replaced namebinding refers to a superior managed object class which has been re-registered in this or another Recommendation.

3)	The replaced namebinding refers to a subordinate managed object class which has been re-registered in this or another Recommendation.

4)	The replaced namebinding refers to a naming attribute which has been re-registered in this or another Recommendation.

In each case where a namebinding is replaced, the new namebinding will be registered within this Recommendation. The textual label for the namebinding will be revised to include the text "R1". For example, in the revision of the G.774.2 (1994) namebinding "au3CTPSink-augSink", the revised label will become "au3CTPSink-augSinkR1". Note the "R1" is placed immediately following the revised class which impacts the namebinding.

Below is a table of namebindings deprecated from ITU-T G.774.2 (1994) and the G.774.2 namebindings which replace them:

Deprecated G.774.2 1994 Namebindings

au3CTPSink-augSink

au4CTPSink-augSink

tu11CTPSink-tug2Sink

tu12CTPSink-tug2Sink

tu2CTPSink-tug2Sink

tu3CTPSink-tug3Sink

tug2Source-vc3TTPSource

tug2Sink-vc3TTPSink

tug3Sink-vc4TTPSink

tug3Source-vc4TTPSource

vc11TTPSink-sdhNE

vc12TTPSink-sdhNE

vc2TTPSink-sdhNE

vc3TTPSink-sdhNE

vc3TTPSource-sdhNE

vc4TTPSink-sdhNE

vc4TTPSource-sdhNE

vcnUserChannelCTPSink-vc3TTPSink

vcnUserChannelCTPSource-vc3TTPSource

vcnUserChannelCTPSink-vc4TTPSink

vcnUserChannelCTPSource-vc4TTPSource

Replacement G.774 Namebindings

au3CTPSinkR1-augSink

au4CTPSinkR1-augSink

tu11CTPSinkR1-tug2Sink

tu12CTPSinkR1-tug2Sink

tu2CTPSinkR1-tug2Sink

tu3CTPSinkR1-tug3Sink

tug2Source-vc3TTPSourceR1

tug2Sink-vc3TTPSinkR1

tug3Sink-vc4TTPSinkR1

tug3Source-vc4TTPSourceR1

vc11TTPSinkR1-sdhNE

vc12TTPSinkR1-sdhNE

vc2TTPSinkR1-sdhNE

vc3TTPSinkR1-sdhNE

vc3TTPSourceR1-sdhNE

vc4TTPSinkR1-sdhNE

vc4TTPSourceR1-sdhNE

vcnUserChannelCTPSink-vc3TTPSinkR1

vcnUserChannelCTPSource-vc3TTPSourceR1

vcnUserChannelCTPSink-vc4TTPSinkR1

vcnUserChannelCTPSource-vc4TTPSourceR1

This Recommendation extends the Name Bindings currently defined in ITU-T G.774 by adding AND SUBCLASSES.

NOTE – Only the sink – sink and source – source Name Bindings are defined in this Recommendation. The also possible sink – bidirectional, source – bidirectional and bidirectional – bidirectional Name Bindings are used implicitly via Inheritance and the AND SUBCLASSES clauses. For the bidirectional – bidirectional case the sink – sink Name Binding shall be used.

au3CTPSinkR1-augSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": au3CTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": augSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": au3CTPId;

BEHAVIOUR

au3CTPSinkR1-augSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to  the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 59 };

au3CTPSource-augSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": au3CTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": augSource  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": au3CTPId;

BEHAVIOUR

au3CTPSource-augSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 2 };

au4CTPSinkR1-augSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": au4CTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": augSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": au4CTPId;

BEHAVIOUR

au4CTPSinkR1-augSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 60 };

au4CTPSource-augSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": au4CTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": augSource  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": au4CTPId;

BEHAVIOUR

au4CTPSource-augSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 4 };

augSink-msTTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": augSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": msTTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": augId;

BEHAVIOUR

augSink-msTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 5 };

augSource-msTTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": augSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": msTTPSource AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": augId;

BEHAVIOUR

augSource-msTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 6 };

electricalSPITTPSink-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": electricalSPITTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 7 };

electricalSPITTPSource-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": electricalSPITTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": electricalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 8 };

msCTPSink-rsTTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msCTPId;

BEHAVIOUR

msCTPSink-rsTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 9 };

msCTPSource-rsTTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msCTPId;

BEHAVIOUR

msCTPSource-rsTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 10 };

msDatacomCTPSink-msTTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msDatacomCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": msTTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSink-msTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 11 };

msDatacomCTPSource-msTTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msDatacomCTPSource AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": msTTPSource AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msDatacomCTPId;

BEHAVIOUR

msDatacomCTPSource-msTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 12 };

msOrderwireCTPSink-msTTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msOrderwireCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": msTTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSink-msTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 13 };

msOrderwireCTPSource-msTTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msOrderwireCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": msTTPSource AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msOrderwireCTPId;

BEHAVIOUR

msOrderwireCTPSource-msTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 14 };

msTTPSink-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msTTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 15 };

msTTPSource-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": msTTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": msTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 16 };

opticalSPITTPSink-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": opticalSPITTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 17 };

opticalSPITTPSource-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": opticalSPITTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": opticalSPITTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 18 };

rsCTPSink-electricalSPITTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": electricalSPITTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsCTPId;

BEHAVIOUR

rsCTPSink-electricalSPITTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 19 };

rsCTPSource-electricalSPITTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": electricalSPITTPSource  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsCTPId;

BEHAVIOUR

rsCTPSource-electricalSPITTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 20 };

rsCTPSink-opticalSPITTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": opticalSPITTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsCTPId;

BEHAVIOUR

rsCTPSink-opticalSPITTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 21 };

rsCTPSource-opticalSPITTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": opticalSPITTPSource  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsCTPId;

BEHAVIOUR

rsCTPSource-opticalSPITTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 22 };

rsDatacomCTPSink-rsTTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsDatacomCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSink-rsTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 23 };

rsDatacomCTPSource-rsTTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsDatacomCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsDatacomCTPId;

BEHAVIOUR

rsDatacomCTPSource-rsTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 24 };

rsOrderwireCTPSink-rsTTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsOrderwireCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSink-rsTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 25 };

rsOrderwireCTPSource-rsTTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsOrderwireCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsOrderwireCTPId;

BEHAVIOUR

rsOrderwireCTPSource-rsTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 26 };

rsTTPSink-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsTTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 27 };

rsTTPSource-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsTTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsTTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 28 };

rsUserChannelCTPSink-rsTTPSink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsUserChannelCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSink-rsTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 29 };

rsUserChannelCTPSource-rsTTPSource  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": rsUserChannelCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": rsTTPSource AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": rsUserChannelCTPId;

BEHAVIOUR

rsUserChannelCTPSource-rsTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 30 };

tu11CTPSinkR1-tug2Sink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu11CTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug2Sink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu11CTPId;

BEHAVIOUR

tu11CTPSinkR1-tug2SinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 61 };

tu11CTPSource-tug2Source  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu11CTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug2Source  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu11CTPId;

BEHAVIOUR

tu11CTPSource-tug2SourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 32 };

tu12CTPSinkR1-tug2Sink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu12CTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug2Sink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu12CTPId;

BEHAVIOUR

tu12CTPSinkR1-tug2SinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 62 };

tu12CTPSource-tug2Source  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu12CTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug2Source  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu12CTPId;

BEHAVIOUR

tu12CTPSource-tug2SourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 34 };

tu2CTPSinkR1-tug2Sink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu2CTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug2Sink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu2CTPId;

BEHAVIOUR

tu2CTPSinkR1-tug2SinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 63 };

tu2CTPSource-tug2Source  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu2CTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug2Source  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu2CTPId;

BEHAVIOUR

tu2CTPSource-tug2SourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 36 };

tu3CTPSinkR1-tug3Sink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu3CTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug3Sink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu3CTPId;

BEHAVIOUR

tu3CTPSinkR1-tug3SinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 64 };

tu3CTPSource-tug3Source NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tu3CTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug3Source  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tu3CTPId;

BEHAVIOUR

tu3CTPSource-tug3SourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 38 };

tug2Sink-tug3Sink  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tug2Sink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug3Sink  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tug2Id;

BEHAVIOUR

tug2Sink-tug3SinkBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 39 };

tug2Source-tug3Source  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tug2Source  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": tug3Source  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tug2Id;

BEHAVIOUR

tug2Source-tug3SourceBehaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 40 };

tug2Sink-vc3TTPSinkR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tug2Sink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc3TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tug2Id;

BEHAVIOUR

tug2Sink-vc3TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 65 };

tug2Source-vc3TTPSourceR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tug2Source  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc3TTPSourceR1  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tug2Id;

BEHAVIOUR

tug2Source-vc3TTPSourceR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 66 };

tug3Sink-vc4TTPSinkR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tug3Sink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc4TTPSinkR1  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tug3Id;

BEHAVIOUR

tug3Sink-vc4TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 67 };

tug3Source-vc4TTPSourceR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": tug3Source  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc4TTPSourceR1  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": tug3Id;

BEHAVIOUR

tug3Source-vc4TTPSourceR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 68 };

vc11TTPSinkR1-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc11TTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 69 };

vc11TTPSource-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc11TTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc11TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 46 };

vc12TTPSinkR1-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc12TTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 70 };

vc12TTPSource-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc12TTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc12TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 48 };

vc2TTPSinkR1-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc2TTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 71 };

vc2TTPSource-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc2TTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc2TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 50 };

vc3TTPSinkR1-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc3TTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 72 };

vc3TTPSourceR1-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc3TTPSourceR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc3TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 73 };

vc4TTPSinkR1-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc4TTPSinkR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 74 };

vc4TTPSourceR1-sdhNE  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vc4TTPSourceR1  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": sdhNE  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vc4TTPId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS { g774-02NameBinding 75 };

vcnUserChannelCTPSink-vc3TTPSinkR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vcnUserChannelCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc3TTPSinkR1  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc3TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 76 };

vcnUserChannelCTPSource-vc3TTPSourceR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vcnUserChannelCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc3TTPSourceR1  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc3TTPSourceR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 77 };

vcnUserChannelCTPSink-vc4TTPSinkR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vcnUserChannelCTPSink  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc4TTPSinkR1  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSink-vc4TTPSinkR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 78 };

vcnUserChannelCTPSource-vc4TTPSourceR1  NAME BINDING

SUBORDINATE OBJECT CLASS    "Recommendation G.774": vcnUserChannelCTPSource  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS    "Recommendation G.774": vc4TTPSourceR1  AND SUBCLASSES;

WITH ATTRIBUTE    "Recommendation G.774": vcnUserChannelCTPId;

BEHAVIOUR

vcnUserChannelCTPSource-vc4TTPSourceR1Behaviour  BEHAVIOUR

DEFINED AS

*  The subordinate managed objects are automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the NE  *

;;

REGISTERED AS { g774-02NameBinding 79 };

# 13	Constraint rules

## 13.1	Constraint rules extended syntax

This clause redefines the grammar of the pointer constraint rules as previously defined in ITU-T G.774. This new grammar should only be applied to the pointer constraint rules defined in this Recommendation.

### 13.1.1	Constraint rules grammar

<constraint-rule-label> CONSTRAINT RULE

OBJECT CLASS <class-label> [AND SUBCLASSES] ;

IS RELATED TO <class-list> ;

USING ATTRIBUTE <attribute-label> ;

<constraint-rule-set> ;

;

<constraint-rule-set>  ::=  <single-constraint-rule> | <named-type-constraint-rule>

<single-constraint-rule>  ::=  ACCORDING TO RULE <constraint-rule>

<named-type-constraint-rule>  ::=  CASE { <named-type-constraint-rule-list> }

<named-type-constraint-rule-list>  ::=  <named-type-constraint-rule-item> |

<named-type-constraint-rule-item> , <named-type-constraint-rule-list>

<named-type-constraint-rule-item>  ::=

<named-type> ACCORDING TO RULE <constraint-rule>

<class-label>  ::=  label string as defined in ISO/IEC IS 10165-4

<attribute-label>  ::=  label string as defined in ISO/IEC IS 10165-4

<class-list>  ::=  <class-spec> | <class-list> ,<class-spec>

<class-spec>  ::=  <class-label> | <class-label> AND SUBCLASSES

<constraint-rule>  ::=  SET { <constraint-members> }

| SEQUENCE { <constraint-members> }

| CHOICE { <constraint-members> }

| SET SIZE ( <ordinality> ) OF <constraint-term>

| SEQUENCE SIZE ( <ordinality> ) OF <constraint-term>

<constraint-members>  ::=     <constraint-term>

| <constraint-term> , <constraint-members>

<constraint-term>  ::=  <class-spec> | <constraint-rule>

<ordinality>  ::=  <valueRange> | <valueList>

<valueRange>  ::=  <lowerValue> .. <upperValue>

<valueList>  ::=  <itemValue> | <itemValue> , <valueList>

<itemValue>  ::=  INTEGER

<lowerValue>  ::=  INTEGER

<upperValue>  ::=  INTEGER | N

### 13.1.2	Constraint rule templates

OBJECT CLASS <class-label> [AND SUBCLASSES];

indicates the class and optionally the subclasses which are governed by this constraint-rule. A class may be governed by more than one constraint-rule with non-overlapping sets of related classes in the RELATES TO OBJECT CLASSES clause.

IS RELATED TO OBJECT CLASSES <class-list>;

indicates the set of classes of related instances that are governed by this constraint-rule.  Any classes that are not in this list are not governed by this constraint-rule.

USING ATTRIBUTE <attribute-label>;

indicates the attribute that represents a relationship by means of a pointer (DN) to the related object instances.

<constraint-rule-set>;

there can be either a single rule, or a set of rules one for each of a set of named choices.  In the latter case the CASE { ... } structure is used.

CASE { ... };

provides a distinct constraint-rule for each of the set of named choices in the attribute syntax.

ACCORDING TO RULE <constraint-rule>

provides the rule

SET { <constraint-members> }

indicates that *all* of the constraint-members must be present in any order.

SEQUENCE { <constraint-members> }

indicates that *all* of the constraint-members must be present in sequence.

CHOICE { <constraint-members> }

indicates that *any one* of the constraint-members must be present.

SET SIZE <ordinality> OF <constraint-term>

indicates the number of <constraint-term> that must be present in any order.

SEQUENCE SIZE <ordinality> OF <constraint-term>

indicates the number of <constraint-term> that must be present in sequence.

## 13.2	Connectivity pointer constraints

This clause defines the allowable values for the downstreamConnectivityPointer and upstreamConnectivityPointer attributes using the object classes defined in this Recommendation. The pointer constraint rules defined in this Recommendation replace those defined in ITU-T G.774.

Revisions that require re-registration

This clause provides replacement constraint rule definitions for the existing ITU-T G.774.2 (1994). Any constraint rule replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a constraint rule are as follows:

1)	The replaced constraint rule is faulty and must be fixed.

2)	The replaced constraint rule refers to a managed object class which has been re-registered in this or another Recommendation.

3)	The replaced constraint rule refers to an attribute which has been re-registered.

In each case where a constraint is replaced, the new constraint will be registered within this Recommendation. The textual label for the constraint will be revised to include the text "R1". For example, in the revision of the G.774.2 (1994) constraint "downstreamConnectivityPointer-au3CTPSink", the revised label will become "downstreamConnectivityPointer-au3CTPSinkR1". Note the "R1" is placed immediately following the revised class which impacts the constraint. In the case where the class within the label has not changed but the constraint is still altered because the

constraint refers to a class that has changed, then the "R1" is placed immediately following the "downstreamConnectivityPointer" text of the revised constraint label. For example, in the revision of the G.774.2 (1994) constraint "downstreamConnectivityPointer-au3CTPSource", the revised label will become "downstreamConnectivityPointerR1-au3CTPSource".

The syntax, grammar and templates used for these constraint rules are defined in ITU-T G.774 (1992) and extended in ITU-T G.774.2 (1994).

Below is a table of constraint rules deprecated from ITU-T G.774.2 (1994) and the G.774.2 constraint rules which replace them:

Deprecated G.774.2 1994 Constraint Rules

downstreamConnectivityPointer-au3CTPSink

upstreamConnectivityPointer-au3CTPSource

downstreamConnectivityPointer-au4CTPSink

upstreamConnectivityPointer-au4CTPSource

downstreamConnectivityPointer-tu11CTPSink

upstreamConnectivityPointer-tu11CTPSource

downstreamConnectivityPointer-tu12CTPSink

upstreamConnectivityPointer-tu12CTPSource

downstreamConnectivityPointer-tu2CTPSink

upstreamConnectivityPointer-tu2CTPSource

downstreamConnectivityPointer-tu3CTPSink

upstreamConnectivityPointer-tu3CTPSource

upstreamConnectivityPointer-vc11TTPSink

downstreamConnectivityPointer-vc11TTPSource

upstreamConnectivityPointer-vc12TTPSink

downstreamConnectivityPointer-vc12TTPSource

upstreamConnectivityPointer-vc2TTPSink

downstreamConnectivityPointer-vc2TTPSource

upstreamConnectivityPointer-vc3TTPSink

downstreamConnectivityPointer-vc3TTPSource

upstreamConnectivityPointer-vc4TTPSink

downstreamConnectivityPointer-vc4TTPSource

Replacement G.774.2 Constraint Rules

downstreamConnectivityPointer-au3CTPSinkR1

upstreamConnectivityPointerR1-au3CTPSource

downstreamConnectivityPointer-au4CTPSinkR1

upstreamConnectivityPointerR1-au4CTPSource

downstreamConnectivityPointer-tu11CTPSinkR1

upstreamConnectivityPointerR1-tu11CTPSource

downstreamConnectivityPointer-tu12CTPSinkR1

upstreamConnectivityPointerR1-tu12CTPSource

downstreamConnectivityPointer-tu2CTPSinkR1

upstreamConnectivityPointerR1-tu2CTPSource

downstreamConnectivityPointer-tu3CTPSinkR1

upstreamConnectivityPointerR1-tu3CTPSource

upstreamConnectivityPointer-vc11TTPSinkR1

downstreamConnectivityPointerR1-vc11TTPSource

upstreamConnectivityPointer-vc12TTPSinkR1

downstreamConnectivityPointerR1-vc12TTPSource

upstreamConnectivityPointer-vc2TTPSinkR1

downstreamConnectivityPointerR1-vc2TTPSource

upstreamConnectivityPointer-vc3TTPSinkR1

downstreamConnectivityPointer-vc3TTPSourceR1

upstreamConnectivityPointer-vc4TTPSinkR1

downstreamConnectivityPointer-vc4TTPSourceR1

downstreamConnectivityPointer-au3CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES,

"Recommendation G.774":vc4TTPSinkR1  AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES,

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES},

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES}

}

};

;

upstreamConnectivityPointerR1-au3CTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":au3CTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc3TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu3CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":vc4TTPSourceR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc3TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu3CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":vc4TTPSourceR1 AND SUBCLASSES}

};

;

downstreamConnectivityPointer-au4CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

au4CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":au4CTPSource AND SUBCLASSES,

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSource AND SUBCLASSES}

};

;

upstreamConnectivityPointerR1-au4CTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":au4CTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":au4CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":vc4TTPSourceR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc4TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSinkR1 AND SUBCLASSES}

};

;

downstreamConnectivityPointer-msCTPSink  CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":msCTPSink AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":msTTPSink AND SUBCLASSES,

"Recommendation G.774":msCTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":msTTPSink AND SUBCLASSES,

"Recommendation G.774":msCTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointer-msCTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":msCTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":msTTPSource AND SUBCLASSES,

"Recommendation G.774":msCTPSink AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":msTTPSource AND SUBCLASSES,

"Recommendation G.774":msCTPSink AND SUBCLASSES }

};

;

upstreamConnectivityPointer-msTTPSink CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":msTTPSink AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":msCTPSink AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":msCTPSink AND SUBCLASSES }

};

;

downstreamConnectivityPointer-msTTPSource  CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":msTTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":msCTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":msCTPSource AND SUBCLASSES }

};

;

downstreamConnectivityPointer-rsCTPSink CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":rsCTPSink AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":rsTTPSink AND SUBCLASSES,

"Recommendation G.774":rsCTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":rsTTPSink AND SUBCLASSES,

"Recommendation G.774":rsCTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointer-rsCTPSource  CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":rsCTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":rsTTPSource AND SUBCLASSES,

"Recommendation G.774":rsCTPSink AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":rsTTPSource AND SUBCLASSES,

"Recommendation G.774":rsCTPSink AND SUBCLASSES }

};

;

upstreamConnectivityPointer-rsTTPSink  CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":rsTTPSink AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":rsCTPSink AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":rsCTPSink AND SUBCLASSES }

};

;

downstreamConnectivityPointer-rsTTPSource  CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":rsTTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":rsCTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100": downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":rsCTPSource AND SUBCLASSES }

};

;

downstreamConnectivityPointer-tu11CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu11CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu11CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu11CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu11CTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointerR1-tu11CTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu11CTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc11TTPSource AND SUBCLASSES,

"Recommendation G.774":tu11CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc11TTPSource AND SUBCLASSES,

"Recommendation G.774":tu11CTPSinkR1 AND SUBCLASSES}

};

;

downstreamConnectivityPointer-tu12CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu12CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu12CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu12CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu12CTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointerR1-tu12CTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu12CTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc12TTPSource AND SUBCLASSES,

"Recommendation G.774":tu12CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc12TTPSource AND SUBCLASSES,

"Recommendation G.774":tu12CTPSinkR1 AND SUBCLASSES}

};

;

downstreamConnectivityPointer-tu2CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu2CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu2CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu2CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu2CTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointerR1-tu2CTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu2CTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc2TTPSource AND SUBCLASSES,

"Recommendation G.774":tu2CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc2TTPSource AND SUBCLASSES,

"Recommendation G.774":tu2CTPSinkR1 AND SUBCLASSES }

};

;

downstreamConnectivityPointer-tu3CTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu3CTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointerR1-tu3CTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":tu3CTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc3TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu3CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc3TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu3CTPSinkR1 AND SUBCLASSES }

};

;

upstreamConnectivityPointer-vc11TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc11TTPSource AND SUBCLASSES,

"Recommendation G.774":tu11CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc11TTPSource AND SUBCLASSES,

"Recommendation G.774":tu11CTPSinkR1 AND SUBCLASSES}

};

;

downstreamConnectivityPointerR1-vc11TTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc11TTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu11CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu11CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu11CTPSource AND SUBCLASSES}

};

;

upstreamConnectivityPointer-vc12TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

vc12TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc12TTPSource AND SUBCLASSES,

"Recommendation G.774":tu12CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc12TTPSource AND SUBCLASSES,

"Recommendation G.774":tu12CTPSinkR1 AND SUBCLASSES }

};

;

downstreamConnectivityPointerR1-vc12TTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc12TTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu12CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu12CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc12TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu12CTPSource AND SUBCLASSES}

};

;

upstreamConnectivityPointer-vc2TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc2TTPSource AND SUBCLASSES,

"Recommendation G.774":tu2CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc2TTPSource AND SUBCLASSES,

"Recommendation G.774":tu2CTPSinkR1 AND SUBCLASSES }

};

;

downstreamConnectivityPointerR1-vc2TTPSource CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc2TTPSource AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES

"Recommendation G.774":tu2CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu2CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu2CTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointer-vc3TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc3TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":tu3CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc3TTPSourceR1,

"Recommendation G.774":au3CTPSinkR1,

"Recommendation G.774":tu3CTPSinkR1 }

};

;

downstreamConnectivityPointer-vc3TTPSourceR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc3TTPSourceR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES},

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

vc3TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES,

"Recommendation G.774":tu3CTPSource AND SUBCLASSES }

};

;

upstreamConnectivityPointer-vc4TTPSinkR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc4TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":upstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc4TTPSourceR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSinkR1 AND SUBCLASSES },

concatenated ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SEQUENCE SIZE(3) OF

"Recommendation G.774":au3CTPSinkR1 AND SUBCLASSES}

};

;

downstreamConnectivityPointer-vc4TTPSourceR1 CONSTRAINT RULE

OBJECT CLASS

"Recommendation G.774":vc4TTPSourceR1 AND SUBCLASSES;

IS RELATED TO

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSource AND SUBCLASSES,

"Recommendation G.774":au3CTPSource AND SUBCLASSES;

USING ATTRIBUTE

"Recommendation M.3100":downstreamConnectivityPointer;

CASE {

single ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSource AND SUBCLASSES },

broadcast ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

"Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES,

"Recommendation G.774":au4CTPSource AND SUBCLASSES},

concatenated ACCORDING TO RULE

SET SIZE(1) OF CHOICE {

SEQUENCE SIZE(3) OF

"Recommendation G.774":au3CTPSource AND SUBCLASSES},

broadcastConcatenated ACCORDING TO RULE

SET SIZE(1..N) OF CHOICE {

SEQUENCE SIZE(3) OF

"Recommendation G.774":au3CTPSource AND SUBCLASSES}

};

;

# 14	Subordination rules

This clause defines the allowable combinations of subordinate object class instances that may be named by a superior object class instance, using the object classes contained in this Recommendation.

Revisions that require re-registration

This clause provides replacement subordination rule definitions for the existing ITU-T G.774.2 (1994). Any subordination rule replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a subordination rule are as follows:

1)	The replaced subordination rule is faulty and must be fixed.

2)	The replaced subordination rule refers to a managed object class which has been re-registered.

In each case where a subordination rule is replaced, the new subordination rule will be registered within this Recommendation. The textual label for the subordination rule will be revised to include the text "R1". For example, in the revision of the G.774.2 (1994) subordination rule "vc3TTPSinkSubordination", the revised label will become "vc3TTPSinkR1Subordination". Note the "R1" is placed immediately following the revised class which impacts the subordination rule. In the case where the class within the label has not changed but the subordination rule is still altered because the subordination rule refers to a class that has changed, then the "R1" is placed at the end of the revised subordination rule label. For example, in the revision of the G.774.2 (1994) subordination rule "modifiableAugSinkSubordination", the revised label will become "modifiableAugSinkSubordinationR1".

Below is a table of subordination rules deprecated from ITU-T G.774.2 (1994) and the G.774.2 subordination rules which replace them.

Deprecated G.774.2 1994 Subordination Rules

modifiableAugSinkSubordination

modifiableAugBidirectionalSubordination

modifiableTug2SinkSubordination

modifiableTug2BidirectionalSubordination

modifiableTug3SinkSubordination

tug3BidirectionalSubordination

modifiableVC3TTPSinkSubordination

vc3TTPSinkSubordination

modifiableVC3TTPSourceSubordination

vc3TTPSourceSubordination

modifiableVC3TTPBidirectionalSubordination

vc3TTPBidirectionalSubordination

modifiableVC4TTPSinkSubordination

vc4TTPSinkSubordination

modifiableVC4TTPSourceSubordination

vc4TTPSourceSubordination

modifiableVC4TTPBidirectionalSubordination

vc4TTPBidirectionalSubordination

Replacement G.774.2 Subordination Rules

modifiableAugSinkSubordinationR1

modifiableAugBidirectionalSubordinationR1

modifiableTug2SinkSubordinationR1

modifiableTug2BidirectionalSubordinationR1

modifiableTug3SinkSubordinationR1

tug3BidirectionalSubordinationR1

modifiableVC3TTPSinkR1Subordination

vc3TTPSinkR1Subordination

modifiableVC3TTPSourceR1Subordination

vc3TTPSourceR1Subordination

modifiableVC3TTPBidirectionalR1Subordination

vc3TTPBidirectionalR1Subordination

modifiableVC4TTPSinkR1Subordination

vc4TTPSinkR1Subordination

modifiableVC4TTPSourceR1Subordination

vc4TTPSourceR1Subordination

modifiableVC4TTPBidirectionalR1Subordination

vc4TTPBidirectionalR1Subordination

modifiableAugSinkSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

modifiableAugSink;

NAMES SUBORDINATES

"Recommendation G.774":au4CTPSinkR1,

"Recommendation G.774":au3CTPSinkR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF "Recommendation G.774":au4CTPSinkR1,

SET SIZE(3) OF "Recommendation G.774":au3CTPSinkR1

};

;

modifiableAugSourceSubordination  SUBORDINATION RULE

SUPERIOR OBJECT CLASS

modifiableAugSource;

NAMES SUBORDINATES

"Recommendation G.774":au4CTPSource,

"Recommendation G.774":au3CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF "Recommendation G.774":au4CTPSource,

SET SIZE(3) OF "Recommendation G.774":au3CTPSource

};

;

modifiableAugBidirectionalSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

modifiableAugBidirectional;

NAMES SUBORDINATES

"Recommendation G.774":au4CTPSinkR1,

"Recommendation G.774":au4CTPSource,

"Recommendation G.774":au4CTPBidirectionalR1,

"Recommendation G.774":au3CTPSinkR1,

"Recommendation G.774":au3CTPSource,

"Recommendation G.774":au3CTPBidirectionalR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

"Recommendation G.774":au4CTPSinkR1,

"Recommendation G.774":au4CTPSource,

"Recommendation G.774":au4CTPBidirectionalR1 },

SET SIZE(3) OF CHOICE {

"Recommendation G.774":au3CTPSinkR1,

"Recommendation G.774":au3CTPSource,

"Recommendation G.774":au3CTPBidirectionalR1 }

};

;

msTTPSinkSubordination  SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":msTTPSink;

NAMES SUBORDINATES

"Recommendation G.774":augSink,

modifiableAugSink,

"Recommendation G.774":msDatacomCTPSink,

"Recommendation G.774":msOrderwireCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF CHOICE {

"Recommendation G.774":augSink,

modifiableAugSink },

SET SIZE(0..1) OF "Recommendation G.774":msDatacomCTPSink,

SET SIZE(0..1) OF "Recommendation G.774":msOrderwireCTPSink

};

;

msTTPSourceSubordination  SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":msTTPSource;

NAMES SUBORDINATES

"Recommendation G.774":augSource,

modifiableAugSource,

"Recommendation G.774":msDatacomCTPSource,

"Recommendation G.774":msOrderwireCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF CHOICE {

"Recommendation G.774":augSource,

modifiableAugSource },

SET SIZE(0..1) OF "Recommendation G.774":msDatacomCTPSource,

SET SIZE(0..1) OF "Recommendation G.774":msOrderwireCTPSource

};

;

msTTPBidirectionalSubordination  SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":msTTPBidirectional;

NAMES SUBORDINATES

"Recommendation G.774":augBidirectional,

modifiableAugBidirectional,

"Recommendation G.774":msDatacomCTPSink,

"Recommendation G.774":msDatacomCTPSource,

"Recommendation G.774":msDatacomCTPBidirectional,

"Recommendation G.774":msOrderwireCTPSink,

"Recommendation G.774":msOrderwireCTPSource,

"Recommendation G.774":msOrderwireCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(1,4,16) OF CHOICE {

"Recommendation G.774":augBidirectional,

modifiableAugBidirectional },

SET SIZE(0..1) OF CHOICE {

"Recommendation G.774":msDatacomCTPSink,

"Recommendation G.774":msDatacomCTPSource,

"Recommendation G.774":msDatacomCTPBidirectional },

SET SIZE(0..1) OF CHOICE {

"Recommendation G.774":msOrderwireCTPSink,

"Recommendation G.774":msOrderwireCTPSource,

"Recommendation G.774":msOrderwireCTPBidirectional }

};

;

modifiableTug2SinkSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

modifiableTug2Sink;

NAMES SUBORDINATES

"Recommendation G.774":tu11CTPSinkR1,

"Recommendation G.774":tu12CTPSinkR1,

"Recommendation G.774":tu2CTPSinkR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF "Recommendation G.774":tu2CTPSinkR1,

SET SIZE(3) OF "Recommendation G.774":tu12CTPSinkR1,

SET SIZE(4) OF "Recommendation G.774":tu11CTPSinkR1

};

;

modifiableTug2SourceSubordination  SUBORDINATION RULE

SUPERIOR OBJECT CLASS

modifiableTug2Source;

NAMES SUBORDINATES

"Recommendation G.774":tu11CTPSource,

"Recommendation G.774":tu12CTPSource,

"Recommendation G.774":tu2CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF "Recommendation G.774":tu2CTPSource,

SET SIZE(3) OF "Recommendation G.774":tu12CTPSource,

SET SIZE(4) OF "Recommendation G.774":tu11CTPSource

};

;

modifiableTug2BidirectionalSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

modifiableTug2Bidirectional;

NAMES SUBORDINATES

"Recommendation G.774":tu11CTPSinkR1,

"Recommendation G.774":tu11CTPSource,

"Recommendation G.774":tu11CTPBidirectionalR1,

"Recommendation G.774":tu12CTPSinkR1,

"Recommendation G.774":tu12CTPSource,

"Recommendation G.774":tu12CTPBidirectionalR1,

"Recommendation G.774":tu2CTPSinkR1,

"Recommendation G.774":tu2CTPSource,

"Recommendation G.774":tu2CTPBidirectionalR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

"Recommendation G.774":tu2CTPSinkR1,

"Recommendation G.774":tu2CTPSource,

"Recommendation G.774":tu2CTPBidirectionalR1 },

SET SIZE(3) OF CHOICE {

"Recommendation G.774":tu12CTPSinkR1,

"Recommendation G.774":tu12CTPSource,

"Recommendation G.774":tu12CTPBidirectionalR1 },

SET SIZE(4) OF CHOICE {

"Recommendation G.774":tu11CTPSinkR1,

"Recommendation G.774":tu11CTPSource,

"Recommendation G.774":tu11CTPBidirectionalR1 }

};

;

modifiableTug3SinkSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

modifiableTug3Sink;

NAMES SUBORDINATES

"Recommendation G.774":tug2Sink,

modifiableTug2Sink,

"Recommendation G.774":tu3CTPSinkR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF "Recommendation G.774":tu3CTPSinkR1,

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Sink,

modifiableTug2Sink }

};

;

tug3SourceSubordination  SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":tug3Source;

NAMES SUBORDINATES

"Recommendation G.774":tug2Source,

modifiableTug2Source,

"Recommendation G.774":tu3CTPSource;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF "Recommendation G.774":tu3CTPSource,

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Source,

modifiableTug2Source }

};

;

tug3BidirectionalSubordinationR1 SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":tug3Bidirectional;

NAMES SUBORDINATES

"Recommendation G.774":tug2Sink,

"Recommendation G.774":tug2Source,

"Recommendation G.774":tug2Bidirectional,

modifiableTug2Sink,

modifiableTug2Source,

modifiableTug2Bidirectional,

"Recommendation G.774":tu3CTPSinkR1,

"Recommendation G.774":tu3CTPSource,

"Recommendation G.774":tu3CTPBidirectionalR1;

ACCORDING TO RULE

CHOICE {

SET SIZE(1) OF CHOICE {

"Recommendation G.774":tu3CTPSinkR1,

"Recommendation G.774":tu3CTPSource,

"Recommendation G.774":tu3CTPBidirectionalR1 },

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Sink,

"Recommendation G.774":tug2Source,

"Recommendation G.774":tug2Bidirectional,

modifiableTug2Sink,

modifiableTug2Source,

modifiableTug2Bidirectional

}

};

;

modifiableVC3TTPSinkR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":modifiableVC3TTPSinkR1;

NAMES SUBORDINATES

"Recommendation G.774":tug2Sink,

modifiableTug2Sink,

"Recommendation G.774":vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Sink,

modifiableTug2Sink },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSink

};

;

vc3TTPSinkR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":vc3TTPSinkR1;

NAMES SUBORDINATES

"Recommendation G.774":tug2Sink,

modifiableTug2Sink,

"Recommendation G.774":vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Sink,

modifiableTug2Sink },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSink

};

;

modifiableVC3TTPSourceR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":modifiableVC3TTPSourceR1;

NAMES SUBORDINATES

"Recommendation G.774":tug2Source,

modifiableTug2source,

"Recommendation G.774":vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Source,

modifiableTug2Source },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSource

};

;

vc3TTPSourceR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":vc3TTPSourceR1;

NAMES SUBORDINATES

"Recommendation G.774":tug2Source,

modifiableTug2source,

"Recommendation G.774":vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Source,

modifiableTug2Source },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSource

};

;

modifiableVC3TTPBidirectionalR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":modifiableVC3TTPBidirectionalR1;

NAMES SUBORDINATES

"Recommendation G.774":tug2Bidirectional,

modifiableTug2Bidirectional,

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Bidirectional,

modifiableTug2Bidirectional },

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional }

};

;

vc3TTPBidirectionalR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":vc3TTPBidirectionalR1;

NAMES SUBORDINATES

"Recommendation G.774":tug2Bidirectional,

modifiableTug2Bidirectional,

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(7) OF CHOICE {

"Recommendation G.774":tug2Bidirectional,

modifiableTug2Bidirectional },

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional }

};

;

modifiableVC4TTPSinkR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":modifiableVC4TTPSinkR1;

NAMES SUBORDINATES

"Recommendation G.774":tug3Sink,

modifiableTug3Sink,

"Recommendation G.774":vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(3) OF CHOICE {

"Recommendation G.774":tug3Sink,

modifiableTug3Sink },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSink

};

;

vc4TTPSinkR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":vc4TTPSinkR1;

NAMES SUBORDINATES

"Recommendation G.774":tug3Sink,

modifiableTug3Sink,

"Recommendation G.774":vcnUserChannelCTPSink;

ACCORDING TO RULE

SET {

SET SIZE(3) OF CHOICE {

"Recommendation G.774":tug3Sink,

modifiableTug3Sink },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSink

};

;

modifiableVC4TTPSourceR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":modifiableVC4TTPSourceR1;

NAMES SUBORDINATES

"Recommendation G.774":tug3Source,

modifiableTug3source,

"Recommendation G.774":vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(3) OF CHOICE {

"Recommendation G.774":tug3Source,

modifiableTug3Source },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSource

};

;

vc4TTPSourceR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":vc4TTPSourceR1;

NAMES SUBORDINATES

"Recommendation G.774":tug3Source,

modifiableTug3source,

"Recommendation G.774":vcnUserChannelCTPSource;

ACCORDING TO RULE

SET {

SET SIZE(3) OF CHOICE {

"Recommendation G.774":tug3Source,

modifiableTug3Source },

SET SIZE(1) OF

"Recommendation G.774":vcnUserChannelCTPSource

};

;

modifiableVC4TTPBidirectionalR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":modifiableVC4TTPBidirectionalR1;

NAMES SUBORDINATES

"Recommendation G.774":tug3Bidirectional,

modifiableTug3Bidirectional,

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(3) OF CHOICE {

"Recommendation G.774":tug3Bidirectional,

modifiableTug3Bidirectional },

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional }

};

;

vc4TTPBidirectionalR1Subordination SUBORDINATION RULE

SUPERIOR OBJECT CLASS

"Recommendation G.774":vc4TTPBidirectionalR1;

NAMES SUBORDINATES

"Recommendation G.774":tug3Bidirectional,

modifiableTug3Bidirectional,

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional;

ACCORDING TO RULE

SET {

SET SIZE(3) OF CHOICE {

"Recommendation G.774":tug3Bidirectional,

modifiableTug3Bidirectional },

SET SIZE(1) OF CHOICE {

"Recommendation G.774":vcnUserChannelCTPSink,

"Recommendation G.774":vcnUserChannelCTPSource,

"Recommendation G.774":vcnUserChannelCTPBidirectional }

};

;

# 15	Supporting ASN.1 productions

SDHConfASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) conf(02) informationModel(0)

asn1Module(2) sdhconf (0)}

DEFINITIONS IMPLICIT TAGS  ::=

BEGIN

--  EXPORTS everything  --

sdhConf OBJECT IDENTIFIER  ::=  { itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) conf(02)

informationModel(0) }

g774-02MObjectClass OBJECT IDENTIFIER  ::=  { sdhConf managedObjectClass(3) }

g774-02Action OBJECT IDENTIFIER  ::=  { sdhConf action(9) }

g774-02NameBinding OBJECT IDENTIFIER  ::=  { sdhConf nameBinding(6) }

g774-02Parameter OBJECT IDENTIFIER  ::=  { sdhConf parameter(5) }

ClientType  ::=  ENUMERATED  {

--  For more information refer to ITU-T G.803 and G.707.  --

noClient (0),

c139264AsynchronousMappingClientType (1),

c44736AsynchronousMappingClientType (2),

c34AsynchronousMappingClientType (3),

c6312AsynchronousMappingClientType (4),

c6312BitSynchronousMappingClientType (5),

c6312ByteSynchronousMappingClientType (6),

c2048AsynchronousMappingClientType (7),

c2048BitSynchronousMappingClientType (8),

c2048ByteSynchronousMappingClientType (9),

c1544AsynchronousMappingClientType (10),

c1544BitSynchronousMappingClientType (11),

c1544ByteSynchronousMappingClientType (12),

aTMClientType (13),

fDDIClientType (14),

mANClientType (15)

}

-- NOTE – The identification of the different type of adaptation functions

-- for a single type of characteristic  information is for further study.--

ConnectionInfo  ::=  ENUMERATED {

crossConnectable		(1),

notCrossConnectable	(2),

unknown				(3)

}

AUGStructureInfo  ::=  CHOICE  {

oneAU4		[0] ConnectionInfo,

threeAU3		[1] SEQUENCE SIZE (1..3) OF  ConnectionInfo

}

DefineClientTypeInfo  ::=  ClientType

DefineSDHStructureError  ::=  ENUMERATED  {

structureNotSupported		(0),

tpNotCrossConnectable		(1),

tpAlreadyCrossConnected		(2),

unknown					(3)

}

TUG3StructureInfo  ::=  CHOICE {

oneTU3		[0] ConnectionInfo,

sevenTUG2		[1] SEQUENCE SIZE (1..7) OF TUG2StructureInfo

}  --  ordered according to the time sequence  --

TUG2StructureInfo  ::=  CHOICE {

oneTU2		[0] ConnectionInfo,

threeTU12		[1] SEQUENCE SIZE (1..3) OF ConnectionInfo,

fourTU11		[2] SEQUENCE SIZE (1..4) OF ConnectionInfo

}  --  ordered according to the time sequence  --

VC4StructureInfo  ::=  CHOICE  {

notSubmultiplexed	[0] ClientType,

threeTUG3			[1] SEQUENCE SIZE (1..3) OF TUG3StructureInfo

}  --  ordered according to the time sequence  --

VC3StructureInfo  ::=  CHOICE  {

notSubmultiplexed	[0] ClientType,

sevenTUG2			[1] SEQUENCE SIZE (1..7) OF TUG2StructureInfo

}  --  In case of mapping the VC3 into a VC4 only the notSubmultiplexed choice is permitted.  --

END  --  end of supporting asn.1 productions  --

APPENDIX I

Inheritance and Naming Diagrams

See Figures I.1 and I.2.

Figure I.1/G.774.2  Naming tree for managed object classes (defined in this Recommendation)

Figure I.2/G.774.2  Inheritance tree for managed object classes
(defined in this Recommendation)

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |