|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.4 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH) – Management of the subnetwork connection protection for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.4 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.4 Synchronous digital hierarchy (SDH)  Management of the subnetwork connection protection for the network element view

- Summary This Recommendation provides an information model for the management of subnetwork connection protection of Synchronous Digital Hierarchy (SDH) subnetwork. This model describes the managed object classes and their properties for the subnetwork connection protection function, as defined in ITU-T G.803 and as related to SDH network elements. These objects are useful to describe information exchanged across interfaces defined in M.3010 Telecommunications Management Network (TMN) architecture for the management of the subnetwork connection protection function.

Document history. Issue; Notes. 2001; First revision incorporated the changes documented in the G.774.4 Corrigendum 1.. 7/1995; Initial version of the Recommendation..

- Source ITU-T Recommendation G.774.4 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2002

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and Definitions		3

4	Abbreviations		3

5	Subnetwork connection protection management model		4

5.1	Overview		4

5.2	SDH subnetwork connection protection requirements		4

6	Managed object class definitions		5

6.1	Connection protection group		6

6.2	Connection protection		6

6.3	Multipoint connection protection		7

6.4	Subnetwork connection protection fabric		9

7	Packages		9

7.1	Hold off time package		9

8	Attributes		9

8.1	Hold off time		9

8.2	Protection criteria		10

8.3	Switch status		10

9	Actions		11

9.1	Protected connect action		11

9.2	Protect unprotect connection action		12

10	Parameters		13

10.1	Switch status		13

11	Name bindings		14

11.1	Connection protection		15

11.2	Connection protection group R1		15

11.3	Cross connection		15

11.4	Multipoint connection protection		16

11.5	Multipoint cross connection		16

12	Subordination rules		16

13	Pointer constraints		16

14	Supporting ASN.1 productions		16

Page

Annex A  Inheritance and naming diagrams		19

Annex B  Examples of subnetwork connection protection		21

B.1	Legends		21

B.2	Subnetwork connection protection examples		22

B.3	Bidirectional subnetwork connection protection in rings		28

B.4	Interconnection of protected rings by a cross-connect		29

B.5	Interconnections of subnetwork with ADMs (SNCP/SNCP) in SNCP interworking		30

B.6	Logical connections in SNCP interworking rings		31

B.7	Logical connections in MS SHARED RINGs interworking		33

B.8	Logical connections in SNCPRing and MS SHARED RING interworking		35

Annex C  State transition diagrams		37

C.1	State diagrams		37

C.2	Allowable values of switchStatus for a protected (mp) connection protection		37

C.3	Allowable values of switchStatus for a protecting (mp) connection protection		38

C.4	State diagram for a protected connection protection (non-revertive)		38

C.5	State diagram for a protecting connection protection (non-revertive)		41

C.6	State diagram for a protected connection protection (revertive)		43

C.7	State diagram for a protecting connection protection (revertive)		45

ITU-T Recommendation G.774.4

Synchronous digital hierarchy (SDH)  Management of the subnetwork connection 
protection for the network element view

# 1	Scope

This Recommendation addresses the management of the automatic protection switching within network element at the high and low order path layers. It covers the subnetwork connection protection as described in ITU-T G.803 and G.841.

This Recommendation provides an information model, as related to the subnetwork connection protection function for the Synchronous Digital Hierarchy (SDH). It identifies the Telecommunications Management Network (TMN) object classes required for the management of the subnetwork connection protection function for SDH network elements. These objects are relevant to information exchanged across standardized interfaces defined in ITU-T M.3010 TMN architecture.

This Recommendation applies to SDH network elements which perform the subnetwork connection protection function and those systems in the TMN that manage SDH network elements.

The new objects defined in this Recommendation supersede those defined in ITU-T G.774.4 (1995). For each object class, attribute, action, notification, parameter defined in this Recommendation it shall be indicated what the impacts upon the existing ITU-T G.774.4 (1995) are.

Structure of this Recommendation

Clause 5.1 provides an overview of the SDH subnetwork connection protection information model. Clauses 6-14 describe the information model using the notation mechanisms defined in ITU-T X.722, Guidelines for the Definition of Managed Objects. Clause 14 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. Naming and Inheritance are illustrated in Annex A. The state transition diagrams are provided in Annex C. Diagrams illustrating the subnetwork connection protection are provided in Annex B.

# 2	References

The following Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy (SDH).

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774 (2001), Synchronous digital hierarchy (SDH)  Management information model for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunications management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview.

	ITU-T X.710 (1997), Information technology – Open Systems Interconnection – Common management information service.

	ITU-T X.711 (1997), Information technology – Open Systems Interconnection – Common management information protocol: Specification.

	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology – Open Systems Interconnection  Structure for management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology – Open Systems Interconnection – Systems Management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology – Open Systems Interconnection – Systems Management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology – Open Systems Interconnection – Systems Management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.734 (1992), Information technology – Open Systems Interconnection – Systems Management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.735 (1992), Information technology – Open Systems Interconnection – Systems Management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

# 3	Terms and Definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, ITU-T G.784 and ITU-T M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

APDU	Application Protocol Data Unit

APS	Automatic Protection Switching

BSHR	Bidirectional Self Healing Ring

CMIP	Common Management Information Protocol

CMIS	Common Management Information Service

CTP	Connection Termination Point

ISO	International Organization for Standardization

ITU-T	International Telecommunication Union – Telecommunication Standardization Sector

LOS	Loss Of Signal

MS	Multiplex Section

MSP	Multiplex Section Protection

NE	Network Element

OS	Operations System

OSI	Open Systems Interconnection

Pkg	Package

RDN	Relative Distinguished Name

RRP	Reliable Resource Pointer

SD	Signal Degrade

SDH	Synchronous Digital Hierarchy

SF	Signal Fail

SNCP	Subnetwork Connection Protection

STM-N	Synchronous Transport Module N

TMN	Telecommunications Management Network

TP	Termination Point

TTP	Trail Termination Point

URP	Unreliable Resource Pointer

WTR	Wait-To-Restore

# 5	Subnetwork connection protection management model

## 5.1	Overview

The model is based on those generic classes and modelling principles outlined in ITU-T M.3100 and on those common classes from ITU-T G.774.3.

It provides a similar way to control and operate the subnetwork connection protection function as the other protection functions. The model defines three subclasses of the generic protection model:

–	connectionProtectionGroupR1 which represents the association of unreliable resources for the purpose of automatic protection switching and which is the focal point for all management operation related to the protection function.

–	connectionProtection which represents the protected or the protecting unit and which represents the assignment of an unreliable resource to one reliable resource. The connectionProtection is contained by a connectionProtectionGroupR1.

–	mpConnectionProtection used to model protected broadcast. Each leg of the protected broadcast is represented by individual crossConnection contained by the mpConnectionProtection.

It defines also a subclass of the fabric: sncpFabric derived from the M.3100 fabric. The sncpFabric is used to handle by means of the protectedConnect and protectUnprotect action, in addition to the connect and disconnect actions of the fabric, the following capabilities:

–	Configure protected cross connections using the connection protections and multipoint connection protections.

–	Change from unprotected to protected connection without traffic cut.

–	Change from protected to unprotected connection without traffic cut.

## 5.2	SDH subnetwork connection protection requirements

The management of the subnetwork connection protection function shall provide the following management capabilities:

–	The ability to manage a 1 + 1 single ended subnetwork connection protection for unidirectional and/or bidirectional subnetwork connection.

–	The ability to manage the protection of broadcast where all legs are protected altogether and they do not act independently.

–	The ability to manage the protection of broadcast where a part of the legs are protected altogether and the other part of the legs are not protected.

–	Support of inter-connection of rings using add and drop multiplexer where no interaction between rings exists and where subnetwork connection protection exists in each ring.

–	Support of inter-connection of rings using add and drop multiplexer where no interaction between rings exists and where a subnetwork connection protection exists in one ring and a Multiplex Section Shared Protection Ring exists in the other ring.

–	Support of inter-connection of rings using add and drop multiplexer where no interaction between rings exists and where a Multiplex Section Shared Protection Ring exists in both rings.

–	Support of inter-connection of rings/subnetwork using cross-connects where subnetwork connection protection exists in each ring/subnetwork.

–	The ability to configure the protection before activating it.

–	The ability to configure the protection without any traffic flow and to resume the traffic afterwards.

–	The ability to change the unreliable (alternate routes) resource while the protection is configured.

–	The ability to pass from an existing non-protected connection to a protected connection without traffic cut.

–	The ability to pass from a protected connection to an unprotected connection without any traffic cut on the transmit direction.

–	The ability to pass from a protected leg of a protected broadcast to an unprotected leg without any traffic cut on the transmit direction.

–	The ability to pass from an unprotected leg of a broadcast to a protected leg without any traffic cut.

–	The ability when passing from a protected connection to an unprotected connection to select which of the two connections will carry the traffic afterwards.

–	The ability to initiate and terminate forced and manual switch.

–	The ability to lock out the protected and the protecting connection.

–	The ability to suspend and resume the traffic on the protected and the protecting connections.

–	The ability to configure the switching criteria.

–	The ability to control a switch hold off time and a wait to restore time.

–	The ability to configure the protection in a single management operation at each end of the subnetwork connection.

–	The ability to add or remove a leg to/from a protected broadcast.

–	The network element should not store any historical information about connection configuration before it has been configured in the protection state.

NOTE – Throughout this Recommendation the terms protecting and protected are used, it is considered that these terms are respectively equivalent to terms used in ITU-T G.841, protection and working, which are no longer used in that Recommendation.

# 6	Managed object class definitions

The following object class can be used to model an automatic 1 + 1 single-ended protection system for subnetwork connection protection.

Revisions that require re-registration

This clause provides replacement managed object class definitions for the existing ITU-T G.774.4 (1995). Any managed object class replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a managed object class are as follows:

1)	The replaced managed object class is faulty and must be fixed.

2)	The replaced managed object class includes an attribute, package, notification or action which has been re-registered in this or another Recommendation.

3)	The replaced managed object class inherits from a managed object class which has been re-registered in this or another Recommendation.

In each case where a class is replaced, the new class will be registered within this Recommendation. The textual label for the class will be revised to include the text "R1". For example, in the revision of the G.774.4 (1995) managed object class "connectionProtectionGroup", the revised label will become "connectionProtectionGroupR1".

Below is a table of classes deprecated from ITU-T G.774.4 (1995) and the G.774.4 classes which replace them:

Deprecated G.774.4 (1995) Classes	Replacement G.774.4 Classes

connectionProtectionGroup			connectionProtectionGroupR1

## 6.1	Connection protection group

connectionProtectionGroupR1  MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774.03":protectionGroupR1;

CHARACTERIZED BY

connectionProtectionGroupPkgR1	PACKAGE

BEHAVIOUR connectionProtectionGroupR1Behaviour;

ATTRIBUTES

protectionCriteria   	GET-REPLACE ADD-REMOVE;

NOTIFICATIONS

"Recommendation G.774.03":protectionSwitchReportingR1 switchStatusParameter;;;

CONDITIONAL PACKAGES

holdOffTimePackage

PRESENT IF "an instance supports it";

REGISTERED AS { g774-04MObjectClass 5 } ;

connectionProtectionGroupR1Behaviour BEHAVIOUR

DEFINED AS	"This object class is used to model an automatic protection

system for subnetwork connection protection. The protectionGroupType attribute indicates that the protection scheme used is 1+1 (plus). This object class is a focal point for management operations and notifications related to management of the protection system. Actual signal flow across the subnetwork connection protection is reflected by the connectivity pointers of the TPs involved in the subnetwork connection protection. At instantiation time of an instance of this class the protectionCriteria attribute is intitialized by

local initial value.";

## 6.2	Connection protection

connectionProtection  MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774.03": protectionUnit;

CHARACTERIZED BY

connectionProtectionPkg  PACKAGE

BEHAVIOUR connectionProtectionBehaviour;

ATTRIBUTES

switchStatus  GET,

"Recommendation X.721": administrativeState GET-REPLACE,

"Recommendation X.721": operationalState GET,

"Recommendation M.3100": signalType  GET,

"Recommendation M.3100": directionality GET;

NOTIFICATIONS

"Recommendation X.721": stateChange,

"Recommendation X.721": objectDeletion,

"Recommendation X.721": objectCreation;;;

REGISTERED AS { g774-04MObjectClass 2 };

connectionProtectionBehaviour BEHAVIOUR

DEFINED AS	"This object class is used to represent the assignment between an unreliable resource (TP) and a reliable resource (TP) for the purpose of subnetwork connection protection. This object class has properties of a cross-connection since it represents a flexible assignment of termination points in the context of subnetwork protection, according to Recommendation G.803. The directionality attribute represents the type of traffic which flows through the connection protection (CP). It can be either:

-	Bidirectional - The traffic flow is passing between the unreliable and the reliable resource in both directions.

-	Unidirectional - The traffic flow is unidirectional and passing from the unreliable to the reliable resource.

The Unreliable Resource Pointer (URP) points to one termination point which represents one of the alternatives for the protection system. The URP points to a TP according to the directionality of the CP, i.e.:

-	the URP of a Bidirectional CP must point to a Bidirectional TP or GTP;

-	the URP of an Unidirectional CP must point to a Sink TP or GTP, or to a Bidirectional TP or GTP.

The Unreliable Resource Pointer points to Null when the corresponding (unreliable) TP has been temporarily disconnected from the connection protection.

The crossConnectionObjectPointer of the corresponding TP points to the connectionProtection.

The Reliable Resource Pointer points either:

-	to Null in the case of a Protecting Connection Protection; or

-	to a Termination Point (CTP or TTP) or GTP instance that represents one end of the subnetwork connection in the case of a protected Connection Protection.

This termination point shall have the same signal type than the CTP or GTP representing the unreliable resource used for protection.

The crossConnectionObjectPointer of the corresponding TP points to the connectionProtection.

The attribute Signal Type describes the signal that is cross-connected. The termination points or GTPs that are cross-connected must have signal types that are compatible.

If the objects listed in the Unreliable Resource Pointer and Reliable Resource Pointer attributes are GTPs, the nth element of the Unreliable Resource GTP is related to the nth element of the Reliable Resource GTP (for every n).

Administrative state

-	Unlocked - The Connection Protection object is administratively unlocked. Traffic is allowed to pass through the Connection Protection.

-	Locked - No traffic is allowed to pass through the Connection Protection. The traffic flow is suspended between the termination points representing the reliable and the unreliable resources. The connectivity pointers in the corresponding unreliable resources and reliable resources are updated to reflect the suspension of traffic flow.

Setting the administrative state to locked does not prevent the protection function from attempting to operate.

Operational state

The operational state reflects only the connection capability of the connection protection:

-	Enabled - The Connection Protection is performing its normal cross-connection function.

-	Disabled - The Connection Protection is incapable of performing its normal cross-connection function.

The operational state of Disabled is considered as a signal failure on the corresponding connection and as such it must be taken into account in the protection mechanism.

The switchStatus indicates the status of the switching requests.";

## 6.3	Multipoint connection protection

mpConnectionProtection  MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774.03": protectionUnit;

CHARACTERIZED BY

mpConnectionProtectionPkg  PACKAGE

BEHAVIOUR mpConnectionProtectionBehaviour;

ATTRIBUTES

switchStatus  GET,

"Recommendation X.721": administrativeState GET-REPLACE,

"Recommendation X.721": operationalState GET,

"Recommendation X.721": availabilityStatus GET,

"Recommendation M.3100": signalType  GET;

NOTIFICATIONS

"Recommendation X.721": stateChange,

"Recommendation X.721": objectDeletion,

"Recommendation X.721": objectCreation;;;

REGISTERED AS { g774-04MObjectClass 3 };

mpConnectionProtectionBehaviour BEHAVIOUR

DEFINED AS	"This object class is used to represent the assignment between an unreliable resource, which is the termination point or GTP object listed in the Unreliable Resource Pointer attribute, and a set of reliable resources for the purpose of protection of point-to-multipoint connection. The reliable resources are the termination point or GTP objects listed in the To Termination attributes of the crossConnectionmanaged objects contained by protected mp Connection Protection.

This object class has properties of a multipoint cross-connection since its represents a flexible assignment of termination points in the context of subnetwork protection according to Recommendation G.803.

The Unreliable Resource Pointer (URP) points to one termination point or GTP which represents one of the two alternatives for the protection system.

The URP must point to a Sink CTP or GTP, or to a Bidirectional TP or GTP.

The Unreliable Resource Pointer points to Null when the corresponding (unreliable) TP has been temporarily disconnected from the multipoint connection protection.

The crossConnectionObjectPointer of the corresponding TP points to the mpConnectionProtection.

The Reliable Resource Pointer has always the value NULL. The reliable resources are represented by all the termination point or GTP objects pointed to by the To Termination attribute of the crossConnection managed objects contained by the protected mpConnectionProtection.

The attribute Signal Type describes the signal that is cross-connected. The termination points or GTPs that are cross-connected must have signal types that are compatible.

Administrative state

-	Unlocked - The Multipoint Connection Protection object is administratively unlocked. It allows traffic to pass through each contained cross-connection depending on its administrative state.

-	Locked - No traffic is allowed to pass through the Cross-Connection between the cross-connected termination points. The effect of this value overrides the effect of the administrative state of each contained cross-connection.

Setting the administrative state to locked does not prevent the protection function from attempting to operate.

Operational state

The operational state of a Multipoint Connection Protection object reflects the overall health of the cross-connection including all the Cross-Connection objects contained in the Multipoint Connection Protection.

-	Enabled - The Cross-Connection is performing its normal function. Note that some (but not all) of the cross-connection objects contained in the protected Multipoint Connection Protection may be disabled.

-	Disabled - The Cross-Connection is incapable of performing its normal cross-connection function. All the cross-connection objects contained in the protected Multipoint Connection Protection are disabled.

The operational state of Disabled is considered as a signal failure on the corresponding connection and as such it must be taken into account in the protection mechanism.

Availability status

The supported values for this attribute are:

-	Degraded - The Multipoint Connection Protection is degraded in some respect. For instance, if one or more (but not all) Cross-Connection objects contained in the protected Multipoint Connection Protection are disabled, the Multipoint Connection Protection will be considered as degraded. The Multipoint Connection Protection remains available for service (i.e. its operational state is enabled) while it is degraded. No automatic protection switch occurs.

-	Empty SET.

The switchStatus indicates the status of the switching requests.";

## 6.4	Subnetwork connection protection fabric

sncpFabric MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100": fabric;

CHARACTERIZED BY

sncpFabricPackage PACKAGE

BEHAVIOUR sncpFabricBehaviour;

ACTIONS

protectUnprotect,

protectedConnect;;;

REGISTERED AS { g774-04MObjectClass 4 };

sncpFabricBehaviour BEHAVIOUR

DEFINED AS	"The subnetwork connection protection fabric object represents the function of managing the establishment and release of protected cross-connections.";

# 7	Packages

## 7.1	Hold off time package

holdOffTimePackage	PACKAGE

ATTRIBUTES

holdOffTime GET-REPLACE;

REGISTERED AS { g774-04Package 1 };

# 8	Attributes

## 8.1	Hold off time

holdOffTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDHSNCPASN1.HoldOffTime;

MATCHES FOR EQUALITY;

BEHAVIOUR holdOffTimeBehaviour;

REGISTERED AS { g774-04Attribute 1 };

holdOffTimeBehaviour BEHAVIOUR

DEFINED AS	"This attribute specifies the amount of 100 ms periods which represents the time to wait before performing a protection switch after a defect has occurred on either the protecting or protected protection unit. The minimum value is zero (no hold off time) and the maximum value is 100 (representing 10 seconds). The hold off time is defined in G.841.";

## 8.2	Protection criteria

protectionCriteria	ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDHSNCPASN1.ProtectionCriteria;

MATCHES FOR EQUALITY;

BEHAVIOUR protectionCriteriaBehaviour;

REGISTERED AS { g774-04Attribute 2 };

protectionCriteriaBehaviour BEHAVIOUR

DEFINED AS	"This attribute specifies the list of configurable criteria that are to be used for automatic switch decision supplementing those criteria which are defined as mandatory and fixed within the SNCP function as stated in G.841. ";

## 8.3	Switch status

switchStatus	ATTRIBUTE

WITH ATTRIBUTE SYNTAX SDHSNCPASN1.ProtectionStatus;

MATCHES FOR EQUALITY;

BEHAVIOUR switchStatusBehaviour;

REGISTERED AS { g774-04Attribute 3 };

switchStatusBehaviour BEHAVIOUR

DEFINED AS	"The following is a list of all the possible status values of the switchStatus attribute for protected connection protection or protected mpConnectionProtection:

-	No request - There is no outstanding request. An active switch hold off time is considered a no request.

-	Do Not Revert - The protected connection protection has been switched to the protecting connection protection and the request to do so has been subsequently cleared. This status is only applicable to 1 + 1 non-revertive systems.

-	Wait-to-Restore - The protected connection protection has recovered from an auto switch condition and the WTR timer is activated.

-	Manual switch to the protecting connection protection completed - The protected connection protection has been switched to the protecting connection protection by a manual switch request.

-	Auto switch to the protecting connection completed - The protected connection protection has been switched to the protecting connection protection due to an auto switch condition.

-	Auto switch pending - An auto switch request cannot been completed due to the unavailability of the protecting connection protection.

-	Forced switch to the protecting connection protection completed - The protected connection protection has been forced to switch to the protecting connection protection.

-	Forced switch to the protecting connection protection completed - auto switch pending - An auto switch request cannot be completed due to a completed forced switch (to protecting connection protection).

-	Lockout of protected connection protection - The protected connection protection is prevented from switching to protection.

-	Lockout of protected connection protection - auto switch pending - An auto switch request cannot be completed due to the lockout of the protected connection protection.

The following is a list of all the possible status values of the switchStatus attribute for protecting connection protection or protecting mpConnectionProtection:

-	No request - There is no outstanding request. An active switch hold off time is considered a no request.

-	Do Not Revert - The protected connection protection has been switched to the protecting connection protection and the request to do so has been cleared. This status is only applicable to 1 + 1 non-revertive systems.

-	Manual switch to the protecting connection protection completed - The protected connection protection has been switched to the protecting connection protection by a manual switch request.

-	Auto switch to the protecting connection protection completed - A protected connection protection has been switched to the protecting connection protection due to an auto switch condition.

-	Signal fail - A signal fail condition exists on the protecting connection protection (no traffic is being carried on the protecting connection protection).

-	Forced switch to the protecting connection completed - The protected connection protection has been forced to switch to the protecting connection protection.

-	Lockout of protection - The protecting connection protection is prevented from providing protection.

-	Lockout of protection - auto switch condition on protecting connection protection - A lockout of protection is in place, and an auto switch condition exists on the protecting connection protection.";

# 9	Actions

## 9.1	Protected connect action

protectedConnect	ACTION

BEHAVIOUR		protectedConnectBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SDHSNCPASN1.ProtectedConnectInformation;

WITH REPLY SYNTAX	SDHSNCPASN1.ProtectedConnectResult;

REGISTERED AS { g774-04Action 1 };

protectedConnectBehaviour BEHAVIOUR

DEFINED AS	"This action offers the capability, in a single action, to:

1)	Establish a protected connection between termination points/GTPs (connect Choice of the syntax). The connection to be established is part of a subnetwork connection protection. The Termination Points/GTP involved in this protected connection must not be already connected for the concerned direction. In this case this connection establishment is done according to the following rules:

-	The protecting TP is used to describe the unreliable TP(s) of the protecting connection. This must be done according to the description of the protection Connection Type.

-	The protection Connection Type is used to describe the type of connection to be established, i.e. unidirectional or bidirectional. The protection Connection Type describes the connection between the unreliable TPs and the reliable TPs for the protected connection.

-	If the protection Connection Type is bidirectional, a connectionProtectionGroupR1 is created. The To TP part of the syntax represents the reliable resource.

-	If the protection Connection Type is unidirectional, it can concern either:

-	A unidirectional point-to-point subnetwork connection protection. In this case a connectionProtectionGroupR1 is created.

-	A unidirectional point-to-multipoint subnetwork connection protection where all legs are protected altogether. In this case a connectionProtectionGroupR1 is created, containing:

a)	the mpConnectionProtection of the working channel, connecting the unreliable TP. The reliable TPs are connected by crossConnection objects contained by the mpConnectionProtection.

b)	the mpConnectionProtection of the protecting channel. This mpConnectionProtection does not contain crossConnection object.

The administrative state of the created connections objects is specified as an optional parameter of this action. If this parameter is omitted, the administrative state will be set to 'unlocked'.

2)	Add TPs/GTPs as destinations of a point-to-multipoint connection. (addLeg Choice of the syntax).

–	The legs may be added as part of the protected legs (corresponding to the sub-choice protected Legs of the syntax). In that case each crossConnection object corresponding to each individual leg is created, these crossConnections are contained by the protected mpConnectionProtection contained by the connectionProtectionGroupR1.

–	The legs may be added as non-protected legs (corresponding to the sub-choice unprotected Legs of the syntax). In that case each crossConnection object corresponding to each individual leg is created, these crossConnections are contained by the mpCrossConnection representing the unprotected broadcast. If no unprotected mpCrossConnection exists then an mpCrossConnection is created, containing all the specified unprotected legs (if only one unprotected leg exists, then a point-to-point unidirectional cross-connection is created). If this action concerns an existing unidirectional point-to-point cross-connection then a point-to-multipoint cross-connection is created which contains the crossConnections representing legs.

The administrative state of the created cross-connection or connection protection objects will be the same as that of the containing multipoint cross-connection or multipoint connection protection object unless otherwise specified in the action parameters.

3)	Connect a TP as the unreliable resource of an existing protected connection (protected or protecting connectionProtection or mpConnectionProtection contained in a connectionProtectionGroupR1). (addUnreliable Choice of the syntax)

–	If the specified TP is not already connected for the corresponding direction (notConnected sub-choice of the syntax), it will be connected by the specified connectionProtection or mpConnectionProtection.

–	If the specified TP is already connected for the corresponding direction (connected sub-choice of the syntax) by a unidirectional connection (point-to-point or multipoint), the TP will point to the connectionProtection or mpConnectionProtection in addition to the existing connection(s).";

## 9.2	Protect unprotect connection action

protectUnprotect ACTION

BEHAVIOUR  protectUnprotectBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SDHSNCPASN1.ProtectUnprotectInformation;

WITH REPLY SYNTAX	SDHSNCPASN1.ProtectedConnectResult;

REGISTERED AS { g774-04Action 2 };

protectUnprotectBehaviour BEHAVIOUR

DEFINED AS	"This action offers the capability, in a single action, to:

1)	Protect an existing unprotected connection (protect Choice of the syntax). The description for protecting an existing connection uses the same syntax as the establishment of a new protected connection, and follows the same rules, except that all the TPs or GTP specified in the protectionConnectionType part must be explicitly specified by their object instances (i.e. no tpPool, no GTP expressed by a sequence of TPs). The connectivity specified by the protectionConnectionType part of the syntax must reflect the same connectivity (signal flow expressed by the connectivity pointers) as the existing one, i.e.:

-	For a unidirectional point-to-point, the TP specified as the From and the To must be already cross-connected the same way as described and they should not be already protected for the concerned direction.

-	For a bidirectional point-to-point, the TPs specified as the from and the To must be already cross-connected the same way as described, i.e.: a bidirectional cross-connection connecting the From TP and the To TP (reliable TP) must already exist.

-	For a point-to-multipoint, the TP specified as the From and all the TPs specified as the To must be already cross-connected the same way and none of them should be already part of a protection scheme for the concerned direction. Only those legs which are specified as part of the protectedConnect Information will be involved in the protection scheme. A connectionProtectionGroupR1, two mpConnectionProtection’s and each crossConnection representing each leg are created as the result of this action. If all legs are involved in the protection scheme then the existing mpCrossConnection is deleted. The administrative state of the new created connection objects will be the same as that of the existing one they are to replace, unless otherwise specified in the action parameters. The result, if OK is given by the connected Choice of the syntax.

2)	Unprotect an existing protected connection (unprotect Choice of the syntax). The description for unprotecting an existing connection uses the same syntax as the establishment of a new unprotected connection, i.e. the ConnectionType and ConnectionTypeBi, and follows the same rules, except that all the described connections must already exist, and all the TPs or GTP specified in this action must be explicitly specified by their object instances. The connectivity specified by the syntax must reflect the same connectivity as the existing one for either the working or the protect part of the ConnectionProtectionGroupR1, i.e.:

-	For a unidirectional point-to-point connection protection, the TP specified as the From and the To must be respectively the unreliable TP and the reliable TP of the (protected or protecting) connectionProtection.

-	For a bidirectional point-to-point, the TPs specified as the From and the To must be respectively the unreliable TP of either the protected or protecting connectionProtection and the reliable TP of the protected connectionProtection. When the choice single of the syntax is used, a bidirectional cross-connection is created (uniform route) connecting the specified Unreliable TP and the Reliable TP. When the choice multiple of the syntax is used, two unidirectional cross-connections are created (diverse route), one connecting the specified Unreliable TP to the Reliable TP, and the other connecting the Reliable TP to the specified Unreliable TP.

–	For a point-to-multipoint, the TP specified as the From and all the TPs specified as the To must be respectively the unreliable TP of the protected mp Connection Protection and the To TPs of the cross-connects contained in the mp connection protection. If TPs are specified which do not pertain to the protection scheme, the requests fail with the logical problem mismatching TP Instances. Only those TPs which are explicitly specified as part of the protectedUnprotectInformation will be unprotected, and if no mpCrossConnection was already existing then one is created which holds each crossConnection corresponding to each unprotected leg. If no other TP remains in the protected scheme, then the connectionProtectionGroupR1 is deleted and consequently all its contained objects. The administrative state of the new created connection objects will be the same as that of the existing one they are to replace, unless otherwise specified in the action parameters.";

# 10	Parameters

## 10.1	Switch status

switchStatusParameter	PARAMETER

CONTEXT EVENT-INFO;

WITH SYNTAX	SDHSNCPASN1.ProtectionStatusParameter;

BEHAVIOUR switchStatusParameterBeh;

REGISTERED AS { g774-04Parameter 1 };

switchStatusParameterBeh BEHAVIOUR

DEFINED AS	"This parameter is included in the additional info parameters of the protection switching reporting notification. This notification is sent by the protection group according to the following rules. There are several cases:

-	The switch from protected to protecting or protecting to protected has been done without preempting an existing switch. In this case, the old and new values of the Switch status attribute of the protecting channel shall be reported in the notification by means of the oldSwitchStatus and newSwitchStatus parameters respectively.

-	A switch is performed by preemting an exiting one. In this case, the old and new values of the Switch status attribute of the protecting channel shall be reported in the notification by means of the oldSwitchStatus and newSwitchStatus parameters respectively.

-	An auto switch condition exists on a channel but the auto switch cannot be served due to the unavailability of the channel that otherwise protects it. In this case the oldSwitchStatus and newSwitchStatus parameters refer to the switch status attribute value of the channel on which the auto switch condition arises. The exception is when the channel is already forced or locked out, in which case no notification is sent.

-	A working channel (protected unit) has been locked out or released from lockout without modifying any existing switch. In this case the oldSwitchStatus and newSwitchStatus parameters refer to the switch status attribute value of the working channel which has been locked out.

-	A protection channel (protecting unit) has been locked out or released from lockout without modifying the existing switch. In this case, the oldSwitchStatus and newSwitchStatus parameters refer to the switch status attribute value of the protection channel which has been locked out.

The protectionSwitchReporting is not sent when the automatic switch condition is toggling between SF and WTR condition. While in the lockout of forced switch state no notification is sent except for ending of release failure.";

# 11	Name bindings

Revisions that require re-registration

This clause provides replacement namebinding definitions for the existing ITU-T G.774.4 (1995). Any namebinding replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a namebinding are as follows:

1)	The replaced namebinding is faulty and must be fixed.

2)	The replaced namebinding refers to a superior managed object class which has been re-registered in this or another Recommendation.

3)	The replaced namebinding refers to a subordinate managed object class which has been re-registered in this or another Recommendation.

4)	The replaced namebinding refers to a naming attribute which has been re-registered in this or another Recommendation.

In each case where a namebinding is replaced, the new namebinding will be registered within this Recommendation. The textual label for the namebinding will be revised to include the text "R1". For example, in the revision of the G.774.4 (1995) namebinding "connectionProtection-connectionProtectionGroup", the revised label will become "connectionProtection-connectionProtectionGroupR1". Note the "R1" is placed immediately following the revised class which impacts the namebinding.

Below is a table of namebindings deprecated from ITU-T G.774.4 (1995) and the G.774.4 namebindings which replace them:

Deprecated G.774.4 (1995) Namebindings

connectionProtection-connectionProtectionGroup

connectionProtectionGroup-sncpFabric

mpConnectionProtection-connectionProtectionGroup

Replacement G.774.4 Namebindings

connectionProtection-connectionProtectionGroupR1

connectionProtectionGroupR1-sncpFabric

mpConnectionProtection-connectionProtectionGroupR1

## 11.1	Connection protection

connectionProtection-connectionProtectionGroupR1 NAME BINDING

SUBORDINATE OBJECT CLASS		connectionProtection AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS connectionProtectionGroupR1

AND SUBCLASSES;

WITH ATTRIBUTE	"Recommendation G.774.03":protectionUnitId;

BEHAVIOUR

connectionProtection-connectionProtectionGroupR1Beh BEHAVIOUR

DEFINED AS	"The connection protection is contained by a

connection protection Group managed object instance. There must be two and only two CP (Connection Protection) in the CPG (Connection Protection Group), with the same signal type and the same directionality. This is used to represent a point-to-point unidirectional or bidirectional connection protection. One of these CPs must be the protected one (protecting attribute with the value 'FALSE'), and the other must be the protecting one (protecting attribute with the value 'TRUE')." ;;

REGISTERED AS { g774-04NameBinding 7 }

;

## 11.2	Connection protection group R1

connectionProtectionGroupR1-sncpFabric NAME BINDING

SUBORDINATE OBJECT CLASS

connectionProtectionGroupR1 AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS

sncpFabric AND SUBCLASSES;

WITH ATTRIBUTE		"Recommendation G.774.03":protectionGroupId;

BEHAVIOUR  connectionProtectionGroupR1-sncpFabricBeh BEHAVIOUR

DEFINED AS	"A Connection Protection Group is created as the

result of the establishment of a protected connection (point-to-point or point-to-multipoint) or of the protection of an existing non-protected connection. When disconnecting a TP involved in a protected connection:

-	if the TP is the reliable resource of a point-to-point connection protection, or the last reliable resource (last leg) of a multipoint connection protection, this leads to the disconnection of the protection, resulting in the deletion of the connection protection group and all contained objects.

-	if the TP is the unreliable resource of a connection protection or multipoint connection protection, the corresponding Unreliable Resource Pointer is set to NULL. Such a disconnection shall be considered as a signal failure. A new unreliable resource (TP) may be connected using the capability to add a new unreliable resource of the protected connect action. When both unreliable TPs are disconnected, this leads to the disconnection of the protection, resulting in the deletion of the connection protection group and all contained objects. ";;

REGISTERED AS { g774-04NameBinding 8 };

## 11.3	Cross connection

crossConnection-mpConnectionProtection NAME BINDING

SUBORDINATE OBJECT CLASS "Recommendation M.3100":

crossConnection AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS mpConnectionProtection AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation M.3100":crossConnectionId;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { g774-04NameBinding 3 };

crossConnection-sncpFabric NAME BINDING

SUBORDINATE OBJECT CLASS "Recommendation M.3100":

crossConnection AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS sncpFabric AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation M.3100":crossConnectionId;

DELETE ONLY-IF-NO-CONTAINED-OBJECTS;

REGISTERED AS { g774-04NameBinding 5 };

## 11.4	Multipoint connection protection

mpConnectionProtection-connectionProtectionGroupR1 NAME BINDING

SUBORDINATE OBJECT CLASS		mpConnectionProtection AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS

connectionProtectionGroupR1 AND SUBCLASSES;

WITH ATTRIBUTE1	"Recommendation G.774.03":protectionUnitId;

BEHAVIOUR	mpConnectionProtection-connectionProtectionGroupR1Beh

BEHAVIOUR

DEFINED AS	"MultiPoint Connection Protection are created as

the result of the establishment of a protected multipoint connection or of the protection of an existing non-protected multipoint connection. The multipoint connection protections are contained directly by the connection protection Group managed object instance. There must be two and only two mpConnectionProtections in the connectionProtectionGroup, with the same signal type. This is used to represent the point to multipoint unidirectional connection protection.

One of these mpConnectionProtection must be the protected one (protecting attribute with the value FALSE), and the other must be the protecting one (protecting attribute with the value TRUE).

Only the protected mpConnectionProtection can contain cross-connections representing the reliable resources. The configuration may be done prior to any existing cross-connection or on an existing multipoint cross-connection." ;;

REGISTERED AS { g774-04NameBinding 9 };

## 11.5	Multipoint cross connection

mpCrossConnection-sncpFabric NAME BINDING

SUBORDINATE OBJECT CLASS "Recommendation M.3100":

mpCrossConnection AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS sncpFabric AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation M.3100":mpCrossConnectionId;

REGISTERED AS { g774-04NameBinding 6 };

# 12	Subordination rules

None.

# 13	Pointer constraints

None.

# 14	Supporting ASN.1 productions

SDHSNCPASN1 { itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) sncp(04)
informationModel(0) asn1Module(2) sdhsncp (0) }

DEFINITIONS IMPLICIT TAGS  ::=

BEGIN

--  EXPORTS everything  --

IMPORTS

ObjectInstance

FROM CMIP-1 { joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3) }

AdministrativeState, OperationalState

FROM Attribute-ASN1Module { joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 1 }

AddLeg, Connected, ConnectionType, ConnectionTypeBi, Directionality, ExplicitPtoMP,
ExplicitPtoP, ExplicitTP, Failed, PointToMultipoint, PointToPoint, PtoMPools, PtoTPPool
FROM ASN1DefinedTypesModule { ccitt recommendation m(13) gnm(3100)
informationModel(0) asn1Modules(2) asn1DefinedTypesModule(0) }

Boolean, Integer, ProtectionStatus, ProtectionStatusParameter
FROM SDHProtASN1 { itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) prot(03)
informationModel(0) asn1Module(2) sdhmsp(0) };

sdhSNCP OBJECT IDENTIFIER ::= { itu-t(0) recommendation(0) g(7) g774(774) hyphen(127)
sncp(04) informationModel(0) }

g774-04MObjectClass OBJECT IDENTIFIER ::=  { sdhSNCP managedObjectClass(3) }

g774-04Attribute OBJECT IDENTIFIER ::=  { sdhSNCP attribute(7) }

g774-04Action OBJECT IDENTIFIER ::=  { sdhSNCP action(9) }

g774-04NameBinding OBJECT IDENTIFIER ::=  { sdhSNCP nameBinding(6) }

g774-04Parameter OBJECT IDENTIFIER ::=  { sdhSNCP parameter(5) }

g774-04Package OBJECT IDENTIFIER ::=  { sdhSNCP package(4) }

g774-04StandardSpecificExtension OBJECT IDENTIFIER ::=  { sdhSNCP standardSpecificExtension(0) }

--  The following value assignments are for the Protection Criteria in the 
--  context of SDH.

--  These values shall always be assigned by this Recommendation in the context 
--  of SDH.

sncpProtectionCriteria  OBJECT IDENTIFIER ::= 
{ g774-04StandardSpecificExtension 0 }

sncpPathTraceMismatchCriteria   OBJECT IDENTIFIER ::= { sncpProtectionCriteria 1 }

sncpExcessiveErrorCriteria  OBJECT IDENTIFIER ::= { sncpProtectionCriteria 2 }

ConnectUnreliableTP ::= CHOICE {

notConnected	[0]	UnreliableTp,

connected 	[1]	UnreliableTp

}

HoldOffTime ::=	INTEGER

PointToPointProtection ::=  SEQUENCE {

protectedPointToPoint	PointToPoint,

protectingPointToPoint	ProtectingConnection

}

--  The object Instance of the xCon of PointToPoint represents the object
--  instance of the created connectionProtection

PointToMultipointProtection ::=  SEQUENCE {

protectedPointToMultipoint	PointToMultipoint,

protectingPointToMultipoint	ProtectingConnection

}

--  The object Instance of the xConnections and mpXCon of PointToMultipoint 
--  represents the object instances of the created crossConnections and 
--  mpConnectionProtection

--  The protectingPointToMultipoint represents the object instance of the 
--  unreliable TP and the object instance of the created mpConnectionProtection 
--  of the protecting connection

ProtectedAddLeg ::=  CHOICE {

protected		[0]	AddLeg,

unProtected	[1]	AddLeg

}

--  The object Instance of the mpXCon of AddLeg represents the object instance of 
--  the mpConnectionProtection to which new legs are added

ProtectedConnectInformation ::=  SEQUENCE OF SEQUENCE {

CHOICE	{

connect		[0]	ProtectedConnection,

addleg		[1]	ProtectedAddLeg,

addUnreliable	[2]	ConnectUnreliableTP

},

administrativeState	AdministrativeState OPTIONAL

}

ProtectedConnection ::=  SEQUENCE {

protectionConnectionType	ProtectionConnectionType,

protectingTP	ExplicitTP,

revertive		Boolean,

waitToRestore		[0]	Integer		OPTIONAL,

holdOffTime 		[1]	HoldOffTime	OPTIONAL

}

ProtectionConnectionType ::=  CHOICE {

unidirectional		[0]	ConnectionType,

bidirectional		[1]	ConnectionTypeBi

}

ProtectedConnectionResult ::=  SEQUENCE {

CHOICE	{

pointToPoint		[0]	PointToPointProtection,

pointToMultipoint	[1]	PointToMultipointProtection

},

connectionPG		ObjectInstance

}

ProtectedConnectResult ::=  SEQUENCE OF CHOICE {

failed		[0]	Failed,

protected		[1]	ProtectedConnectionResult,

unprotected	[2]	Unprotected

}

--  the n-th element in the "SEQUENCE OF" is related to the n-th element in the
--  "SEQUENCE OF" of the "ProtectedConnectInformation" type.

ProtectingConnection ::=  SEQUENCE {

unreliableTp	ObjectInstance,

conProt		ObjectInstance

}

ProtectionCriteria  ::=  SET OF Criterion

Criterion ::=  OBJECT IDENTIFIER

ProtectUnprotectInformation ::=  SEQUENCE OF SEQUENCE {

CHOICE {

protect	[3]	ProtectedConnection,

unprotect	[4]	UnprotectConnection

},

administrativeState  AdministrativeState OPTIONAL

}

Unprotected ::=  CHOICE {

uniform	[0]	Connected,

divers	[1]	UnprotectedMultiple

}

UnprotectedMultiple ::=  SEQUENCE {

sending		PointToPoint,

receiving		PointToPoint

}

UnprotectConnection ::=  CHOICE {

uniPToP		[0]	ExplicitPtoP,

pToMp		[1]	ExplicitPtoMP,

bidirectional	[2]	UnprotectBi

}

UnprotectBi ::=  CHOICE {

uniformRoute		[0]	ExplicitPtoP,

diverseRoute		[1]	UnprotectMultiple

}

UnprotectMultiple ::=  SEQUENCE {

firstXCon		ExplicitPtoP,

secondXCon	ExplicitPtoP

}

UnreliableTp ::=  SEQUENCE {

unreliableTp	ExplicitTP,

connectionProtection	ObjectInstance

}

END

Annex A

Inheritance and naming diagrams

Figure A.1/G.774.4  Inheritance subtree of subnetwork connection protection

Figure A.2/G.774.4  Containment tree of the subnetwork connection protection

Annex B

Examples of subnetwork connection protection

## B.1	Legends

In the following figures, the following legends and symbols will be used as described in Figure B.1.

Figure B.1/G.774.4  Legend

## B.2	Subnetwork connection protection examples

See Figures B.2 to B.7.

Figure B.2/G.774.4  Unidirectional cross-connection and connection protection modelized 
at the transmit end as a normal broadcast and at the receiving end 
by connection protection group

Figure B.3/G.774.4  Bidirectional cross-connection and connection protection (uniform route)

Figure B.4/G.774.4  Cross-connection and connection protection (diverse route)

Figure B.5/G.774.4  Unidirectional upstream point-to-multipoint cross-connection and connection protection (All the legs (TPBX, TPBY and TPBZ) are protected as a whole)

Figure B.6/G.774.4  Unidirectional upstream point-to-multipoint cross-connection and connection protection (Only the legs TPBY and TPBZ are protected)

Figure B.7/G.774.4  Unidirectional upstream point-to-multipoint cross-connection and connection protection (Only the legs TPBY and TPBZ are protected)

## B.3	Bidirectional subnetwork connection protection in rings

See Figure B.8.

Figure B.8/G.774.4  Bidirectional subnetwork connection protection

## B.4	Interconnection of protected rings by a cross-connect

See Figures B.9 and B.10.

Figure B.9/G.774.4  Interconnection of protected rings by a cross-connect

Figure B.10/G.774.4  Interconnection of protected rings by a cross-connect:
managed objects view

## B.5	Interconnections of subnetwork with ADMs (SNCP/SNCP) in SNCP interworking

See Figure B.11.

Figure B.11/G.774.4  Interconnection of subnetwork with ADMs (SNCP/SNCP)
in SNCP interworking

## B.6	Logical connections in SNCP interworking rings

See Figures B.12 and B.13.

Figure B.12/G.774.4  Logical connections in SNCP interworking rings connectivity view

Figure B.13/G.774.4  Logical connections in SNCP interworking rings: management view

## B.7	Logical connections in MS SHARED RINGs interworking

See Figures B.14 and B.15.

Figure B.14/G.774.4  Logical connections in MS SHARED RINGS interworking: 
connectivity view

Figure B.15/G.774.4  Logical connections in MS SHARED RINGS interworking: management view

## B.8	Logical connections in SNCPRing and MS SHARED RING interworking

See Figures B.16 and B.17.

Figure B.16/G.774.4  Logical connections in SNCP Ring and MS SHARED RING interworking: connectivity view

Figure B.17/G.774.4  Logical connections in SNCP Ring and MS SHARED RING interworking: management view

Annex C

State transition diagrams

## C.1	State diagrams

## C.2	Allowable values of switchStatus for a protected (mp) connection protection

| State | Protection status | Description of the state |
| --- | --- | --- |
| STA01 | noRequest | No request is currently active on the protected connection protection. The traffic is passed through on the protected connection protection. |
| STA02 | AutoSwitchSFPending | An automatic switch Signal Failure condition is active but could not be protected due to the unavailability of the protecting connection protection. |
| STA03 | AutoSwitchSFComplete | An automatic switch has been successfully completed to the protecting connection protection due to a Signal Failure condition on the protected connection protection. |
| STA04 | ForcedSwitchComplete | A forced switch has been completed successfully to the protecting connection protection. |
| STA05 | ForcedSwitchComplete-AutoSwitchSFPending | A forced switch to the protecting connection protection has been completed successfully. Additionally, an automatic switch Signal Failed condition is active on the protected connection protection. |
| STA06 | LockoutComplete | The protected connection protection has been locked out. |
| The following are additional values for 1 + 1 non-revertive systems |  |  |
| STA07 | DoNotRevert | The protected connection protection has been switched to the protecting connection protection and the request to do so has been released. The switch to the protecting connection protection is maintained. |
| The following are additional values for 1 + 1 revertive systems |  |  |
| STA08 | manualSwitchToProtectingComplete | The protected connection protection has completed a manual switch to the protecting connection protection. |
| STA09 | AutoSwitchCompleteWaitToRestore | An automatic switch has been successfully completed to the protecting connection protection. The switch condition has cleared on the protected connection protection, and the wait to restore timer is started for switching back (revertive systems only). |

## C.3	Allowable values of switchStatus for a protecting (mp) connection protection

| State | Protection status | Description of the state |
| --- | --- | --- |
| STA01 | noRequest | No request is present on the protecting connection protection. |
| STA02 | AutoSwitchSFToProtectingComplete | The protected connection protection has completed an automatic switch (SF) to the protecting connection protection. |
| STA03 | ForcedSwitchToProtectingComplete | The protected connection protection has completed a Forced Switch to the protecting connection protection. |
| STA04 | ForcedSwitchToProtecting-CompleteSFPresentOn-Protecting | The protected connection protection has completed a Forced Switch to the protecting connection protection. Additionally the protecting connection protection has an SF condition present (1 + 1 unidirectional systems only). |
| STA05 | LockoutComplete | The protecting connection protection has been locked out. |
| STA06 | AutomaticSwitchSFToProtectedPending | A Signal Failure condition is active on the protecting connection protection. |
| The following are additional values for 1 + 1 non-revertive systems |  |  |
| STA07 | DoNotRevert | The protected connection protection has been switched to the protecting connection protection and the request to do so has been released. The switch to the protecting connection protection is maintained. |
| The following are additional values for 1 + 1 revertive systems |  |  |
| STA08 | manualSwitchToProtectingComplete | The protected connection protection has completed a manual switch to the protecting connection protection. |
| STA09 | AutoSwitchCompleteWaitToRestore | An automatic switch has been successfully completed to the protecting connection protection. The switch condition has cleared on the protected connection protection, and the wait to restore timer is started for switching back (Only for Revertive Systems). |

## C.4	State diagram for a protected connection protection (non-revertive)

Condition

c1:	The request on the protected connection protection has a higher priority than the existing one on the protecting connection protection. (Refer to the protection switch decision table.)

c2:	There is an auto switch condition present on the protected connection protection.

c3:	The request has been completed.

c4:	There is an auto switch condition present on the protecting connection protection.

Process

p1:	Request switch to protecting connection protection.

p2: 	Release the current switch.

p3:	Notify switch report. (The protection switch report is sent by the protection group however the condition under which such notification is sent is specified in the connection protection state diagram using such process.)

States

| STA01 | noRequest |
| --- | --- |
| STA02 | AutoSwitchSFPending |
| STA03 | AutoSwitchSFComplete |
| STA04 | ForcedSwitchComplete |
| STA05 | ForcedSwitchCompleteAutoSwitchSFPending |
| STA06 | LockoutComplete |
| STA07 | DoNotRevert |

Event

| Req. Lock. | A lockout request has been issued on the protected connection protection. |
| --- | --- |
| Rel. Lock. | A release request of lockout has been issued on the protecting connection protection. |
| Req. Force | A Forced Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Force | A release request of Forced Switch has been issued on the protected connection protection. |
| Req. Man. | A Manual Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Man. | A Release Manual Switch request has been issued. As Manual Switch is not memorized, this will result as a failed request. |
| A-S Cond. | An automatic switch condition has occurred on the protected connection protection. |
| A-S Clr | The automatic switch condition on the protecting connection protection has disappeared. |
| P-ing Avail. | One or more requests (including Lockout request) have disappeared on the protecting connection protection, so that the protecting connection protection becomes available for use in the protection scheme (i.e. the highest priority request that could remain on the protecting connection protection has lower priority than existing ones on the protected connection protection). |
| P-ing Unav. | One or more requests (including Lockout request) have appeared on the protecting connection protection, so that the protecting connection protection becomes unavailable for use in the protection scheme (i.e. the highest priority request on the protecting connection protection has higher priority than existing ones on the protected connection protection). |
| Man. P-ing | A Manual Switch request has been issued to switch from the protecting connection protection to the protected connection protection. |
| Set Rev. | The connection protection group mode has been changed from non-revertive mode to revertive mode. |

| State diagram for a protected connection protection in a 1 + 1 non-revertive system |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Event | STA01 No Req. | STA02 A-S Pend. | STA03 A-S Comp. | STA04 F-S Comp. | STA05 F-S Comp A-S Pend. | STA06 LockComp (Note 1) | STA07 Do not revert |
| Req. Lock. | p3:STA06 | p3:STA06 | p2,p3: STA06 | p2,p3: STA06 | p2,p3: STA06 | STA06 | p2,p3: STA06 |
| Rel. Lock. | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | c1&c2&c3 p1,p3:STA03 | STA07 (Note 2) |
|  |  |  |  |  |  | (^c1V^c3)& c2:p3:STA02 |  |
|  |  |  |  |  |  | ^c2:p3:STA01 |  |
| Req. Force | c1&c3:p1, p3:STA04 | c1&c3:p1, p3:STA05 | p3:STA05 | STA04 | STA05 | STA06 (Note 2) | p3:STA04 |
|  | ^c1V^c3: STA01 (Note 2) | ^c1V^c3: STA02 (Note 2) |  |  |  |  |  |
| Rel. Force | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | p3:STA07 | c1&c3:p3: STA03 | STA06 (Note 2) | STA07 (Note 2) |
|  |  |  |  |  | ^c1V^c3:p2, p3: STA02 |  |  |
| Req. Man. | c1&c3:p1, p3:STA07 | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | STA07 |
|  | ^c1V^c3: STA01 (Note 2) |  |  |  |  |  |  |
| Rel. Man. | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | STA07 (Note 2) |
| A-S Cond. | c1&c3:p1, p3:STA03 | c1&c3:p1, p3:STA03 | STA03 | STA05 | STA05 | STA06 | p3:STA03 |
|  | ^c1V^c3: p3:STA02 | ^c1V^c3: STA02 |  |  |  |  |  |
| A-S Clr | – | ^c2:p3: STA01 | ^c2 &^c4: p3:STA07 | – | ^c2:STA04 | STA06 | – |
|  |  |  | ^c2 &c4:p2, p3:STA01 |  | c2:STA05 |  | – |
|  |  | c2:STA02 | c2&^c1:p2, p3:STA02 |  |  |  |  |
|  |  |  | c2&c1: STA03 |  |  |  |  |
| P-ing Unav. | STA01 | STA02 | p2,p3: STA02 | p2,p3: STA01 | p2,p3: STA02 | STA06 | p2,p3: STA01 |
| P-ing Avail. | STA01 | c3:p1,p3: STA03 | – | – | – | STA06 | – |
|  |  | ^c3:STA02 |  |  |  |  |  |
| Man. P-ing | STA01 | – | – | – | – | – | p2,p3: STA01 |
| Set Rev. | STA01 (Note 3) | STA02 (Note 3) | STA03 (Note 3) | STA04 (Note 3) | STA05 (Note 3) | STA06 (Note 3) | p2,p3: STA01 (Note 3) |
| NOTE 1  An automatic switch condition may be pending. NOTE 2  It results as a failed request. NOTE 3  The state table to be considered after having changed the state is the state table of the revertive system. |  |  |  |  |  |  |  |

## C.5	State diagram for a protecting connection protection (non-revertive)

Condition

c1:	The request on the protected connection protection has a higher priority than the existing one on the protecting connection protection. (Refer to the protection switch decision table.)

c2:	There is an auto switch condition present on the protected connection protection.

c3:	The request has been completed.

c4:	There is an auto switch condition present on the protecting connection protection.

Process

p1:	Request switch to protecting connection protection.

p2:	Release switch.

p3:	Notify switch report. (The protection switch report is sent by the protection group; however, the condition under which such notification is sent is specified in the connection protection state diagram using such process.)

Event

| Req. Lock. | A lockout request has been issued on the protecting connection protection. |
| --- | --- |
| Rel. Lock. | A release request of lockout has been issued on the protecting connection protection. |
| Req. Force | A Forced Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Force | A release request of Forced Switch has been issued on the protected connection protection. |
| Req. Man. | A Manual Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Man. | A Release Manual Switch request has been issued. As Manual Switch is not memorized, this will result as a failed request. |
| A-S Cond. | An automatic switch condition has occurred on the protecting connection protection. |
| A-S Clr | The automatic switch condition on the protecting connection protection has disappeared. |
| P-ted Lock | The protected connection protection has been locked out. |
| P-ted Avail. | One or more A-S requests have disappeared on the protected connection protection, so that the protected connection protection becomes available for use in the protection scheme (i.e. the highest priority request that could remain on the protected connection protection has lower priority than existing ones on the protecting connection protection). |
| P-ted Unav. | One or more A-S requests have appeared on the protected connection protection, so that the protected connection protection becomes unavailable for use in the protection scheme (i.e. the highest priority request on the protected connection protection has higher priorities than existing ones on the protected connection protection). |
| Set Rev. | The connection protection group mode has been changed from non-revertive to revertive mode. |
| Req. Man. To P-ted | A Manual Switch request has been issued to switch from the protecting connection protection to the protected connection protection. |

States

| STA01 | noRequest |
| --- | --- |
| STA02 | AutoSwitchSFToProtectingComplete |
| STA03 | ForcedSwitchToProtectingComplete |
| STA04 | ForcedSwitchToProtectingCompleteSFPresentOnProtecting |
| STA05 | LockoutComplete |
| STA06 | AutomaticSwitchSFToProtectedPending |
| STA07 | DoNotRevert |

| State diagram for a protected connection protection in a 1 + 1 non-revertive system |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Event | STA01 No Req. | STA02 A-S Comp. | STA03 F-S Comp. | STA04 F-S Comp. A-S Pend. | STA05 LockComp (Note 1) | STA06 A-S Pend. | STA07 Do Not Revert |
| Req. Lock. | p3:STA05 | p2,p3: STA05 | p2,p3: STA05 | p2,p3: STA05 | STA05 | p3: STA05 | p2,p3: STA05 |
| Rel. Lock. | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | ^c4:STA01 | STA06 (Note 2) | STA07 (Note 2) |
|  |  |  |  |  | c4: STA06 |  |  |
| Req. Force | p1,p3: STA03 | p3:STA03 | STA03 | STA04 | STA05 (Note 2) | p1,p3: STA04 | p3:STA03 |
| Rel. Force | STA01 (Note 2) | STA02 (Note 2) | ^c2:p3: STA07 | ^c1:p3: STA02 | STA05 (Note 2) | STA06 (Note 2) | STA07 (Note 2) |
|  |  |  | c2:p3: STA02 | ^c1:p2,p3: STA06 |  |  |  |
| Req. Man. | p1,p3: STA07 | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | STA07 |
| Rel. Man. | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | STA07 (Note 2) |
| A-S Cond. | STA06 | c1:STA02 | STA04 | STA04 | STA05 | STA06 | p2,p3: |
|  |  | ^c1:p2,p3: STA06 |  |  |  |  | STA06 |
| A-S Clr | – | STA02 | – | c4:STA04 | STA05 | c1:p1,p3: STA02 | – |
|  |  |  |  | ^c4:STA03 |  | ^c1^c4:p3: STA01 |  |
|  |  |  |  |  |  | ^c1&c4: STA06 |  |
| P-ted Lock | STA01 | ^c4:p2,p3: STA01 | p2,p3: STA01 | p2,p3: STA06 | STA05 | STA06 | p2,p3: STA01 |
|  |  | c4:p2,p3: STA06 |  |  |  |  |  |
| P-ted Unav. | c3:p1,p3: STA02 | STA02 | STA03 | STA04 | STA05 | c1&c3:p1, p3:STA02 | p3:STA02 |
|  | ^c3:STA01 |  |  |  |  | ^c1V^c3: STA06 |  |
| P-ted Avail. | STA01 | ^c4:p3 STA07 | STA03 | STA04 | STA05 | STA06 | STA07 |
|  |  | c4:p2,p3: STA06 |  |  |  |  |  |
| Req. Man. To P-ted | STA01 | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | p2,p3: STA01 |
| Set Rev. | STA01 (Note 3) | STA02 (Note 3) | STA03 (Note 3) | STA04 (Note 3) | STA05 (Note 3) | STA06 (Note 3) | p2,p3: STA01 (Note 3) |
| NOTE 1  An automatic switch condition may be pending. NOTE 2  It results as a failed request. NOTE 3  The state table to be considered after having changed the state is the state table of the revertive system. |  |  |  |  |  |  |  |

## C.6	State diagram for a protected connection protection (revertive)

Condition

c1:	The request on the protected connection protection has a higher priority than the existing one on the protecting connection protection. (Refer to the protection switch decision table.)

c2:	There is an auto switch condition present on the protected connection protection.

c3:	The request has been completed.

c4:	There is an auto switch condition present on the protecting connection protection.

Process

p1:	Request switch to protecting connection protection.

p2:	Release the current switch.

p3:	Notify switch report. (The protection switch report is sent by the protection group; however, the condition under which such notification is sent is specified in the connection protection state diagram using such process.)

p4:	Start wait to restore timer.

p5:	Stop wait to restore timer.

States

| STA01 | noRequest |
| --- | --- |
| STA02 | AutoSwitchSFPending |
| STA03 | AutoSwitchSFComplete |
| STA04 | ForcedSwitchComplete |
| STA05 | ForcedSwitchCompleteAutoSwitchSFPending |
| STA06 | LockoutComplete |
| STA08 | manualSwitchToProtectingComplete |
| STA09 | AutoSwitchCompleteWaitToRestore |

Event

| Req. Lock. | A lockout request has been issued on the protected connection protection. |
| --- | --- |
| Rel. Lock. | A release request of lockout has been issued on the protected connection protection. |
| Req. Force | A Forced Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Force | A release request of Forced Switch has been issued on the protected connection protection. |
| Req. Man. | A Manual Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Man. | A Release of Manual Switch request has been issued. |
| A-S Cond. | An automatic switch condition has occurred on the protected connection protection. |
| A-S Clr | The automatic switch condition has disappeared. |
| P-ing Avail. | One or more requests (including Lockout request) have disappeared on the protecting connection protection, so that the protecting connection protection becomes available for use in the protection scheme (i.e. the highest priority request that could remain on the protecting connection protection has lower priority than existing ones on the protected connection protection). |
| P-ing Unav. | One or more requests (including Lockout request) have appeared on the protecting connection protection, so that the protecting connection protection becomes unavailable for use in the protection scheme (i.e. the highest priority request on the protecting connection protection has higher priority than existing ones on the protected connection protection). |
| WTR | The Wait-to-Restore Time has elapsed. |
| Set Non-Rev. | The connection protection group mode has been changed from revertive to non-revertive mode. |

| State diagram for a protected connection protection in a 1 + 1 revertive system |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Event | STA01 No Req. | STA02 A-S Pend. | STA03 A-S Comp. | STA04 F-S Comp. | STA05 F-S Comp A-S Pend. | STA06 LockComp (Note 1) | STA08 M-S Comp. | STA09 A-S Comp WTR |
| Req. Lock. | p3:STA06 | p3:STA06 | p2,p3: STA06 | p2,p3: STA06 | p2,p3: STA06 | STA06 | p2,p3: STA06 | p2,p3,p5: STA06 |
| Rel. Lock. | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | c1&c2&c3 p1,p3: STA03 | STA08 (Note 2) | STA09 (Note 2) |
|  |  |  |  |  |  | (^c1V^c3)& c2:p3: STA02 |  |  |
|  |  |  |  |  |  | ^c2:p3: STA01 |  |  |
| Req. Force | c1&c3:p1, p3:STA04 | c1&c3:p1, p3:STA05 | p3:STA05 | STA04 | STA05 | STA06 (Note 2) | p3:STA04 | p3,p5: STA04 |
|  | ^c1V^c3: STA01 (Note 2) | ^c1V^c3: STA02 (Note 2) |  |  |  |  |  |  |
| Rel. Force | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | p2,p3: STA01 | c1:p3: STA03 | STA06 (Note 2) | STA08 (Note 2) | STA09 (Note 2) |
|  |  |  |  |  | ^c1:p2,p3: STA02 |  |  |  |
| Req. Man. | c1&c3:p1, p3:STA08 | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | STA08 | p3,p5: STA08 |
|  | ^c1V^c3: STA01 (Note 2) |  |  |  |  |  |  |  |
| Rel. Man. | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | p2,p3: STA01 | STA09 (Note 2) |
| A-S Cond. | c1&c3:p1, p3:STA03 | c1&c3:p1, p3:STA03 | STA03 | STA05 | STA05 | STA06 | p3:STA03 | p3,p5: STA03 |
|  | ^c1V^c3: p3:STA02 | ^c1V^c3: STA02 |  |  |  |  |  |  |
| A-S Clr | – | ^c2:p3: STA01 | ^c2 &^c4: p3,p4: STA09 | – | ^c2:STA04 | STA06 | – | – |
|  |  |  | ^c2 &c4:p2, p3:STA01 |  |  |  |  |  |
|  |  | c2:STA02 | c2&^c1:p2, p3:STA02 |  | c2:STA05 |  |  |  |
|  |  |  | c2&c1: STA03 |  |  |  |  |  |
| P-ing Unav. | STA01 | STA02 | p2,p3: STA02 | p2,p3: STA01 | p2,p3: STA02 | STA06 | p2,p3: STA01 | p2,p3,p5: STA01 |
| P-ing Avail. | STA01 | c3:p1,p3: STA03 | – | – | – | STA06 | – | – |
|  |  | ^c3:STA02 |  |  |  |  |  |  |
| WTR | – | – | – | – | – | – | – | p2,p3: STA01 |
| Set Non-Rev. | STA01 (Note 3) | STA02 (Note 3) | STA03 (Note 3) | STA04 (Note 3) | STA05 (Note 3) | STA06 (Note 3) | p3:STA07 (Notes 3, 4) | p3,p5: STA07 (Notes 3, 4) |
| NOTE 1  An automatic switch condition may be pending. NOTE 2  It results as a failed request. NOTE 3  The state table to be considered after changing the state is the state table of the non-revertive system. NOTE 4  This state pertains to the state diagram for a protected connection protection in a 1 + 1 non-revertive system. |  |  |  |  |  |  |  |  |

## C.7	State diagram for a protecting connection protection (revertive)

Condition

c1:	The request on the protected connection protection has a higher priority than the existing one on the protecting connection protection. (Refer to the protection switch decision table.)

c2:	There is an auto switch condition present on the protected connection protection.

c3:	The request has been completed.

c4:	There is an auto switch condition present on the protecting connection protection.

Process

p1:	Request switch to protecting connection protection.

p2:	Release switch.

p3:	Notify switch report. (The protection switch report is sent by the protection group; however, the condition under which such notification is sent is specified in the connection protection state diagram using such process.)

p4:	Start wait to restore timer.

p5:	Stop wait to restore timer.

Event

| Req. Lock. | A lockout request has been issued on the protecting connection protection. |
| --- | --- |
| Rel. Lock. | A release request of lockout has been issued on the protecting connection protection. |
| Req. Force | A Forced Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Force | A release request of Forced Switch has been issued on the protected connection protection. |
| Req. Man. | A Manual Switch request has been issued to switch from the protected connection protection to the protecting connection protection. |
| Rel. Man. | A Release of Manual Switch request has been issued. |
| A-S Cond. | An automatic switch condition has occurred on the protecting connection protection. |
| A-S Clr | The automatic switch condition on the protecting connection protection has disappeared. |
| P-ted Lock | The protected connection protection has been locked out. |
| P-ted Avail. | One or more A-S requests have disappeared on the protected connection protection, so that the protected connection protection becomes available for use in the protection scheme (i.e. the highest priority request that could remain on the protected connection protection has lower priority than existing ones on the protecting connection protection). |
| P-ted Unav. | One or more A-S requests have appeared on the protected connection protection, so that the protected connection protection becomes unavailable for use in the protection scheme (i.e. the highest priority request on the protected connection protection has higher priorities than existing ones on the protected connection protection). |
| WTR | The Wait-to-Restore Time has elapsed. |
| Set Non-Rev. | The connection protection group mode has been changed from revertive to non-revertive mode. |

States

| STA01 | noRequest |
| --- | --- |
| STA02 | AutoSwitchSFToProtectingComplete |
| STA03 | ForcedSwitchToProtectingComplete |
| STA04 | ForcedSwitchToProtectingCompleteSFPresentOnProtecting |
| STA05 | LockoutComplete |
| STA06 | AutomaticSwitchSFPending |
| STA08 | manualSwitchToProtectingComplete |
| STA09 | AutoSwitchCompleteWaitToRestore |

| State diagram for a protecting connection protection in a 1 + 1 revertive system |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Event | STA01 No Req. | STA02 A-S Comp. | STA03 F-S Comp. | STA04 F-S Comp. A-S Pend. | STA05 F-S Comp Comp (Note 1) | STA06 A-S Pend. | STA08 M-S Comp. | STA09 A-S Comp WTR |
| Req. Lock. | p3:STA05 | p2,p3: STA05 | p2,p3: STA05 | p2,p3: STA05 | STA05 | p3: STA05 | p2,p3: STA06 | p2,p3,p5: STA06 |
| Rel. Lock | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | ^c4:STA01 | STA06 (Note 2) | STA08 (Note 2) | STA09 (Note 2) |
|  |  |  |  |  | c4: STA06 |  |  |  |
| Req. Force | p1,p3: STA03 | p3:STA03 | STA03 | STA04 | STA05 (Note 2) | p1,p3: STA04 | p3: STA03 | p3,p5: STA03 |
| Rel. Force | STA01 (Note 2) | STA02 (Note 2) | p2,p3: STA01 | p2,p3: STA06 | STA05 (Note 2) | STA06 (Note 2) | STA08 (Note 2) | STA09 (Note 2) |
| Req. Man. | p1,p3: STA08 | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | STA08 | p3,p5: STA08 |
| Rel. Man. | STA01 (Note 2) | STA02 (Note 2) | STA03 (Note 2) | STA04 (Note 2) | STA05 (Note 2) | STA06 (Note 2) | p2,p3: STA01 | STA09 (Note 2) |
| A-S Cond. | STA06 | c1:STA02 | STA04 | STA04 | STA05 | STA06 | p2,p3: STA06 | p2,p3,p5: STA06 |
|  |  | ^c1:p2,p3:STA06 |  |  |  |  |  |  |
| A-S Clr | – | STA02 | – | c4:STA04 | STA05 | c1:p1,p3: STA02 | – | – |
|  |  |  |  | ^c4:STA03 |  | ^c1&^c4: p3:STA01 |  |  |
|  |  |  |  |  |  | ^c1&c4: STA06 |  |  |
| P-ted Lock | STA01 | ^c4: p2,p3: STA01 | p2,p3: STA01 | p2,p3: STA06 | STA05 | STA06 | p2,p3: STA01 | p2,p3,p5: STA01 |
|  |  | c4:p2,p3: STA06 |  |  |  |  |  |  |
| P-ted Unav. | c3:p1,p3: STA02 | STA02 | STA03 | STA04 | STA05 | c1&c3:p1, p3:STA02 | p3: STA02 | p3,p5: STA02 |
|  | ^c3: STA01 |  |  |  |  | ^c1V^c3: STA06 |  |  |
| P-ted Avail. | STA01 | ^c4:p2,p3:STA06 | STA03 | STA04 | STA05 | STA06 | STA08 | – |
|  |  | c4:p2,p3, p4:STA09 |  |  |  |  |  |  |
| WTR | – | – | – | – | – | – | – | p2,p3: STA01 |
| Set Non-Rev. | STA01 (Note 3) | STA02 (Note 3) | STA03 (Note 3) | STA04 (Note 3) | STA05 (Note 3) | STA06 (Note 3) | p3:STA07 (Notes 3, 4) | p3,p5: STA07 (Notes 3, 4) |
| NOTE 1  An automatic switch condition may be pending. NOTE 2  It results as a failed request. NOTE 3  The state table to be considered after having changed the state is the state table of the revertive system. NOTE 4  This state pertains to the state diagram for a protected connection protection in a 1 + 1 non-revertive system. |  |  |  |  |  |  |  |  |

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |