|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.5 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH)  Management of connection supervision functionality (HCS/LCS) for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.5 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.5 Synchronous digital hierarchy (SDH)  Management of connection supervision functionality (HCS/LCS) for the network element view

- Summary This Recommendation provides an information model for the Synchronous Digital Hierarchy (SDH) Network. This model describes the managed object classes and their properties for the management of Connection Supervision Functionality (HCS/LCS), as defined in ITU-T G.783 and as related to SDH network Elements. These objects are useful to describe information exchanged across interfaces defined in ITU-T M.3010 Telecommunications Management Network (TMN) architecture.

Document History. Issue; Notes. 2001; First revision incorporated the changes documented in the G.774.5 Corrigendum 1.. 7/1995; Initial version of the Recommendation..

- Source ITU-T Recommendation G.774.5 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and definitions		3

4	Abbreviations		3

5	Connection supervision information model		3

5.1	Overview		3

5.2	Requirements		4

6	Object classes		4

6.1	Administrative unit 4 CTP bidirectional with HCS		5

6.2	Administrative unit 4 CTP sink with HPOM		5

6.3	Administrative unit 4 CTP source with HUG		5

6.4	Administrative unit 3 CTP bidirectional with HCS		5

6.5	Administrative unit 3 CTP sink with HPOM		5

6.6	Administrative unit 3 CTP source with HUG		5

6.7	Tributary unit 3 CTP bidirectional with LCS		5

6.8	Tributary unit 3 CTP sink with LPOM		6

6.9	Tributary unit 3 CTP source with LUG		6

6.10	Tributary unit 2 CTP bidirectional with LCS		6

6.11	Tributary unit 2 CTP sink with LPOM		6

6.12	Tributary unit 2 CTP source with LUG		6

6.13	Tributary unit 12 CTP bidirectional with LCS		6

6.14	Tributary unit 12 CTP sink with LPOM		6

6.15	Tributary unit 12 CTP source with LUG		7

6.16	Tributary unit 11 CTP bidirectional with LCS		7

6.17	Tributary unit 11 CTP sink with LPOM		7

6.18	Tributary unit 11 CTP source with LUG		7

7	Packages		7

7.1	Virtual container 11-2 supervision bidirectional package		8

7.2	Virtual container 11-2 supervision sink package		8

7.3	Virtual container 11-2 supervision source package		9

7.4	Virtual container 3-4 supervision bidirectional package		9

7.5	Virtual container 3-4 supervision sink package		9

7.6	Virtual container 3-4 supervision source package		10

Page

8	Attributes		11

8.1	Generator enabled		11

8.2	Monitor active		11

8.3	J1 Path trace receive		11

8.4	J1 Path trace send		11

8.5	J2 Path trace expected		11

8.6	J2 Path trace receive		12

8.7	J2 Path trace send		12

9	Actions		12

10	Notifications		12

11	Parameters		12

12	Name bindings		12

13	Constraint rules		14

14	Subordination rules		14

15	Supporting ASN.1 productions		14

Appendix I – Inheritance and naming diagrams		15

ITU-T Recommendation G.774.5

Synchronous digital hierarchy (SDH)  Management of connection supervision functionality (HCS/LCS) for the network element view

# 1	Scope

SDH Connection Supervision Functions are used to configure the supervision of higher and lower order path overhead independent from termination functions.

Configuration is done by modifications of attributes of the relevant managed objects. These attributes are included by subclassing of existing G.774 managed object Classes.

The new objects defined in this Recommendation supersede those defined in ITU-T G.774.5 (1995). For each object class, attribute, action, notification, parameter defined in this Recommendation it shall be indicated what the impacts upon the existing ITU-T G.774.5 (1995) are.

Structure of this Recommendation

Clause 5.1 provides an overview of the SDH Connection Supervision information model. Clauses 6 to 12 describe the information model using the mechanisms defined in X.722. Clause 15 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. Naming and Inheritance are illustrated in informative Appendix I.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy (SDH).

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774 (2001), Synchronous digital hierarchy (SDH)  Management information model for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunications management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview.

	ITU-T X.710 (1997), Information technology – Open Systems Interconnection – Common management information service.

	ITU-T X.711 (1997), Information technology – Open Systems Interconnection – Common management information protocol: Specification.

	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology – Open Systems Interconnection  Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology – Open Systems Interconnection – Systems Management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology – Open Systems Interconnection – Systems Management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology – Open Systems Interconnection – Systems Management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.734 (1992), Information technology – Open Systems Interconnection – Systems Management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd. 1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.735 (1992), Information technology – Open Systems Interconnection – Systems Management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

# 3	Terms and definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, ITU-T G.784 and ITU-T M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AIS	Alarm Indication Signal

CTP	Connection Termination Point

EBER	Excessive Bit Error Ratio

FERF	Far End Receive Failure

HCS	Higher order Connection Supervision

HPA	Higher order Path Adaptation

LCS	Lower order Connection Supervision

MSA	Multiplex Section Adaptation

NE	Network Element

OS	Operations System

SDH	Synchronous Digital Hierarchy

TMN	Telecommunications Management Network

TTP	Trail Termination Point

VC	Virtual Container

# 5	Connection supervision information model

## 5.1	Overview

5.1.1	HCS and LCS may be provided in the case of an open HPC respective LPC (unused connection). This case is modelled by connected AU3/4CTP respective TUxCTP. Therefore:

•	the HCS is modelled by "supervisedAU3/4CTP". These classes are subclasses of AU3/4CTP of ITU-T G.774 which currently model only the MSA function;

•	the LCS is modelled by "supervisedTU3/2/12/11CTP". These classes are subclasses of TU3/2/12/11CTP of ITU-T G.774 which currently model only the HPA function.

5.1.2	If an instance of an AU3/4CTP or TUxCTP has to be created (only possible as a consequence of the creation of a superior TTP or as a side effect of payload configuration – see ITU-T G.774.2) and the equipment is able to provide HCS/LCS functionality (see R3), instead of (ITU-T G.774) CTPs, the supervised CTPs should be created automatically.

5.1.3	The (de-)activation of HCS/LCS is modelled using the attribute generatorEnabled for the unequipped generator subfunction and the attribute monitorActive for the path overhead monitor subfunction. These attributes allow the independent management of both subfunctions (requirement R4).

5.1.4	The (de-)activation of HCS/LCS in the supervised CTP objects does not influence the behaviour derived from ITU-T G.774 (MSA, HPA). A SET-operation of generatorEnabled or of monitorActive to TRUE may be rejected depending upon the dynamic situation of the specific network element if it supports the connection supervision function with less than 100%. This results from requirement R5.

5.1.5	Performance measurement of the connection supervision functionality of supervised CTPs (Sink or Bidirectional) can be done using a subclass of currentData which is applicable for path termination (see ITU-T G.774.1). This subclass provides the parameters background block errors, errored seconds, severely errored seconds and unavailable seconds (some are optional and are also provided for the far end). If the attribute monitorActive of a CTP has the value FALSE during a part of a PM period the performance data is not reliable. This should be indicated by the attribute suspectIntervalFlag in the currentData object.

## 5.2	Requirements

The features of HCS/LCS (defined in ITU-T G.783) which influence the modelling approach are summarized below:

R1	The connection supervision function includes the monitoring of parts of HO/LO-POH to get alarms and performance information about the path segment (sub function H/L-POM (Path Overhead Monitor)).

R2	The connection supervision function includes the generation of supervisory unequipped HO/LO-POH with a Path Trace (sub function H/L-UG (Unequipped Generator)).

R3	The connection supervision function may be optional.

R4	The connection supervision function shall be able to be set into the INACTIVE state and the ACTIVE state (independent for HPOM and HUG respective LPOM and LUG).

R5	The connection supervision function may be supported for a specific equipment at the same time in the range of 0% to 100%.

# 6	Object classes

This clause provides replacement managed object class definitions for the existing ITU-T G.774.5 (1995). Any managed object class replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a managed object class are as follows:

1)	The replaced managed object class is faulty and must be fixed.

2)	The replaced managed object class includes an attribute, package, notification or action which has been re-registered in this or another Recommendation.

3)	The replaced managed object class inherits from a managed object class which has been re-registered in this or another Recommendation.

In each case where a class is replaced the new class will be registered within this Recommendation. The textual label for the class will be revised to include the text "R1". For example in the revision of the G.774.5 (1995) managed object class "au4SupervisedCTPBidirectional", the revised label will become "au4SupervisedCTPBidirectionalR1".

Below is a table of classes deprecated from ITU-T G.774.5 (1995) and the G.774.5 classes which replace them:

Deprecated G.774.5 (1995) Classes	Replacement G.774.5 Classes

au4SupervisedCTPBidirectional		au4SupervisedCTPBidirectionalR1

au4SupervisedCTPSink				au4SupervisedCTPSinkR1

au3SupervisedCTPBidirectional		au3SupervisedCTPBidirectionalR1

au3SupervisedCTPSink				au3SupervisedCTPSinkR1

tu3SupervisedCTPBidirectional		tu3SupervisedCTPBidirectionalR1

tu3SupervisedCTPSink				tu3SupervisedCTPSinkR1

tu2SupervisedCTPBidirectional		tu2SupervisedCTPBidirectionalR1

tu2SupervisedCTPSink				tu2SupervisedCTPSinkR1

tu12SupervisedCTPBidirectional		tu12SupervisedCTPBidirectionalR1

tu12SupervisedCTPSink				tu12SupervisedCTPSinkR1

tu11SupervisedCTPBidirectional		tu11SupervisedCTPBidirectionalR1

tu11SupervisedCTPSink				tu11SupervisedCTPSinkR1

## 6.1	Administrative unit 4 CTP bidirectional with HCS

au4SupervisedCTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM 	"Recommendation G.774":au4CTPBidirectionalR1,

au4SupervisedCTPSinkR1,

au4SupervisedCTPSource;

CHARACTERIZED BY

vc3-4SupervisionBidirectionalPackage;

REGISTERED AS {g774-05MObjectClass 19 };

## 6.2	Administrative unit 4 CTP sink with HPOM

au4SupervisedCTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM 	"Recommendation G.774":au4CTPSinkR1;

CHARACTERIZED BY

vc3-4SupervisionSinkPackageR1;

REGISTERED AS {g774-05MObjectClass 20 };

## 6.3	Administrative unit 4 CTP source with HUG

au4SupervisedCTPSource			MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774": au4CTPSource;

CHARACTERIZED BY

vc3-4SupervisionSourcePackage;

REGISTERED AS {g774-05MObjectClass 3};

## 6.4	Administrative unit 3 CTP bidirectional with HCS

au3SupervisedCTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":au3CTPBidirectionalR1,

au3SupervisedCTPSinkR1,

au3SupervisedCTPSource;

CHARACTERIZED BY

vc3-4SupervisionBidirectionalPackage;

REGISTERED AS {g774-05MObjectClass 21 };

## 6.5	Administrative unit 3 CTP sink with HPOM

au3SupervisedCTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM 	"Recommendation G.774":au3CTPSinkR1;

CHARACTERIZED BY

vc3-4SupervisionSinkPackageR1;

REGISTERED AS {g774-05MObjectClass 22 };

## 6.6	Administrative unit 3 CTP source with HUG

au3SupervisedCTPSource			MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774": au3CTPSource;

CHARACTERIZED BY

vc3-4SupervisionSourcePackage;

REGISTERED AS {g774-05MObjectClass 6};

## 6.7	Tributary unit 3 CTP bidirectional with LCS

tu3SupervisedCTPBidirectionalR1  MANAGED OBJECT CLASS

DERIVED FROM 	"Recommendation G.774":tu3CTPBidirectionalR1,

tu3SupervisedCTPSinkR1,

tu3SupervisedCTPSource;

CHARACTERIZED BY

vc3-4SupervisionBidirectionalPackage;

REGISTERED AS {g774-05MObjectClass 23 };

## 6.8	Tributary unit 3 CTP sink with LPOM

tu3SupervisedCTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation G.774":tu3CTPSinkR1;

CHARACTERIZED BY

vc3-4SupervisionSinkPackageR1;

REGISTERED AS {g774-05MObjectClass 24 };

## 6.9	Tributary unit 3 CTP source with LUG

tu3SupervisedCTPSource			MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774": tu3CTPSource;

CHARACTERIZED BY

vc3-4SupervisionSourcePackage;

REGISTERED AS {g774-05MObjectClass 9};

## 6.10	Tributary unit 2 CTP bidirectional with LCS

tu2SupervisedCTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM 	"Recommendation G.774":tu2CTPBidirectionalR1,

tu2SupervisedCTPSinkR1,
			tu2SupervisedCTPSource;

CHARACTERIZED BY

vc11-2SupervisionBidirectionalPackage;

REGISTERED AS {g774-05MObjectClass 25 };

## 6.11	Tributary unit 2 CTP sink with LPOM

tu2SupervisedCTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM 	"Recommendation G.774":tu2CTPSinkR1;

CHARACTERIZED BY

vc11-2SupervisionSinkPackageR1;

REGISTERED AS {g774-05MObjectClass 26 };

## 6.12	Tributary unit 2 CTP source with LUG

tu2SupervisedCTPSource			MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774": tu2CTPSource;

CHARACTERIZED BY

vc11-2SupervisionSourcePackage;

REGISTERED AS {g774-05MObjectClass 12};

## 6.13	Tributary unit 12 CTP bidirectional with LCS

tu12SupervisedCTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM 		"Recommendation G.774":tu12CTPBidirectionalR1,

tu12SupervisedCTPSinkR1,

tu12SupervisedCTPSource;

CHARACTERIZED BY

vc11-2SupervisionBidirectionalPackage;

REGISTERED AS {g774-05MObjectClass 27 };

## 6.14	Tributary unit 12 CTP sink with LPOM

tu12SupervisedCTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation G.774":tu12CTPSinkR1;

CHARACTERIZED BY

vc11-2SupervisionSinkPackageR1;
REGISTERED AS {g774-05MObjectClass 28 };

## 6.15	Tributary unit 12 CTP source with LUG

tu12SupervisedCTPSource		Managed Object Class

DERIVED FROM		"Recommendation G.774": tu12CTPSource;

CHARACTERIZED BY

vc11-2SupervisionSourcePackage;

REGISTERED AS {g774-05MObjectClass 15};

## 6.16	Tributary unit 11 CTP bidirectional with LCS

tu11SupervisedCTPBidirectionalR1 MANAGED OBJECT CLASS

DERIVED FROM		"Recommendation G.774":tu11CTPBidirectionalR1,

tu11SupervisedCTPSinkR1,

tu11SupervisedCTPSource;

CHARACTERIZED BY

vc11-2SupervisionBidirectionalPackage;

REGISTERED AS {g774-05MObjectClass 29 };

## 6.17	Tributary unit 11 CTP sink with LPOM

tu11SupervisedCTPSinkR1 MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation G.774":tu11CTPSinkR1;

CHARACTERIZED BY

vc11-2SupervisionSinkPackageR1;

REGISTERED AS {g774-05MObjectClass 30 };

## 6.18	Tributary unit 11 CTP source with LUG

tu11SupervisedCTPSource			MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774": tu11CTPSource;

CHARACTERIZED BY

vc11-2SupervisionSourcePackage;

REGISTERED AS {g774-05MObjectClass 18};

# 7	Packages

This clause provides replacement package definitions for the existing ITU-T G.774.5 (1995). Any package replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a package are as follows:

1)	The replaced package is faulty and must be fixed.

2)	The replaced package includes an attribute, package, notification or action which has been re-registered in this Recommendation.

In each case where a package is replaced, the new package will be registered within this Recommendation. The textual label for the package will be revised to include the text "R1". For example, in the revision of the G.774.5 (1995) package "vc11-2SupervisionSinkPackage", the revised label will become "vc11-2SupervisionSinkPackageR1".

Below is a table of packages deprecated from ITU-T G.774.5 (1995) and the G.774.5 packages which replace them:

Deprecated G.774.5 (1995) Packages	Replacement G.774.5 Packages

vc11-2SupervisionSinkPackage			vc11-2SupervisionSinkPackageR1

vc3-4SupervisionSinkPackage			vc3-4SupervisionSinkPackageR1

## 7.1	Virtual container 11-2 supervision bidirectional package

vc11-2SupervisionBidirectionalPackage PACKAGE

BEHAVIOUR	vc11-2SupervisionBidirectionalPackageBehaviour;

REGISTERED AS {g774-05Package 1};

vc11-2SupervisionBidirectionalPackageBehaviour BEHAVIOUR

DEFINED AS

*	A communicationAlarm notification shall be issued if a far end receive failure (V5 Byte) is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure).

If monitoring is suspended due to monitorActive an existing far end receive failure is cleared and removed from the current problem list.

If the (inherited) attributes monitorActive and generatorEnabled have the value TRUE and the upstreamConnectivityPointer has the value NULL (not connected), the originated supervisory unequipped VC can indicate in the V5 byte a far end receive failure according to the conditions given in the functional specification (Recommendation G.783: signal label mismatch, path trace mismatch, AIS or loss of pointer).

*;

## 7.2	Virtual container 11-2 supervision sink package

vc11-2SupervisionSinkPackageR1 PACKAGE

BEHAVIOUR	vc11-2SupervisionSinkPackageR1Behaviour;

ATTRIBUTES

monitorActive			GET-REPLACE,

"Recommendation G.774": v5SignalLabelExpected	GET-REPLACE,

"Recommendation G.774": v5SignalLabelReceive		GET,

j2PathTraceExpected REPLACE-WITH-DEFAULT  		DEFAULT VALUE SDH CSASN1.defaultNull GET-REPLACE,

j2PathTraceReceive		GET;
REGISTERED AS {g774-05Package 7 };

vc11-2SupervisionSinkPackageR1Behaviour BEHAVIOUR

DEFINED AS

*If a SDH-CTP has to be created and the underlying resource is able to provide connection supervision, a supervised CTP should be created. The attribute monitorActive of supervised CTPs does only influence the behaviour related to connection supervision. A SET-operation of monitorActive to TRUE may be rejected dependent from the dynamic situation of the specific network element. In case of rejection the "invalidAttributeValue" error should be returned. If the attribute monitorActive has the value TRUE, then the path overhead is monitored. A communicationAlarm notification shall be issued if the signal label received (V5 Byte) does not match the signal label expected. The probableCause parameter of the notification shall indicate signal label mismatch. A communicationAlarm notification shall be issued if the path trace received (J2 Byte) does not match the path trace expected. The probableCause parameter of the notification

shall indicate path trace mismatch.

If monitoring is suspended due to monitorActive, all outstanding alarms related to the connection supervision (see above) are cleared and removed from the current problem list. In this state the attributes v5SignalLabelReceive and j2PathTraceReceive may contain values which do not reflect the received signal and no alarms (see before) are emitted. If the attribute monitorActive has the value FALSE during a part of a PM period, the performance data is not reliable.

This should be indicated by the attribute suspectIntervalFlag of a possibly contained currentData object.

If the network element supports LCS with restricted capability, the local initial value of the attribute monitorActive should be FALSE.

*;

## 7.3	Virtual container 11-2 supervision source package

vc11-2SupervisionSourcePackage PACKAGE

BEHAVIOUR	vc11-2SupervisionSourcePackageBehaviour;

ATTRIBUTES

generatorEnabled		GET-REPLACE,

j2PathTraceSend		GET-REPLACE;

REGISTERED AS {g774-05Package 3};

vc11-2SupervisionSourcePackageBehaviour BEHAVIOUR

DEFINED AS

*	If an SDH-CTP has to be created and the underlying resource is able to provide connection supervision, a supervised CTP should be created. The attribute generatorEnabled of supervised CTPs does only influence the behaviour related to connection supervision. A SET-operation of generatorEnabled to TRUE may be rejected dependent from the dynamic situation of the specific network element. In case of rejection, the “invalidAttributeValue” error should be returned.

If the supervised CTP is not connected (the upstreamConnectivityPointer has the value NULL) and the attribute generatorEnabled has the value TRUE, then a supervisory unequipped signal with the current value of j2PathTraceSend is originated.

If the supervised CTP is connected (the upstreamConnectivityPointer does not have the value NULL), this package does not influence the transmitted VC (independent from generatorEnabled).

If the network element supports LCS with restricted capability, the local initial value of the attribute generatorEnabled should be FALSE.

*;

## 7.4	Virtual container 3-4 supervision bidirectional package

vc3-4SupervisionBidirectionalPackage PACKAGE

BEHAVIOUR	vc3-4SupervisionBidirectionalPackageBehaviour;

REGISTERED AS {g774-05Package 4};

vc3-4SupervisionBidirectionalPackageBehaviour BEHAVIOUR

DEFINED AS

*	A communicationAlarm notification shall be issued if a far end receive failure (G1 Byte) is detected. The probableCause parameter of the notification shall indicate FERF (Far End Receive Failure). If monitoring is suspended due to monitorActive, an existing far end receive failure is cleared and removed from the current problem list.

If the (inherited) attributes monitorActive and generatorEnabled have the value TRUE and the upstreamConnectivityPointer has the value NULL (not connected), the originated supervisory unequipped VC can indicate in the G1 byte a far end receive failure according to the conditions given in the functional specification (Recommendation G.783: signal label mismatch, path trace mismatch, AIS or loss of pointer).

*;

## 7.5	Virtual container 3-4 supervision sink package

vc3-4SupervisionSinkPackageR1 PACKAGE

BEHAVIOUR   vc3-4SupervisionSinkPackageR1Behaviour;

ATTRIBUTES

monitorActive			GET-REPLACE,

"Recommendation G.774": c2SignalLabelExpected	GET-REPLACE,

"Recommendation G.774": c2SignalLabelReceive		GET,

"Recommendation G.774": j1PathTraceExpected

REPLACE-WITH-DEFAULT

DEFAULT VALUE SDHCSASN1.defaultNull

GET-REPLACE,

j1PathTraceReceive		GET;

REGISTERED AS {g774-05Package 8 };

vc3-4SupervisionSinkPackageR1Behaviour BEHAVIOUR

DEFINED AS

*	If a SDH-CTP has to be created and the underlying resource is able to provide connection supervision, a supervised CTP should be created.

The attribute monitorActive of supervised CTPs does only influence the behaviour related to connection supervision. A SET-operation of monitorActive to TRUE may be rejected dependent from the dynamic situation of the specific network element. In case of rejection the "invalidAttributeValue" error should be returned.

If the attribute monitorActive has the value TRUE, then the path overhead is monitored. A communicationAlarm notification shall be issued if the signal label received (C2 Byte) does not match the signal label expected. The probableCause parameter of the notification shall indicate signal label mismatch. A communicationAlarm notification shall be issued if the path trace received (J1 Byte) does not match the path trace expected. The probableCause parameter of the notification shall indicate path trace mismatch. If monitoring is suspended due to the attribute monitorActive, all outstanding alarms related to the connection supervision (see above) are cleared and removed from the current problem list. In this state the attributes c2SignalLabelReceive and j1PathTraceReceive may contain values which do not reflect the received signal and no alarms (see before) are emitted. If the attribute monitorActive has the value FALSE during a part of a PM period, the performance data is not reliable. This should be indicated by the attribute suspectIntervalFlag of a possibly contained currentData object. If the network element supports HCS with restricted capability, the local initial value of the attribute monitorActive should be FALSE.

*;

## 7.6	Virtual container 3-4 supervision source package

vc3-4SupervisionSourcePackage PACKAGE

BEHAVIOUR		vc3-4SupervisionSourcePackageBehaviour;

ATTRIBUTES

generatorEnabled		GET-REPLACE,

j1PathTraceSend		GET-REPLACE;

REGISTERED AS {g774-05Package 6};

vc3-4SupervisionSourcePackageBehaviour BEHAVIOUR

DEFINED AS

*	If an SDH-CTP has to be created and the underlying resource is able to provide connection supervision, a supervised CTP should be created.

The attribute generatorEnabled of supervised CTPs does only influence the behaviour related to connection supervision. A SET-operation of generatorEnabled to TRUE may be rejected dependent from the dynamic situation of the specific network element. In case of rejection, the “invalidAttributeValue” error should be returned.

If the supervised CTP is not connected (the upstreamConnectivityPointer has the value NULL) and the attribute generatorEnabled has the value TRUE, then a supervisory unequipped signal with the current value of j1PathTraceSend is originated.

If the supervised CTP is connected (the upstreamConnectivityPointer does not have the value NULL), this package does not influence the transmitted VC (independent from generatorEnabled).

If the network element supports HCS with restricted capability, the local initial value of the attribute generatorEnabled should be FALSE.

*;

# 8	Attributes

## 8.1	Generator enabled

generatorEnabled  ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHCSASN1.Boolean;

MATCHES FOR	EQUALITY;

REGISTERED AS {g774-05Attribute 1};

## 8.2	Monitor active

monitorActive  ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHCSASN1.Boolean;

MATCHES FOR	EQUALITY;

REGISTERED AS {g774-05Attribute 2};

## 8.3	J1 Path trace receive

j1PathTraceReceive	  ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHCSASN1.PathTraceRS;

MATCHES FOR	EQUALITY;

BEHAVIOUR

j1PathTraceReceiveBehaviour BEHAVIOUR

DEFINED AS

*	This attribute is used to indicate the value of the incoming J1 VC Path Trace byte message.*

;;

REGISTERED AS {g774-05Attribute 3};

## 8.4	J1 Path trace send

j1PathTraceSend  ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHCSASN1.PathTraceRS;

MATCHES FOR	EQUALITY;

BEHAVIOUR

j1PathTraceSendBehaviour BEHAVIOUR

DEFINED AS

*	This attribute is used to indicate the value of the outgoing J1 VC Path Trace byte message.*

;;

REGISTERED AS {g774-05Attribute 4};

## 8.5	J2 Path trace expected

j2PathTraceExpected  ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHCSASN1.PathTrace;

MATCHES FOR	EQUALITY;

BEHAVIOUR

j2PathTraceExpectedBehaviour BEHAVIOUR

DEFINED AS

*	This attribute is used to specify the value of the expected J2  VC Path Trace byte message. If the value of this attribute is set to NULL then any Received Path Trace shall be considered to match.*

;;

REGISTERED AS {g774-05Attribute 5};

## 8.6	J2 Path trace receive

j2PathTraceReceive  ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHCSASN1.PathTraceRS;

MATCHES FOR	EQUALITY;

BEHAVIOUR

j2PathTraceReceiveBehaviour BEHAVIOUR

DEFINED AS

*	This attribute is used to indicate the value of the incoming J2  VC Path Trace byte message.*

;;

REGISTERED AS {g774-05Attribute 6};

## 8.7	J2 Path trace send

j2PathTraceSend  ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHCSASN1.PathTraceRS;

MATCHES FOR    EQUALITY;

BEHAVIOUR

j2PathTraceSendBehaviour BEHAVIOUR

DEFINED AS

*	This attribute is used to indicate the value of the outgoing J2  VC Path Trace byte message.*

;;

REGISTERED AS {g774-05Attribute 7};

# 9	Actions

None.

# 10	Notifications

None.

# 11	Parameters

None.

# 12	Name bindings

This clause provides replacement namebinding definitions for the existing ITU-T G.774.5 (1995). Any namebinding replaced by one in this clause is considered to be deprecated. The reasons for the replacement of a namebinding are as follows:

1)	The replaced namebinding is faulty and must be fixed.

2)	The replaced namebinding refers to a superior managed object class which has been re-registered in this Recommendation.

3)	The replaced namebinding refers to a subordinate managed object class which has been re-registered in this Recommendation.

4)	The replaced namebinding refers to a naming attribute which has been re-registered in this Recommendation.

In each case where a namebinding is replaced, the new namebinding will be registered within this Recommendation. The textual label for the namebinding will be revised to include the text "R1". For example, in the revision of the G.774.5 (1995) namebinding "pathTerminationCurrentData-au4SupervisedCTPSink", the revised label will become "pathTerminationCurrentData-

au4SupervisedCTPSinkR1". Note the "R1" is placed immediately following the revised class which impacts the namebinding.

Below is a table of namebindings deprecated from ITU-T G.774.5 (1995) and the G.774.5 namebindings which replace them:

Deprecated G.774.5 (1995) Namebindings

pathTerminationCurrentData-au4SupervisedCTPSink

pathTerminationCurrentData-au3SupervisedCTPSink

pathTerminationCurrentData-tu3SupervisedCTPSink

pathTerminationCurrentData-tu2SupervisedCTPSink

pathTerminationCurrentData-tu12SupervisedCTPSink

pathTerminationCurrentData-tu11SupervisedCTPSink

Replacement G.774.5 Namebindings

pathTerminationCurrentData-au4SupervisedCTPSinkR1

pathTerminationCurrentData-au3SupervisedCTPSinkR1

pathTerminationCurrentData-tu3SupervisedCTPSinkR1

pathTerminationCurrentData-tu2SupervisedCTPSinkR1

pathTerminationCurrentData-tu12SupervisedCTPSinkR1

pathTerminationCurrentData-tu11SupervisedCTPSinkR1

For performance monitoring, name bindings which have supervised CTPs (Sink or Bidirectional) are defined as superior class and the subclass of currentData for path termination as subordinate class (defined in ITU-T G.774.1).

pathTerminationCurrentData-au4SupervisedCTPSinkR1		NAME  BINDING

SUBORDINATE OBJECT CLASS

"Recommendation G.774.01":pathTerminationCurrentData	AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS au4SupervisedCTPSinkR1		AND SUBCLASSES;

WITH ATTRIBUTE		"Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-05NameBinding 7};

pathTerminationCurrentData-au3SupervisedCTPSinkR1 NAME  BINDING

SUBORDINATE OBJECT CLASS

"Recommendation G.774.01":pathTerminationCurrentData	AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS	au3SupervisedCTPSinkR1		AND SUBCLASSES;

WITH ATTRIBUTE 	"Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-05NameBinding 8};

pathTerminationCurrentData-tu3SupervisedCTPSinkR1		NAME  BINDING

SUBORDINATE OBJECT CLASS

"Recommendation G.774.01":pathTerminationCurrentData	AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS	tu3SupervisedCTPSinkR1		AND SUBCLASSES;

WITH ATTRIBUTE		"Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-05NameBinding 9};

pathTerminationCurrentData-tu2SupervisedCTPSinkR1	NAME  BINDING

SUBORDINATE OBJECT CLASS

"Recommendation G.774.01":pathTerminationCurrentData	AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS  tu2SupervisedCTPSinkR1		AND SUBCLASSES;

WITH ATTRIBUTE		"Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-05NameBinding 10};

pathTerminationCurrentData-tu12SupervisedCTPSinkR1	NAME BINDING

SUBORDINATE OBJECT CLASS

"Recommendation G.774.01":pathTerminationCurrentData	AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS  tu12SupervisedCTPSinkR1		AND SUBCLASSES;

WITH ATTRIBUTE		"Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-05NameBinding 11};

pathTerminationCurrentData-tu11SupervisedCTPSinkR1	NAME  BINDING

SUBORDINATE OBJECT CLASS

"Recommendation G.774.01":pathTerminationCurrentData	AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS	tu11SupervisedCTPSinkR1	AND SUBCLASSES;

WITH ATTRIBUTE		"Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-05NameBinding 12};

# 13	Constraint rules

None.

# 14	Subordination rules

None.

# 15	Supporting ASN.1 productions

SDHCSASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127)  cs(05)

informationModel(0) asn1Module(2) sdhcs(0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS Everything

IMPORTS

PathTrace FROM SDH {itu-t(0) recommendation(0) g(7) sdhm(774) informationModel(0)

asn1Module(2) sdh(0)};

sdhCS OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) cs(05) informationModel(0)}

g774-05MObjectClass OBJECT IDENTIFIER ::= {sdhCS  managedObjectClass(3)}

g774-05Package OBJECT IDENTIFIER	::={sdhCS  package(4)}

g774-05NameBinding OBJECT IDENTIFIER ::= {sdhCS nameBinding(6)}

g774-05Attribute OBJECT IDENTIFIER	::={sdhCS attribute(7)}

Boolean ::= BOOLEAN

defaultNull Null ::= NULL

Null ::= NULL

PathTraceRS	::= GraphicString

END

Appendix I

Inheritance and naming diagrams

Figure I.1/G.774.5 – Inheritance relationship for supervised au4CTPs

Figure I.2/G.774.5 – Containment relationship for supervised CTPs

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |