|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.6 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous Digital Hierarchy (SDH)  Unidirectional performance monitoring for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.6 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.6 Synchronous Digital Hierarchy (SDH)  Unidirectional performance monitoring for the network element view

- Summary This Recommendation provides an information model for the performance monitoring of Synchronous Digital Hierarchy (SDH) network. This model describes the managed object classes and their properties for the performance monitoring function of unidirectional paths and sections, as defined in ITU-T G.784 and as related to SDH network elements. These objects are useful to describe information exchanged across interfaces defined in ITU-T M.3010 Telecommunications Management Network (TMN) architecture for the management of the performance monitoring function.

History. Issue; Notes. 2001; First revision incorporated the changes documented in the Implementors Guide. The rsCurrentData, rsCurrentDataTR, rsHistoryData, msAdaptationCurrentData, and msAdaptationHistoryData were moved from ITU-T G.774.1 to this Recommendation for unidirectional performance monitoring.. 4/1997; Initial version of the Recommendation..

- Source ITU-T Recommendation G.774.6 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and Definitions		3

4	Abbreviations		3

5	Performance management model		4

5.1	Overview		4

5.2	Requirements		4

5.3	Model overview		6

5.4	Other modelling considerations		6

6	Managed object classes definitions		6

6.1	SDH current data unidirectional		7

6.2	Regenerator section current data		8

6.3	Regenerator section current data threshold reset		8

6.4	Multiplex section current data near-end		9

6.5	Multiplex section current data near-end threshold reset		10

6.6	Path termination current data near-end		10

6.7	Path termination current data near-end threshold reset		11

6.8	Multiplex section adaptation current data		11

6.9	Multiplex section current data far-end		12

6.10	Multiplex section current data far-end threshold reset		12

6.11	Path termination current data far-end		13

6.12	Path termination current data far-end threshold reset		14

6.13	Regenerator section history data		14

6.14	Multiplex section history data near-end		15

6.15	Multiplex section adaptation history data		15

6.16	Path termination history data near-end		15

6.17	Multiplex section history data far-end		16

6.18	Path termination history data far-end		16

7	Package definitions		17

7.1	Near-end unavailable second current data package		17

7.2	Far-end unavailable second current data package		17

7.3	Near-end unavailable second history data package		18

7.4	Far-end unavailable second history data package		18

Page

7.5	Failure counts near-end package		18

7.6	Errored seconds type A near-end package		18

7.7	Errored seconds type B near-end package		19

7.8	Failure counts far-end package		19

7.9	Errored seconds type A far-end package		19

7.10	Errored seconds type B far-end package		19

7.11	Failure counts near-end history data package		19

7.12	Errored seconds type A near-end history data package		20

7.13	Errored seconds type B near-end history data package		20

7.14	Failure counts far-end history data package		20

7.15	Errored seconds type A far-end history data package		20

7.16	Errored seconds type B far-end history data package		21

7.17	Out of frame second current data package		21

7.18	Out of frame second history data package		21

8	Attributes definitions		21

8.1	Near-end unavailable seconds		21

8.2	Far-end unavailable seconds		22

8.3	Failure counts near-end		22

8.4	Errored seconds type A near-end		22

8.5	Errored seconds type B near-end		22

8.6	Failure counts far-end		22

8.7	Errored seconds type A far-end		23

8.8	Errored seconds type B  far-end		23

8.9	Out of frame second		23

8.10	Pointer justification count high		23

8.11	Pointer justification count low		23

9	Actions		24

10	Notifications		24

11	Parameters		24

12	Name binding definitions		24

12.1	MS current data near-end – MS TTP sink		24

12.2	MS current data near-end threshold reset – MS TTP sink		24

12.3	RS current data – RS TTP sink		24

12.4	RS current data threshold reset – RS TTP sink		25

12.5	MS current data far-end – MS TTP sink		25

Page

12.6	MS current data far-end threshold reset – MS TTP sink		25

12.7	Path termination current data near-end – AU4 supervised CTP SinkR1		25

12.8	Path termination current data near-end – AU3 supervised CTP SinkR1		26

12.9	Path termination current data near-end – TU3 supervised CTP SinkR1		26

12.10	Path termination current data near-end – TU2 supervised CTP SinkR1		26

12.11	Path termination current data near-end – TU12 supervised CTP SinkR1		26

12.12	Path termination current data near-end – TU11 supervised CTP SinkR1		27

12.13	Path termination current data far-end – AU4 supervised CTP SinkR1		27

12.14	Path termination current data far-end – AU3 supervised CTP SinkR1		27

12.15	Path termination current data far-end – TU3 supervised CTP SinkR1		27

12.16	Path termination current data far-end – TU2 supervised CTP SinkR1		28

12.17	Path termination current data far-end – TU12 supervised CTP SinkR1		28

12.18	Path termination current data far-end – TU11 supervised CTP SinkR1		28

12.19	Path termination current data near-end TR – AU4 supervised CTP SinkR1		28

12.20	Path termination current data near-end TR – AU3 supervised CTP SinkR1		29

12.21	Path termination current data near-end TR – TU3 supervised CTP SinkR1		29

12.22	Path termination current data near-end TR – TU2 supervised CTP SinkR1		29

12.23	Path termination current data near-end TR – TU12 supervised CTP SinkR1		29

12.24	Path termination current data near-end TR – TU11 supervised CTP SinkR1		30

12.25	Path termination current data far-end TR – AU4 supervised CTP SinkR1		30

12.26	Path termination current data far-end TR – AU3 supervised CTP SinkR1		30

12.27	Path termination current data far-end TR – TU3 supervised CTP SinkR1		30

12.28	Path termination current data far-end TR – TU2 supervised CTP SinkR1		31

12.29	Path termination current data far-end TR – TU12 supervised CTP SinkR1		31

12.30	Path Termination current data far-end TR – TU11 supervised CTP SinkR1		31

12.31	Path termination current data near-end – VC4 TTP SinkR1		31

12.32	Path termination current data near-end – VC3 TTP SinkR1		32

12.33	Path termination current data near-end – VC2 TTP SinkR1		32

12.34	Path termination current data near-end – VC12 TTP SinkR1		32

12.35	Path termination current data near-end – VC11 TTP SinkR1		32

12.36	Path termination current data near-end threshold reset – VC4 TTP SinkR1		33

12.37	Path termination current data near-end threshold reset – VC3 TTP SinkR1		33

12.38	Path termination current data near-end threshold reset – VC2 TTP SinkR1		33

12.39	Path termination current data near-end threshold reset – VC12 TTP SinkR1		33

12.40	Path termination current data near-end threshold reset – VC11 TTP SinkR1		34

12.41	Path termination current data far-end – VC4 TTP SinkR1		34

Page

12.42	Path termination current data far-end – VC3 TTP SinkR1		34

12.43	Path termination current data far-end – VC2 TTP SinkR1		34

12.44	Path termination current data far-end – VC12 TTP SinkR1		35

12.45	Path termination current data far-end – VC11 TTP SinkR1		35

12.46	Path termination current data far-end threshold reset – VC4 TTP SinkR1		35

12.47	Path termination current data far-end threshold reset – VC3 TTP SinkR1		35

12.48	Path termination current data far-end threshold reset – VC2 TTP SinkR1		36

12.49	Path termination current data far-end threshold reset – VC12 TTP SinkR1		36

12.50	Path termination current data far-end threshold reset – VC11 TTP SinkR1		36

12.51	MS adaptation current data – AU4 CTP source		36

12.52	MS adaptation current data – AU3 CTP source		37

13	Subordination rules		37

14	Pointer constraints		37

15	Supporting ASN.1 productions		37

Appendix I – Naming and inheritance diagrams		38

ITU-T Recommendation G.774.6

Synchronous Digital Hierarchy (SDH)  Unidirectional performance monitoring 
for the network element view

# 1	Scope

SDH Performance Monitoring Functions are used to monitor specified performance events of specified Termination Points managed objects and to report these performance data, as well as Quality Of Service Alarms to its managing system according to a given schedule.

ITU-T M.2120 defines maintenance of transport network, ITU-T G.784 defines the management of SDH-based network element. This Recommendation defines the object model based on ITU-T Q.822 according to the requirements described in ITU-T G.784 and ITU-T M.2120 with respect to unidirectional performance monitoring. This model uses generic mechanisms defined in ITU-T Q.822. The information model for bidirectional performance monitoring is covered by ITU-T G.774.1. This Recommendation reuses functionality of ITU-T G.774.1 wherever possible.

Structure of this Recommendation

Clause 5.1 provides an overview of the unidirectional SDH performance monitoring information model. Clauses 6 to 15 describe the information model using the notation mechanisms defined in ITU-T X.722, Guidelines for the definition of managed objects. Clause 15 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. Naming and inheritance are illustrated in Appendix I.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy SDH.

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774 (2001), Synchronous digital hierarchy (SDH)  Management information model for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunication management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview.

	ITU-T X.710 (1997), Information technology – Open Systems Interconnection – Common management information service.

	ITU-T X.711 (1997), Information technology – Open Systems Interconnection – Common management information protocol: Specification.

	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology – Open Systems Interconnection  Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998), and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology – Open Systems Interconnection – Systems Management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology – Open Systems Interconnection – Systems Management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology – Open Systems Interconnection – Systems Management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.734 (1992), Information technology – Open Systems Interconnection – Systems Management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.735 (1992), Information technology – Open Systems Interconnection – Systems Management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.739 (1993), Information technology – Open Systems Interconnection – Systems Management: Metric objects and attributes.

	ANSI T1.231-1997, Digital Hierarchy  Layer 1 in-Service Digital Transmission Performance Monitoring.

# 3	Terms and Definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, ITU-T G.784 and ITU-T M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AIS	Alarm Indication Signal

BBE	Background Block Error

CSES	Consecutive Severely Errored Second

CTP	Connection Termination Point

EBER	Excessive Bit Error Ratio

ES	Errored Second

FEBBE	Far-End Background Block Error

FEEB	Far-End Errored Block

FEES	Far-End Errored Second

FESES	Far-End Severely Errored Second

ISO	International Organization for Standardization

ITU	International Telecommunication Union

LOF	Loss Of Frame

LOS	Loss Of Signal

MS	Multiplex Section

NCSES	Number of Consecutive Severely Errored Seconds

NE	Network Element

OS	Operations System

OSI	Open Systems Interconnection

Pkg	Package

QoS	Quality of Service

RDN	Relative Distinguished Name

RS	Regenerator Section

SDH	Synchronous Digital Hierarchy

SES	Severely Errored Second

SPI	Synchronous Physical Interface

STM-N	Synchronous Transport Module N

TMN		Telecommunications Management Network

TP	Termination Point

TR	Threshold Reset

TTP	Trail Termination Point

UAS	Unavailable Second

# 5	Performance management model

## 5.1	Overview

This clause provides Managed Objects required to support management of unidirectional performance monitoring in SDH Network Elements.

This model defines subclasses of the generic currentData and historyData object classes from ITU-T Q.822 for each kind of monitoring point. For each type of monitoring point, two subclasses of currentData are defined. One is defined for either a 15-minute count or 1-day count period, and provides implicit clearing of threshold crossing alarms at the end of each granularity period. The other subclass is only relevant to 15-minute counting and performs explicit clearing of threshold alarms (Threshold Reset) at the end of a clear 15-minute period (refer to 2.3.4.2/M.2120).

History information could either be collected as part of historyData instances or one of its subclasses or as an eventRecord or one of its subclasses contained in a log. Utilization of a log is not mandatory in this Recommendation.

The threshold reset (TR) mechanism used in this Recommendation corresponds to the reset threshold report (RTR) of ITU-T G.784.

## 5.2	Requirements

The unidirectional SDH Performance Monitoring functions shall provide for:

–	the ability for a managing system to request the collection of the various Performance events relating to a given monitored entity for a given collection period;

–	the ability for a managing system to suspend/resume the performance data collection for a given monitored entity (or set of entities);

–	the ability for a managing system to instruct the NE to reset the performance monitoring counters for a given monitored entity (or set of entities);

–	the scheduling of performance collection activity within specified time periods, for a given monitored entity (or set of entities);

–	the ability for a managing system to request the performance monitoring counters for a given monitored entity (or set of entities);

–	the ability for a managed system to send event reports to a managing system to notify the results of the performance data collection, at the end of the collection period;

–	the ability for a managing system to instruct the NE to maintain Performance historical data for a specified duration, under specified conditions;

–	the ability for a managing system to instruct the NE to remove specific Performance historical data;

–	the ability for a managing system to instruct the NE to establish thresholding criteria for a given monitored entity (or set of entities);

–	the ability for a managed system to send Quality Of Service Alarms upon threshold violation of a performance counter of a monitored entity.

In order to fulfil the needs for maintenance-based collection and error performance-based collection, the following register sets are required for performance management evaluation:

Starting and ending of a unidirectional unavailability period has to be reported by the instance of the managed object class that hold the 24-hour counts (near end or far end).

Maintenance-based collection for regenerator sections

The detailed definitions for the required parameters are included in ITU-T G.784. While the optional requirements are defined in either ITU-T G.784 or ANSI Standard T1.231.

These requirements are covered by this Recommendation.

Required Register sets are:

–	17 (161) 15-minute near-end registers for BBE, ES, and SES;

–	2 (11) 24-hour near-end registers unidirectional for BBE, ES, and SES.

Optional Register sets are:

–	17 (161) 15-minute near-end registers for ESA, ESB, OFS (SEFS) and UAS;

–	2 (11) 24-hour near-end registers unidirectional for ESA, ESB, OFS (SEFS) and UAS.

Maintenance-based collection for paths and multiplex sections

The detailed definitions for the required parameters are included in ITU-T G.784, while the optional requirements are defined in either ITU-T G.784 or ANSI Standard T1.231.

These requirements are covered by this Recommendation.

Required Register sets are:

–	17 (161) 15-minute near-end registers for BBE, ES, and SES;

–	17 (161) 15-minute far-end registers for BBE, ES, and SES;

–	2 (11) 24-hour near-end registers unidirectional for BBE, ES, and SES;

	2 (11) 24-hour far-end registers unidirectional for BBE, ES, and SES.

Optional Register sets are:

–	17 (161) 15-minute near-end registers for ESA, ESB, FC and UAS;

–	17 (161) 15-minute far-end registers for ESA, ESB, FC and UAS;

–	2 (11) 24-hour near-end registers unidirectional for ESA, ESB, FC and UAS;

–	2 (11) 24-hour far-end registers unidirectional for ESA, ESB, FC and UAS.

Error performance-based collection for paths only

These requirements are covered by ITU-T G.774.1.

Register sets needed:

–	2 (11) 24-hour near-end registers for BBE, ES and SES but bidirectional (ITU-T G.826) behaviour;

–	2 (11) 24-hour far-end registers for BBE, ES and SES but bidirectional (ITU-T G.826) behaviour;

–	2 (11) 24-hour bidirectional registers for UAS.

## 5.3	Model overview

ITU-T G.774.1 does not cover unidirectional unavailability of paths and multiplex sections, because the performance counters of the several currentData and historyData Managed Object Classes behave in the G.826 (the bidirectional) way. Therefore, it is necessary to have Managed Object Classes for the unidirectional performance monitoring feature to extend the G.774.1 information model.

The unidirectional requirements are reflected in the Managed Object Classes defined in this Recommendation.

In order to avoid redundant information duplication and unuseful implementation effort the CSES events are managed only in the sdhCurrentData subclass instances that have a 15-minute granularity period.

List of the managed object classes to fulfil the requirements for unidirectional performance monitoring

sdhCurrentDataUnidirectional

msCurrentDataNearEnd, msCurrentDataNearEndTR

pathTerminationCurrentDataNearEnd, pathTerminationCurrentDataNearEndTR

msCurrentDataFarEnd, msCurrentDataFarEndTR

pathTerminationCurrentDataFarEnd, pathTerminationCurrentDataFarEndTR

msHistoryDataNearEnd

pathTerminationHistoryDataNearEnd

msHistoryDataFarEnd

pathTerminationHistoryDataFarEnd

## 5.4	Other modelling considerations

Since there is no far-end signalling in the regenerator section, the rsCurrentData, rsCurrentDataTR and rsHistoryData Managed Object Classes of ITU-T G.774.1 with the uASCurrentDataPackage and the uASHistoryDataPackage shall be used for near-end monitoring.

No additional Managed Object Classes are needed to fulfil the requirement of monitoring 2 (11) 24-hour-registers for bidirectional performance data of paths. The ITU-T G.774.1 pathTerminationCurrentData with the uASCurrentDataPackage plus the farEndCurrentDataPackage and the pathTerminationHistoryData Managed Object Classes with the uASHistoryDataPackage plus the farEndHistoryDataPackage can be used.

New nameBindings and new attributes (far-end unavailable seconds (fEUAS), near-end unavailable seconds (nEUAS)) are added wherever necessary.

All the UAS counters are optional and therefore put in conditional packages.

# 6	Managed object classes definitions

NOTE – The following managed object classes have been moved to this Recommendation from the previous version of ITU-T G.774.1 for supporting unidirectional performance monitoring.

## 6.1	SDH current data unidirectional

sdhCurrentDataUnidirectional MANAGED OBJECT CLASS

DERIVED FROM "Recommendation Q.822": currentData;

CHARACTERIZED BY

"Recommendation Q.822": zeroSuppressionPkg,

"Recommendation Q.822": thresholdPkg,

sdhCurrentDataUnidirectionalPackage PACKAGE

BEHAVIOUR sdhCurrentDataUnidirectionalBehaviour;

ATTRIBUTES

"Recommendation M.3100": currentProblemList GET;;;

CONDITIONAL PACKAGES

"Recommendation G.774.01": historyPackage  PRESENT IF

"an instance does not support flexible assignment of the history length",

"Recommendation G.774.01": unavailableTimeAlarmPackage  PRESENT IF

"starting and ending of unavailable period has to be reported and the

granularity period is 24 hours";

REGISTERED AS {g774-6MObjectClass 1};

sdhCurrentDataUnidirectionalBehaviour BEHAVIOUR

DEFINED AS

"The sdhCurrentDataUnidirectional class is used to define generic characteristic for unidirectional SDH performance monitoring from which subclasses are defined in order to hold performance event counts for a specific monitoring point. Subclasses of this class are used in order to support performance monitoring of SDH trails at various layers as described in Recommendation G.805. The performance monitoring events ES, SES and BBE which are monitored by some of the subclasses of this subclass are defined in 4.1.1/G.826. The granularityPeriod attribute can only be assigned a value at creation time.

This class can only contain one reference to an instance of the thresholdData object class in the thresholdDataInstance attribute.

If a threshold is reached or crossed then the currentProblemList attribute shall indicate it with the probable cause Threshold crossed. Subclass of this class is used to monitor the near-end or far-end performance data of the trail.

The unavailability conditions are kept separate for near-end and far-end monitoring. This means that only near-end conditions apply only to near-end unavailability and vice versa only far-end conditions apply only to far-end unavailability.

For threshold reset subclasses of this object class the following rules apply:

	No more than one QOS alarm shall be generated until there has been a 15-minute rectangular fixed 	window with less error count than the low error count threshold and no unavailable period exists.

	To provision the high and low threshold value, the counterThresholdAttributList attribute of the Q.822-ThresholdDataInstance is used.

In this attribute all the thresholds (high and low) for each necessary counter are stored in a list. This means 	for example, that the high threshold for ES and the low threshold for ES are individually stored in the same list. It is up to the network element to recognize which is the high and which is the low one.

If the unavailableTimeAlarmPackage is present and if an unavailable period starts, then a communication alarm shall be sent with a probable cause of Unavailable and the presence of this unavailable condition is indicated by the currentProblemList attribute. If an unavailable period is ending, then a communication alarm shall be sent with a probable cause of Unavailable and a

severity of Cleared. An available condition is indicated by the absence of the unavailable condition in the current problem list. The unavailable condition has no effect on the operationalState. Each subclass of this class shall define the performance attributes required to hold the mandatory or optional performance events. These performance event counts are inhibited during unavailable time of its own direction (at the near end or at the far end). Attributes which are defined in a subclass of this class shall be included in history information using the historyData, or one of its subclasses, unless it is explicitly specified in the subclass of this class that a particular attribute be not included. Each subclass of this class shall indicate which subclass of the history data is used for history retention. The following conditional packages are not used in this class: filterSuppressionPkg, observedManagedObjectPkg.

Concerning the subclasses of this class the following rule applies:

If a subclass of this class has a granularity period of 15 minutes, it should be either an instance with the threshold reset functionality or an instance without this functionality instantiated (per termination point), but not both.";

## 6.2	Regenerator section current data

rsCurrentData  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774.01":sdhCurrentData;

CHARACTERIZED BY

rsCurrentDataPackage	PACKAGE

BEHAVIOUR	rsCurrentDataBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod REQUIRED VALUES

SDHPMASN1.SDHGranularityPeriod,

"Recommendation G.774.01":bBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01":eS REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01":sES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

oFSCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01":cSESCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01":uASCurrentDataPackage PRESENT IF

"an instance supports it";

REGISTERED AS {g774-01MObjectClass 2};

rsCurrentDataBehaviour BEHAVIOUR

DEFINED AS

*Instances of the rsCurrentData managed object Class are used to hold the current register counts for a regenerator section during a collection period.

This managed object class uses the rsHistoryData managed object class for history retention.

A QOS alarm shall be sent as soon as a threshold is reached or crossed. At the end of the granularity period the QOS alarm is implicitly cleared, providing there are no other outstanding threshold crossing QOS alarms, "Threshold crossing" removed from the currentProblemList (i.e. No Notification is Sent) and a new QOS alarm shall be sent if the threshold is reached or crossed again during the next granularity period. Only one threshold value per performance counter will be supported. *;

## 6.3	Regenerator section current data threshold reset

rsCurrentDataTR  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774.01":sdhCurrentData;

CHARACTERIZED BY

"Recommendation G.774.01":thresholdResetPackage,

rsCurrentDataTRPackage	PACKAGE

BEHAVIOUR rsCurrentDataTRBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod PERMITTED VALUES

SDHPMASN1.SDHPVGranularityPeriod,

"Recommendation G.774.01":bBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01":eS REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01":sES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

oFSCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01":cSESCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01":uASCurrentDataPackage PRESENT IF

"an instance supports it";

REGISTERED AS {g774-01MObjectClass 3};

rsCurrentDataTRBehaviour BEHAVIOUR

DEFINED AS

"Instances of the rsCurrentDataTR managed object Class are used to hold the current register counts for a regenerator section during a collection period.

This managed object class uses the rsHistoryData managed object class for history retention";

## 6.4	Multiplex section current data near-end

msCurrentDataNearEnd MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentDataUnidirectional;

CHARACTERIZED BY

msCurrentDataNearEndPackage PACKAGE

BEHAVIOUR   msCurrentDataNearEndBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod  REQUIRED VALUES

SDHPMUNIASN1.SDHGranularityPeriod,

"Recommendation G.774.01": bBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": eS REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": sES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

nearEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": cSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it",

failureCountsNearEndPackage PRESENT IF

"an instance supports it",

eSANearEndPackage PRESENT IF

"an instance supports it",

eSBNearEndPackage PRESENT IF

"an instance supports it";

REGISTERED AS {g774-6MObjectClass 2} ;

msCurrentDataNearEndBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of the msCurrentDataNearEnd managed object Class are used to hold the current near-end register counts for a multiplex section trail termination point during a collection period.

This managed object class uses the msHistoryDataNearEnd managed object class for history retention.

A QOS alarm shall be sent as soon as a threshold is reached or crossed. At the end of the granularity period, the QOS alarm is implicitly cleared and, providing there are no other outstanding threshold crossing QOS alarms, threshold crossing is removed from the currentProblemList (i.e. No Notification is Sent) and a new QOS alarm shall be sent if the threshold is reached or crossed again during the next granularity period. Only one threshold value per performance counter will be supported.";

## 6.5	Multiplex section current data near-end threshold reset

msCurrentDataNearEndTR  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentDataUnidirectional;

CHARACTERIZED BY

"Recommendation G.774.01": thresholdResetPackage,

msCurrentDataNearEndTRPackage PACKAGE

BEHAVIOUR

msCurrentDataNearEndTRBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod PERMITTED VALUES

SDHPMUNIASN1.SDHPVGranularityPeriod,

"Recommendation G.774.01": bBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": eS REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": sES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

nearEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": cSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it" ;

REGISTERED AS {g774-6MObjectClass 3} ;

msCurrentDataNearEndTRBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of the msCurrentDataNearEndTR managed object Class are used to hold the current near-end register counts for a multiplex section trail termination point during a collection period.

Only the 15 minutes granularity period must be supported.

This managed object class uses the msHistoryDataNearEnd managed object class for history retention. " ;

## 6.6	Path termination current data near-end

pathTerminationCurrentDataNearEnd  MANAGED OBJECT CLASS

DERIVED FROM   sdhCurrentDataUnidirectional;

CHARACTERIZED BY

pathTerminationCurrentDataNearEndPackage PACKAGE

BEHAVIOUR pathTerminationCurrentDataNearEndBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod REQUIRED VALUES

SDHPMUNIASN1.SDHGranularityPeriod,

"Recommendation G.774.01": bBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": eS REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": sES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

nearEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": cSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it",

failureCountsNearEndPackage PRESENT IF

"an instance supports it",

eSANearEndPackage PRESENT IF

"an instance supports it",

eSBNearEndPackage PRESENT IF

"an instance supports it";

REGISTERED AS  {g774-6MObjectClass 4} ;

pathTerminationCurrentDataNearEndBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of the pathTerminationCurrentDataNearEnd managed object Class are used

to hold the current near end register counts for a Higher Order Path or Lower Order Path during a collection period.

This managed object class uses the pathTerminationHistoryDataNearEnd managed object class for history retention.

A QOS alarm shall be sent as soon as a threshold is reached or crossed. At the end of the granularity period, the QOS alarm is implicitly cleared and, providing there are no other outstanding threshold crossing QOS alarms, threshold crossing is removed from the currentProblemList (i.e. No Notification is Sent) and a new QOS alarm shall be sent if the threshold is reached or crossed again during the next granularity period. Only one threshold value per performance counter will be supported.";

## 6.7	Path termination current data near-end threshold reset

pathTerminationCurrentDataNearEndTR  MANAGED OBJECT CLASS

DERIVED FROM     sdhCurrentDataUnidirectional;

CHARACTERIZED BY

"Recommendation G.774.01": thresholdResetPackage,

pathTerminationCurrentDataNearEndTRPackage PACKAGE

BEHAVIOUR pathTerminationCurrentDataNearEndTRBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod PERMITTED VALUES

SDHPMUNIASN1.SDHPVGranularityPeriod,

"Recommendation G.774.01": bBE  REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": eS  REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": sES  REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

nearEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": cSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it";

REGISTERED AS  {g774-6MObjectClass 5} ;

pathTerminationCurrentDataNearEndTRBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of the pathTerminationCurrentDataNearEndTR managed object Class are used to hold the current near-end register counts for a Higher Order Path or Lower Order Path during a collection period.

Only the 15 minutes granularity period must be supported.

This managed object class uses the pathTerminationHistoryDataNearEnd managed object class for history retention.";

## 6.8	Multiplex section adaptation current data

msAdaptationCurrentData  MANAGED OBJECT CLASS

DERIVED FROM "Recommendation G.774.01":sdhCurrentData;

CHARACTERIZED BY

msAdaptationCurrentDataPackage  PACKAGE

BEHAVIOUR	msAdaptationCurrentDataBehaviour;

ATTRIBUTES

pJCHigh	GET,

pJCLow	GET;;;

REGISTERED AS {g774-01MObjectClass 11};

msAdaptationCurrentDataBehaviour  BEHAVIOUR

DEFINED AS

"Instances of this class are used to hold the pointer justification event (PJE) counts.Positive and negative PJEs are counted separately one selectable outgoing AUwithin an STM-N signal after the AU has been resynchronized to the local clock.";

## 6.9	Multiplex section current data far-end

msCurrentDataFarEnd   MANAGED OBJECT CLASS

DERIVED FROM   sdhCurrentDataUnidirectional;

CHARACTERIZED BY

msCurrentDataFarEndPackage PACKAGE

BEHAVIOUR   msCurrentDataFarEndBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod  REQUIRED VALUES

SDHPMUNIASN1.SDHGranularityPeriod,

"Recommendation G.774.01": fEBBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": fEES REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": fESES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

farEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": farEndCSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it",

failureCountsFarEndPackage PRESENT IF

"an instance supports it",

eSAFarEndPackage PRESENT IF

"an instance supports it",

eSBFarEndPackage PRESENT IF

"an instance supports it";

REGISTERED AS {g774-6MObjectClass 6} ;

msCurrentDataFarEndBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of the msCurrentDataFarEnd managed object Class are used to hold the current far-end register counts for a multiplex section trail termination point during a collection period.

This managed object class uses the msHistoryDataFarEnd managed object class for history retention.

A QOS alarm shall be sent as soon as a threshold is reached or crossed. At the end of the granularity period, the QOS alarm is implicitly cleared and, providing there are no other outstanding threshold crossing QOS alarms, threshold crossing is removed from the currentProblemList (i.e. No Notification is Sent) and a new QOS alarm shall be sent if the threshold is reached or crossed again during the next granularity period. Only one threshold value per performance counter will be supported.";

## 6.10	Multiplex section current data far-end threshold reset

msCurrentDataFarEndTR  MANAGED OBJECT CLASS

DERIVED FROM sdhCurrentDataUnidirectional;

CHARACTERIZED BY

"Recommendation G.774.01": thresholdResetPackage,

msCurrentDataFarEndTRPackage PACKAGE

BEHAVIOUR

msCurrentDataFarEndTRBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod PERMITTED VALUES

SDHPMUNIASN1.SDHPVGranularityPeriod,

"Recommendation G.774.01": fEBBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": fEES REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": fESES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

farEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": farEndCSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it";

REGISTERED AS {g774-6MObjectClass 7} ;

msCurrentDataFarEndTRBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of the msCurrentDataFarEnd managed object Class are used to hold the current far-end register counts for a multiplex section trail termination point during a collection period.

Only the 15 minutes granularity period must be supported.

This managed object class uses the msHistoryDataFarEnd managed object class for history retention." ;

## 6.11	Path termination current data far-end

pathTerminationCurrentDataFarEnd  MANAGED OBJECT CLASS

DERIVED FROM   sdhCurrentDataUnidirectional;

CHARACTERIZED BY

pathTerminationCurrentDataFarEndPackage PACKAGE

BEHAVIOUR pathTerminationCurrentDataFarEndBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod REQUIRED VALUES

SDHPMUNIASN1.SDHGranularityPeriod,

"Recommendation G.774.01": fEBBE REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": fEES  REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": fESES REPLACE-WITH-DEFAULT GET;;;

CONDITIONAL PACKAGES

farEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": farEndCSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it",

failureCountsFarEndPackage PRESENT IF

"an instance supports it",

eSAFarEndPackage PRESENT IF

"an instance supports it",

eSBFarEndPackage PRESENT IF

"an instance supports it";

REGISTERED AS  {g774-6MObjectClass 8} ;

pathTerminationCurrentDataFarEndBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of the pathTerminationCurrentDataFarEnd managed object Class are used to hold the current far end register counts for a Higher Order Path or Lower Order Path during a collection period.

This managed object class uses the pathTerminationHistoryDataFarEnd managed object class for history retention.

A QOS alarm shall be sent as soon as a threshold is reached or crossed. At the end of the granularity period, the QOS alarm is implicitly cleared and, providing there are no other outstanding threshold crossing QOS alarms, threshold crossing is removed from the currentProblemList (i.e. No Notification is Sent) and a new QOS alarm shall be sent if the threshold is reached or crossed again during the next granularity period. Only one threshold value per performance counter will be supported.";

## 6.12	Path termination current data far-end threshold reset

pathTerminationCurrentDataFarEndTR  MANAGED OBJECT CLASS

DERIVED FROM	sdhCurrentDataUnidirectional;

CHARACTERIZED BY

"Recommendation G.774.01": thresholdResetPackage,

pathTerminationCurrentDataFarEndTRPackage PACKAGE

BEHAVIOUR pathTerminationCurrentDataFarEndTRBehaviour;

ATTRIBUTES

"Recommendation X.739": granularityPeriod PERMITTED VALUES

SDHPMUNIASN1.SDHPVGranularityPeriod,

"Recommendation G.774.01": fEBBE  REPLACE-WITH-DEFAULT  GET,

"Recommendation G.774.01": fEES  REPLACE-WITH-DEFAULT GET,

"Recommendation G.774.01": fESES  REPLACE-WITH-DEFAULT  GET;;;

CONDITIONAL PACKAGES

farEndUASCurrentDataPackage PRESENT IF

"an instance supports it",

"Recommendation G.774.01": farEndCSESCurrentDataPackage PRESENT IF

"the granularity period is 15 minutes and an instance supports it";

REGISTERED AS  {g774-6MObjectClass 9} ;

pathTerminationCurrentDataFarEndTRBehaviour BEHAVIOUR 
DEFINED AS
"Instances of the pathTerminationCurrentDataFarEndTR managed object Class are used to hold the current far-end register counts for a Higher Order Path or Lower Order Path during a collection period.

Only the 15 minutes granularity period must be supported.

This managed object class uses the pathTerminationHistoryDataFarEnd managed object class for history retention.";

## 6.13	Regenerator section history data

rsHistoryData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822 ": historyData;

CHARACTERIZED BY

rsHistoryDataPackage  PACKAGE

BEHAVIOUR rsHistoryDataBehaviour;

ATTRIBUTES

"Recommendation G.774.01":bBE GET,

"Recommendation G.774.01":eS GET,

"Recommendation G.774.01":sES GET ;;;

CONDITIONAL PACKAGES

oFSHistoryDataPackage PRESENT IF

"the containing rsCurrentData or rsCurrentDataTR instance contains the oFSCurrentDataPackage",

"Recommendation G.774.01":uASHistoryDataPackage PRESENT IF

"the containing rsCurrentData contains the uASCurrentDataPackage";

REGISTERED AS {g774-01MObjectClass 12};

rsHistoryDataBehaviour   BEHAVIOUR

DEFINED AS

"Instances of this class are used to store the observed events of an rsCurrentData or rsCurrentDataTR object at the end of an observation interval. An instance of this managed object is contained by an rsCurrentData or rsCurrentDataTR managed object instance.";

## 6.14	Multiplex section history data near-end

msHistoryDataNearEnd   MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822":historyData;

CHARACTERIZED BY

msHistoryDataNearEndPackage PACKAGE

BEHAVIOUR

msHistoryDataNearEndBehaviour;

ATTRIBUTES

"Recommendation G.774.01": bBE GET,

"Recommendation G.774.01": eS GET,

"Recommendation G.774.01": sES GET;;;

CONDITIONAL PACKAGES

nearEndUASHistoryDataPackage PRESENT IF

"the containing msCurrentDataNearEnd contains the 	nearEndUASCurrentDataPackage",

failureCountsNearEndHistoryDataPackage PRESENT IF

"the containing msCurrentDataNearEnd contains the 	failureCountsNearEndPackage",

eSANearEndHistoryDataPackage PRESENT IF

"the containing msCurrentDataNearEnd contains the eSANearEndPackage",

eSBNearEndHistoryDataPackage PRESENT IF

"the containing msCurrentDataNearEnd contains the eSBNearEndPackage";

REGISTERED AS {g774-6MObjectClass 10} ;

msHistoryDataNearEndBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of this class are used to store the observed events of an msCurrentDataNearEnd or msCurrentDataNearEndTR object at the end of an observation interval. An instance of this managed object is contained by an msCurrentDataNearEnd or msCurrentDataNearEndTR managed object instance";

## 6.15	Multiplex section adaptation history data

msAdaptationHistoryData  MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822": historyData;

CHARACTERIZED BY

msAdaptationHistoryDataPackage  PACKAGE

BEHAVIOUR msAdaptationHistoryDataBehaviour;

ATTRIBUTES

pJCHigh	GET,

pJCLow	GET;;;

REGISTERED AS {g774-01MObjectClass 18};

msAdaptationHistoryDataBehaviour  BEHAVIOUR

DEFINED AS

"Instances of this class are used to store the observed events of an msAdaptationCurrentData object at the end of an observation interval. An instance of this managed object is contained by an msAdaptationCurrentData managed object instance.";

## 6.16	Path termination history data near-end

pathTerminationHistoryDataNearEnd  MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation Q.822":historyData;

CHARACTERIZED BY

pathTerminationHistoryDataNearEndPackage PACKAGE

BEHAVIOUR pathTerminationHistoryDataNearEndBehaviour;

ATTRIBUTES

"Recommendation G.774.01": bBE GET,

"Recommendation G.774.01": eS GET,

"Recommendation G.774.01": sES GET;;;

CONDITIONAL PACKAGES

nearEndUASHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataNearEnd  contains the 	nearEndUASCurrentDataPackage",

failureCountsNearEndHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataNearEnd  contains the 	failureCountsNearEndPackage",

eSANearEndHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataNearEnd  contains the 	eSANearEndPackage",

eSBNearEndHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataNearEnd  contains the 	eSBNearEndPackage";

REGISTERED AS {g774-6MObjectClass 11} ;

pathTerminationHistoryDataNearEndBehaviour BEHAVIOUR 
DEFINED AS 
"Instances of this class are used to store the observed events of a pathTerminationCurrentDataNearEnd or pathTerminationCurrentDataNearEndTR object at the end of an observation interval. An instance of this managed object is contained by a pathTerminationCurrentDataNearEnd or pathTerminationCurrentDataNearEndTR managed object instance";

## 6.17	Multiplex section history data far-end

msHistoryDataFarEnd   MANAGED OBJECT CLASS

DERIVED FROM  "Recommendation Q.822":historyData;

CHARACTERIZED BY

msHistoryDataFarEndPackage PACKAGE

BEHAVIOUR

msHistoryDataFarEndBehaviour;

ATTRIBUTES

"Recommendation G.774.01": fEBBE GET,

"Recommendation G.774.01": fEES GET,

"Recommendation G.774.01": fESES GET;;;

CONDITIONAL PACKAGES

farEndUASHistoryDataPackage PRESENT IF

"the containing msCurrentDataFarEnd  contains the 	farEndUASCurrentDataPackage",

failureCountsFarEndHistoryDataPackage PRESENT IF

"the containing msCurrentDataFarEnd  contains the 	failureCountsFarEndPackage",

eSAFarEndHistoryDataPackage PRESENT IF

"the containing msCurrentDataFarEnd  contains the eSAFarEndPackage",

eSBFarEndHistoryDataPackage PRESENT IF

"the containing msCurrentDataFarEnd  contains the eSBFarEndPackage";

REGISTERED AS {g774-6MObjectClass 12} ;

msHistoryDataFarEndBehaviour BEHAVIOUR
DEFINED AS
"Instances of this class are used to store the observed events of an msCurrentDataFarEnd or msCurrentDataFarEndTR object at the end of an observation interval. An instance of this managed object is contained by an msCurrentDataFarEnd or msCurrentDataFarEndTR managed object instance";

## 6.18	Path termination history data far-end

pathTerminationHistoryDataFarEnd  MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation Q.822":historyData;

CHARACTERIZED BY

pathTerminationHistoryDataFarEndPackage PACKAGE

BEHAVIOUR pathTerminationHistoryDataFarEndBehaviour;

ATTRIBUTES

"Recommendation G.774.01": fEBBE GET,

"Recommendation G.774.01": fEES GET,

"Recommendation G.774.01": fESES GET;;;

CONDITIONAL PACKAGES

farEndUASHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataFarEnd  contains the 	farEndUASCurrentDataPackage",

failureCountsFarEndHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataFarEnd  contains the 	failureCountsFarEndPackage",

eSAFarEndHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataFarEnd  contains the 	eSAFarEndPackage",

eSBFarEndHistoryDataPackage PRESENT IF

"the containing pathTerminationCurrentDataFarEnd  contains the 	eSBFarEndPackage";

REGISTERED AS {g774-6MObjectClass 13} ;

pathTerminationHistoryDataFarEndBehaviour BEHAVIOUR
DEFINED AS
"Instances of this class are used to store the observed events of a pathTerminationCurrentDataFarEnd or pathTerminationCurrentDataFarEndTR object at the end of an observation interval. An instance of this managed object is contained by a pathTerminationCurrentDataFarEnd or pathTerminationCurrentDataFarEndTR  managed object instance";

# 7	Package definitions

## 7.1	Near-end unavailable second current data package

nearEndUASCurrentDataPackage PACKAGE

BEHAVIOUR

nearEndUASCurrentDataPackageBehaviour;

ATTRIBUTES

nEUAS REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 1};

nearEndUASCurrentDataPackageBehaviour BEHAVIOUR
DEFINED AS
"This package is used to store the counter of one-second intervals pertaining to an Unavailable Time at the near end.";

## 7.2	Far-end unavailable second current data package

farEndUASCurrentDataPackage PACKAGE

BEHAVIOUR

farEndUASCurrentDataPackageBehaviour;

ATTRIBUTES

fEUAS REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 2};

farEndUASCurrentDataPackageBehaviour BEHAVIOUR
DEFINED AS
"This package is used to store the counter of one-second intervals pertaining to an Unavailable Time at the far end.";

## 7.3	Near-end unavailable second history data package

nearEndUASHistoryDataPackage PACKAGE

BEHAVIOUR

nearEndUASHistoryDataPackageBehaviour;

ATTRIBUTES

nEUAS GET;

REGISTERED AS {g774-6Package 3};

nearEndUASHistoryDataPackageBehaviour BEHAVIOUR
DEFINED AS
"This package is used to store the corresponding current data counter of one second intervals pertaining to an Unavailable Time at the near end at the end of the granularity period.";

## 7.4	Far-end unavailable second history data package

farEndUASHistoryDataPackage PACKAGE

BEHAVIOUR

farEndUASHistoryDataPackageBehaviour;

ATTRIBUTES

fEUAS GET;

REGISTERED AS {g774-6Package 4};

farEndUASHistoryDataPackageBehaviour BEHAVIOUR
DEFINED AS
"This package is used to store the corresponding current data counter of one-second intervals pertaining to an Unavailable Time at the far end at the end of the granularity period.";

## 7.5	Failure counts near-end package

failureCountsNearEndPackage PACKAGE

BEHAVIOUR

failureCountsNearEndPackageBehaviour;

ATTRIBUTES

fCNearEnd REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 5};

failureCountsNearEndPackageBehaviour BEHAVIOUR

DEFINED AS
"This package is used to store the corresponding failure counts at the near end.";

## 7.6	Errored seconds type A near-end package

eSANearEndPackage PACKAGE

BEHAVIOUR

eSANearEndPackageBehaviour;

ATTRIBUTES

eSANearEnd REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 6};

eSANearEndPackageBehaviour BEHAVIOUR

DEFINED AS
"This package is used to store the errored seconds of type A at the near end.";

## 7.7	Errored seconds type B near-end package

eSBNearEndPackage PACKAGE

BEHAVIOUR

eSBNearEndPackageBehaviour;

ATTRIBUTES

eSBNearEnd REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 7};

eSBNearEndPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the errored seconds of type B at the near end.";

## 7.8	Failure counts far-end package

failureCountsFarEndPackage PACKAGE

BEHAVIOUR

failureCountsFarEndPackageBehaviour;

ATTRIBUTES

fCFarEnd REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 8};

failureCountsFarEndPackageBehaviour BEHAVIOUR

DEFINED AS
"This package is used to store the corresponding failure counts at the far end.";

## 7.9	Errored seconds type A far-end package

eSAFarEndPackage PACKAGE

BEHAVIOUR

eSAFarEndPackageBehaviour;

ATTRIBUTES

eSAFarEnd REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 9};

eSAFarEndPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the errored seconds of type A at the far end.";

## 7.10	Errored seconds type B far-end package

eSBFarEndPackage PACKAGE

BEHAVIOUR

eSBFarEndPackageBehaviour;

ATTRIBUTES

eSBFarEnd REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-6Package 10};

eSBFarEndPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the errored seconds of type B at the far end.";

## 7.11	Failure counts near-end history data package

failureCountsNearEndHistoryDataPackage PACKAGE

BEHAVIOUR

failureCountsNearEndHistoryDataPackageBehaviour;

ATTRIBUTES

fCNearEnd GET;

REGISTERED AS {g774-6Package 11};

failureCountsNearEndHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the corresponding failure counts at the near end at the end of the granularity period.";

## 7.12	Errored seconds type A near-end history data package

eSANearEndHistoryDataPackage PACKAGE

BEHAVIOUR

eSANearEndHistoryDataPackageBehaviour;

ATTRIBUTES

eSANearEnd GET;

REGISTERED AS {g774-6Package 12};

eSANearEndHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the corresponding errored seconds of type A at the near end at the end of the granularity period.";

## 7.13	Errored seconds type B near-end history data package

eSBNearEndHistoryDataPackage PACKAGE

BEHAVIOUR

eSBNearEndHistoryDataPackageBehaviour;

ATTRIBUTES

eSBNearEnd GET;

REGISTERED AS {g774-6Package 13};

eSBNearEndHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the corresponding errored seconds of type B at the near end at the end of the granularity period.";

## 7.14	Failure counts far-end history data package

failureCountsFarEndHistoryDataPackage PACKAGE

BEHAVIOUR

failureCountsFarEndHistoryDataPackageBehaviour;

ATTRIBUTES

fCFarEnd GET;

REGISTERED AS {g774-6Package 14};

failureCountsFarEndHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the corresponding failure counts at the far end at the end of the granularity period.";

## 7.15	Errored seconds type A far-end history data package

eSAFarEndHistoryDataPackage PACKAGE

BEHAVIOUR

eSAFarEndHistoryDataPackageBehaviour;

ATTRIBUTES

eSAFarEnd GET;

REGISTERED AS {g774-6Package 15};

eSAFarEndHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the corresponding errored seconds of type A at the far end at the end of the granularity period.";

## 7.16	Errored seconds type B far-end history data package

eSBFarEndHistoryDataPackage PACKAGE

BEHAVIOUR

eSBFarEndHistoryDataPackageBehaviour;

ATTRIBUTES

eSBFarEnd GET;

REGISTERED AS {g774-6Package 16};

eSBFarEndHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the corresponding errored seconds of type B at the far end at the end of the granularity period.";

## 7.17	Out of frame second current data package

oFSCurrentDataPackage PACKAGE

BEHAVIOUR

oFSCurrentDataPackageBehaviour;

ATTRIBUTES

oFS REPLACE-WITH-DEFAULT GET;

REGISTERED AS {g774-01Package 10};

oFSCurrentDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to store the counter of one second intervals containing one or more Out of Frame.";

## 7.18	Out of frame second history data package

oFSHistoryDataPackage PACKAGE

BEHAVIOUR

oFSHistoryDataPackageBehaviour;

ATTRIBUTES

oFS GET;

REGISTERED AS {g774-01Package 11};

oFSHistoryDataPackageBehaviour BEHAVIOUR

DEFINED AS

"This package is used to record the corresponding current data attribute values at the end of the granularity period.";

# 8	Attributes definitions

## 8.1	Near-end unavailable seconds

nEUAS ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	nEUASBeh BEHAVIOUR
		DEFINED AS
		"The value of the nEUAS attribute represents the count of one-second intervals pertaining to an UnavailableTime at the near end.";;

REGISTERED AS {g774-6Attribute 1};

## 8.2	Far-end unavailable seconds

fEUAS ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	fEUASBeh BEHAVIOUR
		DEFINED AS
		"The value of the fEUAS attribute represents the count of one-second intervals pertaining to an UnavailableTime at the far end.";;

REGISTERED AS {g774-6Attribute 2};

## 8.3	Failure counts near-end

fCNearEnd ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	fCNearEndBeh BEHAVIOUR
		DEFINED AS
		"This parameter is a count of the number of occurrences of near-end failure events.";;

REGISTERED AS {g774-6Attribute 3};

## 8.4	Errored seconds type A near-end

eSANearEnd ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	eSANearEndBeh BEHAVIOUR

DEFINED AS
		"This parameter is a count of errored seconds of type A at the near end.

";;

REGISTERED AS {g774-6Attribute 4};

## 8.5	Errored seconds type B near-end

eSBNearEnd ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	eSBNearEndBeh BEHAVIOUR
		DEFINED AS
		"This parameter is a count of errored seconds of type B at the near end.

";;

REGISTERED AS {g774-6Attribute 5};

## 8.6	Failure counts far-end

fCFarEnd ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	fCFarEndBeh BEHAVIOUR

DEFINED AS
		"This parameter is a count of the number of occurences of far-end failure events.

";;

REGISTERED AS {g774-6Attribute 6};

## 8.7	Errored seconds type A far-end

eSAFarEnd ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	eSAFarEndBeh BEHAVIOUR

DEFINED AS
		"This parameter is a count of the errored seconds of type A at the far end.

";;

REGISTERED AS {g774-6Attribute 7};

## 8.8	Errored seconds type B  far-end

eSBFarEnd ATTRIBUTE

DERIVED FROM "Recommendation X.721": counter;

BEHAVIOUR	eSBFarEndBeh BEHAVIOUR

DEFINED AS
		"This parameter is a count of errored seconds of type B at the far end.

";;

REGISTERED AS {g774-6Attribute 8};

## 8.9	Out of frame second

oFS  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

oFSBeh BEHAVIOUR

DEFINED AS

"The value of the oFS attribute represents the count of seconds with at least one Out Of Frame Event during the available time of the monitored resource during the corresponding granularity period."

;;

REGISTERED AS {g774-01Attribute 14};

## 8.10	Pointer justification count high

pJCHigh  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

pJCHighBehaviour BEHAVIOUR

DEFINED AS

"The value of the pJCHigh attribute represents the positive PJE count on one selectable outgoing AU within an STM-N signal after the AU has been resynchronized to the local clock."

;;

REGISTERED AS {g774-01Attribute 23};

## 8.11	Pointer justification count low

pJCLow  ATTRIBUTE

DERIVED FROM  "Rec. X.721 | ISO/IEC 10165-2": counter;

BEHAVIOUR

pJCLowBehaviour BEHAVIOUR

DEFINED AS

"The value of the pJCLow attribute represents the negative PJE count on one selectable outgoing AU within an STM-N signal after the AU has been resynchronized to the local clock."

;;

REGISTERED AS {g774-01Attribute 24};

# 9	Actions

None.

# 10	Notifications

None.

# 11	Parameters

None.

# 12	Name binding definitions

## 12.1	MS current data near-end – MS TTP sink

msCurrentDataNearEnd-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS msCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":msTTPSink AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739":scannerId;

CREATE

WITH-REFERENCE-OBJECT ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 1};

## 12.2	MS current data near-end threshold reset – MS TTP sink

msCurrentDataNearEndTR-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS msCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":msTTPSink AND SUBCLASSES;

WITH ATTRIBUTE   "Recommendation X.739":scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 2};

## 12.3	RS current data – RS TTP sink

rsCurrentData-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS  rsCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": rsTTPSink AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 7};

## 12.4	RS current data threshold reset – RS TTP sink

rsCurrentDataTR-rsTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS  rsCurrentDataTR  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": rsTTPSink AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-01NameBinding 8};

## 12.5	MS current data far-end – MS TTP sink

msCurrentDataFarEnd-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS msCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":msTTPSink AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 13};

## 12.6	MS current data far-end threshold reset – MS TTP sink

msCurrentDataFarEndTR-msTTPSink NAME BINDING

SUBORDINATE OBJECT CLASS msCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":msTTPSink AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 14};

## 12.7	Path termination current data near-end – AU4 supervised CTP SinkR1

pathTerminationCurrentDataNearEnd-au4SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au4SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 69};

## 12.8	Path termination current data near-end – AU3 supervised CTP SinkR1

pathTerminationCurrentDataNearEnd-au3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 70};

## 12.9	Path termination current data near-end – TU3 supervised CTP SinkR1

pathTerminationCurrentDataNearEnd-tu3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 71};

## 12.10	Path termination current data near-end – TU2 supervised CTP SinkR1

pathTerminationCurrentDataNearEnd-tu2SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu2SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 72};

## 12.11	Path termination current data near-end – TU12 supervised CTP SinkR1

pathTerminationCurrentDataNearEnd-tu12SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu12SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 73};

## 12.12	Path termination current data near-end – TU11 supervised CTP SinkR1

pathTerminationCurrentDataNearEnd-tu11SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu11SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 74};

## 12.13	Path termination current data far-end – AU4 supervised CTP SinkR1

pathTerminationCurrentDataFarEnd-au4SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au4SupervisedCTPSinkR1

AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 75};

## 12.14	Path termination current data far-end – AU3 supervised CTP SinkR1

pathTerminationCurrentDataFarEnd-au3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 76};

## 12.15	Path termination current data far-end – TU3 supervised CTP SinkR1

pathTerminationCurrentDataFarEnd-tu3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 77};

## 12.16	Path termination current data far-end – TU2 supervised CTP SinkR1

pathTerminationCurrentDataFarEnd-tu2SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu2SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 78};

## 12.17	Path termination current data far-end – TU12 supervised CTP SinkR1

pathTerminationCurrentDataFarEnd-tu12SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu12SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 79};

## 12.18	Path termination current data far-end – TU11 supervised CTP SinkR1

pathTerminationCurrentDataFarEnd-tu11SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu11SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 80};

## 12.19	Path termination current data near-end TR – AU4 supervised CTP SinkR1

pathTerminationCurrentDataNearEndTR-au4SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au4SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 81};

## 12.20	Path termination current data near-end TR – AU3 supervised CTP SinkR1

pathTerminationCurrentDataNearEndTR-au3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 82};

## 12.21	Path termination current data near-end TR – TU3 supervised CTP SinkR1

pathTerminationCurrentDataNearEndTR-tu3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 83};

## 12.22	Path termination current data near-end TR – TU2 supervised CTP SinkR1

pathTerminationCurrentDataNearEndTR-tu2SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu2SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 84};

## 12.23	Path termination current data near-end TR – TU12 supervised CTP SinkR1

pathTerminationCurrentDataNearEndTR-tu12SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu12SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 85};

## 12.24	Path termination current data near-end TR – TU11 supervised CTP SinkR1

pathTerminationCurrentDataNearEndTR-tu11SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu11SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 86};

## 12.25	Path termination current data far-end TR – AU4 supervised CTP SinkR1

pathTerminationCurrentDataFarEndTR-au4SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au4SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 87};

## 12.26	Path termination current data far-end TR – AU3 supervised CTP SinkR1

pathTerminationCurrentDataFarEndTR-au3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":au3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 88};

## 12.27	Path termination current data far-end TR – TU3 supervised CTP SinkR1

pathTerminationCurrentDataFarEndTR-tu3SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu3SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 89};

## 12.28	Path termination current data far-end TR – TU2 supervised CTP SinkR1

pathTerminationCurrentDataFarEndTR-tu2SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu2SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 90};

## 12.29	Path termination current data far-end TR – TU12 supervised CTP SinkR1

pathTerminationCurrentDataFarEndTR-tu12SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu12SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 91};

## 12.30	Path Termination current data far-end TR – TU11 supervised CTP SinkR1

pathTerminationCurrentDataFarEndTR-tu11SupervisedCTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS  pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774.05":tu11SupervisedCTPSinkR1 AND

SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE
  DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 92};

## 12.31	Path termination current data near-end – VC4 TTP SinkR1

pathTerminationCurrentDataNearEnd-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 49};

## 12.32	Path termination current data near-end – VC3 TTP SinkR1

pathTerminationCurrentDataNearEnd-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 50};

## 12.33	Path termination current data near-end – VC2 TTP SinkR1

pathTerminationCurrentDataNearEnd-vc2TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 51};

## 12.34	Path termination current data near-end – VC12 TTP SinkR1

pathTerminationCurrentDataNearEnd-vc12TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 52};

## 12.35	Path termination current data near-end – VC11 TTP SinkR1

pathTerminationCurrentDataNearEnd-vc11TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 53};

## 12.36	Path termination current data near-end threshold reset – VC4 TTP SinkR1

pathTerminationCurrentDataNearEndTR-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 54};

## 12.37	Path termination current data near-end threshold reset – VC3 TTP SinkR1

pathTerminationCurrentDataNearEndTR-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 55};

## 12.38	Path termination current data near-end threshold reset – VC2 TTP SinkR1

pathTerminationCurrentDataNearEndTR-vc2TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 56};

## 12.39	Path termination current data near-end threshold reset – VC12 TTP SinkR1

pathTerminationCurrentDataNearEndTR-vc12TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 57};

## 12.40	Path termination current data near-end threshold reset – VC11 TTP SinkR1

pathTerminationCurrentDataNearEndTR-vc11TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataNearEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 58};

## 12.41	Path termination current data far-end – VC4 TTP SinkR1

pathTerminationCurrentDataFarEnd-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 59};

## 12.42	Path termination current data far-end – VC3 TTP SinkR1

pathTerminationCurrentDataFarEnd-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 60};

## 12.43	Path termination current data far-end – VC2 TTP SinkR1

pathTerminationCurrentDataFarEnd-vc2TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 61};

## 12.44	Path termination current data far-end – VC12 TTP SinkR1

pathTerminationCurrentDataFarEnd-vc12TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 62};

## 12.45	Path termination current data far-end – VC11 TTP SinkR1

pathTerminationCurrentDataFarEnd-vc11TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEnd AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 63};

## 12.46	Path termination current data far-end threshold reset – VC4 TTP SinkR1

pathTerminationCurrentDataFarEndTR-vc4TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc4TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 64};

## 12.47	Path termination current data far-end threshold reset – VC3 TTP SinkR1

pathTerminationCurrentDataFarEndTR-vc3TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc3TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 65};

## 12.48	Path termination current data far-end threshold reset – VC2 TTP SinkR1

pathTerminationCurrentDataFarEndTR-vc2TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc2TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 66};

## 12.49	Path termination current data far-end threshold reset – VC12 TTP SinkR1

pathTerminationCurrentDataFarEndTR-vc12TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc12TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 67};

## 12.50	Path termination current data far-end threshold reset – VC11 TTP SinkR1

pathTerminationCurrentDataFarEndTR-vc11TTPSinkR1 NAME BINDING

SUBORDINATE OBJECT CLASS

pathTerminationCurrentDataFarEndTR AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS "Recommendation G.774":vc11TTPSinkR1 AND SUBCLASSES;

WITH ATTRIBUTE "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT  ,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 68};

## 12.51	MS adaptation current data – AU4 CTP source

msAdaptationCurrentData-au4CTPSource NAME BINDING

SUBORDINATE OBJECT CLASS  msAdaptationCurrentData AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": au4CTPSource AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 93};

## 12.52	MS adaptation current data – AU3 CTP source

msAdaptationCurrentData-au3CTPSource NAME BINDING

SUBORDINATE OBJECT CLASS  msAdaptationCurrentData  AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS  "Recommendation G.774": au3CTPSource AND SUBCLASSES;

WITH ATTRIBUTE  "Recommendation X.739": scannerId;

CREATE

WITH-REFERENCE-OBJECT,

WITH-AUTOMATIC-INSTANCE-NAMING;

DELETE

DELETES-CONTAINED-OBJECTS;

REGISTERED AS {g774-6NameBinding 94};

# 13	Subordination rules

None.

# 14	Pointer constraints

None.

# 15	Supporting ASN.1 productions

SDHPMUNIASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) pmUni(06) informationModel(0) asn1Module(2) sdhpmUni (0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

IMPORTS

SDHGranularityPeriod, SDHPVGranularityPeriod FROM SDHPMASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) pm(01) informationModel(0) asn1Module(2) sdhpm (0)}

TimePeriod FROM MetricModule {joint-iso-itu-t ms(9) function(2) part11(11) asn1Module(2) 0};

sdhPMUni OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) pmUni(06) informationModel(0)}

g774-6MObjectClass OBJECT IDENTIFIER ::= {sdhPMUni managedObjectClass(3)}

g774-6Attribute OBJECT IDENTIFIER ::= {sdhPMUni attribute(7)}

g774-6NameBinding OBJECT IDENTIFIER ::= {sdhPMUni nameBinding(6)}

g774-6Package OBJECT IDENTIFIER ::= {sdhPMUni package(4)}

sdhPM OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) pm(01) informationModel(0) }

g774-01MObjectClass OBJECT IDENTIFIER ::= {sdhPM managedObjectClass(3)}

g774-01Attribute OBJECT IDENTIFIER ::= {sdhPM attribute(7)}

g774-01NameBinding OBJECT IDENTIFIER ::= {sdhPM nameBinding(6)}

g774-01Package OBJECT IDENTIFIER ::= {sdhPM package(4)}

END -- end of SDHPMUNIASN1

appendix I

Naming and inheritance diagrams

The naming and inheritance trees cover only the managed object classes of this Recommendation (see Figures I.1, I.2, I.3, I.4 and I.5).

Figure I.1/G.774.6  Object name binding for the unidirectional performance monitoring

Figure I.2/G.774.6  Object name binding for the unidirectional performance monitoring

Figure I.3/G.774.6  Object name binding for the unidirectional performance monitoring

Figure I.4/G.774.6  Object name binding for the unidirectional performance monitoring

Figure I.5/G.774.6  Inheritance tree for the unidirectional performance monitoring

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |