|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.7 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH)  Management of lower order path trace and interface labelling for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.7 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.7 Synchronous digital hierarchy (SDH)  Management of lower order path trace and interface labelling for the network element view

- Summary This Recommendation provides an information model for the Management of Lower Order Path Trace and Interface Labelling in Synchronous Digital Hierarchy (SDH) Networks. This model describes the managed object classes and their properties for the Lower Order Path Trace and Interface Labelling functions as related to SDH Network Elements. These objects are useful to describe information exchanged across interfaces defined in ITU-T M.3010 Telecommunications Management Network (TMN) architecture for the management of the Lower Order Path Trace and Interface Labelling functions.

Document History. Issue; Notes. 2001; First revision incorporated the changes documented in the Implementor's Guide.. 11/1996; Initial version of the Recommendation..

- Source ITU-T Recommendation G.774.7 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and Definitions		3

4	Abbreviations		3

5	Lower order path trace and interface labelling information model		3

5.1	Overview		3

5.2	Requirements		4

6	Object classes		4

6.1	Labelled electrical SPI trail termination point object classes		4

6.2	Labelled optical SPI trail termination point object classes		5

6.3	Virtual container 11 path trace object classes		6

6.4	Virtual container 12 object classes		6

6.5	Virtual container 2 object classes		7

6.6	Modifiable virtual container 2 path trace object classes		8

6.7	Modifiable virtual container 12 path trace object classes		8

6.8	Modifiable virtual container 11 path trace object classes		9

7	Packages		10

7.1	Virtual container 11-2 path trace packages		10

8	Attributes		10

9	Actions		11

10	Notifications		11

11	Parameters		11

12	Name bindings		11

13	Constraint rules		11

14	Subordination rules		11

15	Supporting ASN.1 productions		11

ITU-T Recommendation G.774.7

Synchronous digital hierarchy (SDH)  Management of lower order path trace 
and interface labelling for the network element view

# 1	Scope

This ITU-T Recommendation covers the following functionality:

	the ability to configure and retrieve a label associated with electrical SDH physical interfaces;

	the ability to configure and retrieve a label associated with optical SDH physical interfaces;

	the ability to configure path trace on SDH lower order paths.

The rationale for supporting the above functionality is that the functions were agreed as changes in the ITU-T G.774 Implementor's Guide, but these changes are functional extensions and not defect fixes. Therefore, a specific new Recommendation was created.

Structure of this Recommendation

Clause 5.1 provides an overview of the SDH Interface Label and Lower Order Path Trace information model. Clauses 6-12 describe the information model using the notation mechanisms defined in ITU-T X.722: Guidelines for the Definition of Managed Objects. Clause 15 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy (SDH).

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774 (2001), Synchronous digital hierarchy (SDH)  Management information model for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunications  management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview.

	ITU-T X.710 (1997), Information technology – Open Systems Interconnection – Common management information service.

	ITU-T X.711 (1997), Information technology – Open Systems Interconnection – Common management information protocol: Specification.

	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of management information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology – Open Systems Interconnection   Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed object, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology – Open Systems Interconnection – Systems Management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology – Open Systems Interconnection – Systems Management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology – Open Systems Interconnection – Systems Management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor. 2 (1999).

	ITU-T X.734 (1992), Information technology – Open Systems Interconnection – System Management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor. 2 (1999).

	ITU-T X.735 (1992), Information technology – Open Systems Interconnection – Systems Management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

# 3	Terms and Definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, ITU-T G.784 and ITU-T M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AU	Administrative Unit

AUG	Administrative Unit Group

Bid	Bidirectional

CTP	Connection Termination Point

GTP	Group Termination Point

Id	Identifier

MS	Multiplex Section

NE	Network Element

OS	Operations System

OSI	Open Systems Interconnection

PDH	Plesiochronous Digital Hierarchy

RS	Regenerator Section

SDH	Synchronous Digital Hierarchy

SPI	Synchronous Physical Interface

TMN	Telecommunication Management Network

TP	Termination Point

TTP	Trail Termination Point

TU	Tributary Unit

TUG	Tributary Unit Group

VC-n	Virtual Container n

# 5	Lower order path trace and interface labelling information model

## 5.1	Overview

Labelling of electrical and optical SDH physical interfaces is done using the following managed object classes which contain the M.3100 userlLabel attribute.

New Managed Object Classes

labelledElectricalSPITTPBidirectional
	labelledElectricalSPITTPSink

labelledElectricalSPITTPSource

labelledOpticalSPITTPBidirectional

labelledOpticalSPITTPSink

labelledOpticalSPITTPSource

Configuration of the SDH lower order path trace function is done using the following managed object classes and packages which contain attributes to provide access to and control of the j2PathTrace bytes in the SDH lower order path.

New Managed Object Classes

vc11PathTraceTTPBidirectional

vc11PathTraceTTPSink

vc11PathTraceTTPSource

vc12PathTraceTTPBidirectional

vc12PathTraceTTPSink

vc12PathTraceTTPSource

vc2PathTraceTTPBidirectional

vc2PathTraceTTPSink

vc2PathTraceTTPSource

modifiableVC2PathTraceTTPSink

modifiableVC2PathTraceTTPSource

modifiableVC2PathTraceTTPBidirectional

modifiableVC12PathTraceTTPSink

modifiableVC12PathTraceTTPSource

modifiable VC12PathTraceTTPBidirectional

modifiableVC11PathTraceTTPBidirectional

modifiableVC11PathTraceTTPSink

modifiableVC11PathTraceTTPSource

New Packages

vc11-2PathTraceSinkPackage

vc11-2PathTraceSourcePackage

## 5.2	Requirements

The information model must satisfy the following requirements:

	the ability to set and get the value a user specific label associated with an individual electrical SDH physical interface;

	the ability to set and get the value a user specific label associated with an individual optical SDH physical interface;

	the ability to set and get the value of the transmitted path trace identifier for an individual SDH lower order path;

	the ability to set and get the value of the expected path trace identifier for an individual SDH lower order path;

	the ability to get the value of the received path trace identifier for an individual SDH lower order path.

# 6	Object classes

## 6.1	Labelled electrical SPI trail termination point object classes

labelledElectricalSPITTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM

labelledElectricalSPITTPSink,

labelledElectricalSPITTPSource;

REGISTERED AS { g774-7ObjectClass 1 };

labelledElectricalSPITTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":electricalSPITTPSink;

CHARACTERIZED BY

labelledElectricalSPITTPSinkPkg PACKAGE

BEHAVIOUR

labelledElectricalSPITTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

*This object class provides the ability to label electrical SDH

physical trail termination points.*

;;

ATTRIBUTES

"Recommendation M.3100":userLabel	GET-REPLACE;

;;

REGISTERED AS { g774-7ObjectClass 2 };

labelledElectricalSPITTPSource MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation G.774":electricalSPITTPSource;

CHARACTERIZED BY

labelledElectricalSPITTPSourcePkg PACKAGE

BEHAVIOUR

labelledElectricalSPITTPSourcePkgBehaviour BEHAVIOUR
			DEFINED AS

*This object class provides the ability to label electrical SDH

physical trail termination points.*

;;

ATTRIBUTES

"Recommendation M.3100":userLabel	GET-REPLACE;

;;

REGISTERED AS { g774-7ObjectClass 3 };

## 6.2	Labelled optical SPI trail termination point object classes

labelledOpticalSPITTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM

labelledOpticalSPITTPSink,

labelledOpticalSPITTPSource;
REGISTERED AS { g774-7ObjectClass 4 };

labelledOpticalSPITTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":opticalSPITTPSink;

CHARACTERIZED BY

labelledOpticalSPITTPSinkPkg PACKAGE

BEHAVIOUR

labelledOpticalSPITTPSinkPkgBehaviour BEHAVIOUR

DEFINED AS

*This object class provides the ability to label optical SDH

physical trail termination points.*

;;

ATTRIBUTES

"Recommendation M.3100":userLabel	GET-REPLACE;

;;
REGISTERED AS { g774-7ObjectClass 5 };

labelledOpticalSPITTPSource MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation G.774": opticalSPITTPSource;

CHARACTERIZED BY

labelledOpticalSPITTPSourcePkg PACKAGE

BEHAVIOUR

labelledOpticalSPITTPSourcePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class provides the ability to label optical SDH

physical trail termination points.*

;;

ATTRIBUTES

"Recommendation M.3100":userLabel	GET-REPLACE;

;;

REGISTERED AS { g774-7ObjectClass 6 };

## 6.3	Virtual container 11 path trace object classes

vc11PathTraceTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM

"Recommendation G.774":vc11TTPBidirectionalR1,

vc11PathTraceTTPSink,

vc11PathTraceTTPSource;

REGISTERED AS { g774-7ObjectClass 7 };

vc11PathTraceTTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc11TTPSinkR1;

CHARACTERIZED BY

vc11-2PathTraceSinkPackage,

vc11TTPSinkPathTracePkg PACKAGE

BEHAVIOUR

vc11TTPSinkPathTracePkgBehaviour BEHAVIOUR

DEFINED AS
			*This object class supports the SDH lower order path trace

function. This CLASS shall be instantiated when lower order

path trace is supported.*

;;

;;

REGISTERED AS { g774-7ObjectClass 8 };

vc11PathTraceTTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc11TTPSource;

CHARACTERIZED BY

vc11-2PathTraceSourcePackage,

vc11TTPSourcePkgR1 PACKAGE

BEHAVIOUR

vc11TTPSourcePkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class supports the SDH lower order path trace

function. This CLASS shall be instantiated when lower order

path trace is supported.*

;;

;;

REGISTERED AS { g774-7ObjectClass 9 };

## 6.4	Virtual container 12 object classes

vc12PathTraceTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc12TTPBidirectionalR1,

vc12PathTraceTTPSink,

vc12PathTraceTTPSource;

REGISTERED AS { g774-7ObjectClass 10 };

vc12PathTraceTTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc12TTPSinkR1;

CHARACTERIZED BY

vc11-2PathTraceSinkPackage,

vc12TTPSinkPathTracePkg PACKAGE

BEHAVIOUR

vc12TTPSinkPathTracePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class supports the SDH lower order path trace

function. This CLASS shall be instantiated when lower order

path trace is supported.*

;;

;;

REGISTERED AS { g774-7ObjectClass 11 };

vc12PathTraceTTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc12TTPSource;

CHARACTERIZED BY

vc11-2PathTraceSourcePackage,

vc12TTPSourcePkgR1 PACKAGE

BEHAVIOUR

vc12TTPSourcePkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class supports the SDH lower order path trace

function. This CLASS shall be instantiated when lower order

path trace is supported.*

;;

;;

REGISTERED AS { g774-7ObjectClass 12 };

## 6.5	Virtual container 2 object classes

vc2PathTraceTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc2TTPBidirectionalR1,

vc2PathTraceTTPSink,

vc2PathTraceTTPSource;

REGISTERED AS { g774-7ObjectClass 13 };

vc2PathTraceTTPSink MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc2TTPSinkR1;

CHARACTERIZED BY

vc11-2PathTraceSinkPackage,

vc2TTPSinkPathTracePkg PACKAGE

BEHAVIOUR

vc2TTPSinkPathTracePkgBehaviour BEHAVIOUR

DEFINED AS

*This object class supports the SDH lower order path trace

function. This CLASS shall be instantiated when lower order

path trace is supported.*

;;

;;

REGISTERED AS { g774-7ObjectClass 14 };

vc2PathTraceTTPSource MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":vc2TTPSource;

CHARACTERIZED BY

vc11-2PathTraceSourcePackage,

vc2TTPSourcePkgR1 PACKAGE

BEHAVIOUR

vc2TTPSourcePkgR1Behaviour BEHAVIOUR

DEFINED AS

*This object class supports the SDH lower order path trace

function.This CLASS shall be instantiated when lower order path

trace is supported.*

;;

;;

REGISTERED AS { g774-7ObjectClass 15 };

## 6.6	Modifiable virtual container 2 path trace object classes

modifiableVC2PathTraceTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM vc2PathTraceTTPBidirectional;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC2PathTraceTTPBidPackage PACKAGE

BEHAVIOUR

modifiableVC2PathTraceTTPBidBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower

order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 16  };

modifiableVC2PathTraceTTPSink MANAGED OBJECT CLASS

DERIVED FROM vc2PathTraceTTPSink;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC2PathTraceTTPSinkPackage PACKAGE

BEHAVIOUR

modifiableVC2PathTraceTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower

order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 17  };

modifiableVC2PathTraceTTPSource MANAGED OBJECT CLASS

DERIVED FROM vc2PathTraceTTPSource;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC2PathTraceTTPSourcePackage PACKAGE

BEHAVIOUR

modifiableVC2PathTraceTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower

order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 18  };

## 6.7	Modifiable virtual container 12 path trace object classes

modifiableVC12PathTraceTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM vc12PathTraceTTPBidirectional;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVCPathTrace12TTPBidPackage PACKAGE

BEHAVIOUR

modifiableVC12PathTraceTTPBidBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower

order path trace is supported.*

;;

ACTIONS

“Recommendation G.774.2”:defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 19  };

modifiableVCPathTrace12TTPSink MANAGED OBJECT CLASS

DERIVED FROM vc12PathTraceTTPSink;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC12PathTraceTTPSinkPackage PACKAGE

BEHAVIOUR

modifiableVC12PathTraceTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower 
			order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 20  };

modifiableVC12PathTraceTTPSource MANAGED OBJECT CLASS

DERIVED FROM vc12PathTraceTTPSource;

CHARACTERIZED BY

"Recommendation M.3100":  supportableClientListPackage,

modifiableVC12PathTraceTTPSourcePackage PACKAGE

BEHAVIOUR

modifiableVC12PathTraceTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and

lower order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 21  };

## 6.8	Modifiable virtual container 11 path trace object classes

modifiableVC11PathTraceTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM vc11PathTraceTTPBidirectional;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC11PathTraceTTPBidPackage PACKAGE

BEHAVIOUR

modifiableVC11PathTraceTTPBidBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower

order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 22  };

modifiableVC11PathTraceTTPSink MANAGED OBJECT CLASS

DERIVED FROM vc11PathTraceTTPSink;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC11PathTraceTTPSinkPackage PACKAGE

BEHAVIOUR

modifiableVC11PathTraceTTPSinkBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower

order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 23  };

modifiableVC11PathTraceTTPSource MANAGED OBJECT CLASS

DERIVED FROM vc11PathTraceTTPSource;

CHARACTERIZED BY

"Recommendation M.3100": supportableClientListPackage,

modifiableVC11PathTraceTTPSourcePackage PACKAGE

BEHAVIOUR

modifiableVC11PathTraceTTPSourceBehaviour  BEHAVIOUR

DEFINED AS

*This CLASS shall be instantiated when change of the SDH

frame structure by management operation is supported and lower

order path trace is supported.*

;;

ACTIONS

"Recommendation G.774.2":defineClientType;;;

REGISTERED AS {  g774-7ObjectClass 24  };

# 7	Packages

## 7.1	Virtual container 11-2 path trace packages

vc11-2PathTraceSinkPackage PACKAGE

BEHAVIOUR

vc11-2PathTraceSinkPackageBehaviour BEHAVIOUR

DEFINED AS

*When 16 bytes are supported, the 16 bytes of the path trace shall be conveyed at the management interface in both ways. This is a local issue whether the NE recompute the CRC7 under a replace operation.*

;;

ATTRIBUTES

"Recommendation G.774.5":j2PathTraceExpected

REPLACE-WITH-DEFAULT

DEFAULT VALUE SDHPTLASN1.nullDefault

GET-REPLACE,

"Recommendation G.774.5":j2PathTraceReceive		GET;

;

vc11-2PathTraceSourcePackage PACKAGE

BEHAVIOUR

vc11-2PathTraceSourcePackageBehaviour BEHAVIOUR

DEFINED AS

*When 16 bytes are supported, the 16 bytes of the path trace shall be conveyed at the management interface.*

;;

ATTRIBUTES

"Recommendation G.774.5":j2PathTraceSend		GET-REPLACE;

;

# 8	Attributes

None.

# 9	Actions

None.

# 10	Notifications

None.

# 11	Parameters

None.

# 12	Name bindings

None.

# 13	Constraint rules

None.

# 14	Subordination rules

None.

# 15	Supporting ASN.1 productions

This clause contains all the ASN.1 definitions required to support all the new GDMO definitions within this Recommendation.

SDHPTLASN1 { itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) ptl(7) informationModel(0)

asn1Module(2) sdhptl (0) }

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

sdhPTL OBJECT IDENTIFIER ::= { itu-t(0) recommendation(0) g(7) g774(774)

hyphen(127) ptl(7) informationModel(0) }

g774-7ObjectClass OBJECT IDENTIFIER ::= { sdhPTL managedObjectClass(3) }

g774-7Action OBJECT IDENTIFIER ::= { sdhPTL action(9) }

g774-7NameBinding OBJECT IDENTIFIER ::= { sdhPTL nameBinding(6) }

g774-7Parameter OBJECT IDENTIFIER ::= { sdhPTL parameter(5)}

g774-7Notification OBJECT IDENTIFIER ::= { sdhPTL notification(10) }

g774-7Package OBJECT IDENTIFIER ::= { sdhPTL package(4) }

g774-7Attribute OBJECT IDENTIFIER ::= { sdhPTL attribute(7) }

nullDefault Null ::= NULL

Null ::= NULL

END

-- end of supporting asn.1 productions

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |