|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.8 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH)  Management of radio-relay systems for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.8 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.8 Synchronous digital hierarchy (SDH)  Management of radio-relay systems for the network element view

- Summary This Recommendation provides an information model for the Synchronous Digital Hierarchy (SDH) Radio-Relay Network Elements. This model describes the managed object classes and their properties for the Radio Synchronous Physical Interface and the protection function of sections having such a physical interface. These objects are useful to describe information exchanged across interfaces defined in ITU-T M.3010 Telecommunications Management Network (TMN) architecture for the management of the Radio-Relay Network Elements.

Document History. Issue; Notes. 2001; First revision incorporated the changes documented in the Implementors Guide. 4/1997; Initial version of the Recommendation.

- Source ITU-T Recommendation G.774.8 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and Definitions		2

4	Abbreviations		3

5	SDH radio TP fragment		4

5.1	Object classes definitions		4

5.1.1	Radio synchronous physical interface		4

5.2	Packages definitions		5

5.3	Attributes definitions		6

5.4	Name bindings definitions		7

5.5	Object relations		8

5.6	Supporting ASN.1		9

6	SDH radio protection fragment		10

6.1	Object classes definitions		10

6.1.1	Generic object definitions		10

6.1.2	MS tandem connection protection object definitions		11

6.1.3	High order path connection protection object definitions		15

6.1.4	Multiplex section trail protection (by means of RPS) object definitions		18

6.2	Packages definitions		20

6.3	Attributes definitions		20

6.3.1	hitless		20

6.3.2	radioHoldOffTime		21

6.3.3	rpsSummaryStatus		21

6.3.4	exerciseOn		21

6.3.5	privilegedChannel		22

6.3.6	radioProtectionStatus		22

6.3.7	radioUnprotectedCTPId		22

6.3.8	radioProtectedTTPId		23

6.4	Actions definitions		23

6.5	Parameters definitions		24

6.5.1	Radio protection status parameter		24

6.6	Name bindings definitions		24

6.6.1	augSink		24

6.6.2	augSource		24

6.6.3	msTcCTPSink		25

Page

6.6.4	msTcCTP Source		25

6.6.5	msTcTTP Sink		25

6.6.6	msTcTTP Source		25

6.6.7	vc4HopcTTP Sink		26

6.6.8	vc4HopcTTP Source		26

6.6.9	au4HopcCTPSink		26

6.6.10	au4HopcCTP Source		26

6.7	Supporting ASN.1		27

Appendix I – Figures		29

ITU-T Recommendation G.774.8

Synchronous digital hierarchy (SDH)  Management of radio-relay
systems for the network element view

# 1	Scope

This Recommendation provides an information model to be used at the interface between Network Elements and Management Systems, for the management of radio-relays equipment which use the Synchronous Digital Hierarchy (SDH). It identifies the Telecommunications Management Network (TMN) object classes required for the management of the Radio-Relay SDH network elements. These objects are relevant to information exchanged across standardized interfaces defined in the M.3010 TMN architecture.

Structure of this Recommendation

Clauses 5 to 6 describe the information model using the notation mechanisms defined in ITU-T X.722: Guidelines for the Definition of Managed Objects. Clauses 5.6 and 6.7 contain the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. Naming and Inheritance are diagrammatically shown in informative Appendix I.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

–	ITU-T G.707/Y.1322 (2000), Network-node interface for the synchronous digital hierarchy (SDH).

–	ITU-T G.773 (1993), Protocol suites for Q-interface protocols for management of transmission systems.

–	ITU-T G.774 (2001), Synchronous digital hierarcy (SDH)   Management information model for the network element view.

–	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

–	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

–	ITU-T G.803 (2000), Architectures of transport networks based on the synchronous digital hierarchy (SDH).

–	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

–	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

–	ITU-T M.60 (1993), Maintenance terminology and definitions.

–	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

–	ITU-T M.3010 (2000), Principles for a telecommunications management network.

–	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

–	ITU-T M.3100 (1995), Generic network information model.

–	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

–	ITU-T Q.812 (1997), Higher layer protocol profiles for the Q3 and X interfaces.

–	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

–	ITU-T X.680 to X.683 (1997), Information technology    Abstract Syntax Notation One (ASN.1).

–	ITU-T X.701 (1997), Information technology  Open Systems Interconnection  Systems management overview.

–	ITU-T X.710 (1997), Information technology   Open Systems Interconnection  Common management information service.

–	ITU-T X.711 (1997), Information technology  Open Systems Interconnection  Common management information protocol: Specification.

–	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

–	ITU-T X.721 (1992), Information technology  Open Systems Interconnection  Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

–	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

–	ITU-T X.730 (1992), Information technology  Open Systems Interconnection  Systems Management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

–	ITU-T X.731 (1992), Information technology   Open Systems Interconnection  Systems Management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

–	ITU-T X.733 (1992), Information technology  Open Systems Interconnection  Systems Management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

–	ITU-T X.734 (1992), Information technology  Open Systems Interconnection  Systems Management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

–	ITU-T X.735 (1992), Information technology  Open Systems Interconnection  Systems Management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

# 3	Terms and Definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, ITU-T G.784 and ITU-T M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

AIS		Alarm Indication Signal

AP		Access Point

CMIP		Common Management Information Protocol

CMIS		Common Management Information Service

CP		Connection Point

CTP		Connection Termination Point

DRR		Digital Radio-Relay

GTP		Group Termination Point

HPA		Higher order Path Adaptation

IA		Indirect Adapter

IOS		Intra-Office Section

ISO		International Organization for Standardization

ITU-T		International Telecommunication Union  Telecommunication Standardization sector

LOF		Loss of Frame

LPA		Lower order Path Adaptation

NE		Network Element

OS		Operations System

OSI		Open Systems Interconnection

PDH		Plesiochronous Digital Hierarchy

Pkg		Package

RF		Radio Frequency

RPS		Radio Protection Switching

RRR		Radio-Relay Regenerator

RRT		Radio-Relay Terminal

RS		Regenerator Section

RSPI		Radio Synchronous Physical Interface

SDH		Synchronous Digital Hierarchy

Snk		Sink

Src		Source

STM-N		Synchronous Transport Module N

STM-RR	Synchronous Transport Module for Sub-STM-1 radio-relay

TMN		Telecommunications Management Network

TP		Termination Point

TTP		Trail Termination Point

VC-n		Virtual Container n

# 5	SDH radio TP fragment

This clause provides Managed Objects required to model Radio SDH Physical interfaces.

## 5.1	Object classes definitions

### 5.1.1	Radio synchronous physical interface

This clause describes the object classes required to model the Radio SDH physical interface.

radioSPITTPBidirectional MANAGED OBJECT CLASS

- DERIVED FROM "Recommendation M.3100":trailTerminationPointBidirectional,

radioSPITTPSink,

radioSPITTPSource;

- REGISTERED AS { g774-8ObjectClass 1 };

radioSPITTPSink MANAGED OBJECT CLASS

- DERIVED FROM "Recommendation M.3100":trailTerminationPointSink;

- CHARACTERIZED BY

- "Recommendation X.721":administrativeStatePackage,

- "Recommendation M.3100":createDeleteNotificationsPackage,

- "Recommendation M.3100":stateChangeNotificationPackage,

- "Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

radioSPIPackage,

radioSPITTPSinkPkg  PACKAGE

- BEHAVIOUR

radioSPITTPSinkPkgBehaviour BEHAVIOUR

- DEFINED AS

"This managed object class represents the process of converting the incoming radio frequency signal into an internal logic level STM-N signal and the recovering of the timing from the incoming signal.

- The upstream connectivity pointer is NULL for an instance of this class.

The aforementioned process is composed of two subfunctions, namely the RX subfunction and the Demodulation subfunction.

A communicationsAlarm notification shall be issued if the RX subfunction fails. The probableCause parameter of the notification shall indicate rxFail.

A communicationsAlarm notification shall be issued if the Demodulation subfunction fails. The probableCause parameter of the notification shall indicate demodulationFail.

- When an alarm is pending on an instance, its operationalState is disabled."

;;;;

- CONDITIONAL PACKAGES

rxLOSNotificationPackage PRESENT IF "an instance supports it",

demLOSNotificationPackage PRESENT IF "an instance supports it";

- REGISTERED AS { g774-8ObjectClass 2 };

radioSPITTPSource MANAGED OBJECT CLASS

- DERIVED FROM "Recommendation M.3100":trailTerminationPointSource;

- CHARACTERIZED BY

- "Recommendation X.721":administrativeStatePackage,

- "Recommendation M.3100":createDeleteNotificationsPackage,

- "Recommendation M.3100":stateChangeNotificationPackage,

- "Recommendation M.3100":tmnCommunicationsAlarmInformationPackage,

radioSPIPackage,

radioSPITTPSourcePkg  PACKAGE

BEHAVIOUR

radioSPITTPSourcePkgBehaviour BEHAVIOUR

- DEFINED AS

"This managed object class represents the process of converting an outgoing internal logic level STM-N signal into a radio frequency signal.

- The downstream connectivity pointer is NULL for an instance of this class.

The aforementioned process is composed of two subfunctions, namely the TX subfunction and the Modulation subfunction.

A communicationsAlarm notification shall be issued if the TX subfunction fails. The probableCause parameter of the notification shall indicate txFail.

A communicationsAlarm notification shall be issued if the Modulation subfunction fails. The probableCause parameter of the notification shall indicate modulationFail.

- When an alarm is pending on an instance, its operationalState is disabled.";;

- ATTRIBUTES atpcImplemented GET;;;

- CONDITIONAL PACKAGES

atpcPackage PRESENT IF "the ATPC is implemented and an instance supports it",

txLOSNotificationPackage PRESENT IF "an instance supports it",

modLOSNotificationPackage PRESENT IF "an instance supports it";

- REGISTERED AS { g774-8ObjectClass 3 };

## 5.2	Packages definitions

atpcPackage	PACKAGE

- ATTRIBUTES

atpcEnabled	GET-REPLACE;

- REGISTERED AS { g774-8Package 1 };

radioSPIPackage		PACKAGE

- ATTRIBUTES

radioSPITTPId		GET,

radioFrequency		GET,

"Recommendation G.774":stmLevel	GET;

- REGISTERED AS { g774-8Package 2 };

rxLOSNotificationPackage	PACKAGE

- BEHAVIOUR

rxLOSNotificationPackageBehaviour		BEHAVIOUR

- DEFINED AS

"A communicationsAlarm notification shall be issued if a loss of the incoming signal for the RX subfunctions is detected. The probableCause parameter of the notification shall indicate rxLOS. The rxLOS probableCause in the communicationsAlarm notification should be used only when the distinction between the fail of the RX subfunction and the loss of the incoming signal can be carried out with sufficient degree of confidence."

;;

- REGISTERED AS { g774-8Package 3 };

demLOSNotificationPackage	PACKAGE

- BEHAVIOUR

demLOSNotificationPackageBehaviour	BEHAVIOUR

- DEFINED AS

"A communicationsAlarm notification shall be issued if a loss of the incoming signal for the Demodulation subfunctions is detected. The probableCause parameter of the notification shall indicate demLOS. The demLOS probableCause in the communicationsAlarm notification should be used only when the distinction between the fail of the Demodulation subfunction and the loss of the incoming signal can be carried out with sufficient degree of confidence."

;;

- REGISTERED AS { g774-8Package 4 };

txLOSNotificationPackage	PACKAGE

- BEHAVIOUR

txLOSNotificationPackageBehaviour		BEHAVIOUR

- DEFINED AS

"A communicationsAlarm notification shall be issued if a loss of the OUTgoing signal for the TX subfunctions is detected. The probableCause parameter of the notification shall indicate txLOS. The txLOS probableCause in the communicationsAlarm notification should be used only when the distinction between the fail of the TX subfunction and the loss of the incoming signal can be carried out with sufficient degree of confidence."

;;

- REGISTERED AS { g774-8Package 5 };

modLOSNotificationPackage	PACKAGE

- BEHAVIOUR

modLOSNotificationPackageBehaviour	BEHAVIOUR

- DEFINED AS

"A communicationsAlarm notification shall be issued if a loss of the OUTgoing signal for the Modulation subfunctions is detected. The probableCause parameter of the notification shall indicate modLOS. The modLOS probableCause in the communicationsAlarm notification should be used only when the distinction between the fail of the Modulation subfunction and the loss of the incoming signal can be carried out with sufficient degree of confidence."

;;

- REGISTERED AS { g774-8Package 6 };

## 5.3	Attributes definitions

atpcImplemented ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHRadioTpASN1.Boolean;

- MATCHES FOR EQUALITY;

- BEHAVIOUR

atpcImplementedBeh BEHAVIOUR

- DEFINED AS

- "This attribute specifies whether the ATPC capability is present or not.

A value of TRUE indicates that the ATPC capability is present and a value of FALSE indicates that the ATPC capability is not present." 
;;

- REGISTERED AS {g774-8Attribute 1};

atpcEnabled ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDHRadioTpASN1.Boolean;

- MATCHES FOR EQUALITY;

- BEHAVIOUR

atpcEnabledBeh BEHAVIOUR

DEFINED AS

- "This attribute specifies whether the ATPC device is currently allowed to work or not.

A value of TRUE indicates that the ATPC device is allowed to work and a value of FALSE indicates that the device is not allowed to work (i.e. the transmit power has a fixed value)." ;;

- REGISTERED AS {g774-8Attribute 2};

radioFrequency ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDHRadioTpASN1.RadioFrequency;

- MATCHES FOR EQUALITY;

- BEHAVIOUR

radioFrequencyBehaviour BEHAVIOUR

- DEFINED AS

"This attribute is used to specify the carrier radio frequencies and optionally the related polarization states associated to instances of radioSPITTPSink, radioSPITTPSource and radioSPITTPBidirectional managed object Classes.

- It also indicates if each specified radio frequency is used at transmit or receive side.

- Frequency value are expressed in MHz.

- For instances of radioSPITTPSink (radioSPITTPSource) managed object Class, the FrequencyUsage sub-field can take only the receive (transmit) value."
;;

- REGISTERED AS {g774-8Attribute 3};

radioSPITTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX	SDHRadioTpASN1.NameType;

- MATCHES FOR EQUALITY;

- BEHAVIOUR

radioSPITTPIdBehaviour BEHAVIOUR

- DEFINED AS

- "This attribute is used as a RDN for naming instances of the radioSPITTP object classes."

- ;;

- REGISTERED AS {g774-8Attribute 4};

## 5.4	Name bindings definitions

radioSPITTPSink-managedElement		NAME BINDING

SUBORDINATE OBJECT CLASS		radioSPITTPSink AND SUBCLASSES;

- NAMED BY

SUPERIOR OBJECT CLASS			“Recommendation M.3100”:managedElement AND SUBCLASSES;

WITH ATTRIBUTE	radioSPITTPId;

- BEHAVIOUR

radioSPITTPSink-managedElementBehaviour	BEHAVIOUR

- DEFINED AS

- "The subordinate managed object may be automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment."
;;

- REGISTERED AS { g774-8NameBinding 1};

radioSPITTPSource-managedElement		NAME BINDING

SUBORDINATE OBJECT CLASS		radioSPITTPSource AND SUBCLASSES;

- NAMED BY

SUPERIOR OBJECT CLASS			“Recommendation M.3100”:managedElement AND SUBCLASSES;

WITH ATTRIBUTE		radioSPITTPId;

- BEHAVIOUR

radioSPITTPSource-managedElementBehaviour		BEHAVIOUR

- DEFINED AS

- "The subordinate managed object may be automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment."
;;

- REGISTERED AS { g774-8NameBinding 2};

rsCTPSink-radioSPITTPSink			NAME BINDING

SUBORDINATE OBJECT CLASS		"Recommendation G.774":rsCTPSink AND SUBCLASSES;

- NAMED BY

SUPERIOR OBJECT CLASS		radioSPITTPSink AND SUBCLASSES;

WITH ATTRIBUTE	"Recommendation G.774":rsCTPId;

- BEHAVIOUR

rsCTPSink-radioSPITTPSinkBehaviour	BEHAVIOUR

- DEFINED AS

- "The subordinate managed object may be automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment."

;;

- REGISTERED AS {g774-8NameBinding 3};

rsCTPSource-radioSPITTPSource		NAME BINDING

SUBORDINATE OBJECT CLASS		"Recommendation G.774":rsCTPSource AND SUBCLASSES;

- NAMED BY

SUPERIOR OBJECT CLASS		radioSPITTPSource AND SUBCLASSES;

WITH ATTRIBUTE	"Recommendation G.774":rsCTPId;

- BEHAVIOUR

rsCTPSource-radioSPITTPSourceBehaviour	BEHAVIOUR

- DEFINED AS

- "The subordinate managed object may be automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment."

;;

- REGISTERED AS {g774-8NameBinding 4};

## 5.5	Object relations

radioSPITTPBidirectionalSubordination		SUBORDINATION RULE

- SUPERIOR OBJECT CLASS

radioSPITTPBidirectional;

- NAMES SUBORDINATES

rsCTPSink, rsCTPSource, rsCTPBidirectional;

- ACCORDING TO RULE

- SET SIZE(1) OF CHOICE {

rsCTPSink, rsCTPSource, rsCTPBidirectional };

;

radioSPITTPSinkSubordination		SUBORDINATION RULE

- SUPERIOR OBJECT CLASS

radioSPITTPSink;

- NAMES SUBORDINATES

rsCTPSink;

- ACCORDING TO RULE

- SET SIZE(1) OF rsCTPSink;

;

radioSPITTPSourceSubordination	SUBORDINATION RULE

- SUPERIOR OBJECT CLASS

radioSPITTPSource;

- NAMES SUBORDINATES

rsCTPSource;

- ACCORDING TO RULE

- SET SIZE(1) OF rsCTPSource;

;

## 5.6	Supporting ASN.1

- SDHRadioManagement {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127)

sdhRadioIM(8)}

- DEFINITIONS IMPLICIT TAGS ::= BEGIN

-- EXPORTS Everything

sdhRadioManagement OBJECT IDENTIFIER::= {itu-t(0) recommendation(0) g(7)

g774(774) hyphen(127) sdhRadioIM(8) informationModel(0)}

g774-8ObjectClass OBJECT IDENTIFIER ::= {sdhRadioManagement managedObjectClass(3)}

g774-8Attribute OBJECT IDENTIFIER ::= {sdhRadioManagement attribute(7)}

g774-8NameBinding OBJECT IDENTIFIER ::= {sdhRadioManagement nameBinding(6)}

g774-8Action OBJECT IDENTIFIER ::= {sdhRadioManagement action(9)}

g774-8Notification OBJECT IDENTIFIER ::= {sdhRadioManagement notification(10)}

g774-8Package OBJECT IDENTIFIER ::= {sdhRadioManagement package(4)}

g774-8Parameter OBJECT IDENTIFIER ::= {sdhRadioManagement parameter(5)}

- END

- SDHRadioTpASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127)

sdhRadioIM(8) informationModel(0) asn1Module(2) sdhRadioTp(0)}

- DEFINITIONS IMPLICIT TAGS ::= BEGIN

-- EXPORTS Everything

- IMPORTS

- NameType FROM ASN1DefinedTypesModule {itu-t(0) recommendation(0) m(13)

gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypesModule(0)}

- ProbableCause FROM Attribute-ASN1Module {joint-iso-ccitt ms(9) smi(3)

part2(2) asn1Module(2) 1};

- RadioFrequency ::= SEQUENCE OF SEQUENCE {

frequencyValue [0] INTEGER,

frequencyUsage [1] FrequencyUsage,

polarization	[2] Polarization OPTIONAL

}

- Boolean ::= BOOLEAN

- Integer ::= INTEGER

- Polarization ::= ENUMERATED {vertical (0), horizontal (1),unspecified (2)}

- FrequencyUsage ::= ENUMERATED {receive (0), transmit (1)}

-- The following value assignments specify the Probable Cause value related to

-- Radio-Relay management within the TMN application context. The chosen values

-- are reserved by Recommendation M.3100 for communication alarm-related probable

-- causes.

rxFail ProbableCause ::= localValue : 30

rxLOS ProbableCause ::= localValue : 31

demodulationFail ProbableCause ::= localValue : 32

demLOS ProbableCause ::= localValue : 33

txFail ProbableCause ::= localValue : 34

txLOS ProbableCause ::= localValue : 35

modulationFail ProbableCause ::= localValue : 36

modLOS ProbableCause ::= localValue : 37

- END

# 6	SDH radio protection fragment

This clause provides Managed Objects required for modeling the Radio Protection Switching function.

## 6.1	Object classes definitions

### 6.1.1	Generic object definitions

#### 6.1.1.1	SDH radio protection group

sdhRadioProtectionGroup MANAGED OBJECT CLASS

- DERIVED FROM "Recommendation G.774.03":protectionGroupR1;

- CHARACTERIZED BY

sdhRadioProtectionGroupPkg PACKAGE

- BEHAVIOUR sdhRadioProtectionGroupBeh;

- ATTRIBUTES

rpsSummaryStatus		GET,

"Recommendation G.774.03":protectionSwitchMode  GET,

hitless GET;

NOTIFICATIONS

"Recommendation G.774.03":protectionSwitchReportingR1

radioProtectionStatusParameter;;;

- CONDITIONAL PACKAGES

- "Recommendation G.774.03":protectionMismatchStatusPkg  PRESENT IF

- "An APS protocol is used",

radioHoldOffTimePkg PRESENT IF "the hitless functionality is not 			present and an instance supports it",

singleExercisePkg   PRESENT IF "an instance supports it",

exerciseOnOffPkg PRESENT IF "an instance supports it";

- REGISTERED AS {g774-8ObjectClass 4 } ;

sdhRadioProtectionGroupBeh BEHAVIOUR

- DEFINED AS

"This object class is used to model all radio protection schemes envisaged for the Radio Protection Switch function. This object class is the focal point for management operations and notifications related to management of the protection system.

The protectionMismatchStatus indicates a mismatch between the provisioned protectionGroupType of this protectionGroup and the provisioned protectionGroupType of the far-end. It also indicates mismatch of uni-directional versus bi-directional switch provisioning between the two protection groups.

Actual signal flow across any specific transport entity  is reflected by the connectivity pointers of the TPs involved in the protection scheme represented by an instance of this class.

For instances of this managed object class the REPLACE operation on the attributes revertive, waitToRestoreTime and protectionGroupType is not mandatorily required.

When the exerciseOnOffPkg package is present, it is possible to start and to stop the exercise procedure on the RPS acting on the exerciseOn boolean attribute. When the exerciseOn attribute is TRUE, possible malfunctioning of the exercise procedure shall be signalled setting the 'degraded' component in the availabilityStatus attribute.

If the attributeValueChangeNotification package is present, then changes to the exerciseOn and radioHoldOffTime (if they are present) shall cause an attributeValueChange notification to be emitted.

The protectionUnit sub-field of the protectionSwitchReportingInfo has no meaning in protectionSwitchReporting notification emitted by instances of this managed object class. The conditions for emitting the protectionSwitchReporting notifications are specified in the behaviour of the radioProtectionStatusParameter parameter.";

#### 6.1.1.2	SDH radio protection unit

sdhRadioProtectionUnit MANAGED OBJECT CLASS

- DERIVED FROM "Recommendation G.774.03":protectionUnit;

- CHARACTERIZED BY

sdhRadioProtectionUnitPkg PACKAGE

- BEHAVIOUR sdhRadioProtectionUnitBeh ;

- ATTRIBUTES

"Recommendation G.774.03":channelNumber	GET,

radioProtectionStatus	GET,

"Recommendation G.774.03":reliableResourcePointer	PERMITTED VALUES

SDHRadioProtASN1.SDHRadioResourcePointer,

"Recommendation G.774.03":unreliableResourcePointer PERMITTED VALUES

SDHRadioProtASN1.SDHRadioResourcePointer  ;;;

- CONDITIONAL PACKAGES

"Recommendation G.774.03":extraTrafficControlPkg  PRESENT IF " extra traffic may be suspended and resumed " ,

privilegedChannelPkg

PRESENT IF "an instance supports it and the instance is protecting";

- REGISTERED AS {g774-8ObjectClass 5 } ;

sdhRadioProtectionUnitBeh BEHAVIOUR

- DEFINED AS

"This object class is specific to SDH Radio Protection Systems. Instances of this object class are used to represent the assignment between an unreliable resource (termination point) and a reliable resource (termination point) for the purpose of protection of the transport entity involved in any particular RPS protection scheme.

If this is a protecting protection unit , the reliableResourcePointer points to the protected termination point for extra traffic or NULL if there is no extra traffic.

The channelNumber attribute value represents the number of the channel used by the automatic protection switching protocol, if any.";

### 6.1.2	MS tandem connection protection object definitions

This clause provides the object classes required to model protection of Multiplex Section Tandem Connections.

#### 6.1.2.1	MS tandem connection CTP

msTcCTPBidirectional    MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774":msCTPBidirectional,

msTcCTPSource,

msTcCTPSink  ;

- CHARACTERIZED BY

msTcCTPBidirectionalPkg  PACKAGE

- BEHAVIOUR msTcCTPBidirectionalBeh ;;;

- REGISTERED AS {g774-8ObjectClass 6 } ;

msTcCTPBidirectionalBeh BEHAVIOUR

- DEFINED AS

"The msTcCTPBidirectional object class is a class of objects that represents either the bidirectional protected resources or the bidirectional unprotected resources in a tandem connection made up of one or more link connections at multiplex section layer.

If a bidirectional SDH Radio Protection Switching function is present, this object class shall be supported." ;

msTcCTPSink    MANAGED OBJECT CLASS

DERIVED FROM   "Recommendation G.774": msCTPSink ;

- CHARACTERIZED BY

- "Recommendation M.3100": crossConnectionPointerPackage,

msTcCTPSinkPkg  PACKAGE

- BEHAVIOUR msTcCTPSinkBeh ;;;

- REGISTERED AS {g774-8ObjectClass 7 } ;

msTcCTPSinkBeh BEHAVIOUR

- DEFINED AS

"The msTcCTPSink object class is a class of objects that represents either the protected or the unprotected resources in a tandem connection made up of one or more link connections at multiplex section layer. An instance of this object class defines the tandem connection end-point which terminates a multiplex section connection.

An instance of this object class is pointed to by a reliableResourcePointer attribute or by an unreliableResourcePointer attribute in an instance of the protectionUnit object class according if it represents a protected or an unprotected tandem connection respectively.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer or unreliableResourcePointer attribute pointing back to the instance of this object class.

If an instance of this object class represents a protected tandem connection, then the downStreamConnectivityPointer points to either null or its associated msTcCTP object instance(s) representing the unprotected tandem connection(s).

If an instance of this object class represents an unprotected tandem connection, then the downStreamConnectivityPointer points to either null or its associated msTcCTP or msTcTTP object instance representing the protected tandem connection.

- When a signal is switched to another unit, the value of the pointer is updated." ;

msTcCTPSource    MANAGED OBJECT CLASS

DERIVED FROM   "Recommendation G.774": msCTPSource ;

- CHARACTERIZED BY

- "Recommendation M.3100": crossConnectionPointerPackage,

msTcCTPSourcePkg  PACKAGE

- BEHAVIOUR msTcCTPSourceBeh ;;;

- REGISTERED AS {g774-8ObjectClass 8 } ;

msTcCTPSourceBeh BEHAVIOUR

- DEFINED AS

"The msTcCTPSource object class is a class of objects that represents either the protected resources or the unprotected resources in a tandem connection made up of one or more link connections at multiplex section layer. An instance of this object class defines the tandem connection end-point which originates a multiplex section connection.

An instance of this object class is pointed to by a reliableResourcePointer attribute or by an unreliableResourcePointer attribute in an instance of the sdhRadioProtectionUnit object class according if it represents a protected or an unprotected tandem connection respectively.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer or unreliableResourcePointer attribute pointing back to the instance of this object class.

If an instance of this object class represents a protected tandem connection, then the upStreamConnectivityPointer points to either null or its associated msTcCTP object instance representing the unprotected tandem connection.

If an instance of this object class represents an unprotected tandem connection, then the upStreamConnectivityPointer points to either null or its associated msTcCTP or msTcTTP object instance representing the protected tandem connection.

- When a signal is switched to another unit, the value of the pointer is updated." ;

#### 6.1.2.2	MS tandem connection TTP

msTcTTPBidirectional    MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774": msTTPBidirectional,

msTcTTPSource,

msTcTTPSink  ;

- CHARACTERIZED BY

msTcTTPBidirectionalPkg  PACKAGE

- BEHAVIOUR msTcTTPBidirectionalBeh ;;;

- REGISTERED AS {g774-8ObjectClass 9 } ;

msTcTTPBidirectionalBeh BEHAVIOUR

- DEFINED AS

"The msTcTTPBidirectional object class is a class of objects that represents the bidirectional protected resources in a tandem connection made up of one or more link connections at multiplex section layer.

If a bidirectional SDH Radio Protection Switching function is present, this object class shall be supported." ;

msTcTTPSink    MANAGED OBJECT CLASS

DERIVED FROM   "Recommendation G.774": msTTPSink ;

- CHARACTERIZED BY

- "Recommendation M.3100": crossConnectionPointerPackage,

msTcTTPSinkPkg  PACKAGE

- BEHAVIOUR msTcTTPSinkBeh ;;;

- REGISTERED AS {g774-8ObjectClass 10 } ;

msTcTTPSinkBeh BEHAVIOUR

- DEFINED AS

"The msTcTTPSink object class is a class of objects that represents the protected resources in a tandem connection made up of one or more link connections at multiplex section layer. An instance of this object class defines the tandem connection end-point which terminates a multiplex section trail. An instance of this object class is pointed to by a reliableResourcePointer attribute in a instance of the sdhRadioProtectionUnit object class.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer attribute pointing back to the instance of this object class.

The upStreamConnectivityPointer in an instance of this object class points to either null or its associated msTcCTP object instance representing the unprotected tandem connection. It indicates the actual signal flow and when a signal is switched to another unit, the pointer is updated.

If the attributeValueChangeNotification package is present, then a change in the value of supportedByObjectList shall cause an attributeValueChangeNotification." ;

msTcTTPSource    MANAGED OBJECT CLASS

- DERIVED FROM  "Recommendation G.774": msTTPSource ;

- CHARACTERIZED BY

- "Recommendation M.3100": crossConnectionPointerPackage,

msTcTTPSourcePkg  PACKAGE

- BEHAVIOUR msTcTTPSourceBeh ;;;

- REGISTERED AS {g774-8ObjectClass 11 } ;

msTcTTPSourceBeh BEHAVIOUR

- DEFINED AS

"The msTcTTPSource object class is a class of objects that represents the protected resources in a tandem connection made up of one or more link connections at multiplex section layer. An instance of this object class defines the tandem connection end-point which originates a multiplex section trail. An instance of this object class is pointed to by a reliableResourcePointer attribute in a instance of the sdhRadioProtectionUnit object class.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer attribute pointing back to the instance of this object class.

The downStreamConnectivityPointer in an instance of this object class points to either null or its associated msTcCTP object instance(s) representing the unprotected tandem connection(s). It indicates the actual signal flow and when a signal is switched to another unit, the pointer is updated.

If the attributeValueChangeNotification package is present, then a change in the value of supportedByObjectList shall cause an attributeValueChangeNotification." ;

### 6.1.3	High order path connection protection object definitions

This clause provides the object classes required to model the High Order Path Connection Protection.

#### 6.1.3.1	High order path connection CTP

au4HopcCTPBidirectional    MANAGED OBJECT CLASS

DERIVED FROM   "Recommendation G.774":  au4CTPBidirectionalR1,

au4HopcCTPSource,

au4HopcCTPSink ;

- CHARACTERIZED BY

au4HopcCTPBidirectionalPkg  PACKAGE

- BEHAVIOUR au4HopcCTPBidirectionalBeh ;;;

- REGISTERED AS {g774-8ObjectClass 12 } ;

au4HopcCTPBidirectionalBeh BEHAVIOUR

- DEFINED AS

"The au4HopcCTPBidirectional object class is a class of objects that represents either the bidirectional protected resources or the bidirectional unprotected resources in a High Order Path Connection protected by an RPS function.

If a bidirectional SDH Radio Protection Switching function, acting as high order path connection protection, is present, this object class shall be supported.";

au4HopcCTPSink    MANAGED OBJECT CLASS

DERIVED FROM   "Recommendation G.774": au4CTPSinkR1 ;

- CHARACTERIZED BY

- "Recommendation M.3100" : crossConnectionPointerPackage,

au4HopcCTPSinkPkg  PACKAGE

- BEHAVIOUR au4HopcCTPSinkBeh ;;;

- REGISTERED AS {g774-8ObjectClass 13 } ;

au4HopcCTPSinkBeh BEHAVIOUR

- DEFINED AS

"The au4HopcCTPSink object class is a class of objects that represents either the protected or the unprotected resources in a High Order Path Connection protected by an RPS function. An instance of this object class defines the path connection end-point which terminates a High Order Path Connection.

An instance of this object class is pointed to by a reliableResourcePointer attribute or by an unreliableResourcePointer attribute in an instance of the sdhRadioProtectionUnit object class according if it represents a protected or an unprotected path connection respectively.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer or unreliableResourcePointer attribute pointing back to the instance of this object class.

If an instance of this object class represents a protected path connection, then the downStreamConnectivityPointer points to either null or to its associated au4HopcCTP object instance(s) representing the unprotected path connection(s).

If an instance of this object class represents an unprotected path connection, then the downStreamConnectivityPointer points to either null or to its associated au4HopcCTP or to the vc4HopcTTP object instance representing the protected path connection.

When a signal is switched to another unit, the value of the downStreamConnectivityPointer is updated." ;

au4HopcCTPSource    MANAGED OBJECT CLASS

DERIVED FROM   "Recommendation G.774": au4CTPSource ;

- CHARACTERIZED BY

- "Recommendation M.3100": crossConnectionPointerPackage,

au4HopcCTPSourcePkg  PACKAGE

- BEHAVIOUR au4HopcCTPSourceBeh ;;;

- REGISTERED AS {g774-8ObjectClass 14 } ;

au4HopcCTPSourceBeh BEHAVIOUR

- DEFINED AS

"The au4HopcCTPSource object class is a class of objects that represents either the protected resources or the unprotected resources in a High Order Path Connection protected by an RPS function. An instance of this object class defines the path connection end-point which originates a High Order Path Connection.

An instance of this object class is pointed to by a reliableResourcePointer attribute or by an unreliableResourcePointer attribute in an instance of the sdhRadioProtectionUnit object class according if it represents a protected or an unprotected path connection respectively.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer or unreliableResourcePointer attribute pointing back to the instance of this object class.

If an instance of this object class represents a protected path connection then the upStreamConnectivityPointer points to either null or to its associated au4HopcCTP object instance representing the unprotected path connection.

If an instance of this object class represents an unprotected path connection, then the upStreamConnectivityPointer points to either null or to its associated au4HopcCTP or to the vc4HopcTTP object instance representing the protected path connection.

When a signal is switched to another unit, the value of the upStreamConnectivityPointer is updated." ;

#### 6.1.3.2	Protected high order path connection TTP

vc4HopcTTPBidirectional    MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation G.774": vc4TTPBidirectionalR1,

vc4HopcTTPSource,

vc4HopcTTPSink  ;

- CHARACTERIZED BY

vc4HopcTTPBidirectionalPkg  PACKAGE

- BEHAVIOUR vc4HopcTTPBidirectionalBeh ;;;

- REGISTERED AS {g774-8ObjectClass 15 } ;

vc4HopcTTPBidirectionalBeh BEHAVIOUR

- DEFINED AS

"The vc4HopcTTPBidirectional object class is a class of objects that represents the bidirectional protected resources in a High Order Path Connection protected by an RPS function.

If a bidirectional SDH Radio Protection Switching function, acting as high order path connection protection, is present, this object class shall be supported." ;

vc4HopcTTPSink    MANAGED OBJECT CLASS

DERIVED FROM   "Recommendation G.774": vc4TTPSinkR1 ;

- CHARACTERIZED BY

- "Recommendation M.3100": crossConnectionPointerPackage,

vc4HopcTTPSinkPkg  PACKAGE

- BEHAVIOUR vc4HopcTTPSinkBeh ;;;

- REGISTERED AS {g774-8ObjectClass 16 } ;

vc4HopcTTPSinkBeh BEHAVIOUR

- DEFINED AS

"The vc4HopcTTPSink object class is a class of objects that represents the protected resources in a High Order Path Connection protected by an RPS function. An instance of this object class defines the path end-point which terminates a High Order trail. An instance of this object class is pointed to by a reliableResourcePointer attribute in a instance of the sdhRadioProtectionUnit object class.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer attribute pointing back to the instance of this object class.

The upStreamConnectivityPointer in an instance of this object class points to either null or to its associated au4HopcCTP object instance representing the unprotected path connection. It indicates the actual signal flow and, when a signal is switched to another unit, it is updated.

If the attributeValueChangeNotification package is present, then a change in the value of supportedByObjectList shall cause an attributeValueChangeNotification." ;

vc4HopcTTPSource    MANAGED OBJECT CLASS

- DERIVED FROM  "Recommendation G.774": vc4TTPSourceR1 ;

- CHARACTERIZED BY

- "Recommendation M.3100": crossConnectionPointerPackage,

vc4HopcTTPSourcePkg  PACKAGE

- BEHAVIOUR vc4HopcTTPSourceBeh ;;;

- REGISTERED AS {g774-8ObjectClass 17 } ;

vc4HopcTTPSourceBeh BEHAVIOUR

- DEFINED AS

"The vc4HopcTTPSource object class is a class of objects that represents the protected resources in a High Order Path Connection protected by an RPS function. An instance of this object class defines the path end-point which originates a high order trail. An instance of this object class is pointed to by a reliableResourcePointer attribute in a instance of the sdhRadioProtectionUnit object class.

The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer attribute pointing back to the instance of this object class.

The downStreamConnectivityPointer in an instance of this object class points to either null or to its associated au4HopcCTP object instance(s) representing the unprotected tandem connection(s). It indicates the actual signal flow and when a signal is switched to another unit, it is updated.

If the attributeValueChangeNotification package is present, then a change in the value of supportedByObjectList shall cause an attributeValueChangeNotification." ;

### 6.1.4	Multiplex section trail protection (by means of RPS) object definitions

#### 6.1.4.1	Radio unprotected CTP

radioUnprotectedCTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM	"Recommendation M.3100":

connectionTerminationPointBidirectional,

radioUnprotectedCTPSource,

radioUnprotectedCTPSink;

- CHARACTERIZED BY

radioUnprotectedCTPBidirectionalPkg PACKAGE

- BEHAVIOUR radioUnprotectedCTPBidirectionalBeh;;;

- REGISTERED AS {g774-8ObjectClass 18 };

radioUnprotectedCTPBidirectionalBeh BEHAVIOUR

- DEFINED AS

"The radioUnprotectedCTPBidirectional object class is a class of objects that represents the bidirectional unprotected resources in a protection scheme involving RPS function.";

radioUnprotectedCTPSink MANAGED OBJECT CLASS

DERIVED FROM 	"Recommendation M.3100":connectionTerminationPointSink;

- CHARACTERIZED BY

- "Recommendation M.3100":crossConnectionPointerPackage,

radioUnprotectedCTPSinkPkg PACKAGE

- BEHAVIOUR radioUnprotectedCTPSinkBeh;

- ATTRIBUTES

radioUnprotectedCTPId    GET;;;

- REGISTERED AS {g774-8ObjectClass 19 };

radioUnprotectedCTPSinkBeh BEHAVIOUR

- DEFINED AS

"The radioUnprotectedCTPSink object class is a class of objects that represents the unprotected resources in a protection scheme involving RPS function. An instance of this object class is pointed to by the unreliableResourcePointer attribute in an instance of the sdhRadioProtectionUnit object class. The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit object instance which has the unreliableResourcePointer attribute pointing back to the instance of this object class.";

radioUnprotectedCTPSource MANAGED OBJECT CLASS

DERIVED FROM     "Recommendation M.3100":connectionTerminationPointSink;

- CHARACTERIZED BY

- "Recommendation M.3100":crossConnectionPointerPackage,

radioUnprotectedCTPSourcePkg PACKAGE

- BEHAVIOUR radioUnprotectedCTPSourceBeh;

- ATTRIBUTES

radioUnprotectedCTPId    GET;;;

- REGISTERED AS {g774-8ObjectClass 20 };

radioUnprotectedCTPSourceBeh BEHAVIOUR

- DEFINED AS

"The radioUnprotectedCTPSource object class is a class of objects that represents the unprotected resources in a protection scheme involving RPS function. An instance of this object class is pointed to by the unreliableResourcePointer attribute in an instance of the sdhRadioProtectionUnit object class. The crossConnectionObjectPointer attribute in an instance of this object class points to its associated

sdhRadioProtectionUnit object instance which has the reliableResourcePointer attribute pointing back to the instance of this object class.";

#### 6.1.4.2	Radio protected TTP

radioProtectedTTPBidirectional MANAGED OBJECT CLASS

DERIVED FROM    "Recommendation M.3100":trailTerminationPointBidirectional,

radioProtectedTTPSource,

radioProtectedTTPSink;

- CHARACTERIZED BY

radioProtectedTTPBidirectionalPkg PACKAGE

- BEHAVIOUR radioProtectedTTPBidirectionalBeh;;;

- REGISTERED AS {g774-8ObjectClass 21};

radioProtectedTTPBidirectionalBeh BEHAVIOUR

- DEFINED AS

"The radioProtectedTTPBidirectional object class is a class of objects that represents the bidirectional protected resources in a protection scheme involving RPS function.";

radioProtectedTTPSink MANAGED OBJECT CLASS

DERIVED FROM     "Recommendation M.3100":trailTerminationPointSink;

- CHARACTERIZED BY

- "Recommendation M.3100":crossConnectionPointerPackage,

radioProtectedTTPSinkPkg PACKAGE

- BEHAVIOUR radioProtectedTTPSinkBeh;

- ATTRIBUTES

radioProtectedTTPId	GET;;;

- REGISTERED AS {g774-8ObjectClass 22};

radioProtectedTTPSinkBeh BEHAVIOUR

- DEFINED AS

"The radioProtectedTTPSink object class is a class of objects that represents the protected resources in a protection scheme involving RPS function. An instance of this object class is pointed to by the reliableResourcePointer attribute in an instance of the sdhRadioProtectionUnit object class. The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit instance which has the reliableResourcePointer attribute pointing back to the instance of this object class. The upstreamConnectivityPointer in an instance of this object class points to either null or its associated unprotected CTP object instance; it indicates the actual signal flow, and when a signal is switched to another unit, the value of the upstreamConnectivityPointer is updated. If the attributeValueChangeNotification package is present, then a change in the value of the supportedByObjectList shall cause an attributeValueChange notification.";

radioProtectedTTPSource MANAGED OBJECT CLASS

DERIVED FROM     "Recommendation M.3100":trailTerminationPointSource;

- CHARACTERIZED BY

- "Recommendation M.3100":crossConnectionPointerPackage,

radioProtectedTTPSourcePkg PACKAGE

- BEHAVIOUR radioProtectedTTPSourceBeh;

- ATTRIBUTES

radioProtectedTTPId          GET;;;

- REGISTERED AS {g774-8ObjectClass 23};

radioProtectedTTPSourceBeh BEHAVIOUR

- DEFINED AS

"The radioProtectedTTPSource object class is a class of objects that represents the protected resources in a protection scheme involving RPS function. An instance of this object class is pointed to by the reliableResourcePointer attribute in an instance of the sdhRadioProtectionUnit object class. The crossConnectionObjectPointer attribute in an instance of this object class points to its associated sdhRadioProtectionUnit object instance which has the reliableResourcePointer attribute pointing back to the instance of this object class. The downstreamConnectivityPointer in an instance of this object class points to either null or its associated unprotected CTP object instance(s); it indicates the actual signal flow, and when a signal is switched to another unit, the pointer is updated. If the attributeValueChangeNotification package is present, then a change in the value of the supportedByObjectList shall cause an attributeValueChange notification.";

## 6.2	Packages definitions

exerciseOnOffPkg PACKAGE

- BEHAVIOUR exerciseOnOffPkgBeh;

- ATTRIBUTES

exerciseOn	GET-REPLACE;

- REGISTERED AS {g774-8Package 7};

exerciseOnOffPkgBeh BEHAVIOUR

- DEFINED AS "This package is used to start or stop a continous testing of the RPS functionality.";

singleExercisePkg PACKAGE

- BEHAVIOUR singleExercisePkgBeh;

- ACTIONS

invokeRadioExercise;

- REGISTERED AS {g774-8Package 8};

singleExercisePkgBeh BEHAVIOUR

- DEFINED AS "This package is used to perform a single test operation of the RPS functionality.";

privilegedChannelPkg PACKAGE

- ATTRIBUTES

privilegedChannel		GET-REPLACE;

- REGISTERED AS {g774-8Package 9};

radioHoldOffTimePkg PACKAGE

- ATTRIBUTES

radioHoldOffTime		GET-REPLACE;

- REGISTERED AS {g774-8Package 10};

## 6.3	Attributes definitions

### 6.3.1	hitless

hitless  ATTRIBUTE

- WITH ATTRIBUTE SYNTAX SDHRadioProtASN1.Boolean;

- MATCHES FOR EQUALITY ;

- BEHAVIOUR hitlessBehaviour ;

- REGISTERED AS  {g774-8Attribute 5 } ;

hitlessBehaviour BEHAVIOUR

- DEFINED AS

"This attribute specifies whether the hitless capability is present or not. A value of TRUE indicates that the hitless capability is present in the protection system. A value of FALSE indicates that the hitless capability is not present.";

### 6.3.2	radioHoldOffTime

radioHoldOffTime  ATTRIBUTE

- WITH ATTRIBUTE SYNTAX  SDHRadioProtASN1.Integer ;

- MATCHES FOR EQUALITY, ORDERING ;

- BEHAVIOUR  radioHoldOffTimeBeh ;

- REGISTERED AS { g774-8Attribute 6} ;

radioHoldOffTimeBeh   BEHAVIOUR

- DEFINED AS

"This attribute specifies the amount of 10 ms. periods, within a valid range of 0... 10 s., which represents the time to wait before performing a protection switch after detection of an automatic switching initiation defect on either the protecting or protected protectionUnit.

- The switch is performed only if the defect is still present after the holdOffTime has expired.";

### 6.3.3	rpsSummaryStatus

rpsSummaryStatus				ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDHRadioProtASN1.RPSSummaryStatus;

MATCHES FOR				EQUALITY;

BEHAVIOUR					rpsSummaryStatusBeh;

- REGISTERED AS {g774-8Attribute 7 };

rpsSummaryStatusBeh			BEHAVIOUR

- DEFINED AS

- "This single structured attribute represents the whole status of the RPS function. The statusOfRxProtectionSwitches sub-field allows to know the whole status of the bridges in the Rx side. This is achieved by listing all the protecting protection units which are carrying an extra traffic or traffic from a protected unit. If a protecting unit is not listed, it is intended to be free.

The channelStatus sub-field allows to know information about the highest priority automatic switch request currently active on this protection unit instance together with the last operator switch command accepted (if any) for each protection unit. This is achieved by listing all the protection units which have the value of the channelASRequest sub-field different from 'noOne' and the value of radioSwitchStatus sub-field different from noRequest. If a protection unit is not listed, the status shall be considered (noOne, noRequest).";

### 6.3.4	exerciseOn

exerciseOn	ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDHRadioProtASN1.Boolean;

MATCHES FOR		EQUALITY;

BEHAVIOUR	exerciseOnBeh;

- REGISTERED AS {g774-8Attribute 8 };

exerciseOnBeh	BEHAVIOUR

- DEFINED AS

"This attribute is used to start/stop the exercise procedure on a RPS function. If the value is TRUE the procedure is activated, otherwise it is deactivated.";

### 6.3.5	privilegedChannel

privilegedChannel	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	SDHRadioProtASN1.Privileged;

MATCHES FOR EQUALITY;

BEHAVIOUR		privilegedChannelBeh;

- REGISTERED AS {g774-8Attribute 9 };

privilegedChannelBeh	BEHAVIOUR

- DEFINED AS

"This attribute is used to indicate if the protected channel defined in the value of the attribute is permanently bridged in the TX side (in absence of any switching requests) to this protecting channel. A NULL value indicates that there is no bridge active.";

### 6.3.6	radioProtectionStatus

radioProtectionStatus ATTRIBUTE

- WITH ATTRIBUTE SYNTAX  SDHRadioProtASN1.RadioProtectionStatus;

- MATCHES FOR EQUALITY;

- BEHAVIOUR		radioProtectionStatusBeh;

- REGISTERED AS {g774-8Attribute 10 } ;

radioProtectionStatusBeh  BEHAVIOUR

- DEFINED AS

"This attribute is used to indicate the status of the radio protection switching in a protectionUnit instance.

The protectionUnitStatus sub-field contains information about the highest priority automatic switch request currently active on this protection unit instance together with the last operator switch command accepted (if any).

The associatedChannel sub-field indicates with the value 'itself' that there is no switch performed. In case of switch presence, the fromPU value is used for a protecting unit to indicate the protected unit which has been switched from; the toPU value is used for a protected unit to indicate the protecting unit which has been switched to.

The requestSource sub-field, when present, indicates if the switch request has been 	forwarded locally or remotely.";

### 6.3.7	radioUnprotectedCTPId

radioUnprotectedCTPId ATTRIBUTE

- WITH ATTRIBUTE SYNTAX SDHRadioProtASN1.NameType;

- MATCHES FOR EQUALITY;

- BEHAVIOUR

radioUnprotectedCTPIdBehaviour BEHAVIOUR

- DEFINED AS

- "This attribute is used as a RDN for naming instances of the radioUnprotectedCTP object classes."

- ;;

- REGISTERED AS { g774-8Attribute  11 };

### 6.3.8	radioProtectedTTPId

radioProtectedTTPId ATTRIBUTE

- WITH ATTRIBUTE SYNTAX SDHRadioProtASN1.NameType;

- MATCHES FOR EQUALITY;

- BEHAVIOUR

radioProtectedTTPIdBehaviour BEHAVIOUR

- DEFINED AS

- "This attribute is used as a RDN for naming instances of the radioProtectedTTP object classes."

;;

- REGISTERED AS { g774-8Attribute  12 };

## 6.4	Actions definitions

invokeRadioExercise   ACTION

- BEHAVIOUR  invokeRadioExerciseBeh ;

- MODE CONFIRMED ;

- WITH INFORMATION SYNTAX SDHRadioProtASN1.InvokeRadioExerciseArg ;

- WITH REPLY SYNTAX  SDHRadioProtASN1.InvokeRadioExerciseReply ;

- REGISTERED AS { g774-8Action 1} ;

invokeRadioExerciseBeh   BEHAVIOUR

- DEFINED AS

"The invokeRadioExercise action can be used to request a protection exercise routine to be performed on one or more protectionUnit instances contained in the protectionGroup object.

The action argument contains indications of the protected and protecting protectionUnits to which the request applies.

If a protecting protectionUnit is identified in the protectedUnit field or if a protected protectionUnit is identified in the protectingUnit field, the action fails.

The protectionEntity field may be absent, indicating that the request applies to all contained protectionUnits.

A single exercise consists in initiating a switching process without actually switching and, therefore, involves one protected and one protecting protectionUnit.

For an exercised protected protectionUnit, the exercise result contains the list of each protecting protectionUnit to which the switching process has been applied together with the respective obtained result.

For an exercised protecting protectionUnit, the exercise result contains the list of each protected protectionUnit to which the switching process has been applied together with the respective obtained result.

While an exercise is in progress the value of the radioProtectionStatus attribute for both the involved protected and protecting units shall indicate No Request, on the contrary the exercise result will indicate a denied value.";

## 6.5	Parameters definitions

### 6.5.1	Radio protection status parameter

radioProtectionStatusParameter  PARAMETER

- CONTEXT EVENT-INFO;

- WITH SYNTAX SDHRadioProtASN1.RadioProtectionStatusParameter;

- BEHAVIOUR radioProtectionStatusParameterBeh;

- REGISTERED AS  {g774-8Parameter 1 } ;

radioProtectionStatusParameterBeh BEHAVIOUR

- DEFINED AS

"This parameter is included in the additional info parameters of the protection switching reporting notification.

The protectionSwitchReporting notification is emitted from the sdhRadioProtectionGroup object only in the following cases:

a)	when a failed protection switch request (an automatic request that cannot be satisfied) occurs and the severity of the alarm causing the request is highBER or signalFail.

b)	as a consequence of invoked or released switch requests sent by operator commands and successfully accepted.

c)	when a hardware forcing is performed or released locally on the NE.

d)	when a previously invoked manual switch is released by an automatic switch request.";

## 6.6	Name bindings definitions

### 6.6.1	augSink

augSink-msTcTTPSink NAME BINDING

- SUBORDINATE OBJECT CLASS "Recommendation G.774":augSink AND SUBCLASSES;

- NAMED BY

- SUPERIOR OBJECT CLASS msTcTTPSink AND SUBCLASSES;

- WITH ATTRIBUTE "Recommendation G.774":augId ;

- BEHAVIOUR augSink-msTcTTPSinkBeh;

- REGISTERED AS  {g774-8NameBinding 5 } ;

augSink-msTcTTPSinkBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is 	present.";

### 6.6.2	augSource

augSource-msTcTTPSource NAME BINDING

- SUBORDINATE OBJECT CLASS "Recommendation G.774":augSource  AND SUBCLASSES;

- NAMED BY

- SUPERIOR OBJECT CLASS msTcTTPSource  AND SUBCLASSES;

- WITH ATTRIBUTE "Recommendation G.774": augId ;

- BEHAVIOUR augSource-msTcTTPSourceBeh;

- REGISTERED AS  {g774-8NameBinding 6 } ;

augSource-msTcTTPSourceBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present.";

### 6.6.3	msTcCTPSink

msTcCTPSink-rsTTPSink NAME BINDING

- SUBORDINATE OBJECT CLASS msTcCTPSink  AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774":rsTTPSink AND 	SUBCLASSES;

WITH ATTRIBUTE "Recommendation G.774": msCTPId ;

BEHAVIOUR msTcCTPSink-rsTTPSinkBeh;

- REGISTERED AS  {g774-8NameBinding 7 } ;

msTcCTPSink-rsTTPSinkBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

### 6.6.4	msTcCTP Source

msTcCTPSource-rsTTPSource NAME BINDING

- SUBORDINATE OBJECT CLASS msTcCTPSource AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774": rsTTPSource AND 	SUBCLASSES;

WITH ATTRIBUTE "Recommendation G.774": msCTPId ;

BEHAVIOUR msTcCTPSource-rsTTPSourceBeh;

- REGISTERED AS  {g774-8NameBinding 8 } ;

msTcCTPSource-rsTTPSourceBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

### 6.6.5	msTcTTP Sink

msTcTTPSink-sdhNE NAME BINDING

- SUBORDINATE OBJECT CLASS msTcTTPSink AND SUBCLASSES;

- NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774":sdhNE;

- WITH ATTRIBUTE "Recommendation G.774": msTTPId ;

- BEHAVIOUR msTcTTPSink-sdhNEBeh;

- REGISTERED AS  {g774-8NameBinding 9 } ;

msTcTTPSink-sdhNEBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

### 6.6.6	msTcTTP Source

msTcTTPSource-sdhNE NAME BINDING

- SUBORDINATE OBJECT CLASS msTcTTPSource AND SUBCLASSES;

- NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774":sdhNE;

- WITH ATTRIBUTE "Recommendation G.774":msTTPId ;

- BEHAVIOUR msTcTTPSource-sdhNEBeh;

- REGISTERED AS  {g774-8NameBinding 10 } ;

msTcTTPSource-sdhNEBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

### 6.6.7	vc4HopcTTP Sink

vc4HopcTTPSink-sdhNE NAME BINDING

- SUBORDINATE OBJECT CLASS vc4HopcTTPSink AND SUBCLASSES;

- NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774":sdhNE;

- WITH ATTRIBUTE "Recommendation G.774": vc4TTPId ;

- BEHAVIOUR vc4HopcTTPSink-sdhNEBeh;

- REGISTERED AS  {g774-8NameBinding 11 } ;

vc4HopcTTPSink-sdhNEBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

### 6.6.8	vc4HopcTTP Source

vc4HopcTTPSource-sdhNE NAME BINDING

- SUBORDINATE OBJECT CLASS vc4HopcTTPSource AND SUBCLASSES;

- NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774":sdhNE;

- WITH ATTRIBUTE "Recommendation G.774": vc4TTPId ;

- BEHAVIOUR vc4HopcTTPSource-sdhNEBeh;

- REGISTERED AS  {g774-8NameBinding 12 } ;

vc4HopcTTPSource-sdhNEBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

### 6.6.9	au4HopcCTPSink

au4HopcCTPSink-augSink NAME BINDING

- SUBORDINATE OBJECT CLASS au4HopcCTPSink  AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774":augSink AND 	SUBCLASSES;

WITH ATTRIBUTE "Recommendation G.774": au4CTPId ;

BEHAVIOUR au4HopcCTPSink-augSinkBeh;

- REGISTERED AS  {g774-8NameBinding 13 } ;

au4HopcCTPSink-augSinkBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

### 6.6.10	au4HopcCTP Source

au4HopcCTPSource-augSource NAME BINDING

- SUBORDINATE OBJECT CLASS au4HopcCTPSource AND SUBCLASSES;

NAMED BY SUPERIOR OBJECT CLASS "Recommendation G.774":augSource AND 	SUBCLASSES;

WITH ATTRIBUTE "Recommendation G.774": au4CTPId ;

BEHAVIOUR au4HopcCTPSource-augSourceBeh;

- REGISTERED AS  {g774-8NameBinding 14 } ;

au4HopcCTPSource-augSourceBeh BEHAVIOUR

- DEFINED AS

"The subordinate managed objects are instantiated when the radio protection switching function is present. Instances of this object may also be instantiated when other types of tandem connection protections in multiplex section layer are present.";

## 6.7	Supporting ASN.1

- SDHRadioProtASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127)

sdhRadioIM(8) informationModel(0) asn1Module(2) sdhRadioProtASN1(1)}

- DEFINITIONS IMPLICIT TAGS ::=

- BEGIN

-- EXPORTS everything

- IMPORTS

- RelativeDistinguishedName

- FROM

- InformationFramework {joint-iso-ccitt ds(5) modules(1) informationFramework(1)}

- NameType

- FROM

- ASN1DefinedTypesModule {itu-t(0)

recommendation(0) m(13) gnm(3100) informationModel(0)

asn1Modules(2) asn1DefinedTypesModule(0)}

ProtectionEntity,

- ResourcePointer,

- RequestSource,

- InvokeExerciseArg

FROM SDHProtASN1 { itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) sdhProt(03) 	informationModel(0) asn1Module(2) sdhmsp(0) };

-- supporting productions

- Boolean ::= BOOLEAN

- Integer ::= INTEGER

- InvokeRadioExerciseArg ::= InvokeExerciseArg

- InvokeRadioExerciseReply ::= SET OF SEQUENCE {

exercisedPU		RelativeDistinguishedName,

exerciseResult		SEQUENCE OF SingleExerciseResult}

- SingleExerciseResult ::= SEQUENCE {

protectionUnit		RelativeDistinguishedName,

result			Result }

- Result ::= ENUMERATED {  success (0), denied (1), failed (2) }

- Privileged ::= CHOICE {

noBridge			[0] NULL,

privilegedUnit		[1] RelativeDistinguishedName

}

- RadioProtectionStatusParameter ::= RPSSummaryStatus

- SDHRadioResourcePointer ::= ResourcePointer ( WITH COMPONENTS {..., objectInstances (SIZE(1)) })

RPSSummaryStatus ::= SEQUENCE {

statusOfRxProtectionSwitches		StatusOfRxProtectionSwitches,

channelStatus					ChannelStatus

}

- StatusOfRxProtectionSwitches ::= SET OF SEQUENCE {

protectingSectionId		RelativeDistinguishedName,

protectingSectionStatus		ProtectingSectionStatus

}

- ProtectingSectionStatus ::= CHOICE {

extraTraffic		[0]	NULL,

protectedUnit		[1]	RelativeDistinguishedName

}

ChannelStatus	::=	SET OF SEQUENCE {

protUnitId		RelativeDistinguishedName,

protUnitStatus		ProtUnitStatus

}

- ProtUnitStatus ::= SEQUENCE {

channelASRequest		ChannelASRequest,

radioSwitchStatus		RadioSwitchStatus

}

ChannelASRequest ::= ENUMERATED {noOne(0), waitToRestore(1), earlyWarning(2),

lowBER(3), highBER(4), signalFail(5)}

RadioSwitchStatus  ::= ENUMERATED { noRequest(0), manualSwitch(1),

forcedSwitch(2), lockout(3), hwForcing(4) }

- RadioProtectionStatus ::= SEQUENCE {

protectionUnitStatus	[0] ProtUnitStatus,

associatedChannel		[1] AssociatedChannel,

requestSource			[2] RequestSource OPTIONAL

}

- AssociatedChannel ::= CHOICE {

itself		[0] NULL,

fromPU		[1] RelativeDistinguishedName,

toPU			[2] RelativeDistinguishedName

}

- END

APPENDIX I

Figures

Naming and Inheritance are diagrammatically shown in Figures I.1 to I.8.

Figure I.1/G.774.8 – Radio fragment objects inheritance

Figure I.2/G.774.8  Radio fragment objects naming

- Figure I.3/G.774.8 – Example: 140 Mbit/s signals multiplexed to STM-1 signal

Figure I.4/G.774.8 – SDH radio protection inheritance diagram

- Figure I.5/G.774.8 – SDH radio protection naming diagram

- Figure I.6/G.774.8 – RPS applications for MS-tandem connection protection

Figure I.7/G.774.8 – Example: Radio section  multiplex section 1 + 1
protection – source side

Figure I.8/G.774.8 – Example: Radio section = tandem connection of
regenerator sections 1  1 protection – source side

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |