|  |  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |
| --- | --- | --- | --- | --- |
|  |  | ITU-T | G.774.9 |  |
|  |  | TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (02/2001) |
|  | SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS Digital terminal equipments – Operations, administration and maintenance features of transmission equipment |  |  |  |
|  | Synchronous digital hierarchy (SDH) – Configuration of linear multiplex-section protection for the network element view |  |  |  |
|  | ITU-T Recommendation G.774.9 (Formerly CCITT Recommendation) |  |  |  |

ITU-T G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| --- | --- |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS | G.500–G.599 |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

For further details, please refer to the list of ITU-T Recommendations.

ITU-T Recommendation G.774.9 Synchronous digital hierarchy (SDH) – Configuration of linear multiplex-section protection for the network element view

- Summary This Recommendation provides an information model for the Synchronous Digital Hierarchy (SDH) Network. This model describes the managed object classes and their properties for the configuration of the protection switching function, as defined in ITU-T G.805 and G.774.3 and as related to SDH transmission resources. These objects are useful to describe information exchanged across interfaces defined in M.3010 Telecommunications Management Network (TMN) architecture for the configuration of the protection function. The protection switching scheme of an SDH network element is usually set up autonomously by the network element according to its make-up and mode of operation. When this is not possible, the information model defined by this Recommendation will be used. Post-configuration management of the protection function is described in ITU-T G.774.3.

| Document History |  |
| --- | --- |
| Issue | Notes |
| 2001 | First revision incorporated the changes documented in the G.774 series Implementor Guide. |
| 2/1998 | Initial version of the Recommendation. |

- Source ITU-T Recommendation G.774.9 was revised by ITU-T Study Group 15 (2001-2004) and approved under the WTSA Resolution 1 procedure on 9 February 2001.

FOREWORD

The International Telecommunication Union (ITU) is the United Nations specialized agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of ITU. ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Assembly (WTSA), which meets every four years, establishes the topics for study by the ITU-T study groups which, in turn, produce Recommendations on these topics.

The approval of ITU-T Recommendations is covered by the procedure laid down in WTSA Resolution 1.

In some areas of information technology which fall within ITU-T's purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, ITU had not received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2001

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Terms and definitions		3

4	Abbreviations		3

5	Multiplex-section protection configuration management model		3

5.1	Overview		3

5.2	Requirements		4

5.2.1	Generic protection configuration requirements		4

5.2.2	Specific SDH MS linear protection configuration requirements		4

5.3	Model overview		4

6	Managed object class definitions		5

6.1	Protection coordinator		5

6.2	SDH MS protection coordinator		5

7	Packages		6

8	Attributes		6

8.1	Protection coordinator Id		6

9	Actions		6

9.1	Dismiss protection		6

9.2	Establish protection		6

9.3	Modify protection		7

10	Notifications		7

11	Parameters		7

11.1	MSP configuration error		7

11.2	MSP group configuration parameter		8

11.3	MSP unit configuration parameter		8

11.4	Protection configuration error		8

11.5	Remove protection error		8

12	Name binding definitions		9

12.1	Protection coordinator		9

13	Subordination rules		9

14	Pointer constraints		9

15	Supporting ASN.1 productions		9

Appendix I  Naming and inheritance diagrams		11

ITU-T Recommendation G.774.9

Synchronous digital hierarchy (SDH) – Configuration of linear
multiplex-section protection for the network element view

# 1	Scope

ITU-T G.774.3 defines an information model for management of fixed linear Multiplex Section Protection (MSP) groups. This Recommendation defines the object model that allows to configure flexible linear MSP groups in accordance with the requirements described in ITU-T G.784. This model can be used to establish, modify and remove MSP groups for Network Elements that require external provisioning to configure their protection switching schemes. The protection switching scheme of an SDH network element is usually set up autonomously by the network element according to its make-up and mode of operation. When this is not possible, the information model defined by this Recommendation will be used. Post-configuration control of the protection function is carried out by means of the object model defined in ITU-T G.774.3.

Structure of this Recommendation

Clause 5.1 provides an overview of the SDH protection configuration information model. Clauses 6-15 describe the information model using the notation mechanisms defined in ITU-T X.722 Guidelines for the Definition of Managed Objects. Clause 15 contains the syntax definitions of the information carried in the protocol using Abstract Syntax Notation One (ASN.1) defined in ITU-T X.680-X.683. Naming and Inheritance are illustrated in Appendix I.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision: users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

	ITU-T G.707/Y.1322 (2000), Network node interface for the synchronous digital hierarchy (SDH).

	ITU-T G.773 (1993), Protocol suites for Q-interfaces for management of transmission systems.

	ITU-T G.774 (2001), Synchronous digital hierarchy (SDH) – Management information model for the network element view.

	ITU-T G.783 (2000), Characteristics of synchronous digital hierarchy (SDH) equipment functional blocks.

	ITU-T G.784 (1999), Synchronous digital hierarchy (SDH) management.

	ITU-T G.803 (2000), Architecture of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.831 (2000), Management capabilities of transport networks based on the synchronous digital hierarchy (SDH).

	ITU-T G.958 (1994), Digital line systems based on the synchronous digital hierarchy for use on optical fibre cables.

	ITU-T M.60 (1993), Maintenance terminology and definitions.

	ITU-T M.2120 (2000), PDH path, section and transmission system and SDH path and multiplex section fault detection and localization procedures.

	ITU-T M.3010 (2000), Principles for a telecommunications management network.

	ITU-T M.3013 (2000), Considerations for a telecommunications management network.

	ITU-T M.3100 (1995), Generic network information model.

	ITU-T Q.811 (1997), Lower layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.812 (1997), Upper layer protocol profiles for the Q3 and X interfaces.

	ITU-T Q.822 (1994), Stage 1, Stage 2 and Stage 3 description for the Q3-interface  Performance management.

	ITU-T X.680 to X.683 (1997), Information technology – Abstract Syntax Notation One (ASN.1).

	ITU-T X.701 (1997), Information technology – Open Systems Interconnection – Systems management overview.

	ITU-T X.710 (1997), Information technology – Open Systems Interconnection – Common management information service.

	ITU-T X.711 (1997), Information technology – Open Systems Interconnection – Common management information protocol: Specification.

	ITU-T X.720 (1992), Information technology – Open Systems Interconnection – Structure of Management Information: Management information model, plus Amd.1 (1995) and Cor.1 (1994).

	ITU-T X.721 (1992), Information technology – Open Systems Interconnection – Structure of management information: Definition of management information, plus Cor.1 (1994), Cor.2 (1996), Cor.3 (1998) and Cor.4 (2000).

	ITU-T X.722 (1992), Information technology – Open Systems Interconnection – Structure of management information: Guidelines for the definition of managed objects, plus Amd.1 (1995), Amd.2 (1997) and Cor.1 (1996).

	ITU-T X.730 (1992), Information technology – Open Systems Interconnection – Systems Management: Object management function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.731 (1992), Information technology – Open Systems Interconnection – Systems Management: State management function, plus Amd.1 (1995), Cor.1 (1995) and Amd.1/Cor.1 (1996).

	ITU-T X.733 (1992), Information technology – Open Systems Interconnection – Systems Management: Alarm reporting function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.734 (1992), Information technology – Open Systems Interconnection – Systems Management: Event report management function, plus Cor.1 (1994), Amd.1 (1995), Amd.1/Cor.1 (1996) and Cor.2 (1999).

	ITU-T X.735 (1992), Information technology – Open Systems Interconnection – Systems Management: Log control function, plus Amd.1 (1995) and Amd.1/Cor.1 (1996).

# 3	Terms and definitions

This Recommendation uses the terms and definitions defined in ITU-T G.774, ITU-T G.784 and ITU-T M.3100.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

APDU		Application Protocol Data Unit

APS		Automatic Protection Switching

CMIP		Common Management Information Protocol

CMIS		Common Management Information Service

CTP		Connection Termination Point

ISO		International Organization for Standardization

ITU		International Telecommunication Union

LOS		Loss Of Signal

MS		Multiplex Section

MSP		Multiplex Section Protection

NE		Network Element

OS		Operations System

OSI		Open Systems Interconnection

Pkg		Package

RDN		Relative Distinguished Name

SDH		Synchronous Digital Hierarchy

SF		Signal Fail

STM-N		Synchronous Transport Module N

TMN		Telecommunications Management Network

TP		Termination Point

TTP		Trail Termination Point

WTR		Wait-To-Restore

# 5	Multiplex-section protection configuration management model

## 5.1	Overview

ITU-T G.774.3 defines a generic model for protection schemes and the specific model for SDH MS protection. The model defined in ITU-T G.774.3 allows full operation and control of the MS protection function for Network Elements that set up autonomously their protection switching schemes, according to their make-up and mode of operation.

Some Network Elements, due to equipment flexibility, require external provisioning to determine the configuration of their MS protection schemes, particularly as to the definition of the lines participating in the protection.

This clause provides Managed Objects to support configuration of MS protection groups in SDH Network Elements.

This model is a compatible extension that supports external creation and deletion of SDH MS protection groups and units to the model defined in ITU-T G.774.3.

## 5.2	Requirements

The information model defined in this Recommendation is based on the same approach as used in ITU-T G.774.3 that identifies generic objects to support general protection functional requirements plus specific subclasses for the particular purposes of Multiplex Section Protection.

The functional requirements for configuration management of Multiplex Section Protection groups may be found in ITU-T G.784.

### 5.2.1	Generic protection configuration requirements

The general functional requirements for the configuration of protection schemes are:

–	the ability for a managing system to establish a protection scheme, indicating the protection mode (revertive/non revertive), the entities participating in the protection, their role (protecting/protected), and possibly their priority;

–	the ability for a managing system to modify a protection scheme, adding or removing entities and/or modifying their protection characteristics (role and priority);

–	the ability for a managing system to suppress a protection scheme.

### 5.2.2	Specific SDH MS linear protection configuration requirements

The specific functional requirements for the configuration of SDH MS protection schemes are:

–	the ability for a managing system to specify the type of protection (unidirectional/bidirectional) and to enable or disable the K1, K2 protocol when establishing a protection scheme;1

–	the ability for a managing system to specify the channel number and priority (high/low) associated to each protection line and optionally control extra-traffic on each protection line when establishing or modifying a protection scheme.1

## 5.3	Model overview

The information model specified in this Recommendation defines a generic object (protectionCoordinator) that includes the actions (namely establishProtection, modifyProtection, dismissProtection), necessary to configure a generic protection scheme.

A subclass of protectionCoordinator (sdhMSProtectionCoordinator) is then defined for the specific purposes of SDH MS Protection configuration. Such object class inherits all of the actions and behavior defined in the protectionCoordinator superclass and specifies the additional information needed to establish or modify a protection scheme in the particular case of SDH MSP.

# 6	Managed object class definitions

## 6.1	Protection coordinator

protectionCoordinator MANAGED OBJECT CLASS

DERIVED FROM	 “Recommendation X.721”:top;

CHARACTERIZED BY

protectionCoordinatorPkg PACKAGE

BEHAVIOUR protectionCoordinatorBeh;

ATTRIBUTES

protectionCoordinatorId	GET;

ACTIONS

establishProtection		protectionConfigurationError,

dismissProtection			removeProtectionError,

modifyProtection 			protectionConfigurationError

removeProtectionError;;;

REGISTERED AS {g774-09MObjectClass 1};

protectionCoordinatorBeh BEHAVIOUR

DEFINED AS

*This object class allows the management system to configure the protection schemes of a network element. Instances of this class or of its subclasses are created at system start-up and can never be deleted.*;

## 6.2	SDH MS protection coordinator

sdhMSProtectionCoordinator MANAGED OBJECT CLASS

DERIVED FROM	 	protectionCoordinator;

CHARACTERIZED BY

sdhMSProtectionCoordinatorPkg PACKAGE

BEHAVIOUR sdhMSProtectionCoordinatorBeh;

ACTIONS

establishProtection		mSPGroupConfigurationParameter

mSPUnitConfigurationParameter

mSPConfigurationError,

modifyProtection			mSPUnitConfigurationParameter

mSPConfigurationError;;;

REGISTERED AS {g774-09MObjectClass 2};

sdhMSProtectionCoordinatorBeh BEHAVIOUR

DEFINED AS

*This object class is used specifically to configure SDH MS protection schemes. Only one instance can be created in one NE.

When the establishProtection action is successfully performed one sdhMSProtectionGroup instance and as many sdhMSProtectionUnit instances are created as specified by the action information. The unreliableResourcePointer of each sdhMSProtectionUnit will point to the unprotectedCTP indicated by the unreliableObjects field of the action information. The reliableResourcePointer will point to the protectedTTP connected to the unprotectedCTP, unless the protectionUnit is protecting and extra-traffic is not required or not supported. In this case the reliableResourcePointer is NULL.

UnprotectedCTPs and protectedTTPs are anyhow instantiated for sections which can potentially be included in a protection group irrespective whether protection is actually present or not. When a section is not part of a protection group, the crossConnectionObjectPointer of the unprotectedCTP and protectedTTP point to the sdhMSProtectionCoordinator object instance. When protection is established the crossConnectionObjectPointer points to the associated sdhMSprotectionUnit. When it is possible, unprotectedCTPs and protectedTTPs may be created/deleted as a result of the establish/modify/dismiss actions.*;

# 7	Packages

None.

# 8	Attributes

## 8.1	Protection coordinator Id

protectionCoordinatorId			ATTRIBUTE

WITH ATTRIBUTE SYNTAX		SDHProtCoordASN1.NameType;

MATCHES FOR		EQUALITY;

BEHAVIOUR		protectionCoordinatorIdBeh;

REGISTERED AS {g774-09Attribute 1};

protectionCoordinatorIdBeh BEHAVIOUR

DEFINED AS

*The protectionCoordinatorId attribute is an attribute type whose distinguished value can be used as an RDN when naming an instance of the protectionCoordinator object class.*;

# 9	Actions

## 9.1	Dismiss protection

dismissProtection			ACTION

BEHAVIOUR	dismissProtectionBeh;

MODE CONFIRMED;

WITH INFORMATION SYNTAX		SDHProtCoordASN1.DismissProtectionInfo;

REGISTERED AS {g774-09Action 1};

dismissProtectionBeh BEHAVIOUR DEFINED AS

*This action is used to dismiss a protection scheme. The protectionGroup object instance indicated in the action argument and all the contained protectionUnits are deleted. After the execution of this action all the resources previously participating in the protection scheme will operate in an unprotected mode. Yet, protectedTTPs and unprotectedCTPs will remain to allow for future participation in other protection groups.

The action fails if manual or forced switch commands are active on any of the protection units.

Automatic switches present before the deletion are implicitly released as a side effect. No notification is sent to report the release of automatic switches.*;

## 9.2	Establish protection

establishProtection			ACTION

BEHAVIOUR	establishProtectionBeh;

MODE CONFIRMED;

WITH INFORMATION SYNTAX		SDHProtCoordASN1.EstablishProtectionInfo;

REGISTERED AS {g774-09Action 2};

establishProtectionBeh BEHAVIOUR DEFINED AS

*This action is used to create a protection scheme within a network element. When applied, one protectionGroup object instance is created.

The protectionUnits field indicates the protection resources which will participate in the protection scheme and their role (protected/protecting). One protectionUnit instance is created for each element of the protectionUnits field. The unreliableResourcePointer, the protecting and the priority attributes of each protectionUnit are initialized with the value provided by the ProtectionResource sequence (priority optional). The priority must be assigned either to all protected (or protecting) units or to none. At least one protected and one protecting protection units must be provided by the managing system. The specificPUConfiguration field allows to

define additional protection unit attributes for specific subclasses of the protectionUnit class. The specific parameter to be used is defined in protectionCoordinator subclasses.

The optional protectionGroupType field is used to discriminate whether the protection type is 1+1 (plus) or M:N (colon). The protectionGroupType attribute of the protectionGroup object instance will be set accordingly. This field can be present only when the managing system indicated one protected and one protecting resource; if, in this case, it is absent the NE will set the protectionGroupType attribute according to its own capabilities.

The optional revertiveInformation field allows the managing system to ask for a revertive/non-revertive protection scheme and, only if revertive is set to TRUE, to assign the wait-to-restore time. If the revertiveInformation is not present the NE will set the revertive and waitToRestoreTime attributes according to its own capabilities.

The specificPGConfiguration field allows to define additional protection group attributes for specific subclasses of the protectionGroup class. The specific parameter to be used is defined in protectionCoordinator subclasses.*;

## 9.3	Modify protection

modifyProtection			ACTION

BEHAVIOUR	modifyProtectionBeh;

MODE CONFIRMED;

WITH INFORMATION SYNTAX		SDHProtCoordASN1.ModifyProtectionInfo;

REGISTERED AS {g774-09Action 3};

modifyProtectionBeh BEHAVIOUR DEFINED AS

*This action is used to add and/or remove one or more resources from a protectionGroup. Action requests that would reduce to zero the number of protected or protecting units will be rejected.

If the protectionGroupType was 1+1 (plus) and one or more protection units are added, it automatically switches to M:N (colon).

The action fails if manual or forced switch commands are active on any of the protection units under removal.

Automatic switches present before the deletion of a protectionUnit are implicitly released as a side effect.*;

# 10	Notifications

None.

# 11	Parameters

## 11.1	MSP configuration error

mSPConfigurationError		PARAMETER

CONTEXT SPECIFIC-ERROR;

WITH SYNTAX	SDHProtCoordASN1.MSPConfigurationError;

BEHAVIOUR	mSPConfigurationErrorBeh;

REGISTERED AS {g774-09Parameter 1};

mSPConfigurationErrorBeh	BEHAVIOUR DEFINED AS

*This parameter is included in the error parameter of the CMIP APDU when the establishProtection or modifyProtection action received by the sdhMSPProtectionCoordinator fails for specific MSP configuration inconsistencies.*;

## 11.2	MSP group configuration parameter

mSPGroupConfigurationParameter	PARAMETER

CONTEXT ACTION-INFO;

WITH SYNTAX	SDHProtCoordASN1.MSPGroupConfigurationParameter;

BEHAVIOUR		mSPGroupConfigurationParameterBeh;

REGISTERED AS {g774-09Parameter 2};

mSPGroupConfigurationParameterBeh	BEHAVIOUR DEFINED AS

*This parameter is used in the specificPGConfiguration field of the establishProtection action received by the sdhMSPProtectionCoordinator, when the establishment of a sdhMSProtectionGroup is requested by the management system.

The protectionSwitchMode field indicates whether protection switching is done on a unidirectional or bidirectional basis.

The aPSProtocolPresent boolean field indicates whether the K1/K2 protocol is used. The protectionMismatchStatusPkg conditional package of the sdhMSProtectionGroup object is instantiated if aPSProtocolPresent value is TRUE.*;

## 11.3	MSP unit configuration parameter

mSPUnitConfigurationParameter		PARAMETER

CONTEXT ACTION-INFO;

WITH SYNTAX	SDHProtCoordASN1.MSPUnitConfigurationParameter;

BEHAVIOUR	mSPUnitConfigurationParameterBeh;

REGISTERED AS {g774-09Parameter 3};

mSPUnitConfigurationParameterBeh	BEHAVIOUR DEFINED AS

*This parameter is used in the specificPUConfiguration field of the establishProtection or modifyProtection action received by the sdhMSPProtectionCoordinator, when the establishment or modification of a sdhMSProtectionGroup is requested by the management system.

The channelNumber field indicates the channel number associated with the protection unit.

The sdhPriority field indicated the priority (High/Low) of the protected protection unit. For protecting protection units this field is not present.

If the extraTrafficControl field is present the extraTrafficControlPkg is instantiated in the protectionUnit object instance. The administrativeState is initialized with the value provided by this field. This field can only be present if the aPSProtocolPresent value is set to true and if the protection unit is protecting.*;

## 11.4	Protection configuration error

protectionConfigurationError		PARAMETER

CONTEXT SPECIFIC-ERROR;

WITH SYNTAX	SDHProtCoordASN1.ProtectionConfigurationError;

BEHAVIOUR	protectionConfigurationErrorBeh;

REGISTERED AS {g774-09Parameter 4};

protectionConfigurationErrorBeh		BEHAVIOUR DEFINED AS

*This parameter is included in the error parameter of the CMIP APDU when the establishProtection or modifyProtection action received by a subclass of the protectionCoordinator fails for generic configuration inconsistencies.*;

## 11.5	Remove protection error

removeProtectionError		PARAMETER

CONTEXT SPECIFIC-ERROR;

WITH SYNTAX	SDHProtCoordASN1.RemoveProtectionError;

BEHAVIOUR	removeProtectionErrorBeh;

REGISTERED AS {g774-09Parameter 5};

removeProtectionErrorBeh		BEHAVIOUR DEFINED AS

*This parameter is included in the error parameter of the CMIP APDU when the dismissProtection or modifyProtection action received by a subclass of the protectionCoordinator fails.

The operatorCommandPresent result is returned when a manual or forced switch is present in the protection group. It shall indicate the involved protection units.*;

# 12	Name binding definitions

## 12.1	Protection coordinator

protectionCoordinator-sdhNE			NAME BINDING

SUBORDINATE OBJECT CLASS	protectionCoordinator AND SUBCLASSES;

NAMED BY

SUPERIOR OBJECT CLASS		“Recommendation G.774”:sdhNE AND SUBCLASSES;

WITH ATTRIBUTE		protectionCoordinatorId;

BEHAVIOUR			protectionCoordinator-sdhNEBeh;

REGISTERED AS {g774-09NameBinding 1};

protectionCoordinator-sdhNEBeh	BEHAVIOUR DEFINED AS

*The subordinate managed object is automatically instantiated when the superior managed object is instantiated, according to the make-up and mode of operation of the equipment.*;

# 13	Subordination rules

None.

# 14	Pointer constraints

None.

# 15	Supporting ASN.1 productions

SDHProtCoordASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) protCoord(9) informationModel(0) asn1Module(2) sdhmspcoord(0)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- EXPORTS everything

IMPORTS

AdditionalInformation, AdministrativeState FROM Attribute-ASN1Module {joint-iso-ccitt ms(9) smi(3) part2(2) asn1Module(2) 1}

ObjectInstance FROM CMIP-1 {joint-iso-ccitt ms(9) cmip(1) modules(0) protocol(3)}

NameType FROM ASN1DefinedTypesModule {itu-t(0) recommendation(0) m(13) gnm(3100) informationModel(0) asn1Modules(2) asn1DefinedTypesModule(0)}

ProtectionGroupType, ProtectionSwitchMode, ResourcePointer, SDHMSPriority FROM SDHProtASN1 {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) prot(03) informationModel(0) asn1Module(2) sdhmsp(0)};

sdhProtCoord OBJECT IDENTIFIER ::= {itu-t(0) recommendation(0) g(7) g774(774) hyphen(127) protCoord(9) informationModel(0)}

g774-09MObjectClass OBJECT IDENTIFIER ::= {sdhProtCoord managedObjectClass(3)}

g774-09NameBinding OBJECT IDENTIFIER ::= {sdhProtCoord nameBinding(6)}

g774-09Attribute OBJECT IDENTIFIER ::= {sdhProtCoord attribute(7)}

g774-09Action OBJECT IDENTIFIER ::= {sdhProtCoord action(9)}

g774-09Parameter OBJECT IDENTIFIER ::= {sdhProtCoord parameter(5)}

AddedProtectionUnits ::= ProtectionUnits

DismissedProtectionGroup ::= ObjectInstance

DismissProtectionInfo ::= DismissedProtectionGroup

EstablishProtectionInfo ::= SEQUENCE {

protectionUnits		ProtectionUnits,

protectionGroupType	ProtectionGroupType OPTIONAL,

revertiveInformation	RevertiveInformation OPTIONAL,

specificPGConfiguration	AdditionalInformation OPTIONAL

}

IncompatibleWithNEConfig ::= CHOICE {

alreadyProtected			[0] ObjectInstance,

otherIncompatibility	[1] NULL

}

InconsistentData ::= ENUMERATED {

exceedingProtectingUnits				(0),

exceedingUnitsFor1Plus1					(1),

duplicateUnreliable					(2),

nonRevertiveIncompatibleWith1ToN		(3),

incompletePriorityAssignment			(4),

otherError							(5)

}

ModifiedProtectionGroup ::= ObjectInstance

ModifyProtectionInfo ::= SEQUENCE {

modifiedProtectionGroup	ModifiedProtectionGroup,

addedProtectionUnits	[0] AddedProtectionUnits OPTIONAL,

removedProtectionUnits	[1] RemovedProtectionUnits OPTIONAL

}

MSPConfigurationError ::= ENUMERATED {

protectionSwitchModeNotSupported		(0),

aPSProtocolNotSupported			(1),

invalidChannelNumber				(2),

notSupportedSDHPriority			(3),

extraTrafficControlNotSupported		(4),

otherError						(5)

}

MSPGroupConfigurationParameter ::= SEQUENCE {

protectionSwitchMode		ProtectionSwitchMode,

aPSProtocolPresent		BOOLEAN

}

MSPUnitConfigurationParameter ::= SEQUENCE {

channelNumber			INTEGER,

sdhPriority			SDHMSPriority OPTIONAL,

extraTrafficControl	AdministrativeState OPTIONAL

}

OperatorCommandPresent ::= SET OF ResourcePointer

ProtectionConfigurationError ::= CHOICE {

inconsistentData				[0] InconsistentData,

unsupportedProtConfiguration		[1] UnsupportedProtConfiguration,

incompatibleWithNEConfig		[2] IncompatibleWithNEConfig

}

ProtectionResource ::= SEQUENCE	{

unreliableObjects			ResourcePointer,

protecting				BOOLEAN,

priority					INTEGER OPTIONAL,

specificPUConfiguration		AdditionalInformation OPTIONAL

}

ProtectionUnits ::= SET OF ProtectionResource

RemoveProtectionError ::= OperatorCommandPresent

RemovedProtectionUnits ::= SET OF ObjectInstance

RevertiveInformation ::= SEQUENCE {

revertive				BOOLEAN,

waitToRestoreTime		INTEGER OPTIONAL

}

UnsupportedProtConfiguration ::= ENUMERATED {

notSupportedProtConfig			(0),

notSupportedPGType				(1),

notSupportedRevertiveMode	(2),

invalidWTRTime					(3),

invalidPriority				(4),

otherError					(5)

}

END -- end of SDHProtCoordASN1

Appendix I

Naming and inheritance diagrams

The naming and inheritance trees cover only the managed object classes of this Recommendation. See Figures I.1 and I.2.

Figure I.1/G.774.9 – SDH MS configuration inheritance diagram

Figure I.2/G.774.9 – SDH MS configuration naming diagram

| SERIES OF ITU-T RECOMMENDATIONS |  |
| --- | --- |
| Series A | Organization of the work of ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Cable networks and transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |

These parameters can be configured when establishing the protection scheme. Subsequent modifications may be achieved by using ITU-T G.774.3.