### G776.1 Actions

### Activate LED Test

activateLEDTest ACTION

BEHAVIOUR activateLEDTestBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.ActivateLEDTestInformation;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.ActivateLEDTestResult;

REGISTERED AS {g776.1Action 1};

activateLEDTestBehaviour BEHAVIOUR

DEFINED AS

“This action initiates the LED test. For some equipment the LEDs will extinguish after a fixed interval. For other equipment, it is shut off by the Deactivate LED Test action.”;

### Alarm Cut Off

alarmCutOff ACTION

BEHAVIOUR alarmCutOffBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.G776.1Response

REGISTERED AS {g776.1Action 2};

alarmCutOffBehaviour BEHAVIOUR

DEFINED AS

“Used to silence the audible alarm.”;

### Busiest Period Report Request

busiestPeriodReportRequest ACTION

BEHAVIOUR busiestPeriodReportRequestBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.BusiestPeriodReportRequestResult;

REGISTERED AS {g776.1Action 3};

busiestPeriodReportRequestBehaviour BEHAVIOUR

DEFINED AS

“Used to request a report from the terminal on-demand. The report is calculated upon requst, and sent to the MS.”;

### Cancel Map Change Enable

cancelMapChangeEnable ACTION

BEHAVIOUR cancelMapChangeEnableBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.G776.1Response

REGISTERED AS {g776.1Action 4};

cancelMapChangeEnableBehaviour BEHAVIOUR

DEFINED AS

“Used to cancel the Map Change Enable action.”;

### CC Error Performance ACC Report Request

ccErrorPerformanceACCReportRequest ACTION

BEHAVIOUR ccErrorPerformanceACCReportRequestBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.CCErrorPerformanceACCReportRequestResult;

REGISTERED AS {g776.1Action 5};

ccErrorPerformanceACCReportRequestBehaviour BEHAVIOUR

DEFINED AS

“Used to request a report from the terminal on-demand. The report is calculated upon request, and sent to the MS.”;

### Deactivate LED Test

deactivateLEDTest ACTION

BEHAVIOUR deactivateLEDTestBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.DeactivateLEDTestInformation;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.DeactivateLEDTestResult;

REGISTERED AS {g776.1Action 6};

deactivateLEDTestBehaviour BEHAVIOUR

DEFINED AS

“This action deactivates the LED test initiated by the Activate LED Test for those equipment that do not extinquish the LEDs after a fixed period.”;

### Download Background Map

downloadMap ACTION

BEHAVIOUR downloadMapBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.G776.1Response;

REGISTERED AS {g776.1Action 7};

downloadMapBehaviour BEHAVIOUR

DEFINED AS

“This action causes a download of the background map stored in the Local Management Station to the DCME.”;

### FAX ACC Report Request

faxACCReportRequest ACTION

BEHAVIOUR faxACCReportRequestBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.FAXACCReportRequestResult;

REGISTERED AS {g776.1Action 8};

faxACCReportRequestBehaviour BEHAVIOUR

DEFINED AS

“Used to request a report from the terminal on-demand. The report is calculated upon request, and sent to the MS.”;

### Function Define

functionDefine ACTION

BEHAVIOUR functionDefineBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.FileNameId;

WITH REPLY SYNTAX SSPSDefinedTypesModule.FunctionData;

REGISTERED AS { g776.1Action 9};

functionDefineBehaviour BEHAVIOUR

DEFINED AS

“This action is used to install a SSPS Function definition template. This action makes the Function definition known to SSPS by adding the definition to the library of Function definitions.”;

### Function List Query

functionListQuery ACTION

BEHAVIOUR functionListQueryBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.FileNameId;

WITH REPLY SYNTAX SSPSDefinedTypesModule.FunctionListQueryResult;

REGISTERED AS { g776.1Action 10};

functionListQueryBehaviour BEHAVIOUR

DEFINED AS

“This action is used to present a list of all Device Functions that are current defined in the SSPS. These are Functions that have been installed with the FunctionDefine request. Those Functions that are removed will not appear in this list.”;

### Function Replace

functionReplace ACTION

BEHAVIOUR functionReplaceBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.FileNameId;

WITH REPLY SYNTAX SSPSDefinedTypesModule.FunctionData;

REGISTERED AS { g776.1Action 11};

functionReplaceBehaviour BEHAVIOUR

DEFINED AS

“This action is used to replace an existing Device Function definition. This will replace the Function definition in the Library only. Services and Resource Pools that currently have the chosen Function allocated are not affected. New requests to the Resource Pools to add the specified Function will use the new definition after this command is executed. In order to remove the old version of the Function, commands need to be used that instruct the Resource manager to remove the specific instances that correspond to the older Function Definition.”;

### Install Service Channel

installServiceChannel  ACTION

BEHAVIOUR rootServiceInvokeBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

SSPSDefinedTypesModule.InstallServiceChannelInformation;

WITH REPLY SYNTAX SSPSDefinedTypesModule.ServiceChannelInstallAsnInfo;

REGISTERED AS { g776.1Action 12 };

rootServiceInvokeBehaviour BEHAVIOUR

DEFINED AS

“Install Service Channel or Channel Service Invoke is used to bring an instance of a Service into existence at the Service Channel level of the hierarchy. The Service should have been previously defined and be a part of the Service Library. When the Service is started it is a new entity that can be treated as an independent task. The new Service must conform to the Service model. It must retrieve the necessary components from the Resource manager and make the appropriate connections within its container. Once the new Service is invoked as a Service Channel, the SSPS Root Service will then respond to commands requesting connections to the Service Channel. The Service Channel will be invoked with Relative Priority set to High”;

### Map Change

mapChange ACTION

BEHAVIOUR mapChangeBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.G776.1Response;

REGISTERED AS {g776.1Action 13};

mapChangeBehaviour BEHAVIOUR

DEFINED AS

“Used to switch between the foreground and the background maps of a terminal.”;

### Map Change Enable

mapChangeEnable ACTION

BEHAVIOUR mapChangeEnableBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.G776.1Response;

REGISTERED AS {g776.1Action 14};

mapChangeEnableBehaviour BEHAVIOUR

DEFINED AS

“A change of a foreground configuration map must be preceded with several preparations that must be made by the terminal. The Map Change Enable action is a request for the terminal to commit these actions, in order to enable the replacement of the foreground map.”;

### Remove Service Channel

removeServiceChannel  ACTION

BEHAVIOUR removeServiceChannelBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

SSPSDefinedTypesModule.RemoveServiceChannelInformation;

WITH REPLY SYNTAX SSPSDefinedTypesModule.GenericResponse;

REGISTERED AS { g776.1Action 15 };

removeServiceChannelBehaviour BEHAVIOUR

DEFINED AS

“Remove Service Channel request is used to remove an instance of a Service that has been started by an InstallServiceChannel command.  All internal connections for that Service are automatically removed.  Nothing about that Service instance or its internal states are preserved. The unlink parameter is used to specify if the external connections to the Service are to be removed automatically.”;

### Reset H Register

resetHregister  ACTION

BEHAVIOUR resetHregisterBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX g776.1DefinedTypesModule.G776.1Reply;

REGISTERED AS { g776.1Action16 };

resetHregisterBehaviour BEHAVIOUR

DEFINED AS

“This action is used to reset the echo canceller’s Hregister.”;

### Reset Terminal

resetTerminal ACTION

BEHAVIOUR resetTerminalBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.G776.1Response;

REGISTERED AS {g776.1Action 17};

resetTerminalBehaviour BEHAVIOUR

DEFINED AS

“Used to reset the terminal.”;

### Retrieve Service File

retrieveServiceFile  ACTION

BEHAVIOUR retrieveServiceBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX SSPSDefinedTypesModule.FileNameId;

REGISTERED AS { g776.1Action 18 };

retrieveServiceBehaviour BEHAVIOUR

DEFINED AS

“Retrieve the service monitor file from SSPS. It allows the OS to view the Service Monitor. The Service Monitor is used as a controlled port to the outside world. Services may send messages to the Service Monitor. System error messages are also sent to the Service Monitor; they are stored in the Service Monitor according to the Service that generated them. The messages and data that are sent to the Service Monitor may be viewed by the OS interface by using this command. Once the data for a particular Service is transferred to the OS, it is no longer kept in the Service Monitor. The data area for the Service Monitor, once filled, acts like a circular buffer and overwrites the oldest data with the newest. The size of this data area is fixed and is not under control of the outside interfaces. Services send messages to the Service Monitor using the normal SendMessage API command. The OS interface may retrieve Service Monitor Data using ServiceMonitorQuery.”;

### Retrieve Service Tree

retrieveServiceTree  ACTION

BEHAVIOUR serviceTreeRetrieveBehaviour;

MODE CONFIRMED;

WITH REPLY SYNTAX SSPSDefinedTypesModule.RetrieveServiceTreeResult;

REGISTERED AS { g776.1Action 19 };

serviceTreeRetrieveBehaviour BEHAVIOUR

DEFINED AS

“Retrieve all nodes under a service channel. The SSPS Platform will represent the current Service hierarchy upon request. The Service name and unique system address is presented along with the relationship in the hierarchy. This feature may be used to give a snapshot of the current Services in the SPM as well as to provide the ID's that would be necessary to allow interaction with those Services.”;

### Run EC Self Test Out of Service

runECSelfTestOutofService ACTION

BEHAVIOUR runECSelfTestOutofServiceBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.RunECSelfTestOutofServiceInformation;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.RunECSelfTestOutofServiceResult;

REGISTERED AS {g776.1Action 20};

runECSelfTestOutofServiceBehaviour BEHAVIOUR

DEFINED AS

“This action intitiates an our-of-service self-test on one or more 64 kb/s timeslots of the echo canceller equipment.”;

### Run Soft Reset

runSoftReset ACTION

BEHAVIOUR runSoftResetBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.RunSoftResetInformation;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.RunSoftResetResult;

REGISTERED AS {g776.1Action 21};

runSoftResetBehaviour BEHAVIOUR

DEFINED AS

“This action initiates a soft reset of the SPNE similiar to that which would occur during a power cycle. Configuration parameters would not be affected.”;

### Send Generic Service Message

sendGenericServiceMessage  ACTION

BEHAVIOUR sendGenericServiceMessageBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

SSPSDefinedTypesModule.GenericServiceMessage;

WITH REPLY SYNTAX SSPSDefinedTypesModule.GenericResponse;

REGISTERED AS { g776.1Action 22 };

sendGenericServiceMessageBehaviour BEHAVIOUR

DEFINED AS

“This action may be made by OS to send message involving any Service that exist in the managed system.”;

### Service Define

serviceDefine ACTION

BEHAVIOUR serviceDefineBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.FileNameId;

WITH REPLY SYNTAX SSPSDefinedTypesModule.ServiceData;

REGISTERED AS { g776.1Action 23};

serviceDefineBehaviour BEHAVIOUR

DEFINED AS

“This action is used to install a Service definition template in SSPS. This action makes the Service definition known to SSPS by adding the definition to the library of Service definitions and does not start up any Services.”;

### Service List Query

serviceListQuery ACTION

BEHAVIOUR serviceListQueryBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.FileNameId;

WITH REPLY SYNTAX SSPSDefinedTypesModule.ServiceListQueryResult;

REGISTERED AS { g776.1Action 24};

serviceListQueryBehaviour BEHAVIOUR

DEFINED AS

“This action is used to presents a list of all Services that are current defined in the SSPS. These are Services that have been installed with the ServiceDefine request. Those Services that are removed will not appear in this list.”;

### Service Pool Lock

servicePoolLock	 ACTION

BEHAVIOUR servicePoolLockBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.PoolLockInformation;

WITH REPLY SYNTAX SSPSDefinedTypesModule.GenericResponse;

REGISTERED AS { g776.1Action 25 };

servicePoolLockBehaviour BEHAVIOUR

DEFINED AS

“Pool Lock puts a hold on resources for the specified pool. This allows SSPS Services to continue using the existing allocation, but once they are returned, the function specified can not be assigned to a new request from a Service. An autonomous message will be sent to the OS interface when the Locked Pool becomes totally unallocated. This would be a signal to the OS that the specified Pools may be removed without impacting Services.”;

### Service Pool Size Decrease

servicePoolSizeDecrease  ACTION

BEHAVIOUR servicePoolSizeDecreaseBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.PoolSizeInformation;

WITH REPLY SYNTAX SSPSDefinedTypesModule.GenericResponse;

REGISTERED AS { g776.1Action 26 };

servicePoolSizeDecreaseBehaviour BEHAVIOUR

DEFINED AS

“It allows the OS interface to request modifications to the resource pools.  The modification may alter the size of an existing Pool. The Device Function identified must be a valid entry that has been defined in SSPS. The distribution is an optional parameter that tells SSPS exactly which slots the Function are to be located.”;

### Service Pool Size Increase

servicePoolSizeIncrease  ACTION

BEHAVIOUR servicePoolSizeIncreaseBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.PoolSizeInformation;

WITH REPLY SYNTAX SSPSDefinedTypesModule.GenericResponse;

REGISTERED AS { g776.1Action 27 };

servicePoolSizeIncreaseBehaviour BEHAVIOUR

DEFINED AS

“It allows the OS interface to request modifications to the resource pools.  The modification may alter the size of an existing Pool. The Device Function identified must be a valid entry that has been defined in SSPS. The distribution is an optional parameter that tells SSPS exactly which slots the Function are to be located.”;

### Service Pool Status Query

servicePoolStatusQuery  ACTION

BEHAVIOUR servicePoolQueryBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.ServicePoolQuery;

WITH REPLY SYNTAX SSPSDefinedTypesModule.PoolQueryResult;

REGISTERED AS { g776.1Action 28 };

servicePoolQueryBehaviour BEHAVIOUR

DEFINED AS

“Pool Query retrieves the status of a dynamic pool which is currently configured in SSPS. The request is for a single pool. The returned data gives the size of the current pool allocations as well as current usage.”;

### Service Pool Threshold Warning Get

servicePoolThresholdWarningGet ACTION

BEHAVIOUR servicePoolThresholdGetBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

SSPSDefinedTypesModule.PoolThresholdGetInfo;

WITH REPLY SYNTAX SSPSDefinedTypesModule.PoolThresholdGetResult;

REGISTERED AS { g776.1Action 29 };

servicePoolThresholdGetBehaviour BEHAVIOUR

DEFINED AS

“Retrieve the status of dynamic pool which is currently configured in SSPS. The returned data gives the size of the current pool allocations as well as current usage.”;

### Service Pool Threshold Warning Set

servicePoolThresholdWarningSet ACTION

BEHAVIOUR servicePoolThresholdSetBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.PoolThresholdInfo;

WITH REPLY SYNTAX SSPSDefinedTypesModule.GenericResponse;

REGISTERED AS { g776.1Action 30 };

servicePoolThresholdSetBehaviour BEHAVIOUR

DEFINED AS

“Service Pool Threshold is set to the service pool size  for multiple pools. The returned data gives the size of the current pool allocations as well as current usage.”;

### Service Pool Unlock

servicePoolUnlock  ACTION

BEHAVIOUR servicePoolUnlockBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX SSPSDefinedTypesModule.PoolLockInformation;

WITH REPLY SYNTAX SSPSDefinedTypesModule.GenericResponse;

REGISTERED AS { g776.1Action 31 };

servicePoolUnlockBehaviour BEHAVIOUR

DEFINED AS

“It release any locks on resources for the specified pool.”;

### Terminal In Configuration

terminalInConfiguration ACTION

BEHAVIOUR terminalInConfigurationBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.TerminalInConfigurationInformation;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.TerminalInConfigurationResult;

REGISTERED AS {g776.1Action 32};

terminalInConfigurationBehaviour BEHAVIOUR

DEFINED AS

“Used to set the terminal in configuration.”;

### Terminal Out Of Configuration

terminalOutOfConfiguration ACTION

BEHAVIOUR terminalOutOfConfigurationBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.TerminalOutOfConfigurationInformation;

WITH REPLY SYNTAX

G776.1ASN1DefinedTypesModule.TerminalOutOfConfigurationResult;

REGISTERED AS {g776.1Action 33};

terminalOutOfConfigurationBehaviour BEHAVIOUR

DEFINED AS

“Used to set the terminal out of configuration.”;

### Time Update

timeUpdate ACTION

BEHAVIOUR timeUpdateBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.TimeUpdateInformation;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.TimeUpdateResult;

REGISTERED AS {g776.1Action 34};

timeUpdateBehaviour BEHAVIOUR

DEFINED AS

“Used to synchronize the clock of the terminal and the Management Station (MS).”;

### Upload Map

uploadMap ACTION

BEHAVIOUR uploadMapBehaviour;

MODE CONFIRMED;

WITH INFORMATION SYNTAX

G776.1ASN1DefinedTypesModule.UploadMapInformation;

WITH REPLY SYNTAX G776.1ASN1DefinedTypesModule.UploadMapResult;

REGISTERED AS {g776.1Action 35};

uploadMapBehaviour BEHAVIOUR

DEFINED AS

“Used for uploading a map from the configuration map repository of the terminal to the configuration map repository of the MS.”;