### ADPCM 2bit Overload

adpcm2bitOverload	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR adpcm2bitOverloadBehaviour;

REGISTERED AS {g776.1Attribute 189};

adpcm2bitOverloadBehaviour BEHAVIOUR

DEFINED AS

" This parameter defines the capability of the clique to handle speech overload conditions using instantaneous 2bit ADPCM coding for speech.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is: See comment

Comment: For G.763(1991)=Intelsat Rev2 DCME mode the parameter is fixed at disabled. For G.763(1995)=Intelsat Rev3 the default option setting is enabled. Default Value: If DCME Type=G.763 1991 then disabled, If DCME Type=G.763 1995 then enabled.

Reference: G.763”;

### Alarm Extension Mode

alarmExtensionMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 			 			     		G776.1ASN1DefinedTypesModule.AlarmExtensionMode;

MATCHES FOR EQUALITY;

BEHAVIOUR	alarmExtensionModeBehaviour;

REGISTERED AS {g776.1Attribute 190};

alarmExtensionModeBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines how alarms detected on an incoming receive bearer will be extended to the related primary group channels. When the per channel option is chosen, the alarms will be extended towards the switch only to the trunk channels related to that destination. When the per PRI option is chosen, alarms will be extended on all primary groups which carry channels related to the alarmed destination (AIS on the whole PRI).

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Per Channel, Per PRI

The default value is:

Comment:

Reference: G.763”;

### Application Software ID

appSoftwareId ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR appSoftwareIdBehaviour;

REGISTERED AS {g776.1Attribute 415};

appSoftwareIdBehaviour BEHAVIOUR

DEFINED AS

"The appSoftwareId is an attribute whose distinguished value can be used as a RDN when naming an instance of the appSoftware object class.”;

### BC ID

bcID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.BCID;

MATCHES FOR EQUALITY;

BEHAVIOUR	bcIDBehaviour;

REGISTERED AS {g776.1Attribute 193};

bcIDBehaviour BEHAVIOUR

DEFINED AS

" This parameter identifies a bearer channel (BC).  A bearer channel (BC) is a unidirectional digital transmission path, on a Bearer, used to carry compressed traffic between corresponding DCME. A BC may have an instantaneous bit rate of either: 64,40,32,24,16 kbit/s. The parameter designates a BC in the range of 1 to 61 (normal range) and 64 to 124 (overload range). Any integer percentage can be real with higher resolution.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

1 to 61, 64 to 124

The default value is:

Comment:

Reference: G.763 paragraph 5.9”;

### Bearer Backward Alarm

bearerBackwardAlarm	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	bearerBackwardAlarmBehaviour;

REGISTERED AS {g776.1Attribute 194};

BEHAVIOUR	bearerBackwardAlarmBehaviour

DEFINED AS

“This parameter defines whether an indication of a receive bearer alarm will be sent towards the corresponding DCME through the DCME control channel.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment:

Reference: G.763”;

### CAS Protocol

casProtocol	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.CASProtocol;

MATCHES FOR EQUALITY;

BEHAVIOUR	casProtocolBehaviour;

REGISTERED AS {g776.1Attribute 18};

casProtocolBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines which basic Channel Associated Signalling (CAS) protocol is used on a 2048 kbit/s primary group basis. The protocols defined are: USM (User Signalling Module), Transition Signallng R1 or  R2D, DCMS to ISC Signalling (Q.50) and Circuit Supervision  Signalling (Q.33). In DCME CAS is handled by the User Signalling Module (USM). Q.50 and Q.33 only apply to 2048 kbit/s primary groups.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

CAS, Q50, Q33, Idle

The default value is:

Comment:

Reference: G.763, Q.33, Q.50”;

### CC Average BER

ccAverageBER	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.CCAverageBER;

MATCHES FOR EQUALITY;

BEHAVIOUR	ccAverageBERBehaviour;

REGISTERED AS {g776.1Attribute 195};

ccAverageBERBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the average bit error rate in the receive control channel during the most recent STI. This value is measured to 2 significant digits in the mantissa and to a negative integer in the exponent (e.g., 1.2 x 10-5).

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 0.99

The default value is:

Comment:

Reference: G.763 paragraph 15.2.3”;

### CC BER Ratio

ccBERRatio	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	ccBERRatioBehaviour;

REGISTERED AS {g776.1Attribute 196};

BEHAVIOUR	ccBERRatioBehaviour

DEFINED AS

“This parameter provides the percentage of time that the average bit error rate in the received control channel (averaged over 1 minute intervals) exceeds 1 x 10-3  during the most recent STI.  The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment:

Reference: G.763 paragraph 15.2.3”;

### Channel Check Procedure

channelCheckProcedure	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	channelCheckProcedureBehaviour;

REGISTERED AS {g776.1Attribute 197};

channelCheckProcedureBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines, on a per clique basis, whether channel check procedure is performed.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment:

Reference: G.763 paragraph 10”;

### Clique ID

cliqueID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.CliqueID;

MATCHES FOR EQUALITY;

BEHAVIOUR	cliqueIDBehaviour;

REGISTERED AS {g776.1Attribute 198};

cliqueIDBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies a set or group of BCs (bearer channels) which are independent in operation and control from other BCs. In the multi-clique network operational mode, the bearer is divided into two  independent cliques, each is associated with a separate set of trunk channels. The parameter designates a reference either to the first clique=1 or the second clique=2. A clique is sometimes referred to as a pool. Any integer percentage can be real with higher resolution.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

1, 2

The default value is:

Comment:

Reference: G.763”;

### Clock ID

clockID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ClockID;

MATCHES FOR EQUALITY;

BEHAVIOUR	clockIDBehaviour;

REGISTERED AS {g776.1Attribute 199};

clockIDBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies a synchronization clock in the DCME.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Enumerated Option values of

Trunk In, Trunk Out, Bearer Out, Bearer In

The default value is:

Comment:  "Out" in trunk out and bearer out refers to the direction from the equipment to the transmission line. "In" in bearer  in refers to the direction from the transmission line to the equipment.

Reference: G.763 paragraph 13”;

### Clock Priority Number

clockPriorityNumber	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ClockPriorityNumber;

MATCHES FOR EQUALITY;

BEHAVIOUR	clockPriorityNumberBehaviour;

REGISTERED AS {g776.1Attribute 200};

clockPriorityNumberBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the number of backup clocks. For example, if two PRI's can be designated (one as primary synchronization source and one as backup) then Clock Priority Number is 2. This parameter is by manufacturer. PRI = primary rate interface, Tx = transmit, Rx = receive.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

1-17

The default value is:

Comment:

Reference: “;

### Clock Source

clockSource	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.;

MATCHES FOR EQUALITY;

BEHAVIOUR	clockSourceBehaviour;

REGISTERED AS {g776.1Attribute 201};

clockSourceBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines which synchronization source is appropriate for use in this transmission terminal.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Trunk_PRI_1,..., Tx_Trunk_PRI_12, Rx_Bearer_PRI_1, ..., Rx_Bearer_PRI_4,  External_PRI_1, External_PRI_2, External_64kbps_1, External_64kbps_2, Internal_Clock

The default value is:

Comment: An External_PRI is a primary rate that is neither a bearer nor a trunk primary rate and is used only for synchronization. This parameter is critical to proper synchronization of the terminal and unless properly chosen can lead to service degradation. PRI = primary rate interface, Tx = transmit, Rx = receive.

Reference: G.763 paragraph 5.7”;

### Clock Source Group

clockSourceGroup	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ClockSourceGroup;

MATCHES FOR EQUALITY;

BEHAVIOUR	clockSourceGroupBehaviour;

REGISTERED AS {g776.1Attribute 403};

clockSourceGroupBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines which synchronization source group is appropriate for use in this DCME (or transmission terminal). This capability is only provided in certain SPNE (e.g., between DX-3000F DCME's and between DX-5000 DCME's).

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

TrunkIn, BearerIn, External, Internal

The default value is:

Comment:

Reference:”;

### Clock Source Group Priority ID

clockSourceGroupPriorityID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ClockSourceGroupPriorityID;

MATCHES FOR EQUALITY;

BEHAVIOUR	clockSourceGroupPriorityIDBehaviour;

REGISTERED AS {g776.1Attribute 404};

clockSourceGroupPriorityIDBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies the priority for synchronization clock source group selection in DCME. This capability is only provided in certain SPNE (e.g., between DX-3000F DCME's and between DX-5000 DCME's).

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

1-4

The default value is:

Comment:

Reference:”;

### Clock Source Group Priority ID In Clock Source Group

clockSourceGroupPriorityIDInClockSourceGroup	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.

MATCHES FOR EQUALITY;

BEHAVIOUR	clockSourceGroupPriorityIDInClockSourceGroupBehaviour;

REGISTERED AS {g776.1Attribute 405};

clockSourceGroupPriorityIDInClockSourceGroupBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies the priority in synchronization clock source selection within each Clock Source Group in DCME. This capability is only provided in certain SPNE (e.g., between DX-3000F DCME's and between DX-5000 DCME's).

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

For Trunk_In: 1-12, for Bearer_In: 1-4, for External_Clock: 1-2, for Internal_Clock: 1

The default value is:

Comment:

Reference: “;

### Clock Source Priority ID

clockSourcePriorityID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ClockSourcePriorityID;

MATCHES FOR EQUALITY;

BEHAVIOUR	clockSourcePriorityIDBehaviour;

REGISTERED AS {g776.1Attribute 202};

clockSourcePriorityIDBehaviour BEHAVIOUR

DEFINED AS

“The parameter identifies the main choice or the protection choice (secondary) of the associated clock source.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Enumerated Option values of

Primary, Secondary

The default value is:

Comment:

Reference:”;

### Cmif Generator Program

cmifGeneratorProgram	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.CmifGeneratorProgram;

MATCHES FOR EQUALITY;

BEHAVIOUR	cmifGeneratorProgramBehaviour;

REGISTERED AS {g776.1Attribute 203};

cmifGeneratorProgramBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines which specific generator program is being used.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has String values of

Any string up to 80 characters

The default value is:

Comment:

Reference:”;

### Cmif Syntax

cmifSyntax	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.CmifSyntax;

MATCHES FOR EQUALITY;

BEHAVIOUR	cmifSyntaxBehaviour;

REGISTERED AS {g776.1Attribute 204};

cmifSyntaxBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines which specific syntax is being used.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

SPNE_Map

The default value is:

Comment: Currently only the SPNE configuration map syntax is supported.

Reference:”;

### Cmif Version

cmifVersion	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.CmifVersion;

MATCHES FOR EQUALITY;

BEHAVIOUR	cmifVersionBehaviour;

REGISTERED AS {g776.1Attribute 205};

cmifVersionBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the version of the Cmif document.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has String values of

Any string up to 16 characters

The default value is:

Comment:

Reference:”;

### Com Port 1 Baud Rate

comPort1BaudRate	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.BaudRate;

MATCHES FOR EQUALITY;

BEHAVIOUR	comPort1BaudRateBehaviour;

REGISTERED AS {g776.1Attribute 380};

comPort1BaudRateBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the baud rate of communications port 1.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

300bps, 600bps, 1200bps, 2400bps, 4800bps, 9600bps, 19200bps, 38400bps

The default value is:

Comment:

Reference:”;

### Com Port 1 Type

comPort1Type	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PortType;

MATCHES FOR EQUALITY;

BEHAVIOUR	comPort1TypeBehaviour;

REGISTERED AS {g776.1Attribute 382};

comPort1TypeBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether communications port 1 of the SPNE is either asynchronous or synchronous.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Async, Sync

The default value is:

Comment:

Reference:”;

### Com Port 2 Baud Rate

comPort2BaudRate	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.BaudRate;

MATCHES FOR EQUALITY;

BEHAVIOUR	comPort2BaudRateBehaviour;

REGISTERED AS {g776.1Attribute 381};

comPort2BaudRateBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the baud rate of communications port 2.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

300bps, 600bps, 1200bps, 2400bps, 4800bps, 9600bps, 19200bps, 38400bps

The default value is:

Comment:

Reference:”;

### Com Port 2 Type

comPort2Type	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PortType;

MATCHES FOR EQUALITY;

BEHAVIOUR	comPort2TypeBehaviour;

REGISTERED AS {g776.1Attribute 383};

comPort2TypeBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether communications port 2 of the SPNE is either asynchronous or synchronous.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Async, Sync

The default value is:

Comment:

Reference:”;

### Connected Ratio 64kbps

connectedRatio64kbps	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	connectedRatio64kbpsBehaviour;

REGISTERED AS {g776.1Attribute 206};

connectedRatio64kbpsBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the percentage of trunk channels which were 64 kbit/s during the most recent STI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 15.2.3”;

### Controller Reset State

controllerResetState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ControllerResetState;

MATCHES FOR EQUALITY;

BEHAVIOUR	controllerResetStateBehaviour;

REGISTERED AS {g776.1Attribute 317};

controllerResetStateBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the state of the controllers for the packs.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Normal, Reset

The default value is:

Comment:

Reference:”;

### Current Clock Source

currentClockSrc ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.CurrentClockSrc;

MATCHES FOR EQUALITY;

BEHAVIOUR currentClockSrcBehaviour;

REGISTERED AS {g776.1Attribute 427};

currentClockSrcBehaviour BEHAVIOUR

DEFINED AS

“The parameter identifies the current clock source”;

### Data Activity Ratio

dataActivityRatio	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	dataActivityRatioBehaviour;

REGISTERED AS {g776.1Attribute 209};

dataActivityRatioBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the percentage of trunk channels which were active for voice-band data during the most recent STI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 15.2.3”;

### Data Activity Threshold

dataActivityThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DataActivityThreshold;

MATCHES FOR EQUALITY;

BEHAVIOUR	dataActivityThresholdBehaviour;

REGISTERED AS {g776.1Attribute 210};

dataActivityThresholdBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the threshold level, in dBm0, of the activity detector for trunk channels which are in a VBD state. (i.e., data activity).

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

-40 to -25, adaptive

The default value is: -35

Comment: One manufacturer has this value user setable.

Reference:”;

### Data Hangover First

dataHangoverFirst	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DataHangoverFirst;

MATCHES FOR EQUALITY;

BEHAVIOUR	dataHangoverFirstBehaviour;

REGISTERED AS {g776.1Attribute 211};

dataHangoverFirstBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the hangover  time, in seconds, during which the trunk channel will stay in a VBD state if no activity has been detected, after the first VBD activity of the call has been detected.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Real values of

0-25.5 in steps of .1 sec

The default value is: 10

Comment: The integer value represents seconds. This relates to First_Data_ Hangover in G.763. Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 12.1.2”;

### Data Hangover Second

dataHangoverSecond	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DataHangoverSecond;

MATCHES FOR EQUALITY;

BEHAVIOUR	dataHangoverSecondBehaviour;

REGISTERED AS {g776.1Attribute 212};

dataHangoverSecondBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the hangover  time, in milliseconds, during which the trunk channel will stay in a VBD state if no activity is detected, after the first data hangover has expired.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0 to 255

The default value is: 100

Comment: The integer value represents milliseconds. This relates to Second_Data_Hangover in G.763. Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 12.1.2”;

### Data Queue Freezeout Fraction

dataQueueFreezeoutFraction	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DataQueueFreezeoutFraction;

MATCHES FOR EQUALITY;

BEHAVIOUR	dataQueueFreezeoutFractionBehaviour;

REGISTERED AS {g776.1Attribute 216};

dataQueueFreezeoutFractionBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the percentage of active data trunk channels which were not connected during the most recent STI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100 in steps of .001

The default value is:

Comment: This integer value represents percent (e.g., 50.123 represents 50.123%).

Reference: G.763 paragraph 15.2.3”;

### Date Compiled

dateCompiled ATTRIBUTE

WITH ATTRIBUTE SYNTAX SSPSDefinedTypesModule.DateInstalled;

MATCHES FOR EQUALITY , ORDERING;

BEHAVIOUR dateCompiledBehaviour;

REGISTERED AS { g776.1Attribute 416};

dateCompiledBehaviour BEHAVIOUR

DEFINED AS

“date which template have been compiled”;

### Date Defined By User

dateDefinedByUser ATTRIBUTE

WITH ATTRIBUTE SYNTAX SSPSDefinedTypesModule.DateInstalled;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR dateDefinedByUserBehaviour;

REGISTERED AS { g776.1Attribute 417};

dateDefinedByUserBehaviour BEHAVIOUR

DEFINED AS

“date which devicefunction or service function defined by user thru the OS request.";

### Date Installed

dateInstalled ATTRIBUTE

WITH ATTRIBUTE SYNTAX SSPSDefinedTypesModule.DateInstalled;

MATCHES FOR EQUALITY, ORDERING;

BEHAVIOUR dateInstalledBehaviour;

REGISTERED AS { g776.1Attribute 418};

dateInstalledBehaviour BEHAVIOUR

DEFINED AS

“installation date of the template files ";

### DCME Mode

dcmeMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DCMEMode;

MATCHES FOR EQUALITY;

BEHAVIOUR	dcmeModeBehaviour;

REGISTERED AS {g776.1Attribute 30};

dcmeModeBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines a set of capabilities that are associated with a clique in order to assure compatibility with corespondent DCMEs clique operating in the same mode.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

G 763,  Rev2, Celtic_3G, DTX_240, DX3000, DX-3000F

The default value is:

Comment: G.763 is ITU 1995 recommendation. Rev2 is INTELSAT IESS-501 Revision 2. The others are proprietary model DCME's.

Reference: G.763”;

### Destination ID

destinationID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DestinationID;

MATCHES FOR EQUALITY;

BEHAVIOUR	destinationIDBehaviour;

REGISTERED AS {g776.1Attribute 31};

destinationIDBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies the destination. There is a one-to-one relationship between a particular destination number and its name.

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

SPNE_Type_ID= EC, SPNE_Type_ID=DCME: 1-4

The default value is:

Comment: There is a one-to-one relationship between a particular destination number and its name. Any integer percentage can be real with higher resolution.

Reference:”;

### Destination Name

destinationName	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DestinationName;

MATCHES FOR EQUALITY;

BEHAVIOUR	destinationNameBehaviour;

REGISTERED AS {g776.1Attribute 32};

destinationNameBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines destination name.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has String values of

Any string the end user selects up to 80

The default value is:

Comment: There is a one-to-one relationship between a particular destination name and its destination ID.

Reference:”;

### Destination Name Abbreviated

destinationNameAbbreviated	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DestinationNameAbbreviated;

MATCHES FOR EQUALITY;

BEHAVIOUR	destinationNameAbbreviatedBehaviour;

REGISTERED AS {g776.1Attribute 7};

destinationNameAbbreviatedBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines an abbreviation of the destination name.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has String values of

Any string the end user selects up to 5 characters (e.g., Paris, N.Y.C)

The default value is:

Comment: This abbreviation may be displayed on the equipment display panel where display space is limited. It is an alternative for Destination_Name. There is a one to one relationship between Destination_Name and Destination_Name_ Abbreviated.

Reference:”;

### Destination Number At Far End

destinationNumberAtFarEnd	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DestinationNumberAtFarEnd;

MATCHES FOR EQUALITY;

BEHAVIOUR	destinationNumberAtFarEndBehaviour;

REGISTERED AS {g776.1Attribute 393};

destinationNumberAtFarEndBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the multi-clique and multi-destination operational modes, the near end terminal should know the destination number at the far end that is related to every receive bearer. This number is necessary in order to extract the relevant Tx bearer from the Bearer Backward Alarm data in the synchronous data word and in order to identify the local destination of the received Voice Order Wire.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

1-4

The default value is:

Comment:

Reference: G.763”;

### Disconnect Idle Calls

disconnectIdleCalls	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	disconnectIdleCallsBehaviour;

REGISTERED AS {g776.1Attribute 33};

disconnectIdleCallsBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether idle calls (i.e., no activity detected at trunk channels input) are disconnected from the bearer, even if there is no demand to free the bearer channels for another call, after a period of 60 seconds.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment: The need to disconnect idle calls emanates from the need to be compatible with equipment that does not have enough transcoding resources to deal with multi-destination activity in different time zones. For G.763(1991)=Intelsat Rev2 DCME mode the parameter is fixed at disable. For G.763(1995)=INTELSAT Rev3 the default option setting is enable.

Reference: G.763”;

### DLC 64kbps Control

dlc64kbpsControl	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DLC64kbpsControl;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlc64kbpsControlBehaviour;

REGISTERED AS {g776.1Attribute 377};

dlc64kbpsControlBehaviour BEHAVIOUR

DEFINED AS

“This parameter allows for the manual override of the 64 kbit/s DLC state of the overload control algorithm. The feature values may be manual_on (forcing a congestive condition to be reported towards the switch regardless of the state of the algorithm), manual_off (inhibiting the report of a congestive condition by the algorithm towards the switch), or auto (where manual override is disabled and DLC control is handled by the internal overload algorithm).

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Manual_On, Manual_Off, Auto

The default value is: Auto

Comment:

Reference:”;

### DLC 64kbps Deactivation Timer

dlc64kbpsDeactivationTimer	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DLC64kbpsDeactivationTimer;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlc64kbpsDeactivationTimerBehaviour;

REGISTERED AS {g776.1Attribute 218};

dlc64kbpsDeactivationTimerBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the minimum duration 64 kbit/s DLC can be activated.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

10 to 25, 30, 40, 50, 60

The default value is: 10

Comment: The integer value represents seconds. This is refered to as Ta Timer in G.763 Paragraph 9.3 and Table 10. This parameter is a timer (in seconds) used for "stretching" the DLC active time after conditions for 64 kbit/s DLC deactivation are reached to prevent rapid oscillation at the switch. The values 30, 40, 50, 60 are manufacturer dependent. Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 9.3, Q.50”;

### DLC 64kbps On Ratio

dlc64kbpsOnRatio	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlc64kbpsOnRatioBehaviour;

REGISTERED AS {g776.1Attribute 219};

dlc64kbpsOnRatioBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the percentage of time during the most recent STI that the 64 kbit/s DLC was activated.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 15.2.3”;

### DLC 64kbps State

dlc64kbpsState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DLC64kbpsState;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlc64kbpsStateBehaviour;

REGISTERED AS {g776.1Attribute 217};

dlc64kbpsStateBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the state of the 64 kbit/s DLC.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Enumerated Option values of

Activated, Deactivated

The default value is:

Comment: Activated means informing the switch to stop sending new calls.

Reference: G.763 paragraph 9.3, Q.50”;

### DLC 64kbps Threshold

dlc64kbpsThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DLC64kbpsThreshold;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlc64kbpsThresholdBehaviour;

REGISTERED AS {g776.1Attribute 220};

dlc64kbpsThresholdBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the maximal number of transparent 64 kbit/s circuits accepted before 64 kbit/s DLC is to be activated.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

1 to Pool_Size for Pool_Transmit_ID

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 9.1.1”;

### DLC Allocated Bearer Nibbles

dlcAllocatedBearerNibbles	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DLCAllocatedBearerNibbles;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcAllocatedBearerNibblesBehaviour;

REGISTERED AS {g776.1Attribute 192};

dlcAllocatedBearerNibblesBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the maximum number of bearer nibbles (4-bit units) for traffic to a specific destination.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0 to 61

The default value is:

Comment: This parameter is used to activate a destination selective DLC (when the load is between the HL and LL thresholds) only towards the loading destination. This parameter is effective only in the destination selective DLC mode. Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 9.3.1.2.1”;

### DLC Data High Load Threshold

dlcDataHighLoadThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcDataHighLoadThresholdBehaviour;

REGISTERED AS {g776.1Attribute 213};

dlcDataHighLoadThresholdBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the data bearer occupancy threshold at which the DCME will activate both voice/VB and 64 kbit/s DLC.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Real values of

0 to 100 in steps of 1

The default value is:

Comment: The integer value represents percent (e. g., 1 means 1%). The data bearer occupancy is the percentage of the bearer occupied by VBD 40 kbit/s channels. Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 9”;

### DLC Data Low Load Threshold

dlcDataLowLoadThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcDataLowLoadThresholdBehaviour;

REGISTERED AS {g776.1Attribute 214};

dlcDataLowLoadThresholdBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the data bearer occupancy threshold at which the DCME will activate both voice/VB and 64 kbit/s DLC.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Real values of

0 to 100 in steps of 1

The default value is:

Comment: The integer value represents percent (e. g., 1 means 1%). The data bearer occupancy is the percentage of the bearer occupied by VBD 40 kbit/s channels. Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 9”;

### DLC First Averaging Period

dlcFirstAveragingPeriod	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DLCFirstAveragingPeriod;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcFirstAveragingPeriodBehaviour;

REGISTERED AS {g776.1Attribute 222};

dlcFirstAveragingPeriodBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the interval length between samples of the speech rate taken from the bearer. This parameter also determines the averaging period for each average bit per sample (abs) calculation.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

8, 32, 64, 128, 256

The default value is:

Comment: The integer value represents milliseconds. This parameter also determines the averaging period for each average bit per sample (abs) calculation. Any integer percentage can be real with higher resolution.

Reference: : G.763 paragraph 9.3 and Annex B.1”;

### DLC Mode

dlcMode	 ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DLCMode;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcModeBehaviour;

REGISTERED AS {g776.1Attribute 223};

dlcModeBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the Dynamic Load Control (DLC) for the associated clique."

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Selective, Global

The default value is: See comment

Comment: For multidestination networks, it is fairer to apply DLC only towards the source that exceeds its Allocated_Bearer_Nibbles (i.e., selective). Default Value: If DCME_Type=G.763_1991 then global, If DCME_Type=G.763_1995 then selective.

Reference: G.763 paragraph 9.3”;

### DLC Sympathetic Control

dlcSympatheticControl	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcSympatheticControlBehaviour;

REGISTERED AS {g776.1Attribute 379};

dlcSympatheticControlBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether local DLC is allowed to be controlled by the far DLC congestive control states. When enabled, far DLC is allowed to induce local DLC; when disabled, local DLC is only based on local congestive conditions. The enabled or disabled condition apply to both the 64kbps DLC and Voice DLC states.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is: Enable

Comment: This feature is only applicable to point to point operation. Caution is advised in use of DLC Sympathetic Control because it may cause a degredation in the quality of service under overload conditions.

Reference:”;

### DLC TCH Interaction

dlcTCHInteraction	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcTCHInteractionBehaviour;

REGISTERED AS {g776.1Attribute 224};

dlcTCHInteractionBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the transparent call handling when 64 kbit/s DLC is activated.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment: When this parameter is disabled, TCH ignores 64 kbit/s DLC condition and accepts switch requests (even when 64 kbit/s DLC is activated). When this parameter is enabled, TCH rejects switch requests if 64 kbit/s DLC is activated.

Reference: G.763 paragraph 9.3, Table 10, and Annex B”;

### DLC Voice Control

dlcVoiceControl	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DLCVoiceControl;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcVoiceControlBehaviour;

REGISTERED AS {g776.1Attribute 378};

dlcVoiceControlBehaviour BEHAVIOUR

DEFINED AS

“This parameter allows for the manual override of the Voice DLC state of the overload control algorithm. The feature values may be manual_on (forcing a congestive condition to be reported towards the switch regardless of the state of the algorithm), manual_off (inhibiting the report of a congestive condition by the algorithm towards the switch), or auto (where manual override is disabled and DLC control is handled by the internal overload algorithm).

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Manual_On, Manual_Off, Auto

The default value is: Auto

Comment:

Reference:”;

### DLC Voice Deactivation  Timer

dlcVoiceDeactivationTimer	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DLCVoiceDeactivationTimer;

MATCHES FOR EQUALITY;

BEHAVIOUR dlcVoiceDeactivationTimerBehaviour;

REGISTERED AS {g776.1Attribute 225};

dlcVoiceDeactivationTimerBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the minimum duration Voice/VB DLC can be activated.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

10 to 25, 30, 40, 50, 60

The default value is: 10

Comment: The integer value represents seconds. This is refered to as Ta Timer in G.763 Paragraph 9.3 and Table 10. This parameter is a timer (in seconds) used for "stretching" the DLC active time after conditions for Voice/VBD DLC deactivation are reached to prevent rapid oscillation at the switch. The values 30, 40, 50, 60 are manufacturer dependent. Any integer percentage can be real with higher resolution.

Reference: G.763”;

### DLC Voice High Load Threshold

dlcVoiceHighLoadThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DLCVoiceHighLoadThreshold;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcVoiceHighLoadThresholdBehaviour;

REGISTERED AS {g776.1Attribute 311};

dlcVoiceHighLoadThresholdBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the value of the bearer average bits/sample for voice (abs) the criterion at which the DCME will activate both Voice/VBD and 64 kbit/s DLC.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Real values of

2.00 to 4.00 in steps of 0.05

The default value is:

Comment: See G.763 for activation criteria.

Reference: G.763 paragraph 9”;

### DLC Voice Low Load Threshold

dlcVoiceLowLoadThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.DLCLowLoadThreshold;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcVoiceLowLoadThresholdBehaviour;

REGISTERED AS {g776.1Attribute 312};

dlcVoiceLowLoadThresholdBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the value of the bearer average bits/sample for voice (abs) the criterion at which the DCME will deactivate Voice/VB DLC.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Real values of

2.00 to 4.00 in steps of 0.05

The default value is:

Comment: See G.763 for deactivation criteria.

Reference: G.763 paragraph 9”;

### DLC Voice State

dlcVoiceState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.DLCVoiceState;

MATCHES FOR EQUALITY;

BEHAVIOUR	dlcVoiceStateBehaviour;

REGISTERED AS {g776.1Attribute 226};

dlcVoiceStateBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the state of the voice/VB DLC.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Enumerated Option values of

Activated, Deactivated

The default value is:

Comment: Fill-in

Reference: G.763 paragraph 9.3, Q.50”;

### DS0 ID

ds0ID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Ds0ID;

MATCHES FOR EQUALITY;

BEHAVIOUR	ds0IDBehaviour;

REGISTERED AS {g776.1Attribute 227};

ds0IDBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies the 64 kbit/s time slot on the primary rate link.

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

For PRI_Rate=1544 the values are 1-24; for PRI_Rate=2048 the values are 1-31

The default value is:

Comment: 1 through 24 for PRI_Rate whose rate is 1544, 1 through 31 for PRI_Rate whose rate is 2048.  Any integer percentage can be real with higher resolution.

Reference:”;

### DSI

dsi	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	dsiBehaviour;

REGISTERED AS {g776.1Attribute 228};

dsiBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether digital signal interpolation is performed on a per channel basis.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment: For DCME, DSI is enabled for dynamically assigned channels and is disabled for preassigned channels.

Reference: G.763, G.766”;

### EC Comfort Noise

ecComfortNoise	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECMethods;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecComfortNoiseBehaviour;

REGISTERED AS {g776.1Attribute 338};

ecComfortNoiseBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether or not the Comfort Noise Matching feature is disabled or enabled in the Methods which are manufacturer specific and this provides a standard way to specifiy among a list of manufacturer supplied choices.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Disable

Method1

Method2

Method3

Method4

Method5

Method6

Method7

The default value is:

Comment: If EC_Non-linear_Processor is disabled, EC_Comfort_Noise has no effect. The Comfort Noise Methods are manufacturer specific and this provides a standard way to specify among a list of manufacturer supplied choices.

Reference:”;

### EC Comfort Noise Far

ecComfortNoiseFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecComfortNoiseFarBehaviour;

REGISTERED AS {g776.1Attribute 367};

ecComfortNoiseFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

The default value is:

Comment:

Reference:”;

### EC CRC Detection

ecCRCDetection	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECCrcDetection;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecCRCDetectionBehaviour;

REGISTERED AS {g776.1Attribute 399};

ecCRCDetectionBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines at which port the CRC is checked.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

RCV Only, SND Only, Both, None

The default value is: None

Comment:

Reference:”;

### EC Data Tone Disabler Disable Mode

ecDataToneDisablerDisableMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECDataToneDisablerMode;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecDataToneDisablerDisableModeBehaviour;

REGISTERED AS {g776.1Attribute 339};

ecDataToneDisablerDisableModeBehaviour BEHAVIOUR

DEFINED AS

“The echo canceller is equipped with a tone disabler that responds to a disabling signal and causes the echo cancellation function to be disabled..

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

None - The tone disabler function is not set.

G.164 - The tone disabler function conforms to the echo suppressor tone disabler requirement detailed in Section 5 of ITU-T Recommendation G.164 dealing with the dectection of 2100 Hz tone with or withou phase reversal.

G.165/G.168 - The tone disabler function conforms to the echo canceller tone disabler requirements detailed in Section 4 of ITU-T Recommendations G.165/G.168 dealing with the detection of of 2100 Hz tone with phase reversal.

Switch 56 - Modified G.168 echo canceller tone disabler function dealing with the detection of a combination of 2100 Hz tone and the state of bit 8.

The default value is:

Comment: The data tone (2100 Hz) is defined in V.25,V.8. The behavior of the echo canceller in G164, G.165, G.168.

Reference:”;

### EC Data Tone Disabler Reenable Mode

ecDataToneDisablerReenableMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECDataToneDisablerReenableMode;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecDataToneDisablerReenableModeBehaviour;

REGISTERED AS {g776.1Attribute 340};

ecDataToneDisablerReenableModeBehaviour BEHAVIOUR

DEFINED AS

“The echo canceller is equipped with a tone disabler that responds to a disabling signal and causes the echo cancellation function to be disabled.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

G.164/G.165/G.168 - EC data tone disablers reenable the echo canceller on the absence of energy according to G.164/G.165/G.168.

Switch 56 - EC data tone disablers reenable the echo canceller based in bit 8 and the absence of energy according to G.164/G.165/G.168.

End of Call - EC data tone disablers reenable the echo canceller based upon the channel returning to an idle state.  Hold band energy is completely ignored as is the presence of bit 8 "high".

The default value is:

Comment

Reference:”;

### EC Echo Characteristics

ecEchoCharacteristics	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECMethods;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecEchoCharacteristicsBehaviour;

REGISTERED AS {g776.1Attribute 366};

ecEchoCharacteristicsBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the specific set of internal parameters for the echo canceller function.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Method 1 - Default set of parameters

Method 2 - Debug set of parameters

Method 3 ... Method 7 - Manufacturer specific

The default value is:

Comment:

Reference:”;

### EC Echo Characteristics Far

ecEchoCharacteristicsFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECMethods;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecEchoCharacteristicsFarBehaviour;

REGISTERED AS {g776.1Attribute 333};

ecEchoCharacteristicsFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the sensitivity setting for the remote echo canceller function.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Method 1    Method 2

Method 3    Method 4

Method 5    Method 6

Method 7

The default value is:

Comment: Echo cancellers may be used to cancel echoes originating in several different types of networks, such as terrestrial, satellite and cellular networks. Various operational characteristics of an echo canceller need to be optimized to ensure the best performance in each network environment. This attribute is used to select which of these methods supported by a particular manufacturer is enabled. Note that selection of a method will affect other configurable attributes, such as Comfort Noise and NLP settings.

Reference:”;

### EC Min ERL

ecMinERL	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECMinERL;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecMinERLBehaviour;

REGISTERED AS {g776.1Attribute 233};

ecMinERLBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the near end speech detection threshold for echo cancellation.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0, 1, 2, 3, 4, 5, 6

The default value is:

Comment: The integer value represents dB. The parameter name refers to minimum echo return loss. Any integer percentage can be real with higher resolution.

Reference: G.165”;

### EC Min ERL Far

ecMinERLFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ecMinERLFar;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecMinERLFarBehaviour;

REGISTERED AS {g776.1Attribute 332};

ecMinERLFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the minimum echo return loss in dB for the remote echo canceller function.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0, 1, 2, 3, 4, 5, 6

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference:”;

### EC Non-linear Processor

ecNonlinearProcessor	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECMethods;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecNonlinearProcessorBehaviour;

REGISTERED AS {g776.1Attribute 369};

ecNonlinearProcessorBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether an echo cancellers NLP feature is disabled or enabled in one of seven possible methods. These methods are provided to support different vendor strategies for non-linear processing.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Method 1    Method 2

Method 3    Method 4

Method 5    Method 6

Method 7

The default value is:

Comment: Several components of the Non-linear Processor's function in an echo canceller may be controlled to optimize speech quality for various call types and network configurations. This allows selection of a preconfigured set of these components, determined by the manufacturer to be suitable for specific network environments.

Reference:”;

### EC Non-linear Processor Far

ecNonlinearProcessorFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecNonlinearProcessorFarBehaviour;

REGISTERED AS {g776.1Attribute 368};

ecNonlinearProcessorFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

The default value is:

Comment:

Reference:”;

### EC Offset Delay

ecOffsetDelay	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECOffsetDelay;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecOffsetDelayBehaviour;

REGISTERED AS {g776.1Attribute 328};

ecOffsetDelayBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines offset delay. The offset delay refers to an increment of continuous delay which is intended to move the EC's processing window to a predetermined time relative to the start of the echo signal. This is a configuration parameter set by the user to based on forseen endpaths to some value up to the EC_Offset_Delay_Max.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0,nx16(n=1-70;1120ms maximum)

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples. Offset is sometimes referred to as bulk or fixed delay. Any integer percentage can be real with higher resolution.

Reference:”;

### EC Offset Delay Far

ecOffsetDelayFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECOffsetDelayFar;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecOffsetDelayFarBehaviour;

REGISTERED AS {g776.1Attribute 329};

ecOffsetDelayFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines offset delay when the ECE endpath is towards the far end. The offset delay refers to an increment of continuous delay which is intended to move the EC's processing window to a predetermined time relative to the start of the echo signal. This is a configuration parameter set by the user to based on forseen endpaths to some value up to the EC_Offset_Delay_Max.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0,nx16(n=1-70;1120ms maximum)

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples. Offset is sometimes referred to as bulk or fixed delay. Any integer percentage can be real with higher resolution.

Reference:”;

### EC Offset Delay Max

ecOffsetDelayMax	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECOffsetDelayMax;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecOffsetDelayMaxBehaviour;

REGISTERED AS {g776.1Attribute 330};

ecOffsetDelayMaxBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines offset capability of the ECE. The offset delay refers to an increment of continuous delay which is intended to move the EC's processing window to a predetermined time relative to the start of the echo signal. This is a configuration parameter that is determined by the configuration of the equipment purchased.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read Only attribute that has Integer values of

0,nx16(n=1-70;1120ms maximum)

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples. Offset is sometimes referred to as bulk or fixed delay. A value of 0 is returned when the echo canceller has no offset delay capability. Any integer percentage can be real with higher resolution.

Reference:”;

### EC Offset Delay Max Far

ecOffsetDelayMaxFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECOffsetDelayMaxFar;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecOffsetDelayMaxFarBehaviour;

REGISTERED AS {g776.1Attribute 331};

ecOffsetDelayMaxFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines offset capability of the ECE when the endpath is towards the far end. The offset delay refers to an increment of continuous delay which is intended to move the EC's processing window to a predetermined time relative to the start of the echo signal. This is a configuration parameter that is determined by the configuration of the equipment purchased.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read Only attribute that has Integer values of

0,nx16(n=1-70;1120ms maximum)

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples. Offset is sometimes referred to as bulk or fixed delay. Any integer percentage can be real with higher resolution.

Reference:”;

### EC Per Call CAS

ecPerCallCAS	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecPerCallCASBehaviour;

REGISTERED AS {g776.1Attribute 402};

ecPerCallCASBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines how CAS bit or bits are used to disable the echo canceller.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

None, A=0, A=1, B=0, B=1, C=0, C=1, D=0, D=1, E&M, FXO-GS, FXO-LS, FXS-GS, FXS-LS, signaling_scheme_1, signaling_scheme2

The default value is:

Comment: This attribute is used in conjunction with EC_Per_Call_Control and is only in effect if the CAS value is selected. Typically, single CAS bit control is used when the transmission facility is 2048 kbit/s. Two bits (i.e., A&B) are typically used when the transmission facility is 1544 kbit/s. The values, E&M, FXO-GS, FXO-LS, FXS-GS, FXS-LS, actually represent A&B bit states. Two additional values are also provided for any customized configurations of CAS. None is used when CAS signalling is not being used.

Reference:”;

### EC Per Call Control

ecPerCallControl	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecPerCallControlBehaviour;

REGISTERED AS {g776.1Attribute 335};

ecPerCallControlBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the method by which an echo cancellers signal processing can be controlled per call.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enabled,Disabled,CAS,Idle_Channel_Code,Q.50,Method1-Method7

The default value is:

Comment: If Enabled, the echo canceller is permanently enabled and there are no possible methods available for external control of the echo canceller.  Some of the available methods for control would be the detection of busy and idle conditions by monitoring the bit stream for Channel Associated Signalling as specified in G.704.  Other proprietary methods for per call control from a switch can be accommodated as a vendor specific method. Enabled means that the echo canceller is permanently enabled and there are no possible methods available for external control of the echo canceller. These methods are provided to accommodate different network signalling schemes as well as vendor proprietary switch control schemes. Disabled means that the echo canceller is permanently disabled and there are no possible methods available for external control of the echo canceller.

Reference:”;

### EC Processing Capability

ecProcessingCapability	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECProcessingCapability;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecProcessingCapabilityBehaviour;

REGISTERED AS {g776.1Attribute 323};

ecProcessingCapabilityBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the provisioned processing capability (window). This is a configuration parameter set by the user to based on forseen endpaths to some value up to the EC_Processing_Capability_Max.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

16,32,48,64,80,96,128

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples.

Reference:”;

### EC Processing Capability Far

ecProcessingCapabilityFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECProcessingCapability;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecProcessingCapabilityFarBehaviour

REGISTERED AS {g776.1Attribute 324};

ecProcessingCapabilityFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the provisioned processing capability (window) with its endpath towards the far-end. This is a configuration parameter set by the user to based on forseen endpaths to some value up to the EC Processing Capability Far Max.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

0,16,32,48,64,80,96,128

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples.

Reference:”;

### EC Processing Capability Max

ecProcessingCapabilityMax	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECProcessingCapability;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecProcessingCapabilityMaxBehaviour;

REGISTERED AS {g776.1Attribute 326};

ecProcessingCapabilityMaxBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the maximum processing capability (window) of the ECE. This does not include the offset delay.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read Only attribute that has Enumerated Option values of

16,32,48,64,80,96,128

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples.

Reference:”;

### EC Processing Capability Max Far

ecProcessingCapabilityMaxFar	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECProcessingCapability;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecProcessingCapabilityMaxFarBehaviour;

REGISTERED AS {g776.1Attribute 327};

ecProcessingCapabilityMaxFarBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the maximum processing capability (window) of the ECE with its endpath towards the far end. This does not include the offset delay.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read Only attribute that has Enumerated Option values of

0,16,32,48,64,80,96,128

The default value is:

Comment: The values are in milliseconds. See Section 10.2 for EC offset/processing explanation and examples. A value of 0 is returned if the ECE does not possess this capability.

Reference:”;

### EC RCV CRC

ecRCVCRC	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECRCVCRC;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecRCVCRCBehaviour;

REGISTERED AS {g776.1Attribute 398};

ecRCVCRCBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines how the CRC is generated at the RCV Out port.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Pass, Correct

The default value is: Pass

Comment: If correct is selected, the CRC is always recomputed based on the incoming signal (after the digital signal processing has been done) and inserted on the RCV Out. Incoming CRC errors are ignored. If pass is selected, the CRC is also recomputed based on the incoming RCV signal (after the digital signal processing has been done) and inserted on the RCV Out as long as no incoming CRC errors are detected. In the case of an incoming CRC error, an incorrect CRC is inserted on the RCV Out.

Reference:”;

### EC Self Test In Service

ecSelfTestInService	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecSelfTestInServiceBehaviour;

REGISTERED AS {g776.1Attribute 349};

ecSelfTestInServiceBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the Echo Canceller self-test mode.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment:

Reference:”;

### EC Signalling Tone Disabler

ecSignallingToneDisabler	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECSignallingToneDisabler;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecSignallingToneDisablerBehaviour;

REGISTERED AS {g776.1Attribute 375};

ecSignallingToneDisablerBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines  the presence and operation of an in-band signalling tone disabler.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

None, 2280Hz, 2400Hz, 2600Hz, R1, R2, CCITT#5, Sig_Scheme_1, Sig_Scheme_2

The default value is:

Comment: Signalling tone disablers may be needed to disable the echo canceller and keep it from affecting the in-band signalling tones used in the call control process. None is returned if the echo canceller is not equipped with a Signalling Tone disabler.

Reference:”;

### EC SND CRC

ecSNDCRC	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECSNDCRC;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecSNDCRCBehaviour;

REGISTERED AS {g776.1Attribute 397};

ecSNDCRCBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines how the CRC is generated at the SND Out port.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Pass, Correct

The default value is: Correct

Comment: If correct is selected, the CRC is always recomputed based on the modified SND signal (echo removed) and inserted on the SND Out. Incoming CRC errors are not reinserted in the outgoing bit stream. If pass is selected, the CRC is also recomputed based on the modified SND signal (echo removed) and inserted on the SND Out as long as no incoming CRC errors are detected. In the case of an incoming CRC error, an incorrect CRC is inserted on the SND Out.

Reference:”;

### EC Timing Reference

ecTimingReference	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECTimingReference;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecTimingReferenceBehaviour;

REGISTERED AS {g776.1Attribute 342};

ecTimingReferenceBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the Echo Canceller timing reference.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0 - Number of Facilities

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference:”;

### EC Timing Reference Side

ecTimingReferenceSide	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ECTimingReferenceSide;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecTimingReferenceSideBehaviour;

REGISTERED AS {g776.1Attribute 343};

ecTimingReferenceSideBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the which side is selected for the Echo Canceller  timing reference.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Near, Far

The default value is:

Comment:

Reference:”;

### EC VPA Tone Disable

ecVPAToneDisable	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ECVPAToneDisable;

MATCHES FOR EQUALITY;

BEHAVIOUR	ecVPAToneDisableBehaviour;

REGISTERED AS {g776.1Attribute 374};

ecVPAToneDisableBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the presence and operation of a  Voice Path Assurance (VPA) tone disabler.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

None, Enable or Disable

The default value is:

Comment: The Voice Path Assurance tone is defined in CCITT #6 and #7 Signalling Systems and is used to verify that there is continuity in the end-to-end voice path. None is returned if the echo canceller is not equipped with a VPA tone disabler.

Reference:”;

### Facility Loop Test

facilityLoopTest	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.FacilityLoopTest;

MATCHES FOR EQUALITY;

BEHAVIOUR	facilityLoopTestBehaviour;

REGISTERED AS {g776.1Attribute 352};

facilityLoopTestBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the loop state of the facilities.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Loop_None, Loop_Near, Loop_Far, Loop_Both

The default value is:

Comment:

Reference:”;

### Facsimile Bit Rate ID

facsimileBitRateID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.FacsimileBitRateID;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileBitRateIDBehaviour;

REGISTERED AS {g776.1Attribute 363};

facsimileBitRateIDBehaviour BEHAVIOUR

DEFINED AS

“This identifies the bit rate of fax modems.

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has Enumerated Option values of

For V.17 and V.33 - 7200,9600,12000,14400

For V.27ter - 2400,4800

For V.29 - 7200,9600

The default value is:

Comment:

Reference: G.766 Table 1”;

### Facsimile Call Direction ID

facsimileCallDirectionID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.FacsimileCallDirectionID;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileCallDirectionIDBehaviour;

REGISTERED AS {g776.1Attribute 365};

facsimileCallDirectionIDBehaviour BEHAVIOUR

DEFINED AS

“This identifies the direction for number of fax calls performance reports

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has Enumerated Option values of

Incoming

Outgoing

Both

The default value is: Both

Comment:

Reference:”;

### Facsimile Demodulation

facsimileDemodulation	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileDemodulationBehaviour;

REGISTERED AS {g776.1Attribute 235};

facsimileDemodulationBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether facsimile remodulation/demodulation is done on a per channel basis.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment: This parameter overrides the parameters specified for an individual channel. Disable deactivates facsimile demodulation. In this case, the incoming traffic will not be monitored to detect the presence of facsimile, even on channels that have been set by service administration to have facsimile demodulation.

Reference:G.766 Table 1”;

### Facsimile FEC Activation

faxFECActivation	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.FacsimileFECActivation;

MATCHES FOR EQUALITY;

BEHAVIOUR	faxFECActivationBehaviour;

REGISTERED AS {g776.1Attribute 239};

BEHAVIOUR	faxFECActivationBehaviour

DEFINED AS

“This parameter defines whether and how the Forward Error Correction (FEC) information bits will be added to the facsimile blocks (data) on the bearer.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable, Automatic

The default value is:

Comment:

Reference: G.766 paragraph 5.3.1 and paragraph 10.4”;

### Facsimile FEC State

faxFECState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	faxFECStateBehaviour;

REGISTERED AS {g776.1Attribute 240};

faxFECStateBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies whether forward error correction is currently performed.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Enumerated Option values of

Enabled, Disabled

The default value is:

Comment: If Facsimile FECActivation is automatic, the state is dependent on the link bit error rate.

Reference: G.766”;

### Facsimile Mean Number of Banks Allocated

facsimileMeanNumberofBanksAllocated	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.FacsimileMeanNumberofBanksAllocated;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileMeanNumberofBanksAllocatedBehaviour;

REGISTERED AS {g776.1Attribute 249};

facsimileMeanNumberofBanksAllocatedBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the average number of facsimile banks, per DCME frame, allocated during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 61

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Facsimile Mean Number of Image Blocks Per Call

facsimileMeanNumberofImageBlocksPerCall	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.FacsimileMeanNumberofImageBlocksPerCall;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileMeanNumberofImageBlocksPerCallBehaviour;

REGISTERED AS {g776.1Attribute 250};

facsimileMeanNumberofImageBlocksPerCallBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the average number of image blocks used per facsimile call during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 9999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Facsimile Modulation Scheme ID

facsimileModulationSchemeID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.FacsimileModulationSchemeID;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileModulationSchemeIDBehaviour;

REGISTERED AS {g776.1Attribute 362};

facsimileModulationSchemeIDBehaviour BEHAVIOUR

DEFINED AS

“The indentifies the modulation scheme used in facsimile calls.

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has Enumerated Option values of

V.17

V.27ter

V.29

V.33

The default value is:

Comment:

Reference: G.766 Table 1”;

### Facsimile Remod Output Level

facsimileRemodOutputLevel	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.FacsimileRemodOutputLevel;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileRemodOutputLevelBehaviour;

REGISTERED AS {g776.1Attribute 241};

facsimileRemodOutputLevelBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the level of the remodulated facsimile signal.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

-13 to -39

The default value is: -17

Comment: The integer value represents dBm0.

Reference: Table 7 of G.766”;

### Facsimile Remod Output Level Mode

facsimileRemodOutputLevelMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.FacsimileRemodOutputLevelMode;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileRemodOutputLevelModeBehaviour;

REGISTERED AS {g776.1Attribute 373};

facsimileRemodOutputLevelModeBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines whether the remodulator output level will be fixed (-17 dbm) or matched to the measured transmission level at the far end.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Matched or Fixed

The default value is:

Comment:

Reference:”;

### Facsimile Statistics Time Interval

facsimileStatisticsTimeInterval	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.TimeInterval;

MATCHES FOR EQUALITY;

BEHAVIOUR	facsimileStatisticsTimeIntervalBehaviour;

REGISTERED AS {g776.1Attribute 236};

facsimileStatisticsTimeIntervalBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the resolution of the facsimile traffic statistics time interval for which facsimile traffic statistics are measured.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

10, 20, 30, 40, 50, 60

The default value is: 10

Comment: The integer value represents minutes. Any integer percentage can be real with higher resolution.

Reference:G.766 Section 10.1”;

### Failed Seizures Ratio 64kbps

failedSeizuresRatio64kbps	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	failedSeizuresRatio64kbpsBehaviour;

REGISTERED AS {g776.1Attribute 238};

failedSeizuresRatio64kbpsBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the percentage of 64 kbit/s demand seizure attempts that were negatively acknowledged by the DCME in the most recent STI for whatever reason.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: The integer values represent percent (e.g. 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 15.2.3”;

### FCC Local Error Count

fccLocalErrorCount	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.FCCLocalErrorCount;

MATCHES FOR EQUALITY;

BEHAVIOUR	fccLocalErrorCountBehaviour;

REGISTERED AS {g776.1Attribute 242};

fccLocalErrorCountBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides the accumulated number of facsimile control channel bit errors received by the local terminal in the last 30 seconds.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Enumerated Option values of

0 to 255

The default value is:

Comment: If the count reaches 255, it saturates until the next measurement interval (next FSTI). FCC=fax control channel.

Reference: G.766 paragraph 10.3”;

### FCC Remote Error Count

fccRemoteErrorCount	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.FCCRemoteErrorCount;

MATCHES FOR EQUALITY;

BEHAVIOUR	fccRemoteErrorCountBehaviour;

REGISTERED AS {g776.1Attribute 243};

fccRemoteErrorCountBehaviour BEHAVIOUR

DEFINED AS

“This parameter provides statistic records for accumulated number of facsimile control channel bit errors received by the remote terminal in the last 30 seconds.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Enumerated Option values of

0 to 255

The default value is:

Comment: If the count reaches 255, it saturates until the next measurement interval (next FSTI). FCC=fax control channel.

Reference: G.766 paragraph 10.3”;

### Functional Name

functionName ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR functionNameBehaviour;

REGISTERED AS { g776.1Attribute 419};

functionNameBehaviour BEHAVIOUR

DEFINED AS

“The functionName is an attribute whose distinguished value can be used as a RDN when naming an instance of the functionPool object class. The specific function must be already installed and is represented by the deviceFunctionDefinition MO Class thru the deviceFunctionDefinition request.”;

### Injected Rx Noise Max

injectedRxNoiseMax	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.InjectedRxNoiseMax;

MATCHES FOR EQUALITY;

BEHAVIOUR	injectedRxNoiseMaxBehaviour;

REGISTERED AS {g776.1Attribute 57};

injectedRxNoiseMaxBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines the maximum noise level, in dBm0, to be injected on the receive trunk side even when the matched noise level measured at the corresponding transmit side exceeds this level.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

-68,-65,-60,-55,-50

The default value is: -50

Comment: The integer represents dBm0. The value -68 is for mu-law only

Reference: G.763 Table 6”;

### IT ID

itID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ITID;

MATCHES FOR EQUALITY;

BEHAVIOUR	itIDBehaviour;

REGISTERED AS {g776.1Attribute 246};

itIDBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies an intermediate time slot used for conveying trunk channel (TC) to bearer channel (BC) connectivity.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

1 to 255

The default value is:

Comment: 1 to 216 are used for normal TC to IT connections, IT's above this range are used for special internal DCME purposes. Any integer percentage can be real with higher resolution.

Reference: G.763”;

### Map Change Handler Activation

mapChangeHandlerActivation	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapChangeHandlerActivationBehaviour;

REGISTERED AS {g776.1Attribute 247};

mapChangeHandlerActivationBehaviour BEHAVIOUR

DEFINED AS

“This parameter defines a map change to be when the background map and the foreground map are switched.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment: When this parameter is enabled, the channel check procedure is inhibited and both voice/VB and 64 kbit/s DLCs are activated.

Reference: Section 15.8 of G.763”;

### Map Creation Date

mapCreationDate	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapCreationDate;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapCreationDateBehaviour;

REGISTERED AS {g776.1Attribute 386};

mapCreationDateBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

MMDDYY

The default value is:

Comment:

Reference:”;

### Map Creation Time

mapCreationTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapCreationTime;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapCreationTimeBehaviour;

REGISTERED AS {g776.1Attribute 387};

mapCreationTimeBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

HHMMSS

The default value is:

Comment:

Reference:”;

### Map Last Updated By

mapLastUpdatedBy	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapLastUpdatedBy;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapLastUpdatedByBehaviour;

REGISTERED AS {g776.1Attribute 389};

mapLastUpdatedByBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has String values of

The default value is:

Comment:

Reference:”;

### Map Name

mapName	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapName;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapNameBehaviour;

REGISTERED AS {g776.1Attribute 248};

mapNameBehaviour BEHAVIOUR

DEFINED AS

“ This paramter identifies the configuration map.

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has String values of

Any string up to 8 characters.

The default value is:

Comment:

Reference:”;

### Map Number Of Updates

mapNumberOfUpdates	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapNumberOfUpdates;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapNumberOfUpdatesBehaviour;

REGISTERED AS {g776.1Attribute 388};

mapNumberOfUpdatesBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

The default value is:

Comment:

Reference:”;

### Map Organization

mapOrganization	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapOrganization;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapOrganizationBehaviour;

REGISTERED AS {g776.1Attribute 384};

mapOrganizationBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has String values of

The default value is:

Comment:

Reference:”;

### Map Site Name

mapSiteName	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapSiteName;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapSiteNameBehaviour;

REGISTERED AS {g776.1Attribute 385};

mapSiteNameBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has String values of

The default value is:

Comment:

Reference:”;

### Map Update Date

mapUpdateDate	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapUpdateDate;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapUpdateDateBehaviour;

REGISTERED AS {g776.1Attribute 390};

mapUpdateDateBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

MMDDYY

The default value is:

Comment:

Reference:”;

### Map Update Time

mapUpdateTime	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.MapUpdateTime;

MATCHES FOR EQUALITY;

BEHAVIOUR	mapUpdateTimeBehaviour;

REGISTERED AS {g776.1Attribute 391};

mapUpdateTimeBehaviour BEHAVIOUR

DEFINED AS

“

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

HHMMSS

The default value is:

Comment:

Reference:”;

### Mask Performance Reports

maskPerformanceReports	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.MaskPerformanceReports;

MATCHES FOR EQUALITY;

BEHAVIOUR	maskPerformanceReportsBehaviour;

REGISTERED AS {g776.1Attribute 354};

maskPerformanceReportsBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines whether the autonomous reports are masked.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Normal, Masked

The default value is:

Comment:

Reference:”;

### Module Provisioning State

moduleProvisioningState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.ModuleProvisioningState;

MATCHES FOR EQUALITY;

BEHAVIOUR	moduleProvisioningStateBehaviour;

REGISTERED AS {g776.1Attribute 319};

moduleProvisioningStateBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the restoral of provisioning information in a module or its back up copy.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Normal, Reload_from_non_volatile, Reload_from_backup

The default value is:

Comment:

Reference:”;

### Number Of ADPCM Fallbacks Due To NSF Failure

numberOfADPCMFallbacksDueToNSFFailure	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfADPCMFallbacksDueToNSFFailure;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfADPCMFallbacksDueToNSFFailureBehaviour;

REGISTERED AS {g776.1Attribute 251};

numberOfADPCMFallbacksDueToNSFFailureBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the number of times the facsimile demod/remod units had to cease compressing the call because the non-standard facilities used in the facsimile call could not handle the call during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 9999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Number Of ADPCM Fallbacks Not Due To DCN Detection

numberOfADPCMFallbacksNotDueToDCNDetection	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfADPCMFallbacksNotDueToDCNDetection;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfADPCMFallbacksNotDueToDCNDetectionBehaviour;

REGISTERED AS {g776.1Attribute 252};

numberOfADPCMFallbacksNotDueToDCNDetectionBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the number of times the facsimile demod/remod units had to cease compressing the call for any reason other than normal termination during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 9999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Number Of Facsimile Bank Allocation Failures Due to Near End Congestion

numberOfFacsimileBankAllocationFailuresDuetoNearEndCongestion	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfFacsimileBankAllocationFailuresDuetoNearEndCongestion;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfFacsimileBankAllocationFailuresDuetoNearEndCongestionBehaviour;

REGISTERED AS {g776.1Attribute 253};

numberOfFacsimileBankAllocationFailuresDuetoNearEndCongestionBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the number of times the transmission equipment was unable to allocate a facsimile bank because the near-end was congested with traffic during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 9999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference:”;

### Number Of Facsimile Calls

numberOfFacsimileCalls	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfFacsimileCalls;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfFacsimileCallsBehaviour;

REGISTERED AS {g776.1Attribute 254};

numberOfFacsimileCallsBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the number of facsimile calls in the most recent FSTI in the direction Facsimile Direction ID (incoming, outgoing, both).

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 9999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Number Of Facsimile Calls Using Non-Standard Facilities

numberOfFacsimileCallsUsingNonStandardFacilities	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfFacsimileCallsUsingNonStandardFacilities;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfFacsimileCallsUsingNonStandardFacilitiesBehaviour;

REGISTERED AS {g776.1Attribute 255};

numberOfFacsimileCallsUsingNonStandardFacilitiesBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the number of facsimile calls compressed using non-standard facilites of T.30 during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 9999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Number Of Facsimile Calls Using Standard Facilities

numberOfFacsimileCallsUsingStandardFacilities	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfFacsimileCallsUsingStandardFacilities;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfFacsimileCallsUsingStandardFacilitiesBehaviour;

REGISTERED AS {g776.1Attribute 256};

numberOfFacsimileCallsUsingStandardFacilitiesBehaviour BEHAVIOUR

DEFINED AS

" This parameter provides the number of facsimile calls compressed using standard facilites of T.30 during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 9999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Number Of Facsimile Demod Remod Units

numberOfFacsimileDemodRemodUnits	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfFacsimileDemodRemodUnits;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfFacsimileDemodRemodUnitsBehaviour;

REGISTERED AS {g776.1Attribute 257};

numberOfFacsimileDemodRemodUnitsBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the maximum number of facsimile demod/remod units available to do facsimile compression.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 240

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Number Of Image Blocks At Each Bit Rate

numberOfImageBlocksAtEachBitRate	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NumberOfImageBlocksAtEachBitRate;

MATCHES FOR EQUALITY;

BEHAVIOUR	numberOfImageBlocksAtEachBitRateBehaviour;

REGISTERED AS {g776.1Attribute 258};

numberOfImageBlocksAtEachBitRateBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the bit rates most utilized for facsimile compression and is useful in determining the particular demod/remod capabilities required.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 99999

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Number Received  Bearers

numberReceivedBearers	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.NumberReceivedBearers;

MATCHES FOR EQUALITY;

BEHAVIOUR	NumberReceivedBearersBehaviour;

REGISTERED AS {g776.1Attribute 259};

NumberReceivedBearersBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the number of received bearers.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Operational_Mode=Point_To_Point: 1; Operational_Mode=Multi_clique: 1-2;  Operational_Mode=Multi_destination 2-4;  Operational_Mode=Mixed: 3-4

The default value is:

Comment: For multi-clique mode, Number_Received_Bearers=1 assumes a Digital Cross-connect system is used external to the DCME to combine two sub-PRI receive cliques.

Reference: G.763”;

### Operational Mode

operationalMode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.OperationalMode;

MATCHES FOR EQUALITY;

BEHAVIOUR	operationalModeBehaviour;

REGISTERED AS {g776.1Attribute 71};

operationalModeBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the network configuration of the SPNE.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Point_To_Point, Multi_Clique, Multi_Destination, Mixed

The default value is:

Comment: Mixed is multi-clique where one clique operates in the multi-destination mode. Mixed is referred to as interoperation mode in G.763.

Reference: G.763   4”;

### OS Root Service ID

osRootServiceId ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY, ORDERING, SUBSTRINGS;

BEHAVIOUR	osRootServiceIdBehaviour;

REGISTERED AS { g776.1Attribute 420 };

osRootServiceIdBehaviour BEHAVIOUR

DEFINED AS

"The osRootServiceId is an attribute whose distinguished value can be used as a RDN when naming an instance of the osRootService object class.";

### Pack Maintenance State

packMaintenanceState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.PackMaintenanceState;

MATCHES FOR EQUALITY;

BEHAVIOUR	packMaintenanceStateBehaviour;

REGISTERED AS {g776.1Attribute 318};

packMaintenanceStateBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the maintenance state of a circuit pack.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Normal, Remove, Insert

The default value is:

Comment:

Reference:”;

### Peak Number Of Facsimile Banks Allocated

peakNumberOfFacsimileBanksAllocated	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PeakNumberOfFacsimileBanksAllocated;

MATCHES FOR EQUALITY;

BEHAVIOUR	peakNumberOfFacsimileBanksAllocatedBehaviour;

REGISTERED AS {g776.1Attribute 269};

peakNumberOfFacsimileBanksAllocatedBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the peak number of facsimile banks allocated during the most recent FSTI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 60

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Percent of Time FEC Activated

percentOfTimeFECActivated	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	percentOfTimeFECActivatedBehaviour;

REGISTERED AS {g776.1Attribute 270};

percentOfTimeFECActivatedBehaviour BEHAVIOUR

DEFINED AS

“ Fill-in

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.766 paragraph 10.1”;

### Performance Counts Clear State

performanceCountsClearState	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PerformanceCountsClearState;

MATCHES FOR EQUALITY;

BEHAVIOUR	performanceCountsClearStateBehaviour;

REGISTERED AS {g776.1Attribute 353};

performanceCountsClearStateBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the state of the facility performance counts.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Normal, Clear

The default value is:

Comment:

Reference:”;

### Pool Receive ID

poolReceiveID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PoolReceiveID;

MATCHES FOR EQUALITY;

BEHAVIOUR	poolReceiveIDBehaviour;

REGISTERED AS {g776.1Attribute 271};

poolReceiveIDBehaviour BEHAVIOUR

DEFINED AS

“ This parameter identifies which Receive Pool is being used.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

1,2

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.763”;

### Pool Size

poolSize	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PoolSize;

MATCHES FOR EQUALITY;

BEHAVIOUR	poolSizeBehaviour;

REGISTERED AS {g776.1Attribute 272};

poolSizeBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the size of the transmit (Pool Transmit ID) pools in terms of the number of 64 kbit/s time slots and the receive (Pool Receive ID) pools in terms of the number of 64 kbit/s time slots.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0 to 31

The default value is:

Comment: This parameter must be at least 1 for clique 1 transmit and receive. It is zero for clique 2 when there is not a clique. Any integer percentage can be real with higher resolution.

Reference: G.763”;

### Pool Transmit ID

poolTransmitID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PoolTransmitID;

MATCHES FOR EQUALITY;

BEHAVIOUR	poolTransmitIDBehaviour;

REGISTERED AS {g776.1Attribute 273};

poolTransmitIDBehaviour BEHAVIOUR

DEFINED AS

“ This parameter identifies which Transmit Pool is being used.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

1, 2

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.763”;

### PRI Alarm Delay

priAlarmDelay	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.AlarmDelay;

MATCHES FOR EQUALITY;

BEHAVIOUR	priAlarmDelayBehaviour;

REGISTERED AS {g776.1Attribute 15};

priAlarmDelayBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the delay value, in seconds, between the moment an alarm event is detected on an incoming receive bearer primary group and the moment the alarm condition is declared.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Real values of

0.0 to 3.0 in steps of 0.1

The default value is: 0.0

Comment:

Reference: G.763 paragraph 15.11.3, G.704”;

### PRI BPV Correct

priBPVProcessing	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	priBPVProcessingBehaviour;

REGISTERED AS {g776.1Attribute 401};

priBPVProcessingBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines how incoming Bipolar Violations (BPV's) are treated.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment: If enable is selected, any BPV's in the incoming signal are corrected such that the outgoing signal is free of BPV's. If disable is selected, incoming BPV's are re-inserted (passed) to the outgoing data stream. This attribute affects both SND and RCV transmission paths.

Reference:”;

### PRI E Bit Activation

priEBitActivation	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	priEBitActivationBehaviour;

REGISTERED AS {g776.1Attribute 40};

priEBitActivationBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines whether receiving CRC block errors set the corresponding E bit.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is: Disable

Comment: This feature must be disabled for multidestination mode.

Reference: G.706, G.763 paragraph 15.11”;

### PRI F Bit Error Correct

priFBitError	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	priFBitErrorBehaviour;

REGISTERED AS {g776.1Attribute 400};

priFBitErrorBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines how incoming f-bit errors in the Frame Alignment Signal (FAS) are treated.

This attribute is characteristic of ECE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment: If enable is selected, any errors in the incoming FAS are corrected such that the outgoing FAS is always correct. If disable is selected, errors detected in the incoming FAS are passed to the outgoing data stream. This attribute affects both the SND and RCV transmission paths.

Reference:”;

### PRI Frame Format

priFrameFormat	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PRIFrameFormat;

MATCHES FOR EQUALITY;

BEHAVIOUR	priFrameFormatBehaviour;

REGISTERED AS {g776.1Attribute 1};

priFrameFormatBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the super frame structure of the PRI.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

PRI_Rate=1544: SF, ESF; PRI_Rate=2048: CRC_On, CRC_Off

The default value is:

Comment: For PRI_Rate=1544: SF = superframe = G.704 12-Frame, ESF = Extended superframe = G.704 24 Frame. For PRI_Rate=2048: CRC_On means that CRC-4 is transmitted with the framing information.

Reference: G.704”;

### PRI ID

priID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PRIID;

MATCHES FOR EQUALITY;

BEHAVIOUR	priIDBehaviour;

REGISTERED AS {g776.1Attribute 37};

priIDBehaviour BEHAVIOUR

DEFINED AS

“ This parameter identifies the 1544 or 2048 kbit/s primary rate interface.

This attribute is characteristic of SPNE. It falls in the TMN Identification Category.

It is a Read Only attribute that has Integer values of

SPTT_Type=EC:1 to PRI_Number_Max; SPTT_Type=DCME: Tx_Bearer_PRI_1, ..., Tx_Bearer_PRI_4,Tx_Trunk_PRI_1,..., Tx_Trunk_PRI_12, Rx_Bearer_PRI_1, ..., Rx_Bearer_PRI_4, Rx_Trunk_PRI_1,..., Rx_Trunk_PRI_12

The default value is:

Comment: PRI = primary rate interface, Tx = transmit, Rx = receive. Any integer percentage can be real with higher resolution.

Reference:”;;

### PRI Line Code

priLineCode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PRILineCode;

MATCHES FOR EQUALITY;

BEHAVIOUR	priLineCodeBehaviour;

REGISTERED AS {g776.1Attribute 53};

priLineCodeBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines which line code is used for the PRI.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

PRI_Rate=1544:B8ZS, AMI, ZCS; PRI_Rate=2048:HDB3, CMI

The default value is:

Comment: B8ZS=bipolar with eight-zero substitution, AMI=alternate mark inversion, ZCS= zero code suppression, HDB3=high density bipolar of order 3, CMI=coded mark inversion

Reference: G.703”;

### PRI Line Impedance

priLineImpedance	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PriLineImpedance;

MATCHES FOR EQUALITY;

BEHAVIOUR	priLineImpedanceBehaviour;

REGISTERED AS {g776.1Attribute 54};

priLineImpedanceBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the PRI input impedance.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

PRI_Rate=2048: 75_Ohm, 110_Ohm, 120_Ohm

PRI_Rate=1544: 133_feet, 266_feet, 399_feet, 533_feet, 655_feet

The default value is:

Comment: For 2048 kbit/s the connection is unbalanced 75 Ohm or balanced 120 Ohm or balanced 110 Ohm cable. For 1544 kbit/s these are the equilizer settings in maximum feet to Reference TLP point (e.g. DSX)

Reference: G.703”;

### PRI Modified CRC4 Alignment

priModifiedCRC4Alignment	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	priModifiedCRC4AlignmentBehaviour;

REGISTERED AS {g776.1Attribute 392};

priModifiedCRC4AlignmentBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the modified CRC-4 multiframe alignment algorithm which allows automatic interworking between equipments with and without CRC-4 capability and it is assumed that an equipment with CRC-4 procedure always generates CRC-4.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment:

Reference: G.706 Annex B”;

### PRI Rate

priRate	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PRIRate;

MATCHES FOR EQUALITY;

BEHAVIOUR	priRateBehaviour;

REGISTERED AS {g776.1Attribute 74};

priRateBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the basic physical interface to the equipment of the primary rate interface.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

2048_Kbps, 1544_Kbps

The default value is:

Comment: The electrical physical interface of SPTT is usually at the primary group level. SPTT have the ability to perform 2048 kbit/s to 1544 kbit/s mappings, therefore it is required to define the physical interface to the trunk and bearer sides separately.

Reference: G.703”;

### PRI Signalling

priSignalling	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.PriSignalling;

MATCHES FOR EQUALITY;

BEHAVIOUR	priSignallingBehaviour;

REGISTERED AS {g776.1Attribute 358};

priSignallingBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines whether channel associated signaling is used (robbed bit for 1544 kbit/s and TS16 for 2048 kbit/s).

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

For 1544 kbit/s: rbs, dmi, none; for 2048 kbit/s: cas, none

The default value is:

Comment: When channel associated signalling is none: for a 2048 kbit/s interface, TS 16  is available for use to carry traffic. For 1544 kbit/s interfaces, there is no robbed-bit signalling. Other values are: ABCD - Signaling is used, all bits significant.

Reference: G.763”;

### Q50 Type

q50Type	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Q50Type;

MATCHES FOR EQUALITY;

BEHAVIOUR	q50TypeBehaviour;

REGISTERED AS {g776.1Attribute 275};

q50TypeBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the kind of switch that is controlling the Q.50 interface.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Type A (per Q.50 Supplement), Type A Simplified, Type B (per Q.50 Supplement), 4ESS, None

The default value is:

Comment: For 1544 kbit/s trunks, the line framing format should be ESF. For 2048 kbit/s trunks, channel 16 cannot be assigned if it is already used for other traffic.

Reference: G.763, Q.50”;

### Receive Voice Activity Detector

receiveVoiceActivityDetector	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	receiveVoiceActivityDetectorBehaviour;

REGISTERED AS {g776.1Attribute 75};

receiveVoiceActivityDetectorBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines whether the activity detector on the receive trunk side will be enabled.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment:

Reference: G.763”;

### Remote IT ID

remoteITID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.RemoteITID;

MATCHES FOR EQUALITY;

BEHAVIOUR	remoteITIDBehaviour;

REGISTERED AS {g776.1Attribute 277};

remoteITIDBehaviour BEHAVIOUR

DEFINED AS

“ This parameter identifies the transmit IT number of the corresponding DCME associated with a local IT and TC.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

1 to 360

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.763”;

### Reserve Clock Source

reserveClockSrc ATTRIBUTE

WITH ATTRIBURE SYNTAX G776.1ASN1DefinedTypesModule.ReserveClockSrc;

MATCHES FOR EQUALITY;

BEHAVIOUR reserveClockSrcBehaviour

REGISTERED AS {G776.1Attribute 428};

reserveClockSrcBehaviour BEHAVIOUR

DEFINED AS

“This parameter identifies the secondary (reseve) source clock”;

### Rx BC

rxBC	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.RxBC;

MATCHES FOR EQUALITY;

BEHAVIOUR	rxBCBehaviour;

REGISTERED AS {g776.1Attribute 279};

rxBCBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines a BC in the receive path.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

1 to 61

The default value is:

Comment: 1 to 61 for 2048 kbit/s primary group, 1 to 47 for a 1544 kbit/s primary group. Any integer percentage can be real with higher resolution.

Reference: G.763”;

### ScCTPId

scCTPId ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR scCTPIdBehaviour;

REGISTERED AS { g776.1Attribute 421 };

scCTPIdBehaviour BEHAVIOUR

DEFINED AS

“This attribute is used as an RDN for naming instances of the scCTP object class.”;

### Service Channel ID

serviceChId ATTRIBUTE --<NAME-CONV nameConv>--

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR	serviceChIDBehaviour;

REGISTERED AS { g776.1Attribute 422 };

serviceChIDBehaviour BEHAVIOUR

DEFINED AS

“The serviceChId is an attribute whose distinguished value can be used as a RDN when naming an instance of the serviceChannel object class. For SSPS, the value of this name is given by the OS in the installServiceChannel command, if specified; otherwise it will be given by the SSPS Platform in the response message of the installServiceChannel command. The serviceChannelID is a unique handle returned by the server. The address is used when sending messages to the Service, indentifying messages from the Service or referencing ports associated with the Service.”;

### Service Name

serviceName ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.ServiceName;

MATCHES FOR EQUALITY;

BEHAVIOUR serviceNameBehaviour;

REGISTERED AS { g776.1Attribute 423};

serviceNameBehaviour BEHAVIOUR

DEFINED AS

“An ASCII string that uniquely identifies a Service that has been previously been placed in the Service Library. The Service will be installed as a Service Channel.”;

### Service Template Name

serTemplateName  ATTRIBUTE --<NAME-CONV nameConv>--

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NameType ;

MATCHES FOR EQUALITY ;

BEHAVIOUR serTemplateNameBehaviour;

REGISTERED AS { g776.1Attribute 424 };

serTemplateNameBehaviour BEHAVIOUR

DEFINED AS

“ServiceFunction Template Name”;

### Severely Errored  Seconds Ratio Threshold

severelyErroredSecondsRatioThreshold	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	severelyErroredSecondsRatioThresholdBehaviour;

REGISTERED AS {g776.1Attribute 280};

severelyErroredSecondsRatioThresholdBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the threshold for sending an alarm event to the operator whenever the severely errored seconds ratio exceeds the set threshold.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Real values of

1 to 100

The default value is: 10

Comment: This real value represents percent (e. g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763”;

### Severely Errored Seconds Ratio

severelyErroredSecondsRatio	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	severelyErroredSecondsRatioBehaviour;

REGISTERED AS {g776.1Attribute 281};

severelyErroredSecondsRatioBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the ratio of severely errored seconds for the PRI.

This attribute is characteristic of SPNE. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: This Real value represents percent (e. g., 1 means 1%).

Reference: G.763 section 15.2.3”;

### Signalling Idle Code

signallingIdleCode	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.SignallingIdleCode;

MATCHES FOR EQUALITY;

BEHAVIOUR	signallingIdleCodeBehaviour;

REGISTERED AS {g776.1Attribute 244};

signallingIdleCodeBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the code that will be sent using the a,b,c,d bits of time slot no.16 of the trunk primary group of the incoming primary rate interface towards the switch when the CAS protocol is chosen as idle.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

0001 to 1111

The default value is:

Comment: The integer value represents a binary representation. Some switches require a fixed ABCD pattern even when CCS is used. Any integer percentage can be real with higher resolution.

Reference: G.763”;

### Signalling Transition

signallingTransition	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.SignallingTransition;

MATCHES FOR EQUALITY;

BEHAVIOUR	signallingTransitionBehaviour;

REGISTERED AS {g776.1Attribute 285};

signallingTransitionBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the translation between CAS signalling types.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

None, 2, 4, 16, Clear, Inverted, Zero and One

The default value is:

Comment: For each input code (0000 to 1111) there is a mapping to a output code. This allows translating between signalling bit definitions. For DCME, when USM option is disabled this parameter is not used.

Reference:”;

### SPNE ID

spneID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.SpneID;

MATCHES FOR EQUALITY;

BEHAVIOUR	spneIDBehaviour;

REGISTERED AS {g776.1Attribute 320};

spneIDBehaviour BEHAVIOUR

DEFINED AS

“ This string represents the ID for the equipment.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has String values of

Normal, Reload_from_non_volatile, Reload_from_backup

The default value is:

Comment:

Reference:”;

### Statistics Implementation

statisticsImplementation	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.G776.1Option;

MATCHES FOR EQUALITY;

BEHAVIOUR	statisticsImplementationBehaviour;

REGISTERED AS {g776.1Attribute 83};

statisticsImplementationBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines whether the terminal will perform statistical calculations for performance management display.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Enable, Disable

The default value is:

Comment:

Reference: G.763”;

### Statistics Time Interval

statisticsTimeInterval	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.TimeInterval;

MATCHES FOR EQUALITY;

BEHAVIOUR	statisticsTimeIntervalBehaviour;

REGISTERED AS {g776.1Attribute 289};

statisticsTimeIntervalBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the interval (referred to elsewhere as STI) over which various performance statistics are to be collected.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

10 to 60 in steps of 10 for DCME; 5, 15, 60

The default value is:

Comment: The integer value represents minutes. For DCME each statistic will be calculated every minute using results from every sampled DCME frame (such as every 10 th frame). Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 15.2.3 and G.826”;

### TC Assignment

tcAssignment	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.TCAssignment;

MATCHES FOR EQUALITY;

BEHAVIOUR	tcAssignmentBehaviour;

REGISTERED AS {g776.1Attribute 292};

tcAssignmentBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the type of trunk channel assignment on the bearer. (i.e. the rate at which the trunk channel will be assigned to: 64, 40 ,32, 24 or 16 Kb/s).

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

figuration, Dynamic, Preassigned_64Kbps, Preassigned_40Kbps, Preassigned_32Kbps, Preassigned_24Kbps, Preassigned_16Kbps (default is Out_Of_Configuration)

Value(s):

The default value is:

Comment:

Reference: G.763”;

### TC ID

tcID	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.TCID;

MATCHES FOR EQUALITY;

BEHAVIOUR	tcIDBehaviour;

REGISTERED AS {g776.1Attribute 293};

tcIDBehaviour BEHAVIOUR

DEFINED AS

“ This parameter identifies a 64 kbit/s transmission path between DCME and switch. A TC relates to one channel (time-slot) in a primary PCM group .This parameter identifies a (transmit and receive) port of the equipment and is designated by an integer number.

This attribute is characteristic of DCME. It falls in the TMN Identification Category.

It is a Read/Write attribute that has Integer values of

1 to 360

The default value is:

Comment: Any integer percentage can be real with higher resolution.

Reference: G.763”;

### Template Name

templateName ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.NameType;

MATCHES FOR EQUALITY;

BEHAVIOUR templateNameBehaviour;

REGISTERED AS { g776.1Attribute 425};

templateNameBehaviour BEHAVIOUR

DEFINED AS

“Name of the template within a file that is loaded into SSPS thru the tftp command from OS. It contains API used for DeviceFunctionDefinition and ServiceFunction Definition.;

### Transmit Onhook

transmitOnhook	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.;

MATCHES FOR EQUALITY;

BEHAVIOUR	transmitOnhookBehaviour;

REGISTERED AS {g776.1Attribute 296};

transmitOnhookBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines a situation where due to abnormal trans-hybrid losses and even when the trunk is in the "on-hook" state, some energy may appear at the speech detector at high enough levels to activate the speech detector. The transmit on-hook scheme allows the state of the channel to be identified through a specific pattern of the 2 signalling bits of the 4-bit signalling scheme for 1544 and 2048 kbit/s trunks. In this case, "transmit" refers to the direction into the CME from the channellize side.

This attribute is characteristic of CME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

A/B: 0/0, 0/1, 1/0, 1/1, 0/don't care, 1/don't care, don't care/0, don't care/1, always on-hook, always off-hook

The default value is: always off-hook

Comment: The on-hook signalling scheme should be set to "always off-hook" when common channel signalling is used in an application to prevent degraded performance, for example clipping of speech or transmission errors.

Reference:”;

### TS 16 Usage

ts16Usage	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.TS16Usage;

MATCHES FOR EQUALITY;

BEHAVIOUR	ts16UsageBehaviour;

REGISTERED AS {g776.1Attribute 300};

ts16UsageBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the usage of a 2048Kb/s trunk primary group time slot no. 16.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

CAS, CCS, Traffic

The default value is:

Comment:

Reference: G.763”;

### Tx BC

txBC	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.TXBC;

MATCHES FOR EQUALITY;

BEHAVIOUR	txBCBehaviour;

REGISTERED AS {g776.1Attribute 302};

txBCBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines a BC in the transmit path.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

1 to 61

The default value is:

Comment: 1 to 61 for 2048Kb/s primary group, 1 to 47 for a 1544Kb/s primary group. Any integer percentage can be real with higher resolution.

Reference: G.763”;

### USM Interval

usmInterval	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.USMInterval;

MATCHES FOR EQUALITY;

BEHAVIOUR	usmIntervalBehaviour;

REGISTERED AS {g776.1Attribute 303};

usmIntervalBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the time interval (in 2ms DCME frame units) between two consecutive signalling messages sent through the bearer to the corresponding side. Each signalling message contains signalling information relating to two trunk channels.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Integer values of

2 to 16

The default value is: 8

Comment: Any integer percentage can be real with higher resolution.

Reference: G.763”;

### USM Type

usmType	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.USMType;

MATCHES FOR EQUALITY;

BEHAVIOUR	usmTypeBehaviour;

REGISTERED AS {g776.1Attribute 304};

usmTypeBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines which type of channel associated transition signalling is employed on a primary group: R1 or R2D

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

R1, R2D

The default value is:

Comment:

Reference: G.763”;

### Voice Activity Ratio

voiceActivityRatio	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	voiceActivityRatioBehaviour;

REGISTERED AS {g776.1Attribute 307};

voiceActivityRatioBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the percentage of TCs which are voice-active to those which are incoming to the terminal.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Integer values of

0 to 100

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763 paragraph 15.2.3”;

### Voice Average Bits Per Sample

voiceAverageBitsPerSample	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.VoiceAverageBitsPerSample;

MATCHES FOR EQUALITY;

BEHAVIOUR	voiceAverageBitsPerSampleBehaviour;

REGISTERED AS {g776.1Attribute 308};

voiceAverageBitsPerSampleBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the average voice quality provided by the terminal during the most recent STI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

2 to 4 in steps of .01

The default value is:

Comment:

Reference: G.763 Table 11”;

### Voice DLC On Ratio

voiceDLCOnRatio	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	voiceDLCOnRatioBehaviour;

REGISTERED AS {g776.1Attribute 309};

voiceDLCOnRatioBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the percentage of time that DLC for voice or voice-band data is active during the most recent STI.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%).

Reference: G.763 section 15.2.3”;

### Voice Freezeout Excess

voiceFreezeoutExcess	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	voiceFreezeoutExcessBehaviour;

REGISTERED AS {g776.1Attribute 310};

voiceFreezeoutExcessBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the percent of 1 minute intervals in the most recent STI that the Voice Queue Freezeout Fraction for that 1 minute interval exceeded .5%.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763 section 15.2.3”;

### Voice Or Data

voiceOrData	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.VoiceOrData;

MATCHES FOR EQUALITY;

BEHAVIOUR	voiceOrDataBehaviour;

REGISTERED AS {g776.1Attribute 313};

voiceOrDataBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines whether a DS0 is a voiceband or digital data channel.

This attribute is characteristic of DCME. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

Voice, Data

The default value is:

Comment: This parameter can also be flipped by the Q.50 interface.

Reference:”;

### Voice Queue Freezeout Fraction

voiceQueueFreezeoutFraction	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.Percentage;

MATCHES FOR EQUALITY;

BEHAVIOUR	voiceQueueFreezeoutFractionBehaviour;

REGISTERED AS {g776.1Attribute 314};

voiceQueueFreezeoutFractionBehaviour BEHAVIOUR

DEFINED AS

“ This parameter provides the ratio of competitive clip duration to voice spurt duration in the most recent STI.  Ideally, this parameter would be 0.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Real values of

0 to 100 in steps of .001

The default value is:

Comment: The integer value represents percent (e.g., 1 means 1%). Any integer percentage can be real with higher resolution.

Reference: G.763 section 15.2.3”;

### Voiceband Coding Algorithm

voicebandCodingAlgorithm	ATTRIBUTE

WITH ATTRIBUTE SYNTAX 	G776.1ASN1DefinedTypesModule.VoicebandCodingAlgorithm;

MATCHES FOR EQUALITY;

BEHAVIOUR	voicebandCodingAlgorithmBehaviour;

REGISTERED AS {g776.1Attribute 315};

voicebandCodingAlgorithmBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the Voiceband Coding Algorithm of a given signal.

This attribute is characteristic of SPNE. It falls in the TMN Configuration Category.

It is a Read/Write attribute that has Enumerated Option values of

8-bit A_ law G.711 PCM,8-bit Mu_ law G.711 PCM, 5,4,3,2-bit G.726 ADPCM, 5,4,3,2-bit G.727 ADPCM

The default value is:

Comment: This is the parameter the user sets on controlable channel.

Reference: G.711, G.726, G.727, G.761,G.762,G.763, G.766”;

### Voiceband Coding Algorithm In Use

voicebandCodingAlgorithmInUse	ATTRIBUTE

WITH ATTRIBUTE SYNTAX G776.1ASN1DefinedTypesModule.VoicebandCodingAlgorithmInUse;

MATCHES FOR EQUALITY;

BEHAVIOUR	voicebandCodingAlgorithmInUseBehaviour;

REGISTERED AS {g776.1Attribute 316};

voicebandCodingAlgorithmInUseBehaviour BEHAVIOUR

DEFINED AS

“ This parameter defines the coding algorithm currently in use.

This attribute is characteristic of DCME. It falls in the TMN Performance Category.

It is a Read Only attribute that has Enumerated Option values of

8_bit_G711_PCM, 5_bit_G726_ADPCM, 4_bit_G726_ADPCM, 3_bit_G726_ADPCM, 2_bit_G726_ADPCM, 5_bit_G727_ADPCM, 4_bit_G727_ADPCM,  3_bit_G727_ADPCM, 2_bit_G727_ADPCM

The default value is:

Comment: The user retrieves the algorithm the equipment is currently using via this parameter. For DCME, this parameter is per channel per DCME frame.

Reference:”;