G776.1

Appendix I

SPNE Tutorial

(This appendix does not form an integral part of this Recommendation)

Table of Contents

1.0  Overall Context

2.0  Primary Rate Interface Related Nomenclature

3.0  Echo Canceller

3.1  General

3.2  Echo Path Capabilities

3.2.1  Distant End Echo Cancellation

3.3  Data Tone Disablers

4.0  Digital Circuit Multiplication Equipment (DCME)

4.1  General:

4.2  Compression:

4.3  Dynamic Load Control:

4.4  Application Configurations:

4.5  DCME Synchronization

5.0  Signal/Service Processing System (SSPS)

5.1  General

# 1.0  Overall Context

This Tutorial Annex is included to provide users of the G.776.1 documentation with an overview of  Signal Processing Network Element applications and a description of their capabilities using the G.776.1 parameter names. A discussion on specific and unique terminology is also included.

Note that several of the attributes include a large number of possible values. These options have been incorporated to ensure that all possible cases of implementation have been covered for TMN purposes. This does not imply that all of these values should be available from a given SPNE, nor that it corresponds to recommended utilization.

# 2.0  Primary Rate Interface Related Nomenclature

Because of the different SPNE vendor nomenclature, equipment functions and equipment applications, several parameters are associated to that of the PRI. The PRIs may be of two different rates: 1.544 Mbps, for a DS1, and 2.048 Mbps, for an E1. Each of these two rates is formed with a fixed multiple of 64 kbps bi-directional channels referred to as DS0s. Related to the DCME function, a trunk channel (TC) is an unidirectional channel within a DS1 or E1 with a bit rate of 64 kbps; two of these channels (transmit and receive) conform a DS0. A bearer channel (BC) in found on the compressed side of the equipment and is an unidirectional transmission path that may have any of the following instantaneous bit rates: 64, 40, 32, 24, or 16 kbps. A bearer is a collection of BCs; it is the name generally given to the compressed facility link

# 3.0  Echo Canceller

This section provides a basic definition of an echo canceller. Additionally, terminology specific to echo cancellers which may need clarification is discussed.

## 3.1  General

An echo canceller is a signal processing function placed in the four-wire portion of a circuit, which reduces the echo by subtracting an estimated echo from the actual echo. See ITU-T Recommendation G.168 for further information.

## 3.2  Echo Path Capabilities

Historically, echo cancellers echo paths have been typically between 32 and 64ms up to a maximum of 128ms. This resulted from the fact that the service providers deployed echo cancellers relatively close to the echo source. Figure 3.2.1 depicts this typical application. However, with the advent of Digital Cellular systems (which have delays in excess of 200ms), echo paths capacities have increased significantly. Additionally, some service providers have begun to deploy echo cancellers in a central location to cancel the echo that originates physically very large distances away (these large distances can equate to echo paths of over 1000ms in length). This is called Distant-End echo cancellation. See sub-section 3.2.1. and Figure 3.2.2.

Actual implementation of echo canceller devices has historically contained only one echo canceller (i.e. one cancelled and non-cancelled side known as a half echo canceller) per channel. See Figure 3.2.1. Again more recently, echo canceller devices have appeared in the network which have two echo cancellers on a single device. In this case, the device has two cancelled sides with both non-cancelled sides electrically connected to each other. This physical implementation (back-to-back, co-located echo cancellers known as a full echo canceller) reflects actual although not typical echo canceller deployment in some networks. Figure 3.2.2 is a full echo canceller deployment.

To accommodate this physical implementation of a full echo canceller, G.776.1 has attributes to provision both echo cancellers. Whenever an attribute is available for the second echo canceller, the term “Far” is appended to the attribute name to create a second attribute. For example, there are two attributes for the setting the echo path capacity: EC_Processing_Capability and EC_Processing_Capability_Far. Although it is arbitrary, in a full echo canceller, EC_Processing_Capability_Far would typically be used for the Distant-End application.

### 3.2.1  Distant End Echo Cancellation

The terms EC_Processing_Capability and EC_Offset_Delay are separate but related terms used to define the echo path capacity of an echo canceller. EC_Processing_Capability defines the portion of the echo path capacity where echo signals are cancelled.  Within this region, active echo cancellation is occurring.  All echoes which appear within the EC_Processing_Capability will be cancelled. EC_Offset_Delay on the other hand defines a fixed delay which is inserted ahead of the EC_Processing_Capability portion of the echo path capacity.  The EC_Offset_Delay, in effect, extends the effective echo path capacity of the echo canceller.  No echo cancellation occurs within this portion of the echo path.  EC_Offset_Delay is used in applications where there is a known, relatively fixed delay (within the range of the EC_Offset_Delay_Max) in the echo path such that the echo can be located within a known EC_Processing_Capability (e.g. 32ms or 64ms).  An example of such an application is satellite.

In geosynchronous satellite applications, the round trip delay is known (520ms for a single hop circuit).  See figure 3.2.1. Additionally, it is known for certain that the echo will appear after 520ms and that no echo will appear within that 520ms.  Figure 3.2.2 illustrates that a typical echo canceller with an EC_Processing_Capability of 32ms cannot possibly treat the echo which occurs after 520ms. With this in mind an echo canceller could be applied which utilizes EC_Offset_Delay in conjunction with EC_Processing_Capability.  For this application, an EC_Offset_Delay of 520ms or less would be used.  The value of EC_Processing_Capability used is dependent on the amount of EC_Offset_Delay which is used. Assuming that a 512ms EC_Offset_Delay is used, a service provider may then choose to use an EC_Processing_Capability of 32ms or 64ms (typical application values). For a 64ms EC_Processing_Capability, see figure 3, the echo canceller has an effective echo path capacity of 576ms (512ms +64ms) even though no echo processing is occurring in the first 512ms.  If an EC_Offset_Delay of 512ms was not available, the next lowest value of EC_Offset_Delay would be used and the EC_Processing_Capability would be increased to provide an overall 576ms of echo path coverage.

The terms EC_Processing_Capability_Max and EC_Offset_Delay_Max define the maximum values available on the particular echo canceller version. An echo canceller may be selected for use by an administration for multiple applications in which case the value of EC_Processing_Capability_Max and EC_Offset_Delay_Max could be significantly larger than the normal or typically expected values. In all cases, the values of EC_Processing_Capability and EC_Offset_Delay will always be less than or equal to these maximum values.

Figure 3.2.1

Figure 3.3.2

## 3.3  Data Tone Disablers

Most commercially available echo cancellers provide multiple modes of data tone disabler operation. These different modes of operation determine upon which signals (a.k.a. tones) the echo canceller will disable its echo canceller function.

G.165 and G.168 both specify that an echo canceller will disable its cancellation function upon receipt of a 2100Hz tone with periodic phase reversals per ITU Recommendation V.25. This is commonly referred to as the G165 mode of operation. Typically, this type of disable tone is sent by higher speed (9.6 kbit/s) modems (e.g., V.32, V.32bis, and V.34). Additionally, it is common to find that echo cancellers also offer another mode of operation commonly referred to as the G.164 mode of operation. In this mode of operation, the echo canceller will disable its cancellation function upon receipt of a 2100 Hz tone with or without phase reversals (per V.25). Typically, the non-phase reversal disable tone is sent by lower speed (9.6 kbit/s) modems and most facsimile machines. This mode of operation originates from G.164 which is the ITU recommendation on Echo Cancellers. The complexities of passing voice band data through an echo canceller error free have lead to acceptance of multiple modes of echo canceller operation although the G.164 mode of operation is beginning to fade in use.

See ITU Recommendation G.168, Appendix A, Section 4 for more discussion on this subject.

# 4.0  Digital Circuit Multiplication Equipment (DCME)

## 4.1  General:

Digital Circuit Multiplication Equipment are network transmission elements designed for the compression of switched voiceband traffic. They are used between International Switching Centers (ISCs) to achieve bandwidth savings over the different types of transmission facilities: fiber and copper cable, satellite, and microwave radio.  Signal processing functions allow for the proper treatment of the incoming traffic. In general, these functions consist of a trunk activity detector, a traffic signal discriminator, and an coder/decoder or a modem function depending on the type of traffic. The traffic is multiplexed together with the assistance of a process known as digital speech interpolation (dsi). Traffic congestion control  is implemented between the connecting switches and the DCME to maintain a chosen level of traffic quality. Additionally, the DCME includes other functions such as traffic statistics, synchronization, facility performance and alarms and hardware and/or software redundancy and alarms.

The descriptions included in this section refer to the DCME capabilities addressed in Recommendation G.763.

## 4.2  Compression:

Compression is achieve through the selective implementation of different signal processing functions. The sequence of these functions and the availability of the resources to implement them (whether dedicated or pooled) varies slightly from manufacturer to manufacturer.

Trunk activity detection: this process is used to detect the presence or absence of traffic activity on each of the trunk channels. It is usually associated with Digital Speech Interpolation (DSI), a function that connects a trunk channel to a bearer channel only when activity is actually present. In speech, for example, it allows for a gain of about 2.5 : 1.

The activity detection threshold may be programmed to occur at a fixed or variable energy level; hangover, the amount of time it takes for the bearer channel to be released after no activity has been detected, varies with the type of traffic and it may be programmable. The activity detection process also measures the transmit idle channel noise level and encodes the information that is then sent to the receive unit that regenerates and inserts the noise when the trunk channel is disconnected from the bearer channel.

Traffic signal discrimination: this process is used on a per trunk channel to distinguish between voiceband data and speech for optimized processing and encoding. In addition, a further refinement is done to detect the presence of ITU-T Signalling System No. 5  line signalling.

Encoding/Decoding: to further increase bandwidth efficiency, DCME utilizes several coding schemes to reduce the number of bits used for transmission. With input from the traffic signal discriminator, and depending on the amount of traffic load, signals are encoded at different rates of ADPCM (40, 32, 24 or 16 kbps) found in ITU Recommendation G.726.  For example, voiceband data signals are generally encoded at 40 kbps; speech signals receive 32 kbps, however, during periods where there are more active trunks than bearer channels, the encoding rate for speech may be reduced to 24 or even 16 kbps. This is done by selecting an appropriate encoder, communicating the change to the far-end and rearranging the bearer to accommodate for the new type of bandwidth demands. Codec resource implementation varies from vendor to vendor: sometimes they are dedicated, sometimes they are pooled. A condition called freeze-out may occur when the instantaneous number of codecs have been exceeded or the instantaneous number of active trunk channels exceeds the available number of bearer channels. It reflects itself as missing segments of information and, if present, it may affect the different types of traffic.

Facsimile Demodulator/Remodulator: this process further discriminates facsimile traffic from voiceband data to achieve higher compression gains than the one introduced by using ADPCM on the analog-modulated signal. It recovers the baseband digital signal from the facsimile machine, transmits it over the bearer and remodulates it using an inverse process at the DCME receive unit. Compression ratios are about tripled with the use of this mechanism.

## 4.3  Dynamic Load Control:

DLC reduces the probability of extreme congestion by notifying the connecting ISC that traffic overload conditions exist and not to route any new calls to that DCME. The process compares provisioned traffic related thresholds with those being measured by the DCME. These measurements include the average number of encoding bits, the average data bearer occupancy and the number of on-demand clear channels. When any of the thresholds are exceeded, in either an upward or downward direction, the DCME communicates with the ISC in accordance with Recommendation Q.50.

## 4.4  Application Configurations:

DCME applications may be divided into point-to-point and point-to-multi-point:

Point-to-point:

In this applications, the most used, all available bearer bandwidth is used to compress traffic between two given destinations.  See Figure 4.4.1.

Point-to-multi-point:

Two applications are available: multi-clique and multi-destination:

In multi-clique applications, the bearer channels are subdivided into independent pools of fixed capacity (cliques). Even though they share the same physical termination at the DCME they generally have different destinations. The clique separation is done outside the DCME by other network equipment.  See Figure 4.4.2.

Multi-destination applications are implemented solely on satellite networks. This is because multi-destination is based on the broadcast nature of  a satellite transmitted signal. Compressed transmission capacity from a DCME  is shared amongst traffic with different destinations. At the receive location, each DCME selects its corresponding traffic.  See Figure 4.4.3.

The applications differ in that for multi-clique compressed bandwidth is fixed and independent for each of the cliques, whereas that for multi-destination compressed bandwidth on the transmit direction. In addition, the transmit and receive directions of a multi-destination application utilize different facility paths to and from each of the DCMEs.

## 4.5  DCME Synchronization

The selection of synchronization clock sources and the usage of elastic/slip buffers in DCME is explained in this section. A number of system synchronization examples can be found in Section I.6 of Appendix I of Recommendation G.763.

The uniqueness in the synchronization of DCME has to do with the potential implementation of plesiochronous slip buffers. These types of buffers may be used when a pair of DCMEs are connected to two independent networks. Their function is to absorb the difference in clock frequencies between the two networks. The buffers can be implemented in transmission terminals (e.g. optical fiber or satellite transmission systems), on the international switching system, or on the DCME. Another possibility is the use of Doppler absorbing buffers. In this case, the elastic buffers absorb the short-term clock drifts caused by satellite movement.

There are four potential elastic/slip buffer positions for DCME as indicated in Figure I-1, namely the Trunk In Buffer, Bearer Out Buffer, Bearer In Buffer and Trunk Out Buffer. There are also four potential clock signals to be used to read out the data from the corresponding buffers, namely Trunk In Clock, Bearer Out Clock, Bearer In Clock and Trunk Out Clock. It is assumed that all buffers will derive their write clocks from the input bit streams (refer to Section I.6 of Appendix I of Recommendation G.763). (In actual DCMEs, not all potential buffers and synchronization clocks are implemented by the manufacturers.)

Figure I-1

Potential Elastic/Slip Buffers and Corresponding

Synchronization Clocks

Potential clock sources for synchronization are:

- Trunk input bit-streams (TX Trunk PRI, 1:12),

- Bearer input bit-streams (RX Bearer PRI, 1:4),

- External Clock inputs (External PRI, 1:2),

and

- Internal Clock (Internal Clock).

To enhance the reliability of operation, clock sources may be designated as Master Clock Source or as Backup Clock Source. The  maximum number of Backup Clock Sources varies with the design of the DCME.

# 5.0  Signal/Service Processing System (SSPS)

## 5.1  General

SSPS is a digital signal processing/software platform equipment that allows for the implementation of network signal processing functions such as digital circuit multiplication, echo cancellation and/or automatic level control. For operation, SSPS requires its own set of attributes, such as those used to manipulate the aforementioned functions.