|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

| ITU-T | G.776.3 |
| --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU | (04/2000) |

SERIES G: TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

Digital transmission systems – Terminal equipments – Operations, administration and maintenance features of transmission equipment

ADPCM DCME configuration map report

ITU-T  Recommendation  G.776.3

(Formerly  CCITT  Recommendation)

ITU-T  G-SERIES  RECOMMENDATIONS

TRANSMISSION SYSTEMS AND MEDIA, DIGITAL SYSTEMS AND NETWORKS

| For further details, please refer to ITU-T List of Recommendations. |  |
| --- | --- |
| INTERNATIONAL TELEPHONE CONNECTIONS AND CIRCUITS | G.100–G.199 |
| INTERNATIONAL ANALOGUE CARRIER SYSTEM |  |
| GENERAL CHARACTERISTICS COMMON TO ALL ANALOGUE CARRIER-TRANSMISSION SYSTEMS | G.200–G.299 |
| INDIVIDUAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON METALLIC LINES | G.300–G.399 |
| GENERAL CHARACTERISTICS OF INTERNATIONAL CARRIER TELEPHONE SYSTEMS ON RADIO-RELAY OR SATELLITE LINKS AND INTERCONNECTION WITH METALLIC LINES | G.400–G.449 |
| COORDINATION OF RADIOTELEPHONY AND LINE TELEPHONY | G.450–G.499 |
| TESTING EQUIPMENTS |  |
| TRANSMISSION MEDIA CHARACTERISTICS | G.600–G.699 |
| DIGITAL TRANSMISSION SYSTEMS |  |
| TERMINAL EQUIPMENTS | G.700–G.799 |
| General | G.700–G.709 |
| Coding of analogue signals by pulse code modulation | G.710–G.719 |
| Coding of analogue signals by methods other than PCM | G.720–G.729 |
| Principal characteristics of primary multiplex equipment | G.730–G.739 |
| Principal characteristics of second order multiplex equipment | G.740–G.749 |
| Principal characteristics of higher order multiplex equipment | G.750–G.759 |
| Principal characteristics of transcoder and digital multiplication equipment | G.760–G.769 |
| Operations, administration and maintenance features of transmission equipment | G.770–G.779 |
| Principal characteristics of multiplexing equipment for the synchronous digital hierarchy | G.780–G.789 |
| Other terminal equipment | G.790–G.799 |
| DIGITAL NETWORKS | G.800–G.899 |
| DIGITAL SECTIONS AND DIGITAL LINE SYSTEM | G.900–G.999 |

ITU-T RECOMMENDATION G.776.3 ADPCM DCME CONFIGURATION MAP REPORT

Summary This Recommendation specifies the configuration report for Recommendations G.763 [7]/G.766 DCME [8] and the associated TMN parameters found in Recommendation G.776.1 [9]. The report presents the local DCME configuration map and highlights those parameters affecting system and traffic interoperability. Standardized configuration reports greatly assist field personnel in the end-to-end provisioning and trouble-shooting of DCME systems.

Source ITU-T Recommendation G.776.3 was prepared by ITU-T Study Group 15 (1997-2000) and was approved under the WTSC Resolution No. 1 procedure on 4 April 2000.

FOREWORD

ITU (International Telecommunication Union) is the United Nations Specialized Agency in the field of telecommunications. The ITU Telecommunication Standardization Sector (ITU-T) is a permanent organ of the ITU. The ITU-T is responsible for studying technical, operating and tariff questions and issuing Recommendations on them with a view to standardizing telecommunications on a worldwide basis.

The World Telecommunication Standardization Conference (WTSC), which meets every four years, establishes the topics for study by the ITU-T Study Groups which, in their turn, produce Recommendations on these topics.

The approval of Recommendations by the Members of the ITU-T is covered by the procedure laid down in WTSC Resolution No. 1.

In some areas of information technology which fall within ITU-T’s purview, the necessary standards are prepared on a collaborative basis with ISO and IEC.

NOTE

In this Recommendation, the expression "Administration" is used for conciseness to indicate both a telecommunication administration and a recognized operating agency.

INTELLECTUAL PROPERTY RIGHTS

The ITU draws attention to the possibility that the practice or implementation of this Recommendation may involve the use of a claimed Intellectual Property Right. The ITU takes no position concerning the evidence, validity or applicability of claimed Intellectual Property Rights, whether asserted by ITU members or others outside of the Recommendation development process.

As of the date of approval of this Recommendation, the ITU had received notice of intellectual property, protected by patents, which may be required to implement this Recommendation. However, implementors are cautioned that this may not represent the latest information and are therefore strongly urged to consult the TSB patent database.

  ITU  2000

All rights reserved. No part of this publication may be reproduced or utilized in any form or by any means, electronic or mechanical, including photocopying and microfilm, without permission in writing from the ITU.

CONTENTS

Page

1	Scope		1

2	References		1

3	Definitions		1

4	Abbreviations		2

5	Recommendation overview		2

6	Total configuration		3

6.1	Map Identification		3

6.2	Modes (Network and DCME)		4

6.3	Trunk Bit Streams		4

6.4	Bearer Bit Streams		5

6.5	Pools and Destinations		5

6.6	Trunk Channel Mapping		6

6.7	System Clocks		6

6.7.1	Clocking Method 1		7

6.7.2	Clocking Method 2		7

6.8	Signalling Interface		8

6.9	Fax Demodulation		8

6.10	Alarm Handling		9

6.11	Activity Management		11

6.12	DLC		11

6.13	Statistics		13

Annex A  List of interoperability parameters		13

Appendix I  Example G.776.3 printout		14

Recommendation G.776.3

ADPCM DCME CONFIGURATION MAP REPORT

(Geneva, 2000)

# 1	Scope

This Recommendation specifies the configuration report for ADPCM DCME (G.763 [7]/G.766 [8]) and the associated TMN parameters found in Recommendation G.776.1 [9].

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[1]	ITU-T Recommendation G.701 (1993), Vocabulary of digital transmission and multiplexing, and pulse code modulation (PCM) terms.

[2]	ITU-T Recommendation G.703 (1998), Physical/electrical characteristics of hierarchical digital interfaces.

[3]	ITU-T Recommendation G.704 (1998), Synchronous frame structures used at 1544, 6312, 2048, 8448 and 44 736 kbit/s hierarchical levels.

[4]	CCITT Recommendation G.706 (1991), Frame alignment and cyclic redundancy check (CRC) procedures relating to basic frame structures defined in Recommendation G.704.

[5]	CCITT Recommendation G.711 (1988), Pulse code modulation (PCM) of voice frequencies.

[6]	CCITT Recommendation G.726 (1990), 40, 32, 24, 16 kbit/s adaptive differential pulse code modulation (ADPCM).

[7]	ITU-T Recommendation G.763 (1998), Digital circuit multiplication equipment using G.726 ADPCM and digital speech interpolation.

[8]	ITU-T Recommendation G.766 (1996), Facsimile demodulation/remodulation for digital circuit multiplication equipment.

[9]	ITU-T Recommendation G.776.1 (1998), Managed objects for signal processing network elements.

[10]	ITU-T Recommendation M.3100 (1995), Generic network information model.

[11]	ITU-T Recommendation Q.50 (1997), Signalling between circuit multiplication equipments (CME) and international switching centres (ISC).

[12]	CCITT Recommendation X.721 (1992) | ISO/IEC 10165-2:1992, Information technology  Open Systems Interconnection  Structure of management information: Definition of management information.

# 3	Definitions

The definitions used in this Recommendation are used in the above-listed Recommendation as well.

# 4	Abbreviations

This Recommendation uses the following abbreviations:

ADPCM	Adaptive Differential Pulse Code Modulation

BC		Bearer Channel

BER		Bit Error Ratio

CAS		Channel Associated Signalling

CC		Control Channel

CRC		Cyclic Redundancy Check

dB		decibel

DCME		Digital Circuit Multiplication Equipment

DCN		Disconnect

DLC		Dynamic Load Control

E1		Digital signal level 1 at 2048 kbit/s

FCC		Facsimile Control Channel

FE		Far End

FEC		Forward Error Correction

ID		Identifier

IT		Intermediate Trunk

kbit/s		kilobits per second

Max		Maximum

Min		Minimum

ms		milliseconds

PCM		Pulse Code Modulation

PRI		Primary Rate Interface

Rx		Receive

STI		Statistics Time Interval

T1		Digital signal level 1 at 1544 kbit/s

TC		Trunk Channel

TCH		Transparent Circuit Handler

TS		Time Slot

Tx		Transmit

VBD		Voice-band Data

# 5	Recommendation overview

This Recommendation specifies the configuration report for Recommendations G.763 [7]/G.766 DCME [8] and the associated TMN parameters found in Recommendation G.776.1 [9]. The report presents the local DCME configuration map and highlights those parameters affecting system and traffic interoperability. Standardized configuration reports greatly assist field personnel in the

end-to-end provisioning and troubleshooting of DCME systems. These reports are used not only by local personnel, but exchanged between administrations at each end to validate the current configuration for interoperability.

The map data is italicized and bracketed (e.g. <data>). Parameters affecting system and traffic interoperability are shown in bold. Given their significance it is recommended that these parameters be presented in a similar manner in the actual implementation. To enhance useability of this Recommendation, these interoperability parameters are also listed in Annex A.

This Recommendation specifies the format of the configuration map printout of a G.763/G.766 DCME. The configuration map parameters in the report are divided into sections. Each section covers a specific subject. The parameters are arranged in tables. Appendix I contains an example of a configuration map report.

# 6	Total configuration

## 6.1	Map Identification

Map Identification

Create Information

Organization: <mapOrganization> Site Name: <mapSiteName> Map Name: <mapName> Creation Date: <mapCreationTime> S/W Version: <softwareNumber>

Update Information

Number of Updates: <mapNumberOfUpdates> Last Updated by: <mapLastUpdatedBy> Update Date: <mapUpdateDate>

Comments

## 6.2	Modes (Network and DCME)

Modes (Network and DCME)

Network Operational Mode:		<operationalMode>

| Clique # | DCME Mode | Fax Modulation | FEC | Channel Check Procedure | ADPCM 2bit Ovrld | Disconnect Idle Calls |
| --- | --- | --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) |

(1)	<cliqueID>

(2)	<dcmeMode>

(3)	<facsimileDemodulation>

(4)	<faxFECActivation>

(5)	<channelCheckProcedure>

(6)	<adpcm2bitOverload>

(7)	<disconnectIdleCalls>

## 6.3	Trunk Bit Streams

Trunk Bit Streams (2048 kbit/s)

Trunk Side Interface:	<priRate>

Line Impedance:			<priLineImpedance>

| Trunk # | CRC-4 | Modified CRC Alignment | E-bit Indicator |
| --- | --- | --- | --- |
| (1) | (2) | (3) | (4) |

(1)	<priID>

(2)	<priFrameFormat>

(3)	<priModifiedCRC4Alignment>

(4)	<priEBitActivation>

Trunk Bit Streams (1544 kbit/s)

Trunk Side Interface:	<priRate>

| Trunk # | Line Equalizer | Frame Format | Line Code |
| --- | --- | --- | --- |
| (1) | (2) | (3) | (4) |

(1)	<priID>

(2)	<priLineImpedance>

(3)	<priFrameFormat>

(4)	<priLineCode>

## 6.4	Bearer Bit Streams

Bearer Bit Streams (2048 kbit/s)

Bearer Side Interface:	<priRate>

2048 kbit/s Impedance:	<priLineImpedance>

2048 kbit/s Bearer Bit Streams:

| Bearer # | CRC-4 | Modified CRC Alignment | E-bit Indicator |
| --- | --- | --- | --- |
| (1) | (2) | (3) | (4) |

(1)  	<priID>

(2) 	<priFrameFormat>

(3) 	<priModifiedCRC4Alignment>

(4) 	<priEBitActivation>

Bearer Bit Streams (1544 kbit/s)

Bearer Side Interface:	<priRate>

| Bearer # | Line Equalizer | Frame Format | Line Code |
| --- | --- | --- | --- |
| (1) | (2) | (3) | (4) |

(1) 	<priID>

(2)	<priLineImpedance>

(3)	<priFrameFormat>

(4)	<priLineCode>

## 6.5	Pools and Destinations

Pools and Destinations

Destinations

| Dest. # | Rx Bearer | Destination Name | Abbreviated Name | Tx Pool | FE Dest. # |
| --- | --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) | (6) |

Tx Pool #

Pool Size:	<poolsize>

Bearer Channel Configuration Table

| BC # | Assignment |
| --- | --- |
| (7) | (8) |

Rx Pool #

Pool Size:	<poolsize>

Bearer Channel Configuration Table

| BC # | Assignment |
| --- | --- |
| (7) | (8) |

NOTE  For Tx Pool # and Rx Pool #, only special BCs that are not related to TCs are displayed.

(1) 	<destinationID>

(2) 	<priID>

(3)	<destinationName>

(4)	<destinationNameAbbreviated>

(5) 	<cliqueID>

(6)	<destinationNumberAtFarEnd>

(7) 	<bcID>

(8)	<bcAssignment> (For each bearer channel: <bcID>)

## 6.6	Trunk Channel Mapping

Trunk Channel Mapping

| TC | IT | Remote IT | Dest. # | Assignment | Tx BC | Rx BC |
| --- | --- | --- | --- | --- | --- | --- |
| (1) | (2) | (3) | (4) | (5) | (6) | (7) |

(1)  	<tcID>

(2) 	<itID>

(3) 	<remoteITID>

(4) 	<destinationID>

(5) 	<tcAssignment>

(6)	<txBC>

(7)	<rxBC>

## 6.7	System Clocks

Dependent on the manufacturer used, select the report format in 6.7.1 or 6.7.2.

### 6.7.1	Clocking Method 1

System Clocks

| Clock ID | Trunk In | Trunk Out | Bearer In |
| --- | --- | --- | --- |
| Main | (1) | (1) | (1) |
| Reserved | (1) | (1) | (1) |

(1)	<clockSource>

### 6.7.2	Clocking Method 2

System Clocks

| Priority | Trunk In Clock | Bearer In Clock |
| --- | --- | --- |
| 1 | (1) | (1) |
| 2 | (1) | (1) |
| 3 | (1) | (1) |
| 4 | (1) | (1) |

(1)  	<clockSourceGroupID>

Tx Trunk Clocks

| Priority In Clock Group | Trunk In Clock Group | Bearer In Clock Group | External Clock Group | Internal Clock |
| --- | --- | --- | --- | --- |
| 1 | (1) | (1) | (1) | (1) |
| 2 | (1) | (1) | (1) | not applicable |
| 3 | (1) | (1) | not applicable | not applicable |
| 4 | (1) | (1) | not applicable | not applicable |
| 5 | (1) | not applicable | not applicable | not applicable |
| 6 | (1) | not applicable | not applicable | not applicable |
| 7 | (1) | not applicable | not applicable | not applicable |
| 8 | (1) | not applicable | not applicable | not applicable |
| 9 | (1) | not applicable | not applicable | not applicable |
| 10 | (1) | not applicable | not applicable | not applicable |
| 11 | (1) | not applicable | not applicable | not applicable |
| 12 | (1) | not applicable | not applicable | not applicable |

(1) 	<clockSource>

Rx Bearer Clocks

| Priority in Clock Group | Trunk In Clock Group | Bearer In Clock Group | External Clock Group | Internal Clock |
| --- | --- | --- | --- | --- |
| 1 | (1) | (1) | (1) | (1) |
| 2 | (1) | (1) | (1) | not applicable |
| 3 | (1) | (1) | not applicable | not applicable |
| 4 | (1) | (1) | not applicable | not applicable |
| 5 | (1) | not applicable | not applicable | not applicable |
| 6 | (1) | not applicable | not applicable | not applicable |
| 7 | (1) | not applicable | not applicable | not applicable |
| 8 | (1) | not applicable | not applicable | not applicable |
| 9 | (1) | not applicable | not applicable | not applicable |
| 10 | (1) | not applicable | not applicable | not applicable |
| 11 | (1) | not applicable | not applicable | not applicable |
| 12 | (1) | not applicable | not applicable | not applicable |

(1) 	<clockSource>

## 6.8	Signalling Interface

Signalling Interface

| Trunk # | Idle Code (abcd) | Type | Q.50 Type |
| --- | --- | --- | --- |
| (1) | (2) | (3) | (4) |

DLC Off (per TS):	<q50aSimpDLConAbcd>

DLC On (per TS):	<q50aSimpDLConAbcd>

(1)	<priID>

(2)	<signallingIdleCode>

(3)	<casProtocol>

(4)	<q50Type>

## 6.9	Fax Demodulation

Fax Demodulation

| Clique # | Fax Demodulation | FEC |
| --- | --- | --- |
|  | (1) | (2) |

NSS-code Fax machines not to be demodulated

| Manufacturer Code | Machine Code |
| --- | --- |
| (3) | (4) |

Fax Selective Deactivation TCs

(5)

(1)	<facsimileDemodulation>

(2)	<faxFECActivation>

(3)	<nssNotSupportedManufacturerCode>

(4)	<nssNotSupportedMachineCode>

(5)	<facsimileDemodulationDS0>

## 6.10	Alarm Handling

Alarm Handling

Alarm Extension Mode:	<alarmExtensionMode>

Bearer backward alarm: 	<bearerBackwardAlarm>

Alarm Delay:

Trunk Set Delay:	<priAlarmDelay>

Trunk Reset Delay:	<priAlarmResetDelay>

Bearer Set Delay:	<priAlarmDelay>

Bearer Reset Delay:	<priAlarmResetDelay>

PCM (E1) Alarms

| Alarms | Classification |
| --- | --- |
| Failure of I/C primary group | (1) |
| CRC MFR on primary group | (1) |
| LBER on primary group | (1) |
| Trunk BS AIS Failure | (1) |
| Bearer BS AIS Failure | (1) |
| Trunk BS RAI (TS 0) | (1) |
| Bearer BS RAI | (1) |
| Trunk BS MFR alarm (TS 16) | (1) |
| Trunk BS remote MFR alarm (TS 16) | (1) |
| Abnormal circuit supervision on TC | (1) |
| Trunk BS HSLIP  Tx | (1) |
| Trunk BS LSLIP  Tx | (1) |
| Trunk BS HSLIP  Rx | (1) |
| Trunk BS LSLIP  Rx | (1) |
| Bearer BS HSLIP  Rx | (1) |
| Bearer BS LSLIP  Rx | (1) |

Alarm Handling

PCM (T1) Alarms

| Alarms | Classification |
| --- | --- |
| LOS  CFA on I/C primary group | (1) |
| LOF  CFA on I/C primary group | (1) |
| AIS  CFA on I/C primary group | (1) |
| YELLOW  CFA on I/C primary group | (1) |
| LOF or LOS  Defect on I/C primary group | (1) |
| AIS  Defect on I/C primary group | (1) |
| Abnormal circuit supervision on TC | (1) |
| HBER on bearer interface primary group | (1) |
| HBER on trunk interface primary group | (1) |
| LBER on primary group | (1) |
| BS HSLIP  Tx (Trunk) | (1) |
| BS LSLIP  Tx (Trunk) | (1) |
| BS HSLIP  Rx (Bearer) | (1) |
| BS LSLIP  Rx (Bearer) | (1) |

DCME CC Alarms

| Alarms | Classification |
| --- | --- |
| CC AIS failure | (1) |
| CC HBER | (1) |
| CC LBER | (1) |
| Loss of DCME Frame (LDFA) | (1) |
| Bearer backward alarm in DCME CC | (1) |

CC Performance Alarms

| Alarms | Classification |
| --- | --- |
| Bearer Average BER  per STI | (1) |
| Bearer BER excess  per STI | (1) |
| Bearer severely errored seconds  per STI | (1) |

(1)	<alarmSeverityClassification>

## 6.11	Activity Management

Activity Management

Data Activity Threshold:	<dataActivityThreshold>

Max Injected Rx Noise:		<injectedRxNoiseMax>

VBD Hangover

First data hangover value:	<dataHangoverFirst>

Second data hangover value:	<dataHangoverSecond>

## 6.12	DLC

DLC

DLC Mode:		<dlcMode>

Voice/VBD Thresholds

| Clique # | Average Bits per Sample |  | Data Bearer Occupancy |  |
| --- | --- | --- | --- | --- |
|  | High Load | Low Load | High Load | Low Load |
| (2) | (3) | (4) | (5) | (6) |

DLC timers

| Clique # | First Stage Averaging Period | Voice/VBD DLC Deactivation Timer |
| --- | --- | --- |
| (2) | (7) | (8) |

64 kbit/s DLC

| Clique # | 64 kbit/s Circuit Limit DLC Activation | 64 kbit/s DLC Deactivation Timer |
| --- | --- | --- |
| (2) | (9) | (10) |

| Destination # | DLC/TCH Interaction |
| --- | --- |
| (11) | (12) |

Destination Selective DLC

| Destination # | Allocated Bearer Nibbles |
| --- | --- |
| (11) | (13) |

(2)	<cliqueID>

(3)	<dlcVoiceHighLoadThreshold>

(4)	<dlcVoiceLowLoadThreshold>

(5) 	<dlcDataHighLoadThreshold>

(6) 	<dlcDataLowLoadThreshold>

(7)  	<dlcFirstAveragingPeriod>

(8) 	<dlcVoiceDeactivationTime>

(9) 	<dlc64kbpsThreshold>

(10)	<dlc64kbpsDeactivationTimer>

(11)	<destinationID>

(12)	<dlcTCHInteraction>

(13)	<dlcAllocatedBearerNibbles>

## 6.13	Statistics

Statistics

Statistics Implementation:	<statisticsImplementation>

| Characteristic | Definition |
| --- | --- |
| Statistics Time Interval | (1) |
| Fax Statistics Time Interval | (2) |

(1)	<statisticsTimeInterval>

(2)	<facsimileStatisticsTimeInterval>

Annex A

List of interoperability parameters

The parameters in 6.1 are not themselves required for interoperability. However, Administrations should use these parameters to ensure that the map in question is both correct and current.

The following parameters are critical to interoperability:

Network Operational Mode	<operationalMode>

DCME Mode				<dcmeMode>

Fax Modulation			<facsimileDemodulation>

FEC					<faxFECActivation>

ADPCM 2bit Ovrld		<adpcm2bitOverload>

Disconnect Idle Calls		<disconnectIdleCalls>

Bearer Side Interface		<priRate>

Bearer CRC-4			<priFrameFormat>

Bearer Modified CRC Alignment	<priModifiedCRC4Alignment>

Bearer E-bit Indicator		<priEBitActivation>

Bearer Frame Format		<priFrameFormat>

Bearer Line Format		<priLineCode>

Pool Size					<poolsize>

Bearer Channel Assignment	<bcAssignment> (for each bearer channel: <bcID>)

IT					<itID>

Remote IT				<remoteITID>

Tx BC					<txBC>

Rx BC					<rxBC>

Signalling Interface Type	<casProtocol>

Fax Demodulation			<facsimileDemodulation>

FEC					<faxFECActivation>

Fax Selective Deactivation TCs	<facsimileDemodulationDS0>

Data Activity Threshold	<dataActivityThreshold>

Max Injected Rx Noise		<injectedRxNoiseMax>

First data hangover value	<dataHangoverFirst>

Second data hangover value	<dataHangoverSecond>

Appendix I

Example G.776.3 printout

Following is an example printout. The information contained in the fields is fictitious but attempts to show a realistic example.

Map Identification

Create Information. Organization: Xanadu Telcom Site Name: Kublaville Map Name: Cubicon Map2 Creation Date: 23-June-1999 S/W Version: 2.3. Update Information. Number of Updates: 2 Last Updated by: Jill A. Comtek Update Date: 4-April-2000. Comments. This map is for the Kublaville, Xanadu to Cubicon Link 2 and was last updated to fix the configuration problems found when we exchanged Map updates (Thanks to G.776.3 printout!)..

Modes (Network and DCME)

Network Operational Mode: Point-to-Point Clique #; DCME Mode; Fax Modulation; FEC; Channel Check Procedure; ADPCM 2bit Ovrld; Disconnect Idle Calls. (1); G.763; Enable; Auto; Enable; Enable; Enable.

Cubicon Map2 Page 1

Trunk Bit Streams

Trunk Side Interface:; 2048 kbit/s. Line Impedance:; 75 ohms. Trunk #; CRC-4; Modified CRC Alignment; E-bit Indicator. 1; Disable; Disable; Disable. 2; Disable; Disable; Disable. 3; Disable; Disable; Disable. 4; Enable; Disable; Disable. 5; Enable; Disable; Disable.

Bearer Bit Streams (2048 kbit/s)

Bearer Side Interface:; 2048 kbit/s. 2048 kbit/s Impedance:; 75 ohms. 2048 kbit/s Bearer Bit Streams:. Bearer #; CRC-4; Modified CRC Alignment; E-bit Indicator. 1; Enabled; Disabled; Enabled.

Cubicon Map2 Page 2

Pools and Destinations

Destinations

Dest. #; Rx Bearer; Destination Name; Abbreviated Name; Tx Pool; FE Dest. #. 1; 1; Cubicon; Cbcn; 1; 1.

Tx Pool 1

Pool Size:; 1-31. NOTE  Dynamic mappings are excluded.. BC #; Assignment. 1; None.

Rx Pool 1

Pool Size:; 1-31. NOTE  Dynamic mappings are excluded.. BC #; Assignment. None.

Trunk Channel Mapping

TC; IT; Remote IT; Dest. #; Assignment; Tx BC; Rx BC. 1/1 – 1/31; 1-30; 1-30; 1; Dynamic; Dynamic; Dynamic. 2/1 – 2/31; 31-60; 31-60; 1; Dynamic; Dynamic; Dynamic. 3/1 – 3/31; 61-90; 61-90; 1; Dynamic; Dynamic; Dynamic. 4/1 – 4/31; 91-120; 91-120; 1; Dynamic; Dynamic; Dynamic. 5/1 – 5/31; 121-150; 121-150; 1; Dynamic; Dynamic; Dynamic.

Cubicon Map2 Page 3

System Clocks

Clock ID; Trunk In; Trunk Out; Bearer In. Main; Trunk 1; Trunk 1; Bearer 1. Reserved; Trunk 2; Trunk 2; Internal.

Signalling Interface

Trunk #; Idle Code (abcd); Type; Q.50 Type. 1; 1101; Idle; A. 2; 1101; Idle; A. 3; 1101; Idle; A. 4; 1101; Idle; A. 5; 1101; Idle; A.

Fax Demodulation

Clique #; Fax Demodulation; FEC. 1; Enabled; Enabled. NSS-code Fax machines not to be demodulated. Manufacturer Code; Machine Code. Garage Shop Fax; Model T. Fax Selective Deactivation TCs. 1, 16.

Cubicon Map2 Page 4

Alarm Handling

Alarm Extension Mode:; Per Time Slot. Bearer Backward Alarm:; Enabled. Trunk Alarm Delay:; 2 s; Bearer Alarm Delay:; 2 s. Trunk Alarm Reset Delay:; 10 s; Bearer Alarm Reset Delay:; 10 s. PCM (E1) Alarms. Alarms; Classification. Failure of I/C primary group; PMA. CRC MFR on primary group; PMA. LBER on primary group; DMA. Trunk BS AIS Failure; SA. Bearer BS AIS Failure; SA. Trunk BS RAI (TS 0); SA. Bearer BS RAI; SA. Trunk BS MFR alarm (TS 16); SA. Trunk BS remote MFR alarm (TS 16); PMA. Abnormal circuit supervision on TC; SA. Trunk BS HSLIP  Tx; MEI. Trunk BS LSLIP  Tx; MEI. Trunk BS HSLIP  Rx; MEI. Trunk BS LSLIP  Rx; MEI. Bearer BS HSLIP  Rx; PMA. Bearer BS LSLIP  Rx; MEI.

Cubicon Map2 Page 5

Alarm Handling

DCME CC Alarms. Alarms; Classification. CC AIS failure; SA. CC HBER; PMA. CC LBER; DMA. Loss of DCME Frame (LDFA); PMA. Bearer backward alarm in DCME CC; SA. CC Performance Alarms. Alarms; Classification. Bearer Average BER  per STI; SA. Bearer BER excess  per STI; PMA. Bearer severely errored seconds  per STI; DMA.

Activity Management

Data Activity Threshold [dBm]:; 35. Max Injected Rx Noise:; 50. VBD Hangover. First data hangover value [s]:; 10. Second data hangover value [s]; 100.

Cubicon Map2 Page 6

DLC

DLC Mode: Global. Voice/VBD Thresholds. Clique #; Average Bits per Sample; Data Bearer Occupancy. High Load; Low Load; High Load; Low Load. 1; 3.6; 3.0; 90; 70. DLC timers. Clique #; First Stage Averaging Period (ms); Voice/VBD DLC Deactivation Timer (Ta) [s]. 1; 128; 10. 64 kbit/s DLC. Clique #; 64 kbit/s Circuit Limit DLC Activation; 64 kbit/s DLC Deactivation Timer (Tb) [s]. 1; 30; 10. Destination #; DLC/TCH Interaction. 1; Enable. Destination Selective DLC. Destination #; Allocated Bearer Nibbles. 1; 62.

Cubicon Map2 Page 7

Statistics

Statistics

Statistics Implementation: Enabled. Characteristic; Definition. Statistics Time Interval [min]; 10. Fax Statistics Time Interval [min]; 10.

Cubicon Map2 Page 8

| ITU-T RECOMMENDATIONS SERIES |  |
| --- | --- |
| Series A | Organization of the work of the ITU-T |
| Series B | Means of expression: definitions, symbols, classification |
| Series C | General telecommunication statistics |
| Series D | General tariff principles |
| Series E | Overall network operation, telephone service, service operation and human factors |
| Series F | Non-telephone telecommunication services |
| Series G | Transmission systems and media, digital systems and networks |
| Series H | Audiovisual and multimedia systems |
| Series I | Integrated services digital network |
| Series J | Transmission of television, sound programme and other multimedia signals |
| Series K | Protection against interference |
| Series L | Construction, installation and protection of cables and other elements of outside plant |
| Series M | TMN and network maintenance: international transmission systems, telephone circuits, telegraphy, facsimile and leased circuits |
| Series N | Maintenance: international sound programme and television transmission circuits |
| Series O | Specifications of measuring equipment |
| Series P | Telephone transmission quality, telephone installations, local line networks |
| Series Q | Switching and signalling |
| Series R | Telegraph transmission |
| Series S | Telegraph services terminal equipment |
| Series T | Terminals for telematic services |
| Series U | Telegraph switching |
| Series V | Data communication over the telephone network |
| Series X | Data networks and open system communications |
| Series Y | Global information infrastructure and Internet protocol aspects |
| Series Z | Languages and general software aspects for telecommunication systems |